|  | United Nations | ECE/TRANS/WP.29/GRE/2026/16_NL |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 12 February 2026 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Ninety-fourth session

Geneva, 27-30 April 2026

Item 8 of the provisional agenda

UN Regulation No. 10 (Electromagnetic Compatibility)

Proposal for a Supplement to 07 series of amendments to UN Regulation No. 10 (Electromagnetic compatibility)

Submitted by the experts from the Informal Working Group on Electromagnetic Compatibility*

The text reproduced below was prepared by the experts from the Informal Working Group on Electromagnetic Compatibility (IWG EMC) with the aim to fit Automated Driving System (ADS) requirements into UN Regulation No.10. The proposed modifications to the existing text of the UN Regulation are marked in bold for new or strikethrough for deleted characters. Modifications to ECE/TRANS/WP.29/GRE/2026/16 in blue

I.	Proposal

Insert new paragraphs 0. and 0.1. to 0.1.3., to read:

"0.		Introduction

0.1.		Supplement [3] to the 07 series of amendments is introduced to take into account vehicles of categories X and Y 1, as well as conventional vehicles which are equipped with an Automated Driving System (ADS) 1.

0.1.1. 		The Regulation was originally drafted for vehicles with driver’s compartment and manual driving controls. It is the intention of this amendment to keep the spirit of the Regulation and to extend its application to vehicles without driver, driver’s compartment and without manual driving controls in the vehicle. In the absence of driver/driver’s compartment/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.2. 		In case of vehicles equipped with an ADS no special provisions or exemptions apply.

[0.1.3.		While performing the EMC tests, it is important that all sensors etc., used by the ADS to perform the DDT 1, and to perform non-DDT related tasks, if applicable, are operational and function as they would in real-life. The Regulation is amended to clarify this.

Vehicles equipped with an ADS can be tested using an ADS test configuration. The aim of the ADS test configuration is to enable the functioning of an ADS feature in the test facility. Different test configurations may be created for the different test conditions (e.g. for emissions, immunity testing). A test configuration may be achieved by e.g. modified software installed by the vehicle manufacturer to modify the ODD1 of the ADS feature for the purpose of running the test, systems installed in the test facility to simulate a camera view or to create a virtual distance for a radar, [to feed the ADS with simulated signals] etc. ]"

Paragraph 1.1, insert new footnote 1, amend to read:

"1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.68, para. 2"

Paragraph 1.3., amend to read:

"1.3.	It covers:

- Requirements regarding the immunity to radiated and conducted disturbances for functions related to direct control of the vehicle (including control by an ADS), related to driver, ADS, passenger and other road users' protection (if applicable), related to disturbances, which would cause confusion to the driver, ADS or other road users, related to vehicle data bus functionality, related to disturbances, which would affect vehicle statutory data;

…"

Insert new paragraph 2.9.4., to read:

“2.9.4.	Presence of a type of ADS feature”

Paragraph 2.12., add subparagraph (g) to read:

"2.12.	"Immunity related functions" are the following functions; this list is not exhaustive and shall be adapted to the technical evolution of vehicle/technology:

(a) 	Functions related to the direct control of the vehicle:

(i) 	By degradation or change in: e.g. engine, gear, brake, suspension, active steering, speed limitation devices;

(ii) 	By affecting drivers position, if applicable: e.g. seat or steering wheel positioning;

(iii) 	By affecting driver's visibility, if applicable: e.g. dipped beam, windscreen wiper, indirect vision systems, blind spot systems.

(iv) 	By affecting degradation or change of the ADS performance whilste an ADS feature is active; e.g. camera, radar.

(b) 	Functions related to driver, passenger and other road user protection:

(i) 	E.g. airbag and safety restraint systems, emergency calling systems;

(c) 	Functions which, when disturbed, cause confusion to the driver[, ADS] or other road users:

(i) 	Optical disturbances: incorrect operation of e.g. direction indicators, stop lamps, end outline marker lamps, rear position lamp, light bars for emergency system, wrong information from warning indicators, lamps or displays related to functions in Subparagraphs (a) or (b) which might be observed in the direct view of the driver.

(ii) 	Acoustical disturbances: incorrect operation of e.g. anti-theft alarm, horn.

[(iii) 	Disturbances in the logic signals transmitted to the ADS, if applicable.]

(d) 	Functions related to vehicle data bus functionality:

(i) 	By blocking data transmission on vehicle data bus-systems, which are used to transmit data, required to ensure the correct functioning of other immunity related functions.

(e) 	Functions which when disturbed affect vehicle statutory data: e.g. tachograph, odometer;

(f) 	Functions related to charging mode when coupled to the power grid:

(i) 	For vehicle test: by leading to unexpected vehicle motion;

(ii) 	For ESA test: by leading to an incorrect charging condition (e.g. over-current, over-voltage)."

(g) 	Functions, which result in degradation or change of the ADS performance.

Insert a new paragraph 2.34., to read:

“2.34.	“ADS test configuration” means a mode of operation of an ADS that enables the correct execution of the EMC test in the test facility, with the ADS feature active and representative of on-road behaviour.”

Insert a new paragraph 3.1.9., to read:

"3.1.9.	If a driver’s seat is fitted but it is not the front seat nearest to the opposing traffic, the applicant shall specify this seating position."

Insert a new paragraph 3.1.10., to read:

"3.1.10.	If remote operation is part of the ADS system, which can influence the longitudinal or lateral movement of the vehicle, this remote operation shall be included in the application for approval."

Insert a new paragraph 6.11. and its subparagraphs, to read:

"6.10.91.	Special requirements for vehicles equipped with an ADS

6.10.9.1.1.	If not otherwise specified, all requirements to this Regulation related to the “driver” or “manual operation”, assume that no “ADS feature” is active.

6.10.91.2.	The references to ADS features are only applicable whilste an ADS feature is active.

6.11.3.	ADS test configuration

The ADS test configuration for the immunity test, shall fulfil at least the following functional requirements:

(a)	It shall allow the vehicle to keep longitudinal control

(b)	It shall allow the vehicle to steer or keep the steering in the centre;

(c)	It shall allow the vehicle to be operated at the designated test facility

An ADS test configuration may be achieved by e.g. modified software installed by the vehicle manufacturer to modify the ODD1 of the ADS feature for the purpose of running the test, systems installed in the test facility to simulate a camera view or to create a virtual distance for a radar/lidar, etc.

The ADS test configuration shall not influence or manipulate the results of the testing. The ADS and other vehicle systems shall not be deactivated by the ADS test configuration, unless this is needed for safe operation on the vehicle dynamometer.

If deactivation of systems cannot be avoided, the manufacturer shall provide a list of deactivated devices and/or functionalities with a justification for the deactivation and a proposal how compliance with the provisions of this Regulation can be shown. This may be achieved by sub-system testing or component-based testing.

The ADS test configuration shall be subject to approval by the Type Approval Authority and its use shall be recorded and described in the test report. At the request of the Type Approval Authority, the correct functioning of the ADS test configuration shall be verified by means of checks performed on a chassis dynamometer including the monitoring of relevant on-board signals and/or the measurement of vehicle parameters."

Paragraph 9.3.2., amend to read:

"9.3.2.	When the conformity of a vehicle, component or ESA taken from the series is being verified, production shall be deemed to conform to the requirements of this Regulation in relation to immunity to electromagnetic radiation if the vehicle ESA does not exhibit any degradation relating to the direct control of the vehicle by the ADS or which could be observed by the driver or other road user when the vehicle is in the state defined in Annex 6, Paragraph 4., and is subjected to a field strength, expressed in V/m, up to 80% of the reference limits prescribed in Paragraph 6.4.2.1., and, if applicable, Paragraph 7.7.2.1. for vehicles and Paragraph 6.8.2.1. and, if applicable, Paragraph 7.18.2.1. for ESAs above."

Annex 2A, add new items 74. to 76., to read:

"74.	Automated Driving System (ADS): yes/no/optional1……………………………………

75.	For vehicles with an ADS, description of system operation (including any electronic parts) ……………………………………………………………………………….

76.	A driver’s seat is fitted: yes/no/optional1…………………………………………………

The driver’s seat is the front seat nearest to the opposing traffic: yes/no1……...

If not, the position is to be specified: ……………………………………………."

Annex 6, paragraph 2.1.1.1., amend to read:

“2.1.1.1.	The engine shall normally turn the driving wheels at a steady speed of 50 km/h if there is no technical reason due to the vehicle to define a different condition. For vehicles of categories L1 and L2 the steady speed shall normally be turned at 25 km/h. The vehicle shall be on an appropriately loaded dynamometer or alternatively supported on insulated axle stands with minimum ground clearance if no dynamometer is available. Where appropriate, transmission shafts, belts or chains may be disconnected (e.g. trucks, two- and three-wheel vehicles). For vehicles equipped with an ADS, the steering system(s) shall be activated and follow a smooth simulated continuous road path. Adjustments between +[30]° and –[30]° e.g. by using the ADS EMC Command Control Script. The simulated road path shall be selected in agreement between the manufacturer and Type Approval Authority. If the wheel steering angle is not attainable (because the steering angle available to the ADS is limited, or because full lock is reached first), the maximum attainable steering angle shall be used and shall be described in the test report. To allow the steering system to be active during the test, the axle may e.g. rest on insulated axle stands or the steering functionality may be mechanically disconnected from the wheels.”

Annex 6, paragraph 2.1.1.2., amend to read:

"2.1.1.2.	Basic Vehicle Conditions

The Paragraph defines minimum test conditions (as far as applicable) and failure criteria for vehicle immunity tests. Other vehicle systems, which can affect immunity related functions, shall be tested in a way to be agreed between manufacturer and Technical Service.

| "50 km/h mode" vehicle test conditions | Failure criteria |
| --- | --- |
| Vehicle speed 50 km/h (respectively 25 km/h for L1, L2 vehicles) 20 per cent (vehicle driving the rollers). or Vmax of the vehicle with the ADS feature activated – 5 km/h, when max speed with the ADS feature activated is lower than 50 km/h If the vehicle is equipped with a cruise control system, it shall be used to maintain the required constant vehicle speed and maintained without any deactivation. | Speed variation greater than 10 per cent of the nominal speed. In case of automatic gearbox: change of gear ratio inducing a speed variation greater than 10 per cent of the nominal speed. |
| Dipped beams ON (manual mode) | Lighting OFF (front light and rear light) |
| Specific warning (e.g Rotating/flashing light, signaling bar, siren…) ON | Specific warning OFF |
| Cluster operate in normal mode | Unexpected warning Inconsistent variation of the odometer |
| Rear view system | Unexpected movement of rear view mirror Loss or freezing of the display (CMS) |
| Front wiper ON (manual mode) maximum speed | Complete stop of front wiper |
| Direction indicator on driver's the side nearest to the opposing traffic ON | Frequency change (lower than 0.75 Hz or greater than 2.25 Hz). Duty cycle change (lower than 25 per cent or greater than 75 per cent). |
| Adjustable suspension in normal position | Unexpected significant variation |
| Driver's seat (if fitted) and steering wheel (if fitted) in medium position | Unexpected variation greater than 10 per cent of total range |
| Alarm unset | Unexpected activation of alarm |
| Horn OFF | Unexpected activation of horn |
| Airbag and safety restraint systems operational with inhibited passenger airbag if this function exists | Unexpected activation |
| Automatic doors closed | Unexpected opening |
| Adjustable endurance brake lever in normal position | Unexpected activation |
| Brake pedal not depressed | Unexpected activation of brake and unexpected activation of stop lights |
| ADS shall be operational but without ADS feature(s) activated (1) | ADS does not remain in a failure safe mode or expected failure operational mode |
| [ADS feature activated] | [Unexpected behaviour, any change in ADS operational functions] |
| (1): ADS are turned on by the driver but some or all ADS functions may revert to a mode where system is monitoring sensors but is not actively ‘driving’ the vehicle due to plausibility issues caused by the EMC laboratory environment. |  |

| "Brake mode" vehicle test conditions | Failure criteria |
| --- | --- |
| Vehicle in a state that allows the braking system to operate normally, parking brake released, vehicle speed 0 km/h. Brake pedal depressed to activate the brake function and the stop lights without any dynamic cycle. | Stop lights inactivated during mode Brake warning light ON with loss of brake function. |
| Day running light (DRL) ON | DRL inactivated during mode |
| ADS shall be operational but without ADS feature(s) activated (1) | ADS does not remain in a failure safe mode or expected failure operational mode |
| [ADS testing mode activated and braking initiated by the ADS] | [Unexpected behaviour, any change in ADS operational functions] |
| (1): ADS are turned on by the driver but some or all ADS functions may revert to a mode where system is monitoring sensors but is not actively ‘driving’ the vehicle due to plausibility issues caused by the EMC laboratory environment. |  |

…"

Annex 6, paragraph 2.1.1.3., amend to read:

"2.1.1.3.	All equipment which can be switched on permanently by the driver, or passenger, or ADS feature, shouldshall be in normal operation."

Annex 6, paragraph 2.1.1.4., amend to read:

"2.1.1.4.	All other systems which affect the driver's or ADS feature’s control of the vehicle shall be (on) as in normal operation of the vehicle.

[In case of a conflict between the functioning of ADS sensors in the EMC (laboratory) test environment, the manufacturer, in agreement with the Type Approval Authority and Technical Service, shall propose alternative methods and/or subtests, to make sure the ADS feature(s) are tested as they operate on the road, and comply with the provisions of this Regulation.]"

Annex 6, paragraph 2.1.2., amend to read:

“2.1.2.	If there are vehicle electrical/electronic systems which form an integral part of the immunity related functions, which will not operate under the conditions described in paragraph 2.1., it will be permissible for the manufacturer to provide a report or additional evidence to the Technical Service that the vehicle electrical/electronic system meets the requirements of this Regulation. Such evidence shall be retained in the type approval documentation.

This paragraph does not apply to vehicle types which are equipped with an ADS.“

Annex 6, paragraph 2.2.1.3., amend to read:

"2.2.1.3.	All other equipment which can be switched ON by the driver, or passengers, or ADS feature shall be OFF."

Insert a new Annex 2A paragraph 74., to read:

“74.	A brief description of the ADS (if any)”

Insert a new Annex 2A paragraph 74., to read:

“75.	A brief description of the remote operation and it’s connectivity to the vehicle (if any)”

Annex 3A, Appendix, paragraph 7, amend to read:

“7.	ADS feature of type 1 or type 2 (if any)”

Former paragraph 7, renumber as 8.

“78. Remarks: (e.g. valid for both left-hand drive and right-hand drive vehicles): .............. "

II.	Justification

- This proposal is a result of a small group working on the fitness of ADS into 07 series of amendments to UN Regulation No.10.

- IWG EMC has still to validate few items in square brackets.

- An informal document will be submitted prior to GRE April session to amend this this working document.

*	In accordance with the programme of work of the Inland Transport Committee for 2026 as outlined in proposed programme budget for 2026 (A/80/6 (Sect. 20), table 20.7), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.