Proposal for the 03 Series of Amendments to UN Regulation No. 160 (Event Data Recorder)

I.	Proposal

Annex 4 - Data elements and format1

Table 1, amend to read:

Table 1

| […] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Advanced driver distraction warning (ADDW), status | Mandatory | Event | N/A | N/A | N/A | On, Off | Planar VRU Rollover AEBS |
| Advanced driver distraction warning (ADDW), time since warning issued23 | Mandatory | Event | N/A | 0 to 240 sec | ±1 sec | 1 sec | Planar VRU Rollover AEBS |
| […] |  |  |  |  |  |  |  |
| Driver control assistance system (DCAS) status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Active, Passive, Inactive, Off | Planar VRU Rollover AEBS |
| [Driver control assistance system (DCAS), driver monitoring system, time since disengagement warning issued24 | Mandatory | Event | N/A | 0 to 1000 sec | ± 0.5sec | 1 sec | Planar VRU Rollover AEBS] |
| [Driver control assistance system (DCAS), driver monitoring system, warning type | Mandatory | -5.0 to 0 seconds | 4Hz | N/A | N/A | Hands-on request, Eyes-on request, Direct Control Alert (due to driver disengagement), Direct Control Alert (due to exiting system boundaries), No warning | Planar VRU Rollover AEBS] |
| Driver drowsiness and attention warning (DDAW), status | Mandatory | Event | N/A | N/A | N/A | On, Off | Planar VRU Rollover AEBS |
| Driver drowsiness and attention warning (DDAW), time since warning issued23 | Mandatory | Event | N/A | 0 to 60 min | ±1 min | 1 min | Planar VRU Rollover AEBS |
| […] |  |  |  |  |  |  |  |

[…]

23 List this element up to 5 times for the most recent warnings given on current trip (once for each warning)

24List this element up to a maximum of the most recent [15] warnings