Proposal for the 03 Series of Amendments to UN Regulation No. 160 (Event Data Recorder)

The modifications to the text of the 02 Series of Amendments to UN Regulation No. 160 	are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Paragraph 2.2. on "Accident Emergency Call System", renumber as paragraph 2.25.

Paragraph 2.18., amend to read:

“2.18. 	"End of event time" means,

- for planar events, the moment at which the cumulative delta-V within a 20 ms time period becomes 0.8 km/h or less, or the moment at which the crash protection algorithm of the air bag control unit all algorithms resets, or the moment at 300 ms after time zero,

- for all other events, the moment at 300 ms after time zero.”

Paragraph 2.20., amend to read:

“2.20.	"Engine throttle, percent full" means the driver-requested acceleration as measured by the throttle position sensor on the accelerator control compared to the fully depressed position.”

Paragraph 2.25., deleted:

“2.25.	"If recorded" means if data is recorded in non-volatile memory for the purpose of subsequent downloading”

Paragraph 2.35., amend to read:

“2.35.	"Non-volatile memory" means the memory reserved for maintaining recorded EDR data in a semi-permanent fashion. Data recorded in non-volatile memory is retained after a loss of power and can be retrieved with EDR data extraction tools and methods.”

Paragraph 2.45., amend to read:

“2.45.	"Service brake, on and off status" means the status of the device that is 		installed in or connected to the brake pedal system to detect whether the 		pedal was pressed. The device can include the brake pedal switch or other 		driver-operated service brake control.”

Paragraph 2.51., amend to read:

“2.51. 	"Time from event 1 to 2interval between current event and previous 			event" means the elapsed time from time zero of the first previous event 		to until time zero of the second current event of a multi-event crash.”

Paragraph 2.65., amend to read:

“2.65.	"Vulnerable road userVRU secondary safety system" means a deployable 		vehicle system outside the occupant compartment designed to mitigate 		injury consequences to vulnerable road users during a collision.”

Insert new paragraphs 2.71 – 2.82:

“2.71.	“Rollover protection system” means any protection system that protects occupants during a rollover, which is activated by a vehicle's crash sensing system.

2.72.	“Advanced emergency braking system (AEBS)” means a system which can automatically detect an imminent forward collision and activate the vehicle braking system to decelerate the vehicle with the purpose of avoiding or mitigating a collision. The system may also be referred to as "Automatic emergency braking system" in other publications or countries.

2.73.	“Trigger activated” indicates the first trigger that was activated to cause the recording of the event.

2.74.	"Pre-Event Synchronization Timer" means the relative time from last data sampling point before T0 to T0. Used for temporal alignment of pre-crash data with event time zero."

2.75.	“Blind spot monitoring system” means a system that detects and communicates to the driver the presence of another vehicle in a zone to the side of the subject vehicle or the approach of another vehicle into that zone.

2.76.	"Emergency stop signal" means a light-signaling function to indicate to other road users to the rear of the vehicle that a high retardation force is being applied to the vehicle relative to the prevailing road conditions.

2.77.	“Turn signal” means the logical state of the signal for the direction indictors on a motor vehicle or trailer which indicates the intention to change the direction in which the vehicle is proceeding.

2.78.	“Acceleration control for pedal error (ACPE)” means a system to detect misapplication of the accelerator control by the driver and to control unintended acceleration.

2.79.	“Advanced driver distraction warning (ADDW)” means a system that helps the driver to continue to pay attention to the traffic situation and that warns the driver when he or she is distracted.

"Advanced Driver Distraction Warning (ADDW) system" means a system that assesses the driver’s visual attention and warns the driver via the vehicle’s Human-Machine Interface when assessed to be distracted.

2.80.	“Current trip” means the period of time extending from the event back to the last change of Ignition cycle.

2.81.	“Driver control assistance system (DCAS)” means the hardware and software collectively capable of assisting a driver in controlling the longitudinal and lateral motion of the vehicle on a sustained basis.

2.82.	“Driver drowsiness and attention warning (DDAW)” means a system that assesses the driver’s alertness through vehicle systems analysis and warns the driver if needed”

"Driver Drowsiness and Attention Warning (DDAW) system" means a system that assesses the driver’s reduced alertness and warns the driver via the vehicle’s Human-Machine Interface when they are assessed to be drowsy.”

Paragraph 5.3.1., amend to read:

“5.3.1.	Conditions for triggering recording of data

An event shall be recorded by the EDR if one of the following threshold values mentioned under paragraphs 5.3.1.1. to 5.3.1.2. is met or exceeded.

If a vehicle is not fitted with a system referred to in paragraphs 5.3.1.1.3. to 5.3.1.1.5 or 5.3.1.2., this document requires neither triggering recording of data according to these paragraphs nor fitting of such systems.

5.3.1.1.	For planar event, VRU event, rollover event:

5.3.1.1.1.	Change in longitudinal vehicle velocity more than 8 km/h within a 150 ms or less interval.

5.3.1.1.2.	Change in lateral vehicle velocity more than 8 km/h within a 150 ms or less interval

5.3.1.1.3.	Activation Deployment of Non-reversible occupant restraint system,

5.3.1.1.4.	Activation Deployment of Vulnerable road user VRU secondary safety system.

If a vehicle is not fitted with any Vulnerable Road User (VRU) secondary safety system, this document requires neither recording of data nor fitting of such systems. However, if the vehicle is fitted with such a system, then it is mandatory to record the event data following activation of this system.

5.3.1.1.5.	Deployment of the rollover protection system.

5.3.1.2.	For AEBS event:

Emergency braking demand by the advanced emergency braking system in response to detecting the possibility of an imminent collision.”

Paragraph 5.3.2, amend to read:

“5.3.2.	Conditions for triggering locking of data

In the circumstances provided below, the memory for the event shall be locked to prevent any future overwriting of the data by subsequent events.

If a locking trigger according to 5.3.2.1 or 5.3.2.2 occurs in short succession (<300 ms) after another trigger and an overlap of data between both events would result, only the planar event shall be locked.”

Paragraphs 5.3.2.3., amend to read:

“5.3.2.3.	Activation Deployment of Vulnerable road user VRU secondary safety system.”

Insert new paragraph 5.3.2.4., to read:

“5.3.2.4.	Deployment of the rollover protection system.”

Paragraph 5.3.3., amend to read:

“5.3.3.	Conditions for establishment of time zero

Time zero is established at the time when any of the following first occurs:

5.3.3.1.	For planar event, VRU event, rollover event:

5.3.3.1.1	For systems with "wake-up" air bag control systems, the time at which the occupant restraint control algorithm is activated; or

5.3.3.1.2.	For continuously running algorithms,

5.3.3.1.2.1.	The first point in the interval where a longitudinal, cumulative delta-V of over 0.8 km/h is reached within a 20 ms time period; or

5.3.3.1.2.2.	For vehicles that record "delta-V, lateral," the first point in the interval where a lateral, cumulative delta-V of over 0.8 km/h is reached within a 5 ms time period; or

5.3.3.1.3.	Deployment of a non-reversible deployable occupant restraint or activation of VRU secondary safety protection system.

5.3.3.1.4.	Deployment of VRU secondary safety system

5.3.3.1.5.	Deployment of the rollover protection system.

5.3.3.2.	 For AEBS Event

Occurrence of the emergency braking demand trigger of the advanced emergency braking system.”

Paragraph 5.4.3, amend to read:

“5.4.3.	The data elements required by paragraph 5.1, shall be recorded in the format specified by paragraph 5.2, exist at the completion of the crash test and the complete data recorded element shall read "yes" after the test. Elements that are not operating normally in braking, etc.) are not required to meet the accuracy or resolution requirements in these crash tests.
The data shall be retrievable even after an impact of a severity level set by UN Regulations Nos. 94, 95 or 137 95 and either 94 or 137.”

Paragraph 6.1., amend to read:

“6.1.	The accuracy of the measurement of longitudinal and lateral acceleration the two data elements (post-crash longitudinal and lateral acceleration) shall be verified using a component test fixture that subjects the EDR/airbag control module acceleration sensors to a sinusoidal acceleration motion in accordance with the following:

at= -40g*sinπt20 ms
			        +/- 2g”

12.	Transitional provisions

Insert new paragraphs 12.12. – 12.16., to read:

12.12.	As from the official date of entry into force of the 03 series of amendments, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type-approvals under this Regulation as amended by the 03 series of amendments.

12.13.	As from 1 September [2031], Contracting Parties applying this Regulation shall not be obliged to accept type-approvals to any of the preceding series of amendments, first issued after 1 September [2031].

12.14.	Contracting Parties applying this Regulation shall continue to accept type approvals issued according to the preceding series of amendments to this Regulation first issued before 1 September [2031].

12.15.	Contracting Parties applying this Regulation may grant type approvals according to any preceding series of amendments to this Regulation.

12.16.	Contracting Parties applying this Regulation shall continue to grant extensions of existing approvals to any preceding series of amendments to this Regulation

Annex 1 - Communication

Annex 1, amend to read:

“of a vehicle type with regard to its Event Darta Recorder (EDR) pursuant to UN Regulation No. 160”

Annex 4 - Data elements and format1

Table 1, amend to read:

Table 1

| Data element | Condition for requirement2,3 | Recording interval/time4,5 (relative to time zero) | Data sample rate (samples per second) | Minimum range | Accuracy6,7 | Resolution8 | Event(s) recorded for9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Speed, vehicle indicated | Mandatory | [-5.0 to 0 sec] | 210 | 0 km/h to 250 km/h | ±1 km/h | 1 km/h | Planar Rollover VRU AEBS |
| Engine throttle, % full (or Accelerator pedal, % full) | Mandatory | -5.0 to 0 sec | 2 | 0 to 100 % | ± 5 % | 1% | Planar Rollover VRU |
| Engine throttle, % full | If Recorded Mandatory | -5.0 to 0 sec | 10 | 0 to 100 % | ± 5 % | 1% | Planar Rollover VRU AEBS |
| Accelerator pedal, % full | Mandatory | -5.0 to 0 sec | 10 | 0 to 100 % | ± 5 % | 1% | Planar Rollover VRU AEBS |
| Service brake, on/off status | Mandatory | -5.0 to 0 sec | 210 | On or OffN/A | N/A | On,or Off | Planar Rollover VRU AEBS |
| Ignition cycle, crash | Mandatory | -1.0 sec | N/A | 0 to 60,000 | ±1 cycle | 1 cycle | Planar Rollover VRU AEBS |
| Ignition cycle, download | Mandatory | At time of download10 | N/A | 0 to 60,000 | ±1 cycle | 1 cycle | Planar Rollover VRU AEBS |
| Safety belt status, driver | Mandatory | -1.0 sec | N/A | Fastened, not fastened N/A | N/A | Fastened, not fastened | Planar Rollover AEBS |
| Air bag warning lamp | Mandatory | -1.0 sec | N/A | On or Off N/A | N/A | On,or Off | Planar Rollover |
| Multi-event crash, number of event | If Recorded | Event | N/A | 1 or more | N/A | 1 or more | Planar Rollover VRU AEBS |
| Time from Event 1 to 2Time interval between current event and previous event | Mandatory | As needed | N/A | 0 to 5 sec | ±0.1 sec | 0.1 sec | Planar Rollover VRU AEBS |
| Complete file recorded | Mandatory | Following other data | N/A | Yes or No N/A | N/A | Yes,or No | Planar Rollover VRU AEBS |
| Normal acceleration (post-crash) | If Recorded | 0 to at least 250 300 ms | 10 | -5g to +5g | ±10 % | 0.5g | Rollover |
| Engine rpm | Mandatory | -5.0 to 0 sec | 210 | 0 to 10,000 20,000 rpm | ±100 rpm17 | 100 rpm | Planar Rollover AEBS |
| Vehicle roll angle18 | If Recorded | 0 -2.5 sec to at least 250 300 ms | 10 | -1080 deg to +1080 deg | ±10 % | 10 deg | Rollover |
| Vehicle roll rate18 | Mandatory if fitted and used for rollover protection system control algorithm | 0 -2.5 sec to at least 250 300 ms | 10 | -240 to +240 deg/sec | ±10 % | 4 deg/sec | Rollover |
| Anti-lock braking system activity | Mandatory | -5.0 to 0 sec | 210 | Faulted, Non-Engaged, Engaged N/A | N/A | Faulted, Non-Engaged, Engaged | Planar VRU Rollover AEBS |
| Stability control | Mandatory | -5.0 to 0 sec | 210 | Faulted, On, Off, Engaged N/A | N/A | Faulted, On, Off, Engaged | Planar VRU Rollover AEBS |
| Steering input | Mandatory | -5.0 to 0 sec | 210 | -250 deg CW to + 250 deg CCW. | ±5 % | 1 % 5 deg | Planar VRU Rollover AEBS |
| Safety belt status, front passenger12 | Mandatory | -1.0 sec | N/A | Fastened, Not fastened N/A | N/A | Fastened, not fastened | Planar Rollover AEBS |
| Passenger air bag suppression status, front12 | Mandatory | -1.0 sec | N/A | Suppressed or not suppressed N/A | N/A | Suppressed,or not suppressed | Planar Rollover |
| Seat track position switch, foremost, status, driver | Mandatory if fitted and used for deployment decision | -1.0 sec | N/A | Yes or NoN/A | N/A | Yes, or No | Planar Rollover |
| Seat track position switch, foremost, status, front passenger9 | Mandatory if fitted and used for deployment decision | -1.0 sec | N/A | Yes or NoN/A | N/A | Yes, or No | Planar Rollover |
| Occupant size classification, driver (if applicable) | If recorded | -1.0 sec | N/A | 5th percentile female or larger. | N/A | Yes, or No | Planar Rollover |
| Occupant size classification, front passenger9 | If recorded | -1.0 sec | N/A | 6yr old HIII US ATD or Q6 ATD or smaller | N/A | Yes, or No | Planar Rollover |
| Safety belt status, rear passengers13 | Mandatory | -1.0 sec | N/A | Fastened, not fastened N/A | N/A | Fastened, not fastened | Planar Rollover AEBS |
| Tyre Pressure Monitoring System Warning Lamp Status | Mandatory | -1.0 second relative to time zero | N/A | N/A | N/A | On, Off | Planar Rollover AEBS |
| Longitudinal acceleration (pre-crash) | Mandatory | -5.0 to 0 second relative to time zero | 2 | -1.5g to +1.5g | ±10 % | 0.1g | Planar VRU AEBS |
| Lateral acceleration (pre-crash) | Mandatory | -5.0 to 0 second relative to time zero | 2 | -1.0g to +1.0g | ±10 % | 0.1g | Planar AEBS |
| Yaw Rate18 | Mandatory | -5 to 0 seconds relative to time zero | 2 | -75 to +75 degrees/second deg/sec | ± 10% of the full range of the sensor | 0.1 deg/sec | Planar Rollover AEBS |
| Traction Control Status | Mandatory if not fitted with Stability control | -5.0 to 0 second relative to time zero | 210 | Faulted, On, Off, Engaged N/A | N/A | Faulted, On, Off, Engaged | Planar Rollover |
| Advanced emergency braking system status | Mandatory | -5.0 to 0 second relative to time zero | 210 | N/A | N/A | Faulted, Deactivated, On but Non-engaged, Warning but Non-engaged, Engaged | Planar Rollover VRU AEBS |
| Cruise Control System Status | Mandatory | -5.0 to 0 second relative to time zero | 2 | N/A | N/A | Engaged, Faulted, Off, Non-engaged | Planar Rollover VRU AEBS |
| Adaptive Cruise Control Status (driving automation system level 1) | Mandatory | -5.0 to 0 second relative to time zero | 2 | N/A | N/A | Engaged, Faulted, Off, Non-engaged | Planar Rollover VRU AEBS |
| Vulnerable road user secondary safety system warning indicator status18 | Mandatory | -1.1 to 0 sec relative to time zero | N/A | N/A | N/A | On, or Off | VRU |
| Safety belt status mid-position front | Mandatory | -1.0 sec | N/A | Fastened, not fastened N/A | N/A | Fastened, not fastened | Planar Rollover AEBS |
| Lane departure warning system status | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, Off, On but not warning, On – Warning left, On – Warning right | Planar Rollover AEBS |
| Corrective steering function status | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, Off, On but not engaged, Engaged | Planar Rollover AEBS |
| Emergency steering function status | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, Off, On but not engaged, Engaged | Planar Rollover AEBS |
| Automatically commanded steering function category A status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Off, Stand-By Active20 | Planar Rollover AEBS |
| Automatically commanded steering function category B1 status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Off, Stand-By Active20 | Planar Rollover AEBS |
| Automatically commanded steering function category B2 status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Off, Stand-By Active20 | Planar Rollover AEBS |
| Automatically commanded steering function category C status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Off, Stand-By Active20 | Planar Rollover AEBS |
| Automatically commanded steering function category D status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Off, Stand-By Active20 | Planar Rollover AEBS |
| Automatically commanded steering function category E status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Off, Stand-By Active20 | Planar Rollover AEBS |
| Accident emergency call system status | Mandatory | Event 0-300 ms or 0 to end of event time plus 30 ms, whichever is shorter | N/A | N/A | N/A | Faulted, On but emergency call not automatically triggered, On – emergency call automatically triggered21 | Planar Rollover VRU |
| Pre-Event Synchronization Timer22 | Mandatory | Event | N/A | 0 to 500 ms | ±2 ms | 1 ms | Planar VRU Rollover AEBS |
| Trigger activated | Mandatory | Event | N/A | N/A | N/A | Planar, VRU, Rollover, [AEBS] | Planar VRU Rollover AEBS |
| Acceleration control for pedal error (ACPE) status | Mandatory | -5.0 to 0 sec | 10 2 | N/A | N/A | Faulted, Off, On but not intervening, Intervening | Planar VRU Rollover |
| [Advanced driver distraction warning (ADDW), number of warnings issued on current trip | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar VRU Rollover AEBS] |
| [Advanced driver distraction warning (ADDW), time elapsed since its last warning issued on current trip | Mandatory | Event | N/A | 0 to 240 sec | ±1 sec | 1 sec | Planar VRU Rollover AEBS] |
| Blind spot monitoring system status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Off, Faulted, On not warning, On warning left- side, On warning right-side | Planar VRU Rollover |
| Turn signal status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Off, Left, Right | Planar VRU Rollover |
| Driver control assistance system (DCAS) status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Active, Passive, Inactive, Off | Planar VRU Rollover AEBS |
| [Driver control assistance system (DCAS), driver monitoring system, number of hands-on requests issued on current trip due to driver disengagement | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar VRU Rollover AEBS] |
| [Driver control assistance system (DCAS), driver monitoring system, number of eyes-on requests issued on current trip due to driver disengagement | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar VRU Rollover AEBS] |
| [Driver control assistance system (DCAS), driver monitoring system, number of direct control alerts issued on current trip due to driver disengagement | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar VRU Rollover AEBS] |
| [Driver control assistance system (DCAS), driver monitoring system, time elapsed since its last warning issued on current trip | Mandatory | Event | N/A | 0 to 60 sec | N/A | 1 sec | Planar VRU Rollover AEBS] |
| [Driver control assistance system (DCAS), driver monitoring system, last warning type issued on current trip | Mandatory | Event | N/A | N/A | N/A | Hands-on request, Eyes-on request, Direct Control Alert (due to driver disengagement), Direct Control Alert (due to exiting system boundaries), No request | Planar VRU Rollover AEBS] |
| [Driver drowsiness and attention warning (DDAW), number of warnings issued on current trip | Mandatory | Event | N/A | 0 to 10 | N/A | 1 | Planar VRU Rollover AEBS] |
| [Driver drowsiness and attention warning (DDAW), time elapsed since its last warning issued on current trip | Mandatory | Event | N/A | 0 to 60 min | ±1 min | 1 min | Planar VRU Rollover AEBS] |
| Emergency stop signal status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | On, Off | Planar VRU |

2 "Mandatory" is subject to the conditions detailed in Section 1.

12 3  "If recorded" means if the data is recorded in non-volatile memory for the purpose of subsequent downloading.

13 For rollover events the time at which the event is determined to have started as defined by the manufacturer.

3 4  Pre-crash data and crash data are asynchronous. The sample time accuracy requirement for pre-crash timedata [having single data point (e.g., safety belt status, driver)] is -0.1 to 1.0 sec (e.g., T = -1 would need to occur between -1.1 and 0 seconds.).

5  Pre-crash data and crash data are asynchronous. The sample time accuracy requirement for pre-crash data having multiple data points (e.g., vehicle speed) is the sample interval (inverse of sample rate) of the data element. i.e. the last data point may have been captured just before time zero but no more than one sample interval (e.g., data with 2Hz sample rate may have its last sample captured up to 0.5 sec before time zero). All subsequent pre-crash data values are referenced from this data point.

5 6  Accuracy requirement only applies within the range of the physical sensor. If measurements captured by a sensor exceed the design range of the sensor, the reported element shall indicate when the measurement first exceeded the design range of the sensor.

16 7  Relative to the full range of the sensor.

4 8  For data elements with system states, the term "engaged" also means "actively controlling" or "actively intervening" and "not-engaged" also means "on but not controlling". Likewise, "off" also means "deactivated".

6 9  "Planar" includes triggered events in sections 5.3.1.1.1., 5.3.1.1.2. and 5.3.1.1.3.

"VRU" includes triggered events in section 5.3.1.1.4.

"Rollover" includes triggered events in section 5.3.1.1.5.

"AEBS" includes triggered events in section 5.3.1.2.

7 10  The ignition cycle at the time of download is not required to be recorded at the time of the crash but shall be reported during the download process.

8 11  The air bag warning lamp is the readiness indicator specified in national air bag requirements and may also illuminate to indicate a malfunction in another part of the deployable restraint system.

9 12 List this element n times, once for each device

18 13 List this element n times, once for each device in 2nd, 3rd, row.

17 14 List this element n - 1 times, once for each stage of a multi-stage air bag system.

10 15 +/- 10 per cent of the full range of the accelerometer used in the Electronic Control Unit (ECU) containing the EDR function as specified in paragraph 6.1.5.

11 16 Format for lateral acceleration recorded in rollover is at the option of the manufacturer.

14 17 These elements do not need to meet the accuracy and resolution requirements in specified crash tests.

15 18 The manufacturer will indicate the direction of positive roll /rate, yaw rate, and roll angle

19 Multiple safety system status indications can be combined into the air bag warning.

20 Faulted = Faulted, Per R79 Off = Off, Standby – ACSF can’t control, Active = ACSF is on but not controlling or ACSF is on and controlling.

21 Triggered means the airbag control unit has sent a message on the bus that emergency call shall be sent via another unit.

22 List this element n times, once for each data sample rate of pre-crash data.

II.	Justification

Proposal by the Informal Working Group on EDR/DSSAD amending the 02 Series of Amendments to UN Regulation No. 160 introducing new requirements in line with the ‘Step 2’ workstream (Programme of Work ECE/TRANS/WP.29/2025/1 as revised).

- Paragraph 2.2. ("Accident Emergency Call System"). Numbering corrected.

- Paragraph 2.25. deleted. This definition is already covered by a footnote and therefore redundant.

- Paragraph 2.35. The deleted text part is already covered by para. 5.35. and therefore redundant.

- Paragraph 2. All other modifications are due to new or amended data elements and, where applicable, based on existing definitions in other UN Regulations.

- Paragraphs 5.3.1., 5.3.2. and 5.3.3.: Intended to resolve interpretation issues during type approval on whether a rollover protection system should be considered a non-reversible occupant restraint system triggering an event. The amendments now clarify rollover events and explicitly include deployment of the rollover protection system as a condition for triggering recording and locking of data.

Paragraph 5.3.1. further ensures that events are recorded based on other trigger conditions even when the rollover algorithm is still in operation.

- Paragraph 5.3.2.: The amendment aims at recording (and subsequently locking) any event based on locking triggers (i.e. those definitively indicating a collision), while events triggered shortly before, and therefore relating to the same real-world incidents, may be excluded (i.e. not recorded to non-volatile memory). This ensures that the most relevant record relating to a real-world incident (around the actual time of collision) is available and locked and that all data elements relevant for the actual collision are captured.

- Paragraph 5.4.3.: To resolve interpretation issues during type approval on which collision tests are required to evidence the crash test performance and survivability. Based on para. 5.4.1. and 5.4.2., it is now clarified that a side AND a frontal collision, being either full width OR offset, must be conducted.

- Paragraphs 5.3.1., 5.3.3. and Annex 4: An important improvement of this proposal is the capturing of events involving vulnerable road users. These may be missed by the currently defined triggers. To improve the capture rate of collisions (including but not limited to VRU collisions) advanced emergency braking system interventions are added as trigger criterion.

- Paragraph 6.1.: Clarified that this concerns post-crash. Also missing units are added to the equation.

- Transitional provisions added [the actual transitional time still under discussion].

- Annex 1 : typo corrected.

- Annex 4. Table 1:

Data element “Engine throttle, % full or Accelerator pedal, % full” splitted to distinguish the element for internal combustion engines from that of electric vehicles.

Recording interval/time for normal acceleration and vehicle roll angle and rate increased to fully cover the recording of (pre- and post-crash) rollover events.

[Recording interval/time amended to 0 to [2.5] sec. for data elements triggered by the AEBS. This value is based on a typical TTC for start of AEBS braking of 1.0 to 1.5 seconds and, assuming constant deceleration, doubling this time to bring the vehicle to a standstill in front of the target.]

Sample rate increased to 10 Hz for several data elements to improve the granularity of the event data.

Data elements Pre-Event Synchronization Timer, Trigger activated, ACPE, ADDW, DDAW, Blind spot monitoring system status, Turn Signal Status, DCAS, Emergency stop signal status added. This is intended to further extend the usefulness of EDRs for collision research purposes in relation to active safety and driver assistance systems and their impact on road safety. EDRs are used for more than just collision investigation, the data they produce can be aggregated to assess the effectiveness of new driver assistance systems. It would also allow evaluation of whether the assistance systems are making a positive impact on road safety.
Note: The inclusion of the ADDW, DDAW and DCAS (non-status) elements is opposed by some IWG delegations as this would not contribute to crash investigation.

Data element “Accident emergency call system status” recording interval/time (and clarifying footnote) amended to ensure recording the full event.

Data element “Engine rpm” minimum range increased. The original value was based on the characteristics of internal combustion engine vehicles. This value is now expanded to accommodate the motor rpm range of actual electric vehicles.

Other modifications concern editorial improvements and corrections.

_________________