Proposal for the 03 Series of Amendments to UN Regulation No. 160 (Event Data Recorder)

I.	Proposal

Insert new paragraphs 2.71 – 2.82:

[…]

2.83.	“Time to collision (TTC)” means the value of time obtained by dividing the longitudinal distance (in the direction of travel of the subject vehicle) between the subject vehicle and the AEBS target by the longitudinal relative speed of the subject vehicle and the target

[…]

Paragraph 5.3.2, amend to read:

“5.3.2.	Conditions for triggering locking of data

In the circumstances provided below, the memory for the event shall be locked to prevent any future overwriting of the data by subsequent events.

If a locking trigger according to 5.3.2.1 or 5.3.2.2 occurs in short succession (<300 ms) after another trigger and an overlap of data between both events would result, only the planar event shall be locked.”

[…]

12.	Transitional provisions

Insert new paragraphs 12.12. – 12.16., to read:

[…]

12.13.	As from 1 September [2029], Contracting Parties applying this Regulation shall not be obliged to accept type-approvals to any of the preceding series of amendments, first issued after 1 September [2029].

12.14.	Contracting Parties applying this Regulation shall continue to accept type approvals issued according to the preceding series of amendments to this Regulation first issued before 1 September [2029].

[…]

Annex 4 - Data elements and format1

Table 1, amend to read:

Table 1

| Data element | Condition for requirement2,3 | Recording interval/time4,5 (relative to time zero) | Data sample rate (samples per second) | Minimum range | Accuracy6,7 | Resolution8 | Event(s) recorded for9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Speed, vehicle indicated | Mandatory | [-5.0 to 0 sec (Planar, Rollover, VRU) [-5.0] to 2.5 sec (AEBS)]]] | 210 | 0 km/h to 250 km/h | ±1 km/h | 1 km/h | Planar Rollover VRU AEBS |
| […] |  |  |  |  |  |  |  |
| Safety belt status, driver | Mandatory | -1.0 sec | N/A | Fastened, not fastened N/A | N/A | Fastened, not fastened | Planar Rollover VRU AEBS |
| Air bag warning lamp | Mandatory | -1.0 sec | N/A | On or Off N/A | N/A | On,or Off | Planar Rollover VRU |
| […] |  |  |  |  |  |  |  |
| Engine rpm | Mandatory | -5.0 to 0 sec | 210 | 0 to 10,000 20,000 rpm | ±100 rpm17 | 100 rpm | Planar Rollover VRU AEBS |
| […] |  |  |  |  |  |  |  |
| Safety belt status, front passenger12 | Mandatory | -1.0 sec | N/A | Fastened, Not fastened N/A | N/A | Fastened, not fastened | Planar Rollover VRU AEBS |
| […] |  |  |  |  |  |  |  |
| Tyre Pressure Monitoring System Warning Lamp Status | Mandatory | -1.0 second relative to time zero | N/A | N/A | N/A | On, Off | Planar Rollover VRU AEBS |
| Longitudinal acceleration (pre-crash) | Mandatory | -5.0 to 0 second relative to time zero | 10 | -1.5g to +1.5g | ±10 % | 0.1g | Planar Rollover VRU AEBS |
| Lateral acceleration (pre-crash) | Mandatory | -5.0 to 0 second relative to time zero | 10 | -1.0g to +1.0g | ±10 % | 0.1g | Planar Rollover VRU AEBS |
| […] |  |  |  |  |  |  |  |
| Vulnerable road user secondary safety system warning indicator status18 | Mandatory | -1.1 to 0 sec relative to time zero | N/A | N/A | N/A | On, or Off | VRU |
| […] |  |  |  |  |  |  |  |
| Lane departure warning system status | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, Off, On but not warning, On – Warning left, On – Warning right | Planar Rollover VRU AEBS |
| Corrective steering function status | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, Off, On but not engaged, Engaged | Planar Rollover VRU AEBS |
| Emergency steering function status | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, Off, On but not engaged, Engaged | Planar Rollover VRU AEBS |
| […] |  |  |  |  |  |  |  |
| Trigger activated | Mandatory | Event | N/A | N/A | N/A | Planar, VRU, Rollover, [AEBS (for suspected vulnerable road user), AEBS (for suspected non-vulnerable road user)]]] | Planar VRU Rollover AEBS |
| […] |  |  |  |  |  |  |  |
| [Time to collision (TTC) | Mandatory | Event | N/A | 0 s to 5.0 s | ±0.2 s | 0.1 s | AEBS] |

[…]