| [@@@ 8.2.3.tt[hase | ECE/TRANS/WP.29/GRPE/2025/15 |  |
| --- | --- | --- |
|  | Informal Document GRPE-93-29 Submitted by the experts from the EC, Japan, UK and OICA 93rd GRPE, 14-17 October 2025 Agenda item 3(a) Updates to GRPE/2025/15e are shown using tracked changes Economic and Social Council | Distr.: General 31 July 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety third session

Geneva, 14-17 October 2025

Item 3(a) of the provisional agenda

Light vehicles:
UN Regulations Nos. 68 (Measurement of the maximum speed,
including electric vehicles), 83 (Emissions of M1 and N1 vehicles),
101 (CO2 emissions/fuel consumption), 
103 (Replacement pollution control devices) and 
154 (Worldwide harmonized Light vehicles Test Procedures (WLTP))

Proposal for a new Series of amendments to UN Regulation No. 154 (WLTP)

Submitted by the experts from the European Commission, Japan, the United Kingdom of Great Britain and Northern Ireland and the International Organization of Motor Vehicle Manufacturers *

This document proposes a new 04 series of amendments to UN Regulation No. 154, as a consolidated version. It is a single series which replaces the previous approach for UN Regulation No. 154, which had separate series for the regional levels and the harmonised procedure, which contained the most stringent procedures/limits subject to full mutual recognition.

This new series introduces new regional requirements from Contracting Parties, in particular the requirements from Euro 7 in the EU. These include, amongst others, new particle number (PN10) provisions, a new SHED limit for evaporative emissions, updated On-board Fuel Consumption Monitoring provisions, as well as other adaptations to technical progress. It introduces new annexes with requirements relating to in-vehicle battery durability and a new test for range of Pure Electric Vehicles at low temperatures.

UN Regulation No. 154

Uniform provisions concerning the approval of light duty passenger and commercial vehicles with regards to criteria emissions, emissions of carbon dioxide and fuel consumption and/or the measurement of electric energy consumption and electric range (WLTP)

Annexes Part A

The Type Approval requirements and documentation included in Annexes Part A cover the requirements for Level 1A, Level 1B, and Level 2. This means that certain elements may not be required for the level of approval being sought. In such an instance the element may be omitted.

Where relevant, the Type Approval requirements and documentation included in Annexes Part A provide separate reporting tables/fields for the results after 3-phases and the results after 4-phases.

Annex A1

Engine and vehicle characteristics and information concerning the conduct of tests (‘information document’)

The following information, when applicable, shall be supplied in triplicate and include a list of contents.

If there are drawings, they shall be to an appropriate scale and show sufficient detail; they shall be presented in A4 format or folded to that format. Photographs, if any, shall show sufficient detail.

If the systems, components or separate technical units have electronic controls, information concerning their performance shall be supplied.

| Level of approval being applied for (L1A, L1B, L2): … |  |  |  |
| --- | --- | --- | --- |
| 0 | GENERAL |  |  |
| 0.1. | Make (trade name of manufacturer): … |  |  |
| 0.2. | Type: … |  |  |
| 0.2.1. | Commercial name(s) (if available): … |  |  |
| 0.2.3. | Family identifiers (where applicable): |  |  |
| 0.2.3.1. | Interpolation family: … |  |  |
| 0.2.3.2. | ATCT family(s): … |  |  |
| 0.2.3.4. | Roadload family |  |  |
| 0.2.3.4.1. | Roadload family of VH: … |  |  |
| 0.2.3.4.2. | Roadload family of VL: … |  |  |
| 0.2.3.4.3. | Roadload families applicable in the interpolation family: … |  |  |
| 0.2.3.5. | Roadload Matrix family(s): … |  |  |
| 0.2.3.6. | Periodic regeneration family(s): … |  |  |
| 0.2.3.7. | Evaporative test family(s): … |  |  |
| 0.2.3.8. | OBD family(s): … |  |  |
| 0.2.3.9. | Durability family(s): … |  |  |
| 0.2.3.10. | ER family(s): … |  |  |
| 0.2.3.11. | Gas Fuelled Vehicle family(s): … |  |  |
| 0.2.3.12. | (Reserved) |  |  |
| 0.2.3.13. | KCO2 correction factor family: … |  |  |
| 0.2.3.14. | Low temperature range family: … |  |  |
| 0.2.3.15. | Battery durability family(s): … |  |  |
| 0.2.3.15.1. | Monitor family(s): … |  |  |
| 0.2.3.16. | Virtual distance family: … |  |  |
| 0.2.3.17. | Lower limit pressure family for OVC-FCHVs and NOVC-FCHVs |  |  |
| 0.2.4. | other family(s): … |  |  |
| 0.4. | Category of vehicle (c): … |  |  |
| 0.5. | Name and address of manufacturer: ... |  |  |
| 0.8. | Name(s) and address(es) of assembly plant(s): … |  |  |
| 0.9. | Name and address of the manufacturer's representative (if any): … |  |  |
| 1. | GENERAL CONSTRUCTION CHARACTERISTICS |  |  |
| 1.1. | Photographs and/or drawings of a representative vehicle/component/separate technical unit (1): |  |  |
| 1.3.3. | Powered axles (number, position, interconnection): … |  |  |
| 2. | MASSES AND DIMENSIONS (f) (g) (7) (in kg and mm) (Refer to drawing where applicable) |  |  |
| 2.6. | Mass in running order (h) (a) maximum and minimum for each variant: … |  |  |
| 2.6.3. | Rotational mass: 3 % of the sum of mass in running order and 25 kg or value, per axle (kg): … |  |  |
| 2.8. | Technically permissible maximum laden mass stated by the manufacturer (i) (3): … |  |  |
| 3. | PROPULSION ENERGY CONVERTER (k) |  |  |
| 3.1. | Manufacturer of the propulsion energy converter(s): … |  |  |
| 3.1.1. | Manufacturer's code (as marked on the propulsion energy converter or other means of identification): … |  |  |
| 3.2. | Internal combustion engine |  |  |
| 3.2.1.1. | Working principle: positive ignition/compression ignition/dual fuel (1) Cycle: four stroke/two stroke/rotary (1) |  |  |
| 3.2.1.2. | Number and arrangement of cylinders: … |  |  |
| 3.2.1.2.1. | Bore (1): … mm |  |  |
| 3.2.1.2.2. | Stroke (1): … mm |  |  |
| 3.2.1.2.3. | Firing order: … |  |  |
| 3.2.1.3. | Engine capacity (m): … cm3 |  |  |
| 3.2.1.4. | Volumetric compression ratio (2): … |  |  |
| 3.2.1.5. | Drawings of combustion chamber, piston crown and, in the case of positive ignition engines, piston rings: … |  |  |
| 3.2.1.6. | Normal engine idling speed (2): … min–1 |  |  |
| 3.2.1.6.1. | High engine idling speed (2): … min–1 |  |  |
| 3.2.1.8. | Rated engine power (n): … kW at … min–1 (manufacturer's declared value) |  |  |
| 3.2.1.9. | Maximum permitted engine speed as prescribed by the manufacturer: … min–1 |  |  |
| 3.2.1.10. | Maximum net torque (n): … Nm at … min–1 (manufacturer's declared value) |  |  |
| 3.2.2. | Fuel |  |  |
| 3.2.2.1. | Diesel/Petrol/LPG/NG or Biomethane/Ethanol (E 85)/Biodiesel/Hydrogen (1), |  |  |
| 3.2.2.1.1. | RON, unleaded: … |  |  |
| 3.2.2.4. | Vehicle fuel type: Mono fuel, Bi fuel, Flex fuel (1) |  |  |
| 3.2.2.5. | Maximum amount of biofuel acceptable in fuel (manufacturer's declared value): … % by volume |  |  |
| 3.2.4. | Fuel feed |  |  |
| 3.2.4.1. | By carburettor(s): yes/no (1) |  |  |
| 3.2.4.2. | By fuel injection (compression ignition or dual fuel only): yes/no (1) |  |  |
| 3.2.4.2.1. | System description (common rail/unit injectors/distribution pump etc.): … |  |  |
| 3.2.4.2.2. | Working principle: direct injection/pre-chamber/swirl chamber (1) |  |  |
| 3.2.4.2.3. | Injection/Delivery pump |  |  |
| 3.2.4.2.3.1. | Make(s): … |  |  |
| 3.2.4.2.3.2. | Type(s): … |  |  |
| 3.2.4.2.3.3. | Maximum fuel delivery (1) (2): … mm3 /stroke or cycle at an engine speed of: … min–1 or, alternatively, a characteristic diagram: … (When boost control is supplied, state the characteristic fuel delivery and boost pressure versus engine speed) |  |  |
| 3.2.4.2.4. | Engine speed limitation control |  |  |
| 3.2.4.2.4.2.1. | Speed at which cut-off starts under load: … min–1 |  |  |
| 3.2.4.2.4.2.2. | Maximum no-load speed: … min–1 |  |  |
| 3.2.4.2.6. | Injector(s) |  |  |
| 3.2.4.2.6.1. | Make(s): … |  |  |
| 3.2.4.2.6.2. | Type(s): … |  |  |
| 3.2.4.2.8. | Auxiliary starting aid |  |  |
| 3.2.4.2.8.1. | Make(s): … |  |  |
| 3.2.4.2.8.2. | Type(s): … |  |  |
| 3.2.4.2.8.3. | System description: … |  |  |
| 3.2.4.2.9. | Electronic controlled injection: yes/no (1) |  |  |
| 3.2.4.2.9.1. | Make(s): … |  |  |
| 3.2.4.2.9.2. | Type(s): |  |  |
| 3.2.4.2.9.3 | Description of the system: … |  |  |
| 3.2.4.2.9.3.1. | Make and type of the control unit (ECU): … |  |  |
| 3.2.4.2.9.3.1.1. | Software version of the ECU: … |  |  |
| 3.2.4.2.9.3.2. | Make and type of the fuel regulator: … |  |  |
| 3.2.4.2.9.3.3. | Make and type of the air-flow sensor: … |  |  |
| 3.2.4.2.9.3.4. | Make and type of fuel distributor: … |  |  |
| 3.2.4.2.9.3.5. | Make and type of the throttle housing: … |  |  |
| 3.2.4.2.9.3.6. | Make and type or working principle of water temperature sensor: … |  |  |
| 3.2.4.2.9.3.7. | Make and type or working principle of air temperature sensor: … |  |  |
| 3.2.4.2.9.3.8. | Make and type or working principle of air pressure sensor: … |  |  |
| 3.2.4.3. | By fuel injection (positive ignition only): yes/no (1) |  |  |
| 3.2.4.3.1. | Working principle: single-/multi-point/direct injection/other (specify) (1): … |  |  |
| 3.2.4.3.2. | Make(s): … |  |  |
| 3.2.4.3.3. | Type(s): … |  |  |
| 3.2.4.3.4. | System description (In the case of systems other than continuous injection give equivalent details): … |  |  |
| 3.2.4.3.4.1. | Make and type of the control unit (ECU): … |  |  |
| 3.2.4.3.4.1.1. | Software version of the ECU: … |  |  |
| 3.2.4.3.4.3. | Make and type or working principle of air-flow sensor: … |  |  |
| 3.2.4.3.4.8. | Make and type of throttle housing: … |  |  |
| 3.2.4.3.4.9. | Make and type or working principle of water temperature sensor: … |  |  |
| 3.2.4.3.4.10. | Make and type or working principle of air temperature sensor: … |  |  |
| 3.2.4.3.4.11. | Make and type or working principle of air pressure sensor: … |  |  |
| 3.2.4.3.5. | Injectors |  |  |
| 3.2.4.3.5.1. | Make: … |  |  |
| 3.2.4.3.5.2. | Type: … |  |  |
| 3.2.4.3.7. | Cold start system |  |  |
| 3.2.4.3.7.1. | Operating principle(s): … |  |  |
| 3.2.4.3.7.2. | Operating limits/settings (1) (2): … |  |  |
| 3.2.4.4. | Feed pump |  |  |
| 3.2.4.4.1. | Pressure (2): … kPa or characteristic diagram (2): … |  |  |
| 3.2.4.4.2. | Make(s): … |  |  |
| 3.2.4.4.3. | Type(s): … |  |  |
| 3.2.5. | Electrical system |  |  |
| 3.2.5.1. | Rated voltage: … V, positive/negative ground (1) |  |  |
| 3.2.5.2. | Generator |  |  |
| 3.2.5.2.1. | Type: … |  |  |
| 3.2.5.2.2. | Nominal output: … VA |  |  |
| 3.2.6. | Ignition system (spark ignition engines only) |  |  |
| 3.2.6.1. | Make(s): … |  |  |
| 3.2.6.2. | Type(s): … |  |  |
| 3.2.6.3. | Working principle: … |  |  |
| 3.2.6.6. | Spark plugs |  |  |
| 3.2.6.6.1. | Make: … |  |  |
| 3.2.6.6.2. | Type: … |  |  |
| 3.2.6.6.3. | Gap setting: … mm |  |  |
| 3.2.6.7. | Ignition coil(s) |  |  |
| 3.2.6.7.1. | Make: … |  |  |
| 3.2.6.7.2. | Type: … |  |  |
| 3.2.7. | Cooling system: liquid/air (1) |  |  |
| 3.2.7.1. | Nominal setting of the engine temperature control mechanism: … |  |  |
| 3.2.7.2. | Liquid |  |  |
| 3.2.7.2.1. | Nature of liquid: … |  |  |
| 3.2.7.2.2. | Circulating pump(s): yes/no (1) |  |  |
| 3.2.7.2.3. | Characteristics: … or |  |  |
| 3.2.7.2.3.1. | Make(s): … |  |  |
| 3.2.7.2.3.2. | Type(s): … |  |  |
| 3.2.7.2.4. | Drive ratio(s): … |  |  |
| 3.2.7.2.5. | Description of the fan and its drive mechanism: … |  |  |
| 3.2.7.3. | Air |  |  |
| 3.2.7.3.1. | Fan: yes/no (1) |  |  |
| 3.2.7.3.2. | Characteristics: … or |  |  |
| 3.2.7.3.2.1. | Make(s): … |  |  |
| 3.2.7.3.2.2. | Type(s): … |  |  |
| 3.2.7.3.3. | Drive ratio(s): … |  |  |
| 3.2.8. | Intake system |  |  |
| 3.2.8.1. | Pressure charger: yes/no (1) |  |  |
| 3.2.8.1.1. | Make(s): … |  |  |
| 3.2.8.1.2. | Type(s): … |  |  |
| 3.2.8.1.3. | Description of the system (e.g. maximum charge pressure: … kPa; wastegate if applicable): … |  |  |
| 3.2.8.2. | Intercooler: yes/no (1) |  |  |
| 3.2.8.2.1. | Type: air-air/air-water (1) |  |  |
| 3.2.8.3. | Intake depression at rated engine speed and at 100 % load (compression ignition engines only) |  |  |
| 3.2.8.4. | Description and drawings of inlet pipes and their accessories (plenum chamber, heating device, additional air intakes, etc.): … |  |  |
| 3.2.8.4.1. | Intake manifold description (include drawings and/or photos): … |  |  |
| 3.2.8.4.2. | Air filter, drawings: … or |  |  |
| 3.2.8.4.2.1. | Make(s): … |  |  |
| 3.2.8.4.2.2. | Type(s): … |  |  |
| 3.2.8.4.3. | Intake silencer, drawings: … or |  |  |
| 3.2.8.4.3.1. | Make(s): … |  |  |
| 3.2.8.4.3.2. | Type(s): … |  |  |
| 3.2.9. | Exhaust system |  |  |
| 3.2.9.1. | Description and/or drawing of the exhaust manifold: … |  |  |
| 3.2.9.2. | Description and/or drawing of the exhaust system: … |  |  |
| 3.2.9.3. | Maximum allowable exhaust back pressure at rated engine speed and at 100 % load (compression ignition engines only): … kPa |  |  |
| 3.2.10. | Minimum cross-sectional areas of inlet and outlet ports: … |  |  |
| 3.2.11. | Valve timing or equivalent data |  |  |
| 3.2.11.1. | Maximum lift of valves, angles of opening and closing, or timing details of alternative distribution systems, in relation to dead centres. For variable timing system, minimum and maximum timing: … |  |  |
| 3.2.11.2. | Reference and/or setting ranges (1): … |  |  |
| 3.2.12. | Measures taken against air pollution |  |  |
| 3.2.12.1. | Device for recycling crankcase gases (description and drawings): … |  |  |
| 3.2.12.2. | Pollution control devices (if not covered by another heading) |  |  |
| 3.2.12.2.1. | Catalytic converter |  |  |
| 3.2.12.2.1.1. | Number of catalytic converters and elements (provide the information below for each separate unit): … |  |  |
| 3.2.12.2.1.2. | Dimensions, shape and volume of the catalytic converter(s): … |  |  |
| 3.2.12.2.1.3. | Type of catalytic action: … |  |  |
| 3.2.12.2.1.4. | Total charge of precious metals: … |  |  |
| 3.2.12.2.1.5. | Relative concentration: … |  |  |
| 3.2.12.2.1.6. | Substrate (structure and material): … |  |  |
| 3.2.12.2.1.7. | Cell density: … |  |  |
| 3.2.12.2.1.8. | Type of casing for the catalytic converter(s): … |  |  |
| 3.2.12.2.1.9. | Location of the catalytic converter(s) (place and reference distance in the exhaust line): … |  |  |
| 3.2.12.2.1.10. | Heat shield: yes/no (1) |  |  |
| 3.2.12.2.1.11. | Normal operating temperature range: … °C |  |  |
| 3.2.12.2.1.12. | Make of catalytic converter: … |  |  |
| 3.2.12.2.1.13. | Identifying part number: … |  |  |
| 3.2.12.2.2. | Sensors |  |  |
| 3.2.12.2.2.1. | Oxygen and/or lambda sensor(s): yes/no (1) |  |  |
| 3.2.12.2.2.1.1. | Make: … |  |  |
| 3.2.12.2.2.1.2. | Location: … |  |  |
| 3.2.12.2.2.1.3. | Control range: … |  |  |
| 3.2.12.2.2.1.4. | Type or working principle: … |  |  |
| 3.2.12.2.2.1.5. | Identifying part number: … |  |  |
| 3.2.12.2.2.2. | NOx sensor: yes/no (1) |  |  |
| 3.2.12.2.2.2.1. | Make: … |  |  |
| 3.2.12.2.2.2.2. | Type: … |  |  |
| 3.2.12.2.2.2.3. | Location |  |  |
| 3.2.12.2.2.3. | Particulate sensor: yes/no (1) |  |  |
| 3.2.12.2.2.3.1. | Make: … |  |  |
| 3.2.12.2.2.3.2. | Type: … |  |  |
| 3.2.12.2.2.3.3. | Location: … |  |  |
| 3.2.12.2.3. | Air injection: yes/no (1) |  |  |
| 3.2.12.2.3.1. | Type (pulse air, air pump, etc.): … |  |  |
| 3.2.12.2.4. | Exhaust gas recirculation (EGR): yes/no (1) |  |  |
| 3.2.12.2.4.1. | Characteristics (make, type, flow, high pressure/low pressure/combined pressure, etc.): … |  |  |
| 3.2.12.2.4.2. | Water-cooled system (to be specified for each EGR system e.g. low pressure/high pressure/combined pressure: yes/no (1) |  |  |
| 3.2.12.2.5. | Evaporative emissions control system (petrol and ethanol engines only): yes/no (1) |  |  |
| 3.2.12.2.5.1. | Detailed description of the devices: … |  |  |
| 3.2.12.2.5.2. | Drawing of the evaporative control system: … |  |  |
| 3.2.12.2.5.3. | Drawing of the carbon canister: … |  |  |
| 3.2.12.2.5.4. | Mass of dry charcoal: … g |  |  |
| 3.2.12.2.5.5. | Schematic drawing of the fuel tank (petrol and ethanol engines only): … |  |  |
| 3.2.12.2.5.5.1. | Fuel tank system capacity, material and construction: … |  |  |
| 3.2.12.2.5.5.2. | Description of vapour hose material, fuel line material and connection technique of the fuel system: … |  |  |
| 3.2.12.2.5.5.3. | Sealed tank system: yes/no |  |  |
| 3.2.12.2.5.5.4. | Description of fuel tank relief valve setting (air ingestion and relief): … |  |  |
| 3.2.12.2.5.5.5. | Description of the purge control system: … |  |  |
| 3.2.12.2.5.6. | Description and schematic of the heat shield between tank and exhaust system: … |  |  |
| 3.2.12.2.5.7. | Permeability factor: … |  |  |
| 3.2.12.2.6. | Particulate trap (PT): yes/no (1) |  |  |
| 3.2.12.2.6.1. | Dimensions, shape and capacity of the particulate trap: … |  |  |
| 3.2.12.2.6.2. | Design of the particulate trap: … |  |  |
| 3.2.12.2.6.3. | Location (reference distance in the exhaust line): … |  |  |
| 3.2.12.2.6.4. | Make of particulate trap: … |  |  |
| 3.2.12.2.6.5. | Identifying part number: … |  |  |
| 3.2.12.2.7. | On-board-diagnostic (OBD) system: yes/no (1) |  |  |
| 3.2.12.2.7.1. | Written description and/or drawing of the MI: … |  |  |
| 3.2.12.2.7.2. | List and purpose of all components monitored by the OBD system: … |  |  |
| 3.2.12.2.7.3. | Written description (general working principles) for |  |  |
| 3.2.12.2.7.3.1. | Positive-ignition engines |  |  |
| 3.2.12.2.7.3.1.1. | Catalyst monitoring: … |  |  |
| 3.2.12.2.7.3.1.2. | Misfire detection: … |  |  |
| 3.2.12.2.7.3.1.3. | Oxygen sensor monitoring: … |  |  |
| 3.2.12.2.7.3.1.4. | Other components monitored by the OBD system: … |  |  |
| 3.2.12.2.7.3.2. | Compression-ignition engines: … |  |  |
| 3.2.12.2.7.3.2.1. | Catalyst monitoring: … |  |  |
| 3.2.12.2.7.3.2.2. | Particulate trap monitoring: … |  |  |
| 3.2.12.2.7.3.2.3. | Electronic fuelling system monitoring: … |  |  |
| 3.2.12.2.7.3.2.5. | Other components monitored by the OBD system: … |  |  |
| 3.2.12.2.7.4. | Criteria for MI activation (fixed number of driving cycles or statistical method): … |  |  |
| 3.2.12.2.7.5. | List of all OBD output codes and formats used (with explanation of each): … |  |  |
| 3.2.12.2.7.6. | The following additional information shall be provided by the vehicle manufacturer for the purposes of enabling the manufacture of OBD-compatible replacement or service parts and diagnostic tools and test equipment. |  |  |
| 3.2.12.2.7.6.1. | A description of the type and number of the preconditioning cycles or alternative preconditioning methods used for the original type approval of the vehicle and the reason for their usage. |  |  |
| 3.2.12.2.7.6.2. | A description of the type of the OBD demonstration cycle used for the original type-approval of the vehicle for the component monitored by the OBD system. |  |  |
| 3.2.12.2.7.6.3. | A comprehensive document describing all sensed components with the strategy for fault detection and MI activation (fixed number of driving cycles or statistical method), including a list of relevant secondary sensed parameters for each component monitored by the OBD system. A list of all OBD output codes and format used (with an explanation of each) associated with individual emission related powertrain components and individual non-emission related components, where monitoring of the component is used to determine MI activation, including in particular a comprehensive explanation for the data given in service $05 Test ID $21 to FF and the data given in service $06.dIn the case of vehicle types that use a communication link in accordance with ISO 15765-4 ‘Road vehicles, diagnostics on controller area network (CAN) — Part 4: requirements for emissions-related systems’, a comprehensive explanation for the data given in service $06 Test ID $00 to FF, for each OBD monitor ID supported, shall be provided. | A comprehensive document describing all sensed components with the strategy for fault detection and MI activation (fixed number of driving cycles or statistical method), including a list of relevant secondary sensed parameters for each component monitored by the OBD system. A list of all OBD output codes and format used (with an explanation of each) associated with individual emission related powertrain components and individual non-emission related components, where monitoring of the component is used to determine MI activation, including in particular a comprehensive explanation for the data given in service $05 Test ID $21 to FF and the data given in service $06.dIn the case of vehicle types that use a communication link in accordance with ISO 15765-4 ‘Road vehicles, diagnostics on controller area network (CAN) — Part 4: requirements for emissions-related systems’, a comprehensive explanation for the data given in service $06 Test ID $00 to FF, for each OBD monitor ID supported, shall be provided. |  |
| 3.2.12.2.7.6.4. | The information required above may be defined by completing a table as described below. |  |  |
| 3.2.12.2.7.6.4.1. | Light-duty vehicles |  |  |
| Component; Fault code; Monitoring strategy; Fault detection criteria; MI activation criteria; Secondary parameters; Preconditioning; Demonstration test. Catalyst; P0420; Oxygen sensor 1 and sensor 2 signals; Difference between sensor 1 and sensor 2 signals-; 3rd cycle; Engine speed, engine load, A/F mode, catalyst temperature; Two Type 1 cycles; Type 1. |
| 3.2.12.2.8. | Other system: … |  |  |
| 3.2.12.2.8.2. | Driver inducement system |  |  |
| 3.2.12.2.8.2.3. | Type of inducement system: no engine restart after countdown/no start after refuelling/fuel-lockout/performance restriction |  |  |
| 3.2.12.2.8.2.4. | Description of the inducement system |  |  |
| 3.2.12.2.8.2.5. | Equivalent to the average driving range of the vehicle with a complete tank of fuel: … km |  |  |
| 3.2.12.2.10. | Periodically regenerating system: (provide the information below for each separate unit) |  |  |
| 3.2.12.2.10.1. | Method or system of regeneration, description and/or drawing: … |  |  |
| 3.2.12.2.10.2. | The number of Type 1 operating cycles, or equivalent engine test bench cycles, between two cycles where regenerative phases occur under the conditions equivalent to Type 1 test (Distance ‘D’): … |  |  |
| 3.2.12.2.10.2.1. | Applicable Type 1 cycle: … |  |  |
| 3.2.12.2.10.2.2. | The number of complete applicable test cycles required for regeneration (distance ‘d’) |  |  |
| 3.2.12.2.10.3. | Description of method employed to determine the number of cycles between two cycles where regenerative phases occur: … |  |  |
| 3.2.12.2.10.4. | Parameters to determine the level of loading required before regeneration occurs (i.e. temperature, pressure etc.): … |  |  |
| 3.2.12.2.10.5. | Description of method used to load system: … |  |  |
| 3.2.12.2.11. | Catalytic converter systems using consumable reagents (provide the information below for each separate unit) yes/no (1) |  |  |
| 3.2.12.2.11.1. | Type and concentration of reagent needed: … |  |  |
| 3.2.12.2.11.2. | Normal operational temperature range of reagent: … |  |  |
| 3.2.12.2.11.3. | International standard: … |  |  |
| 3.2.12.2.11.4. | Frequency of reagent refill: continuous/maintenance (where appropriate): |  |  |
| 3.2.12.2.11.5. | Reagent indicator: (description and location) |  |  |
| 3.2.12.2.11.6. | Reagent tank |  |  |
| 3.2.12.2.11.6.1. | Capacity: … |  |  |
| 3.2.12.2.11.6.2. | Heating system: yes/no |  |  |
| 3.2.12.2.11.6.2.1. | Description or drawing |  |  |
| 3.2.12.2.11.7. | Reagent control unit: yes/no (1) |  |  |
| 3.2.12.2.11.7.1. | Make: … |  |  |
| 3.2.12.2.11.7.2. | Type: … |  |  |
| 3.2.12.2.11.8. | Reagent injector (make type and location): … |  |  |
| 3.2.12.2.11.9. | Reagent quality sensor (make, type and location): … |  |  |
| 3.2.12.2.12. | Water injection: yes/no (1) |  |  |
| 3.2.14. | Details of any devices designed to influence fuel economy (if not covered by other items):.… |  |  |
| 3.2.15. | LPG fuelling system: yes/no (1) |  |  |
| 3.2.15.1. | Approval number (approval number of UN Regulation No. 67): … |  |  |
| 3.2.15.2. | Electronic engine management control unit for LPG fuelling |  |  |
| 3.2.15.2.1. | Make(s): … |  |  |
| 3.2.15.2.2. | Type(s): … |  |  |
| 3.2.15.2.3. | Emission-related adjustment possibilities: … |  |  |
| 3.2.15.3. | Further documentation |  |  |
| 3.2.15.3.1. | Description of the safeguarding of the catalyst at switch-over from petrol to LPG or back: … |  |  |
| 3.2.15.3.2. | System lay-out (electrical connections, vacuum connections compensation hoses, etc.): … |  |  |
| 3.2.15.3.3. | Drawing of the symbol: … |  |  |
| 3.2.16. | NG fuelling system: yes/no (1) |  |  |
| 3.2.16.1. | Approval number (approval number of UN Regulation No. 110): |  |  |
| 3.2.16.2. | Electronic engine management control unit for NG fuelling |  |  |
| 3.2.16.2.1. | Make(s): … |  |  |
| 3.2.16.2.2. | Type(s): … |  |  |
| 3.2.16.2.3. | Emission-related adjustment possibilities: … |  |  |
| 3.2.16.3. | Further documentation |  |  |
| 3.2.16.3.1. | Description of the safeguarding of the catalyst at switch-over from petrol to NG or back: … |  |  |
| 3.2.16.3.2. | System lay-out (electrical connections, vacuum connections compensation hoses, etc.): … |  |  |
| 3.2.16.3.3. | Drawing of the symbol: … |  |  |
| 3.2.18. | Hydrogen fuelling system: yes/no (1) (*) |  |  |
| 3.2.18.1. | Type approval number according to UN Regulation No. 134 (if applicable): ………... |  |  |
| 3.2.18.1.1. | Nominal working pressure of hydrogen tank(s): … MPa |  |
| 3.2.18.1.2. | Total volume of hydrogen tank(s): …m3 |  |
| 3.2.18.2. | Electronic engine management control unit for hydrogen fuelling |  |  |
| 3.2.18.2.1. | Make(s): … |  |  |
| 3.2.18.2.2. | Type(s): … |  |  |
| 3.2.18.2.3. | Emission-related adjustment possibilities: … |  |  |
| 3.2.18.3. | Further documentation |  |  |
| 3.2.18.3.1. | Description of the safeguarding of the catalyst at switch-over from petrol to hydrogen or back: … |  |  |
| 3.2.18.3.2. | System lay-out (electrical connections, vacuum connections compensation hoses, etc.): … |  |  |
| 3.2.18.3.3. | Drawing of the symbol: … |  |  |
| 3.2.19.4. | Further documentation |  |  |
| 3.2.19.4.2. | System lay-out (electrical connections, vacuum connections compensation hoses, etc.): … |  |  |
| 3.2.19.4.3. | Drawing of the symbol: … |  |  |
| 3.2.20. | Heat storage information |  |  |
| 3.2.20.1. | Active heat storage device: yes/no (1) |  |  |
| 3.2.20.1.1. | Enthalpy: … (J) |  |  |
| 3.2.20.2. | Insulation materials: yes/no (1) |  |  |
| 3.2.20.2.1. | Insulation material: … (x) |  |  |
| 3.2.20.2.2. | Insulation nominal volume: … (l) (x) |  |  |
| 3.2.20.2.3. | Insulation nominal weight: … (kg) (x) |  |  |
| 3.2.20.2.4. | Insulation location: … (x) |  |  |
| 3.2.20.2.5. | Worst case approach vehicle cool down: yes/no (1) |  |  |
| 3.2.20.2.5.1. | (not worst case approach) Minimum soaking time, tsoak_ATCT (hours):… (x) |  |  |
| 3.2.20.2.5.2. | (not worst case approach) Location of the engine temperature measurement: … (x) |  |  |
| 3.2.20.2.6. | Single interpolation family within the ATCT family approach: yes/no (1) |  |  |
| 3.2.20.2.7. | Worst case approach with regards to insulation: yes/no (1) |  |  |
| 3.2.20.2.7.1. | Description of the ATCT measured reference vehicle regarding insulation: … |  |  |
| 3.3. | Electric powertrain (for PEV only) |  |  |
| 3.3.1. | General description of electric powertrain |  |  |
| 3.3.1.1. | Make: .......................... |  |  |
| 3.3.1.2. | Type: .......................... |  |  |
| 3.3.1.3. | Use (1): Monomotor/multimotors (number): .......................... |  |  |
| 3.3.1.4. | Transmission arrangement: parallel/transaxial/others, to precise: .......................... |  |  |
| 3.3.1.5. | Test voltage: .......................... V |  |  |
| 3.3.1.6. | Motor nominal speed: .......................... min-1 |  |  |
| 3.3.1.7. | Motor maximum speed: ..........................min-1 or by default: reducer outlet shaft/gear box speed (specify gear engaged): .......................... min-1 |  |  |
| 3.3.1.9. | Maximum power: .......................... kW |  |  |
| 3.3.1.10. | Maximum thirty minutes power: .......................... kW |  |  |
| 3.3.1.11. | Flexible range (where P > 90 per cent of max. power): speed at the beginning of range: .......................... min-1 speed at the end of range: .......................... min-1 |  |  |
| 3.3.2. | Traction REESS |  |  |
| 3.3.2.1. | Trade name and mark of the REESS: .......................... |  |  |
| 3.3.2.2. | Kind of electro-chemical couple: .......................... |  |  |
| 3.3.2.3. | Nominal voltage: .......................... V |  |  |
| 3.3.2.4. | REESS maximum thirty minutes power (constant power discharge): .......................... kW |  |  |
| 3.3.2.5. | REESS performance in 2 h discharge (constant power or constant current): (1) |  |  |
| 3.3.2.5.1. | REESS energy: .......................... kWh |  |  |
| 3.3.2.5.2. | REESS capacity: .......................... Ah in 2 h |  |  |
| 3.3.2.5.3. | End of discharge voltage value: .......................... V |  |  |
| 3.3.2.6. | Indication of the end of the discharge that leads to a compulsory stop of the vehicle: (1) .......................... |  |  |
| 3.3.2.7. | REESS mass: .......................... kg |  |  |
| 3.3.2.8. | Number of cells:…… |  |  |
| 3.3.2.9. | REESS position:………… |  |  |
| 3.3.2.10. | Type of coolant : air/liquid (1) |  |  |
| 3.3.2.11. | Battery management system control unit |  |  |
| 3.3.2.11.1. | Make: ……….. |  |  |
| 3.3.2.11.2. | Type: ……. |  |  |
| 3.3.2.11.3. | Identification number: ….. |  |  |
| 3.3.3. | Electric Motor |  |  |
| 3.3.3.1. | Working principle: |  |  |
| 3.3.3.1.1. | direct current/alternating current (1) /number of phases: .......................... |  |  |
| 3.3.3.1.2. | separate excitation/series/compound (1) |  |  |
| 3.3.3.1.3. | synchronous/asynchronous (1) |  |  |
| 3.3.3.1.4. | coiled rotor/with permanent magnets/with housing (1) |  |  |
| 3.3.3.1.5. | number of poles of the motor: .......................... |  |  |
| 3.3.3.2. | Inertia mass: .......................... |  |  |
| 3.3.4. | Power controller |  |  |
| 3.3.4.1. | Make : .......................... |  |  |
| 3.3.4.2. | Type : .......................... |  |  |
| 3.3.4.2.1. | Identification number: ….. |  |  |
| 3.3.4.3. | Control principle: vectorial/open loop/closed/other (to be specified): (1) .......................... |  |  |
| 3.3.4.4. | Maximum effective current supplied to the motor: (2) .......................... A during .......................... seconds |  |  |
| 3.3.4.5. | Voltage range use: .......................... V to .......................... V |  |  |
| 3.3.5. | Cooling system: Motor: liquid/air (1) Controller: liquid/air (1) |  |  |
| 3.3.5.1. | Liquid-cooling equipment characteristics: |  |  |
| 3.3.5.1.1. | Nature of the liquid .......................... circulating pumps: yes/no (1) |  |  |
| 3.3.5.1.2. | Characteristics or make(s) and type(s) of the pump: .......................... |  |  |
| 3.3.5.1.3. | Thermostat: setting: .......................... |  |  |
| 3.3.5.1.4. | Radiator: drawing(s) or make(s) and type(s): .......................... |  |  |
| 3.3.5.1.5. | Relief valve: pressure setting: .......................... |  |  |
| 3.3.5.1.6. | Fan: characteristics or make(s) and type(s): .......................... |  |  |
| 3.3.5.1.7. | Fan duct: .......................... |  |  |
| 3.3.5.2. | Air-cooling equipment characteristics |  |  |
| 3.3.5.2.1. | Blower: characteristics or make(s) and type(s): .......................... |  |  |
| 3.3.5.2.2. | Standard air ducting: .......................... |  |  |
| 3.3.5.2.3. | Temperature regulating system: yes/no (1) |  |  |
| 3.3.5.2.4. | Brief description: .......................... |  |  |
| 3.3.5.2.5. | Air filter: .......................... make(s): .......................... type(s): |  |  |
| 3.3.5.3. | Temperatures admitted by the manufacturer (maximum) |  |  |
| 3.3.5.3.1. | Motor outlet: ..........................° C |  |  |
| 3.3.5.3.2. | controller inlet: ..........................° C |  |  |
| 3.3.5.3.3. | at motor reference point(s): ..........................° C |  |  |
| 3.3.5.3.4. | at controller reference point(s): ..........................° C |  |  |
| 3.3.6. | Insulating category: .......................... |  |  |
| 3.3.7. | International protection (IP)-code: .......................... |  |  |
| 3.3.8. | Lubrication system principle: (1) Bearings: friction/ball Lubricant: grease/oil Seal: yes/no Circulation: with/without |  |  |
| 3.3.9. | Charger |  |  |
| 3.3.9.1. | Charger: on board/external (1) in case of an external unit, define the charger (trademark, model): .......................... |  |  |
| 3.3.9.2. | Description of the normal profile of charge: |  |  |
| 3.3.9.3. | Specification of mains: |  |  |
| 3.3.9.3.1. | Type of mains: single phase/three phase (1) |  |  |
| 3.3.9.3.2. | Voltage: .......................... |  |  |
| 3.3.9.4. | Rest period recommended between the end of the discharge and the start of the charge: .......................... |  |  |
| 3.3.9.5. | Theoretical duration of a complete charge: .......................... |  |  |
| 3.3.10. | Electric energy converters |  |  |
| 3.3.10.1. | Electric energy converter between the electric machine and traction REESS |  |  |
| 3.3.10.1.1. | Make : .......................... |  |  |
| 3.3.10.1.2. | Type : .......................... |  |  |
| 3.3.10.1.3. | Declared nominal power : .......................... W |  |  |
| 3.3.10.2. | Electric energy converter between the traction REESS and low voltage power supply |  |  |
| 3.3.10.2.1. | Make : .......................... |  |  |
| 3.3.10.2.2. | Type : .......................... |  |  |
| 3.3.10.2.3. | Declared nominal power : .......................... W |  |  |
| 3.3.10.3. | Electric energy converter between the recharge-plug-in and traction REESS |  |  |
| 3.3.10.3.1. | Make : .......................... |  |  |
| 3.3.10.3.2. | Type : .......................... |  |  |
| 3.3.10.3.3. | Declared nominal power : .......................... W |  |  |
| 3.4. | Combinations of propulsion energy converters |  |  |
| 3.4.1. | Hybrid electric vehicle: yes/no (1) |  |  |
| 3.4.2. | Category of hybrid electric vehicle: off-vehicle charging/not off-vehicle charging: (1) |  |  |
| 3.4.3. | Operating mode switch: with/without (1) |  |  |
| 3.4.3.1. | Selectable modes |  |  |
| 3.4.3.1.1. | Pure electric: yes/no (1) |  |  |
| 3.4.3.1.2. | Pure fuel consuming: yes/no (1) |  |  |
| 3.4.3.1.3. | Hybrid modes: yes/no (1) (if yes, short description): … |  |  |
| 3.4.4. | Description of the energy storage device: (REESS, capacitor, flywheel/generator) |  |  |
| 3.4.4.1. | Make(s): … |  |  |
| 3.4.4.2. | Type(s): … |  |  |
| 3.4.4.3. | Identification number: … |  |  |
| 3.4.4.4. | Kind of electrochemical couple: … |  |  |
| 3.4.4.5. | Energy: … (for REESS: voltage and capacity Ah in 2 h, for capacitor: J, …) |  |  |
| 3.4.4.6. | Charger: on board/external/without (1) |  |  |
| 3.4.4.7. | Type of coolant : air/liquid (1) |  |  |
| 3.4.4.8. | Battery management system control unit |  |  |
| 3.4.4.8.1. | Make: ……….. |  |  |
| 3.4.4.8.2. | Type: ……. |  |  |
| 3.4.4.8.3. | Identification number: ….. |  |  |
| 3.4.5. | Electric machine (describe each type of electric machine separately) |  |  |
| 3.4.5.1. | Make: … |  |  |
| 3.4.5.2. | Type: … |  |  |
| 3.4.5.3. | Primary use: traction motor/generator (1) |  |  |
| 3.4.5.3.1. | When used as traction motor: single-/multimotors (number) (1): … |  |  |
| 3.4.5.4. | Maximum power: … kW |  |  |
| 3.4.5.5. | Working principle |  |  |
| 3.4.5.5.5.1 | Direct current/alternating current/number of phases: … |  |  |
| 3.4.5.5.2. | Separate excitation/series/compound (1) |  |  |
| 3.4.5.5.3. | Synchronous/asynchronous (1) |  |  |
| 3.4.5.6. | Cooling system: Motor: liquid/air (1) Controller: liquid/air (1) |  |  |
| 3.4.5.6.1. | Liquid-cooling equipment characteristics: |  |  |
| 3.4.5.6.1.1. | Nature of the liquid .......................... circulating pumps: yes/no (1) |  |  |
| 3.4.5.6.1.2. | Characteristics or make(s) and type(s) of the pump: .......................... |  |  |
| 3.4.5.6.1.3. | Thermostat: setting: .......................... |  |  |
| 3.4.5.6.1.4. | Radiator: drawing(s) or make(s) and type(s): .......................... |  |  |
| 3.4.5.6.1.5. | Relief valve: pressure setting: .......................... |  |  |
| 3.4.5.6.1.6. | Fan: characteristics or make(s) and type(s): .......................... |  |  |
| 3.4.5.6.1.7. | Fan duct: .......................... |  |  |
| 3.4.5.6.2. | Air-cooling equipment characteristics |  |  |
| 3.4.5.6.2.1. | Blower: characteristics or make(s) and type(s): .......................... |  |  |
| 3.4.5.6.2.2. | Standard air ducting: .......................... |  |  |
| 3.4.5.6.2.3. | Temperature regulating system: yes/no (1) |  |  |
| 3.4.5.6.2.4. | Brief description: .......................... |  |  |
| 3.4.5.6.2.5. | Air filter: .......................... make(s): .......................... type(s): |  |  |
| 3.4.5.6.3. | Temperatures admitted by the manufacturer (maximum) |  |  |
| 3.4.5.6.3.1. | Motor outlet: ..........................° C |  |  |
| 3.4.5.6.3.2. | controller inlet: ..........................° C |  |  |
| 3.4.5.6.3.3. | at motor reference point(s): ..........................° C |  |  |
| 3.4.5.6.3.4. | at controller reference point(s): ..........................° C |  |  |
| 3.4.6. | Control unit |  |  |
| 3.4.6.1. | Make(s): … |  |  |
| 3.4.6.2. | Type(s): … |  |  |
| 3.4.6.3. | Identification number: … |  |  |
| 3.4.7. | Power controller |  |  |
| 3.4.7.1. | Make: … |  |  |
| 3.4.7.2. | Type: … |  |  |
| 3.4.7.3. | Identification number: … |  |  |
| 3.4.9. | Manufacturer's recommendation for preconditioning: … |  |  |
| 3.4.10. | FCHV: yes/no (1) (*) |  |  |
| 3.4.10.1. | Type of Fuel Cell |  |  |
| 3.4.10.1.2. | Make: … |  |  |
| 3.4.10.1.3. | Type: … |  |  |
| 3.4.10.1.4. | Nominal Voltage (V): … |  |  |
| 3.4.10.1.5. | Type of coolant: air/liquid (1) |  |  |
| 3.4.10.2. | System description (working principle of the fuel cell, drawing, etc.): … |  |  |
| 3.4.11. | Electric energy converters |  |  |
| 3.4.11.1. | Electric energy converter between the electric machine and traction REESS |  |  |
| 3.4.11.1.1. | Make : .......................... |  |  |
| 3.4.11.1.2. | Type : .......................... |  |  |
| 3.4.11.1.3. | Declared nominal power : .......................... W |  |  |
| 3.4.11.2. | Electric energy converter between the traction REESS and low voltage power supply |  |  |
| 3.4.11.2.1. | Make : .......................... |  |  |
| 3.4.11.2.2. | Type : .......................... |  |  |
| 3.4.11.2.3. | Declared nominal power : .......................... W |  |  |
| 3.4.11.3. | Electric energy converter between the recharge-plug-in and traction REESS |  |  |
| 3.4.11.3.1. | Make : .......................... |  |  |
| 3.4.11.3.2. | Type : .......................... |  |  |
| 3.4.11.3.3. | Declared nominal power : .......................... W |  |  |
| 3.5. | Manufacturer’s declared values for determination of CO2 emissions/fuel consumption/electric energy consumption/electric range/driving range of hydrogen |  |  |
| 3.5.7. | Manufacturer’s declared values |  |  |
| 3.5.7.1. | Test vehicle parameters Vehicle; Vehicle Low (VL) if existing; Vehicle High (VH); VM if existing; V representative (only for road load matrix family*); Default values. Vehicle bodywork type; -. Road load method used (measurement or calculation by road load family); -; -. Road load information:. Tyres make and type, if measurement; -. Tyre dimensions (front/rear), if measurement; -. Tyre rolling resistance (front/rear) (kg/t). Tyre pressure (front/rear) (kPa), if measurement. Delta CD × A of vehicle L compared to vehicle H (IP_H minus IP_L); -; -; -. Delta CD × A compared to road load family vehicle L (IP_H/L minus RL_L), if calculation by road load family; -; -. Vehicle test mass (kg). Road load coefficients. f0 (N). f1 (N/(km/h)). f2 (N/(km/h)²). Frontal area m² (0.000 m²); -; -; -. Cycle Energy Demand (J). * representative vehicle is tested for the road load matrix family. |
| 3.5.7.1.1. | Fuel used for the Type 1 test and selected for the measurement of the net power in accordance with UN Regulation No. 85 (for LPG or NG vehicles only): … |  |  |
| 3.5.7.2. | Combined CO2 emissions |  |  |
| 3.5.7.2.1. | CO2 emission for pure ICE vehicles and NOVC-HEVs |  |  |
| 3.5.7.2.1.0. | Minimum and maximum CO2 values within the interpolation family: … g/km |  |  |
| 3.5.7.2.1.1. | Vehicle high: … g/km |  |  |
| 3.5.7.2.1.2. | Vehicle low (if applicable): … g/km |  |  |
| 3.5.7.2.1.3. | Vehicle M (if applicable): … g/km |  |  |
| 3.5.7.2.2. | Charge-Sustaining CO2 emission for OVC-HEVs |  |  |
| 3.5.7.2.2.1. | Charge-Sustaining CO2 emission vehicle high: g/km |  |  |
| 3.5.7.2.2.2. | Charge-Sustaining CO2 emission vehicle low (if applicable): g/km |  |  |
| 3.5.7.2.2.3. | Charge-Sustaining CO2 emission vehicle M (if applicable): g/km |  |  |
| 3.5.7.2.3. | Charge-Depleting CO2 emission and weighted CO2 emission for OVC-HEVs |  |  |
| 3.5.7.2.3.1. | Charge-Depleting CO2 emission of Vehicle high: … g/km |  |  |
| 3.5.7.2.3.2. | Charge-Depleting CO2 emission of Vehicle low (if applicable): … g/km |  |  |
| 3.5.7.2.3.3. | Charge-Depleting CO2 emission of Vehicle M (if applicable): … g/km |  |  |
| 3.5.7.2.3.4. | Minimum and maximum weighted CO2 values within the OVC interpolation family: … g/km |  |  |
| 3.5.7.3. | Electric range for electrified vehicles |  |  |
| 3.5.7.3.1. | Pure Electric Range (PER) for PEVs at ambient temperature (23 °C) |  |  |
| 3.5.7.3.1.1. | Vehicle high: … km |  |  |
| 3.5.7.3.1.2. | Vehicle low (if applicable): … km |  |  |
| 3.5.7.3.2. | All Electric Range (AER) and Equivalent All Electric Range (EAER) for OVC-HEVs and OVC-FCHVs (as applicable) |  |  |
| 3.5.7.3.2.1. | Vehicle high: AER: … km , EAER: … km |  |  |
| 3.5.7.3.2.2. | Vehicle low (if applicable): AER: … km , EAER: … km |  |  |
| 3.5.7.3.2.3. | Vehicle M (if applicable): AER: … km , EAER: … km |  |  |
| 3.5.7.3.3. | Pure Electric Range (PER) for PEVs at low temperature (-7 °C) |  |  |
| 3.5.7.3.3.1. | Declared low temperature pure electric range ratio, KPER,WLTC,LT,dec |  |  |
| 3.5.7.3.3.1.1. | Low temperature pure electric range, vehicle low: … km |  |  |
| 3.5.7.3.3.1.2. | Low temperature pure electric range vehicle high: … km |  |  |
| 3.5.7.4. | Fuel consumption (FCCS) for NOVC-FCHVs and OVC-FCHVs (as applicable) |  |  |
| 3.5.7.4.1. | Charge-Sustaining fuel consumption for NOVC-FCHVs and OVC-FCHVs (as applicable) |  |  |
| 3.5.7.4.1.1. | Vehicle high: … kg/100 km |  |  |
| 3.5.7.4.1.2. | Vehicle low (if applicable): … kg/100 km |  |  |
| 3.5.7.4.1.3. | Vehicle M (if applicable): … kg/100 km |  |  |
| 3.5.7.4.2. | Charge-Depleting fuel consumption for OVC-FCHVs (as applicable) |  |  |
| 3.5.7.4.2.1. | Vehicle high: … kg/100 km |  |  |
| 3.5.7.4.2.2. | Vehicle low (if applicable): … kg/100 km |  |  |
| 3.5.7.4.2.3. | Vehicle M (if applicable): … kg/100 km |  |
| 3.5.7.5. | Electric energy consumption for electrified vehicles |  |  |
| 3.5.7.5.1. | Combined electric energy consumption (ECWLTC) for Pure electric vehicles |  |  |
| 3.5.7.5.1.1. | Vehicle high: … Wh/km |  |  |
| 3.5.7.5.1.2. | Vehicle low (if applicable): … Wh/km |  |  |
| 3.5.7.5.2. | UF-weighted charge-depleting electric energy consumption ECAC,CD (combined) |  |  |
| 3.5.7.5.2.1. | Vehicle high: … Wh/km |  |  |
| 3.5.7.5.2.2. | Vehicle low (if applicable): … Wh/km |  |  |
| 3.5.7.5.2.3. | Vehicle M (if applicable): … Wh/km |  |  |
| 3.5.7.6. | Fuel efficiency and Driving Range of hydrogen |  |  |
| 3.5.7.6.1. | Fuel efficiency for pure ICE vehicles and NOVC-HEVs |  |  |
| 3.5.7.6.1.1. | Vehicle high: … km/l |  |  |
| 3.5.7.6.1.2. | Vehicle low (if applicable): … km/l |  |  |
| 3.5.7.6.1.3. | Vehicle M (if applicable): … km/l |  |  |
| 3.5.7.6.2. | Charge-Sustaining fuel efficiency for OVC-HEVs |  |  |
| 3.5.7.6.2.1. | Charge-Sustaining fuel efficiency vehicle high: km/l |  |  |
| 3.5.7.6.2.2. | Charge-Sustaining fuel efficiency vehicle low (if applicable): km/l |  |  |
| 3.5.7.6.2.3. | Charge-Sustaining fuel efficiency vehicle M (if applicable): km/l |  |  |
| 3.5.7.6.3. | Charge-Depleting fuel efficiency for OVC-HEVs |  |  |
| 3.5.7.6.3.1. | Charge-Depleting fuel efficiency of Vehicle high: … km/l |  |  |
| 3.5.7.6.3.2. | Charge-Depleting fuel efficiency of Vehicle low (if applicable): … km/l |  |  |
| 3.5.7.6.3.3. | Charge-Depleting fuel efficiency of Vehicle M (if applicable): … km/l |  |  |
| 3.5.7.6.4. | Fuel efficiency and Driving Range of hydrogen for NOVC-FCHVs and OVC-FCHVs |  |  |
| 3.5.7.6.4.1. | Charge-Sustaining fuel efficiency and Driving Range of hydrogen for NOVC-FCHVs and OVC-FCHVs (as applicable) |  |  |
| 3.5.7.6.4.1.1. | Vehicle high: … km/kg |  |  |
| 3.5.7.6.4.1.2. | Vehicle low (if applicable): … km/kg |  |  |
| 3.5.7.6.4.1.3. | Vehicle M (if applicable): … km/kg |  |
| 3.5.7.6.4.1.4. | PLL for determination of Driving Range of hydrogen (if applicable): …MPa |  |  |
| 3.5.7.6.4.2. | Charge-Depleting fuel efficiency for OVC-FCHVs (if applicable) |  |  |
| 3.5.7.6.4.2.1. | Vehicle high: … km/kg |  |  |
| 3.5.7.6.4.2.2. | Vehicle low (if applicable): … km/kg |  |  |
| 3.5.7.6.4.2.3. | Vehicle M (if applicable): … km/kg |  |  |
| 3.6. | Temperatures permitted by the manufacturer |  |  |
| 3.6.1. | Cooling system |  |  |
| 3.6.1.1. | Liquid cooling Maximum temperature at outlet: … K |  |  |
| 3.6.1.2. | Air cooling |  |  |
| 3.6.1.2.1. | Reference point: … |  |  |
| 3.6.1.2.2. | Maximum temperature at reference point: … K |  |  |
| 3.6.2. | Maximum outlet temperature of the inlet intercooler: … K |  |  |
| 3.6.3. | Maximum exhaust temperature at the point in the exhaust pipe(s) adjacent to the outer flange(s) of the exhaust manifold or turbocharger: … K |  |  |
| 3.6.4. | Fuel temperature Minimum: … K — maximum: … K For diesel engines at injection pump inlet, for gas fuelled engines at pressure regulator final stage |  |  |
| 3.6.5. | Lubricant temperature Minimum: … K — maximum: … K |  |  |
| 3.8. | Lubrication system |  |  |
| 3.8.1. | Description of the system |  |  |
| 3.8.1.1. | Position of lubricant reservoir: … |  |  |
| 3.8.1.2. | Feed system (by pump/injection into intake/mixing with fuel, etc.) (1) |  |  |
| 3.8.2. | Lubricating pump |  |  |
| 3.8.2.1. | Make(s): … |  |  |
| 3.8.2.2. | Type(s): … |  |  |
| 3.8.3. | Mixture with fuel |  |  |
| 3.8.3.1. | Percentage: … |  |  |
| 3.8.4. | Oil cooler: yes/no (1) |  |  |
| 3.8.4.1. | Drawing(s): … or |  |  |
| 3.8.4.1.1. | Make(s): … |  |  |
| 3.8.4.1.2. | Type(s): … |  |  |
| 3.8.5. | Lubricant specification: …W… |  |  |
| 4. | TRANSMISSION (p) |  |  |
| 4.3. | Moment of inertia of engine flywheel: … |  |  |
| 4.3.1. | Additional moment of inertia with no gear engaged: … |  |  |
| 4.4. | Clutch(es) |  |  |
| 4.4.1. | Type: … |  |  |
| 4.4.2. | Maximum torque conversion: … |  |  |
| 4.5. | Gearbox |  |  |
| 4.5.1. | Type (manual/automatic/CVT (continuously variable transmission)) (1) |  |  |
| 4.5.1.4. | Torque rating: … |  |  |
| 4.5.1.5. | Number of clutches: … |  |  |
| 4.6. | Gear ratios |  |  |
|  | Gear; Internal gearbox ratios (ratios of engine to gearbox output shaft revolutions); Final drive ratio(s) (ratio of gearbox output shaft to driven wheel revolutions); Total gear ratios. Maximum for CVT. 1. 2. 3. …. Minimum for CVT. |
| 4.6.1. | Gearshift |  |  |
| 4.6.1.1. | Gear 1 excluded: yes/no (1) |  |  |
| 4.6.1.2. | n95_high for each gear: … min–1 |  |  |
| 4.6.1.3. | nmin_drive |  |  |
| 4.6.1.3.1. | 1st gear: … min–1 |  |  |
| 4.6.1.3.2. | 1st gear to 2nd: … min–1 |  |  |
| 4.6.1.3.3. | 2nd gear to standstill: … min–1 |  |  |
| 4.6.1.3.4. | 2nd gear: … min–1 |  |  |
| 4.6.1.3.5. | 3rd gear and beyond: … min–1 |  |  |
| 4.6.1.4. | nmin_drive_set for acceleration/constant speed phases (n_min_drive_up): … min–1 |  |  |
| 4.6.1.5. | nmin_drive_set for deceleration phases (nmin_drive_down): |  |  |
| 4.6.1.6. | initial period of time |  |  |
| 4.6.1.6.1. | tstart_phase: … s |  |  |
| 4.6.1.6.2. | nmin_drive_start: … min–1 |  |  |
| 4.6.1.6.3. | nmin_drive_up_start: … min–1 |  |  |
| 4.6.1.7. | use of ASM: yes/no (1) |  |  |
| 4.6.1.7.1. | ASM values: … at … min-1 |  |  |
| 4.7. | Maximum vehicle design speed (in km/h) (q): … |  |  |
| 4.12. | Gearbox lubricant: …W… |  |  |
| 6. | SUSPENSION |  |  |
| 6.6. | Tyres and wheels |  |  |
| 6.6.1. | Tyre/wheel combination(s) |  |  |
| 6.6.1.1. | Axles |  |  |
| 6.6.1.1.1. | Axle 1: … |  |  |
| 6.6.1.1.1.1. | Tyre size designation |  |  |
| 6.6.1.1.2. | Axle 2: … |  |  |
| 6.6.1.1.2.1. | Tyre size designation |  |  |
|  | etc. |  |  |
| 6.6.2. | Upper and lower limits of rolling radii |  |  |
| 6.6.2.1. | Axle 1: … |  |  |
| 6.6.2.2. | Axle 2: … |  |  |
| 6.6.3. | Tyre pressure(s) as recommended by the vehicle manufacturer: … kPa |  |  |
| 9. | BODYWORK |  |  |
| 9.1. | Type of bodywork (c): … |  |  |
| 9.10.3. | Seats |  |  |
| 9.10.3.1. | Number of seating positions: … |  |  |
| 12. | MISCELLANEOUS |  |  |
| 12.10. | Devices or systems with driver selectable modes which influence CO2 emissions, fuel consumption, electric energy consumption and/or criteria emissions and do not have a predominant mode: yes/no (1) |  |  |
| 12.10.1. | Charge-sustaining test (if applicable) (state for each device or system) |  |  |
| 12.10.1.0. | Predominant mode under CS condition: yes/no (1) |  |  |
| 12.10.1.0.1. | Predominant mode under CS condition: … (if applicable) |  |  |
| 12.10.1.1. | Best case mode: … (if applicable) |  |  |
| 12.10.1.2. | Worst case mode: … (if applicable) |  |  |
| 12.10.1.3. | Mode which enables the vehicle to follow the reference test cycle: … (in case no predominant mode under CS condition and only one mode is able to follow the reference test cycle) |  |  |
| 12.10.2. | Charge-depleting test (if applicable) (state for each device or system) |  |  |
| 12.10.2.0. | Predominant mode under CD condition: yes/no (1) |  |  |
| 12.10.2.0.1. | Predominant mode under CD condition: … (if applicable) |  |  |
| 12.10.2.1. | Most energy consuming mode: … (if applicable) |  |  |
| 12.10.2.2. | Mode which enables the vehicle to follow the reference test cycle: … (in case no predominant mode under CD condition and only one mode is able to follow the reference test cycle) |  |  |
| 12.10.3. | Type 1 test (if applicable) (state for each device or system) |  |  |
| 12.10.3.1. | Best case mode: … |  |  |
| 12.10.3.2. | Worst case mode: … |  |  |

Explanatory notes

(1)	Delete where not applicable (there are cases where nothing needs to be deleted when more than one entry is applicable).

(2)	Specify the tolerance.

(3)	Please fill in here the upper and lower values for each variant.

(6)	(Reserved)

(7)	Optional equipment that affects the dimensions of the vehicle shall be specified.

(x)	For insulation nominal volume and insulation nominal weight, state to 2 decimal places. A tolerance of +/- 10 per cent shall be applied for insulation volume and insulation weight. Not to be documented if “no” in paragraph 3.2.20.2.5. or 3.2.20.2.7.

(c)	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.6, paragraph 2. - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html.

(f)	Where there is one version with a normal cab and another with a sleeper cab, both sets of masses and dimensions are to be stated.

(g)	Standard ISO 612: 1978 — Road vehicles — Dimensions of motor vehicles and towed vehicles — terms and definitions.

(h)	The mass of the driver is assessed at 75 kg.

The liquid containing systems (except those for used water that must remain empty) are filled to 100 % of the capacity specified by the manufacturer.

(i)	For trailers or semi-trailers, and for vehicles coupled with a trailer or a semi-trailer, which exert a significant vertical load on the coupling device or the fifth wheel, this load, divided by standard acceleration of gravity, is included in the maximum technically permissible mass.

(k)	In the case of a vehicle that can run either on petrol, diesel, etc., or also in combination with another fuel, items shall be repeated.

In the case of non-conventional engines and systems, particulars equivalent to those referred to here shall be supplied by the manufacturer.

(l)	This figure shall be rounded off to the nearest tenth of a millimetre.

(m)	This value shall be calculated (π = 3.1416) and rounded off to the nearest cm3.

(n)	Determined in accordance with the requirements of UN Regulation No. 85.

(p)	The specified particulars are to be given for any proposed variants.

(q)	With respect to trailers, maximum speed permitted by the manufacturer.

(*)	Paragraph 3.2.18 shall apply to hydrogen fuel system of OVC-FCHV and NOVC-FCHV.

Annex A1 - Appendix 1

WLTP Test Report

Test Reports

A Test Report is the report issued by the technical service responsible for conducting the tests according this regulation.

Part I

The following information, if applicable, is the minimum data required for the Type 1 test.

Report number

| APPLICANT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manufacturer |  |  |  |  |  |  |  |
| SUBJECT | … |  |  |  |  |  |  |
|  | Level approval requested (tick a box) |  |  |  | Level 1A | Level 1B | Level 2 |
| Roadload family identifier(s) |  |  | : |  |  |  |  |
| Interpolation family identifier(s) |  |  | : |  |  |  |  |
| Object submitted to tests |  |  |  |  |  |  |  |
|  |  | Make | : |  |  |  |  |
|  |  | IP identifier | : |  |  |  |  |
| CONCLUSION |  | The object submitted to tests complies with the requirements mentioned in the subject. |  |  |  |  |  |

| place, | DD/MM/YYYY |
| --- | --- |

General notes:

If there are several options (references), the one tested should be described in the test report.

If there are not, a single reference to the information document at the start of the test report may be sufficient.

Every Technical Service is free to include some additional information.

Characters are included in the sections of the test report relating to specific vehicle types, as follows:

"(a)"	 Specific to positive ignition engine vehicles or vehicles ‘G’ (as specified in Table 1B of UN Regulation No. 154) (as applicable).

"(b)"	Specific to compression ignition engine vehicles or vehicles ‘D’ (as specified in Table 1B of UN Regulation No. 154) (as applicable).

1. 	Description of tested vehicle(s): high, low and m (if applicable)

1.1. 	General

| Vehicle numbers | : | Prototype number and VIN |
| --- | --- | --- |
| Category | : |  |
| Bodywork | : |  |
| Drive wheels | : |  |

1.1.1. 	Powertrain Architecture

| Powertrain architecture | : | pure ICE, hybrid, electric or fuel cell |
| --- | --- | --- |

1.1.2. 	Internal Combustion Engine (if applicable)

For more than one ICE, please repeat the point

| Make | : |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | : |  |  |  |  |  |  |  |
| Working principle | : |  | two/four stroke |  |  |  |  |  |
| Cylinders number and arrangement | : |  |  |  |  |  |  |  |
| Engine capacity (cm3) | : |  |  |  |  |  |  |  |
| Engine idling speed (min-1) | : |  |  |  | + - |  |  |  |
| High engine idling speed (min-1) (a) | : |  |  |  | + - |  |  |  |
| Rated engine power | : |  |  | kW |  | at |  | rpm |
| Maximum net torque | : |  |  | Nm |  | at |  | rpm |
| Engine lubricant | : | make and type |  |  |  |  |  |  |
| Cooling system | : | Type: air/water/oil |  |  |  |  |  |  |
| Insulation | : | material, amount, location, nominal volume and nominal weight(4) |  |  |  |  |  |  |

(4)	a tolerance of +/- 10 per cent is permitted for volume and weight

1.1.3. 	Test fuel for Type 1 test (if applicable)

For more than one test fuel, please repeat the point

| Make | : |  |
| --- | --- | --- |
| Type | : | Petrol - Diesel – LPG – NG - … … |
| Density at 15°C | : |  |
| Sulphur content | : | Only for Diesel and Petrol |
|  | : |  |
| Batch number | : |  |
| Willans factors (for ICE) for CO2 emission (gCO2/MJ) | : |  |

1.1.4. 	Fuel feed system (if applicable)

For more than one fuel feed system, please repeat the point

| Direct injection | : | yes/no or description |
| --- | --- | --- |
| Vehicle fuel type | : | Monofuel / bifuel / flex fuel |
| Control unit |  |  |
| Part reference | : | same as information document |
| Software tested | : | read via scantool, for example |
| Air flowmeter | : |  |
| Throttle body | : |  |
| Pressure sensor | : |  |
| Injection pump | : |  |
| Injector(s) | : |  |

1.1.5. 	Intake system (if applicable)

For more than one intake system, please repeat the point

| Pressure charger | : | Yes/no make & type (1) |
| --- | --- | --- |
| Intercooler | : | yes/no type (air/air – air/water) (1) |
| Air filter (element) (1) | : | make & type |
| Intake silencer (1) | : | make & type |

1.1.6. 	Exhaust system and anti-evaporative system (if applicable)

For more than one, please repeat the point

| First catalytic converter | : | make & reference (1) principle: three way / oxidising / NOx trap / NOx storage system / Selective Catalyst Reduction… |
| --- | --- | --- |
| Second catalytic converter | : | make & reference (1) principle: three way / oxidising / NOx trap / NOx storage system / Selective Catalyst Reduction… |
| Particulate trap | : | with/without/not applicable catalysed: yes/no make & reference (1) |
| Reference and position of oxygen and/or lambda sensor(s) | : | before catalyst / after catalyst |
| Air injection | : | with/without/not applicable |
| Water injection | : | with/without/not applicable |
| EGR | : | with/without/not applicable cooled/non-cooled HP/LP |
| Evaporative emission control system | : | with/without/not applicable |
| Reference and position of NOx sensor(s) | : | Before/ after |
| Openings in the exhaust system designed to remove condensate (if applicable) | : | Position |
| General description (1) | : |  |

1.1.7. 	Heat Storage Device (if applicable)

For more than one Heat Storage System, please repeat the point

| Heat storage device | : | yes/no |
| --- | --- | --- |
| Heat capacity (enthalpy stored J) | : |  |
| Time for heat release (s) | : |  |

1.1.8. 	Transmission (if applicable)

For more than one Transmission, please repeat the point

| Gearbox | : | manual / automatic / continuous variation |
| --- | --- | --- |
| Gear shifting procedure |  |  |
| Predominant mode* | : | yes/no normal / drive / eco/… |
| Best case mode for CO2 emissions and fuel consumption (if applicable) | : |  |
| Worst case mode for CO2 emissions and fuel consumption (if applicable) | : |  |
| Highest electric energy consumption mode (if applicable) | : |  |
| Control unit | : |  |
| Gearbox lubricant | : | make and type |
| Tyres |  |  |
| Make | : |  |
| Type | : |  |
| Dimensions front/rear | : |  |
| Dynamic circumference (m) | : |  |
| Tyre pressure (kPa) | : |  |

* for OVC-HEV, specify for charge-sustaining and for charge-depleting operating conditions.

Transmission ratios (R.T.), primary ratios (R.P.) and (vehicle speed (km/h)) / (engine speed (1000 (min-1)) (V1000) for each of the gearbox ratios (R.B.).

| R.B. | R.P. | R.T. | V1000 |
| --- | --- | --- | --- |
| 1st | 1/1 |  |  |
| 2nd | 1/1 |  |  |
| 3rd | 1/1 |  |  |
| 4th | 1/1 |  |  |
| 5th | 1/1 |  |  |
| … |  |  |  |

1.1.9. 	Electric machine (if applicable)

For more than one Electric Machine, please repeat the point

| Make | : |  |
| --- | --- | --- |
| Type | : |  |
| Peak Power (kW) | : |  |

1.1.10. Traction REESS (if applicable)

For more than one Traction REESS, please repeat the point

| Make | : |  |
| --- | --- | --- |
| Type | : |  |
| Capacity (Ah) | : |  |
| Nominal Voltage (V) | : |  |

1.1.11. Fuel cell (if applicable)

For more than one Fuel Cell stack, please repeat the point

| Make | : |  |
| --- | --- | --- |
| Type | : |  |

1.1.12. Power electronics (if applicable)

Can be more than one PE (propulsion converter, low voltage system or charger)

| Make | : |  |
| --- | --- | --- |
| Type | : |  |
| Power (kW) | : |  |

1.2. 	Vehicle high description

1.2.1. 	Mass

| Test mass of VH (kg) | : |  |
| --- | --- | --- |

1.2.2. 	Road load parameters

| f0 (N) | : |  |
| --- | --- | --- |
| f1 (N/(km/h)) | : |  |
| f2 (N/(km/h)²) | : |  |
| Cycle energy demand (J) | : |  |
| Road load test report reference | : |  |
| Road load family’s identifier | : |  |

1.2.3. 	Cycle selection parameters

| Cycle (without downscaling) | : | Class 1 / 2 / 3a / 3b |
| --- | --- | --- |
| Ratio of rated power to mass in running order – 75kg (PMR)(W/kg) | : | (if applicable) |
| Capped speed process used during measurement | : | yes/no |
| Maximum speed of the vehicle (km/h) | : |  |
| Downscaling (if applicable) | : | yes/no |
| Downscaling factor fdsc | : |  |
| Cycle distance (m) | : |  |
| Constant speed (in the case of the shortened test procedure) | : | if applicable |

1.2.4. 	Gear shift point (if applicable)

| Version of Gear Shift calculation |  | indicate the applicable amendment to UN GTR No 15 |
| --- | --- | --- |
| Gear shifting | : | Average gear for v ≥ 1 km/h, x.xxxx |
| nmin_drive |  |  |
| 1st gear | : | …min-1 |
| 1st gear to 2nd | : | …min-1 |
| 2nd gear to standstill | : | …min-1 |
| 2nd gear | : | …min-1 |
| 3rd gear and beyond | : | …min-1 |
| Gear 1 excluded | : | yes/no |
| n95_high for each gear | : | …min-1 |
| nmin_drive_set for acceleration/constant speed phases (nmin_drive_up) | : | …min-1 |
| nmin_drive_set for deceleration phases (nmin_drive_down) | : | …min-1 |
| tstart_phase | : | …s |
| nmin_drive_start | : | …min-1 |
| nmin_drive_up_start | : | …min-1 |
| use of ASM | : | yes/no |
| ASM values | : |  |

1.3. 	Vehicle low description (if applicable)

1.3.1. 	Mass

| Test mass of VL(kg) | : |  |
| --- | --- | --- |

1.3.2. 	Road load parameters

| f0 (N) | : |  |
| --- | --- | --- |
| f1 (N/(km/h)) | : |  |
| f2 (N/(km/h)²) | : |  |
| Cycle energy demand (J) | : |  |
| Δ(CD×Af)LH (m2) | : |  |
| Road load test report reference | : |  |
| Road load family’s identifier | : |  |

1.3.3. 	Cycle Selection parameters

| Cycle (without downscaling) | : | Class 1 / 2 / 3a / 3b |
| --- | --- | --- |
| Ratio of rated power to mass in running order – 75kg (PMR)(W/kg) | : | (if applicable) |
| Capped speed process used during measurement | : | yes/no |
| Maximum speed of the vehicle | : |  |
| Downscaling (if applicable) | : | yes/no |
| Downscaling factor fdsc | : |  |
| Cycle distance (m) | : |  |
| Constant speed (in the case of the shortened test procedure) | : | if applicable |

1.3.4. 	Gear shift point (if applicable)

| Gear shifting | : | Average gear for v ≥ 1 km/h, x.xxxx |
| --- | --- | --- |

1.4. 	Vehicle M description (if applicable)

1.4.1. 	Mass

| Test mass of VM(kg) | : |  |
| --- | --- | --- |

1.4.2. 	Road load parameters

| f0 (N) | : |  |
| --- | --- | --- |
| f1 (N/(km/h)) | : |  |
| f2 (N/(km/h)²) | : |  |
| Cycle energy demand (J) | : |  |
| Δ(CD×Af)LH (m2) | : |  |
| Road load test report reference | : |  |
| Road load family’s identifier | : |  |

1.4.3. 	Cycle Selection parameters

| Cycle (without downscaling) | : | Class 1 / 2 / 3a / 3b |
| --- | --- | --- |
| Ratio of rated power to mass in running order – 75kg (PMR)(W/kg) | : | (if applicable) |
| Capped speed process used during measurement | : | yes/no |
| Maximum speed of the vehicle | : |  |
| Downscaling (if applicable) | : | yes/no |
| Downscaling factor fdsc | : |  |
| Cycle distance (m) | : |  |
| Constant speed (in the case of the shortened test procedure) | : | if applicable |

1.4.4. 	Gear shift point (if applicable)

| Gear shifting | : | Average gear for v ≥ 1 km/h, x.xxxx |
| --- | --- | --- |

2. 	Test results

2.1. 	Type 1 test

| Method of chassis dyno setting | : | Fixed run / iterative / alternative with its own warmup cycle |
| --- | --- | --- |
| Dynamometer in 2WD/4WD operation | : | 2WD/4WD |
| For 2WD operation, was the non-powered axle rotating | : | yes/no/not applicable |
| Dynamometer operation mode . |  | yes/no |
| Coastdown mode | : | yes/no |
| Additional preconditioning | : | yes/no description |
| Deterioration factors | : | assigned / tested |

2.1.1. 	Vehicle high

| Date(s) of test(s) |  |  | : | (day/month/year) |
| --- | --- | --- | --- | --- |
| Place of the test(s) | : | Chassis dyno, location, country |  |  |
| Height of the lower edge above ground of cooling fan (cm) | : |  |  |  |
| Lateral position of fan centre (if modified as request by the manufacturer) | : | in the vehicle centre-line/… |  |  |
| Distance from the front of the vehicle (cm) | : |  |  |  |
| IWR: Inertial Work Rating 4 phase cycle (%) | : | x.x |  |  |
| IWR: Inertial Work Rating 3 phase cycle (%) | : | x.x |  |  |
| RMSSE: Root Mean Squared Speed Error 4 phase cycle (km/h) | : | x.xx |  |  |
| RMSSE: Root Mean Squared Speed Error 3 phase cycle (km/h) | : | x.xx |  |  |
| Description of the accepted deviation of the driving cycle | : | PEV before break off criteria or Fully operated acceleration pedal |  |  |

2.1.1.1. Criteria emissions (if applicable)

2.1.1.1.1. Criteria emissions of vehicles with at least one combustion engine, of NOVC-HEVs and of OVC-HEVs in case of a charge-sustaining Type 1 test

For each driver selectable mode tested the points below shall be repeated (predominant mode or best case mode and worst case mode, if applicable)

Test 1a – Results after 4 Phase cycle

| Pollutants | CO | THC (a) | NMHC (a) | NOx | THC+NOx (b) | Particulate Matter | Particle Number |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (#.1011/km) |
| Measured values |  |  |  |  |  |  |  |
| Regeneration factors (Ki)(2) Additive |  |  |  |  |  |  |  |
| Regeneration factors (Ki)(2) Multiplicative |  |  |  |  |  |  |  |
| Deterioration factors (DF) additive |  |  |  |  |  |  |  |
| Deterioration factors (DF) multiplicative |  |  |  |  |  |  |  |
| Final values |  |  |  |  |  |  |  |
| Limit values |  |  |  |  |  |  |  |

Test 1b – Results after 3 Phase cycle

| Pollutants | CO | THC | NMHC | NOx | Particulate Matter | Particle Number |
| --- | --- | --- | --- | --- | --- | --- |
|  | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (#.1011/km) |
| Measured values |  |  |  |  |  |  |
| Regeneration factors (Ki)(2) Additive |  |  |  |  |  |  |
| Regeneration factors (Ki)(2) Multiplicative |  |  |  |  |  |  |
| Deterioration factors (DF) additive |  |  |  |  |  |  |
| Deterioration factors (DF) multiplicative |  |  |  |  |  |  |
| Final values |  |  |  |  |  |  |
| Limit values |  | - |  |  |  |  |

| (2) See Ki family report(s) (if applicable) | : |  |
| --- | --- | --- |
| Type 1 performed for Ki determination (if applicable) | : |  |
| Regeneration family’s identifier | : |  |

Tests 2a and 2b if applicable: for CO2 reason (dCO21) / for pollutants reason (90% of the limits) / for both

Record test results in accordance with the table of Tests 1a and 1b, as applicable

Tests 3a and 3b if applicable: for CO2 reason (dCO22)

Record test results in accordance with the table of Tests 1a and 1b, as applicable

2.1.1.1.2.  Criteria emissions of OVC-HEVs in case of a charge-depleting Type 1 test

Test 1a – Results after 4 Phase cycle

Criteria emission limits have to be fulfilled and the following point has to be repeated for each driven test cycle.

| Pollutants | CO | THC (a) | NMHC (a) | NOx | THC+NOx (b) | Particulate Matter | Particle Number |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (#.1011/km) |
| Measured single cycle values |  |  |  |  |  |  |  |
| Limit single cycle values |  |  |  |  |  |  |  |

Test 1b – Results after 3 Phase cycle

Criteria emission limits have to be fulfilled and the following point has to be repeated for each driven test cycle.

| Pollutants | CO | THC | NMHC | NOx | Particulate Matter | Particle Number |
| --- | --- | --- | --- | --- | --- | --- |
|  | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (#.1011/km) |
| Measured single cycle values |  |  |  |  |  |  |
| Limit single cycle values |  |  |  |  |  |  |

Tests 2a and 2b (if applicable): for CO2 reason (dCO21) / for pollutants reason (90% of the limits) / for both

Record test results in accordance with the table of Tests 1a and 1b, as applicable

Tests 3a and 3b (if applicable): for CO2 reason (dCO22)

Record test results in accordance with the table of Tests 1a and 1b, as applicable

2.1.1.1.3. UF-weighted criteria emissions of OVC-HEVs

| Pollutants | CO | THC (a) | NMHC (a) | NOx | THC+NOx (b) | Particulate Matter | Particle Number |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (#.1011/km) |
| Calculated values |  |  |  |  |  |  |  |

2.1.1.2. CO2 emission (if applicable)

2.1.1.2.1. CO2 emission of vehicles with at least one combustion engine, of NOVC-HEV and of OVC-HEV in the case of a charge-sustaining Type 1 test

For each driver selectable mode tested the points below have to be repeated (predominant mode or best case mode and worst case mode, if applicable)

Test 1a – Results after 4 Phase cycle

| CO2 emission | Low | Medium | High | Extra High | Combined |
| --- | --- | --- | --- | --- | --- |
| Measured value MCO2,p,1 / MCO2,c,2 |  |  |  |  |  |
| Speed and distance corrected value MCO2,p,2b / MCO2,c,2b |  |  |  |  |  |
| RCB correction coefficient: (5) |  |  |  |  |  |
| MCO2,p,3 / MCO2,c,3 |  |  |  |  |  |
| Regeneration factors (Ki) Additive |  |  |  |  |  |
| Regeneration factors (Ki) Multiplicative |  |  |  |  |  |
| MCO2,c,4 | - |  |  |  |  |
| AFKi= MCO2,c,3 / MCO2,c,4 | - |  |  |  |  |
| MCO2,p,4 / MCO2,c,4 |  |  |  |  | - |
| ATCT correction (FCF) (4) |  |  |  |  |  |
| Temporary values MCO2,p,5 / MCO2,c,5 |  |  |  |  |  |
| Declared value | - | - | - | - |  |
| dCO21 * declared value | - | - | - | - |  |

| (4) FCF: family correction factor for correcting for representative regional temperature conditions (ATCT) |  |  |
| --- | --- | --- |
| See ATCT family report(s) (if applicable) | : |  |
| ATCT family’s identifier | : |  |
| (5) correction as referred to in Appendix 2 to Annex B6 of UN Regulation No. 154 for pure ICE vehicles, and Appendix 2 to Annex B8 of UN Regulation No 154 for HEVs (KCO2) |  |  |

Tests 2a and 2b (if applicable)

Record test results in accordance with the table of Tests 1a and 1b, as applicable

Tests 3a and 3b (if applicable)

Record test results in accordance with the table of Tests 1a and 1b, as applicable

Conclusion after 4 phase cycle

| CO2 emission (g/km) | Low | Medium | High | Extra High | Combined |
| --- | --- | --- | --- | --- | --- |
| Averaging MCO2,p,6/ MCO2,c,6 |  |  |  |  |  |
| Alignment MCO2,p,7 / MCO2,c,7 |  |  |  |  |  |
| Final values MCO2,p,H / MCO2,c,H |  |  |  |  |  |

Information for Conformity of Production for OVC-HEV

|  | Combined |
| --- | --- |
| CO2 emission (g/km) MCO2,CS,COP |  |
| AFCO2,CS |  |

2.1.1.2.2. CO2 emission of OVC-HEVs in case of a charge-depleting Type 1 test

Test 1

| CO2 emission (g/km) | Combined |
| --- | --- |
| Calculated value MCO2,CD |  |
| Declared value |  |
| dCO21 |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion

| CO2 emission (g/km) | Combined |
| --- | --- |
| Averaging MCO2,CD |  |
| Final value MCO2,CD |  |

2.1.1.2.3. UF-weighted CO2 emission of OVC-HEVs

| CO2 emission (g/km) | Combined |
| --- | --- |
| Calculated value MCO2,weighted |  |

2.1.1.3. Fuel consumption (if applicable)

2.1.1.3.1. Fuel consumption of vehicles with only a combustion engine, of NOVC-HEVs and of OVC-HEVs in case of a charge-sustaining Type 1 test

For each driver selectable mode tested the points below has to be repeated (predominant mode or best case mode and worst case, mode if applicable)

| Fuel consumption (l/100 km) or fuel efficiency (km/l) (as applicable) | Low | Medium | High | Extra High | Combined |
| --- | --- | --- | --- | --- | --- |
| Final values FCp,H / FCc,H (6), FEp, FEc |  |  |  |  |  |

(6)  Calculated from aligned CO2 values

On-board Fuel and/or Energy Consumption Monitoring for vehicles referred to in paragraph 5.11. of this Regulation

Data accessibility

The parameters listed in paragraph 3. of Appendix 5 to this Regulation are accessible: yes/not applicable

Accuracy (if applicable)

| Fuel_ConsumedWLTP (litres) (8) | Vehicle HIGH - Test 1 | x.xxx |
| --- | --- | --- |
|  | Vehicle HIGH - Test 2 (if applicable) | x.xxx |
|  | Vehicle HIGH - Test 3 (if applicable) | x.xxx |
|  | Vehicle LOW - Test 1 (if applicable) | x.xxx |
|  | Vehicle LOW Test 2 (if applicable) | x.xxx |
|  | Vehicle LOW - Test 3 (if applicable) | x.xxx |
|  | Total | x.xxx |
| Fuel_ConsumedOBFCM (litres) (8) | Vehicle HIGH - Test 1 | x.xxx(9) |
|  | Vehicle HIGH - Test 2 (if applicable) | x.xxx(9) |
|  | Vehicle HIGH - Test 3 (if applicable) | x.xxx(9) |
|  | Vehicle LOW - Test 1 (if applicable) | x.xxx(9) |
|  | Vehicle LOW Test 2 (if applicable) | x.xxx(9) |
|  | Vehicle LOW - Test 3 (if applicable) | x.xxx(9) |
|  | Total | x.xxx(9) |
| Accuracy (8) |  | x.xxx |

(8)  in accordance with Appendix 5 to this Regulation

(9)  In the case that the OBFCM signal can only be read-out to 2 decimal places, the third 
decimal place shall be introduced as a zero

2.1.1.3.2. Fuel consumption of OVC-HEVs and OVC-FCHVs (as applicable) in case of a charge-depleting Type 1 test

Test 1

| Fuel consumption (l/100 km or kg/100 km) or fuel efficiency (km/l) (as applicable) | Combined |
| --- | --- |
| Calculated value FCCD, FECD |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion

| Fuel consumption (l/100km or kg/100 km) or fuel efficiency (km/l ) (as applicable) | Combined |
| --- | --- |
| Averaging FCCD, FECD |  |
| Final value FCCD, FECD |  |

2.1.1.3.3. UF-Weighted Fuel consumption of OVC-HEVs and OVC-FCHVs (as applicable)

| Fuel consumption (l/100 km or kg/100 km) | Combined |
| --- | --- |
| Calculated value FCweighted |  |

2.1.1.3.4. Fuel consumption of vehicles of NOVC-FCHVs and OVC-FCHVs (as applicable) in case of a charge-sustaining Type 1 test

For each driver selectable mode tested the points below has to be repeated (predominant mode or best case mode and worst case, mode if applicable)

Test1

| Fuel consumption (kg/100 km) or fuel efficiency (km/kg) (as applicable) |  | Low |  | Medium |  | High |  | Extra High |  |  | Combined |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured values FCCS, |  |  |  |  |  |  |  |  |  |  |  |  |
| RCB correction coefficient |  |  |  |  |  |  |  |  |  |  |  |  |
| Calculated value FCCS, FECS |  |  |  |  |  |
| Declared value | - | - | - | - |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion

| Fuel consumption (kg/100 km) or fuel efficiency (km/kg) (as applicable) | Low | Medium | High | Extra High | Combined |
| --- | --- | --- | --- | --- | --- |
| Averaging FCCS, FECS | - | - | - | - |  |
| Alignment FECS |  |  |  | - | - |
| Final value FCCS, FECS |  |  |  | - |  |

2.1.1.4. Ranges (if applicable)

2.1.1.4.1. Ranges for OVC-HEVs and OVC-FCHVs (as applicable)

2.1.1.4.1.1. All electric range

| Test 1 AER (km) | City | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- | --- |
| Measured/Calculated values AER |  |  |  |
| Declared value | - |  |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion

| AER (km) | City | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- | --- |
| Averaging AER (if applicable) |  |  |  |
| Final values AER |  |  |  |

2.1.1.4.1.2. Equivalent All electric Range

Results after 4 Phase cycle

| EAER (km) | Low | Medium | High | Extra High | City | Combined |
| --- | --- | --- | --- | --- | --- | --- |
| Calculated value EAER |  |  |  |  |  |  |
| Declared value | - | - | - | - | - |  |
| Final values EAER |  |  |  |  |  |  |

Results after 3 Phase cycle

| EAER (km) | Low | Medium | High | Combined |
| --- | --- | --- | --- | --- |
| Calculated value EAER |  |  |  |  |
| Declared value | - | - | - |  |
| Final values EAER |  |  |  |  |

2.1.1.4.1.3. Actual charge-depleting range

| RCDA (km) | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- |
| Final value RCDA |  |  |

2.1.1.4.1.4. Charge-Depleting Cycle Range

Test 1

| RCDC (km) | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- |
| Final value RCDC |  |  |
| Index Number of the transition cycle |  |  |
| REEC of confirmation-cycle (%) |  |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

2.1.1.4.2. Ranges for PEVs - Pure electric range (if applicable)

Test 1

| PER (km) | Low | Medium | High | Extra High | City | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated values PER |  |  |  |  |  |  |  |
| Declared value | - | - | - | - | - |  |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion

| PER (km) | Low | Medium | High | Extra High | City | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Averaging PER |  |  |  |  |  |  |  |
| Final values PER |  |  |  |  |  |  |  |

2.1.1.4.3.	Ranges for OVC-FCHVs and NOVC-FCHVs - Driving Range of hydrogen (if applicable)

Test 1

| PLL, Lower limit pressure (MPa) | － |
| --- | --- |
| Measured value PLL |  |
| Declared value PLL |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion

| PLL (MPa) | － |
| --- | --- |
| Final value PLL |  |

| Driving Range of hydrogen (km) | Combined |
| --- | --- |
| Calculated value |  |

2.1.1.5. Electric energy consumption (if applicable)

2.1.1.5.1. Electric energy consumption of OVC-HEVs and OVC-FCHVs (as applicable)

2.1.1.5.1.1. Recharged electric energy (EAC)

|  | 4 phases cycle | 3 phases cycle |
| --- | --- | --- |
| EAC (Wh) |  |  |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2.(p) or paragraph 3.3.(j) of Appendix 5 to this Regulation is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (10) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (10) | x.xxx(11) |
| Accuracy vehicle energy charged (10) | x.xxx(11) |

(10)	in accordance with Appendix 5 to this Regulation

(11)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

2.1.1.5.1.2.	Electric energy consumption (EC)

Test 1a – Results after 4 phase cycle

| EC (Wh/km) | Low | Medium | High | Extra High | City | Combined |
| --- | --- | --- | --- | --- | --- | --- |
| Final values EC |  |  |  |  |  |  |

Test 1b – Results after 3 phase cycle

| EC (Wh/km) | Low | Medium | High | Combined |
| --- | --- | --- | --- | --- |
| Final values EC |  |  |  |  |

2.1.1.5.1.3.	UF-weighted charge-depleting electric energy consumption

Test 1

| ECAC,CD (Wh/km) | Combined |
| --- | --- |
| Calculated value ECAC,CD |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion (if applicable)

| ECAC,CD (Wh/km) | Combined |
| --- | --- |
| Averaging ECAC,CD |  |
| Final value |  |

2.1.1.5.1.4.	UF-weighted electric energy consumption

Test 1

| ECAC,weighted (Wh) | Combined |
| --- | --- |
| Calculated value ECAC,weighted |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion (if applicable)

| ECAC,weighted (Wh/km) | Combined |
| --- | --- |
| Averaging ECAC,weighted |  |
| Final value |  |

2.1.1.5.1.5. Information for COP

|  | Combined (4 phase cycle) | Combined (3 phase cycle) |
| --- | --- | --- |
| Electric energy consumption (Wh/km) ECDC,CD,COP or ECAC,CD,COP (as applicable) |  |  |
| AFEC,AC,CD (as applicable) |  |  |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2.(p) or paragraph 3.3.(j) of Appendix 5 to this Regulation is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (12) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (13) | x.xxx(14) |
| Accuracy vehicle energy charged (13) | x.xxx(14) |

(12)	in accordance with paragraph 3.4. of Appendix 2 to this Regulation

(13)	in accordance with Appendix 5 to this Regulation

(14)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

2.1.1.5.2. Electric energy consumption of PEVs (if applicable)

Test 1

| EAC(Wh) |  |
| --- | --- |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2.(p) or paragraph 3.3.(j) of Appendix 5 to this Regulation is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (15) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (15) | x.xxx(16) |
| Accuracy vehicle energy charged (15) | x.xxx(16) |

(15)	in accordance with Appendix 5 to this Regulation

(16)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

Test 1a – Results after 4 phase cycle

| EC (Wh/km) | Low | Medium | High | Extra High | City | Combined |
| --- | --- | --- | --- | --- | --- | --- |
| Calculated values EC |  |  |  |  |  |  |
| Declared value |  |  |  |  | - |  |

Test 1b – Results after 3 phase cycle

| EC (Wh/km) | Low | Medium | High | Combined |
| --- | --- | --- | --- | --- |
| Calculated values EC |  |  |  |  |
| Declared value |  |  |  |  |

Tests 2a and 2b  (if applicable)

Record test results in accordance with the table of Test 1

Tests 3a and 3b –

Record test results in accordance with the table of Test 1

Conclusion after 4 phase cycle

| EC (Wh/km) | Low 4 phase cycle | Medium 4 phase cycle | High 4 phase cycle | Extra High 4 phase cycle | City 4 phase cycle | Combined 4 phase cycle |
| --- | --- | --- | --- | --- | --- | --- |
| Averaging EC |  |  |  |  |  |  |
| Final values EC |  |  |  |  |  |  |

Conclusion after 3 phase cycle

| EC (Wh/km) | Low 3 phase cycle | Medium 3 phase cycle | High 3 phase cycle | Combined 3 phase cycle |
| --- | --- | --- | --- | --- |
| Averaging EC |  |  |  |  |
| Final values EC |  |  |  |  |

Information for COP - Results after 4 Phase cycle (For Level 1A - if applicable)

|  | Combined 4 phase cycle |
| --- | --- |
| Electric Energy Consumption (Wh/km) ECDC,COP |  |
| AFEC |  |

Information for COP - Results after 3 Phase cycle (For Level 1B - if applicable)

|  | Combined 3 phase cycle |
| --- | --- |
| Electric Energy Consumption (Wh/km) ECDC,COP |  |
| AFEC |  |

For Level 1A and Level 2 only

The parameter listed in paragraph 3.2.(p) or paragraph 3.3.(j) of Appendix 5 to this Regulation is accessible: yes/not applicable

Accuracy (if applicable)

| Vehicle energy chargedREESS_charging (kWh) (17) | x.xxx |
| --- | --- |
| Vehicle energy chargedOBFCM (kWh) (18) | x.xxx(19) |
| Accuracy vehicle energy charged (18) | x.xxx(19) |

(17)	in accordance with paragraph 3.4. of Appendix 2 to this Regulation

(18)	in accordance with Appendix 5 to this Regulation

(19)	in the case that the OBFCM signal can only be read-out to 2 decimal places, the third decimal place shall be introduced as a zero

For Level 1B and Level 2 only

The parameters listed in paragraph 3 of Appendix 5 to this Regulation and listed in Appendix 1 of Annex C1 are accessible: yes/not applicable

2.1.2. 	Vehicle low (if applicable)

Repeat paragraph 2.1.1.

2.1.3. 	Vehicle M (if applicable)

Repeat paragraph 2.1.1.

2.1.4. 	Final criteria emissions values (if applicable)

| Pollutants | CO | THC (a) | NMHC (a) | NOx | THC+NOx (b) | PM | PN |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (mg/km) | (#.1011/km) |
| Highest values(3) |  |  |  |  |  |  |  |

(3)  for each pollutant within all test results of VH, VL (if applicable) and VM (if applicable)

2.4. 	Type 4 (a) test

| Family’s identifier | : |  |
| --- | --- | --- |
| See report(s) | : |  |

2.5. 	Type 5 test

| Family’s identifier | : |  |
| --- | --- | --- |
| See durability family report (s) (if applicable) | : |  |
| Type 1 cycle for criteria emissions testing (if applicable) | : |  |
| Durability family identifier |  |  |

2.7.	Declared pure electric range ratio at low temperature

| Low temperature family’s identifier | : |  |
| --- | --- | --- |
| KPER,WLTC,LT,dec | : |  |

*Duplicate the table in case more than one low temperature is considered.

2.8. 	On Board Diagnostic System

| Family’s identifier | : |  |
| --- | --- | --- |
| See family report(s) (if applicable) | : |  |

2.11. 	Temperature information related to vehicle high (VH)

Unless an ambient temperature correction test is required by the granting type-approval authority for the purpose of type-approval this information shall not be required.

| Worst case approach with regards to vehicle insulation | : | yes/no (7) |
| --- | --- | --- |
| Worst case approach vehicle cool down | : | yes/no (7) |
| ATCT family composed of a single Interpolation family | : | yes/no (7) |
| Engine coolant temperature at the end of soaking time (°C) | : |  |
| Average soak area temperature over the 3 last hours (°C) | : |  |
| Difference between engine coolant end temperature and average soak area temperature of the last 3 hours ∆T_ATCT (°C) | : |  |
| The minimum soaking time tsoak_ATCT (s) | : |  |
| Location of temperature sensor | : |  |
| Measured engine temperature | : | oil/coolant |

(7)  if “yes” then the six last lines are not applicable

2.12. 	Exhaust after-treatment system using reagent

| Family’s identifier | : |  |
| --- | --- | --- |
| See family report(s) (if applicable) | : |  |

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.