UNECE R10.08 - 
immunity related function and 
annex 6 test cycle proposal

Agreed

To be reviewed for next meeting

WRITER

From: R. PERROT (S&E/EXP/ESA)

Phone: +33 169 801 743 / +33 680 467 440

Email: remy.perrot@utac.com

OBJECT: text proposal related to doc IWG-EMC-47-18e (FR) Annex 6 and immunity related function updates.

Next to proposal IWG-EMC-47-18e, hereafter the associated proposal of modification of R10.08

Add a new definition

2.X : Speed Limitation Device (SLD) means a device whose primary function is to control the fuel feed to the engine in order to limit the vehicle speed to the specific value

Immunity related unction (clause 2.12)

2.12. "Immunity related functions" are the following functions; this list is not exhaustive and shall be adapted to the technical evolution of vehicle/technology:

(a) Functions related to the direct control of the vehicle:

(i) By degradation or change in: e.g. engine, gear, brake, suspension, active steering, speed limitation devices;

(ii) By affecting driver’s position: e.g. seat or steering wheel positioning;

(iii) By affecting driver's visibility: e.g. dipped beam, windscreen wiper, indirect vision systems, blind spot systems, rear view camera systems, front and lateral view camera systems, moving off information systems.

(b) Functions related to driver, passenger and other road user protection:

(i) E.g. airbag and safety restraint systems, emergency calling systems;

(c) Functions which, when disturbed, cause confusion to the driver or other road users:

(i) Optical disturbances: incorrect operation of e.g. direction indicators, stop lamps, end outline marker lamps, rear position lamp, light bars for emergency system, wrong information from warning indicators or Tire Pressure Monitoring System function, lamps or displays related to functions in subparagraphs (a) or (b) which might be observed in the direct view of the driver;

(ii) Acoustical disturbances: incorrect operation of e.g. anti-theft alarm, horn.

(d) Functions related to vehicle data bus functionality:

(i) By blocking data transmission on vehicle data bus-systems, which are used to transmit data, required to ensure the correct functioning of other immunity related functions.

(e) Functions which when disturbed affect vehicle statutory data: e.g. tachograph, odometer;

(f) Functions related to charging mode when coupled to the power grid:

(i) For vehicle test: by leading to unexpected vehicle motion;

(ii) For ESA test: by leading to an incorrect charging condition (e.g. over-current, over-voltage).

Annex 6 (clause 2.1.1.2)

2.1.1.2. Basic vehicle conditions

Basic vehicle conditions

The paragraph defines minimum test conditions (as far as applicable) and failure criteria for vehicle immunity tests. Other vehicle systems, which can affect immunity related functions, shall be tested in a way to be agreed between manufacturer and Technical Service.

| "50 km/h mode" vehicle test conditions | Failure criteria |
| --- | --- |
| Vehicle speed 50 km/h (respectively 25 km/h for L1, L2 vehicles) ± 20 per cent (vehicle driving the rollers). If the vehicle is equipped with a cruise control system(1), it shall be used to maintain the required constant vehicle speed and maintained without any deactivation. | Speed variation greater than ±10 per cent of the nominal speed. In case of automatic gearbox: change of gear ratio inducing a speed variation greater than ±10 per cent of the nominal speed. no change of state of the immobilizer and/or device to prevent unauthorized use. |
| Dipped beams ON (manual mode) | Lighting OFF (front light and rear light) |
| Specific warning (e.g Rotating/flashing light, signaling bar, siren…) ON | Specific warning OFF |
| Instrument cluster operate in normal mode | Unexpected warning of telltales or messages related to immunity related functions Inconsistent variation of the odometer |
| Rear view system (internal and external) | Unexpected movement of rear-view mirror |
| Camera Monitoring Systems for indirect vision | Loss of function (full or partial image loss, blank screen, or frozen image) Loss or freezing of the display (Camera Monitoring S) Change of state of display (ON/OFF) |
| Front wiper ON (manual mode) maximum speed | Complete stop of front wiper |
| Direction indicator on driver's side ON | Frequency change (lower than 0.75 Hz or greater than 2.25 Hz). Duty cycle change (lower than 25 per cent or greater than 75 per cent). |
| Adjustable suspension in normal position | Unexpected significant variation |
| Driver's seat and steering wheel in medium position | Unexpected variation greater than 10 per cent of total range |
| Alarm unset | Unexpected activation of alarm |
| Horn OFF | Unexpected activation of horn |
| Airbag and safety restraint systems operational with inhibited passenger airbag if this function exists | Unexpected activation |
| Automatic doors closed | Unexpected opening of automatic doors Unexpected change of the window or sunroof positions |
| Adjustable endurance brake lever in normal position | Unexpected activation |
| Brake pedal not depressed | Unexpected activation of brake and unexpected activation of stop lights |
| Automatic gearbox in Driving mode | change of gear mode of the gearbox |
| Tachograph / odometer operating | Inconsistent distance travelled compared to odometer during the test Recording of data with error (check the difference between beginning and of test, shall be less than 6 km) No freeze of the travelled distance |
| Blindspot detection operating with or without object in the blindspot detection area | modification of the function state (e.g. unexpected deactivation) Unexpected Inadvertent warning indications of the function. |
| lane departure warning system enabled (no road markings required during the test) (2) | modification of the function state (e.g. unexpected deactivation). Unexpected Inadvertent warning indications of the function (Inadvertent warnings or haptic feedback, cluster telltale, …). |
| ADS shall be operational(12) | ADS does not remain in a failure safe mode or expected failure operational mode |
| (1): Due to issues caused by the EMC laboratory environment, it may not be possible to adaptive automatic cruise control of the vehicle, is this case use non adaptative cruise control. (2): Due to issues caused by the EMC laboratory environment, the lane departure warning system may not detect any lane markers. (23): ADS are turned on by the driver but some or all ADS functions may revert to a mode where system is monitoring sensors but is not actively ‘driving’ the vehicle due to plausibility issues caused by the EMC laboratory environment. |  |

| "Brake mode" vehicle test conditions | Failure criteria |
| --- | --- |
| Vehicle in a state that allows the braking system to operate normally, parking brake released, vehicle speed 0 km/h. Brake pedal depressed to activate the brake function and the stop lights without any dynamic cycle. | Stop lights inactivated during mode Brake warning light ON with loss of brake function. |
| Day running light (DRL) ON | DRL inactivated during mode |
| ADS shall be operational(1) | ADS does not remain in a failure safe mode or expected failure operational mode |
| Airbag and safety restraint systems operational with non-inhibited passenger airbag | unexpected activation |
| Speed limitation function is activated(2) | Speed limiting function is disabled |
| Detection of pedestrian and cyclists while moving Off information system enabled in default operating mode | modification of the function state (e.g. unexpected deactivation). Unexpected Inadvertent warning indications of the function. |
| System for taking control of throttle pedal in case of accelerating by error instead of breaking enabled | change in the engine speed / torque applied to wheels, or transmission shaft Unexpected Inadvertent warning indications of the function. |
| Vehicle in front drive, with the Front and Lateral View Camera System enabled. | modification of the function state (e.g. unexpected deactivation). Unexpected Inadvertent warning indications of the function. Freezing of Front and/or lateral view camera systems |
| (1): ADS are turned on by the driver but some or all ADS functions may revert to a mode where system is monitoring sensors but is not actively ‘driving’ the vehicle due to plausibility issues caused by the EMC laboratory environment. (2) only applies to speed limit devices feature installed on M2, M3, N2 or N3 vehicles |  |

| "Speed limit (1)" vehicle test conditions | Failure criteria |
| --- | --- |
| Speed limit feature activated (e.g. 30 km/h) pushing the throttle pedal to reach a speed above the limit if the feature would not be activated | Speed exceedance of the limit. |
| (1) speed limit feature installed on for cycle test condition is the one covered by UNECE R89 |  |

| "Rear view close proximity (1)" vehicle test conditions | Failure criteria |
| --- | --- |
| Vehicle close proximity rearview device, is activated with the feature enabled | modification of the function state (e.g. unexpected deactivation). Inadvertent warning indications of the function. Freezing of rear-view systems (only if no redundancy with a standard mirror). |
| (1) this test condition only applies when M or N vehicle is equipped with a device for reversing motion and motor vehicles with regard to the driver’s awareness of vulnerable road users behind vehicles covered by UNECE R158. If the device can be activated in brake test condition then it can be combined with brake mode, otherwise the test is performed in rear gear. |  |

Note to the UNECE Secretariat:

The author and the speaker of this presentation confirm that they have authorization to use all content including photos and visual elements.

The material is either copyright-free or the author/speaker hold the necessary copyright or permission.

The UNECE will remove any material from its events and supporting websites if there is unlawful use of copyrighted material.

The author/speaker takes responsibility for any infringement on copyright and holds the UNECE harmless to this effect.