|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/37 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 19 September 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-eighth session

Geneva, 1–4 December 2025

Item 12 of the provisional agenda

UN Regulation No. 100 (Electric power trained vehicles)

Proposal for supplement 1 to the 05 series of amendments to UN Regulation No. 100 (Electric power trained vehicles)

Submitted by the expert from France *

The text reproduced below was prepared by the experts from France to correct inconsistencies following the amendment of regulation 100 by vehicles of category O. It is based on informal document GRSP-77-42. The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Paragraph 5.1.4.4., amend to read:

“5.1.4.4.	If an isolation resistance monitoring system is provided, and the isolation resistance less than the requirements given in paragraph 5.1.3. is detected, a warning shall be indicated to the driver. The function of the on-board isolation resistance monitoring system shall be confirmed as described in Annex 6.  In case of vehicles of category O, the trailer shall provide an optical and/or audible warning to the driver of the towing vehicle.  In addition, in case of vehicles of category O3 and O4, the trailer may provide to the towing vehicle a signal to address an optical warning according to this paragraph and/or an audible warning (e.g. transmission via CAN-Bus according to ISO 11992-2).”

Paragraph 5.2.3., amend the last paragraph to read:

“	In addition, in case of vehicles of category O3 and O4, the trailer may provide to the towing vehicle a signal to address an optical warning according to this paragraph and/or an audible warning (e.g. transmission via CAN-Bus according to ISO 11992-2) in the event specified in paragraphs 6.13. to 6.15.

The requirements of this paragraph related to paragraph 6.15 do not apply to REESS that are solely installed on vehicles of category O [that are not designed to carry occupants at any time] nor to batteries on vehicles of category O [that are not designed to carry occupants at any time,] used for recuperation only.”

Paragraph 5.3.3., amend the third paragraph to read:

“	In case of vehicles of category O a trailer parking brake or a service brake shall be automatically activated as long as the trailer connector is physically connected to the trailer inlet.”

Paragraph 5.4.1., amend to read:

“5.4.1.	This test shall be carried out on all vehicles equipped with open type traction batteries. If the REESS has been approved under Part II of this Regulation and installed in accordance with paragraph 7.1 5.2.1.1., this test can be omitted for the approval of the vehicle.”

Paragraph 6.4.1.3., amend the fourth paragraph of (c2) to read :

“	For a high voltage REESS the isolation resistance of the Tested-Device shall ensure at least 100 Ω/Volt for the whole REESS measured after the test in accordance with Annex 5A to this Regulation after the vehicle based test (paragraph 6.4.1.1.) or in accordance with Annex 5B to this Regulation after the component based test (paragraph 6.4.1.2.) to this Regulation, or the protection degree IPXXB shall be fulfilled for the Tested-Device.”

Paragraph 6.15., amend to read:

“6.15.	Thermal propagation.

For a REESS containing flammable electrolyte or alkali metal anodes, the vehicle occupants shall not be exposed to any hazardous environment caused by thermal propagation which is triggered by an internal short circuit leading to a single cell thermal runaway. To ensure this, the requirements of paragraphs 6.15.1. and 6.15.2. shall be satisfied in accordance with the verification procedure described in paragraph 6.15.3.4..

The requirements of this paragraph do not apply to REESS that are solely installed on vehicles of category O [that are not designed to carry occupants at any time] nor to batteries on vehicles of category O [that are not designed to carry occupants at any time,] used for recuperation only.”

II.	Justification

1.	During the 194th session of WP.29 (November 2024), the scope of UN Regulation No. 100 was extended to vehicles of category O.

2.	Paragraph 5.2.3. on warning in the event of failure in REESS has been amended to add requirements for category O vehicles, with the following justification: “it is recommended that the trailer is able to provide a signal to the towing vehicle and/or the trailer will give a direct warning” (informal document GRSP-75-02-Rev.1).

3.	Some specific indications have been added for vehicles of category O3 and O4: “In case of vehicles of category O3 and O4, the trailer may provide to the towing vehicle a signal to address an optical warning according to this paragraph and/or an audible warning (e.g. transmission via CAN-Bus according to ISO 11992-2) in the event specified in paragraphs 6.13. to 6.15.”.

4.	Those specific indications are non-mandatory and only apply in addition to the following prescriptions for vehicles of category O: “In case of vehicles of category O, the trailer shall provide an optical and/or audible warning to the driver of the towing vehicle in the event specified in paragraphs 6.13. to 6.15.”.

5.	The modification presented in this informal document clarifies the prescriptions for vehicles of category O and avoid any misunderstanding.

6.	The main purpose of paragraph 6.15. requirements is to ensure occupant safety in a vehicle. Most vehicles of category O are not designed to carry occupants. However, some of them can carry occupant, and should be then subject to paragraph 6.15. requirements. Paragraphs 6.15. and 5.2.3. are amended accordingly.

7.	Paragraph 5.2.3. on warning in the event of failure in REESS is amended to add clarifications concerning category O vehicles.

8.	Paragraph 5.1.4.4. is amended to include provisions related to vehicles of category O. The warning requirements of this paragraph should be aligned with the requirements of paragraph 5.2.3.

9.	According to paragraph 5.3.3., in case of vehicles of category O, a trailer brake shall be automatically activated as long as the trailer connector is physically connected to the trailer inlet, to prevent accidental or unintended vehicle movement. This requirement should allow different braking system (parking brake or service brake).

10.	Paragraph 5.2.1.1. has been replaced by paragraph 7.1. in the 05 series of amendments.

11.	Paragraph 6.4.1.3. has been amended for clarification purposes.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.