Modifications to ECE/TRANS/WP.29/GRSP/2025/24*

I.	Proposal

Insert a new paragraph 0., to read:

“0.	Introduction

0.1.	For supplement 2 to the 01 series of amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.2. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of category X, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.”

Paragraph 1. (a), add a call to footnote 1 after “M1” and insert a new footnote 1,  to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 1., at the end, add a new paragraph to read:

“	This regulation does not apply to vehicles of category Y.”

Paragraph 2.5., amend to read:

“2.5.	"A front passenger seat" means any seat where the "foremost H-point" of the seat in question is in or in front of the vertical transverse plane through the driver's R-point. For vehicles of category X, the "R" point of the most forward passenger seat shall be taken instead of the R-point of the driver's seat.”

Paragraph 2.15. (b), amend to read:

“	(b)	An anti-rotation device for an ISOFIX semi-universal child restraint system consists of either a top tether, the vehicle dashboard or, a support leg or any other means intended to limit the rotation of the restraint during a frontal impact;”

Paragraph 4.4.1., former footnote 1, renumber as footnote 2 and modify the reference to ECE/TRANS/WP.29/78/Rev.8.

Paragraph 5.3.1., amend the third paragraph to read:

“	The type and number of ISOFIX fixtures, defined in UN Regulations Nos. 16 or 173, which can be installed on each ISOFIX position are defined in UN Regulations Nos. 16 or 173.”

II.	Justification

See paragraph 0. in the proposal.

________________

*	Proposal for supplement 2 to the 01 series of amendments to UN Regulation No. 145 (ISOFIX anchorage systems, ISOFIX top tether anchorages and i-Size seating positions)