Proposal for amendments to informal document GRBP-83-32 to ensure robust tyre abrasion measurements regardless of the test method used

The text reproduced below was prepared by the experts from the European Commission to introduce the procedure for demonstrating the equivalence of test facilities (Vehicle and Indoor Drum) and determining the Transfer Function for Indoor Drum facilities. The modifications to the text of the draft Regulation (GRBP-83-32) are marked in bold for new or strikethrough for deleted characters.

- Proposal

Paragraph 6.1., amend to read:

"6.1. 	The abrasion level of the tyre, determined in accordance with the test method specified in Annex 3, shall be measured using a test facility (circuit or indoor drum) that has demonstrated equivalence according to the procedure described in Annex 4."

Insert two new paragraphs 12.8. and 12.9, to read:

["12.8.	Those test facilities that participated in the inter-facility campaign used to develop this regulation and provided the data for the definition of the abrasion limits and that are listed in the UN Document "XXX" are deemed equivalent facilities for the purpose of the equivalence procedure described in Annex 4, for C1 tyres.

12.9.	Until 1 July 2028, results of C1 tyre tests performed not earlier than 9 February 2024 and until the entry into force of this Regulation, pursuant to the test methods defined in document ECE/TRANS/GRBP/2024/10, as amended by GRBP-79-12 and para. 29 of ECE/TRANS/WP.29/GRBP/77, may be used for the purposes of both the inter-facility equivalence exercise defined in Annex 4 of this Regulation and of tyre type approval when measured by the equivalent facilities indicated in paragraph 12.8."]

Annex 3, insert a new paragraph 1.4., to read:

"1.4. 	Prior to conducting any testing for Type Approval, the test facility (whether indoor drum or circuit) shall have successfully completed the inter-facility equivalence assessment specified in Annex 4 and established its status as an equivalent circuit or equivalent drum."

Annex 3, amend paragraph 2.8., to read:

"6.1. 	Processing of Measurement Results

The calculation method for the abrasion index and the abrasion level shall follow the equations:

MlT = MTb – MTa

MlR = MRb - MRa

Where:

MlT	is the mass loss of the candidate tyre, in grams

MlR	is the mass loss of the reference tyre, in grams

MTb	is the mass of the candidate tyre before test cycle, in grams

MTa	is the mass of the candidate tyre after test cycle, in grams

MRb	is the mass of the reference tyre before test cycle, in grams

MRa	is the mass of the reference tyre after test cycle, in grams

The abrasion index (AICT) shall be calculated according to the following equation:

AICT=ALCTALRT-Amargin

AICT=aALCTALRT-1+1-Amargin

Where:

a 	is the coefficient of the transfer function for the drum as defined in Annex 4, paragraph 2.17,

ALCT 	is the abrasion level (mg/(km∙t)) of the candidate tyre,

ALCT = 1000 (mg/kg) × MlT (g)/DT(km)/Fz,T(t)

ALRT 	is the abrasion level (mg/(km∙t)) of the reference tyre,

ALRT = 1000 (mg/kg) × MlR (g)/DR(km)/Fz,R(t)

DT	is the testing mileage of candidate tyre (km)

DR	is the testing mileage of the reference tyre (km)

Fz,T	is the test load of the candidate tyre (t)

Fz,R	is the test load of the reference tyre (t)

Amargin 	is the dimensionless abrasion margin, which accounts for the complexities of the abrasion level measurement methods, and is equal to 0.22."

Annex 3, Appendix 6 (Example of a test report for indoor drum test method), insert new items 1.1. to 1.4., to read:

"1. 	Test facility equivalence status

1.1. 	Has the test facility demonstrated equivalence according to Annex 4?   Yes /  No

(If ‘No’, the test results are not valid for Type Approval)

1.2. 	Reference to the equivalence report (Unique ID/Date): ......................................

1.3. 	Transfer function applied:

Generic

Specific

Not applicable (circuit facility)

If ‘generic’ or ‘specific’ transfer function is applied, indicate the value of transfer function coefficient a (value between 0.80 and 2.00): ......................................

1.4. 	Verification of test parameters (drum method):

| Parameter | Value in equivalence report (according to Annex 4) | Value in current test |  |
| --- | --- | --- | --- |
| Surface texture | (e.g. specific sandpaper grit) | .................................................. |  |
| Third-body type | (e.g. silica) | .................................................. |  |
| Powder delivery rate [g/h] |  | .................................................. |  |
| Third-body distribution system | (General description) | .................................................. |  |

1.5. 	Test parameters (circuit method):

| Parameter | Value in equivalence report (according to Annex 4) | Value in current test |  |
| --- | --- | --- | --- |
| SRTT abrasion level |  |  |  |
| SRTT abrasion level at reference temperature |  |  |  |

"

Insert a new Annex 4, to read:

Annex 4

Procedure for Demonstrating Equivalence of Vehicle Test and Indoor Drum Test Facilities for Tyre Abrasion and Determining the Transfer Function for Indoor Drum Test Facilities

1.	Scope and purpose

1.1.	This Annex sets out a procedure to demonstrate that the results of a tyre abrasion test carried out at a test facility are equivalent to those obtained with other facilities. The aim is to ensure that test facilities yield equivalent abrasion performance results, so that approvals based on the results of any test facility or method remain comparable and mutually recognised.

1.2.	The procedure provides a structured approach for selecting test tyres and test facilities, establishing a baseline and evaluating test facilities. It applies to both indoor drum test facilities and vehicle test facilities.

1.3 	Furthermore, this procedure establishes the methodology for determining a transfer function, which mathematically relates the results obtained from the indoor drum test method to those of the vehicle test method. The procedure allows for the establishment of either a generic transfer function applicable to any indoor drum test facility or a specific transfer function applicable to a single indoor drum test facility.

2. Definitions

For the purposes of this Annex, the following definitions apply.

2.1.	"Test facility" means a testing centre, carrying out either vehicle tests at a circuit or indoor drum tests.

2.2.	"Circuit" means the specific test route on public roads selected for the execution of the vehicle test method in accordance with Annex 3.

2.3.	"Indoor drum test facility" means the test machine and associated equipment used for the execution of the indoor drum test method in accordance with Annex 3.

2.4.	"Inter-facility equivalence exercise" means the coordinated testing campaign described in this Annex, involving the testing of a common set of exercise tyres by the facility under assessment and participating facilities, used to demonstrate equivalence and, where applicable, to determine the transfer function coefficient a.

2.5.	"Coordinating body" means the Technical Service, Type Approval Authority, the facility under assessment, an equivalent facility or another entity responsible for organising the inter-facility equivalence exercise, collecting the test data, and issuing the final equivalence report.

2.6.	"Equivalent drum" means an indoor drum test facility that proved equivalence to a Type Approval Authority in a previous inter‑facility equivalence exercise.

2.7.	"Equivalent circuit" means a circuit that proved equivalence to a Type Approval Authority in a previous inter‑facility equivalence exercise.

2.8.	"Drum under assessment" (DUA) means a drum facility undergoing evaluation in the current inter-facility equivalence exercise to prove equivalence.

2.9.	"Circuit under assessment" (CUA) means a circuit undergoing evaluation in the current inter-facility equivalence exercise to prove equivalence.

2.10.	"Facility under assessment" means the drum or circuit under assessment participating in the inter-facility equivalence exercise for the purpose of demonstrating equivalence.

2.11.	"Participating facilities" means the designated facilities, other than the facility under assessment, selected to test the exercise tyres for the purpose of establishing the baseline values according to paragraph 4.1.2.1 of this Annex.

2.12.	"Reference tyre" means the Standard Reference Test Tyre (SRTT) specified in the vehicle test method or indoor drum test method of Annex 3, used to normalize the abrasion level of the exercise tyres.

2.13.	"Exercise tyres" are the candidate tyres used for the inter-facility equivalence exercise. These tyres shall be tested by the participating facilities and the facility under assessment and fulfil the requirements of paragraph 4.1.1 of this Annex.

2.14.	"Baseline value" of an exercise tyre is the average abrasion level index (ALI) obtained from all participating facilities as defined in paragraph 2.11 of this Annex. It represents the value against which under assessment facilities are compared to demonstrate equivalence.

2.15.	"Abrasion level index" (ALI) means the abrasion performance of a tyre, expressed as the ratio of the exercise tyre’s abrasion level (ALCT) to that of a reference tyre (ALRT), without any correction or subtraction of any test‑specific abrasion margin:

ALI=ALCTALRT

2.16.	"Equivalence" means that a facility produces abrasion level indices for the exercise tyres that are statistically consistent with the baseline values fulfilling the requirements of paragraph 4.3.3 of this Annex.

2.17.	"Transfer function" means a linear relationship used to relate indoor drum test results to vehicle test results. The transfer function is expressed as:

ALIFDUA,TF=a∙ALIFDUA-1+1

where

ALIDUA 	is the abrasion level index obtained by the drum under assessment using the test method defined in Annex 3, and

ALIDUA,TF 	is the abrasion level index after application of the transfer function.

2.18.	"Generic transfer function" means a transfer function where the transfer function coefficient a is a fixed value equal to [1.X] established by a statistical analysis of multiple facilities, intended for use by any drum facility.

2.19.	"Specific transfer function" means a transfer function where the transfer function coefficient a is determined individually for a specific indoor drum test facility through a dedicated inter-facility equivalence exercise. The transfer function coefficient a may take a value between [0.80 and 2] and is valid only for that specific facility and its operational conditions.

ALIFUA,TF= ALIFUA

3.	Organisation of an inter-facility equivalence exercise

3.1.	The inter-facility equivalence exercise shall be organised by a coordinating body as defined in paragraph 2.5 of this Annex.

3.2.	Upon completion of the tests, each participating facility shall submit the data resulting from the application of the procedure described in paragraph 4 of this Annex to the coordinating body. The abrasion levels of the reference tyres shall also be reported; for circuit facilities, the abrasion level normalised to the relevant reference temperature shall be included.

3.3.	The coordinating body shall compile the results, calculate the baseline values, determine the specific transfer function for the drum under assessment, if applicable, and issue an equivalence report for the facility under assessment.

3.4 	The equivalence report shall be submitted to the Type Approval Authority.

4.	Equivalence procedure outline

The equivalence procedure consists of three main steps.

4.1.	Step 1: Selection of exercise tyres and facilities

4.1.1.	Exercise tyres’ selection (Step 1a)

4.1.1.1.	At least [six] tyres shall be selected so that

(i)	they cover an ALI range of at least [0.50] units at at least one facility, with an approximately even distribution across the range and

(ii)	they are a mix of normal tyres and tyres for use in severe snow conditions with a ratio between 1:2 and 2:1, inclusive.

4.1.1.2.	If a facility under assessment is determining a specific transfer function the ALI range under point (i) above shall be at least [0.80].

4.1.1.3.	If a facility under assessment is designed for testing only normal tyres or tyres for use in severe snow conditions, the equivalence of the test facility shall be restricted to the corresponding tyre category and the requirement under paragraph 4.1.1.1 (ii) of this Annex shall not apply.

4.1.2.	Selection of participating facilities (Step 1b)

4.1.2.1.	The participating facilities shall be selected according to the type of assessment being performed:

(a)	Assessment of a circuit or indoor drum test facility with a generic transfer function: In addition to the facility under assessment, the participating facilities shall include either:

(i)	At least [one] equivalent facility; or

(ii)	At least [three] circuits (regardless of their equivalence status), with no participating indoor drums.

(b)	Assessment of an indoor drum test facility using a specific transfer function: If the assessed drum intends to determine a specific transfer function coefficient a, the participating facilities shall include a

t least [two] equivalent facilities, with the number of circuits being at least as many as the number of drums;

4.2.	Step 2: Establishment of baseline values

4.2.1.	All test facilities shall carry out the tests according to Annex 3 of this Regulation. Each participating facility shall test each exercise tyre at least once.

4.2.2.	The results of each equivalent drum shall be adjusted using their transfer function defined in paragraph 2.17 of this Annex.

4.2.3.	For each exercise tyre, if more than one test result is available, the baseline value shall be the arithmetic mean of the abrasion level indices reported by the participating facilities as defined in paragraph 2.11. The calculation of the baseline requires a minimum of two data points, if at least two participating facilities are required according to paragraph 4.1.2.

4.2.4.	Any result whose absolute difference from the mean exceeds [0.25] shall be excluded and a new arithmetic mean shall be calculated with the remaining results. Such exclusions shall be applied iteratively until all remaining results lie within ±[0.25] of their mean. If less than two valid data points remain after exclusion, data from an additional facility shall be added or an additional tyre shall be tested.

4.3.	Step 3: Demonstration of equivalence of a facility

4.3.1.	The facility under assessment shall compare its measurements with the baseline values.

4.3.2.	The results of a drum facility under assessment shall be adjusted using a transfer function as defined in paragraph 2.17, before comparing to the baseline values according to paragraph 4.3.3 of this Annex.

4.3.2.1.	Assessed drums using a generic transfer function shall apply the value of the generic transfer function coefficient a..

4.3.2.2.	For assessed drums determining a specific transfer function, the value of the transfer function coefficient a shall be calculated based on the results of the inter-facility equivalence exercise.

The coefficient acalc is calculated using the least squares regression on the shifted coordinates as:

acalc=i=1n(ALIFDUA,i-1)∙ALIbaseline,i-1i=1n(ALIFDUA,i-1)2

where

ALIDUA,i	is the abrasion level index obtained for exercise tyre i by the facility under assessment using the test method defined in Annex 3,

ALIbaseline,i	is the baseline abrasion level index obtained for exercise tyre i pursuant to paragraph 4.2 of this Annex, and

n	is the number of exercise tyres

The final transfer function coefficient a used for type-approval tests shall be determined as follows:

If acalc < 0.80, then a = 0.80,

If acalc > 2.00, then a = 2.00,

Otherwise, a = acalc.

4.3.3.	The facility under assessment shall demonstrate equivalence by fulfilling the following criteria:

(i)	For each exercise tyre i, the abrasion level index obtained by the facility under assessment ALIFUA,i (without corrections for circuits ALICUA,i, but adjusted by the transfer function for drums ALIDUA,TF,i) differs from the baseline value by no more than ±[0.25]:

ΔALI= abs(ALIFUA,TF,i-ALIbaseline,i)≤0.25

(ii)	The root mean square deviation RMSE between baseline and facility under assessment for all exercise tyres shall be less than or equal to [0.12]:

RMSE=1ni=1nALIFUA,TF,i-ALIbaseline,i2≤0.12

(iii)	For the case that a specific transfer function is calculated, in addition to the previous criteria, for all exercise tyres, the correlation coefficient (R²) between the results of the facility under assessment and the baselines shall be greater than [0.80]. The correlation coefficient shall be calculated using the Ordinary Least Squares (OLS) method between the results of the facility under assessment and the baseline values, according to the following formula:

R2=1-i=1n(ALIFUA,TF,i-ALIbaseline,i)2i=1nALIbaseline,i-ALIbaseline 2

where

ALIbaseline 	is the arithmetic mean of the baseline values: ALIbaseline=1ni=1nALIbaseline,i;

i	is the exercise tyre index and

n	the total number of exercise tyres tested.

4.3.4.	Indoor drum test facilities that prove equivalence under this procedure (whether using a specific transfer function, or a generic transfer function) are recognised for the specific test parameters used during the exercise. Any relevant modification of the applied parameters shall be documented, justified and agreed by the Type Approval Authority.

4.3.5.	Reference parameters for circuits

The equivalence status of a circuit is established based on the specific test circuit route used during the equivalence exercise. Any relevant modification to the circuit shall be documented, justified and agreed by the Type Approval Authority.