|  | TBD |  |
| --- | --- | --- |
|  |  | xx February 2027 |

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be  used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

_________

Addendum 50 – Regulation No. 51

Revision 4

04 series of amendments – Date of entry into force: XX January 2028

Uniform provisions concerning the approval of motor vehicles having at least four wheels with regard to their sound emissions

This document is meant purely as documentation tool. The authentic and legal binding text is: ECE/TRANS/WP.29/20XX/XX

_________

UNITED NATIONS

Regulation No. 51

Uniform provisions concerning the approval of motor vehicles having at least four wheels with regard to their sound emissions

Contents

Page

Regulation

1.		Scope			  5

2.		Definitions			  5

3.		Application for approval			20

4.		Markings			21

5.		Approval			21

6.		Specifications			23

7.		Modification and extension of approval of a vehicle type			27

8.		Conformity of production			27

9.		Penalties for non-conformity of production			27

10.	Production definitively discontinued			28

11.	Transitional provisions			28

12.	Names and addresses of Technical Services   responsible for conducting approval tests and of
Type Approval Authorities			30

Annexes

1		Communication			32

Appendix 1 - Addendum to the communication form No …			34

Appendix 2 - Technical Information Document			36

2		Arrangements of the approval mark			40

3		Methods and instruments for measuring the sound made by motor vehicles			41

Appendix 1 – Addendum to the communication form No. 			73

Appendix 2 - Correction for the tyre rolling sound component of pass-by sound measurements …xx Appendix 3 - Correction for the tyre rolling sound component of pass-by sound measurements …xx

4		Exhaust silencing systems containing acoustically absorbing fibrous materials			110

Appendix			112

5		Compressed air noise			114

Appendix			115

6		Checks on conformity of production			116

7		Measuring method to evaluate compliance with the Additional Sound Emission Provisions			118

Appendix 1 - Statement of Compliance with the Additional Sound Emission Provisions 			128

Appendix 2 - Flowcharts 										xx

8	Indoor testing	 											xx

9 		Measurement method to evaluate compliance with the Real Driving Additional Sound Emission Provisions (RD-ASEP)

Appendix 1 - Sound Expectation Model

Appendix 2 - Parameter Table for the Sound Expectation Model

Appendix 3 - Symbols, Abbreviations and Acronyms

Appendix 4 - Formulas

Appendix 5 - Test report sheet

10 	Special provisions for the testing of vehicles equipped with an ADS

1.		Scope

This Regulation contains provisions on the sound emitted by motor vehicles and applies to vehicles of categories M and N.

The specifications in this Regulation are intended to reproduce the sound levels which are generated by vehicles during normal driving in urban traffic.

This regulation provides, as well, additional sound emission provisions for vehicles of categories M1 and N1 referring to driving conditions with extreme accelerations in an extended speed range representative for urban and suburban traffic.

2.		Definitions

For the purpose of this Regulation,

2.1.	"Approval of a vehicle" means the approval of a vehicle type with regard to sound;

2.2.	"Vehicle type" means a category of motor vehicles which do not differ in such essential respects as:

2.2.1.	For vehicles tested according to Annex 3, paragraph 3.1.2.1.:

2.2.1.1.	The shape or materials of the engine compartment and its soundproofing;

2.2.1.2.	The type of engine (positive or compression ignition, two- or four-stroke, reciprocating or rotary piston), number and capacity of cylinders, number and type of carburettors or injection system, arrangement of valves, or the type of electric motor;

2.2.1.3.	Rated maximum net power and corresponding rated engine speed(s); however if the rated maximum net power and the corresponding rated engine speed differs only due to different engine mappings, these vehicles may be regarded as from the same type;

2.2.1.4.	The silencing system.

2.2.2.	For vehicles tested according to Annex 3, paragraph 3.1.2.2.:

2.2.2.1.	The shape or materials of the engine compartment and its soundproofing;

2.2.2.2.	The type of engine (positive or compression ignition, two- or four-stroke, reciprocating or rotary piston), number and capacity of cylinders, type of injection system, arrangement of valves, rated engine speed (S), or the type of electric motor;

2.2.2.3.	Vehicles having the same type of engine and/or different overall gear ratios may be regarded as vehicles of the same type;

2.3.	However, if the differences in paragraph 2.2.2. provide for different target conditions, as described in paragraph 3.1.2.2. of Annex 3, these differences are to be considered as a change of type;

2.4.	"Mass of a vehicle in running order (mro)"  means

(a) 	In the case of a motor vehicle:

The mass of the vehicle, with its fuel tank(s) filled to at least 90 per cent of its or their capacity/ies, of the fuel and liquids, fitted with the standard equipment in accordance with the manufacturer's specifications and, when they are fitted, the mass of the bodywork, the cabin, the coupling and the spare wheel(s) as well as the tools and the mass of the driver (75 kg) except in the case of vehicles of category X or category Y;

(b) 	In the case of a trailer:

The mass of the vehicle including the fuel and liquids, fitted with the standard equipment in accordance with the manufacturer's specifications, and, when they are fitted, the mass of the bodywork, additional coupling(s), the spare wheel(s) and the tools.

2.5.	"Technically permissible maximum laden mass (M)"  means the maximum mass allocated to a vehicle on the basis of its construction features and its design performances; the technically permissible laden mass of a trailer or of a semi-trailer includes the static mass transferred to the towing vehicle when coupled;

2.6.	"Vehicle length" means a dimension which is measured according to ISO standard 612-1978, term No. 6.1. In addition to the provisions of that standard, when measuring the vehicle structural length the following devices shall not be taken into account:

(a)	Wiper and washer devices;

(b)	Front or rear marker-plates;

(c)	Customs sealing devices and their protection;

(d)	Devices for securing the tarpaulin and their protection;

(e)	Lighting equipment;

(f)	Rear view mirrors;

(g)	Rear space watching aids;

(h)	Air-intake pipes;

(i)	Length stops for demountable bodies;

(j)	Access steps;

(k)	Ram rubbers;

(l)	Lifting platforms, access ramps and similar equipment in running order, not exceeding 200 mm, provided that the loading capacity of the vehicle is not increased;

(m)	Coupling devices for motor vehicles.

2.7	"Vehicle width" means a dimension which is measured according to ISO standard 612-1978, term No. 6.2. In addition to the provisions of that standard, when measuring the vehicle structural width the following devices shall not be taken into account:

(a)	Customs sealing devices and their protection;

(b)	Devices for securing the tarpaulin and their protection;

(c)	Tyre failure tell-tale devices;

(d)	Protruding flexible parts of a spray-suppression system;

(e)	Lighting equipment.

2.8.		"Maximum net power, Pn" means the declared engine power available for propulsion expressed in kW and measured dependent on the drive train concepts pursuant to UN Regulation No. 85 or UN Regulation No. 177..

Applicable power sources are those, which provide drive power for forward motion to the vehicle.

2.8.1.

For vehicles with combustion engine(s) only (ICE)

The maximum engine power is the net power Pn of the combustion engine(s) measured at full engine load pursuant to UN Regulation No.85 paragraph 5.2.

2.8.2.	For battery electric vehicles (BEV) or fuel cell electric vehicle (FCEV) that have only one propulsion energy converter

The net power Pn of the electric motor of the electric drive train is determined pursuant to UN Regulation No.85 paragraph 5.3.

2.8.3.	For hybrid electric vehicles (HEV), or pure electric vehicles that have more than one propulsion energy converter

The maximum engine power is the "vehicle system power rating" according to the arithmetic sum of parallel propulsive engines on the vehicle or the “sustained vehicle system power according to UN Regulation No. 177, paragraph 8.9.1.(b).

2.9.	"Rated engine speed, S" means the declared engine speed in min-1 (rpm) at which the engine develops its rated maximum net power pursuant to Regulation No. 85 or, where the rated maximum net power is reached at several engine speeds, the highest one of those speeds.

2.10.	"Power to Mass Ratio index (PMR)" means a numerical quantity (see Annex 3, paragraph 3.1.2.1.1.) with no dimension used for the calculation of acceleration.

2.11.	"Reference point" means one of the following points:

2.11.1.	In the case of vehicles of categories M1, N1 and M2 < 3,500 kg technically permissible maximum laden mass:

(a)	For front engine vehicles: the front end of the vehicle;

(b)	For mid-engine vehicles: the centre of the vehicle;

(c)	For rear engine vehicles: the rear end of the vehicle.

For vehicles having multiple propulsion sources, the reference point is determined by the position of the propulsion source having the highest power. If there are multiple propulsion sources of equivalent power, then the position of the most forward propulsion source shall prevail.

2.11.2.	In the case of vehicles of categories M2 > 3,500kg technical permissible maximum laden mass, M3, N2, N3:

(a)	For front-engine vehicles, the front end of the vehicle;

(b)	For all other vehicles, the border of the engine closest to the front of the vehicle;

2.11.3.	For bidirectional vehicles, the reference point shall be determined separately for each forward direction.

2.12.	"Engine" means the power source without detachable accessories.

Power source includes in this context all sources of motive power; for example, electric or hydraulic power sources used alone or in combination with other power sources.

2.13.	"Target acceleration" means acceleration at a partial throttle condition in urban traffic and is derived from statistical investigations.

2.14.	"Reference acceleration" means the required acceleration during the acceleration test on the test track.

2.15.	"Gear ratio weighting factor k" means a dimensionless numerical quantity used to combine the test results of two gear ratios for the acceleration test and the constant speed test.

2.16.	"Partial power factor kp" means a numerical quantity with no dimension used for the weighted combination of the test results of the acceleration test and the constant speed test for vehicles.

2.17.	"Pre-acceleration" means application of acceleration control device prior to AA' for the purpose of achieving stable acceleration between AA' and BB' as referred to in Figure 1 of Appendix 1 to Annex 3.

2.18.	Gear

2.18.1.		"Gear ratios"

2.18.1.1.	“Internal gearbox ratio” means the ratios of engine to gearbox output shaft revolutions.

2.18.1.2.	“Final drive ratio” means the ratio(s) of gearbox output shaft to driven wheel revolutions.

2.18.1.3.	“Total gear ratio” means the ratios between vehicle speed and engine speed during the passage of the vehicle through the test track.

2.18.1.4.	“Gear ratio” used in context with vehicles tested according to 3.1.2.1. of Annex  3 and Annex 7 is the total gear ratio as defined in 2.18.1.3. above.

2.18.2.	"Locked gear ratio" means the control of the transmission such that the gear will not change during a test.

2.18.3.	"Gear" means in the context of this Regulation a discrete gear ratio either selectable by the driver or by an external device.

2.18.4.	For vehicles tested according to paragraph 3.1.2.1. of Annex 3 and Annex 7, "geari" and "geari+1" are defined as two gears in sequence, where geari either provides an acceleration within the 5 per cent tolerance according to paragraph 3.1.2.1.4.1. (a) of Annex 3 or an acceleration greater than the reference acceleration, and geari+1 an acceleration lower than the reference acceleration according to paragraph 3.1.2.1.4.1. (b) or (c) of Annex 3.

2.19.	"Silencing system" means a complete set of components necessary for limiting the sound produced by an engine, its intake and its exhaust (the exhaust manifold(s), the catalyst(s) and emission after-treatment device(s) are not considered part of the silencing system; these parts belong to the engine).

2.20.	"Design family of exhaust silencing system or exhaust silencing system components" means a group of silencing systems or components thereof in which all of the following characteristics are the same:

(a)	The presence of net gas flow of the exhaust gases through the absorbing fibrous material when in contact with that material;

(b)	The type of the fibres;

(c)	Where applicable, binder material specifications;

(d)	Average fibre dimensions;

(e)	Minimum bulk material packing density in kg/m³;

(f)	Maximum contact surface between the gas flow and the absorbing material.

2.21.	"Exhaust silencing system of different types" means silencing systems which significantly differ in respect of at least one of the following:

(a)	Trade names or trademarks of their components;

(b)	The characteristics of the materials constituting their components, except for the coating of those components;

(c)	The shape or size of their components;

(d)	The operating principles of at least one of their components;

(e)	The assembly of their components;

(f)	The number of exhaust silencing systems or components.

2.22.	"Replacement silencing system" means any part of the silencing system or its components intended for use on a vehicle, other than a part of the type fitted to this vehicle when submitted for type-approval pursuant to this Regulation.

2.23.	"R-point" means R-point as defined in paragraph 2.4. of Annex 1 to the Consolidated Resolution on the Construction of Vehicles (R.E.3.).

2.24.	Table of symbols

| Symbol | Unit |  | Annex |  | Paragraph |  |  | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mro | kg |  | Annex 3 |  | 2.2.1. |  |  | mass in running order; value to be reported and used for calculations to a precision of 10 kg |
| mt | kg |  | Annex 3 |  | 2.2.1. |  |  | test mass of the vehicle; value to be reported and used for calculations to a precision of 10 kg |
| mtarget | kg |  | Annex 3 |  | 2.2.1. |  |  | target mass of the vehicle |
| mxload | kg |  | Annex 3 |  | 2.2.1. |  |  | extra loading |
| mfa load unladen | kg |  | Annex 3 |  | 2.2.1. |  |  | front axle load in unladen condition |
| mra load unladen | kg |  | Annex 3 |  | 2.2.1. |  |  | rear axle load in unladen condition |
| munladen | kg |  | Annex 3 |  | 2.2.1. |  |  | unladen vehicle mass |
| mac ra max | kg |  | Annex 3 |  | 2.2.1. |  |  | Technically permissible maximum laden mass allowed for the rear axle as declared by the manufacturer |
| md | kg |  | Annex 3 |  | 2.2.1. |  |  | mass of driver |
| mchassis M2M3 | kg |  | Annex 3 |  | 2.2.1. |  |  | mass of the incomplete vehicle (M2 or M3) |
| mxload M2M3 | kg |  | Annex 3 |  | 2.2.1. |  |  | extra load to be added to the incomplete vehicle (M2 or M3) to reach the mass of the vehicle in running order as chosen by the manufacturer |
| mfa load laden | kg |  | Annex 3 |  | 2.2.7.2. |  |  | front axle load in laden condition |
| mra load laden | kg |  | Annex 3 |  | 2.2.7.2. |  |  | rear axle load in laden condition |
| mt (2 axles virtual) | kg |  | Annex 3 |  | 2.2.7.4. |  |  | test mass of a virtual vehicle with two axles (4x2 or 4x4) |
| vrf | — |  | Annex 3 |  | 2.2.7.4. |  |  | vehicle with more than two axles representing the vehicle family |
| munladen (2 axles virtual) | kg |  | Annex 3 |  | 2.2.7.4. |  |  | unladen vehicle mass of the virtual vehicle with two axles |
| mxload (2 axles virtual) | kg |  | Annex 3 |  | 2.2.7.4. |  |  | extra loading for the virtual vehicle with two axles |
| mac ra max (chosen) | kg |  | Annex 3 |  | 2.2.7.4. |  |  | Technically permissible maximum laden mass allowed for the chosen rear axle as defined in paragraph 2.2.7.4. in Annex 3 |
| AA' | — |  | Annex 3 |  | 3.1.1. |  |  | line perpendicular to vehicle travel which indicates beginning of zone in which to record sound pressure level during test |
| BB' | — |  | Annex 3 |  | 3.1.1. |  |  | line perpendicular to vehicle travel which is 10.00 m behind line PP' |
| CC' | — |  | Annex 3 |  | 3.1.1. |  |  | line of vehicle travel through test surface defined in ISO 10844 |
| PP' | — |  | Annex 3 |  | 3.1.1. |  |  | line perpendicular to vehicle travel which indicates location of microphones |
| vtest | km/h |  | Annex 3 |  | 3.1.2.1. |  |  | vehicle test speed |
| PMR | — |  | Annex 3 |  | 3.1.2.1.1. |  |  | power-to-mass ratio index to be used for calculations; value to be reported and used for calculations to the first decimal place |
| Pn | kW |  | Annex 3 |  | 3.1.2.1.1. |  |  | rated total engine net power |
| l | m |  | Annex 3 |  | 3.1.2.1.2. |  |  | reference length; value to be reported and used for calculations to a precision of 0.01 m (1 cm) |
| lveh | m |  | Annex 3 |  | 3.1.2.1.2. |  |  | length of vehicle; value to be reported and used for calculations to a precision of 0.01 m (1 cm) |
| vAA' | km/h |  | Annex 3 |  | 3.1.2.1.2. |  |  | vehicle velocity when the reference point passes line AA' (see paragraph 2.11. for definition of reference point); value to be reported and used for calculations to the first decimal place |
| vBB' | km/h |  | Annex 3 |  | 3.1.2.1.2. |  |  | vehicle velocity when the reference point or rear of vehicle passes line BB' (see paragraph 2.11. for definition of reference point); value to be reported and used for calculations to the first decimal place |
| vPP' | km/h |  | Annex 3 |  | 3.1.2.1.2. |  |  | vehicle velocity when the reference point passes line PP' (see paragraph 2.11. for definition of reference point); value to be reported and used for calculations to the first decimal place |
| awot test | m/s2 |  | Annex 3 |  | 3.1.2.1.2.1. |  |  | acceleration at wide-open throttle from AA' to BB'; value to be reported and used for calculations to the second decimal place |
| awot test,i | m/s2 |  | Annex 3 |  | 3.1.2.1.2.1. |  |  | acceleration at wide-open throttle achieved in a particular gear i; value to be reported and used for calculations to the second decimal place |
| lpa | m |  | Annex 3 |  | 3.1.2.1.2.1. |  |  | point of depressing the accelerator before line AA'; value to be reported in full meter |
| awot test, PP-BB | m/s2 |  | Annex 3 |  | 3.1.2.1.2.2. |  |  | acceleration at wide-open throttle from PP' to BB'; value to be reported and used for calculations to the second decimal place |
| aurban | m/s2 |  | Annex 3 |  | 3.1.2.1.2.3. |  |  | target acceleration representing urban traffic acceleration; value to be reported and used for calculations to the second decimal place |
| awot ref | m/s2 |  | Annex 3 |  | 3.1.2.1.2.4. |  |  | reference acceleration for the wide-open-throttle test; value to be reported and used for calculations to the second decimal place |
| kP | — |  | Annex 3 |  | 3.1.2.1.3. |  |  | partial power factor; value to be reported and used for calculations to the second decimal place |
| awot i | m/s2 |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | acceleration at wide-open-throttle in gear ratio i; value to be reported and used for calculations to the second decimal place |
| awot (i + 1) | m/s2 |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | acceleration at wide-open-throttle in gear ratio (i + 1); value to be reported and used for calculations to the second decimal place |
| gear ratio i | — |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | gear ratio which provides an acceleration within the 5 per cent tolerance of the reference acceleration awot_ref or greater than the reference acceleration awot_ref |
| gear ratio i+1 | — |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | second of two gear ratios, with an acceleration lower than gear ratio i |
| gear ratio i+2, i+3, … | — |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | gear ratios selectable for the pass-by test of Annex 3, if gear ratio i and gear ratio i+1 exceed an acceleration of 2.0 m/s² |
| k | — |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | gear ratio weighting factor; value to be reported and used for calculations to the second decimal place |
| nMAX | 1/min |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | Maximum engine rotational speed permitted for M1, N1, and M2 less than 3500 kg; value to be reported and used for calculations to a precision of 10 min-1 (xxx0) |
| aanchor | m/s² |  | Annex 7 |  | 3.1. |  |  | reported vehicle acceleration for gear ratio i from Annex 3; value to be reported and used for calculations to the first decimal place |
| S | 1/min |  | Annex 3 |  | 3.1.2.1.4.1. |  |  | rated engine rotational speed in revs per minute, synonymous with the engine rotational speed at maximum power |
| nBB' | 1/min |  | Annex 3 |  | 3.1.2.2. |  |  | engine rotational speed of the vehicle, when the reference point passes BB'; value to be reported and used for calculations to a precision of 10 min-1 |
| ntarget BB' | 1/min |  | Annex 3 |  | 3.1.2.2.1.1.(a) |  |  | target engine rotational speed of the vehicle when the reference point has to pass line BB' (see 2.11.2. for definition of reference point) |
| vtarget BB' | km/h |  | Annex 3 |  | 3.1.2.2.1.1.(a) |  |  | target vehicle velocity when it is necessary that the reference point has to pass line BB' (see 2.11.2. for definition of reference point) |
| vBB' gear I | km/h |  | Annex 3 |  | 3.1.2.2.1.1.(b) |  |  | target vehicle velocity when certain conditions are met |
| vBB' gear i, i= 1,2 | km/h |  | Annex 3 |  | 3.1.2.2.1.1. (c) |  |  | target vehicle velocity when certain conditions are met |
| gearx | - |  | Annex 3 |  | 3.1.2.2.1.1. (d) |  |  | first of two gear ratios used for testing of M2 having a maximum authorized mass of more than 3 500 kg, M3, N2, and N3 where certain criteria on test conditions are met |
| geary | - |  | Annex 3 |  | 3.1.2.2.1.1. (d) |  |  | second of two gear ratios used for testing of M2 having a maximum authorized mass of more than 3,500 kg, M3, N2, and N3 where certain criteria on test conditions are met |
| vBB'x | km/h |  | Annex 3 |  | 3.1.2.2.1.1. (d) |  |  | target vehicle velocity when certain conditions are met |
| vBB'y | km/h |  | Annex 3 |  | 3.1.2.2.1.1. (d) |  |  | target vehicle velocity when certain conditions are met |
| vBB'1 | km/h |  | Annex 3 |  | 3.1.2.2.1.2. (b) |  |  | target vehicle velocity when certain conditions are met |
| vBB'2 | km/h |  | Annex 3 |  | 3.1.2.2.1.2. (b) |  |  | target vehicle velocity when certain conditions are met |
| nBB'i, i=1,2 | 1/min |  | Annex 3 |  | 3.1.2.2.1.2. (d) |  |  | engine rotational speed when the reference point passes BB' when certain conditions are met |
| Lcrs (i) | dB(A) |  | Annex 3 |  | 3.1.3.4.1.2. |  |  | vehicle sound pressure level at constant speed test for gear i; value to be reported and used for calculations to the first decimal place |
| Lcrs (i + 1) | dB(A) |  | Annex 3 |  | 3.1.3.4.1.2. |  |  | vehicle sound pressure level at constant speed test for gear (i + 1); value to be reported and used for calculations to the first decimal place |
| Lcrs rep | dB(A) |  | Annex 3 |  | 3.1.3.4.1.2. |  |  | reported vehicle sound pressure level at constant speed test; value to be reported and used for calculations to the first decimal place |
| Lwot (i) | dB(A) |  | Annex 3 |  | 3.1.3.4.1.2. |  |  | vehicle sound pressure level at wide-open-throttle test for gear i; value to be reported and used for calculations to the first decimal place |
| Lwot (i + 1) | dB(A) |  | Annex 3 |  | 3.1.3.4.1.2. |  |  | vehicle sound pressure level at wide-open-throttle test for gear (i + 1); ; value to be reported and used for calculations to the first decimal place |
| Lwot rep | dB(A) |  | Annex 3 |  | 3.1.3.4.1.2. |  |  | reported vehicle sound pressure level at wide-open-throttle; value to be reported and used for calculations to the first decimal place |
| Lurban | dB(A) |  | Annex 3 |  | 3.1.3.4.1.2. |  |  | reported vehicle sound pressure level representing urban operation; value to be reported mathematically rounded to the nearest integer |
| LTR,Jref,(vTR,ref) |  | dB(A) |  | Annex 3 - Appendix 2 Annex 3 – Appendix 3 |  | 3.1. 4.3 | Reported reference test result of the tyre rolling sound measurements at left/right side according to method described in Annex 3 Appendix 3 |  |  |
| slpref |  | dB(A)/log(v) |  | Annex 3 - Appendix 2 |  | 3.1. | Slope of the tyre rolling sound measurements as determined by Annex 3 Appendix 3 |  |  |
| vTR,ref |  | km/h |  | Annex 3 - Appendix 2 Annex 3 – Appendix 3 |  | 3.1. 4.3. | The reference vehicle speed for the reference tyre rolling sound; this speed may be different from vwot or Vcrs if tyre rolling sound data have been generated independently from the type approval test concerned. (see Annex 3 Appendix 3 paragraph 2.4.1. (b)) |  |  |
| vcrs,j |  | km/h |  | Annex 3 - Appendix 2 |  | 3.2./4.3. | Vehicle speed when the reference point of the vehicle passes line PP' during a pass-by test according Annex paragraph 3.1.2.1.6. |  |  |
| vwot,PP’,j |  | km/h |  | Annex 3 - Appendix 2 |  | 3.3./4.4. | Vehicle speed when the reference point of the vehicle passes line PP' during a pass-by test according Annex paragraph 3.1.2.1.5. |  |  |
| vwot,BB’,j |  | km/h |  | Annex 3 - Appendix 2 |  | 3.3./4.4. | Vehicle speed when the rear of the vehicle passes line BB' during a pass-by test according Annex paragraph 3.1.2.1.5. |  |  |
| Jref |  | °C |  | Annex 3 - Appendix 2 |  | 3. | Reference air temperature: 20 °C |  |  |
| Jcrs,j |  | °C |  | Annex 3 - Appendix 2 |  | 3.2. | Air temperature representative for one pass-by test run j under constant speed condition |  |  |
| Jwot,j. |  | °C |  | Annex 3 - Appendix 2 |  | 3.3. | Air temperature representative for one pass-by test run j under acceleration condition |  |  |
| LTR,crs,j,Jcrs |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.2.3. | Tyre rolling sound adjusted to the speed condition of the constant speed test |  |  |
| LPT,crs,j |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.2.4. | Extracted power train component from each valid constant speed test |  |  |
| LTR,crs,j,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.2.2. | Tyre rolling sound adjusted to the speed condition of the constant speed test and the reference temperature |  |  |
| Lcrs,j,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.2.5. | Air temperature adjusted constant speed test result |  |  |
| LTR,wot,j,Jwot |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.3.3. | Tyre rolling sound adjusted to the speed condition of the acceleration test |  |  |
| LPT,wot,j |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.3.4 | Extracted power train component from each valid acceleration test |  |  |
| LTR,wot,j,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.3.2. | Tyre rolling sound adjusted to the speed condition of the acceleration test and the reference temperature |  |  |
| Lwot,j,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 3.3.5. | Air temperature adjusted acceleration test result |  |  |
| LTR,DB,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 4.1. | Reported reference test result of the tyre rolling sound measurement left/right side according to Annex 3 Appendix 3 taken from a database |  |  |
| LTR,DB,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 4.1. | Reported reference test result of the tyre rolling sound measurement at vTEST left/right side according to Annex 3 Appendix 3 taken from a database |  |  |
| slpDB,ref |  | kg |  | Annex 3 - Appendix 2 |  | 4.1. | Slope of the tyre rolling sound measurements as determined by Annex 3 Appendix 3 taken from a database |  |  |
| vDB,TR,ref |  | km/h |  | Annex 3 - Appendix 2 |  | 4.1. | The reference vehicle speed for the reference tyre rolling sound; this speed may be different from vcrs or vwot, if tyre rolling sound data have been generated independently from the type approval test concerned. (see Annex 3 Appendix 3 paragraph 2.4.1. (b)) |  |  |
| LTR,DB,crs,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 4.1. | Reported reference test result of the tyre rolling sound measurement at vcrs left/right side according to Annex 3 Appendix 3 taken from a database |  |  |
| LTR,DB,wot,Jref |  | dB(A) |  | Annex 3 - Appendix 2 |  | 4.1. | Reported reference test result of the tyre rolling sound measurement at vwot left/right side according to Annex 3 Appendix 3 taken from a database |  |  |
| awot_ASEP | m/s2 |  | Annex 7 |  | 2.3. |  |  | maximum required acceleration at wide-open-throttle |
| κ | — |  | Annex 7 |  | 2.3. |  |  | gears to be tested under "Additional Sound Emission Provisions" (ASEP) |
| nBB_ASEP | 1/min |  | Annex 7 |  | 2.3. |  |  | maximum test engine speed; value to be reported and used for calculations to a precision of 10 min-1 |
| vAA’_ASEP | km/h |  | Annex 7 |  | 2.3. |  |  | Target vehicle velocity for test point P1 of the assessment method according paragraph 2.4. |
| vBB’_ASEP | km/h |  | Annex 7 |  | 2.3. |  |  | Target vehicle velocity for test point P4 of the assessment method according paragraph 2.4. |
| Pj | — |  | Annex 7 |  | 2.4. |  |  | test point(s) under ASEP |
| j | — |  | Annex 7 |  | 2.4. |  |  | index for the test points under ASEP |
| vBB_j | km/h |  | Annex 7 |  | 2.4. |  |  | vehicle test speed at BB' for a particular ASEP test point |
| awot,test, κj | m/s2 |  | Annex 7 |  | 2.5. |  |  | acceleration at wide-open throttle achieved in gear κ and at test point j |
| Lwot,κj | dB(A) |  | Annex 7 |  | 2.5. |  |  | sound pressure level measured for a gear κ and at a test point j; value to be reported and used for calculations to the first decimal place |
| nBB,κj | 1/min |  | Annex 7 |  | 2.5. |  |  | vehicle test engine speed at BB' for a gear κ and at test point j |
| vAA,κj | km/h |  | Annex 7 |  | 2.5. |  |  | vehicle test speed at AA' for a gear κ and at test point j; value to be reported and used for calculations to the first decimal place |
| vBB,κj | km/h |  | Annex 7 |  | 2.5. |  |  | vehicle test speed at BB' for a gear κ and at test point j; value to be reported and used for calculations to the first decimal place |
| vPP,κj | km/h |  | Annex 7 |  | 2.5. |  |  | vehicle test speed at PP' for a gear κ and at test point j; value to be reported and used for calculations to the first decimal place |
| Lanchor | dB(A) |  | Annex 7 |  | 3.1. |  |  | reported vehicle sound pressure level for gear ratio i from Annex 3; value to be reported and used for calculations to the first decimal place |
| nanchor | 1/min |  | Annex 7 |  | 3.1. |  |  | reported vehicle engine speed for gear ratio i from Annex 3 |
| vanchor | km/h |  | Annex 7 |  | 3.1. |  |  | reported vehicle test speed for gear ratio i at BB’ from Annex 3; value to be reported and used for calculations to the first decimal place |
| Lκj | dB(A) |  | Annex 7 |  | 3.5. |  |  | sound pressure level measured for a gear κ and at a test point j; value to be reported and used for calculations to the first decimal place |
| LASEP κ,j | dB(A) |  | Annex 7 |  | 3.3. |  |  | sound pressure level calculated for a gear k at test point j to reflect the estimated linear sound level increase as a function of the engine speed (ICEs) or the vehicle speed (EVs) |
| ΔLASEP, j | dB |  | Annex 7 |  | 3.4. |  |  | added sound pressure level for ASEP at test point j to reflect the relation of the measured performance versus performance at the anchor point; value to be used for calculations to the first decimal place |
| kP_ASEP | — |  | Annex 7 |  | 4.2.1. |  |  | partial power factor determined for the Lurban principle of ASEP |
| Lwot_ASEP | dB(A) |  | Annex 7 |  | 4.2.1. |  |  | vehicle sound pressure level measured for the Lurban principle of ASEP; value to be reported and used for calculations to the first decimal place |
| Lurban_measured_ASEP | dB(A) |  | Annex 7 |  | 4.2.1. |  |  | interim result for calculation of ΔLurban_ASEP; value to be reported and used for calculations to the first decimal place |
| Lurban_normalized | dB(A) |  | Annex 7 |  | 4.2.1. |  |  | interim result for calculation of ΔLurban_ASEP; value to be reported and used for calculations to the first decimal place |
| ΔLurban_ASEP | dB(A) |  | Annex 7 |  | 4.2.1. |  |  | estimated deviation from urban sound pressure level; value to be reported to the first decimal place |
| α | __ |  | Annex 7 |  | 5.2 |  |  | gear to be determined for the reference sound assessment according to the type of transmission |
| Lref | dB(A) |  | Annex 7 |  | 5.3. |  |  | reference sound pressure level for reference sound assessment; value to be reported and used for calculations to the first decimal place |
| nBB’_ref | 1/min |  | Annex 7 |  | 5.3. |  |  | Reference vehicle test engine speed for reference sound assessment |
| vBB’_ref | km/h |  | Annex 7 |  | 5.3. |  |  | reference vehicle test speed for reference sound assessment |

2.25.	Modes

2.25.1.	"Mode" means a distinct driver-selectable condition which does affect the sound emission of the vehicle.

2.26.	Stable acceleration

For the purpose of this Regulation, three conditions are defined for stable 	acceleration.

2.26.1.	"Stable acceleration" applicable to all vehicles subject to this Regulation for low engine speed conditions, which eliminates power train reactions such as bumping and jerking.

2.26.2.	"Stable acceleration" applicable to vehicles of category M1, N1 and M2 < 3,500 kg technically permissible maximum laden mass avoids acceleration delays in acceleration due to engine control application, at the moment when the acceleration unit is depressed. This is typically ensured by using pre-acceleration.

2.26.3.	"Stable acceleration" for the purpose of Annex 7 is based on the assumption of a constant acceleration over the complete measurement distance between AA' and BB' plus the vehicle length.

2.27.	"Kickdown" means a driver initiated automated gear shift to a test condition outside the specific target conditions for the vehicle as defined in Annex 3.

2.28.		"Prevention of downshift" means a measure by the vehicle manufacturer to ensure that the vehicle is tested within its specific target conditions as defined in Annex 3 and Annex 7.

### 2.29.	“Automated Driving System (ADS)” means the vehicle hardware and software that are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis. 1

### 2.30.	“Dynamic Driving Task (DDT)” means the real-time operational and tactical functions required to operate the vehicle. 1

### 2.31.	“Bidirectional vehicle” means a vehicle that can operate in two opposite forward directions. 1

### 2.32.	“Forward direction” (of a vehicle) means a direction that is intended to be used as main travelling direction of the vehicle. 1

2.33.	“Accelerator” means

(a)	for manually driven vehicles:

the device used by the driver to control the engine speed and the vehicle speed;

(b)	for vehicles equipped with an ADS:

the function in the ADS used to control the engine speed and the vehicle speed.

2.34.		Categories of vehicles equipped with an ADS 1

2.34.1.	“Category X vehicles” are vehicles of categories M, N, L and T meeting all of the following conditions:

(a)	They are equipped with an ADS

(b)	They are not capable of being driven manually at speed exceeding 6 km/h; and

(c)	They are designed to carry occupants

2.34.2.	“Category Y vehicles” are vehicles of categories N, L and T meeting all of the following conditions:

(a)	They are equipped with an ADS

(b)	They are not capable of being driven manually at speed exceeding 6 km/h; and

(c)	They are not designed to carry occupants at any time

2.35.	"Enhanced Exterior Sound (EES)" means any sound

- generated by one or more active systems installed on a vehicle for the purpose of enhancing the vehicle’s exterior sound emission, and

- which is not subject to another UN Regulation."

2.36.	"Vehicle Master Control Switch" means the device by which the vehicle’s on-board electronics system is brought, from being switched off, as in the case where a vehicle is parked without the driver being present, to normal operation mode (i.e. the vehicle status is "ready to drive").

3.		Application for approval

3.1.	The application for approval of a vehicle type with regard to sound shall be submitted by its manufacturer or by his duly accredited representative.

3.2.	It shall be accompanied by the undermentioned documents and the following particulars in triplicate:

3.2.1.	A description of the vehicle type with regard to the items mentioned in paragraph 2.2. above. The numbers and/or symbols identifying the engine type and the vehicle type shall be specified;

3.2.2.	A list of the components, duly identified, constituting the sound reduction system;

3.2.3.	A drawing of the assembled sound reduction system and an indication of its position on the vehicle;

3.2.4.	Detailed drawings of each component to enable it to be easily located and identified, and a specification of the materials used.

3.2.5.	A technical information document including the information as outlined in Annex 1, Appendix 2.

3.3.	In the case of paragraph 2.2.2. the single vehicle, representative of the type in question, will be selected by the Technical Service conducting approval tests, in accordance with the vehicle manufacturer, following the specification laid down in paragraph 3.1.2.2. in Annex 3.

3.4.	Approval tests

3.4.1.	At the request of the Technical Service conducting approval tests, the vehicle manufacturer shall, in addition, submit a sample of the sound reduction system and an engine of at least the same cylinder capacity and rated maximum net power as that fitted to the vehicle in respect of which type-approval is sought.

3.4.2.	Tyre rolling sound reference measurements according to Annex 3 Appendix 3 which are carried out independent of the type approval tests of a vehicle (see Case 2 of Annex 3 Appendix 2) are not mandatory but can be performed at the option and responsibility of the vehicle manufacturer.

Where the vehicle manufacturer decides to perform such tests, they shall

(a)	either be carried out by the vehicle manufacturer witnessed by the Type Approval Authority or by a Technical Service, or

(b)	be carried out by the vehicle manufacturer by its laboratories and test facilities which may be designated as an approved laboratory, or

(c)	by laboratories and test facilities of a Technical Service designated by the Type Approval Authority and selected by the vehicle manufacturer.

The test results shall be submitted to the Type Approval Authority as reference data to be used when tests other than type approval test are carried out on a different test track.

Where no reference data have been established, no test track compensation is applicable for above mentioned tests. Therefore, only Case 1 temperature correction is applicable.

3.5.	The Type Approval Authority shall verify the existence of satisfactory arrangements for ensuring effective control of the conformity of production before type approval is granted.

4.		Markings

4.1.	The components of the sound reduction system, excluding fixing hardware and piping, shall bear:

4.1.1.	The trade name or mark of the manufacturer of the sound reduction system and of its components; and

4.1.2.	The manufacturer's trade description;

4.2.	These markings shall be clearly legible and be indelible even after fitting.

4.3.	A component may carry several approval numbers if it has been approved as component of several replacement silencing systems.

5.		Approval

5.1.	Type approval shall only be granted if the vehicle type meets the requirements of paragraphs 6. and 7. below.

5.1.1. 	Starting from 1 July 2023 and for a period of twelve months, during type approval of a vehicle, measurements in accordance with Annex 9 (RD-ASEP) shall be performed. The complete test report (in pdf format) shall be communicated to the Type Approval Authority by their Technical Services and in addition, the monitoring information shall be entered into the data entry file "RD-ASEP Monitoring Data Sheet", which is published on the UN-ECE website.

For the purpose of type approval, it is not mandatory to comply with the provisions of Annex 9.

For vehicles with PMR not exceeding 60, the performance of RD-ASEP tests is not mandatory.

RD-ASEP tests are not applicable to any tests done for the purpose of extension of existing approvals according to UN Regulation No. 51.

In case the type approval tests of Annex 3 and Annex 7 were carried out in an indoor facility, the test and the delivery of data according to Annex 9 are not mandatory.

Notwithstanding the provisions above, the Type Approval Authorities, Technical Services, and vehicle manufacturers may digitally submit additional vehicle tests of approved vehicles in the format as specified above, provided that all data have been determined on a single vehicle. This is applicable but not limited to

- extensions of type approval, or

- vehicles not falling under the scope of RD-ASEP, or

- approvals not falling within the monitoring period.

All tests for Annex 3 and for Annex 9 shall be carried out on the same test track and under similar environmental conditions.

5.2.	An approval number shall be assigned to each type approved. Its first two digits (at present 03 corresponding to the 03 series of amendments) shall indicate the series of amendments incorporating the most recent major technical amendments made to the Regulation at the time of issue of the approval. The same Contracting Party shall not assign the same number to another vehicle type.

5.3.	Notice of approval or of extension or of refusal or withdrawal of approval or of production definitively discontinued of a vehicle type pursuant to this Regulation shall be communicated to the Parties to the Agreement applying this Regulation, by means of a form conforming to the model in Annex 1 to this Regulation.

5.4.	There shall be affixed, conspicuously and in a readily accessible place specified on the approval form, to every vehicle conforming to a vehicle type approved under this Regulation an international approval mark consisting of:

5.4.1.	A circle surrounding the letter "E" followed by the distinguishing number of the country which has granted approval;

5.4.2.	The number of this Regulation, followed by the letter "R", a dash and the approval number to the right of the circle prescribed in paragraph 5.4.1.

5.5.	If the vehicle conforms to a vehicle type approved under one or more other Regulations annexed to the Agreement, in the country which has granted approval under this Regulation, the symbol prescribed in paragraph 5.4.1. need not be repeated; in such a case the regulation and approval numbers and the additional symbols of all the Regulations under which approval has been granted in the country which has granted approval under this Regulation shall be placed in vertical columns to the right of the symbol prescribed in paragraph 5.4.1.

5.6.	The approval mark shall be clearly legible and be indelible.

5.7.	The approval mark shall be placed close to or on the vehicle data plate affixed by the manufacturer.

5.8.	Annex 2 to this Regulation gives examples of arrangements of the approval mark.

6.		Specifications

6.1.	General specifications for durability and against manipulation

6.1.1.	The vehicle, its engine and its sound reduction system shall be so designed, constructed and assembled as to enable the vehicle, in normal use, despite the vibration to which it may be subjected, to comply with the provisions of this Regulation.

6.1.2.	The sound reduction system shall be so designed, constructed and assembled as to be able to reasonably resist the corrosive phenomena to which it is exposed having regard to the conditions of use of the vehicle, including regional climate differences, and against manipulation.

6.2.	Specifications regarding sound levels

6.2.1.	Methods of measurement

6.2.1.1.	The sound made by the vehicle type submitted for approval shall be measured either indoors or outdoors by the methods described in Annex 3 to this Regulation . The specific conditions for indoor testing are provided in Annex 8 to this Regulation. The results of the outdoor and indoor tests are deemed equivalent.

For each specific test condition for vehicles, the manufacturer can select to test the vehicle either indoors or outdoors. The Type Approval Authority shall always have the option to mandate an outdoor test for verification.  The option of the type approval authority to mandate an outdoor test shall apply to any test specified in this Regulation, including conformity of production testing.

In addition, the sound shall be measured on the vehicle when stationary; in the case of a vehicle where an internal combustion engine cannot operate when the vehicle is stationary, the emitted sound shall only be measured in motion. In the case of a hybrid electrical vehicle of category M1 where an internal combustion engine cannot operate when the vehicle is stationary, the emitted sound shall be measured according to Annex 3, paragraph 4.

Vehicles having a technically maximum permissible laden mass exceeding 2,800 kg shall be subjected to an additional measurement of the compressed air noise with the vehicle stationary in accordance with the specifications of Annex 5, if the corresponding brake equipment is part of the vehicle.

6.2.1.2.	The values measured in accordance with the provisions of paragraph 6.2.1.1. above shall be entered in the test report and a certificate corresponding to the model shown in Annex 1.

6.2.2.	Sound level limits

The sound level measured in accordance with the provisions of paragraph 3.1. of Annex 3 to this Regulation, mathematically rounded to the nearest integer value, shall not exceed the following limits:

| Vehicle category | Vehicles used for the carriage of passengers | Limit Values (dB(A)) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Phase 1 | Phase 2 | Phase 3 |
| M1 | PMR ≤ 120 | 72 | 70 | 68 |
|  | 120 < PMR ≤ 160 | 73 | 71 | 69 |
|  | PMR > 160 | 75 | 73 | 71 |
|  | PMR > 200, no. of seats ≤ 4, R-point height < 450mm from the ground | 75 | 74 | 72 |
| M2 | M ≤ 2.5 t | 72 | 70 | 69 |
|  | 2.5 t < M ≤ 3.5 t | 74 | 72 | 71 |
|  | M > 3.5 t; Pn ≤ 135 kW | 75 | 73 | 72 |
|  | M > 3.5 t; Pn > 135 kW | 75 | 74 | 72 |
| M3 | Pn ≤ 150 kW | 76 | 74 | 73 |
|  | 150 kW < Pn ≤ 250 kW | 78 | 77 | 76 |
|  | Pn > 250 kW | 80 | 78 | 77 |
| Vehicle category | Vehicles used for the carriage of goods | Phase 1 | Phase 2 | Phase 3 |
| N1 | M ≤ 2.5 t | 72 | 71 | 69 |
|  | M > 2.5 t | 74 | 73 | 71 |
| N2 | Pn ≤ 135kW | 77 | 75 | 74 |
|  | Pn > 135 kW | 78 | 76 | 75 |
| N3 | Pn ≤ 150 kW | 79 | 77 | 76 |
|  | 150 kW < Pn ≤ 250 kW | 81 | 79 | 77 |
|  | Pn > 250 kW | 82 | 81 | 79 |

6.2.2.1.	For vehicle types of category M1 derived from N1 vehicle types having a technically permissible maximum laden mass above 2.5 tons and a R-point height greater than 850 mm from the ground, the limits of vehicles types of category N1 having a technically permissible maximum laden mass above 2.5 tons apply.

6.2.2.2.	For vehicle types designed for off-road use, the limit values shall be increased by 2 dB(A) for M3 and N3 vehicles category and 1 dB(A) for any other vehicle category.

For vehicle types of category M1 the increased limit values for off-road vehicles are only valid if the technically permissible maximum laden mass > 2 tons.

6.2.2.3.	Limit values shall be increased by 2 dB(A) for wheelchair accessible vehicles of category M1 constructed or converted specifically so that they accommodate one or more persons seated in their wheelchairs when travelling on the road, and armoured vehicles, as defined in paragraph 2.5.2. of R.E.3.

6.2.2.4.		For vehicle types of category M3 having a gasoline only engine, the applicable limit value is increased by 2 dB(A).

6.2.2.5.	For vehicle types of category N1 having a technically permissible maximum laden mass of less than or equal to 2.5 tons, the engine capacity not exceeding 660 cc and the power-to-mass ratio (PMR) calculated by using the technically permissible maximum laden mass not exceeding 35 and a horizontal distance "d" between the front axle and the driver's seat R point of less than 1,100 mm, the limits of the vehicle types of category N1 having a technically permissible maximum laden mass above 2.5 tons apply.

6.2.3.	Additional Sound Emission Provisions (ASEP)

The Requirements of Annex 7ASEP apply to vehicles of categories M1 and N1.

Vehicles are deemed to fulfil the requirements of Annex 7, if the vehicle manufacturer provides technical documents to the type approval authority showing, that the difference between maximum and minimum engine speed of vehicles at BB' for any test condition inside the ASEP control range defined in paragraph 2.3. of Annex 7 of this Regulation (including Annex 3 conditions) does not exceed 0.15 x S. This article is intended especially for non-lockable transmissions with variable gear ratios (CVT).

Vehicles are exempted from ASEP if one of the following conditions is fulfilled:

(a)	For vehicles of category N1, if the engine capacity does not exceed 660 cc and the power-to-mass ratio PMR calculated by using the technically permissible maximum laden mass does not exceed 35.

(b)	For vehicles of category N1, if the payload is at least 850 kg and the power-to-mass ratio calculated by using the technically permissible maximum laden mass does not exceed 40.

(c) 	For vehicles of category N1 or M1 derived from N1 if the technically permissible maximum laden mass is greater than 2.5 tons and the R-point height is greater than 850 mm from the ground and the power- to-mass ratio calculated by using the technically permissible maximum laden mass does not exceed 40.

The sound emission of the vehicle under typical on-road driving conditions, which are different from those under which the type-approval test set out in Annex 3 and Annex 7 was carried out, shall not deviate from the test result in a significant manner11.

Any emission of EES shall be activated during the type-approval test.

6.2.3.1.	The vehicle manufacturer shall not intentionally alter, adjust, or introduce any mechanical, electrical, thermal, or other device or procedure solely for the purpose of fulfilling the sound emission requirements as specified under this Regulation which is not operational during typical on-road operation.

6.2.3.2.	The vehicle shall meet the requirements of Annex 7 to this Regulation.

6.2.3.3.	In applying for type approval, the manufacturer shall provide a statement, in conformity with Appendix 1 of Annex 7, that the vehicle type to be approved complies with the requirements of paragraph 6.2.3. of this Regulation.6.2.3.3.	Specifications for EVs and HEVs in the operating conditions in accordance with Annex 7 paragraph 1.1.

6.2.3.3.1.	Operation of EES

When the vehicle is switched on by the vehicle master control switch, any EES shall be deactivated.

Any activation of EES shall require an intentional action of the driver.

The EES shall be capable of being deactivated.

6.2.3.3.2.	Vehicle Sound Limits

In the speed range equal to or lower than 20 km/h including standstill the sound emitted by the vehicle shall comply with the requirements of UN-Regulation No. 138 paragraph 6.

In the speed range from 20 km/h to 80 km/h the sound emitted by the vehicle shall comply with the requirements of this Regulation.

6.2.3.4.	In applying for type approval, the manufacturer shall provide a statement, in conformity with Appendix 1 of Annex 7, that the vehicle type to be approved complies with the requirements of paragraph 6.2.3. of this Regulation.

6.3.	Specifications regarding exhaust systems containing fibrous materials

6.3.1.	Requirements of Annex 4 shall be applied.

7.		Modification and extension of approval of a vehicle type

7.1.		Every modification of the vehicle type shall be notified to the Type Approval Authority which approved the vehicle type. The Type Approval Authority may then either:

7.1.1.	Consider that the modifications made are unlikely to have an appreciable adverse effect and that in any case the vehicle still complies with the requirements, or

7.1.2.	Require a further test report from the Technical Service responsible for conducting the tests.

7.2.	Confirmation or refusal of approval, specifying the alterations shall be communicated by the procedure specified in paragraph 5.3. above to the Parties to the Agreement applying this Regulation.

7.3.		The Type Approval Authority issuing the extension of approval shall assign a series number for such an extension and inform thereof the other Parties to the 1958 Agreement applying this Regulation by means of a communication form conforming to the model in Annex 1 to this Regulation.

8.		Conformity of production

8.1	The conformity of production procedures shall comply with those set out in the Agreement, Appendix 2 (E/ECE/324-E/ECE/TRANS/505/Rev.2) with the following requirements:

8.1.1.	Vehicles approved according to this Regulation shall be so manufactured as to conform to the type approved by meeting the requirements of paragraph 6. above.

8.1.2.	The minimum requirements for conformity of production control procedures of Annex 6 to this Regulation shall be complied with.

8.2.	The authority which has granted type approval may at any time verify the conformity control methods applied in each production facility. The normal frequency of these verifications shall be one every two years.

9.		Penalties for non-conformity of production

9.1.		The approval granted in respect of a vehicle type pursuant to this Regulation may be withdrawn if the requirements set forth above are not met.

9.2.	If a Contracting Party to the Agreement applying this Regulation withdraws an approval it has previously granted, it shall forthwith so notify the other Contracting Parties applying this Regulation, by means of a communication form conforming to the model in Annex 1 to this Regulation.

10.		Production definitively discontinued

10.1.	If the holder of the approval completely ceases to manufacture a vehicle type approved in accordance with this Regulation, he shall so inform the authority which granted the approval. Upon receiving the relevant communication that authority shall inform thereof the other Parties to the 1958 Agreement applying this Regulation by means of a communication form conforming to the model in Annex 1 to this Regulation.

11.		Transitional provisions

  - As from the official date of entry into force of the 03 series of amendments to this Regulation, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type-approvals under this Regulation as amended by the 03 series of amendments.

  - Transitional Provisions for Phase 1 (see paragraph 6.2.2. above)

    - As from 1 July 2016, Contracting Parties applying this Regulation shall grant type-approvals only if the vehicle type to be approved meets the requirements of phase 1 (see paragraph 6.2.2. above) of this Regulation as amended by the 03 series of amendments.

As from the official date of entry into force of the 03 series of amendments, Contracting Parties applying this Regulation shall grant type approvals to the vehicle type which meets the requirements of phase 2 or phase 3 of this Regulation as amended by the 03 series of amendments.

    - Contracting Parties applying this Regulation shall not refuse to grant extensions of type approvals for existing types which have been granted according to the 02 series of amendments to this Regulation.

    - Until 30 June 2022, no Contracting Party applying this Regulation shall refuse national or regional type approval of a vehicle type-approved to the 02 series of amendments to this Regulation.

    - As from 1 July 2022, Contracting Parties applying this Regulation shall not be obliged to accept for the purpose of national or regional type approval, a vehicle type approved to the preceding series of amendments to this Regulation.

    - Even after the date of entry into force of the 03 series of amendments to this Regulation, Contracting Parties applying this Regulation may continue for national or regional purposes granting type approvals and extensions of type approvals to the preceding series of amendments to this Regulation.

  - Transitional Provisions for Phase 2 (see paragraph 6.2.2. above)

    - As from 1 July 2020 for vehicle types other than N2 and as from 1 July 2022 for vehicles types of category N2, Contracting Parties applying this Regulation shall grant type approvals only if the vehicle type to be approved meets the requirements of phase 2 (see paragraph 6.2.2. above) of this Regulation as amended by the 03 series of amendments.

Furthermore, as from the official date of entry into force of the 03 series of amendments, Contracting Parties applying this Regulation shall grant type approvals to the vehicle type which meets the requirements of phase 3 of this Regulation as amended by the 03 series of amendments.

    - Contracting Parties applying this Regulation shall not refuse to grant extensions of type approvals for existing types which have been granted according to phase 1 (see paragraph 6.2.2. above) or the 02 series of amendments to this Regulation.

    - Until 30 June 2022 for vehicle types other than N2 and until 30 June 2023 for vehicles types of category N2, no Contracting Party applying this Regulation shall refuse national or regional type approval of a vehicle type-approved to phase 1 (see paragraph 6.2.2. above) or the 02 series of amendments to this Regulation.

    - As from 1 July 2022 for vehicle types other than N2 and as from 1 July 2023 for vehicles types of category N2, Contracting Parties applying this Regulation shall not be obliged to accept for the purpose of national or regional type approval, a vehicle type approved to phase 1 (see paragraph 6.2.2.1. above) or the preceding series of amendments to this Regulation.

    - Even after the date of entry into force of the 03 series of amendments to this Regulation, Contracting Parties applying this Regulation may continue for national or regional purposes granting type approvals and extensions of type approvals to phase 1 (see paragraph 6.2.2. above) or the preceding series of amendments to this Regulation.

  - Transitional Provisions for Phase 3 (see paragraph 6.2.2. above)

    - As from 1 July 2024 for vehicle types other than N2, N3 and M3 and as from 1 July 2026 for vehicles types of category N2, N3 and M3, Contracting Parties applying this Regulation shall grant type-approvals only if the vehicle type to be approved meets the requirements of phase 3 (see paragraph 6.2.2. above) of this Regulation as amended by the 03 series of amendments.

    - Contracting Parties applying this Regulation shall not refuse to grant extensions of type approvals for existing types which have been granted according to phase 2 according to paragraph 6.2.2. above.

    - Until 30 June 2026 for vehicle types other than N2, N3 and M3 and until 30 June 2027 for vehicles types of category N2, N3 and M3, no Contracting Party applying this Regulation shall refuse national or regional type approval of a vehicle type-approved to phase 2 according to paragraph 6.2.2. above.

    - As from 1 July 2026 for vehicle types other than N2, N3 and M3 and as from 1 July 2027 for vehicles types of category N2, N3 and M3, Contracting Parties applying this Regulation shall not be obliged to accept for the purpose of national or regional type approval, a vehicle type approved to phase 2 according to paragraph 6.2.2. above.

    - Even after the date of entry into force of the 03 series of amendments to this Regulation, Contracting Parties applying this Regulation may continue for national or regional purposes granting type approvals and extensions of type approvals to phase 1 or phase 2 (see paragraph 6.2.2. above) or the preceding series of amendments to this Regulation.

  - Notwithstanding the transitional provisions above, Contracting Parties whose application of this Regulation comes into force after the date of entry into force of the most recent series of amendments are not obliged to accept type approvals which were granted in accordance with any of the preceding series of amendments to this Regulation.

  - "Until 30 June 2028, vehicles with a serial hybrid drive train which have a combustion engine with no mechanical coupling to the power train are excluded from the requirements of paragraph 6.2.3. above.”

  - Until 30 June 2019, Contracting Parties applying this Regulation can continue to grant approvals using test sites which comply with the specifications of the preceding series of amendments to this Regulation as an alternative to Annex 3, paragraph 2.1. of this Regulation.

  - Until 31 December 2023 for vehicle types of category N1 or for vehicle types of category M1 derived from N1 the limits according to paragraph 6.2.2. of the vehicle types of category N1 having a technically permissible maximum laden mass above 2.5 tons apply, if all the following specifications are met:

  - Having a technically permissible maximum laden mass of less than or equal to 2.5 tons;

  - An R-point height greater or equal to 800 mm from the ground;

  - An engine capacity exceeding 660cc but less than 1495cc;

  - An engine where the centre point of gravity of the engine is between 300 mm and 1,500 mm behind the front axle;

  - And having a rear axle drive.

11.9. 	Until 18 months after the date of entry into force of the Supplement 3, it shall not apply to extensions of existing approvals, originally granted prior to the date of entry into force of Supplement 3.

11.10.		Until May 1, 2020 Supplement 4 does not apply to existing approvals, originally granted prior to the date of entry into force of Supplement 4.

11.11.		Until May 1, 2020 Supplement 5 does not apply to existing approvals, originally granted prior to the date of entry into force of Supplement 5.

11.12.	Until 12 months after the date of entry into force of the Supplement 6, it shall not apply to extensions of existing approvals, originally granted prior to the date of entry into force of Supplement 6.

11.13. 		Supplement 7 (in particular, but not limited to Appendices 2 and 3 to Annex 3 and Annex 9) does not apply to existing type approvals, originally granted prior to the date of entry into force of Supplement 7.

11.14. 		Supplement 8 does not apply to existing type approvals, originally granted prior to the date of entry into force of Supplement 7.

11.15. 		From the entry into force of Supplement 8, ISO 10844:2021 shall be accepted for all approvals granted under this Regulation. Until five years from the entry into force of Supplement 8, ISO 10844:2014 shall be accepted for all approvals granted under this Regulation."

“11.16.	 Supplement 9 does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement 9.”

11.17. 	Supplement 10 does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement 10.

11.18. 	Supplement 11 does not apply to existing type approvals and their extensions, granted prior to the date of entry into force of Supplement 11.

11.19.	Until 24 months after the date of entry into force of the Supplement 11 to the 03 series of amendments to this UN Regulation, Contracting Parties applying this UN Regulation can continue to grant extensions to existing UN type approvals to the 03 series of amendments to this UN Regulation, granted prior to the date of entry into force of Supplement 11, without taking into account the provisions of Paragraph 6.2.3.3.1.

11.20. 		From the entry into force of Supplement 11, ISO 362-3:2022 shall be accepted for all approvals granted under this Regulation. Until five years from the entry into force of Supplement 11, ISO 362-3:2016 shall be accepted for all approvals granted under this Regulation.

12.		Names and addresses of Technical Services responsible for conducting approval tests and of Type Approval Authorities

12.1. 		The Contracting Parties to the 1958 Agreement applying this Regulation shall communicate to the United Nations Secretariat the names and addresses of the Technical Services responsible for conducting approval tests and of the Type Approval Authorities which grant approval and to which forms certifying approval or extension or refusal or withdrawal of approval, issued in other countries, are to be sent.

12.2. 	The Contracting Parties to the 1958 Agreement which apply this Regulation may designate laboratories of vehicle manufacturers as approved test laboratories for the purpose of tyre rolling sound measurements according to paragraph 3.4.2.

12.3. 		Where a Contracting Party to the 1958 Agreement applies paragraph 12.2. above, it may, if it so desires, be represented at the tests by one or more persons of its choice.

Annex 1

Communication

(maximum format: A4 (210 x 297 mm))

issued by:		Name of administration:

......................................

......................................

......................................

1

concerning:	Approval granted

Approval extended

Approval refused

Approval withdrawn

Production definitively discontinued

of a vehicle type with regard to its sound emission pursuant to Regulation No. 51

Approval No. ……………………………..	Extension No. ……………………………

### Section I

0.1.	Make (trade name of manufacturer):

0.2.	Type:

0.3.	Means of identification of type if marked on the vehicle:

0.3.1.	Location of that marking:

0.4.	Category of vehicle:

0.4.1.	Subcategory according to paragraph 6.2.2., the 2nd column of the table and the paragraphs 6.2.2.1. to 6.2.2.5.

0.4.1.	Vehicle equipped with an ADS: yes/no

0.5.	Company name and address of manufacturer:

0.6.	Names and Address(es) of assembly plant(s):

0.7.	Name and address of the manufacturer's representative (if any):

### Section II

1.	Additional information (where applicable): See Addendum (Appendix 1)

2.	Technical Service responsible for carrying out the tests:

3.	Date of test report:

4.	Number of test report:

5.	Remarks (if any): See Addendum (Appendix 1)

6.	Place:

7.	Date:

8.	Signature:

9.	Reasons for extensions:

Attachments:

Information package

Test report(s)

Annex 1 – Appendix 1

Addendum to the communication form No …

1.	Additional information

1.1.	Power plant

1.1.1.	Manufacturer of the engine:

1.1.2.	Manufacturer's engine code:

1.1.3	Rated maximum net power: ........... kW at ........... min-1
or maximum continuous rated power (electric motor) ........... kW

1.1.4.	Pressure charger(s), Make and Type:

1.1.5.	Air filter, Make and Type:

1.1.6.	Intake silencer(s), Make and Type:

1.1.7.	Exhaust Silencer(s), Make and Type:

1.1.8.	Catalyst(s), Make and Type:

1.1.9.	Particulate Trap(s), Make and Type:

1.1.10	System(s) generating EES installed: yes/no

1.1.10.1.	System(s) generating EES, Make and Type:

1.2.	Transmission

1.2.1.	Type (mechanical, hydraulic, electric, etc.):

1.3.	Non-engine devices designed to reduce noise:

2.	Test results

2.1.	Sound level of moving vehicle (Annex 3): 	 dB(A)

2.1.1.	Selected mode for tests of the vehicle in motion: ................................

2.2.	Sound level of stationary vehicle: .......... dB(A) at 	. min-1 in mode ………………

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

2.2.1.	Sound level of compressed air, service brake: 	 dB(A)

2.2.2.	Sound level of compressed air, parking brake: 	 dB(A)

2.2.3.	Sound level of compressed air, during the pressure regulator actuation: 
….. dB(A)

2.3.	Data to facilitate in-use compliance test of hybrid vehicles, where an internal combustion engine cannot operate when the vehicle is stationary

2.3.1.	Gear (i) or position of the gear selector chosen for the test:

2.3.2.	Position of the operating switch during measurement Lwot (i), (if switch is fitted)

2.3.3.	If applicable, pre-acceleration length lPA (Point of the accelerator depression in meter before line AA'). If the pre-acceleration length differs per gear, reporting per gear is required."

2.3.4.	Sound pressure level L wot (i) 	 dB(A)

2.4. 		Testing method information

2.4.1. 	Test method selected: Outdoor/Indoor

3.	Remarks:

Annex 1 – Appendix 2

Technical Information Document

0.	General

0.1.	Make (trade name of manufacturer):

0.2. 	Type: ……………………………………

0.3.	Means of identification of type, if marked on the vehicle:

0.3.1.	Location of that marking:

0.4.	Category of vehicle:

0.5.	Company name and address of manufacturer:

0.6.	Name and address of the manufacturer's representative (if any):

0.7.	Name(s) and address(es) of assembly plant(s):

1.	General construction characteristics of the vehicle

1.1.	Photographs and/or drawings of a representative vehicle:

1.2.	Number of axles and wheels:

1.2.1.	Powered axles (number, position, interconnection):

1.3.	Position and arrangement of the engine:

2.	Masses and dimensions (in kg and mm) (Refer to drawing where applicable):

2.1.	Range of vehicle dimensions (overall):

2.1.1.	For chassis without bodywork:

2.1.1.1.	Length:

2.1.1.2.	Width:

2.1.2.	For chassis with bodywork

2.1.2.1.	Length:

2.1.2.2.	Width:

2.2.	Mass in running order

(a) 	Minimum and maximum for each variant:

(b) 	Mass of each version (a matrix shall be provided):

2.3.	Technically permissible maximum laden mass stated by the manufacturer:,

3.	Power plant

3.1.	Manufacturer of the engine:

3.1.1.	Manufacturer's engine code (as marked on the engine, or other means of identification):

3.2.		Internal combustion engine

3.2.1.	Specific engine information

3.2.1.1.	Working principle: positive ignition/compression ignition, cycle four stroke/two stroke/rotary

3.2.1.2.	Number and arrangement of cylinders:

3.2.1.2.1.	Firing order:

3.2.1.3.		Engine capacity: …. cm3

3.2.1.4.		Rated maximum net power: …. kW at ….. min–1 (manufacturer's declared value)

3.2.2.		Fuel feed

3.2.2.1.		By fuel injection (compression ignition only): yes/no9

3.2.2.1.1.	Working principle: Direct injection/pre-chamber/swirl chamber9

3.2.2.1.2.	Governor

3.2.2.1.2.1.	Type:

3.2.2.1.2.2.	Speed at which Cut-off starts under load: …. min–1

3.2.2.2.		By fuel injection (positive ignition only): yes/no9

3.2.2.2.1.	Working principle: Intake manifold (single-/multi-point2)/direct injection/other (specify)9

3.2.3.		Intake system

3.2.3.1.	Air filter, drawings, or

3.2.3.1.1.	Make(s):

3.2.3.1.2.	Type(s):

3.2.3.2.	Intake silencer, drawings,

3.2.3.2.1.	Make(s):

3.2.3.2.2.	Type(s):

3.2.4.	Exhaust system

3.2.4.1.	Description and/or drawing of the exhaust system:

3.2.4.2.	Exhaust silencer(s):

Type, marking of exhaust silencer(s):

Where relevant for exterior noise, reducing measures in the engine compartment and on the engine:

3.2.4.3.	Location of the exhaust outlet:

3.2.4.4.	Exhaust silencer containing fibrous materials:

3.2.5.	Catalytic convertor: yes/no9

3.2.5.1.	Number of catalytic convertors and elements (provide the information below for each separate unit):

3.2.6. 	Pressure charger(s)

3.2.6.1. 	Make(s): ……………………………………

3.2.6.2. 	Type(s): …………………………………….

3.3. Electric motor (describe each type of electric motor separately)

3.3.1. Make: ...........................................................................................................

3.3.2. Type: ............................................................................................................

3.3.3. Rated maximum net power: …. kW

3.3.4. Operating voltage: …. V”

3.4.	Engine or motor combination:

3.4.1.	Hybrid electric vehicle: yes/no9

3.4.2.	Category of hybrid electric vehicle: off-vehicle charging/not off-vehicle charging:9

3.4.3.	Operating mode switch: with/without9

3.4.3.1.	Selectable modes

3.4.3.1.1.	Pure electric: yes/no9

3.4.3.1.2.	Pure fuel consuming: yes/no9

3.4.3.1.3.	Hybrid modes: yes/no9 (if yes, short description):

3.4.4.	Electric motor (describe each type of electric motor separately)

3.4.4.1.	Make:

3.4.4.2.	Type:

3.4.4.3.	Rated maximum net power: …… kW

4.	Transmission

4.1.	Type (mechanical, hydraulic, electric, etc.):

4.2.	Gear ratios

| Gear | Internal gearbox ratios (ratios of engine to gearbox output shaft revolutions) | Final drive ratio(s) (ratio of gearbox output shaft to driven wheel revolutions) | Total gear ratios |
| --- | --- | --- | --- |
| Maximum for CVT |  |  |  |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| … |  |  |  |
| Minimum for CVT |  |  |  |
| Reverse |  |  |  |

4.3.	Maximum vehicle design speed (in km/h):

5.	Suspension

5.1.	Tyres and wheels

5.1.1.	Tyre/wheel combination(s)

(a) 	For tyres indicate size designation, load-capacity index and speed category symbol;

(b) 	For wheels indicate rim size(s) and off-set(s).

5.1.2.	Upper and lower limits of rolling radii

5.1.2.1.	Axle 1:

5.1.2.2.	Axle 2:

5.1.2.3.	Axle 3:

5.1.2.4.	Axle 4:

etc.

6.	Bodywork

6.1.	Type of bodywork:

6.2.	Materials used and methods of construction:

7.	Miscellaneous

7.1.	Details of any non-engine devices designed to reduce noise (if not covered by other items):

7.2.	Details of any system(s) generating EES (e.g., number and position of actuators and active sound sources) including a description of a possibility to activate and deactivate the EES

Signed:

Position in company:

Date:

Annex 2

Arrangements of the approval mark

Model A

(See paragraph 5.4. of this Regulation)

a = 8 mm min.

The above approval mark affixed to a vehicle shows that the vehicle type concerned has, with regard to its noise emission, been approved in the Netherlands (E 4) pursuant to Regulation No. 51 under approval No. 032439.

The first two digits of the approval number indicate that Regulation No. 51 already included the 03 series of amendments when the approval was granted.

Model B

(See paragraph 5.5. of this Regulation)

a = 8 mm min.

The above approval mark affixed to a vehicle shows that the vehicle type concerned has been approved in the Netherlands (E 4) pursuant to Regulations Nos. 51 and 33. The approval numbers indicate that, at the dates when the respective approvals were granted, Regulation No. 51 included the 03 series of amendments while Regulation No. 33 was in its original form.

Annex 3

Methods and instruments for measuring the sound made by motor vehicles

1.	Measuring instruments

1.1.	Acoustic measurements

The apparatus used for measuring the sound level shall be a precision sound-level meter or equivalent measurement system meeting the requirements of Class 1 instruments (inclusive of the recommended windscreen, if used). These requirements are described in "IEC 61672-1:2002: Precision sound level meters", secondedition, of the International Electrotechnical Commission (IEC).

For indoor testing, when no general statement or conclusion can be made about conformance of the sound level meter by each channel of the array conformance (e.g. when pass-by sound level simulation algorithms do not compute the overall level but spectrum or temporal to recompose it), a simulated pass-by run shall be performed at a constant speed of 50 km/h while a constant tone signal is supplied to all channels of the arrays. The simulated A-weighted sound level is processed and the deviation from a reference tone signal shall be checked in accordance to IEC 61672-3.

Measurements shall be carried out using the "fast" response of the acoustic measurement instrument and the "A" weighting curve also described in "IEC 61672-1:2002". When using a system that includes a periodic monitoring of the A-weighted sound pressure level, a reading should be made at a time interval not greater than 30 ms.

The instruments shall be maintained and calibrated in accordance to the instructions of the instrument manufacturer.

1.2.	Verification and Adjustment of the Entire Acoustic Measurement System for Every Measurement Session

The Technical Service shall determine the duration of a measurement session, which shall be no more than one day.

At the beginning of every measurement session the entire measurement system shall be checked by means of a sound calibrator that fulfils the requirements for sound calibrators of at least precision Class 1 according to IEC 60942:2003, and – if the equipment allows it – adjusted to the reference values given by the calibrator.

At the end of every measurement session, the entire acoustic measurement system shall be re-checked by means of the same sound calibrator which was used for the check at the beginning. Without any further adjustment the difference between the readings at the beginning and at the end shall be less than or equal to 0.5 dB.

If the difference is greater than 0.5 dB, the results of the whole measurement session shall be discarded.

For indoor testing, the entire measurement system shall be checked at the beginning and at the end of a series of sessions.

Section 6 of ISO 362-3:2022 describes a qualified calibration method (i.e. electrical calibration). Such a method is recommended to be provided by the hardware supplier and, in that case, shall be implemented in the measurement software used. Simulation algorithms using sound source localization detection should deactivate that feature for these tests.

1.3.	Compliance with requirements

Compliance of the acoustic measurement instrumentation shall be verified by the existence of a valid certificate of compliance. These certificates shall be deemed to be valid if certification of compliance with the standards was conducted within the previous 12 month period for the sound calibration device and within the previous 24 month period for the instrumentation system. All compliance testing shall be conducted by a laboratory, which is authorized to perform calibrations traceable to the appropriate standards.

1.4.	Instrumentation for speed measurements

The engine speed shall be measured with instrumentation having an accuracy of ±2 per cent or better at the engine speeds required for the measurements being performed.

The road speed of the vehicle shall be measured with a continuous speed measuring device having an accuracy of at least ±0.5 km/h.

1.5.	Meteorological instrumentation

The meteorological instrumentation used to monitor the environmental conditions during the test shall include the following devices, which meet at least the given accuracy:

(a)	Temperature measuring device, ±1 °C;

(b)	Wind speed-measuring device, ±1.0 m/s;

(c)	Barometric pressure measuring device, ±5 hPa;

(d)	A relative humidity measuring device, ±5 per cent.

A monitoring of the wind speed is not mandated when tests are carried out in an indoor facility.

2.	Conditions of measurement

2.1.	Test Site and ambient conditions

The specifications for the test site provide the necessary acoustic environment to carry out the vehicle tests documented in this Regulation. Outdoor and indoor test environments that meet the specifications of this Regulation provide equivalent acoustic environments and produce results that are equally valid.

2.1.1. 		Test Site Outdoor

The surface of the test track and the dimensions of the test site shall be in accordance with ISO 10844:2021.

2.1.2. 		Test Site Indoor

Test Site Indoor requirements shall be as specified below.

- The test room dimensions are described in paragraph 7.2. of ISO 362-3:2022. All room dimensions may be adjusted to meet the specific application for the products being tested according to Annex 8, paragraph 4.

- The test facility shall meet the requirements of ISO 26101:2012 with the qualification criteria and measurement requirements appropriate to this test method as described in ISO 362-3:2022, paragraph 7.3.

- Condition of the floor is described in ISO 362-3:2022, paragraph 7.4.

- Cooling, ventilation, and exhaust gas management are described in ISO 362-3:2022, paragraph 7.5.

- Dynamometer requirements are described in ISO 362-3:2022, paragraph 8.

- Vehicle fixing system is described in ISO 362-3:2022, paragraph 9.3.

2.1.3. 		Ambient conditions

2.1.3.1.	Ambient condition indoor

2.1.3.1.1.	General

Meteorological conditions are specified to provide a range of normal operating temperatures and to prevent abnormal readings due to extreme environmental conditions.

The meteorological instrumentation shall deliver data representative for the test site and values of temperature, relative humidity, and barometric pressure shall be recorded during the measurement interval.

2.1.3.1.2.	Temperature

The measurements shall be made when the ambient air temperature is within the range from 5 °C to 40 °C.

The ambient temperature may of necessity be restricted to a narrower temperature range such that all key vehicle functionalities (e.g. start/stop, hybrid propulsion, battery propulsion, fuel-cell stack operation) are enabled according to manufacturer's specifications.

2.1.3.1.3.	Wind

n.a.

2.1.3.1.4.	Background noise

For indoor testing, background noise shall take into account noise emissions produced by the dynamometer rollers, ventilation systems, and facility exhaust gas systems.

2.1.3.2.	Ambient condition outdoor

2.1.3.2.1.	General

The surface of the site shall be free of powdery snow, tall grass, loose soil or cinders. There shall be no obstacle which could affect the sound field within the vicinity of the microphone and the sound source. The observer carrying out the measurements shall so position themself as not to affect the readings of the measuring instrument.

Measurements shall not be made under adverse weather conditions. It shall be ensured that the results are not affected by gusts of wind.

The meteorological instrumentation should be positioned adjacent to the test area at a height of 1.20 m ± 0.02 m.

A value representative of air and road surface temperature, wind speed and direction, relative humidity, and barometric pressure shall be recorded during the sound measurement interval.

2.1.3.2.2.	Temperature

The measurements shall be made when the ambient air temperature is within the range from 5 °C to 40 °C and the test surface temperature within the range from 5 °C to 60 °C.

Tests carried out on request of the manufacturer at air temperatures below 5° C shall be accepted as well.

The ambient temperature may of necessity be restricted to a narrower temperature range such that all key vehicle functionalities (e.g. start/stop, hybrid propulsion, battery propulsion, fuel-cell stack operation) are enabled according to manufacturer's specifications.

2.1.3.2.3.	Wind

The tests shall not be carried out if the wind speed, including gusts, at microphone height exceeds 5 m/s, during the sound measurement interval.

2.1.3.2.4.	Background noise

Any sound peak which appears to be unrelated to the characteristics of the general sound level of the vehicle shall be ignored in taking the readings.

The background noise shall be measured for duration of 10 seconds immediately before and after a series of vehicle tests. The measurements shall be made with the same microphones and microphone locations used during the test. The A-weighted maximum sound pressure level shall be reported.

The background noise (including any wind noise) shall be at least 10 dB(A) below the A-weighted sound pressure level produced by the vehicle under test. If the difference between the ambient noise and the measured sound is between 10 and 15 dB(A), in order to calculate the test results the appropriate correction shall be subtracted from the readings on the sound-level meter, as in the following table:

| Difference between ambient noise and sound to be measured dB(A) | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- |
| Correction dB(A) | 0.5 | 0.4 | 0.3 | 0.2 | 0.1 | 0.0 |

2.2.	Vehicle

2.2.1.		Vehicle selection

The vehicle shall be representative of vehicles to be put on the market as specified by the manufacturer in agreement with the Technical Service to fulfil the requirements of this Regulation.

Measurements shall be made without any trailer, except in the case of non-separable vehicles. At the request of the manufacturer, measurements may be made on vehicles with lift axle(s) in a raised position.

2.2.2.	Vehicle test mass mt and vehicle target mass mtarget

2.2.2.1.	Measurements shall be made on vehicles at the test mass mt specified according to the Table 2 below.

When testing indoors, the test mass, mt shall be utilized by the control system of the dyno roller.  Actual mass of the vehicle has no effect on results and it is permitted to load the vehicle as necessary to prevent slip between the tyres and the dyno rolls. To detect excessive slip, it is recommended to control the ratio of engine rotational speed and vehicle speed between the acceleration phase and the constant-speed status. To avoid slip, it is possible to increase the axle load.

2.2.2.2.	Target mass, mtarget, is used to denote the mass that N2 and N3 vehicles should be tested at. The actual test mass of the vehicle can be less due to limitations on vehicle and axle loading.

For vehicles in category N2, N3 equipped with an ADS, md = 0 if there is no driver. In this case, if a person is inside the vehicle to perform the test, the mass of this person shall be considered part of the extra load mxload.

Table 2: Specification of test mass for the various vehicle categories

| Vehicle category | Vehicle test mass |
| --- | --- |
| M1 | The test mass mt of the vehicle shall be between 0.9 mro ≤ mt ≤ 1.2 mro For vehicles in category X and Y, 0.9 mro ≤ mt- 75 kg ≤1.2 mro in case a person is inside the vehicle to perform the test For vehicles equipped with an ADS, other than those of category X and Y, 0.9 mro ≤ mt +75 kg ≤ 1.2 mro in case there are no occupants in the vehicle |
| N1 | The test mass mt of the vehicle shall be between 0.9 mro ≤ mt ≤ 1.2 mro For vehicle in category X and Y, 0.9 mro ≤ mt- 75 kg ≤1.2 mro in case a person is inside the vehicle to perform the test. For vehicles equipped with an ADS, other than those of category X and Y, 0.9 mro ≤ mt +75 kg ≤ 1.2 mro in case there are no occupants in the vehicle |
| N2, N3 | mtarget = 50 [kg/kW] x Pn [kW] Extra loading, mxload, to reach the target mass, mtarget, of the vehicle shall be placed above the rear axle(s). If the test mass mt is equal to the target mass mtarget, the test mass mt shall be 0.95 mtarget ≤ mt ≤ 1.05 mtarget The sum of the extra loading and the rear axle load in an unladen condition, mra load unladen, is limited to 75 per cent of the technically permissible maximum laden mass allowed for the rear axle, mac ra max. If the test mass mt is lower than the target mass mtarget, the test mass mt shall be achieved with a tolerance of ±5 per cent. If the centre of gravity of the extra loading cannot be aligned with the centre of the rear axle, the test mass, mt, of the vehicle shall not exceed the sum of the front axle in an unladen condition, mfa load unladen, and the rear axle load in an unladen condition, mra load unladen plus the extra loading, mxload, and the mass of the driver md except in the case of vehicles of category X or category Y where md = 0.. The test mass for vehicles with more than two axles shall be the same as for a two-axle vehicle. If the vehicle mass of a vehicle with more than two axles in an unladen condition, munladen, is greater than the test mass for the two-axle vehicle, then this vehicle shall be tested without extra loading. If the vehicle mass of a vehicle with two axles, munladen, is greater than the target mass, then this vehicle shall be tested without extra loading. |
| M2 (M ≤ 3,500 kg) | The test mass mt of the vehicle shall be between 0.9mro ≤ mt ≤ 1.2mro |
| Complete M2 (M > 3,500 kg), M3 | If the tests are carried out with a complete vehicle having a bodywork, mtarget = 50 [kg/kW] x Pn [kW] is calculated either in compliance with conditions above (see N2, N3 category) or the test mass mt of the vehicle shall be 0.9 mro ≤ mt ≤ 1.1 mro . |
| Incomplete M2 (M > 3,500 kg), M3 | If the tests are carried with an incomplete vehicle not having a bodywork, mtarget = 50 [kg/kW] x Pn [kW] is calculated either in compliance with conditions above (see N2, N3 category), or the test mass mt of the vehicle shall be 0.9 mro ≤ mt ≤ 1.1 mro . where mro = mchassisM2M3 + mxloadM2M3 |

2.2.2.3.	Calculation procedure to determine extra loading of N2 and N3 vehicles only

2.2.2.3.1.	Calculation of extra loading

The target mass, mtarget, (per kW rated power) for two-axle vehicles of category N2 and N3 is specified in the Table in paragraph 2.2.1: above.

mtarget = 50 [kg / kW] × Pn [kW]	(1)

To reach the required target mass, mtarget, for a vehicle being tested, the unladen vehicle, including the mass of the driver, md, shall be loaded with an extra mass, mxload, which shall be placed above the rear axle as given in Formula (8):

mtarget = munladen + md + mxload 	(2)

The target mass, mtarget, shall be achieved with a tolerance of ±5 per cent.

The vehicle mass of the test vehicle in the unladen condition, munladen, is calculated by measuring on a scale the unladen front axle load, mfa load unladen, and the unladen rear axle load, mra load unladen, as given in Formula (3):

munladen = mfa load unladen + mra load unladen 	(3)

By using Formulae (2) and (3), the extra loading, mxload, is calculated as given in Formulae (4) and (5):

mxload = mtarget – (md + munladen) 	(4)

mxload = mtarget − (md + mfa load unladen + mra load unladen) 	(5)

The sum of the extra loading, mxload, and the unladen rear axle load, mra load unladen, is limited to 75 per cent of the technically permissible maximum laden mass for the rear axle, mac ra max, as given in Formula (6):

0.75 mac ra max ≥ mxload + mra load unladen 	(6)

The mxload is limited according to Formula (7):

mxload ≤ 0.75 mac ra max − mra load unladen 	(7)

If the calculated extra loading, mxload, in Formula (5) fulfils Formula (7), then the extra loading is equal to Formula (5). The test mass, mt, of the vehicle is as calculated from Formula (8):

mt = mxload + md + mfa load unladen + mra load unladen 	(8)

In this case, the test mass of the vehicle is equal to the target mass

mt = mtarget	(9)

If the calculated extra loading, mxload, in Formula (5) does not fulfil Formula (7), but rather fulfils Formula (10)

mxload > 0.75 mac ra max − mra load unladen 	(10)

then, the extra loading, mxload, shall be as given by Formula (11):

mxload = 0.75 mac ra max − mra load unladen 	(11)

and the test mass, mt, of the vehicle shall be as given by Formula (12):

mt = 0.75 mac ra max + md + mfa load unladen 	(12)

In this case, the test mass of the vehicle is lower than the target mass

mt < mtarget 	(13)

The test mass, mt, shall be achieved with a tolerance of ± 5 per cent.

2.2.2.3.2.	Loading considerations if load cannot be aligned with the centre of rear axle

If the centre of gravity of the extra loading, mxload, cannot be aligned with the centre of the rear axle, the test mass of the vehicle, mt, shall not exceed the sum of the unladen front axle load, mfa load unladen, and the unladen rear axle load, mra load unladen, plus the extra loading, mxload, and the mass of the driver, md.

This means that if the actual front and rear axle loads are measured on a scale when the extra loading, mxload, is placed onto the vehicle and it is aligned with the centre of the rear axle, the test mass of the vehicle minus the mass of the driver is as given by Formula (14):

mt − md = mfa load laden + mra load laden 	(14)

Where:

mfa load laden = mfa load unladen	(15)

If the centre of gravity of the extra loading cannot be aligned with the centre of the rear axle, Formula (14) is still fulfilled, but

mfa load laden > mfa load unladen 	(16)

because the extra loading has partly distributed its mass to the front axle. In that case, it is not allowed to add more mass onto the rear axle to compensate for the mass moved to the front axle.

2.2.2.3.3.	Test mass for vehicles with more than two axles

If a vehicle with more than two axles is tested, then the test mass of this vehicle shall be the same as the test mass for the two-axle vehicle.

If the unladen vehicle mass of a vehicle with more than two axles is greater than the test mass for the two-axle vehicle, then this vehicle shall be tested without extra loading.

2.2.2.3.4.	Calculation of the test mass of a virtual vehicle with two axles:

When a vehicle family is not represented by a two-axle vehicle because it is physically not available, the vehicle family can be represented by a vehicle with more than two axles (vrf). In that case the test mass of a virtual two-axle vehicle (mt (2 axles virtual)) can be calculated in the following way:

For the calculation of the unladen vehicle mass of the virtual two-axle vehicle (munladen (2 axles virtual)), take from the vehicle with more than two axles (vrf) the measured unladen front axle load (mfa (vrf) load unladen) and the measured unladen rear axle load of that driven rear axle (mra (vrf) load unladen) which has the highest unladen load.

If the vehicle (vrf) has more than one front axle, take the one with the highest unladen front axle load.

munladen (2 axles virtual) = mfa (vrf) load unladen + mra (vrf) load unladen

mxload (2 axles virtual) = mtarget − (md + munladen (2 axles virtual))

Due to the requirement that the sum of the extra loading (mxload (2 axles virtual)) and the unladen rear axle load, mra (vrf) load unladen, is limited to 75 per cent of the technically permissible maximum laden mass allowed for the rear axle, mac ra max (2 axles virtual),  this value, mac ra max (2 axles virtual), has to be chosen in such a way that it represents the rear axle of the forecasted highest production-volume in the manufacturer's variation with a technically permissible maximum laden mass allowed for the rear axle (mac ra max (chosen) ) for the vehicle family as declared by the manufacturer.

mac ra max (4x2 virtual) = mac ra max (chosen)

If mxload (2 axles virtual) ≤ 0.75 mac ra max (chosen) − mra (vrf) load unladen

then

mt (2 axles virtual) = mxload (2 axles virtual) + md + mfa (vrf) load unladen + mra (vrf) load unladen

and

mt (2 axles virtual) = mtarget

If mxload (2 axles virtual) > 0.75 mac ra max (chosen) − mra (vrf) load unladen

then

mt (2 axles virtual) = 0.75 mac ra max (chosen) + md + mfa (vrf) load unladen

and

mt (2 axles virtual) < mtarget

The test mass of the vehicle with more than two axles representing the vehicle family is defined as followed:

mt (vrf) = mt (2 axles virtual)

and the extra loading is calculated as

mxload (vrf) = mt (2 axles virtual) – md – munladen (vrf)

2.2.2.4.		At the applicant's request the vehicle of a category M2, M3, N2 or N3 is deemed representative of its completed type if the tests are carried out to an incomplete vehicle not having a bodywork. In the test of an incomplete vehicle all relevant soundproofing materials, panels and noise reduction components and systems shall be fitted on the vehicle as designed by the manufacturer except a part of bodywork which is built in a later stage.

No new test shall be required due to fitting of a supplement fuel tank or re-location of the original fuel tank on condition that other parts or structures of the vehicle apparently affecting sound emissions have not been altered.

2.2.3.	Preparation of the vehicle before testing

2.2.3.1.	General

The vehicle shall be equipped as specified by the vehicle manufacturer. Before the measurements are started, the vehicle shall be brought to its normal operating conditions, which means that essential components for the operation of the vehicle are at their nominal temperatures as specified by the manufacturer. This applies especially, but is not limited to

-	the cooling water (if applicable);

-	oil temperature (if applicable).

2.2.3.2.	Battery state of charge

If so equipped, propulsion batteries shall have a state-of-charge sufficiently high to enable all key functionalities according to the specifications of the vehicle manufacturer. Propulsion batteries shall be within their component temperature window to enable all key functionalities. Any other type of rechargeable energy storage system shall be ready to operate during the test.

2.2.3.3.	Systems generating EES

Any system generating EES, subject to paragraph 6.2.3. and Annex 7 of this Regulation,, shall operate as foreseen by the vehicle manufacturer and not be interfered with during the measurements.

2.2.3.4.	Tyres

2.2.3.4.1.	Tyre Selection

The tyres and rims to be used for the test shall be representative for the vehicle and shall be selected by the vehicle manufacturer and recorded in Addendum to the Communication form (Annex 1, Appendix 1). They shall correspond to one of the tyre sizes designated for the vehicle as original equipment. The tyre is or will be commercially available on the market at the same time as the vehicle. The tyres shall be inflated to the pressure recommended by the vehicle manufacturer for the test mass of the vehicle. The tyres shall have at least 1.6 mm tread depth.

When performing indoor testing, tyre/road sound is evaluated independently on the test track with the tyres to be used, according to this paragraph. Propulsion sound is independently evaluated on the dynamometer using tyres and other sound control measures to produce tyre/road sound which does not influence the measurement result.

2.2.3.4.2.	Tyre conditioning

Tyres with special fitment requirements, such as asymmetric or directional design, shall also be mounted in accordance with these requirements.

Before testing, tyres shall be conditioned (broken-in). Tyre break-in shall be representative to about 100 km of normal on-road operation. Tyres with special fitment requirements shall be broken-in in accordance with these requirementsThe tyres fitted to the test vehicle shall rotate in the same direction as when they were broken-in.

Test tyres shall be warmed-up immediately prior to testing for at least 10min in the range of the test speed, with moderate lateral & longitudinal acceleration. The lateral acceleration shall be selected in a way to avoid excessive tire tread wear effects.

If test tyres have operational temperature limits that do not cover the full temperature range of this regulation, the tyres shall be conditioned to their operational temperature regarding the provisions described in paragraph 2.2.3.1.”

2.2.3.5.	If the vehicle is fitted with more than two-wheel drive, it shall be tested in the drive which is intended for normal road use.

2.2.3.6.	If the vehicle is fitted with fan(s) having an automatic actuating mechanism, this system shall not be interfered with during the measurements.

2.2.3.7.	If the vehicle is equipped with an exhaust system containing fibrous materials, it might be necessary to carry out a conditioning test prior to testing. The provisions of Annex 4, paragraph 1. in conjunction with the flowchart (Figure 2) of the appendix to Annex 4 shall be followed .

2.2.3.8.	Suspension Trim Level

If fitted, the trim level of a height adjustable suspension shall be set to its normal level for on-road operation as specified by the vehicle manufacturer.

3.	Methods of testing

Outdoor tests shall be performed according to paragraph 3.1.

Indoor tests shall be performed according to paragraph 3.1. using the specifications of ISO 362-3:2022, variant A. For indoor application, the manufacturer shall provide to the technical service, documentation according to Annex 8, paragraph 1.  Variant A is a combination of indoor testing (power train sound) and outdoor testing (tyre/road sound).

3.1.	Measurement of sound of vehicles in motion

3.1.1.	General conditions of test

For outdoor testing, two lines, AA' and BB', parallel to line PP' and situated respectively 10 m ± 0.05m forward and 10 m ± 0.05m rearward of line PP’ shall be marked out on the test runway.

For indoor testing, the virtual line AA' indicates the beginning of the test track, PP' indicates the virtual position of the two pass-by microphones, and BB' indicates the end of the test track. The simulated vehicle speed at AA', vAA', or vehicle speed at PP', vPP', is defined by the roller speed when the reference point of the vehicle passes the virtual line AA' or PP', respectively. The simulated vehicle speed at BB', vBB', is defined when the rear of the vehicle passes the virtual line BB'.

At least four measurements shall be made on each side of the vehicle and for each gear. Preliminary measurements may be made for adjustment purposes, but shall be disregarded.

For outdoor testing the microphone shall be located on both sides of the pathway at a distance of 7.5 m ± 0.05 m from the reference line CC' of the track and 1.2 m ± 0.02 m above the ground. For indoor testing microphones on one side of the roller bench can be used.

The reference axis for free field conditions (see IEC 61672-1:2002) shall be horizontal and directed perpendicularly towards the path of the vehicle line CC'.

3.1.2.	Specific test conditions for vehicles

3.1.2.1.	Vehicles of category M1, N1 and M2 ≤ 3,500 kg technically permissible maximum laden mass:

The path of the centreline of the vehicle shall follow line CC' as closely as possible throughout the entire test, from the approach to line AA' until the rear of the vehicle passes line BB' +20 m.

If the vehicle is fitted with an auxiliary manual transmission or a multi-gear axle, the position used for normal urban driving shall be used. In all cases, the gear ratios for slow movements, parking or braking shall be excluded.

The test mass of the vehicle shall be according to the table of paragraph 2.2.1.

The test speed vtest is 50 km/h  1 km/h. The test speed shall be reached, when the reference point is at line PP'.

If the test speed is modified according to paragraph 3.1.2.1.4.1.(d) of Annex 3 to this Regulation, the modified test speed shall be used for both the acceleration and constant speed test.

3.1.2.1.1.	Power to mass ratio index (PMR)

PMR is defined as follows:

PMR = (Pn / mro) * 1000 kg/kW, where Pn is measured in kW and defined according to paragraph 2.8. of the main body and mro is measured in kg and defined according to paragraph 2.4. of the main body.

The PMR with no dimension is used for the calculation of acceleration.

3.1.2.1.2.	Calculation of acceleration

Acceleration calculations are applicable to M1, N1 and M2 ≤ 3,500 kg technically permissible maximum laden mass categories only.

All accelerations are calculated using different speeds of the vehicle on the test track.  The formulae given are used for the calculation of awot i, awot i+1 and awot test. The speed either at AA' or PP' is defined as the vehicle speed when the reference point passes AA' (vAA') or PP' (vPP'). The speed at BB' is defined when the rear of the vehicle passes BB' (vBB'). The method used for determination of the acceleration shall be indicated in the test report.

Due to the definition of the reference point for the vehicle the length of the vehicle (lveh) is considered differently in the formula below. If the reference point is in the front of the vehicle, then l = lveh, mid: l = ½ lveh and rear: l = 0.

At the choice of the vehicle manufacturer, front engine vehicles may use 
l = 5 m, and mid-engine vehicles may use l = 2.5 m.

3.1.2.1.2.1.	Calculation procedure for vehicles with manual transmission, automatic transmission, adaptive transmissions and transmissions with continuous variable transmissions (CVT's) tested with locked gear ratios:

awot test = ((vBB'/3.6)² - (vAA'/3.6)²) / (2*(20+l))

awot test used in the determination of gear selection shall be the average of the four awot test, i during each valid measurement run.

Pre-acceleration may be used. The point of depressing the accelerator before line AA' shall be reported in the Addendum to the Communication form (Annex 1, Appendix 1).

3.1.2.1.2.2.	Calculation procedure for vehicles with automatic transmissions, adaptive transmissions and CVT's tested with non-locked gear ratios:

awot test used in the determination of gear selection shall be the average of the four awot test, i during each valid measurement run.

If devices or measures described in paragraph 3.1.2.1.4.2. can be used to control transmission operation for the purpose of achieving test requirements, calculate awot test using the equation:

awot test = ((vBB'/3.6)² - (vAA'/3.6)²) / (2*(20+l))

Pre-acceleration may be used.

If devices or measures described in paragraph 3.1.2.1.4.2. are not used, calculate awot test using the equation:

awot_testPP-BB = ((vBB'/3.6)² - (vPP'/3.6)²) / (2*(10+l))

Pre-acceleration shall not be used.

The location of depressing the accelerator shall be where the reference point of the vehicle passes line AA'.

3.1.2.1.2.3.	Target acceleration

The target acceleration aurban defines the typical acceleration in urban traffic and is derived from statistical investigations. This function depends on the PMR of a vehicle.

The target acceleration aurban is defined by:

aurban = 0.63 * log10 (PMR) - 0.09

3.1.2.1.2.4.	Reference acceleration

The reference acceleration awot ref defines the required acceleration during the acceleration test on the test track. It is a function depending on the power-to-mass ratio of a vehicle. That function is different for specific vehicle categories.

The reference acceleration awot ref is defined by:

awot ref = 1.59 * log10 (PMR) -1.41	for PMR ≥ 25

awot ref = a urban = 0.63 * log10 (PMR) - 0.09	for PMR < 25

3.1.2.1.3.	Partial power factor kP

The partial power factor kP (see paragraph 3.1.3.4.1.2.) is used for the weighted combination of the test results of the acceleration test and the constant speed test for vehicles of category M1 and N1 and M2 < 3,500 kg technically permissible maximum laden mass

In cases other than a single gear test, awot ref shall be used instead of awot test (see paragraph 3.1.3.4.1.2.).

3.1.2.1.4.	Gear ratio selection

The selection of gear ratios for the test depends on their specific acceleration potential awot under full throttle condition, according to the reference acceleration awot ref required for the full throttle acceleration test.

If the vehicle allows different transmission setups like automatic or manual gear selection and/or has different software programs or modes (e.g. sporty, winter, adaptive) leading to valid accelerations, the vehicle manufacturer shall prove to the satisfaction of the Technical Service, that the vehicle is tested in the mode which achieves an acceleration being closest to awot ref.

The vehicle transmission, gear, or gear ratio may be controlled by electronic or mechanical measures to avoid the activation of a kick-down function.”

Appendix 1, Figure 4a to Figure 4f, give gear selection criteria and test run criteria for categories M1 and M2 having a technically permissible maximum laden mass not exceeding 3.500 kg and for category N1, in a flowchart form as an aid to test operation.

3.1.2.1.4.1.	Vehicles with manual transmission, automatic transmissions, adaptive transmissions or CVTs tested with locked gear ratios

The following conditions for selection of gear ratios are possible:

(a)	If one specific gear ratio gives an acceleration in a tolerance band of 5 per cent of the reference acceleration awot ref, not exceeding 2.0 m/s2, test with that gear ratio.

(b)	If none of the gear ratios give the required acceleration, then choose a gear ratio i, with an acceleration higher and a gear ratio i+1, with an	acceleration lower than the reference acceleration. If the acceleration value in gear ratio i does not exceed 2.0 m/s2, use both gear ratios for the test. The weighting ratio in relation to the reference acceleration awot ref is calculated by:

k = (awot ref – awot (i+1))/(awot (i) – awot (i+1))

(c)	If the acceleration value of gear ratio i exceeds 2.0 m/s2, the first gear ratio shall be used that gives an acceleration below 2.0 m/s2 unless gear ratio i+1 (or i+2, or i+3 or …) provides acceleration less than aurban. In this case, two gears, i and i+1 (or i+2, or i+3 or …) shall be used, including the gear i with acceleration exceeding 2.0 m/s2. In other cases, no other gear shall be used. The achieved acceleration awot test during the test shall be used for the calculation of the part power factor kP instead of awot ref.

(d)	If maximum engine speed nMAX is exceeded in a gear ratio i before the vehicle	passes BB' the next higher gear i+1 shall be used. If the next higher gear i+1 results in an acceleration below aurban, the vehicle test speed, vtest, in the gear ratio i shall be reduced by 2.5 km/h and the gear ratio selection shall proceed as specified by the options given in this paragraph. In no case shall the vehicle test speed be reduced below 40 km/h.

If the maximum engine speed nMAX is exceeded in gear ratio i before the vehicle passes BB' and the vehicle test speed is equal to 40 km/h, the higher gear ratio i+1 is allowed even if awot test does not exceed aurban.

The vehicle test speed in the higher gear ratio i+1 shall be 50 km/h.

The maximum engine speed nMAX is given by the formula below:

nMAX=1.56 ×PMR-0.227×S, but not more than 80% of S.

(e) 	If no gear ratio is available with an acceleration below 2.0 m/s², the manufacturer shall, if possible take measures to avoid an acceleration value awot test greater than 2.0 m/s².

Table 1 in Appendix to Annex 3 provides examples for valid measures to control the downshift of gears or to avoid accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

3.1.2.1.4.2.	Vehicles with automatic transmission, adaptive transmissions and CVTs tested with non-locked gear ratios:

Manufacturers may take measures to lock discrete gear ratios by electronic or mechanical measures and follow the gear selection previsions of paragraph 3.1.2.1.4.1. above. If so selected, this shall be stated in the test report.

Otherwise, the gear selector position for full automatic operation shall be used.

The acceleration value awot test shall be calculated as defined in paragraph 3.1.2.1.2.2.

The test may then include a gear change to a lower range and a higher acceleration or a higher engine speed. A gear change to a higher range and a lower acceleration is not allowed. A gear shifting to a gear ratio which is not representative for urban traffic shall be avoided.

Therefore, it is permitted to establish and use electronic or mechanical devices, including alternate gear selector positions, to avoid :

(a)	accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. The achieved acceleration awot_test shall be greater or equal to aurban.

(b)	a test engine speed exceeding nMAX (see Appendix 1, figure 4f).

(i)	Therefore, the vehicle test speed vtest may be reduced in steps by 2.5 km/h. In no case the vehicle test speed shall be reduced to a vehicle speed below 40 km/h, or

(ii)		The engine load is reduced to avoid a downshift to a gear ratio where nMAX is exceeded.

If possible, the manufacturer shall take measures to avoid an acceleration value awot test greater than 2.0 m/s2.

If possible, the manufacturer shall take measures to avoid an engine speed higher than nMAX.

Table 1 in Appendix to Annex 3 provides examples for valid measures to  enable a test condition within the above specified boundaries. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

The achieved acceleration awot test is then used for the calculation of the partial power factor kp (see paragraph 3.1.2.1.3.) instead awot ref.

3.1.2.1.4.3.	Vehicles with only one gear ratio, like but not limited to Battery Electric Vehicles (BEV) and Fuel Cell Vehicles (FCV)

The gear selector position for forward driving shall be used. The acceleration value awot test shall be calculated as defined in paragraph 3.1.2.1.2.1.

The achieved acceleration awot test, but no more than 2.0 m/s², shall be greater or equal to aurban  is then used for the calculation of the partial power factor kp (see paragraph 3.1.2.1.3.) instead of awot ref.

If possible, the manufacturer shall take measures to avoid an acceleration value awot test greater than 2.0 m/s².

Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to avoid accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

3.1.2.1.5.	Acceleration test

The manufacturer shall define the position of the reference point in front of line AA' of fully depressing the accelerator. The accelerator shall be fully depressed (as rapidly as is practicable) when the reference point of the vehicle reaches the defined point. The accelerator shall be kept in this depressed condition until the rear of the vehicle reaches line BB'. The accelerator shall then be released as rapidly as possible. The measurement reading shall not end before the rear of the vehicle is 20 m behind the BB' line. The point of fully depressing the accelerator shall be reported in Addendum to the Communication form (Annex 1, Appendix 1). The Technical Service shall have the possibility of pretesting.

If the vehicle length was set according to the provisions of 3.1.2.1.2. the accelerator shall be kept in the depressed condition until the reference point reaches BB’ + 5 m for front engine vehicles, and BB’ + 2.5 m for mid-engine vehicles.

In the case of articulated vehicles consisting of two non-separable units regarded as a single vehicle, the semi-trailer shall be disregarded in determining when line BB' is crossed.

Specific provisions for the testing of vehicles equipped with an ADS are provided in Annex 10.

3.1.2.1.6.	Constant speed test

The constant speed test shall be carried out with the same gear(s) specified for the acceleration test and a constant speed of 50 km/h with a tolerance of ±1 km/h between AA' and BB', or if applicable at the speed determined for the acceleration test according 3.1.2.1.4.1. (d) or 3.1.2.1.4.2. with a tolerance of ±1 km/h between AA' and BB'. During the constant speed test the accelerator shall be positioned to maintain a constant speed between AA' and BB' as specified. If the gear is locked for the acceleration test, the same gear shall be locked for the constant speed test.

The constant speed test is not required for vehicles with a PMR < 25.

Specific provisions for the testing of vehicles equipped with an ADS are provided in Annex 10.

3.1.2.2.	Vehicles of categories M2 > 3,500 kg technically permissible maximum laden mass, M3, N2, N3:

The path of the centreline of the vehicle shall follow line CC' as closely as possible throughout the entire test, from the approach to line AA' until the rear of the vehicle passes line BB'. The test shall be conducted without a trailer or semi-trailer. If a trailer is not readily separable from the towing vehicle the trailer shall be ignored when considering the crossing of line BB'. If the vehicle incorporates equipment such as a concrete mixer, a compressor, etc., this equipment shall not be in operation during the test. The test mass of the vehicle shall be according to the table of paragraph 2.2.1. of Annex 3 to this Regulation.

The value of nBB' and vBB' used in the determination of gear and vehicle speed selection shall be the average of the four nBB', j and vBB', j values during each valid measurement run.

The value of nBB' shall be reported to a precision of 10 revolutions per minute. The reported nBB' shall be used in all subsequent calculation.

The value of vBB' shall be reported to the first digit after the decimal (xx,x).  The reported vBB'' shall be used in all subsequent calculation.

Target conditions of category M2 > 3,500 kg technically permissible maximum laden mass, N2:

When the reference point passes line BB', the engine speed nBB' shall be between 70 and 74 per cent of speed S, at which the engine develops its rated maximum net power, and the vehicle speed shall be 35 km/h ± 5 km/h. Between line AA' and line BB' a stable acceleration condition according to definition 2.26.1. shall be ensured.

Target conditions of category M3, N3:

When the reference point passes line BB', the engine speed nBB' shall be between 85 and 89 per cent of speed S, at which the engine develops its rated maximum net power, and the vehicle speed shall be 35 km/h ± 5 km/h. Between line AA' and line BB' a stable acceleration condition according to definition 2.26.1. shall be ensured.

3.1.2.2.1.	Gear ratio selection

It is the responsibility of the manufacturer to determine the correct manner of testing to achieve the required conditions.

The vehicle transmission, gear, or gear ratio, shall be chosen to be able to fulfil the target conditions according to paragraphs 3.1.2.2.1.1. or 3.1.2.2.1.2. of Annex 3 to this Regulation. The vehicle transmission, gear, or gear ratio may be controlled by electronic or mechanical measures including exclusion of kick-down function.

Appendix 3, Figure 5a to Figure 5d, give gear selection criteria and test run criteria for categories M2 having a technically permissible maximum laden mass exceeding 3,500 kg, and for category N2, M3 and N3, in a flowchart as an aid to test operation.

3.1.2.2.1.1.	Manual transmission, automatic transmissions, adaptive transmissions or transmissions with continuously variable gear ratios (CVTs) tested with locked gear ratios

Stable acceleration condition according to definition 2.26.1. shall be ensured. The gear choice is determined by the target conditions.

The following conditions for fulfilling the target conditions in paragraph 3.1.2.2. of Annex 3 to this Regulation are possible:

(a) 	If one gear choice fulfils both target conditions for the rotational engine speed ntarget BB’ and for the vehicle speed vtarget BB’, test with that gear.

(b)	If more than one gear choice fulfils both target conditions for the rotational engine speed ntarget BB’ and for the vehicle speed vtarget BB’, test in gear i that gives velocity vBB’ gear i closest to 35 km/h.

(c)	If two gear choices fulfil both target conditions for the rotational engine speed ntarget BB’ and for the vehicle speed vtarget BB’, and fulfil the following condition.

(vtarget BB’ – vBB’ gear i) = (vBB’ gear i+1 – vtarget BB’)

then both gears are taken for further calculation of Lurban.

(d)	If one gear choice fulfils the target condition for the rotational engine speed ntarget BB’ but not the target condition for the vehicle speed 
vtarget BB’, use two gears, gearx and geary. The target conditions for the vehicle speed for these two gears are as follows:

gearx

25 km/h ≤ vBB’x ≤ 30 km/h

and

geary

40 km/h ≤ vBB’y ≤ 45 km/h

Both gears, gearx and geary shall fulfil the target rotational engine speed ntarget BB’. Both gears shall be used for further calculation of Lurban.

If only one of the gears fulfils the target rotational engine speed, 
ntarget BB’, test with that gear. This gear shall be used for further calculation of Lurban.

(e)	If none of the two gears fulfils the target rotational engine speed 
ntarget BB’ under condition d) then condition f) shall be chosen.

(f)	If no gear choice fulfils the target rotational engine speed choose the gear that fulfils the target vehicle velocity vtarget BB’ and is closest to the target rotational engine speed ntarget BB’, but not higher than ntarget BB’.

vBB’ gear i  =  vtarget BB’

nBB’ gear i  ≤  ntarget BB’

A stable acceleration condition according to definition 2.26.1. shall be ensured. If such a stable acceleration cannot be ensured in a gear, this gear shall be disregarded. In all conditions, the rated engine speed shall not be exceeded while the reference point of the vehicle is in the measurement zone. If the rated engine speed is exceeded within the measurement zone, this gear shall be disregarded.

3.1.2.2.1.2.	Automatic transmission, adaptive transmissions, and transmissions with variable gear ratio tested with non-locked gear ratios

The gear selector position for full automatic operation shall be used.

The test may then include a gear change to a lower range and a higher acceleration. A gear change to a higher range and a lower acceleration is not allowed. In any case a gear change to a gear ratio that is typically not used at the specified condition as defined by the manufacturer in urban traffic shall be avoided.

Therefore, it is permitted to establish and use electronic or mechanical devices, including alternative gear selector positions, to prevent a downshift to a gear ratio that is typically not used at the specified test condition as defined by the manufacturer in urban traffic.

Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to control the downshift of gears. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

The following conditions for fulfilling the target conditions in paragraph 3.1.2.2. of Annex 3 to this Regulation are possible:

(a) 	If the choice of the gear selector position fulfils both target conditions for the rotational engine speed ntarget BB’ and for the vehicle speed 
vtarget BB’, test with the gear selector in that position.

(b) 	If the choice of the gear selector position fulfils the target condition for the rotational engine speed ntarget BB’ but not the target condition for the vehicle speed vtarget BB’, change the target condition for the vehicle speed to two vehicle target speeds as follows:

Define vBB’1 as

25 km/h ≤ vBB’1 ≤ 35 km/h

and

Define vBB’2 as

35 km/h ≤ vBB’2 ≤ 45 km/h.

Conduct two tests, one with vBB’1 and one with vBB’2.

Both test conditions are used for further calculation of Lurban.

(c)	If under condition b) the target rotational engine speed ntarget BB’ cannot be fulfilled, condition d) shall be chosen.

(d)	If the choice of the gear selector position cannot fulfil the target condition for the rotational engine speed ntarget BB’ but the target condition for the vehicle speed vtarget BB’, change the target condition for the vehicle speed to two vehicle target speeds as follows:

Define vBB’1 as

25 km/h ≤ vBB’1 ≤ 30 km/h

and

Define vBB’2 as

40 km/h ≤ vBB’2 ≤ 45 km/h.

Conduct two tests, one with vBB’1 and one with vBB’2.

Use the test where nBB’ is closest to the target rotational engine speed ntarget BB’ but not higher than ntarget BB’.

nBB’ i ≤ ntarget BB’ for i = 1, 2

If the vehicle cannot fulfil the condition:

nBB’ i ≤ ntarget BB’ for i = 1, 2

condition (e) shall be used.

(e)	If the choice of the gear selector position cannot fulfil the target conditions for the rotational engine speed ntarget BB’ and the target condition for the vehicle speed vtarget BB’, change the target condition for the vehicle speed to the following:

vBB’ = vtarget BB’ + 5 km/h

Conduct the test with that vehicle speed vBB’ where nBB’ is closest to the target rotational engine speed ntarget BB’. A gear change to a higher range and a lower acceleration is allowed after the vehicle passes line PP’.

(f)	If the vehicle includes a transmission design that provides only a single gear selection (D) that limits engine speed during the test, the vehicle shall be tested using only the target vehicle speed vtarget BB’.

3.1.2.2.1.3.	Powertrain with no combustion engine rotational engine speed available

Vehicles with a powertrain where no combustion engine rotational engine speed is available shall fulfil only the target condition for the vehicle speed vtarget BB’

The following conditions for fulfilling the target condition vtarget BB’ in paragraph 3.1.2.2. of Annex 3 to this Regulation are possible.

(a)	If no rotational engine speed is available, it is necessary to fulfil only the target vehicle speed vtarget BB’.

(b)	If no rotational engine speed is available and the target vehicle speed vtarget BB’ cannot be fulfilled, two test conditions shall be conducted as follows:

vBB’1 for the first test condition is defined as

25 km/h ≤ vBB’1 ≤ 35 km/h

and

vBB’2 for the second test condition is defined as

35 km/h ≤ vBB’2 ≤ 45 km/h

Both test conditions are used for further calculation of Lurban.

(c)	If no rotational engine speed is available and the target vehicle speed vtarget BB’ and vBB’1 defined as

25 km/h ≤ vBB’1 ≤ 35 km/h

cannot be fulfilled, it is necessary to conduct, only one test with vBB’2 where vBB’2 is defined as

35 km/h ≤ vBB’2 ≤ 45 km/h

The test condition for vBB’2 is taken for further calculation of Lurban.

3.1.2.2.2.	Acceleration test

When the reference point of the vehicle reaches the line AA' the accelerator shall be fully depressed (without operating the automatic downshift to a lower range than normally used in urban driving) and held fully engaged until the reference point reaches BB’ + 5 m. The acceleration control unit can then be released on request of the manufacturer.

In the case of articulated vehicles consisting of two non-separable units regarded as a single vehicle, the semi-trailer shall be disregarded in determining when line BB' is crossed.

Specific provisions for the testing of vehicles equipped with an ADS are provided in Annex 10.

3.1.3.	Interpretation of results

3.1.3.1.	Measurement readings for outdoor tests

For vehicles of categories M1 and N1, and for vehicles of category M2 having a maximum authorized mass not exceeding 3,500 kg, the maximum A-weighted sound pressure level indicated during each passage of the vehicle according to paragraphs 3.1.2.1.5. and 3.1.2.1.6. shall be rounded to the first significant digit after the decimal place (e.g. XX.X).

For vehicles of category M2 having a maximum authorized mass exceeding 3,500 kg and for vehicles of categories M3, N2, and N3 the maximum A-weighted sound pressure level indicated during each passage of the reference point of the vehicle between line AA’ and line BB’ + 5 m shall be rounded, to the first significant digit after the decimal place (e.g. XX.X).

3.1.3.2.	Measurement readings for indoor tests

The pass-by sound of a vehicle is determined by energetical addition of the power train sound measured in an indoor facility according to paragraph Annex 8, paragraph 2. of this regulation and the separately determined tyre/road sound measured on an outdoor test track according to Annex 8, paragraph 2.3. of this Regulation.

3.1.3.3.	Validation of individual test runs

If a sound peak obviously out of character with the general sound pressure level is observed, the measurement shall be discarded. At least four measurements for each test condition shall be made on each side of the vehicle and for each gear ratio. For outdoor tests left and right shall be measured simultaneously, for indoor tests simultaneous measurement is recommended if it is possible. The first four valid consecutive measurement results, within 2 dB(A), allowing for the deletion of non-valid results (see paragraph 2.1.), shall be used for the further calculations below.

3.1.3.4.	Calculation of results

3.1.3.4.1.	Calculation for vehicles of category M1 and N1, and for vehicles of category M2 having a maximum authorized mass not exceeding 3,500 kg

3.1.3.4.1.1.	Each valid test run of the acceleration and – if applicable – of the constant speed tests per vehicle side and per gear ratio shall be subjected to a temperature and if applicable a test track correction according to Appendix 2 to Annex 3.

3.1.3.4.1.2.	Per gear, test condition (acceleration and constant speed) and vehicle side the four valid and corrected test results shall be averaged and mathematically rounded to the first significant digit after the decimal place.

All further calculations to derive Lurban shall be done separately for the left and right vehicle side. The final value Lurban mathematically rounded to the nearest integer shall be the higher value of the two sides.

The speed measurements at AA', BB', and PP' used for reporting and further calculations shall be rounded to the first significant digit after the decimal place.

The calculated acceleration aacc test used for reporting and further calculations shall be rounded to the second significant digit after the decimal place.

The calculated interim values for the acceleration test and the constant speed test are given by:

Lwot rep = Lwot (i+n) + k * (Lwot(i) – Lwot (i+n))

Lcrs rep = Lcrs(i+n) + k * (Lcrs (i) – Lcrs (i+n))

where 	k = (awot ref – awot (i+n))/(awot (i) – awot (i+n)) 
	with n as determined by paragraph 3.1.2.1.4.1.

In the case of a single gear ratio test inclusive the non-locked condition according to paragraphs 3.1.2.1.4.2. and 3.1.2.1.4.3. the interim values Lacc rep and Lcrs rep are the averaged test results of each test condition (acceleration and constant speed).

The final result is calculated by combining Lacc rep and Lcrs rep. The equation is:

Lurban = Lwot rep – kP * (Lwot rep – Lcrs rep)

The partial power factor kP is given for urban driving. In cases other than a single gear test, kP is calculated by:

kP = 1 – (aurban / awot ref)

If only one gear was specified for the test, kP is given by:

kP = 1 – (aurban / awot test)

In cases where awot test is less than aurban:

kP = 0

In cases where the PMR of the vehicle is lower than 25, the final result Lurban is the result of the acceleration test:

Lurban = Lwot rep

In cases where Lwot,rep is less than Lcrs,rep:

kP=1

In cases where Lwot,rep is less than Lcrs,rep the final result Lurban is the result of the cruise test:

Lurban= Lcrs,rep

3.1.3.4.2.	Calculation for vehicles of category M2 having a maximum authorized mass exceeding 3,500 kg and for vehicles of categories M3, N2, and N3

For each gear and vehicle side, the valid test runs shall be averaged separately, rounded to the first decimal place, and be reported as interim results.

All further calculations to derive Lurban shall be done separately for the left and right vehicle side. The final value Lurban to be reported as the test result mathematically rounded to the nearest integer shall be the higher value of the two sides.

The speed measurements at line BB' shall be noted and used in calculations to the first significant digit after the decimal place.

The engine speed measurements (if applicable) at line BB' shall be noted and used in calculations to the full integer.

In the case of a single gear test, inclusive the non-locked condition, the final result Lurban is equal to the intermediate result.

In the case of a two-gear test, the final result is the arithmetic mean of the intermediate results. The final result Lurban is the higher value of the two calculated averages.

3.2.	Measurement of sound emitted by stationary vehicles

3.2.1.	Sound level in the vicinity of vehicles

The measurement results shall be entered into the Addendum to the Communication form (Annex 1, Appendix 1).

3.2.2.	Acoustic measurements

A precision sound level meter as defined in paragraph 1.1 of this annex shall be used for the measurements.

3.2.3.	Test site - local conditions (see Appendix 1 of Annex 3, Figure 2)

3.2.3.1.	In the vicinity of the microphone, there shall be no obstacle that could influence the acoustical field and no person shall remain between the microphone and the sound source. The meter observer shall be positioned so as not to influence the meter reading.

3.2.4.	Disturbance noise and wind interference

Readings on the measuring instruments produced by ambient noise and wind shall be at least 10 dB(A) below the sound level to be measured. A suitable windscreen may be fitted to the microphone provided that account is taken of its effect on the sensitivity of the microphone (see paragraph 1.1. of this annex).

3.2.5.	Measuring method

3.2.5.1.	Nature and number of measurements

The maximum sound level expressed in A-weighted decibels (dB(A)) shall be measured during the operating period referred to in paragraph 3.2.5.3.2.1.

At least three measurements shall be taken at each measuring point.

3.2.5.2.	Positioning and preparation of the vehicle

The vehicle shall be located in the centre part of the test area with the gear selector in neutral position and the clutch engaged. If the design of the vehicle does not allow this, the vehicle shall be tested in conformity with the manufacturer's prescriptions for stationary engine testing. Before each series of measurements, the engine shall be brought to its normal operating condition, as specified by the manufacturer.

If the vehicle is fitted with fan(s) having an automatic actuating mechanism, this system shall not be interfered with during the sound level measurements.

The engine hood or compartment cover, if so fitted, shall be closed.

3.2.5.3.	Measuring of noise in proximity to the exhaust (see Appendix 1 of Annex 3, Figure 3a)

3.2.5.3.1.	Positions of the microphone

3.2.5.3.1.1.	The microphone shall be located at a distance of 0.5 m ± 0.01 m from the reference point of the exhaust pipe defined in Figure 2 and at an angle of 45º (±5º) to the vertical plane containing the flow axis of the pipe termination. The microphone shall be at the height of the reference point, but not less than 0.2 m from the ground surface. The reference axis of the microphone shall lie in a plane parallel to the ground surface and shall be directed towards the reference point on the exhaust outlet.

If two microphone positions are possible, the location farthest laterally from the vehicle longitudinal centreline shall be used.

If the flow axis of the exhaust outlet pipe is at 90º to the vehicle longitudinal centreline, the microphone shall be located at the point, which is furthest from the engine.

3.2.5.3.1.2.	For vehicles having an exhaust provided with outlets spaced more than 0.3 m apart or more than one silencer, one set of measurement is made for each outlet.

3.2.5.3.1.3.

If a vehicle has two or more exhaust outlets spaced less than or equal to 0.3 m apart and connected to a single silencer, only one set of measurement shall be made. The microphone shall be located relative to the outlet furthest away from the vehicle longitudinal centreline, or when such outlet does not exist, to the outlet, which is highest above the ground.

3.2.5.3.1.4.	For vehicles with a vertical exhaust (e.g. commercial vehicles) the microphone shall be placed at the height of the exhaust outlet. Its axis shall be vertical and oriented upwards. It shall be placed at a distance of 0.5 m ± 0.01 m from the exhaust pipe reference point, but never less than 0.2 m from the side of the vehicle nearest to the exhaust.

3.2.5.3.1.5.

For vehicles, where the reference point of the exhaust pipe is not accessible, or located under the vehicle body, as shown in Figure 3b and 3c in Annex 3, because of the presence of obstacles which form part of the vehicle (e.g. spare wheel, fuel tank, battery compartment), the microphone shall be located at least 0.2 m from the nearest obstacle, including the vehicle body, and shall not be located under the vehicle. Its axis of maximum sensitivity shall face the exhaust outlet from the position least concealed by the above-mentioned obstacles. In case the distance from the exhaust outlet to the other side of the vehicle is larger than 0.2 m (Figures 3c and 3d in Annex 3), the following distances of d1 and d2 shall be chosen:

Case 1:

d1 shall be equal to 0.5 m and the distance from the side (outer border of the vehicle) shall be at least 0.2 m.

d2 shall be equal to 0.5 m and the distance from the side (outer border of the vehicle) shall be at least 0.2 m.

Case 2 (if Case 1 is not fulfilled):

d1 shall be at least 0.5 m and the distance from the side (outer border of the vehicle) shall be equal to 0.2 m.

d2 shall be at least 0.5 m and the distance from the side (outer border of the vehicle) shall be equal to 0.2 m.

When several positions are possible, as shown in Figure 3c, the microphone position giving the lowest value of d1 or d2 shall be used.

3.2.5.3.1.6.	Examples of the position of the microphone, depending on the location of the exhaust pipe, are given in Figures 3a-3d in Appendix 1 to Annex 3.

3.2.5.3.2.	Operating conditions of the engine

3.2.5.3.2.1.	Target engine speed

The target engine speed is defined as:

(a)	75 per cent of the rated engine speed S for vehicles with a rated engine 
	speed ≤ 5,000 min-1;

(b)	3.750 min-1 for vehicles with a rated engine speed above 5.000 min-1 
	and below 7,500 min-1;

(c)	50 per cent of the rated engine speed S for vehicles with a rated engine 
	speed ≥ 7,500 min-1.

If the vehicle cannot reach the engine speed as stated above, the target engine speed shall be 5 per cent below the maximum possible engine speed for that stationary test.

For vehicles for which the engine speed is a fixed value, (for example, but not limited to, series hybrids) either above or below the target engine speed and cannot be adjusted by the accelerator, the test shall be carried out at the fixed engine speed.

In case the engine speed deviates from the applicable target engine speed, the engine speed used for the test and the reason for the deviation shall be documented in the test report and at paragraph 2.2. of Appendix 1 to Annex 1.

3.2.5.3.2.2.	Test procedure

The engine speed shall be gradually increased from idle to the target engine speed, not exceeding the tolerance band of ±3 per cent of the target engine speed, and held constant. Then the throttle control shall be rapidly released and the engine speed shall be returned to idle. The sound pressure level shall be measured during a period of operation consisting of a maintaining constant engine speed of 1 second and throughout the entire deceleration period. The maximum sound level meter reading during this period of operation, mathematically rounded to the first decimal place, is taken as the test value.

Specific provisions for the testing of vehicles equipped with an ADS are provided in Annex 10.

3.2.5.3.2.3.	Test validation

The measurement shall be regarded as valid if the test engine speed does not deviate from the target engine speed by more than ±3 per cent for at least 1 second.

3.2.6.	Results for sound emitted by stationary vehicles

3.2.6.1.	Single test position (outlet)

For the vehicles equipped with one exhaust outlet or two or more exhaust outlets as specified in paragraph 3.2.5.3.1.3., the stationary sound of the vehicle shall be determined for one test position.

At least three measurements for a test position (outlet) shall be made.

The maximum A-weighted sound pressure level indicated during each of the three measurements shall be recorded to the first decimal place.

The first three valid consecutive measurement results, within 2 dB(A), allowing for the deletion of non-valid results (see paragraph 2.1. except the specifications of the test site), shall be used for the determination of the final result for the given measurement position.

The result for a test position (outlet) is the arithmetic average of the three valid measurements, mathematically rounded to the nearest integer value (e.g. 72.5 shall be noted as to 73 while 72.4 shall be noted as to 72).

3.2.6.2.	Multiple test positions (outlets)

For vehicles equipped with multiple exhaust outlets as specified in paragraph 3.2.5.3.1.2., the stationary sound of the vehicle shall be determined for each test position, following the measurement and calculation principles above.

The reported sound pressure level shall be for the test position having the highest average sound pressure level.

3.2.6.3.	Modes

If the vehicle has different modes according to the definition 2.25.1. the stationary sound of the vehicle shall be determined for each mode, following the measurement and calculation principles above.

The sound pressure level for each mode shall be reported according to 3.2.6.1. in case the vehicle has only a single test position (outlet) and according to 3.2.6.2. in case of multiple test positions (outlets).

3.2.7.	Stationary sound pressure level representative for the vehicle type

If the vehicle has only one mode and a single test position (outlet), the representative sound pressure level for the vehicle type is determined by the measurement result according to paragraph 3.2.6.1.

If the vehicle has only one mode but multiple test positions (outlets), the representative sound pressure level for the vehicle type is determined by the measurement result according to paragraph 3.2.6.2.

If the vehicle has multiple modes and one or more test positions (outlets), the representative sound pressure level for the vehicle type is determined by the measurement result according to paragraph 3.2.6.3. In the test report and paragraph 2.2. of Appendix 1 to Annex 1 the representative test result determined by the principles above and the name of the mode shall be documented for every mode. The representative sound pressure level for the vehicle type and its registration papers is the highest reported sound pressure level of all modes as documented in paragraph 2.2. of Appendix 1 to Annex 1.

4.	Sound from the hybrid vehicle of categories M1 in motion, where an internal combustion engine cannot operate when the vehicle is stationary (data reported to facilitate testing of the vehicle in use).

4.1.	In order to facilitate in-use compliance test of hybrid vehicles – where an internal combustion engine cannot operate when the vehicle is stationary –, the following information relating to the sound-pressure level measurements carried out in accordance with paragraph 3.1. of Annex 3 for the motor vehicles in motion is referred to as in-use compliance reference data:

(a) 	Gear (i) or, for vehicles tested with non-locked gear ratios, the position of the gear selector chosen for the test;

(b) 	Position of the operating switch during measurement of the sound pressure level Lwot (i) (if switch is fitted);

(c) 	The pre-acceleration length lPA in m;

(d)	The average vehicle speed in km/h at the beginning of the full throttle acceleration for tests in gear (i); and

(e) 	The sound pressure level Lwot (i) in dB(A) of the wide-open-throttle tests in gear (i), defined as the maximum of the two values resulting from averaging the individual measurement results at each microphone position separately.

4.2.	The in-use compliance reference data shall be entered in the type approval certificate as specified in paragraph 2.3. of the Addendum to the Communication form (Annex 1, Appendix 1).

Annex 3 – Appendix 1

# Figure 1

Measuring positions for vehicles in motion

# Figure 2

Reference point for measurement of sound emitted by stationary vehicles

# Figure 3a

# Figure 3b

# Figure 3c

# Figure 3d

# Figure 4a

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – Lurban computation

Determine PMR for test vehicle 3.1.2.1.1.

Determine target acceleration aurban    (3.1.2.1.2.3.) and reference acceleration awot ref    (3.1.2.1.2.4.)

Select test method (3.1.2.1.4.)

locked gears (3.1.2.1.4.1.)  See Figures 4b, 4c and 4d

non-locked gears (3.1.2.1.4.2.) See Figure 4e

Acceleration test (3.1.2.1.5.)

Constant speed test (3.1.2.1.6.)

Calculation of gear weighting factor k if 2-gear test (3.1.2.1.4.1.)

Calculate L wot rep and L crs rep (3.1.3.4.1.2.)

Calculate k P (3.1.3.4.1.2.)

Calculate L urban (3.1.3.4.1.2.)

# Figure 4b

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation - 
Gear selection using locked gear PART 1

Select Gear

Is acceleration stable according to 2.26.2.?

Calculate test acceleration according to 3.1.2.1.2.1.

Select Pre-acceleration and entry speed

Yess

No

Is acceleration within a wot_ref tolerance band?

Yess

No

Is acceleration less than or equal 2,0 m/s2? and engine speed less than nMAX prior to BB'?

Use gear and compute k P  according to 3.1.3.4.1.2.

Compute L wot rep using results of valid runs

No

Select gears to obtain gear i with stable acceleration above a wot_ref and gear i+1 with stable acceleration below a wot_ref

See Case 2 in Figure 4c

See Case 1 in Figure 4c

Yess

Testing locked gears according to 3.1.2.1.4.1.

# Figure 4c

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – 
Gear selection using locked gear PART 2

Yess

No

Is acceleration of gear i less than or equal 2,0 m/s2? and engine speed less than nMAX prior to BB’?

Use both gears i and i+1, (i+2,  i+3, or,..)   and compute k P  according to 3.1.3.4.1.2. and k by 3.1.2.1.4.1.

Compute L wot rep using results of valid runs

Case 1:

Two gears, gear i with stable acceleration above a wot ref and gear i+1 with stable acceleration below a wot ref

Case 2:

One gear with stable acceleration above 2,0 m/sec2 or engine speed greater than nMAX prior to BB'

Determine first gear i + n (n=1,2,…) with stable acceleration less than or equal to 2,0 m/s2 and engine speed less than nMAX prior to BB'

Is acceleration of gear i +n more than a urban?

Yess

Use gear i+n (n=1, 2, …) and compute k P  according to 3.1.3.4.1.2.

No

Use both gears i with acceleration higher than 2,0 m/sec2 and i+1, (i+2, i+3, or,..)  with acceleration less than a urban

Is engine speed of gear i more than nMAX prior to BB’ ?

See Case 3 in Figure 4d

No

Yess

# Figure 4d

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – 
Gear selection using locked gear PART 3

Yess

No

Yes

Use gear i+n (n=1, 2, ...) with test speed 50 km/h and compute kP according to 3.1.3.4.1.2.

Reduce test speed vtest by 2,5 km/h with gear i

Use both gears i and i+n (n=1, 2, ...) with new test speed for gear i and 50 km/h for gear i+n and compute kP according to 3.1.3.4.1.2.

Is engine speed of gear i less than nMAX S prior to BB’?

No

# Case 3:

# No gear with acceleration more than aurban and engine speed less than nMAX S prior to BB’

Is test speed vtest = 40 km/h?

# Figure 4e

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – 
Gear Selection using non-locked gears

No

Yes

engine speed greater than nMAX prior to BB'

See flowchart 4f.

Yess

No

Compute L wot rep using results of valid runs

Calculate test acceleration according to 3.1.2.1.2.2.  Pre-acceleration is not allowed

Compute k P  according to 3.1.3.4.1.2.

Can measures be taken to control downshifts?

Is acceleration stable?  i.e. there is no delay.

Calculate test acceleration according to 3.1.2.1.2.2.

Select Pre-acceleration and entry speed

No

Yess

If possible, control downshift to obtain acceleration less than or equal to 2,0 m/s2 or a wot_ref, whichever is lower.  If not possible, run higher than 2,0 m/s2 is valid.

Testing unlocked gears according to 3.1.2.1.4.2.

Select entry speed

# Figure 4f

# Flowchart for vehicles tested according to paragraph 3.1.2.1.4.2. of Annex 3 to this Regulation – Gear Selection using non-locked gears

Possibility 2

Test according to 3.1.2.1.4.2. with specified engine load

Possibility 1

Yess

No

Yes

Not valid test condition.

Reduce test speed vtest by 2.5 km/h

Test according to 3.1.2.1.4.2. with new test speed

Is engine speed less than nMAX prior to BB’?

No

# Engine speed exceed nMAX prior to BB’

Is test speed vtest = 40 km/h?

Reduce engine load (by using partial load) such that nBB’ is between 95 % nMAX and nMAX

Yess

# Figure 5a*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test in locked gears

Select test method (3.1.2.2.1.)

Determine vehicle load (2.2.1.)

Locked gear ratios (3.1.2.2.1.1.)

Fulfilling target conditions (3.1.2.2.)?

Establish target conditions (3.1.2.2.)

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.1. d)

nBB’ = ntarget BB’

25 km/h ≤  vBB’ x ≤ 30 km/h

40 km/h ≤  vBB’ y ≤ 45 km/h

Test criteria

(3.1.2.2.1.1. a)

nBB’ = ntarget BB’

vBB’ = vtarget BB’

Test criteria

(3.1.2.2.1.1. b)

nBB’ = ntarget BB’

Gear i with vBB’gear i

Closest to  vtarget BB’

Test criteria

(3.1.2.2.1.1. f)

vBB’ gear i = vtarget BB’

and

nBB’ gear i  ≤ ntarget BB’

vtarget BB’ = Yes

Yes, on one gear

ntarget BB’ = Yes

Lurban x for vBB’ x

Lurban y for vBB’ y

(3.1.3.)

Lurban

(3.1.3.)

Lurban for gear i

with vBB’ gear i

closest to  vtarget BB’

(3.1.3.)

Lurban for nBB’ gear i

closest to but

≤ ntarget BB’

(3.1.3.)

If (3.1.2.2.1.1. c)

(vtarget BB’ - vBB’ gear i )

is equal to

(vBB’ gear i+1 - vtarget BB’)

At least two test

conditions

Two test

conditions

At least two test

conditions

Yes, on more than one gear

vtarget BB’ = Yes

ntarget BB’ = No

vtarget BB’ = No

If (3.1.2.2.1.1 e)

For both gears

For one gear only

Calculate average (3.1.3.2.)

If nBB’ = ntarget BB’

# Figure 5b*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test in non-locked gears PART 1

Select test method (3.1.2.2.1.)

Determine vehicle load (2.2.1.)

Non-locked gear ratios (3.1.2.2.1.2.)

Fulfilling target conditions (3.1.2.2.)?

Establish target conditions (3.1.2.2.)

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.2. d)

25 km/h ≤ vBB’ 1 ≤ 30 km/h

40 km/h ≤ vBB’ 2 ≤ 45 km/h

nBB’ i ≤ ntarget BB’

Test criteria

(3.1.2.2.1.2. a)

nBB’ = ntarget BB’

vBB’ = vtarget BB’

Test criteria

(3.1.2.2.1.2. b)

nBB’ = ntarget BB’

25 km/h ≤ vBB’ 1 ≤ 35 km/h

35 km/h ≤ vBB’ 2 ≤ 45 km/h

Test criteria

(3.1.2.2.1.2. e)

vBB’ = vtarget BB’ + 5km/h

nBB’  closest to

ntarget BB’

vtarget BB’ = Yes

ntarget BB’ = Yes

Lurban for nBB’ i

closest to but

≤ ntarget BB’ for i=1,2

(3.1.3.)

Lurban

(3.1.3.)

Lurban 1 for vBB’ 1

Lurban 2 for vBB’ 2

(3.1.3.)

Lurban

for nBB’   closest to

ntarget BB’

(3.1.3.)

One test

conditions

Two test

conditions

Two test

conditions

vtarget BB’ = No

vtarget BB’ = Yes

If (3.1.2.2.1.2. c)

Calculate average (3.1.3.2.)

vtarget BB’ = No

ntarget BB’ = No

f

If nBB’ i > ntarget BB’

# Figure 5c*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test in non-locked gears PART 2

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.2. f)

vBB’ = vtarget BB’

vtarget BB’ = Limited

by

gear selection D

Lurban

(3.1.3.)

f

# Figure 5d*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test for no- combustion engine speed available

Select test method (3.1.2.2.1.)

Determine vehicle load (2.2.1.)

No rotational engine speed available (3.1.2.2.1.3.)

Fulfilling target conditions (3.1.2.2.)?

Establish target conditions (3.1.2.2.)

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.3. a)

vBB’ = vtarget BB’

Test criteria

(3.1.2.2.1.3. b)

25 km/h ≤ vBB’ 1 ≤ 35 km/h

35 km/h ≤ vBB’ 2 ≤ 45 km/h

Test criteria

(3.1.2.2.1.3. c)

35 km/h ≤ vBB’ 2 ≤ 45 km/h

vtarget BB’ = Yes

Lurban

(3.1.3.)

Lurban 1 for vBB’ 1

Lurban 2 for vBB’ 2

(3.1.3.)

Lurban 2 for vBB’ 2

(3.1.3.)

One test

conditions

Two test

conditions

vBB’ 1 = No

Calculate average (3.1.3.2.)

vtarget BB’ = No

If

Note:

*	Broaden the window for the target vehicle speed vtarget BB’ for vehicles of category M2>3,500 kg technically permissible maximum laden mass and for vehicles of category N2, M3 and N3.

The target vehicle velocity vtarget BB’ is defined as vtarget BB’ = 35 km/h ± 5 km/h which results in a window for the velocity vBB’, when the reference point passes line BB’, from 30 km/h to 40 km/h. If the target vehicle velocity vtarget BB’ is changed into two target vehicle velocities, a lower and a higher one, the following is meant: The lower target vehicle velocity is defined as the target vehicle velocity vtarget BB’ reduced by 5 km/h (v target BB’ - 5 km/h) which results in a window for the velocity vBB’ 1, when the reference point passes line BB’, from 25 km/h to 35 km/h.

25 km/h ≤ vBB’1 ≤ 35 km/h.

The higher target vehicle velocity is defined as the target vehicle velocity vtarget BB’ increased by 5 km/h (vtarget BB’ + 5 km/h) which results in a window for the velocity vBB’2, when the reference point passes line BB’, from 35 km/h to 45 km/h.

35 km/h ≤ vBB’2 ≤ 45 km/h.

Figure 6

Flowchart for measurement and data processing of stationary sound according to paragraph 3.2.

Stationary sound Measurements according to paragraph 3.2.

Setup of the vicinity and vehicle according paragraphs 3.2.1. to 3.2.4.

Stationary sound measurement at a test position (outlet) according para. 3.2.5.

Multiple

test positions

available?

NO

All test positions

are measured?

NO

NO

NO

Driver

selectable modes

available?

YES

Outlet(s)

are measured in all

modes?

Select next mode, repeat all measurements at any test position (outlet)

YES

Report sound pressure level according to paragraph 3.2.6.1.

YES

Select next test position (outlet) for measurement.

YES

Report for each mode the representative sound pressure level

Report highest sound pressure level of all modes and outlets as representative sound pressure level for the vehicle type

Report sound pressure level for according to paragraph 3.2.6.2.

Table 1

# Examples for Devices and Measures to Enable a Vehicle Tested within the Acceleration Boundaries

| No. | Impact | Sub No. | Measure | Additional Requirements |
| --- | --- | --- | --- | --- |
| 1 | Lock of a discrete gear ratio | 1* | A discrete gear ratio can be locked by the driver | none |
|  |  | 2 | A discrete gear ratio is available onboard, but is not available to the driver. Locking can be activated by the manufacturer with an onboard (hidden) function or with an external device | none |
| 2 | Controlled gear shift management: Applicable to transmissions which cannot be locked, or where no locked gear provides a valid test result | 1* | Kickdown is deactivated | none |
|  |  | 2 | Gear shift change(s) can happen during the test, gear shift is controlled by activation of an internal function or external device | Acceleration** shall be between aurban and awot,ref , not exceeding 2.0 m/s². |
| 3 | Defined partial load driving**** | 1 | Acceleration is limited by a mechanical device | Defined acceleration** shall be between aurban and awot,ref , not exceeding 2.0 m/s². For ASEP**, the anchor point parameter are calculated by: Lanchor = (Ltest - kp*Lcrs) / (1-kp) with kP = 1-atest/awot,ref and awot,ref according to 3.1.2.1.2.4. but not higher than 2.0 m/s² nanchor = nbb,test * 3.6 / vbb,test * (awot,ref*(20+2*l)+192,9)0,5 |
|  |  | 2 | External Programming for partial load acceleration***) |  |
| 4 | Mixed Solution (Mode): This measure will be a mix of the above solutions combined in a specific mode | 1* | Mode is available onboard and can be selected by the driver | none |
|  |  | 2 | Mode is available onboard and can only be activated by the manufacturer with a hidden function or an external device | none |
|  |  | 3 | Mode is not available onboard, an external software overrides the internal software | Acceleration** shall be between aurban and awot,ref, not exceeding 2.0 m/s². |

*	Comment: This is a standard situation, already covered by the Regulation text.

** 	Applicable to vehicles of category M1, N1 and M2 ≤ 3,500 kg.

*** 	Partial load shall be achieved by simulation of the travel restriction of the accelerator. It is not allowed to interfere with the engine control management.

****	Applicable

to vehicles of categories M1, N1 and M2 ≤ 3,500 kg, for the further calculation of Lurban in Annex 3 the sound level measured under partial load shall replace the sound level under wide open throttle.

The achieved acceleration under partial load during the test shall be used for the calculation of the part power factor kP instead of awot ref.

The test procedure and the data processing follow the same principle. Although it is tested in partial load, symbols xwot (e.g. Lwot, awot, …) shall be used.

Annex 3 – Appendix 2

Correction for the tyre rolling sound component of pass-by sound measurements

1.	Scope of the correction

This Appendix contains provisions on the correction for the tyre rolling sound component of pass-by sound measurements of Annex 3 and applies to vehicles of categories M1 and N1, and for vehicles of category M2 having a maximum authorized mass not exceeding 3,500 kg.

2.	General (see the flowcharts in this Appendix 2, Figure 7a to Figure 7c)

This Appendix provides correction for temperature and test track dependent on the tyre category and purpose.

For the correction, tyre rolling sound reference values are needed. Tyre rolling sound measurements shall be carried out according to the test procedure of Appendix 3 to Annex 3 of this regulation.

2.1.	These measurements might be carried out during the type approval of a vehicle type (Case 1 as described in paragraph 3. of this appendix) or be performed as an independent test to be used for type approval tests of different vehicle types (Case 2 as described in paragraph 4. of this appendix).

For the further processing of data, the following tyre rolling sound reference information shall be available from the test according to Appendix 3 to Annex 3:

(a)	The tyre rolling sound LTR,ref separately for the left and the right side of the vehicle

(b)	The slope of the tyre rolling sound slpref separately for the left and for the right side of the vehicle

(c)	The reference speed vTR,ref to which these sound levels are assigned. If tyre rolling sound measurements are directly carried out in junction with the pass-by measurements, the reference speed vTR,ref shall be determined in a way to be equal to the vehicle test speed vcrs and vwot.

2.2.	The test results for each gear as determined according to Annex 3 paragraphs 3.1.2.1.4., test condition (acceleration or constant speed) and per vehicle side, are subject to the temperature correction.

2.3.	For simplicity, the formula below uses the index x as place holder for the applicable gear ratio(s) i or i+n. No index is introduced for left and right side, but all calculations shall be done separately for left and right side of the vehicle.

2.4.	If tests are carried out at air temperatures below 5 °C according to paragraph 2.1.3. of Annex 3, the temperature correction is applicable down to an air temperature of 0 °C. For any tests carried out at air temperatures below 0°C, the temperature correction shall be calculated with 0 °C, regardless of the measured air temperature.

3.	Case 1

The temperature correction is based on tyre rolling sound measurements carried in junction with pass-by tests according to Annex 3.

3.1.	Tyre Rolling Sound Reference

The rolling sound of the tyre LTR,ref,vTR,ref and the tyre sound level slope slpref for left and right side of the vehicle shall be determined for a reference vehicle speed vTR,ref at the reference temperature ref according to Appendix 3 to Annex 3.

3.2.	Temperature correction for constant speed test results

3.2.1.	The reference speed shall be identical to the reference test speed of the constant speed test vcrs determined in Annex 3. In most cases this will be 50 km/h. If the tyre reference speed vTR,ref, differs from vcrs, adjust the tyre rolling sound per vehicle side to the test speed vcrs by:

LTR,crs,j,ϑref=LTR,ϑref, vTR,ref+ slpref×lgvcrsvTR,ref

3.2.2.	For each valid pass-by test run j under constant speed the following values per gear are available from the measurements according to Annex 3 paragraph 3.1.2.1.:

(a)	the reported sound levels Lcrs,j,

(b)	the vehicle speed vcrs,PP’,j, and

(c)	the air temperature crs,j.

3.2.3.	For each individual test run (gear, condition and vehicle side), a tyre rolling sound reference shall be calculated for the applicable air temperature crs,j.

LTR,crs,j,ϑcrs=LTR,crs,j,ϑref+K1×lgϑref+K2ϑcrs,j+K2

where 	ϑref 	= 	20 °C 	and 
K1 	=	 3.4 	for C1 and C2 tyres and
K2	=	 3.0 	for C1 tyres and
K2 	= 	15.0 	for C2 tyres

3.2.4.	For each gear, run and vehicle side under constant speed extract the power train component LPT,crs,j from the test result Lcrs,j, by calculation.

LPT,crs,j=10×lg100.1×Lcrs,j-100.1×LTR,crs,j,ϑcrs

In case that LTR,crs,crs is greater than Lcrs,j the power train component LPT,crs,j is determined by

LPT,crs,j=10×lg0.01×100.1×Lcrs,j

with LTR,crs,j,ϑcrs redefined as

LTR,crs,j,ϑcrs = 10 × lg(0.99 × 100.1×Lcrs,j)

The redefined LTR,crs,j,ϑcrs shall then be subjected to temperature correction in 3.2.3 to obtain the corresponding LTR,crs,j,ϑref.”

3.2.5.	Calculate per gear, run and vehicle side the air temperature adjusted constant speed test result Lcrs,j,ref using the temperature normalized tyre rolling sound LTR,ref calculated by

Lcrs,j,ϑref=10×lg100.1×LPT,crs,j+100.1×LTR,crs,j,ϑref

3.3.	Temperature correction for acceleration test results

3.3.1.	For each gear, run and vehicle side, adjust the tyre rolling sound to the speed condition of the acceleration test

LTR,wot,j,ϑref=LTR,ϑref, vTR,ref+slpref×lg0.5×vBB',wot+vPP',wotvTR, ref

3.3.2.	For each valid pass-by test run under acceleration the following values per gear are available from the measurements according to Annex 3 paragraph 3.1.2.1.:

(a)	the reported sound levels Lwot,j,

(b)	the vehicle speeds vwot,PP’,j and vwot,BB’,j , and

(c)	the air temperature wot,j.

3.3.3.	For each individual test run (gear, condition and vehicle side), a tyre rolling sound reference shall be calculated for the applicable air temperature or wot,j.

LTR,wot, j,ϑwot=LTR,wot,j,ϑref+K1×lgϑref+K2ϑwot,j+K2

where	ϑref 	=	20 °C 	and
	K1 	=	3.4 	for C1 and C2 tyres and
	K2 	=	3.0 	for C1 tyres and
	K2 	= 	15.0 	for C2 tyres

3.3.4.	For each gear, run and vehicle side extract the power train component LPT,wot,j from the reported acceleration test Lwot,j, by calculation.

LPT,wot,j=10×lg100.1×Lwot,j-100.1×LTR,wot,j,ϑwot

In case that

100.1×LTR,wot,j,ϑwot ≥ 0.99 × 100.1×Lwot,j

the power train component LPT,wot,j is determined by

LPT,wot,j=10×lg0.01×100.1×Lwot,j

with LTR,wot,j,ϑwot redefined as

LTR,wot,j,ϑwot = 10 × lg(0.99 × 100.1×Lwot,j)

The redefined LTR,wot,j,ϑwot shall then be subjected to temperature correction in 3.3.3 to obtain the corresponding LTR,wot,j,ϑref

3.3.5.	Calculate per gear the acceleration test result Lwot,j,ref

Lwot,j,ϑref=10×lg100.1×LPT,wot,j+100.1×LTR,wot,j,ϑref

3.4.	Proceed to calculate Lurban using the temperature normalized sound pressure levels Lcrs,j,ref and Lwot,j,ref according to the procedure of Annex 3 paragraph 3.1.3.4.1.2.

4.	Case 2

The temperature correction based on tyre rolling sound measurements that have been performed independent from the pass-by tests subject to the temperature correction.

Case 2 is applicable, when pass-by tests carried out according to Annex 3 shall be compared with already existing results – e.g., from type approval, that have been performed under a different temperature condition and on a different test track.

4.1.	The necessary information on tyre rolling sound representative for the tyre used on the vehicle is available from former type approval tests or have been carried out separately according to Appendix 3 to Annex 3 of this UN Regulation. The essential information is provided by the test report of that Appendix and is:

(a)	the tyre rolling sound LTR,DB,ref at the reference temperature ref,

(b)	the reference vehicle speed vTR,DB,ref, and

(c)	the tyre rolling sound slope slpDB,ref.

4.2.	Determine the tyre rolling sound for the vehicle according to Case 1 above and extract the power train relevant components LPT,crs,j and LPT,wot,j for each gear and run accordingly.

4.3.	Temperature correction for constant speed test results

4.3.1.	The reference speed shall be identical to the reference test speed of the constant speed test vtest determined in Annex 3. In most cases this will be 50 km/h. If the tyre reference speed vTR,DB,ref differs from vcrs, adjust the tyre rolling sound per vehicle side to the test speed vcrs by:

LTR,DB, crs,j,ϑref=LTR,DB,ϑref +slpDB,ref×lgvcrs,jvTR,DB,ref

4.3.2.	For each gear, test run and vehicle side, calculate the air temperature and test track adjusted constant speed test results Lcrs,j,ref by

Lcrs,j,ϑref=10×lg100.1×LPT,crs,j+100.1×LTR,DB,crs,j,ϑref

4.4.	Temperature correction for acceleration test results

4.4.1.	For each gear, test run and vehicle side, adjust the tyre rolling sound LTR,DB,ref to the speed condition of the acceleration test

LTR,DB,wot,j,ϑref=LTR,DB,ϑref+slpDB,ref×lg0.5×vBB',wot,j+vPP',wot,jvTR,DB,ref

4.4.2.	For each gear, test run and vehicle side, calculate the acceleration test result Lwot,j,ref by

Lwot,j,ϑref=10×lg100.1×LPT,wot,j+100.1×LTR,DB,wot,j,ϑref

4.5.	Proceed to calculate Lurban with the temperature normalized sound pressure levels Lcrs,j,ref and Lwot,j,ref according to the procedure of Annex 3 paragraph 3.1.3.4.1.2.

# Figure 7a

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – Correction of pass-by measurements for temperature and if applicable for test track differences

# Correction of pass-by measurements according to Annex 3
for temperature and for test track differences (if applicable)

Case 1
Correction for temperature only

Case 2
Correction for temperature and test track

Perform pass-by sound measurements 
according to Annex 3

# Tyre rolling sound has already been 
determined independently and 
the normalized parameters are available:

# - tyre rolling sound LTR,DB,ref,
- the reference speed vTR,DB,ref , and 
- the tyre sound level slope slpDB,ref

Perform tyre rolling sound measurements in combination with the tests above 
according to Appendix 2 to Annex 3

# Per gear and run, extract 
the power train component LPT,crs,j
from the constant speed test result

# Follow the procedure of Case 1 
to determine the power train components
LPT,crs,j and LPT,wot,j

# Per gear and run, adjust 
the tyre rolling sound to 
the vehicle speed condition of 
the individual test run

# Per gear and run, extract 
the power train component LPT,wot,j
from the acceleration test result

# Recalculate Lcrs,j and Lwot,j 
with power train components and 
normalized tyre rolling sound components

# Calculate Lurban with the corrected 
constant speed and acceleration test results 
according to the procedure in Annex 3

# Per gear and run, normalize
the tyre rolling sound to 
the reference temperature

# Figure 7b

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – Temperature Correction for Tyre Rolling Sound Components Case 1

Correction for Tyre Rolling Sound Components

Case 1: Temperature Correction
Tyre rolling sound LTR,ref is determined during a set of pass-by measurements according to Annex 3 
and used to normalize the tyre rolling sound component of each individual measurement run 
to the reference air temperature ref = 20 °C

The rolling sound of the tyre LTR,ref, vTR,ref for left and right side of the vehicle 
shall be determined for a reference vehicle speed vTR,ref according to Appendix 3 to Annex 3.
The reported values are the tyre rolling sound level LTR,ref , vTR,ref
the tyre sound level slope slpref and the temperature TR at which these tests have been performed.

For each individual pass-by measurement run j under constant speed the following values per gear are available:
the reported sound levels Lcrs,j, 
the vehicle speed vcrs,PP’,j  
and the air temperature crs, j

Adjust the tyre rolling sound LTR,ref, vTR,ref 
to the test speed condition of the constant speed test:

LTR,,crs,j,ϑref=LTR,ϑref, vTR,ref+ slpref×lgvcrsvTR,ref

Adjust the tyre rolling sound to 
the applicable air temperature crs,j  
of the individual run and vehicle side:

LTR,crs,j,ϑcrs=LTR,crs, j,ϑref+K1×lgϑref+K2ϑcrs,j+K2

with 	K1 	= 	3.4 	for C1 and C2 tyres and 
K2 	= 	3.0 	for C1 tyres and 
K2	=	15.0 	for C2 tyres

For the per gear and run reported 
constant speed test result Lcrs,j, extract 
the power train component LPT,crs,j by calculation.

LPT,crs,j=10×lg100.1×Lcrs,j-100.1×LTR,crs,j,ϑcrs

Calculate per gear and run the temperature adjusted 
constant speed test result Lcrs,j,ref

Lcrs,j,ϑref=10×lg100.1×LPT,crs,j+100.1×LTR,crs,j,ϑref

For each individual pass-by measurement under acceleration the following values per gear are available:
the reported sound levels Lwot,j, 
the vehicle speeds vwot,PP’,j and vwot,BB’,j 
and the air temperature wot,j

Adjust the tyre rolling sound LTR,ref, vTR,ref to 
the speed condition of the acceleration test:

LTR,wot,j,ϑref=LTR,ϑref,vTR,ref+slpref×lgvBB',wot+vPP',wot2 × vTR, ref

Adjust the tyre rolling sound to 
the applicable air temperature wot,j  
of the individual run and vehicle side:

LTR,wot, j,ϑwot=LTR,wot,j,ϑref+K1×lgϑref+K2ϑwot,j+K2

with 	K1 	= 	3.4 	for C1 and C2 tyres and 
K2 	= 	3.0 	for C1 tyres and 
K2	=	15.0 	for C2 tyres

For the per gear and run reported 
acceleration test Lwot,j, extract 
the power train component LPT,wot,j by calculation.

LPT,wot,j=10×lg100.1×Lwot,j-100.1×LTR,wot,j,ϑwot

Calculate per gear and run  the temperature adjusted acceleration test result Lwot,REP,ref

Lwot,j,ϑref=10×lg100.1×LPT,wot,j+100.1×LTR,wot,j,ϑref

Proceed to calculate Lurban using the temperature normalized sound pressure levels 
Lcrs,j,ref and Lwot,j,ref according to the procedure of Annex 3

# Figure 7c

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – Temperature/Test Track Correction for Tyre Rolling Sound Components Case 2

Correction for Tyre Rolling Sound Components

Case 2: Temperature & Test Track Correction
Tyre rolling sound has already been determined independently from the type approval: 
The air temperature normalized tyre rolling sound LTR,DB,ref, 
the reference speed vTR,DB,ref and the tyre sound level slope slpDB,ref are available.

Calculate per gear and run the temperature & track adjusted constant speed test result Lcrs,j,ref

Lcrs,j,ϑref=10×lg100.1×LPT,crs,j+100.1×LTR,DB,crs,j,ϑref

Calculate per gear and run  the temperature & track adjusted acceleration test result Lwot,j,ref

Lwot,j,ϑref=10×lg100.1×LPT,wot,j+100.1×LTR,DB,wot,j,ϑref

Proceed to calculate Lurban using the temperature normalized sound pressure levels 
Lcrs,j,ref and Lwot,j,ref according to the procedure of Annex 3

Adjust the tyre rolling sound LTR, DB, ref 
to the test speed condition of the constant speed test:

LTR,DB,crs,j,ϑref=LTR,DB, ϑref+slpDB,ref×lgvcrsvTR,DB,ref

Adjust the tyre rolling sound LTR,DB, ref to 
the speed condition of the acceleration test:

LTR,DB,wot,j,ϑref=LTR,DB, ϑref+slpDB,ref×lgvBB',wot+vPP',wot2 × vTR,DB, ref

For each gear and run, determine the tyre rolling sound for the vehicle according to Case 1

Extract the power train relevant components LPT,crs,j

Extract the power train relevant components LPT,wot,j

Annex 3 – Appendix 3

Coast-by test method for measuring tyre-rolling sound emission

1.	Measuring instruments

Unless otherwise specified, the measuring instruments shall comply with the provisions of Annex 3 of this UN Regulation.

1.1.	Meteorological equipment

Unless otherwise specified, the meteorological equipment shall comply with the provisions of Annex 3 of this UN Regulation.

2.		Conditions of measurement

2.1.		Test site

The test site shall comply with the provisions of Annex 3 of this UN Regulation.

2.2.	Meteorological conditions

Tests carried out on request of the manufacturer at temperatures below 5° C shall be accepted as well, however temperature correction to be applied is limited to a minimum air temperature of 0 °C. See paragraph 2 of Appendix 2 to Annex 3 of this Regulation as well.

2.3.		Ambient noise

The background noise provisions shall be in line with Annex 3 of this UN Regulation.

2.4.		Test vehicle requirements

2.4.1.	General

The test vehicle shall be either

(a)	the vehicle directly used for tests according to Annex 3 of this UN Regulation. In this case, the requirements of paragraphs 2.4.2. to 2.4.4. inclusive the subparagraphs do not apply. The vehicle shall comply with the specifications of Annex 3 of this UN Regulation, or

(b)	a motor vehicle compliant with the provisions of paragraphs 2.4.2. to 2.4.4.

2.4.2.	Vehicle load

The vehicle shall be loaded such as to comply with the test tyre loads as specified in paragraph 2.5.2. below.

2.4.3.		Wheelbase

The wheelbase between the first and the second axles fitted with the test tyres shall for Class C1 be less than 3.50 m and for Class C2 tyres be less than 5 m.

2.4.4.		Measures to minimize vehicle influence on sound level measurements

The test vehicle shall be suitable for vehicles to which this tyre will be fitted, this is fulfilled, if the vehicle has been cross-checked to the vehicle type to which the tyres are dedicated with regard to the design criteria below:

2.4.4.1.	Requirements:

(a)	Spray suppression flaps or another extra device to suppress spray;

(b)	Retention of elements in the immediate vicinity of the rims and tyres, which may screen the emitted sound;

(c)	Wheel alignment (toe in, camber and caster) shall be in full accordance with the vehicle manufacturer's recommendations;

(d)	Sound absorbing material in the wheel housings or under the underbody;

(e)	Ground clearance: if available, the body level shall be adjusted to a comparable ground clearance as applicable for the vehicle type.

2.4.4.2.	Recommendations to avoid parasitic noise:

(a)	During testing it should be ascertained that brakes are not poorly released, causing brake noise;

(b)	It should be ascertained that electric cooling fans are not operating;

(c)	Windows and sliding roof of the vehicle shall be closed during testing.

2.5.	Tyres

2.5.1.	General

Four tyres shall be fitted on the test vehicle to be representative for the tyre configuration as it will be used for the type approval of a vehicle. Tyres with special fitting requirements shall be tested in accordance with these requirements (e.g., rotation direction). The tyres shall have a minimum tread depth of 80%.

New tyres shall be "run-in" prior to testing to remove compound nodules or other tyre pattern characteristics resulting from the moulding process. This will normally require the equivalent of about 100 km of normal use on the road.

Tyres are to be tested on rims specified by the vehicle manufacturer.

2.5.2.		Tyre loads

2.5.2.1.	If the test vehicle is a vehicle subject to tests according to Annex 3 according to this UN Regulation, the provisions on the tyre loads below do not apply.

2.5.2.2.	In other cases, the loads on the tyres shall be representative for the vehicle to which these tyres are dedicated with a tolerance +/- 20% not exceeding 90% of the maximum tyre load.

2.5.3.	Tyre inflation pressure

2.5.3.1.	If the test vehicle is a vehicle subject to type approval according to this Regulation, the tyre inflation pressure shall be according to paragraph 2.2.2. of Annex 3.

2.5.3.2.	In other cases, the tyre pressure shall be adjusted according to the manufacturer’s specification for the tyre load as selected according to paragraph 2.5.2.2. above.

2.5.4.		Preparations prior to testing

Prior to testing tyres shall be warmed up by running under test conditions for at least 10 min to allow the rubber compound to warm-up.

3.		Method of testing

3.1.	General conditions

For all measurements the vehicle shall be driven in a straight line over the measuring section (AA' to BB') in such a way that the median longitudinal plane of the vehicle is as close as possible to the line CC'.

When the front end of the test vehicle has reached the line AA' the vehicle shall be brought to coast-down by full release of the accelerator. If applicable, the influence of the power train noise shall be minimized, e.g. the driver shall have put the gear selector to neutral position and switched off the engine. If abnormal noise (e.g. ventilator, self-ignition) is emitted by the test vehicle during the measurement, the test shall be disregarded.

As an alternative test method, the accelerator may be positioned such to maintain a constant speed between line AA’ with an accuracy of +/- 1 km/h. The procedure is recommended especially for electric vehicles when a release of the acceleration pedal would result is a forced deceleration (recuperation) with higher negative torque on the tyre.

Specific provisions for the testing of vehicles equipped with an ADS are provided in Annex 10.

3.2.	Nature and number of measurements

The maximum sound level expressed in A-weighted decibels (dB(A)) shall be measured simultaneously for the left and right side of the vehicle and be reported to the first decimal place as the vehicle is coasting between lines AA' and BB' (front end of the vehicle on line AA', rear end of the vehicle on line BB').

For each pass-by measurement n the vehicle speeds vPP’,n shall be reported, when the reference point of the vehicle (see definition 2.11) passes the lines PP’. In case of a test vehicle according 2.4.1. (b) test might be needed at various reference points, if the tyre configuration shall be used on vehicles with different reference points. The vehicles speeds shall be mathematically rounded to the first decimal place.

At least six measurements shall be made on each side of the test vehicle approximately equally spaced over the speed range specified in paragraph 3.3. below.

3.3.		Test speed range

The test vehicle speeds shall be within the range from 40 km/h to 60 km/h.

4.		Interpretation of results

The measurement shall be invalid if an abnormal discrepancy between the values is recorded (see background noise and measurement reading provisions of annex 3).

4.1.	Determination of test result

Reference speed vTR,ref used to determine the final result will be 50 km/h, unless the reference speed is reduced during the type approval test according to the provisions of paragraph 3.1.2.1.4.1. (d) of Annex 3 of this UN Regulation.

4.2.	Temperature correction

Each test result LTR,i shall be normalized to the air temperature ref by applying a temperature correction, according to the following:

LTR,i,ϑref= LTR,i,TR+ K1× lgϑTR,i + K2ϑref + K2

where	ϑref 	=	20 °C 	and
	TR,i	=  	the measured air temperature per run i and
	K1 	=	3.4 	for C1 and C2 tyres and
	K2 	=	3.0 	for C1 tyres and
	K2 	=	 15.0 	for C2 tyres.

4.3.	Regression analysis of rolling sound measurements

The tyre-road rolling sound level LTR,ref,vTR,ref is determined by a regression analysis for each vehicle side separately according to:

LTR,ϑref,vTR,ref= L-slpref×v

where		L	is the mean value of the rolling sound levels Li, 
		measured in dB(A):

L=1ni=1nLTR,i,ref

n	is the measurement number (n ≥ 6),
	 	is the mean value of logarithms of speeds vi:

=1ni=1ni
with 	i=lgvivTR,ref

slpref	is the slope of the regression line in dB(A):

slpref=i=1ni-LTR,i,ref-Li=1ni-2

4.4.	The final result LTR,ref,vTR,ref for the reference speed vTR,ref and the slope slpref of the regression line shall be reported per vehicle side to the first decimal place.

5.	Test report

5.1.	Authority present during the tests:

5.1.1.	Name and address of applicant:

5.1.2.	Test report No.:

5.1.3.	Date of test:

5.1.4.	Location of test track:

5.1.4.1.	Date of track certification to ISO 10844:2021*:

* Delete what does not apply according to the transitional provisions in this Regulation.

5.1.4.2.	Issued by:

5.1.4.3.	Method of certification:

5.1.5.	Test vehicle

5.1.5.1.	Vehicle used for tyre testing (strike trough what is not applicable):

type approval vehicle / tyre test vehicle

5.1.5.2.	In case of a type approval vehicle

5.1.5.2.1.	Type description:

5.1.5.3.	In case of a tyre test vehicle

5.1.5.3.1.	Make, model, year, modifications, etc.:

5.1.5.3.2.	Test vehicle wheelbase: 	 mm

5.1.6.	Tyre Information

5.1.6.1.	Manufacturer and Brand Name or Trade description:

5.1.6.2.	Tyre Class:

5.1.6.3.	Category of use: (M1, N1 or N2 < 3.5 t)

5.1.6.4.	Tyre test details (front/rear axle):

5.1.6.5.	Tyre size designation:

5.1.6.6.	Tyre service description:

5.1.6.7.	Reference inflation pressure: 	 kPa

5.1.7.	Reported values

5.1.7.1.	Tyre Rolling Sound Level LTR,ref,vTR,ref (left/right side of the vehicle):	dB(A)

5.1.7.2.	Reference speed vTR,ref according to paragraph 4.1:	km/h

5.1.7.3.	Regression slopes slpref (left/right side of the vehicle): 	dB(A)/log(v)

5.1.8.	Comments (if any):

5.1.9.	Date:

5.1.9.1.	Signature:

Annex 4

Exhaust silencing systems containing acoustically absorbing fibrous materials

1.	General

Sound absorbing fibrous materials may be used in silencing systems or components thereof only if

(a)	The exhaust gas is not in contact with the fibrous materials; or if

(b)	The silencing system or components thereof are of the same design family as systems or components for which it has been proven, in the course of type approval process in accordance with the requirements of this regulation for another vehicle-type, that they are not subject to deterioration.

Unless one of these conditions is fulfilled, the complete silencing system or components thereof shall be submitted to a conventional conditioning using one of three installations and procedures described below, or - on request of the manufacturer - by removing the fibrous materials from the silencer.

1.1.	Continuous road operation for 10,000 km

1.1.1.	50 ± 20 per cent of this operation shall consist of urban driving and the remaining operation shall be long-distance runs at high speed; continuous road operation may be replaced by a corresponding test-track programme.

1.1.2.	The two speed regimes shall be alternated at least twice.

1.1.3.	The complete test programme shall include a minimum of 10 breaks of at least three hours duration in order to reproduce the effects of cooling and any condensation which may occur.

1.2.	Conditioning on a test bench

1.2.1.	Using standard parts and observing the vehicle manufacturer's instructions, the silencing system or components thereof shall be fitted to the vehicle referred to in paragraph 3.3. of this Regulation or the engine referred to in paragraph 3.4. of this Regulation. In the former case the vehicle shall be mounted on a roller dynamometer. In the second case, the engine shall be coupled to a dynamometer.

1.2.2.	The test shall be conducted in six six-hour periods with a break of at least 12 hours between each period in order to reproduce the effects of cooling any condensation which may occur.

1.2.3.	During each six-hour period, the engine shall be run, under the following conditions:

(a)	Five minutes at idling speed;

(b)	One-hour sequence under 1/4 load at 3/4 of rated maximum speed (S);

(c)	One-hour sequence under 1/2 load at 3/4 of rated maximum speed (S);

(d)	10-minute sequence under full load at 3/4 of rated maximum speed (S);

(e)	15-minute sequence under 1/2 load at rated maximum speed (S);

(f)	30-minute sequence under 1/4 load at rated maximum speed (S).

Each period shall comprise two sequenced sets of the six above-mentioned conditions in consecutive order from (a) to (f).

1.2.4.	During the test, the silencing system or components thereof shall not be cooled by a forced draught simulating normal airflow around the vehicle. Nevertheless, at the request of the manufacturer, the silencing system or components thereof may be cooled in order not to exceed the temperature recorded at its inlet when the vehicle is running at maximum speed.

1.3.	Conditioning by pulsation

1.3.1.	The silencing system or components thereof shall be fitted to the vehicle referred to in paragraph 3.3. of this Regulation or the engine referred to in paragraph 3.4. of this Regulation. In the former case the vehicle shall be mounted on a roller dynamometer.

In the second case, the engine shall be mounted on a dynamometer. The test apparatus, a detailed diagram of which is shown in Figure 1 of the appendix to this annex shall be fitted at the outlet of the silencing system. Any other apparatus providing equivalent results is acceptable.

1.3.2.	The test apparatus shall be adjusted in such a way that the exhaust-gas flow is alternatively interrupted and re-established by the quick-action valve for 2,500 cycles.

1.3.3.	The valve shall open when the exhaust-gas back pressure, measured at least 100 mm downstream of the intake flange, reaches a value of between 35 and 40 kPa. It shall close when this pressure does not differ by more than 10 per cent from its stabilized value with the valve open.

1.3.4.	The time-delay switch shall be set for the duration of gas exhaust resulting from the provisions laid down in paragraph 1.3.3. above.

1.3.5.	Engine speed shall be 75 per cent of the rated engine speed (S) at which the engine develops rated maximum net power.

1.3.6.	The power indicated by the dynamometer shall be 50 per cent of the full-throttle power measured at 75 per cent of rated engine speed (S).

1.3.7.	Any drain holes shall be closed off during the test.

1.3.8.	The entire test shall be completed within 48 hours.

If necessary, one cooling period will be observed after each hour.

Annex 4 - Appendix

# Figure 1

Test apparatus for conditioning by pulsation

1.	Inlet flange or sleeve for connection to the rear of the test exhaust system.

2.	Hand-operated regulating valve.

3.	Compensating reservoir with a maximum capacity of 40 l and a filling time of not less than one second.

4.	Pressure switch with an operating range of 0.05 to 2.5 bar.

5.	Time delay switch.

6.	Pulse counter.

7.	Quick-acting valve, such as exhaust brake valve 60 mm in diameter, operated by a pneumatic cylinder with an output of 120 N at 4 bar. The response time, both when opening and closing, shall not exceed 0.5 second.

8.	Exhaust gas evacuation.

9.	Flexible pipe.

10.	Pressure gauge.

Figure 2: Flowchart for the check on the necessity for a silencer conditioning test

Check on Necessity for a Silencer Conditioning

Does the silencer
contain a fibrous

material?

Provide documentation that the silencer is belonging to a family of silencers for which it was proven that the fibrous material will not deteriorate.

Conditioning test IS necessary

Conditioning test

IS NOT necessary

NO

YES

Is the fibrous material in contact with exhaust gas?

NO

YES

Is the silencer member of a design family for which it has been proven that the fibrous material will not deteriorate?

Provide information about the family representative silencer and the way of proof for the non-deterioration

Proceed to the 
sound emission tests

YES

NO

Perform either a conditioning test or – on request of the manufacturer – remove the fibrous material

Annex 5

Compressed air noise

1.	Method of measurement

The measurement is performed at microphone positions 2 and 6 according to Figure 1, with the vehicle stationary. The highest A-weighted sound level is registered during venting the pressure regulator and during ventilating after the use of both the service and parking brakes.

The noise during venting the pressure regulator is measured with the engine at idling speed. The ventilating noise is registered while operating the service and parking brakes; before each measurement, the air-compressor unit has to be brought up to the highest permissible operating pressure, and then the engine switched off.

2.	Evaluation of the results

For all microphone positions two measurements are taken. In order to compensate for inaccuracies of the measuring equipment, the meter reading is reduced by 1 dB(A), and the reduced value is taken as the result of measurement. The results are taken as valid if the difference between the measurements at one microphone position does not exceed 2 dB(A). The highest value measured is taken as the result. If this value exceeds the sound limit by 1 dB(A), two additional measurements are to be taken at the corresponding microphone position.

In this case, three out of the four results of measurement obtained at this position have to comply with the sound limit.

3.	Limiting value

The sound level shall not exceed the limit of 72 dB(A).

Annex 5 - Appendix

# Figure 1

# Microphone positions for measurement of compressed air noise

The measurement is performed at the stationary vehicle according to Figure 1, using two microphone positions at a distance of 7 m from the contour of the vehicles, and at 1.2 m above ground.

Annex 6

Checks on conformity of production

1.	General

These requirements are consistent with the test for checking Conformity of Production (COP) according to paragraph 8. of this Regulation.

2.	Testing procedure

The test site and measuring instruments shall be those as described in Annex 3 and Annex 10 for vehicles equipped with an ADS.

2.1.	The vehicle(s) under test shall be subjected to the test for measurement of sound of vehicle in motion as described in paragraph 3.1. of Annex 3.

For vehicles of category M1, N1 and for vehicles of category M2 having a maximum authorized mass not exceeding 3,500 kg,

(a)	the same mode, gear(s)/gear ratio(s), gear weighting factor k and partial power factor kP as determined during the type approval process may be used, provided this information is available from the type approval test report for the applicable vehicle variant of the family. If not, this information shall be determined anew. The test report shall document which way of data processing was selected;

(b)    the test mass mt of the vehicle shall be between 0.9mro ≤ mt ≤ 1.2mro.

Notwithstanding the provisions of paragraph 2.2.3.4.2. on tyre conditioning for testing, manufacturer may use a simplified conditioning according to the vehicle manufacturers specification to avoid excessive use of the tyres during the conditioning.

2.2.	Compressed air noise

Vehicles having maximum mass exceeding 2,800 kg and equipped with compressed air systems shall be subjected to an additional test for measurement of the compressed air noise as described in paragraph 1. of Annex 5.

2.3.	Additional Sound Emission Provisions (ASEP)

The vehicle manufacturer shall assess the compliance with ASEP by an appropriate evaluation (for example, but not limited to, part checks) or may perform the test described in Annex 7.

3.	Sampling and evaluation of the results

One vehicle shall be chosen and subjected to the tests set out in point 2. If the sound level of the vehicle tested does not exceed by more than 1 dB(A) the limit value specified in paragraph 6.2.2. of this Regulation, and, where appropriate, paragraph 3. of Annex 5, the vehicle type shall be considered to conform to the requirements of this Regulation.

If one of the test results does not fulfil the COP requirements of this annex and of paragraph 8. of the main body of this Regulation two more vehicles of the same type shall be tested pursuant to paragraph 2. above.

If the test results for the second and the third vehicle fulfil the COP requirements of this annex and of paragraph 8. of the main body of this Regulation, the vehicle is considered in compliance with regard to the COP.

If one of the test results of the second or third vehicle does not fulfil the COP requirements of this annex and of paragraph 8. of the main body of this Regulation the vehicle type shall be considered not to conform to the requirements of this Regulation and the manufacturer shall take the necessary measures to re-establish the conformity.

Annex 7

Measurement method to evaluate compliance with the Additional Sound Emission Provisions

Only applicable for vehicles as specified in paragraph 6.2.3. of this Regulation.

1.	General (see the flowchart in Appendix 2, Figure 1)

This annex describes a measurement method to evaluate compliance of the vehicle with the additional sound emission provisions (ASEP) conforming to paragraph 6.2.3. of this Regulation.

It is not mandatory to perform actual tests when applying for type-approval. The manufacturer shall sign the declaration of compliance set out in Appendix 1. The approval authority may ask for additional information about the declaration of compliance and carry out the tests described below.

The procedure set out in this annex requires the performance of a test in accordance with Annex 3.

If the tests according to Annex 7 are carried out in the course of type approval, all tests either for Annex 3 and for Annex 7 shall be carried out on the same test track and under similar environmental conditions.

If Annex 7 tests are carried out when type approval has already been granted, e.g. during tests for conformity of production or for in-use compliance, the tests in motion specified in Annex 3 shall be carried out with the same mode, gear(s)/gear ratio(s), gear weighting factor k and partial power factor kP as determined during the type approval process.

The test results of Annex 3 shall be used within Annex 7 without any temperature correction.

1.1.		Vehicles, when electrically propelled without internal combustion engines operating shall comply with Analysis method 1 (Slope assessment method) only.

2.	Measurement method (see the flowchart in Appendix 2, Figure 3)

2.1.	Measurement instruments and condition of measurements

Unless otherwise specified, the measurement instruments, the conditions of the measurements and the condition of the vehicle are equivalent to those specified in Annex 3, paragraphs 1. and 2.

If the vehicle has different modes which affect sound emission, all modes shall comply with the requirements in this annex. In the case where the manufacturer has performed tests to prove to the approval authority compliance with the above requirements, the modes used during those tests shall be reported in a test report.

2.2.	Method of testing

Unless otherwise specified, the conditions and procedures of Annex 3 shall be used. For the purpose of this annex, one run per test condition is measured and evaluated.

2.3.	Control Range

The ASEP requirements apply to every gear ratio κ that leads to test results within the control range as defined below.

The control range is specified as:

|  |  | For vehicles in operating conditions in accordance with Annex 7, paragraph 1.1. | For vehicles in all other operating conditions |
| --- | --- | --- | --- |
| Vehicle speed vAA_ASEP |  | vAA ≥ > 20 km/h | vAA ≥ 20 km/h |
| Vehicle acceleration aWOT_ASEP |  | aWOT ≤ 5.0 m/s2 |  |
| Engine speed nBB_ASEP |  | not applicable | nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤ 0.9 * S, whichever is the lowest |
| Vehicle speed vBB_ASEP for vehicles tested in Annex 3 with |  |  |  |  |
|  | - locked gear | vBB ≤ 80 km/h | If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but no more than 80 km/h. For any other gear, the maximum vehicle speed is 70 km/h. |
|  | - non-locked gears | vBB ≤ 80 km/h |  |
| Gear selection |  | only if applicable, e.g. for hybrid electric vehicles: gears κ ≤ gear i as determined in Annex 3 | gears κ ≤ gear i as determined in Annex 3 |
| Transmission condition for vehicles tested in Annex 3 with |  |  |  |  |
|  | - locked gear | Geari, geari-1,… | Geari, geari-1,… |
|  | - non-locked gears | Non-locked gears | Non-locked gears |

For vehicles subject to paragraph 1.1. of this Annex, the manufacturer shall take measures to achieve an acceleration within the control range in such a way that each test point P1 to P4 as specified in paragraph 2.4. of this Annex reaches the given target speed within the specified tolerance.

Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to enable a test condition within the above specified acceleration boundaries. Any measure used by manufacturers for the above-mentioned purposes shall be documented in the test report.

2.4.	Target conditions

The sound emission shall be measured in each valid gear ratio at the four test points as specified below. For all test points the boundary conditions as specified in paragraph 2.3. shall be met.

The gear ratio is valid if all four points and the anchor point meet the specifications of paragraph 2.3. above. Any gear ratio for which this criteria is not fulfilled is invalid and not analysed further.

The first test point P1 is defined by using an entry speed vAA,κ1 of 20 km/h ≤ vAA,κ1 < 20 km/h + 3 km/h.

For P1, if a stable acceleration condition according to the definition of 2.26.1 cannot be achieved, the speed vAA,κ1 shall be increased in steps of 5 km/h until a stable acceleration is reached.

For all points, stable acceleration condition according to the definition of 2.26.3. shall be verified by comparing the acceleration awot_test,AA-BB calculated between line AA’ and line BB’ with the acceleration awot_test,PP-BB calculated between line PP’ and BB’.

If the ratio awot_test,PP-BB / awot_test,AA-BB does not exceed or is equal to 1,20, then proceed with the acceleration calculation between line AA’ and Line BB’.

If the ratio awot_test,PP-BB / awot_test,AA-BB does exceed 1,20, then proceed with the acceleration calculation between line PP’ and line BB’.

In case of non-locked transmission conditions where nBB_ASEP is exceeded during the test, the following measures shall be considered separately or together:

- provisions of paragraph 2.5.1.

- increased speed in steps of 5 km/h.

The test speed for the fourth test point P4 in any gear is defined by either

- 0.95 x nBB_ASEP ≤ nBB,κ4  ≤nBB_ASEP   or

vBB_ASEP - 3 km/h ≤ VBB,κ4 ≤ VBB_ASEP with VBB_ASEP as defined in paragraph 2.3. The test speed for the other two test points is defined by the following formula:

Test Point Pj: vBB, κj = vBB,κ1 + ((j - 1) / 3) * (vBB,κ4 - vBB,κ1) for j = 2 and 3 with a tolerance of ±3 km/h

Where:

vBB,κ1 = vehicle speed at BB' of test point P1

vBB,κ4 = vehicle speed at BB' of test point P4

2.5.	Test of the vehicle

2.5.1. 		The path of the centreline of the vehicle shall follow line CC' as closely as possible throughout the entire test, starting from the approach of the reference point according to definition 2.11. of the main body to line AA' until the rear of the vehicle passes line BB'.

At line AA' the accelerator shall be fully depressed. To achieve a more stable acceleration according definition 2.26.2. or to avoid a down shift between line AA' and BB' pre-acceleration before line AA' may be used according to the provisions of paragraphs 3.1.2.1.2.1. and 3.1.2.1.2.2. of Annex 3. The accelerator shall be kept in depressed condition until the rear of the vehicle reaches line BB'.

In case of non-locked transmission conditions, the test may include a gear ratio change to a lower range and a higher acceleration. A gear change to a higher range and a lower acceleration is not allowed.

If possible, the manufacturer shall take measures to avoid that a gearshift leads to a condition not in compliance with the boundary conditions. For that, it is permitted to establish and use electronic or mechanical devices, such as alternate gear selector positions. If no such measures can be applied, the rationale shall be provided and documented in the technical report.

Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to control the downshift of gears. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

2.5.2. 		Measurements reading:

Per test point, one single run is carried out.

For every separate test run, the following parameters shall be determined and noted:

The maximum A-weighted sound pressure level of both sides of the vehicle, indicated during each passage of the vehicle according to paragraph 3.1.2.1.5. of Annex 3, shall be mathematically rounded to the first decimal place (Lwot,κj). If a sound peak obviously out of character with the general sound pressure level is observed, the measurement shall be discarded. Left and right side may be measured simultaneously or separately. For further processing, the higher sound pressure level of both sides shall be used.

The vehicle speed readings at AA', PP’ and BB' shall be rounded and reported with the first significant digit after the decimal place. (vAA,κj; vPP,κj; vBB,κj)

If applicable, the engine speed readings at BB' shall be reported as a full integer value (nBB,κj).

2.5.3. 		The calculated acceleration shall be determined in accordance to the formula in paragraph 3.1.2.1.2. of Annex 3 and reported to the second digit after the decimal place (awot,test,κj).

3.	Analysis method 1: Slope-Assessment

3.1.	Determination of the anchor point

The anchor point is the same for each gear ratio κ falling under the control range according to paragraph 2.3. The parameters for the anchor point are taken from the acceleration test of Annex 3 as follows:

In the case the test has been carried out with two gear ratios:

Lanchor is the higher sound pressure level of Lwot,(i) of left and right side of gear ratio i;

nanchor is the average of nBB,wot of the 4 runs of gear ratio i reported from Annex 3;

In the case the test has been carried out in a single gear:

Lanchor is the higher sound pressure level of Lwot of left and right side of gear ratio selected for the test;

nanchor is the average of nBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3;

For vehicles subject to paragraph 1.1. of this Annex:

Lanchor is the higher sound pressure level of Lwot of left and right sides of gear ratio selected for the test;

vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3, but limited to 60 km/h for further calculation.

aanchor is the measured acceleration and is not limited to 2.0 m/s² as required in paragraph 3.1.2.1.4.3. of Annex 3 for the determination of the kp-factor, and is the average of awot,test reported from Annex 3, as defined in paragraph 3.1.2.1.2. of Annex 3.

3.2.	Slope of the regression line for each gear ratio κ

The sound measurements shall be evaluated as function of engine speed according to paragraph 3.2.1.

For vehicles subject to paragraph 1.1. of this annex, the sound measurements shall be evaluated as function of vehicle speed and Slopeκ shall be set to a constant value of 0.25 dB/(km/h) for further calculation.

3.2.1.	Calculation of the slope of the regression line for each gear ratio κ

The linear regression line is calculated using the anchor point and the four correlated additional measurements with the results for engine speeds and sound levels as reported under 2.5.2. of this annex.

(in dB(A)/1,000 min-1)

With	  and    ;

Where nj = engine speed measured at line BB'

3.2.2.	Slope of the regression line for each gear ratio κ

The slopeκ of a particular gear for the further calculation is the derived result of the calculation in paragraph 3.2.1. rounded to the first decimal place, but not higher than 5 dB(A)/1,000 min-1.

In case of non-locked conditions, if Slopeκ  < 0, the selected transmission setup is not valid. In that case the Lurban-Assessment as specified in paragraph 4. shall be applied.

3.3.	Calculation of the linear sound level increase expected for each measurement

The sound level LASEP,κj for measurement point j and gear ratio κ shall be calculated using the engine speeds measured for each measurement point, using the slope specified in paragraph 3.2. above to the specific anchor point for each gear ratio.

For nBB_κ,j ≤ nanchor:LASEP_κ,j = Lanchor + (Slopeκ - Y) * (nBB_κ,j - nanchor) / 1,000

For nBB_κ,j > nanchor:LASEP_κ,j = Lanchor + (Slopeκ + Y) * (nBB_κ,j - nanchor) / 1,000

Where Y= 1

For vehicles subject to paragraph 1.1. of this Annex:

The sound level LASEP,κ,j for measurement point j and gear ratio κ shall be calculated using the vehicle speed measured for each measurement point, using the constant slope specified in paragraph 3.2.2. above to the specific anchor point for each gear ratio.

For vBB_κ,j ≤ vanchor:

LASEP κ,j=Lanchor+Slopeκ-Y×(vBB_κ,j- vanchor)

For vBB_κ,j > vanchor:

LASEP κ,j=Lanchor+Slopeκ×vBB_κ,j- vanchor

Where Y= 0.05 dB/(km/h)

3.4.	Calculation of the sound level increase regarding performance

For tests of purely electrically propelled vehicles, the sound level ΔLASEP,j shall be calculated:

For vBB, test, j × awot, test, j≤ vanchor× aanchor:

ΔLASEPj =0 dB

For vBB test,j × awot, test, κ,j> vanchor× aanchor :

ΔLASEP,j =6 dB ×log10 vBB test,j ×awot,test,jvanchor× aanchor

For all other tests the sound level ΔLASEP,j is set to 0 dB.

3.5.	Additional Samples

On request of the type approval authority, two additional runs within the boundary conditions according to paragraph 2.3. of this annex shall be carried out.

For vehicles subject to paragraph 1.1. of this annex, the additional runs can be selected under any operating condition within the control range, inclusive partial load acceleration or constant speed."

3.6.	Specifications

Every individual sound measurement shall be evaluated.

The sound level of every specified measurement point shall not exceed the limits given below:

Lκj - ΔLASEP,j ≤ LASEP_κ.j + x

With:

x =	2 dB+ (limit value - Lurban),

and only for vehicles

- tested with non-locked transmission conditions given by multiple gear ratios, or

- having multiple electric propulsion sources or

- EV and HEV when electrically propelled without internal combustion engines operating having an Lcrs_rep greater than Lwot_rep of Annex 3:

x = 3 dB + (limit value - Lurban) of Annex 3.

If at any point the measured sound level exceeds the limit, two additional measurements at the same point shall be carried out to verify the measurement uncertainty. The vehicle is still in compliance with ASEP, if the average of the three valid measurements at this specific point fulfils the specification.

4.	Analysis method 2: Lurban Assessment

4.1.	General

This evaluation procedure is an alternative selected by the vehicle manufacturer to the procedure described in paragraph 3. of this annex and is applicable for all vehicle technologies. It is the responsibility of the vehicle manufacturer to determine the correct manner of testing. Unless otherwise specified, all testing and calculation shall be as specified in Annex 3 to this Regulation.

The measurement method is defined in paragraph 2. Each testing point shall be evaluated individually.

4.2.	Calculation of ΔLurban_ASEP

4.2.1.	Data-processing

From any Lwot_ASEP as measured according to this annex, ΔLurban_ASEP shall be calculated as follows:

(a)	Calculate awot_test_ASEP using acceleration calculation from paragraph 3.1.2.1.2.1. or 3.1.2.1.2.2. of Annex 3 to this Regulation, as applicable;

(b)	Determine the vehicle speed (vBB_ASEP) at BB during the Lwot_ASEP test;

(c)	Calculate kP_ASEP as follows:

kP_ASEP = 1 - (aurban / awot_test_ASEP)

Test results where awot_test_ASEP are less than aurban shall be disregarded.

(d)	Calculate Lurban_measured_ASEP as follows:

Lurban_measured_ ASEP = Lwot_ASEP - kP_ASEP * (Lwot_ASEP - L_crs rep)

For further calculation, use the Lurban from Annex 3 to this Regulation without rounding, including the digit after the decimal (xx.x).

(e)	Calculate Lurban_normalized to normalize the speed from vBB_ASEP to 50 km/h as follows:

Lurban_normalized = Lurban_measured_ASEP - (0.15 * (V_BB_ASEP - 50))

(f)	Calculate the deviation ΔLurban_ASEP relative to Lurban as follows:

ΔLurban_ASEP = Lurban_normalized - Lurban –4.2.2. 		Specifications

Compliance with limits:

ΔLurban_ASEP shall be less than or equal to 3.0 dB(A) + limit value - Lurban

5.	Reference sound assessment (see the flowchart in Appendix 2, Figure 2)

5.1.	General

The reference sound can be obtained by simulation or from direct measurement. The result of one assessment method has to comply with the specification of paragraph 5.4.

5.1.1.		Simulation method

For simulation, the reference sound is assessed at a single point in one discrete gear, simulating an acceleration condition assuming an exit speed vBB’ equal to 61 km/h. The sound compliance is calculated using the slope results of paragraph 3.2.2.

If the result of slope of 3.2.2. is not available for the gear specified in paragraph 5.2, this slope of the missing gear can be determined according to paragraphs 2.4., 3.1.  and 3.2.

5.1.2.		Direct measurement method

For direct measurement, the reference sound is assessed at a single run in an acceleration condition started at line AA’ as specified in paragraph 2.5. The gear shall be as specified in paragraph 5.2. for vehicles tested in locked condition or in a gear selected position for normal driving as specified by the manufacturer for vehicles tested in non-locked condition.

The target test speed vAA is equal to 50 km/h ± 1 km/h unless vBB exceeds 61 km/h.

If vBB exceeds 61 km/h, the target test speed vBB shall be set to 61 km/h ± 1 km/h. The entry speed shall be adjusted to achieve the target test speed.

5.2.	The determination of gear α is as follows:

For vehicles tested in locked condition

-	α = 3 for manual transmission and for automatic transmission with up to 5 gears;

-	α = 4 for manual transmission and for automatic transmission with 6 or more gears. If the acceleration calculated from AA to BB + vehicle length in gear 4 exceeds 1.9 m/s², the first higher gear α > 4 with an acceleration lower than or equal to 1.9 m/s² shall be chosen. If there is no gear with an acceleration less than or equal to 1.9 m/s² available, the highest available gear shall be chosen.

For vehicles tested under non-locked condition, the gear ratio for further calculation shall be determined from the acceleration test result in Annex 3 using the reported engine speed and vehicle speed at line BB'.

5.3. 		Data-processing for simulation assessment5.3.1.	Determination of reference engine speed nBB’_ref_α

The reference engine speed, nBB’_ref_α shall be calculated using the gear ratio of gear α at the reference speed of vBB’_ref = 61 km/h.

5.3.2.		Calculation of Lref

Lref = Lanchor + Slopeα  * (nBB’_ref_α  - nanchor) / 1,000

5.4. 		Specifications

For vehicles of category M1, Lref shall be less than or equal to 76 dB(A).

For vehicles of category M1 fitted with a manual  transmission having more than four forward gears and equipped with an engine developing a rated maximum net power greater than 140 kW (according to Regulation No. 85) and having a maximum-power/maximum-mass ratio greater than 75, Lref shall be less than or equal to 79 dB(A).

For vehicles of category M1 fitted with an automatic transmission having more than four forward gears and equipped with an engine developing a rated maximum net power greater than 140 kW (according to Regulation No. 85) and having a maximum-power/maximum-mass ratio greater than 75, Lref shall be less than or equal to 78 dB(A).

For vehicles of category N1 with a technically permissible maximum laden mass below 2,000 kg, Lref shall be less than or equal to 78 dB(A).

For vehicles of category N1 with a technically permissible maximum laden mass above 2,000 kg and below 3,500 kg, Lref shall be less than or equal to 79 dB(A).

For vehicles of category M1 and N1 equipped with a compression-ignition and direct injection internal combustion engine, the sound level shall be increased by 1 dB(A).

For vehicles of category M1 and N1 designed for off-road use and with a technically permissible maximum laden mass above 2 tonnes, the sound level shall be increased by 1 dB(A) if they are equipped with an engine having a rated maximum net power of less than 150 kW (according to UN Regulation No. 85) or by 2 dB(A) if they are equipped with an engine having a rate maximum net power of 150 kW (according to UN Regulation No. 85) or higher.

Annex 7 – Appendix 1

Statement of compliance with the additional sound emission provisions

(Maximum format: A4 (210 x 297 mm))

(Name of manufacturer) attests that vehicles of this type 	 (type with regard to its sound emission pursuant to Regulation No. 51) comply with the requirements of paragraph 6.2.3. of Regulation No. 51.

(Name of manufacturer) makes this statement in good faith, after having performed an appropriate evaluation of the sound emission performance of the vehicles.

Date:

Name of authorized representative:

Signature of authorized representative:

Annex 7 – Appendix 2

Figure 1

Flowchart for the assessment concept for ASEP according to Annex 7

ASEP Annex 7

Statement of Compliance or Test

Manufacturer Statement of Compliance

Perform Type Approval Test according to Annex 3; Report Lwoti, nBB_woti,

Establish Test Conditions According to 
Paragraphs 2.1. and 2.2.

Perform ASEP Tests 
Within the Control Range According to 
Paragraphs 2.3. to 2.5.

Select Analysis Method

Assessment According

to Paragraph 4.

"Lurban-Assessment"

Assessment According

to Paragraphs 3.

"Slope-Assessment"

Evaluation According

to Paragraph 5.

"Reference Sound Assessment"

Manufacturer Statement of Compliance Based on the ASEP Tests

Statement

Test

Figure 2

Flowchart for the vehicle sound assessment according to Annex 7, paragraph 5. "Reference sound assessment"

ASEP - Annex 7

(Paragraph 5.)

"Reference Sound Assessment"

Test or Calculation?

Determine the
correct gear  as given in

paragraph 5.2.

Determine the
target speed according 
to paragraph 5.1.2.

Determine the
correct gear  as given in

paragraph 5.2.

Perform test and
report the maximum 
sound level

Sound level < Limit ?

Vehicle NON-Compliant with provisions of the

"Reference Sound Assessment"

Vehicle Compliant with provisions of the

"Reference Sound Assessment"

Determine nref,

for specified gear according 5.3.1.

Take or determine slope for gear  according to paragraphs 2.4., 3.1. up to 
3.2.2.

CALC

TEST

YES

Calculate sound level according to 
paragraph 5.3.2.

NO

Figure 3

Flowchart for the determination of the individual test points Pj according to Annex 7, paragraph 2. "Measurement method"

Choose lowest gear 
(e.g. first gear)

Gear  ≤ i

Test P1

Target : VAA = 20 km/h 
(or 25, 30, … km/h)

ASEP Annex 7

(Paragraph 2.)

Measurement method

nBB <  nBB_ASEP

or vBB   <  80 km/h

YES

Move on to

Paragraph 3.

"Slope-
Assessment"

or

Paragraph 4.

"Lurban-
Assessment"

Lowest valid gear

Test P4

Target : 80 km/h

or nBB_ASEP

Test P4

Target : 70 km/h

or nBB_ASEP

Test P2 and P3

Target calculated using P1 and P4 measurement

awot< 5m/s²

awot < 5m/s²

NO

Non Valid 
Gear

Select next 
higher gear

Valid Gear , report data for
P1 to P4

YES

YES

NO

NO

YES

NO

YES

NO

awot < 5m/s²

NO

YES

Non Valid 
Gear

Annex 8

Indoor testing

Indoor testing is only for Annex 3 and Annex 7 measurements.

1.		Documentation for indoor application

Documentation shall include:

(a)	Validation of facility, e.g. free field propagation, dyno and air handling background noise level, dyno dynamic performance, software.

(b)	Procedures to be applied for indoor testing, e.g. dyno and software set-up, loading and tie-down, air-handling and vehicle’s temperature management.

(c)	Coast down and tyre sound level data used for calculation of dynamometer load coefficients and tyre sound data used for determination of final reported results.

(d)	Test results on a representative selection of the manufacturer’s production to demonstrate that indoor testing delivers comparable results as outdoor testing within acceptable accuracy.

2.		Vehicle tested indoor using Variant A

Indoor pass-by test is simulated by measurement of power train sound on the dynamometer and energetical addition of the tyre/road sound (measured separately on an outdoor test track).

2.1.	 	General

This method is a combination of indoor testing (power train sound) and outdoor testing (tyre/road sound). It is not necessary to repeat the measurement of the tyre/road sound every time a vehicle is tested. The data of several tyres can be stored in a database and a matching data set from the database can then be used for the test.

2.2.	 	Power train sound

It shall be ensured that there is no remaining tyre/road sound affecting the measurements. In any case it shall be ensured that the remaining tyre/road sound shall be at least 10 dB below the maximum A-weighted sound pressure level produced by the vehicle under test. If this condition cannot be fulfilled, a correction shall be carried out. This correction procedure is described in ISO 362-3:2022 Annex B, paragraph B.6.

The vehicle shall be measured according to the operating condition specified in paragraphs 3.1.2.1. or 3.1.2.2. of Annex 3 of this Regulation.

2.3.  		Tyre/road sound

The measurements of the tyre/road sound shall be performed on a test track as described paragraph 2.1.1. of Annex 3 of this Regulation. The evaluation of tyre/road sound consists of two procedures, namely:

(a) Evaluation of free rolling sound;

(b) Evaluation of tyre/road sound including torque influence which can be derived from a) by a simplified method.

All conditions for evaluation of tyre/road sound shall be done according to paragraph 3. of this Annex.

2.4.  		Calculation of the total vehicle sound

The total vehicle sound is the energetical sum of tyre/road sound and power train sound. This calculation shall be carried out for each single run as describe in ISO 362-3.2022, paragraph 10.2.4.

3.	Procedure for measurement, evaluation, and calculation of tyre/road sound when using variant A

All conditions for evaluation of tyre/road sound, free rolling sound, and torque influence are described in ISO 362-3:2022, Annex B.

4. 		Adjustment of room dimensions

To cater for the smaller size test rooms, the maximum levels shall be evaluated with caution though to avoid missing them according to ISO 362-3:2022, Annex F.

Annex 9

Measurement method to evaluate compliance with the Real Driving Additional Sound Emission Provisions (RD-ASEP)

1.	General

The Real Driving Additional Sound Emission Provisions (RD-ASEP) apply only to vehicles of categories M1 and N1 equipped with:

-	an internal combustion engine (ICE) for propulsion of the vehicle, or

-	any other propulsion technology fitted with an exterior sound enhancement system.

1.1.	Notwithstanding the provisions of Annex 7, paragraph 1., section 4 inclusive its footnote, tests according to Annex 9 done in the course of type approval shall be witnessed by the Authority present during the tests.

Tests shall be carried out on the same test track and under similar environmental conditions subject to the limitations in paragraph 3.3.

1.2.	Exemptions

Notwithstanding the requirements above, vehicles which have no ICE for propulsion are exempted from RD-ASEP, if a sound enhancement system is fitted to the vehicle solely for the purpose of fulfilling the provisions of UN Regulation No. 138, and the sound emitting device (AVAS) does not emit a sound pressure level of more than 75 dB(A)  under any operation conditions exceeding the specification range of UN Regulation No.138.

1.3.	All symbols’ abbreviations and acronyms used in this Annex are listed and defined in Appendix 3 to this Annex.

1.4.	All formulas used in this Annex and Appendix 1 are listed in Appendix 4 to this Annex.

2.	Definitions

2.1.	"Exhaust silencing system with variable geometry" means a silencing system, not including pressure charging, containing one or more active, passive, or self-actuated moving parts or devices.

These parts or devices will cause a change of the gas flow through the exhaust silencing system and result in a change of the sound reduction performance, by opening or closing one or more valves in the exhaust gas flow as a function of varying driving or engine conditions (engine speed, load, vehicle speed, etc.).

Active devices mean actuators, controlled by any means.

Passive or self-actuated devices mean units controlled by exhaust flow.

2.2.	"Exterior sound enhancement system" means a system that is installed to a vehicle for producing exterior sound, such as but not limited to sound actuators, either integrated into an exhaust silencing system or mounted as an individual unit.

2.3.	"Deceleration" means the vehicle deceleration stipulated by the release of the acceleration control unit only, without any driver applied braking (service brake, retarder, parking brake, etc…).

2.4.	"Performance" means the product of acceleration and vehicle speed as quantity of the achieved vehicle performance.

2.5.	"Power trains" means a propulsion system as combination of the energy storage system, the energy supply system and the powertrain according to the UN Mutual Resolution No. 2 (for example PEV, HEV, FCHEV).

3.	Facilities

3.1.	Due to spatial limitations of test facilities not every test condition may be performed on every test facility.

3.2.	Notwithstanding such restrictions, the RD-ASEP tests shall be done on these test facilities.

3.3.	Tests for Annex 9 may be carried out on different test facilities in case of facility limitation(s). However, it is recommended to carry out all tests on one test facility and under similar environmental conditions to reduce measurement uncertainties.

4.	Measurement method

4.1.	Measurement instruments and condition of measurements

Unless otherwise specified, the measurement instruments, the conditions of the measurements and the condition of the vehicle are equivalent to those specified in Annex 3, paragraphs 1. and 2.

4.2.	Method of testing

Unless otherwise specified, the conditions and procedures of Annex 3, paragraphs 3.1. to 3.1.2.1.2.2. shall be used. For the purpose of this Annex, single test runs are performed and evaluated.

4.3.	Control range

A measurement for RD-ASEP is valid, if all parameters are within the specifications of the table below during the test run between lines AA' and BB'.

| Parameter | Minimum | Maximum |
| --- | --- | --- |
| Vehicle Speed | > 0 km/h at line AA' | 100 km/h at line BB' |
| Acceleration | 0 m/s² | 4 m/s² |
| Performance | 0 m²/s³ | 35 m²/s³ |
| Gear | ANY for forward driving |  |
| Mode | ANY |  |

In any operation condition, the engine speed of a vehicle, which can be propelled with an ICE operating, is limited to 80% of S.

4.4.	Target operation conditions

The target operation condition for a single test run is randomly selected by the authority present during the tests carried out for type approval.

The operation condition during measurements per run is defined by

-	the gear selector position,

-	the vehicle mode,

-	the vehicle entry speed at line AA', and

-	the percentage of accelerator depression, either for constant speed or in steps of 25 % acceleration depression.

The manufacturer may establish a mechanical or electronic device to enable the requested percentage of accelerator depression.

The requested accelerator depression shall be achieved during the test with a tolerance of ± 10 % of full range.

The requested vehicle speed at AA' shall be achieved during the test with a tolerance of ± 3 km/h.

If under the chosen operation condition, a stable acceleration according to paragraph 2.26.1. of the main body cannot be enabled, the authority present during the test shall decide how to modify the operation condition (e.g. different gear selector position, speed, acceleration, driving mode).

The chosen operation condition shall result in a run within the control range. Each operation condition shall differ substantially from the test conditions of Annex 3 and all other operation conditions already chosen for this type approval already carried out under this Annex. For vehicles with a combustion engine for propulsion, the operating conditions shall be chosen to differ substantially in engine speed.

The number of operation conditions per vehicle in total is dependent on the vehicles technology and provided by the table below.

|  | D-Range | M (locked) |
| --- | --- | --- |
| Automatic Transmissions (lockable) | 5 | 10 |
| Automatic Transmissions (non-lockable) | 15 | n.a.(*) |
| Vehicles with only one gear | 15 | n.a.(*) |
| Manual Transmissions | n.a.(*) | 15 |

(*) Not applicable

The operation conditions and test results shall be entered into the test report sheet according to the table of the Appendix 5 to this Annex.

4.5.	Test of the vehicle

4.5.1. 	The path of the centerline of the vehicle shall follow line CC' as closely as possible throughout the entire test, starting from the approach of the reference point according to definition in paragraph 2.11. of the main body to line AA' until the rear of the vehicle passes line BB' + 20 m.

The accelerator shall be positioned such to achieve the requested operation condition for this run latest when the reference point of the vehicle reaches line AA'. The accelerator shall be kept in its position until the rear of the vehicle passes line BB'. The accelerator shall then be fully released between BB' and BB'+ 5 m and kept in this released position until the rear of the vehicle passes the line BB'+20 m.

4.5.2. 	Non-locked transmissions

In case of non-locked transmission conditions, the test may include a gear change to a lower range and a higher acceleration under accelerated conditions.

A gear change to a higher range could occur under cruise and low load acceleration conditions. Such upshifts shall be avoided. The authority present during the test shall modify operation conditions so that these upshifts are avoided between AA' and BB'.

In the approach to line AA', the vehicle shall be driven in a way to allow the transmission to stabilize the gear.

4.5.3. 	Measurement readings

Per operation condition, one test run is carried out.

If a measurement within the control range is invalid due to background noise disturbances, wind gusts or other reasons, the measurement shall be discarded and repeated.

For every test run, the following parameters shall be determined and noted:

-	The maximum A-weighted sound pressure level of both sides of the vehicle, indicated during each passage of the vehicle between the two lines AA' and BB' + 20 m, shall be measured and shall be mathematically rounded to the first decimal place (LTEST).

If a sound peak obviously out of character with the general sound pressure level is observed, the measurement shall be discarded.

For further processing, the higher sound pressure level of both sides shall be used.

-	The vehicle speed reading at lines BB', when the rear end of the vehicle passes this line, shall be rounded and reported with the first significant digit after the decimal place (vBB’_TEST).

-	If applicable, the engine speed readings at line AA' and BB' shall be rounded to 10 min-1 and reported (nAA'_TEST; nBB'_TEST).

All measured values shall be entered into the test report sheet according to the table of Appendix 5 to this Annex.

4.5.4. 	Calculated values

All calculated values shall be entered into the test report sheet according to the table of Appendix 5 to this Annex.

4.5.4.1. 	Acceleration a

The accelerations shall be calculated between lines PP' to BB', in accordance with the formula provided in paragraph 3.1.2.1.2.2. of Annex 3 and be reported to the second digit after the decimal place (aTEST) as results.

4.5.4.2. 	Performance va

The performance shall be calculated from the reported vehicle speed at line BB' and the acceleration result from paragraph 4.5.4.1. and rounded to the first digit after the decimal place.

4.5.4.3. 	Expected sound pressure level LTEST_EXP

For the calculation of the expected sound pressure level per test run, the measured values according to paragraph 4.5.3. and calculated values according to paragraphs 4.5.4.1. and 4.5.4.2. shall be used. All calculations are done according to Appendix 1 to this Annex.

5.	Compliance assessment

5.1.	Case 1

The compliance of the vehicle is acceptable if the measured sound pressure levels of all valid test runs are lower than or equal to the expected sound pressure levels of paragraph 4.5.4.3.

LTEST ≤ LTEST_EXP

5.2.	Case 2

If not more than two valid runs of the specified runs exceed the expected sound pressure level of paragraph 4.5.4.3. by not more than 2 dB the compliance of the vehicle is acceptable.

5.3.	Case 3

If more than two valid runs of the specified runs exceed the expected sound pressure level of paragraph 4.5.4.3. then the vehicle is non-compliant with RD-ASEP.

5.4.	Case 4

If one or more valid runs exceed the expected sound pressure level of paragraph 4.5.4.3. by more than 2 dB, the vehicle is non-compliant with RD-ASEP.

5.5	The case of compliance according to this paragraph 5 and the final result (compliance yes/no) have to be mentioned in the Test report of Appendix 5 to this Annex 9.

Annex 9 – Appendix 1

Sound Expectation Model

1.	General

All the formulas and values coming from Annex 3 are identified with a suffix of ‘ANCHOR’ in the symbols.

For example, vTEST_ANCHOR in Annex 9 is equal to vTEST from Annex 3.

2.	Extraction of parameters from measurements according to Annex 3

2.1.	The procedure set out in this Annex requires the performance of tests in accordance with Annex 3.

2.2.	Determination of reference data from Annex 3:

2.2.1.	The necessary reference data to establish the sound expectation model are taken from the pass-by and from the cruise-by test of one gear of the Annex 3 tests.

2.2.1.1.	In the case the test has been carried out with two gear ratios, the reported parameters for gear i shall be selected. In the case the test has been carried out in a single gear, the reported parameters for this single gear shall be selected.

2.2.1.2.	The parameters taken from Annex 3 tests are in all cases the arithmetic average of the four valid runs as determined by the measurements in Annex 3. All values shall be taken over without any temperature or test track correction. The engine speed is not requested for Annex 3. However, for the purpose of Annex 9 it needs to be measured.

The parameters to be reported from the acceleration test are:

-	The sound pressure level LACC_ANCHOR which is the higher value of the measured sound pressure levels of the left and right side of the vehicle, rounded to the first decimal. If applicable, the value shall be corrected according to Table 1 of the Appendix 1 to Annex 3, measures No. 3, sub Nos. 1 or 2.

-	The vehicle speed vBB'_ACC_ANCHOR when the rear of the vehicle passes line BB', rounded to the first decimal.

-	The engine speed nBB'_ACC_ANCHOR when the rear of the vehicle passes line BB', rounded to 10 min-1. If applicable, the value shall be corrected according to Table 1 of the Appendix 1 to Annex 3, measures No. 3, sub Nos. 1 or 2.

The parameters to be reported from the constant speed test are:

-	The sound pressure level LCRS_ANCHOR which is the higher value of the measured sound pressure levels of the left and right side of the vehicle, rounded to the first decimal.

-	The reference vehicle speed vTEST is 50 km/h, unless the vehicle was tested in Annex 3 at a different vehicle speed. In this case use the reported vehicle speed vBB'_CRS_ANCHOR, rounded to the first decimal.

-	The engine speed nBB'_CRS_ANCHOR when the rear of the vehicle passes line BB', rounded to 10 min-1.

2.3.	Selection of parameter coefficients

The necessary coefficients are dependent on the vehicle design and listed in the table in Appendix 2 to this Annex.

2.3.1.	Discrete Determination of the factor x

On request of the manufacturer the factor x may be determined by discrete coast-down measurement to determine LREF_TR directly, according to Appendix 3 of Annex 3 to this Regulation for the reference vehicle speed. Rounding and temperature correction shall not apply.

2.4.	Calculate the Reference Tyre/Rolling Sound Level LREF_TR

LREFTR=10 ×lgx×100.1×LCRSANCHOR

2.5.	Calculate the Reference Power Train Mechanics Sound Level LREF_PT

LREFPT=10 ×lg1-x×100.1×LCRSANCHOR

2.6.	Calculate the Reference Dynamic Sound Level LREF_DYN

LREF_DYN=LREF_PT -15 dB(A)

2.7.	Determine the Vehicle Dynamic Delta Sound Level ∆LDYN

If the arithmetic sound level difference between the reported acceleration sound level LACC_ANCHOR and the reported constant speed sound level LCRS_ANCHOR is at least 1.1 dB(A) or higher, the vehicle dynamic delta sound level ∆LDYN is calculated by :

∆LDYN=10 ×lg100.1× LACCANCHOR-100.1× LREFTRADJ - 100.1×LREFPTADJ-LREFDYN

LREF_TR_ADJ=TR_LO × lg(vBB'_ACC_ANCHORvTEST)+ LREF_TR

LREF_PT_ADJ=PT_LO × lg(nBB'_ACC_ANCHOR+nSHIFT_PT)(nBB'_CRS_ANCHOR + nSHIFT_PT)+ LREF_PT

If the arithmetic sound level difference between the reported acceleration sound level LACC_ANCHOR and the reported constant speed sound level LCRS_ANCHOR is less than 1,1 dB, the vehicle dynamic delta sound level ∆LDYN is set to 10 dB.

∆LDYN=10 dB

In cases where the arithmetic sum of sound energy adjusted reference tyres rolling sound LREF_TR_ADJ and the adjusted reference power train LREF_PT_ADJ  is equal or greater than the sound energy of the anchor point LACC_ANCHOR , the vehicle dynamic delta sound level ∆LDYN is set to 10 dB:

If

100.1x LREF_TR_ADJ  + 100.1xLREF_PT_ADJ  ≥ 100.1x LACC_ANCHOR

then ∆LDYN  = 10 dB

2.8.	Having established the sound expectation model for a given vehicle based on its particular pass-by test results according to Annex 3 of this Regulation, proceed to the single point evaluation for each test run performed according to paragraphs 4.4. and 4.5. of Annex 9.

3.	Calculation of the expected sound level LTEST_EXP

3.1.	For each single test run, performed for the purpose of Annex 9, an expected sound level LTEST_EXP shall be calculated.

3.2.	Necessary input data for the sound model are taken from the pass-by measurement according to paragraph 4.5.1. of Annex 9.

3.2.1.	For the calculation of the expected sound level the parameters listed in paragraphs 4.5.3. and 4.5.4.1. and 4.5.4.2. of Annex 9 are needed.

In addition, the vehicle speed to engine speed ratio TEST of the test run shall be determined, expressed in km/h per 1000 min-1 and calculated by the formula below, rounded to the second decimal:

LREF_PT_ADJ=PT_LO × lg(nBB'_ACC_ANCHOR+nSHIFT_PT)(nBB'_CRS_ANCHOR + nSHIFT_PT)+ LREF_PT

3.2.2.	Virtual engine speed for vehicles without internal combustion engine

When testing vehicles without an internal combustion engine for direct forward propulsion, an engine speed information will not be available. In such cases the engine speed is simulated on the basis of the measured vehicle speed vBB'_TEST by using a virtual uniform gear ratio of 30 km/h per 1000 min-1.

nBB'_TEST=vBB'_TEST30×10003.2.3.

Virtual engine speed for hybrid electric vehicles

In case that an internal combustion engine is mechanically coupled with drive axle whenever internal combustion engine is operating, this paragraph shall be applied.

In case of the other HEV systems, paragraph 3.2.4. shall be used.

Hybrid electric vehicle may have been tested in Annex 3 partly or fully in electric condition. For evaluation according to RD-ASEP, engine speeds and, if applicable corrected sound pressure levels, will have to be assigned to the cruise and the acceleration test.

3.2.3.1.	Case 1 – Internal combustion engine is operational during acceleration test and constant speed test:

3.2.3.1.1.	Assignment of engine speed

For the acceleration test and the constant speed test, use the engine speed information from the test result of Annex 3.

3.2.3.1.2.	Adjustment of sound pressure level

No adjustment is applied.

3.2.3.2.	Case 2 – Internal combustion engine is operational during acceleration test but not during the constant speed test:

3.2.3.2.1.	Assignment of engine speed

For the acceleration test, use the engine speed information from the test result of Annex 3.

For the constant speed test, determine the highest gear in which the vehicle can drive at the target speed of the vehicle vTEST (usually 50 km/h) as selected for the constant speed test in Annex 3. Calculate the engine speed with the gear ratio of that gear.

3.2.3.2.2.	Adjustment of sound pressure level

No adjustment is applied to the acceleration test result.

The adjusted cruise test result LCRS_ANCHOR' is determined by:

LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)

3.2.3.3.	Case 3 – Internal combustion engine is operational during constant speed test but not during the acceleration test

3.2.3.3.1.	Assignment of engine speed

For the constant speed, use the engine speed information from the test result of Annex 3.

For the acceleration test, determine the highest gear that provides an acceleration greater than the reference acceleration aACC_REF but not exceeding 2.0 m/s². Calculate the engine speed with the gear ratio of that gear.

3.2.3.3.2. 	Adjustment of sound pressure level

No adjustment is applied to the constant speed test result.

The adjusted sound pressure level for the acceleration test is determined by:

LACC_ANCHOR'=Limit-kP×LCRS_ANCHOR/(1-kP)

where Limit is the applicable limit value for this vehicle type according to paragraph 6.2.2. of the main body and kP is the determined kP-factor from the Annex 3 test.

3.2.3.4.	Case 4 – Internal combustion engine does neither operate during the acceleration test nor the constant speed test

3.2.3.4.1.		Assignment of engine speed

For the constant speed test, determine the highest gear in which the vehicle can drive at the target speed of the vehicle vTEST (usually 50 km/h) as selected for the constant speed test in Annex 3. Calculate the engine speed with the gear ratio of that gear.

For the acceleration test, determine the highest gear that provides an acceleration greater than the reference acceleration aACC_REF but not exceeding 2.0 m/s². Calculate the engine speed with the gear ratio of that gear.

3.2.3.4.2.	Adjustment of sound pressure level

The adjusted sound pressure level for the constant speed test is:

LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)	The

adjusted sound pressure level for the acceleration test is:

LACC_ANCHOR'=Limit-kP×LCRS_ANCHOR'/(1-kP)

where Limit is the applicable limit for this vehicle type according to paragraph 6.2.2. of the main body and kP is the determined kP-factor from the Annex 3 test.

3.2.4.	Virtual engine speed for hybrid electric vehicle the other system than paragraph 3.2.3.

3.2.4.1.	Case 1 – Internal combustion engine is operational during acceleration test and constant speed test

3.2.4.1.1.	Assignment of engine speed

For the acceleration test and the constant speed test, use the engine speed information from the test result of Annex 3.

3.2.4.1.2.	Adjustment of sound pressure level

No adjustment is applied

3.2.4.2.	Case 2 – Internal combustion engine is operational during acceleration test but not during the constant speed test

3.2.4.2.1. 	Assignment of engine speed

For the acceleration test, use the engine speed information from the test result of Annex 3.

For the constant speed test, determine a virtual uniform gear ratio of 30 km/h per 1000 min-1 at the target speed of the vehicle vTEST as selected for the constant speed test in Annex 3. Calculate the engine speed with the gear ratio of that gear.

3.2.4.2.2.	Adjustment of sound pressure level

No adjustment is applied to the acceleration test result.

The adjusted cruise test result LCRS_ANCHOR' is determined by:

LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)

3.2.4.3.	Case 3 – Internal combustion engine is operational during constant speed test but not during the acceleration test

3.2.4.3.1. 	Assignment of engine speed

For the constant speed, use the engine speed information from the test result of Annex 3.

For the acceleration test, determine a virtual uniform vehicle speed to engine speed ratio of 20 km/h per 1000 min-1. Calculate the engine speed with the vehicle speed vBB‘_ACC_ANCHOR.

nACC_ANCHOR=vBB'_ANCHOR20×1000

3.2.4.3.2.	Adjustment of sound pressure level	
No adjustment is applied to the constant speed test result.

The sound pressure level for the acceleration test is determined by:

LACCANCHOR'=Limit-kP×LCRSANCHOR1-kP

where Limit is the applicable limit for this vehicle type according to paragraph 6.2.2. of the main body and kP is the determined kP-factor from the Annex 3 test.

3.2.4.4.	Case 4 – Internal combustion engine is neither operational during the acceleration test nor during the constant speed test

3.2.4.4.2.	Assignment of engine speed

For the constant speed test, determine a virtual uniform vehicle speed to engine speed ratio of 30 km/h per 1000 min-1 at the target speed of the vehicle vTEST as selected for the constant speed test in Annex 3. Calculate the engine speed with the vehicle speed.

nCRS_ANCHOR=vTEST30×1000

For the acceleration test, determine a virtual uniform vehicle speed to engine speed ratio of 20 km/h per 1000 min-1. Calculate the engine speed with the vehicle speed:

nACC_ANCHOR=vTEST20×1000

3.2.4.4.3.	Adjustment of sound pressure level

The adjusted sound pressure level for the constant speed test is:

LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)

The adjusted sound pressure level for the acceleration test is:

LACC_ANCHOR'=Limit-kP×LCRS_ANCHOR'/(1-kP)

where Limit is the applicable limit for this vehicle type and kP is the determined kP-factor from the Annex 3 test.

3.2.5.	Virtual constant speed test for PMR < 25

A vehicle having a PMR lower than 25 is tested in Annex 3 without constant speed test. For the purpose of RD-ASEP a constant speed test result has to be assigned in Annex 9.

3.2.5.1.	The virtual constant speed test result LCRS_ANCHOR' is determined by

LCRS_ANCHOR'=LACC_ANCHOR-1.1 dB(A)

3.2.5.2 	Assignment of engine speed

3.2.5.2.1. 	Annex 3 acceleration test done in locked gear

For the acceleration test, use the engine speed information from the test result of Annex 3.

If an engine speed information is not available for the acceleration test result (e.g. EV or HEV), the engine speed for the acceleration test is calculated by the formula below:

nBB'_ACC_ANCHOR=vBB'_ACC_ANCHOR20×1000

For the constant speed test, the engine speed is calculated with the parameters determined above for the acceleration test with the formula below:

nBB'_CRS_ANCHOR=vTESTvBB'_ACC_ANCHOR×nBB'_ACC_ANCHOR

For constant speed test, depending on the situation different cases using internal combustion engine and/or electric engine, use the formulas of paragraph 3.2.4.

3.2.5.2.2. 	Annex 3 acceleration test done in non-locked gear or one gear

The engine speed for the constant speed test is calculated with a virtual uniform gear ratio of 30 km/h per 1000 min-1 at the target speed of the vehicle vTEST as selected for the constant speed test in Annex 3.

nBBCRSANCHOR'=vTEST30×1000

3.3.	Calculation of expected tyre rolling sound component LTR_EXP

The expected tyre rolling sound component LTR_EXP is calculated dependent on the achieved vehicle speed vBB'_TEST during the test.

For vehicles speeds up to and inclusive vTEST, LTR_EXP is calculated by:

LTR_EXP=θTR_LO×lgvBB'_TESTvTEST+LREF_TR

For vehicle speeds vBB' TEST exceeding vTEST, LTR EXP is calculated by;

LTR_EXP=θTR_HI×lgvBB'_TESTvTEST+LREF_TR

The parameters TR_LO and TR_HI are taken from the parameter table as applicable for the vehicle.

3.4.	Calculation of expected power train mechanical sound component LPT_EXP

The expected power train base mechanical sound component LPT_EXP is calculated dependent on the achieved engine speed nBB'_TEST during the test.

For engine speeds up to and inclusive nBB'_CRS_ANCHOR, LPT_EXP is calculated by;

LPT_EXP=θPT_LO×lgnBB'_TEST+nSHIFT_PTnBB'_CRS_ANCHOR+nSHIFT_PT+LREF_PT

For engine speeds exceeding nBB'_CRS_ANCHOR, LPT_EXP is calculated by;

LPT_EXP= θPT_HI ×lg((nBB'_TEST + nSHIFT_PT) / (nBB'_CRS_ANCHOR + nSHIFT_PT)) + LREF_PT

The parameters PT_LO,PT_HI and nSHIFT_PT are taken from the parameter table as applicable for the vehicle.

3.5.	Calculation of expected base dynamic sound component LDYN_EXP

The expected base dynamic sound component LDYN_EXP is calculated dependent on the achieved engine speed nBB'_TEST during the test.

For engine speeds up to and inclusive nBB'_ACC_ANCHOR, LDYN_EXP is calculated by;

LDYN_EXP=θDYN_LO×lgnBB'_TEST+nSHIFT_DYNnBB'_ACC_ANCHOR+nSHIFT_DYN+LREF_DYN

For engine speeds exceeding nBB' ACC ANCHOR, LDYN_EXP is calculated by

LDYN_EXP=θDYN_HI×lgnBB'_TEST+nSHIFT_DYNnBB'_ACC_ANCHOR+nSHIFT_DYN+LREF_DYN

The parameters DYN_LO, DYN_HI and nSHIFT_DYN are taken from the parameter table as applicable for the vehicle.

3.6.	Calculation of expected dynamic delta sound component ∆LDYN_EXP

3.6.1.	Determination of the maximum reference acceleration aMAX_REF

3.6.1.1.	The maximum reference acceleration aMAX_REF is the maximum acceleration performance determined in a low gear under full load condition.

A test run not part of the RD-ASEP assessment shall be performed to determine the maximum acceleration performance aMAX_REF of the vehicle. This value will be used in the model to determine the load achieved during a RD-ASEP test run.

This test run is recommended to be carried out in a gear ratio and at a vehicle entry speed such, that the vehicle engine speed nBB'_TEST is between the 50% of S and 80% of S. The acceleration  aTEST and the performance vaTEST may exceed the control range during this test.

This operation condition is determined by the vehicle manufacturer in agreement with the authority present during the test.

The acceleration aMAX_REF is calculated according to the provisions of paragraph 4.5.4.1. of Annex 9.

The vehicle speed to engine speed ratio TEST of this operation condition is defined as the reference vehicle speed to the engine speed ratio REF. For calculation, see paragraph 3.2.1. of this Appendix.

3.6.2.	Calculation of the partial load LOADTEST achieved during the test run

The partial load normalized with the maximum load is calculated based on the achieved acceleration aTEST, relative to the reference acceleration aMAX_i with the formula below

LOADTEST=aTESTaMAX iwhere  aMAX_i=κREF κTEST ×aMAX_REF

3.6.3.	Performance related calculations

3.6.3.1.	Calculation of the performance vaTEST

The performance achieved during the test is calculated from the achieved acceleration expressed in m/s² and the vehicle speed expressed in km/h by:

vaTEST=vBB'_TEST3,6×aTEST [m²/s³]

3.6.3.2.

Calculation of the dynamic performance component ∆LDYN_va

The dynamic performance component of the vehicle dynamic sound is calculated based on the achieved performance vaTEST relative to the achieved performances from Annex 3 Type-approval acceleration test.

vaANCHOR=vBB'_ACC_ANCHOR/3.6×aACC_ANCHOR

If the achieved performance does not exceed the reference performance vaANCHOR, the dynamic performance component ∆LDYN_va is equal zero.

If the achieved performance exceeds the reference performance, the dynamic performance component ∆LDYN_va is calculated by:

∆LDYN_va=β× lgvaTESTvaANCHOR

The parameter  is taken from the parameter table as applicable for the vehicle.

The maximum dynamic performance component ∆LDYN_va is limited to 10 dB.

3.6.3.3.	Aggregation of dynamic sound components

The final dynamic delta sound component ∆LDYN_EXP dynamic is calculated by

∆LDYN_EXP=∆LDYN+∆LDYN_va×1-α1LOADTEST+α21-α1/1+α2+0.3

The parameters 1 and 2 are taken from the parameter table as applicable for the vehicle.

3.7.	For vehicles falling under the scope of UN Regulation No. 138 equipped with a sound system covering the specification range of UN Regulation No. 138 a tolerance LAVAS on the RD-ASEP model is applied to ensure compatibility with the maximum permissible sound level emitted by the AVAS according to UN Regulation No.138.

For the vehicle speed range up to vREF of Annex 3 of this UN Regulation, the additional tolerance is dependent on the achieved vehicle speed vTEST during the RD-ASEP test.

∆LAVAS=LACC_ANCHOR-58×1-vBB'_TESTvTEST0.75

For vehicle speeds vBB’_TEST exceeding vTEST, no additional tolerance is applied, LAVAS is set to zero in that case.

3.8.	Calculation of the expected sound level LTEST_EXP

The calculation results of the paragraphs 3.3. to 3.7. are used to calculate the expected sound level for an individual run to be compared with the measured maximum sound pressure level, by the following formula:

LTEST_EXP=10×lg100.1×LTR_EXP+100.1×LPT_EXP+100.1×LDYN_EXP+∆LDYN_EXP+∆LAVAS+2dB(A)

3.9.	Proceed with the compliance assessment according to paragraph 5. of Annex 9.

Annex 9 – Appendix 2

Parameter Table for the Sound Expectation Model

The table below provides the necessary parameters for establishing the sound expectation model of Annex 9 Appendix 1. The parameters to be selected depend on the propulsion technology.

-	Column A: ICE

-	Column B: BEV, FCEV

-	Column C: HEV

|  |  |  |  | M1/N1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model Part | Parameter | Symbol | Unit | A | B | C |
| SOUND FROM TYRE ROLLING SOUND UNDER NO LOAD | Reference Vehicle Speed (as reported from Annex 3) | vTEST | km/h | 50 (min.40) | 50 (min.40) | 50 (min.40) |
|  | Tyre Rolling Sound Energy Fraction of Annex 3 Cruise Test LCRS_ANCHOR | x | % | 90 or measure | 95 | 90 or measure |
|  | Tyre Rolling Sound Slope < vTEST | TR_LO | dB | 20 | 20 | 20 |
|  | Tyre Rolling Sound Slope > vTEST | TR_HI | dB | 40 | 40 | 40 |
| SOUND FROM THE MECHANICAL SYSTEM UNDER NO LOAD | Power Train Sound Slope < nBB’_CRS’_ANCHOR | PT_LO | dB | 60 | 60 | 60 |
|  | Power Train Sound Slope > nBB’_CRS’_ANCHOR | PT_HI | dB | 115 | 85 | 115 |
|  | Form Factor for the logarithm function of the mechanical sound model | nSHIFT_PT | 1/min | 5000 | 5000 | 5000 |
| SOUND FROM DYNAMICS SYSTEM UNDER LOAD | Dynamic Sound Slope < nBB’_ACC_ANCHOR | DYN_LO | dB | 50 | 50 | 50 |
|  | Dynamic Sound Slope > nBB’_ACC_ANCHOR | DYN_HI | dB | 105 | 75 | 105 |
|  | Form Factor for the logarithm function of the dynamic sound model | nSHIFT_DYN | 1/min | 5000 | 5000 | 5000 |
| SOUND FROM DYNAMICS SYSTEM UNDER EXTENDED PERFORMANCE va | Dynamic va Factor |  | dB(A) | 8 | 8 | 8 |
|  | Partial Load Form Factor 1 | 1 | --- | 0.17 | 0.17 | 0.17 |
|  | Partial Load Form Factor 2 | 2 | --- | 0.40 | 0.40 | 0.40 |

Annex 9 – Appendix 3

Symbols, Abbreviations and Acronyms

| Annex 9 |  |  |  |
| --- | --- | --- | --- |
| Symbol | Unit | Paragraph | Explanation |
| aMAX_REF | m/s2 | 3.4. | maximum reference acceleration as determined in a low gear under full load condition. |
| LTEST | dB(A) | 3.5.3. | sound pressure level measured for any target operation condition; value to be reported and used for calculations to the first decimal place |
| vAA’_TEST | km/h | 3.5.3. | vehicle speed measured for target operation condition when the reference point passes line AA'; value to be reported and used for calculations to the first decimal place |
| vPP’_TEST | km/h | 3.5.3. | vehicle speed measured for target operation condition when the reference point passes line PP; value to be reported and used for calculations to the first decimal place |
| vBB’_TEST | km/h | 3.5.3. | vehicle speed measured for target operation condition when the rear end of the vehicle passes line BB'; value to be reported and used for calculations to the first decimal place |
| nAA’_TEST | 1/min | 3.5.3. | engine speed measured for target operation condition when the reference point of the vehicle passes line AA'; value to be reported and used for calculations to a precision of 10 min-1 |
| nBB’_TEST | 1/min | 3.5.3. | engine speed measured for target operation condition when the rear end of the vehicle passes line BB'; value to be reported and used for calculations to a precision of 10 min-1 |
| aTEST | m/s2 | 3.5.4.1. | acceleration from PP' to BB'; value to be reported and used for calculations to the second decimal place |
| vaTEST | m²/s³ | 3.5.4.2. | performance calculated from the reported vehicle speed at line BB’ in meters per second and the acceleration result from paragraph 3.5.4.1. and rounded to the first digit after the decimal place. |
| LEXP | dB(A) | 3.5.4.3. | the expected sound pressure level for a discrete test run |
| Annex 9 - Appendix 1 |  |  |  |
| LACC_ANCHOR | dB(A) | 2.2.1.2. | vehicle sound pressure level for the acceleration test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| vBB’_ACC_ANCHOR | km/h | 2.2.1.2. | vehicle speed value when the rear of the vehicle passes line BB' for the acceleration test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| nBB’_ACC_ANCHOR | 1/min | 2.2.1.2. | engine speed value when the rear of the vehicle passes line BB' for the acceleration test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to a precision of 10 min-1 |
| LCRS_ANCHOR | dB(A) | 2.2.1.2. | vehicle sound pressure level at constant speed test; value to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| vBB’_CRS_ANCHOR | km/h | 2.2.1.2. | vehicle speed value when the rear of the vehicle passes line BB' for the constant speed test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| nBB’_CRS_ANCHOR | 1/min | 2.2.1.2. | engine speed value when the rear of the vehicle passes line BB' for the constant speed test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to a precision of 10 min-1 |
| x | - | 2.3.1. | energy contribution ratio of tyre/rolling sound level at LCRS_ANCHOR |
| LREF_TR | dB(A) | 2.3.1. | calculated Reference Tyre Rolling Sound Level |
| LREF_PT | dB(A) | 2.5. | calculated Reference Power Train Mechanics Sound Level |
| LREF_DYN | dB(A) | 2.6. | calculated Reference Dynamic Sound Level |
| ∆LDYN | dB(A) | 2.7. | Vehicle Dynamic Delta Sound Level |
| LREF_TR_ADJ | dB(A) | 2.7. | Tyre Rolling Sound Level with adjusted vehicle speed for ∆LDYN calculation |
| LREF_PT_ADJ | dB(A) | 2.7. | Power Train Mechanical Sound Level with adjusted engine speed for ∆LDYN calculation |
| TR_LO | dB(A) | 2.7. | sound slope for tyre rolling sound when vehicle speed is not greater than the reference speed |
| PT_LO | dB(A) | 2.7. | sound slope for Power Train Sound when vehicle engine speed is not greater than nBB’_CRS’_ANCHOR |
| nSHIFT_PT | 1/min | 2.7. | constant (shifting factor) for the regression curve of the mechanic sound model |
| kTEST | km/h·min | 3.2.1. | vehicle speed to engine speed ratio under "Real Driving Additional Sound Emission Provisions" (RD-ASEP) as expressed in km/h per 1000 min-1 during a test run, rounded to the second decimal |
| kREF | km/h·min | 3.2.1. | Reference vehicle speed to engine speed ratio defined in paragraph 3.6.1. |
| LCRS_ANCHOR’ | dB(A) | 3.2.3.2.2. | corrected cruise test result for HEV |
| LACC_ANCHOR’ | dB(A) | 3.2.3.3.2. | corrected acceleration test result for HEV |
| Limit | dB(A) | 3.2.3.3.2. | sound level limits in the table of paragraph 6.2.2. of main body to this Regulation for the valid phase |
| LTR_EXP | dB(A) | 3.3. | expected tyre rolling sound |
| qTR_HI | dB(A) | 3.3. | sound slope for tyre rolling sound when vehicle speed is greater than the reference speed |
| LPT_EXP | dB(A) | 3.4. | expected power train mechanics sound |
| PT_HI | dB(A) | 3.4. | sound slope for Power Train Mechanics Sound when vehicle engine speed is exceeding nBB’_CRS’_ANCHOR |
| LDYN_EXP | dB(A) | 3.5. | expected base dynamic sound |
| DYN_LO | dB(A) | 3.5. | sound slope for Dynamic Sound when vehicle engine speed is not greater than nBB’_ACC’_ANCHOR |
| DYN_HI | dB(A) | 3.5. | sound slope for Dynamic Sound when vehicle engine speed is exceeding nBB’_ACC’_ANCHOR |
| ∆LDYN_EXP | dB | 3.6. | expected dynamic delta sound |
| LOADTEST |  | 3.6.2. | load achieved during the test run |
| aMAX_κ | m/s2 | 3.6.2. | Maximum acceleration in gear κ |
| ∆LDYN_v×a | dB | 3.6.3.2. | dynamic performance component calculated based on the achieved performance vaTEST relative to a reference performance. |
| β |  | 3.6.3.2. | coefficient for calculating dynamic performance component |
| vaANCHOR | m²/s³ | 3.6.3.2. | performance value to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| a1 |  | 3.6.3.3. | coefficient for calculating dynamic sound components |
| a2 |  | 3.6.3.3. | coefficient for calculating dynamic sound components |

Annex 9 – Appendix 4

Formulas

Formula 2.4	
LREF_TR=10 ×lg(x×100.1×LCRS_ANCHOR )

Formula 2.5	
LREF_PT=10 ×lg((1-x)×100.1×LCRS_ANCHOR)

Formula 2.6	
LREF_DYN=LREF_PT -15 dB(A)

Formula 2.7 No.1
∆LDYN=10 ×lg100.1× LACC_ANCHOR-100.1× LREF_TR_ADJ - 100.1×LREF_PT_ADJ-LREF_DYN

Formula 2.7 No.2

LREF_TR_ADJ=TR_LO × lg(vBB'_ACC_ANCHORvTEST)+ LREF_TR

Formula 2.7 No.3
LREF_PT_ADJ=PT_LO × lg(nBB'_ACC_ANCHOR+nSHIFT_PT)(nBB'_CRS_ANCHOR + nSHIFT_PT)+ LREF_PT

Formula 3.2.1
LREF_PT_ADJ=PT_LO × lg(nBB'_ACC_ANCHOR+nSHIFT_PT)(nBB'_CRS_ANCHOR + nSHIFT_PT)+ LREF_PT

Formula 3.2.2.
nBB'_TEST=vBB'_TEST30×1000

Formula 3.2.3.2.2.

LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)

Formula 3.2.3.3.2.

LACC_ANCHOR'=Limit-kP×LCRS_ANCHOR/(1-kP)

Formula 3.2.3.4.2 No.1 
LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)

Formula 3.2.3.4.2 No.2

LACC_ANCHOR'=Limit-kP×LCRS_ANCHOR'/(1-kP)
Formula 3.2.4.2.2 
LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)

Formula 3.2.4.3.1.

nACC_ANCHOR=vBB'_ANCHOR20×1000

Formula 3.2.4.3.2

LACC_ANCHOR'=Limit-kP×LCRS_ANCHOR/(1-kP)

Formula 3.2.4.4.2 No.1

nCRS_ANCHOR=vTEST30×1000
Formula 3.2.4.4.2 No.2

nACC_ANCHOR=vTEST20×1000

Formula 3.2.4.4.3 No.1

LCRS_ANCHOR'=LCRS_ANCHOR+0.5 dB(A)

Formula 3.2.4.4.3 No.2

LACC_ANCHOR'=Limit-kP×LCRS_ANCHOR'/(1-kP)
Formula 3.2.5.1.

LCRS_ANCHOR'=LACC_ANCHOR-1.1 dB(A)

Formula 3.2.5.2.1 No.1

nBB'_ACC_ANCHOR=vBB'_ACC_ANCHOR20×1000
Formula 3.2.5.2.1 No.2

nBB'_CRS_ANCHOR=vTESTvBB'_ACC_ANCHOR×nBB'_ACC_ANCHOR

Formula 3.2.5.2.2.

nBB'_CRS_ANCHOR=vTEST30×1000
Formula 3.3. No.1 
LTR_EXP=θTR_LO×lgvBB'_TESTvTEST+LREF_TR

Formula 3.3. No.2 
LTR_EXP=θTR_HI×lgvBB'_TESTvTEST+LREF_TR

Formula 3.4. No.1

LPT_EXP=θPT_LO×lgnBB'_TEST+nSHIFT_PTnBB'_CRS_ANCHOR+nSHIFT_PT+LREF_PT

Formula 3.4. No.2

LPT_EXP= θPT_HI ×lg((nBB'_TEST + nSHIFT_PT) / (nBB'_CRS_ANCHOR + nSHIFT_PT)) + LREF_PT

Formula 3.5. No.1
LDYN_EXP=θDYN_LO×lgnBB'_TEST+nSHIFT_DYNnBB'_ACC_ANCHOR+nSHIFT_DYN+LREF_DYN

Formula 3.5. No.2

LDYN_EXP=θDYN_HI×lgnBB'_TEST+nSHIFT_DYNnBB'_ACC_ANCHOR+nSHIFT_DYN+LREF_DYN

Formula 3.6.2.  LOADTEST=aTESTaMAX i

where  aMAX_i=κREF κTEST ×aMAX_REF

Formula 3.6.3.1.

vaTEST=vBB'_TEST3,6×aTEST [m²/s³]

Formula 3.6.3.2. No.1
vaANCHOR=vBB'_ACC_ANCHOR/3.6×aACC_ANCHOR

Formula 3.6.3.2. No. 2

∆LDYN_va=β× lgvaTESTvaANCHOR
Formula 3.6.3.3.

∆LDYN_EXP=∆LDYN+∆LDYN_va×1-α1LOADTEST+α21-α1/1+α2+0.3
Formula 3.7.

∆LAVAS=LACC_ANCHOR-58×1-vBB'_TESTvTEST0.75
Formula 3.8.

LTEST_EXP=10×lg100.1×LTR_EXP+100.1×LPT_EXP+100.1×LDYN_EXP+∆LDYN_EXP+∆LAVAS+2dB(A)

Annex 9 – Appendix 5 		Test report sheet

Compliance of the test results to Annex 9: YES/NO                                     Number Case of compliance according to paragraph 4 of Annex 9:

Annex 10

Specific provisions for the testing of vehicles equipped with an ADS

- General

This Annex adapts Annexes 3 and 7 to vehicles equipped with an ADS. It does not add nor remove any requirement.

- Specifications

2.1.		Preparations

When performing the tests of this Regulation, the conditions prescribed in chapter 6 “Specifications” shall be fulfilled. Additional preparation of the test track, the vehicle or other equipment may be needed for vehicles equipped with an ADS. This additional preparation shall be approved by the Type Approval Authority and its designated Technical Service and described in the test report.

2.2.	For vehicles equipped with an ADS, other than those of category X or Y, it is sufficient to test either in manual operation or ADS operation, when the two following conditions are fulfilled:

- 	the manufacturer declares that the vehicle is designed in such a way that there should be no difference between the manual and the automated modes with regards to this Regulation and

- 	the Type Approval Authority and its designated Technical Service agree to this single mode testing.

    - If tested in manual operation, the sound shall be measured according to Annex 3 and, if applicable, Annex 7

    - If tested in ADS operation, the sound shall be measured according to Annex 3 and, if applicable, Annex 7, and this Annex.

- Adaptation of the accelerator commands

Several paragraphs in Annexes 3 and 7 prescribe actions on the accelerator. For vehicles equipped with an ADS and tested in ADS operation, these should be interpreted as commands from the ADS to achieve specific vehicle behaviours. Table 1 below translates the actions on the accelerator into the required actions from the ADS.

Table 1: Translation of actions on the accelerator into required actions from the ADS.

| Chapter | Action for vehicles tested in manual operation | Equivalent action for vehicles tested in ADS operation |
| --- | --- | --- |
| Annex 3, paragraph 3.1.2.1. Annex 3, paragraph 3.1.2.2. Annex 7, paragraph 2.3. | Fully depressing the accelerator | Command from the ADS to apply the maximum acceleration |
| Annex 3, paragraph 3.1.2.1.5. | Keeping the accelerator in the depressed position | Command from the ADS to continue the maximum acceleration |
| Annex 3, Appendix, paragraph 3. and paragraph 3.1. | Releasing the accelerator | Command from the ADS to achieve coast down condition (no torque requested from the engine) |
| Annex 3, Appendix 3, paragraph 3.1. | Positioning the accelerator as such to maintain a constant speed | Command from the ADS to achieve a constant speed |
| Annex 3, paragraph 3.2.5.3.2.2. | Rapidly release the throttle control | Command from the ADS to the ICE to release the throttle to go to idling. (Only applies to Category X and Y vehicles with an ICE.) |

*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document (ECE/TRANS/WP.29/78/Rev.8)."

Additional Sound Emission Provisions refer to the specifications of paragraph 6.2.3. of the main body and Annex 7. A new Annex 9 was introduced for a sound evaluation of a vehicle under Real Driving Conditions (RD-ASEP) for evaluation purposes without impact on the type approval. RD-ASEP covers an extended control range with driving conditions outside of normal driving with higher accelerations and vehicle speeds up to 100 km/h. See also the provisions in paragraph 5.1.

Note: The common understanding of a "low gear" or a "high gear" shall not apply to gear ratios. For example, the lowest gear for forward driving, the first gear, has the highest gear ratio of all forward driving gears. While manual transmission has discrete gears, many non-manual transmissions can have more gear ratios engaged by the control unit of the transmission.

For example, but not limited to, conformity of production, or extensions of already existing approvals, or in-service conformity.

https://unece.org/documents-reference-only-0
In the “monitoring data sheet” the address, where the file shall be submitted to, is on the top of the sheet table “(0) instructions”.

Where the Type Approval Authorities, Technical Services or vehicle manufacturers decide to submit additional test reports, the measured vehicles shall be in such a condition that they can be used for type approval (e.g., the vehicles should not be older than one year, originally equipped, without manipulation and maintained according to the manufacturer's specifications) and type approved according to the 03 series of UN Regulation No. 51.

The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3)

A test is made on a stationary vehicle in order to provide a reference value for administrations which use this method to check vehicles in use.

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), ECE/TRANS/WP.29/78/Rev.6

See guidance document for interpretation of paragraph 6.2.3. ASEP of UN-R 51, which is included in the list of documents for reference of GRBP.

Example: changing a mode or operating a switch.

Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the Regulation).

Delete what does not apply.

If the means of identification of type contains characters not relevant to describe the vehicle types covered by the type-approval certificate such characters shall be represented in the documentation by the symbol: '?' (e.g. ABC??123??).

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.3, para. 2 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html

If applicable

Delete what does not apply.

If the means of identification of type contains characters not relevant to describe the vehicle types covered by the type-approval certificate such characters shall be represented in the documentation by the symbol: '?' (e.g. ABC??123??).

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.3, para. 2 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html

Only for the purpose of defining "off-road vehicles".

Standard ISO 612: 1978 — Road vehicles — Dimensions of motor vehicles and towed vehicles - terms and definitions.
(a) 	Where there is one version with a normal cab and another with a sleeper cab, both sets of masses and dimensions are to be stated. 
(b) 	Optional equipment that affects the dimensions of the vehicle shall be specified.

The mass of the driver is assessed at 75 kg. The liquid containing systems (except those for used water that shall remain empty) are filled to 90 per cent of the capacity specified by the manufacturer. The information referred to in points 2.2. (b) do not need to be provided for vehicle categories N2, N3, M2 and M3.

For vehicles coupled with a trailer or a semi-trailer, which exert a significant vertical load on the coupling device or the fifth wheel, this load, divided by standard acceleration of gravity, is included in the maximum technically permissible mass. Please fill in here the upper and lower values for each variant.

Please fill in here the upper and lower values for each variant.

In the case of a vehicle that can run either on petrol, diesel, etc., or also in combination with another fuel, items shall be repeated. In the case of non-conventional engines and systems, particulars equivalent to those referred here shall be supplied by the manufacturer.

Delete what does not apply.

This value shall be calculated (π = 3.1416) and rounded off to the nearest cm³.

The specified particulars are to be given for any proposed variants.

Continuous Variable Transmission (CVT): transmission with variable gear ratios.

With respect to trailers, maximum speed permitted by the manufacturer.

The latter number is given as an example only.

Given that the tyre contribution for overall sound emission is significant, regard shall be given for existing regulatory provisions concerning tyre/road sound emissions. Traction tyres, snow tyres and special-use tyres as defined in paragraph 2. of UN Regulation No. 117 shall be excluded during type-approval and conformity of production measurements at the request of the manufacturer in accordance with UN Regulation No. 117.

2	See Annex 3, Appendix, Figure 1.

See Figure 6 of Annex 3, Appendix

Measurements for Annex 7 for a particular vehicle type may be carried out on a different test tracks or under different environmental conditions, each according to the provisions of this Regulation, if the test results  of the lower gear used for the calculation of Lurban in Annex 3 and representing the anchor point, do not differ by more the +/- 1.0 dB from the test results at the time when the tests according to Annex 3 have been carried out.

The Analysis method 2 (Lurban assessment) and the Reference Sound Assessment are not applicable for these vehicles.

As applicable for the approved type of vehicle

As applicable for the approved  type of vehicle

Simulation may not always be applicable as the test result of Annex 3 and the elaborated slopes according to paragraph 3. of Annex 7 might not provide consistent data for the simulation. In that case, it is recommended to carry out direct measurements.

See footnote 4 in paragraph 6.2.8. of UN Regulation No. 138 "The maximum overall sound pressure level of 75 dB(A) measured at a distance of 2 m is corresponding to the overall sound pressure level of 66 dB(A) measured at a distance of 7,5 m."

Facilities may provide restriction for safety reasons, such as for vehicle speed.

Tests for Annex 3, Annex 7, and Annex 9 may be carried out on different test facilities if documentation exists that demonstrates that the differences in sound performance are neglectable.