| Submitted by the experts from GTB | Informal document GRE-94-09 (94th GRE, 27-30 April 2026, agenda item 7 (b)) |
| --- | --- |

This paper provides additional information and justification regarding document GRE/2026/12

I.		Proposal for a Supplement to the 01 series of amendments to UN Regulation No. 149

Paragraph 6.2.1.1., amend to read:

“6.2.1.1. 	If not otherwise specified, no measured value deviates unfavourably by more than 20 per cent from the value prescribed in this Regulation and, for driving-beams of class A, B or RA, if HV is situated within the isocandela 0.75 Imax.”

Paragraph 6.2.1.1.1.2.1., delete:

“6.2.1.1.1.2.1.and if, for the driving-beam, HV being situated within the isocandela 0.75 Imax a tolerance of + 20 per cent for maximum values and -20 per cent for minimum values is observed for the photometric values at any measuring point specified in paragraph 5.1. of this Regulation.”

II.	Justification

- In UN Regulation No. 149 (both 00 and 01 series), the Conformity of Production (CoP) requirements for the driving-beam are currently embedded as a sub-paragraph within the passing-beam CoP requirements. This structure creates ambiguity regarding the exact requirements that shall apply to the driving-beam.

- By contrast, UN Regulation No. 123 addresses both beam types at the same hierarchical level (Annex 5, paragraphs 1.2.1.1 and 1.2.1.2), ensuring clarity and consistency in compliance checks.

- The proposed solution aims to improve the structure of the text and its clarity.

____________