Proposal for supplement 6 to the original version of UN Regulation No. 162 (Immobilisers)

Submitted by the expert from the Kingdom of the Netherlands on behalf of the Task Force on Automated Vehicles Regulatory Screening*

The text reproduced below was prepared by the Task Force on Automated Vehicles Regulatory Screening (TF-AVRS), with the aim to amend and supersede document ECE/TRANS/GRSG/2025/42. The modifications to ECE/TRANS/GRSG/2025/42 are marked in (bold) red for new or strikethrough for deleted characters.

I.	Proposal

Insert new paragraphs 0. and 0.1. to 0.1.2., to read:

"0.		Introduction

0.1.		Supplement 6 to the original version is introduced to take into account vehicles of category X and Y 1, as well as vehicles with a manual mode operating at speeds above 6 km/h which are equipped with an Automated driving System (ADS). 1

0.1.1. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the Regulation and to extend its application to vehicles without driver and without manual driving controls in the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.2. 	In case of vehicles equipped with an ADS other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements have to be fulfilled apply."

Paragraph 1.1., footnote 1, amend to read:

"1		As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.68, para. 2 - https://unece.org/transport/standards/vehicle-regulations/wp29/resolutions."

Insert new paragraphs 2.4. and 2.5., to read:

"2.4.	"ADS feature of type 1 (ADSF-1)" means an ADS feature which includes an ADS fallback response requiring a fallback user;

2.5.	"ADS feature of type 2 (ADSF-2)" means an ADS feature which does not include an ADS fallback response requiring a fallback user."

Paragraph 4.4.1., footnote 3, amend to read:

"3	The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev. 6, Annex 3 - https://unece.org/transport/standards/transport/vehicle-regulations-wp29/resolutions.

As defined in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev. 8 (https://unece.org/transport/vehicle-regulations/wp29/resolutions)"

Paragraph 5.2.4.1., amend to read:

"5.2.4.1.	The immobilizer shall be set without supplementary action from the driver (if applicable) by at least one of the following means:

(a)		at rotation of the ignition key into the "0" position in the ignition lock the finalisation of each ignition cycle (or run cycle, as relevant) and activation of a door; in addition, immobilizers which unset immediately before or during the normal starting procedure of the vehicle are permitted to set on turning the ignition off, finalising the ignition cycle (or run cycle, as relevant)

(b)		a maximum of 1 minute after removing the key of the ignition 					lock."

Annex 9, paragraph 4.3.2., amend to read:

"4.3.2.	The requirements in paragraph 4.3.1. shall not apply:

- during a remote-control manoeuvring and remote-control parking as defined in UN Regulation No. 79;

- for vehicles of categories X and Y1; or

- when an ADSF-2 is activated by an authorized registered digital key".

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.