|  | United Nations | ECE/TRANS/WP.29/GRSG/2025/27 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 18 July 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on General Safety Provisions

130th session

Geneva, 6–9 October 2025

Item 14(b) of the provisional agenda

Exchange of views on vehicle automation:

Categorization of automated vehicles and autonomous vehicle regulation screening

Proposal for supplement 2 to the original version of UN Regulation No. 93 (Front Underrun Protective Devices)

Submitted by the expert from the Kingdom of the Netherlands on behalf of the Task Force on Automated Vehicles Regulatory Screening*

The text reproduced below was prepared by the expert from the International Organization of Motor Vehicle Manufacturers (OICA) on behalf of the Task Force on Automated Vehicles Regulatory Screening (TF-AVRS). The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

Proposal

Insert new paragraphs 0. and 0.1. to 0.1.3., to read:

"0.		Introduction

0.1.		Supplement 1 to the 02 series of amendments is introduced to take into account vehicles of categories X and Y 1, as well as vehicles with a manual mode operating at speeds above 6 km/h which are equipped with an Automated Driving System (ADS). 1

0.1.1. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.2. 	In case of vehicles equipped with an ADS other than vehicles of categories X and Y, in any driving mode (manual or automated) no special provisions or exemptions apply."

Paragraph 1., footnote 1, amend to read:

"1	See Consolidated Resolution on the Construction of Vehicles (R.E.3, annex 7) (TRANS/SC1/WP29/78/Amend.3) As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev. 8, para. 2 - https://unece.org/transport/vehicle-regulations/wp29/resolutions"

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.