Working document for the IWG on EDR/DSSAD & SG-EDR. 	This document serves as	a place holder for proposals under 	consideration as part of the ‘EDR Step 2’ workstream.

Proposal by the Informal Working Group on EDR/DSSAD amending the 02 Series of Amendments to UN Regulation No. 160 introducing new requirements in line with the ‘Step 2’ workstream (Programme of Work ECE/TRANS/WP.29/2025/1 as revised).

In addition, editorial improvements are included applicable to previous Series of Amendments. In a later stage it will be identified which items will be introduced as a new Series of Amendment and which go into a Supplement to the existing and previous Series of Amendments.

The modifications to the text of the 02 Series of Amendments to UN Regulation No. 160 are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

2.		Definitions

Paragraph 2.18., amend to read:

2.18. 	"End of event time" means,

- For planar events, the moment at which the cumulative delta-V within a 20 ms time period becomes 0.8 km/h or less, or the moment at which the crash protection algorithm of the air bag control unit all algorithms resets, or the moment at 300 ms after time zero.

- For all other events, the moment at 300 ms after time zero. ”

Insert new paragraphs 2.71 – 2.83:

“2.71.	“Rollover protection system” means any protection system that protects occupants during a rollover, which is activated by a vehicle's crash sensing system.

2.72.	“Advanced emergency braking system” means a system which can automatically detect an imminent forward collision and activate the vehicle braking system to decelerate the vehicle with the purpose of avoiding or mitigating a collision. The system may also be referred to as "Automatic emergency braking system" in other publications or countries.

2.73.	“Trigger activated” indicates the first trigger that was activated to cause the recording of the event.

2.74.	"Vulnerable Road User (VRU)" means an adult or child pedestrian or an adult or child cyclist”

2.75.	"Pre-Event Synchronization Timer" means the relative time from last data sampling point before T0 to T0. Used for temporal alignment of pre-crash data with event time zero."

2.76.      	“Blind spot monitoring system” means a system that detects and communicates to the driver the presence of another vehicle in a zone to the side of the subject vehicle or the approach of another vehicle into that zone.

2.77.	"Emergency stop signal" means a light-signaling function to indicate to other road users to the rear of the vehicle that a high retardation force is being applied to the vehicle relative to the prevailing road conditions.

2.78.	“Turn signal” means a device mounted on a motor vehicle or trailer which, when operated, signals the latter's intention to change the direction in which the vehicle is proceeding.

2.79.	“Acceleration control for pedal error (ACPE)” means a system to detect misapplication of the accelerator control by the driver and to control unintended acceleration.

2.80.	“Advanced driver distraction warning (ADDW)” means a system that helps the driver to continue to pay attention to the traffic situation and that warns the driver when he or she is distracted.

2.81	“Current trip” means the period of time extending from the event back to the last change of Ignition cycle.

2.82.	“Driver control assistance system (DCAS)” means the hardware and software collectively capable of assisting a driver in controlling the longitudinal and lateral motion of the vehicle on a sustained basis.

2.83.	“Driver drowsiness and attention warning (DDAW)” means a system that assesses the driver’s alertness through vehicle systems analysis and warns the driver if needed”

5.		  Definitions

Paragraph 5.3.1., amend to read:

“5.3.1.	Conditions for triggering recording of data

An event shall be recorded by the EDR if one of the following threshold values mentioned under paragraphs 5.3.1.1. to 5.3.1.2. is met or exceeded.

If a vehicle is not fitted with a system referred to in paragraphs 5.3.1.1. to 5.3.1.2., this document requires neither recording of data according to these paragraphs nor fitting of such systems.

5.3.1.1.	For planar event, VRU event, rollover event:

5.3.1.1.1.	Change in longitudinal vehicle velocity more than 8 km/h within a 150 ms or less interval.

5.3.1.1.2.	Change in lateral vehicle velocity more than 8 km/h within a 150 ms or less interval

5.3.1.1.3.	Activation of Non-reversible occupant restraint system,

5.3.1.1.4.	Activation of Vulnerable road user VRU secondary safety system.

5.3.1.1.5.	Activation of the rollover protection system.

If a vehicle is not fitted with any Vulnerable Road User (VRU) secondary safety system, this document requires neither recording of data nor fitting of such systems.  However, if the vehicle is fitted with such a system, then it is mandatory to record the event data following activation of this system.

5.3.1.2.	For AEBS event:

Emergency braking demand by the advanced emergency braking system in response to detecting the possibility of an imminent collision.”

Paragraph 5.3.2, amend to read:

5.3.2. Conditions for triggering locking of data

In the circumstances provided below, the memory for the event shall be locked to prevent any future overwriting of the data by subsequent events.

If a locking trigger occurs in short succession (< 300ms) after another trigger and an overlap of data between both events would result, only one event shall be locked.

Paragraph 5.3.2.3., amend to read:

“5.3.2.3. 	Activation of Vulnerable road user VRU secondary safety system.

“5.3.2.4. 	Activation of the rollover protection system.”

Paragraph 5.3.3., amend to read:

5.3.3.	Conditions for establishment of time zero

Time zero is established at the time when any of the following first occurs:

5.3.3.1            For planar event, VRU event, rollover event:

5.3.3.1.1	For systems with "wake-up" air bag control systems, the time at which the occupant restraint control algorithm is activated; or

5.3.3.1.2.	For continuously running algorithms,

5.3.3.1.2.1.	The first point in the interval where a longitudinal, cumulative delta-V of over 0.8 km/h is reached within a 20 ms time period; or

5.3.3.1.2.2.	For vehicles that record "delta-V, lateral," the first point in the interval where a lateral, cumulative delta-V of over 0.8 km/h is reached within a 5 ms time period; or

5.3.3.1.3.	Deployment of a non-reversible deployable restraint or activation of VRU secondary safety protection system.

5.3.3.1.4.	Activation of VRU secondary safety system

5.3.3.1.5.	Activation of the rollover protection system.

5.3.3.2.           For AEBS Event

Occurrence of the emergency braking demand trigger of the advanced emergency braking system.

Paragraph 5.4., amend to read:

"5.4.	Crash test performance and survivability

5.4.1.	Each vehicle subject to the requirements of national or regional frontal crash test regulations, shall conform with the specifications in paragraph 5.4.3.

5.4.2.	Each vehicle subject to the requirements of national or regional side impact crash test regulations shall conform with the specifications of paragraph 5.4.3.

5.4.3.	The data elements required by paragraph 5.1, shall be recorded in the format specified by paragraph 5.2, exist at the completion of the crash test and the complete data recorded element shall read "yes" after the test. Elements that are not operating normally in braking, etc.) are not required to meet the accuracy or resolution requirements in these crash tests.
The data shall be retrievable even after an impact of a severity level set by UN Regulations Nos. 94, 95 or 137 95 and either 94 or 137.”

6.		  Verification Procedures

Paragraph 6.1., amend to read:

6.1.	The accuracy of the measurement of longitudinal and lateral acceleration of the 2 data elements (post-crash longitudinal and lateral acceleration) shall be verified using a component test fixture that subjects the EDR/airbag control module acceleration sensors to a sinusoidal acceleration motion in accordance with the following:

“	at= -40g*sinπt20 ms
			        +/- 2g ”

12.	Transitional provisions

Paragraph 12., amend to read:

12.11.            As from the official date of entry into force of the 03 series of amendments, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type-approvals under this Regulation as amended by the 03 series of amendments.

12.12.            As from 1 September [2031], Contracting Parties applying this Regulation shall not be obliged to accept type-approvals to any of the preceding series of amendments, first issued after 1 September [2031].

12.13.            Contracting Parties applying this Regulation shall continue to accept type approvals issued according to any of the preceding series of amendments to this Regulation, first issued before 1 September [2031], provided the transitional provisions in these respective previous series of amendments foresee this possibility.

12.14.            Contracting Parties applying this Regulation may grant type approvals according to any preceding series of amendments to this Regulation.

12.15.            Contracting Parties applying this Regulation shall continue to grant extensions of existing approvals to any preceding series of amendments to this Regulation

Annex 4 - Data elements and format1

The following table, amend to read:

| Table-1 Data elements (unchanged) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data element | Condition for requirement2,3 | Recording interval/time4,5 | Data sample rate | Minimum range | Accuracy6,7 | Resolution8 | Event(s) recorded for9 |
| Delta-V, longitudinal | Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz with sufficient range and resolution to calculate delta-v with required accuracy | 0 to 250 ms or 0 to End of Event Time plus 30 ms, whichever is shorter | 100 | -100 km/h to + 100 km/h. | ±10% | 1 km/h. | Planar |
| Maximum delta-V, longitudinal | Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz | 0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter. | N/A | -100 km/h to + 100 km/h. | ±10% | 1 km/h. | Planar |
| Time, maximum delta-V, longitudinal | Mandatory - not required if longitudinal acceleration recorded at ≥500 Hz | 0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter. | N/A | 0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter. | ±3 ms | 2.5 ms | Planar |
| Speed, vehicle indicated | Mandatory | -5.0 to 0 sec | 2 | 0 km/h to 250 km/h | ±1 km/h | 1 km/h | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | VRU |  |
| Engine throttle, % full (or Accelerator pedal, % full) | Mandatory | -5.0 to 0 sec | 2 | 0 to 100 % | ± 5 % | 1% | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | VRU |  |
| Service brake status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | On, Off | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | VRU |  |
| Ignition cycle, crash | Mandatory | -1.0 sec | N/A | 0 to 60,000 | ±1 cycle | 1 cycle | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | AEBS |  |

| Ignition cycle, download | Mandatory | At time of download10 | N/A | 0 to 60,000 | ±1 cycle | 1 cycle | Planar |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | AEBS |  |
| Safety belt status, driver | Mandatory | -1.0 sec | N/A | N/A | N/A | Fastened, not fastened | Planar |  |
|  |  |  |  |  |  |  | Rollover [AEBS] |  |
| Air bag warning lamp11 | Mandatory | -1.0 sec | N/A | N/A | N/A | On, Off | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Frontal air bag deployment, time to deploy, in the case of a single stage air bag, or time to first stage deployment, in the case of a multi-stage air bag, driver | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
| Frontal air bag deployment, time to deploy, in the case of a single stage air bag, or time to first stage deployment, in the case of a multi-stage air bag, passenger12 | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
| Multi-event crash, number of event | If Recorded | Event | N/A | 1 or more | N/A | 1 or more | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | AEBS |  |
| Time from Event 1 to 2 | Mandatory | As needed | N/A | 0 to 5 sec | ±0.1 sec | 0.1 sec | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Complete file recorded | Mandatory | Following other data | N/A | N/A | N/A | Yes, No | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | AEBS |  |
| Lateral acceleration | If Recorded | 0–250 ms or 0 to End of Event Time plus 30 ms, whichever is shorter. | 500 | -50g to +50g | ±10 %15 | 1g | Planar |  |
| (post-crash) |  |  |  |  |  |  | Rollover16 |  |
| Longitudinal acceleration | If Recorded | 0–250 ms or 0 to End of Event Time plus 30 ms, whichever is shorter. | 500 | -50g to +50g | ±10 %15 | 1g | Planar |  |
| (post-crash) |  |  |  |  |  |  |  |  |
| Normal acceleration | If Recorded | 0 to at least 250 300 ms | 10 | -5g to +5g | ±10 % | 0.5g | Rollover |  |
| (post-crash) |  |  |  |  |  |  |  |  |
| Delta-V, lateral | Mandatory - not required if lateral acceleration recorded at ≥500 Hz with sufficient range and resolution to calculate delta-v with required accuracy | 0 to 250 ms or 0 to End of Event Time plus 30 ms, whichever is shorter | 100 | -100 km/h to + 100 km/h. | ±10% | 1 km/h. | Planar |  |
| Maximum delta-V, lateral | Mandatory - not required if lateral acceleration recorded at ≥500 Hz | 0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter. | N/A | -100 km/h to + 100 km/h. | ±10% | 1 km/h. | Planar |  |
| Time, maximum delta-V, lateral | Mandatory - not required if lateral acceleration recorded at ≥500 Hz | 0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter. | N/A | 0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter. | ±3 ms | 2.5 ms | Planar |  |
| Time, maximum delta-V, resultant | Mandatory - not required if relevant acceleration recorded at ≥500 Hz | 0–300 ms or 0 to End of Event Time plus 30 ms, whichever is shorter. | N/A | 0–300 ms, or 0-End of Event Time plus 30 ms, whichever is shorter. | ±3 ms | 2.5 ms | Planar |  |
| Engine RPM | Mandatory | -5.0 to 0 sec | 2 | 0 to 10,000 rpm | ±100 rpm16 | 100 rpm | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Vehicle roll angle18 | If Recorded | 0 to at least 250 ms | 10 | -1080 deg to +1080 deg | ±10 % | 10 deg | Rollover |  |
| Vehicle roll rate18 | Mandatory if fitted and used for rollover protection system control algorithm | 0 to at least 250 ms | 10 | -240 deg/sec to +240 deg/sec | ±10 % | 4 deg/sec | Rollover |  |
| Anti-lock braking system activity | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, Non-Engaged, Engaged | Planar |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | Rollover |  |
| Stability control | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, On, Off, Engaged | Planar |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | Rollover |  |
| Steering input | Mandatory | -5.0 to 0 sec | 2 | -250 deg CW to + 250 deg CCW. | ±5 % | 1% | Planar |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | Rollover |  |
| Safety belt status, front passenger12 | Mandatory | -1.0 sec | N/A | N/A | N/A | Fastened, not fastened | Planar |  |
|  |  |  |  |  |  |  | Rollover [AEBS] |  |
| Passenger air bag suppression status, front12 | Mandatory | -1.0 sec | N/A | N/A | N/A | Suppressed, not suppressed | Planar Rollover |  |
| Frontal air bag deployment, time to nth stage, driver14 | Mandatory if fitted with a driver’s frontal air bag with a multi-stage inflator. | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
| Frontal air bag deployment, time to nth stage, passenger12,14 | Mandatory if fitted with a front passenger’s frontal air bag with a multi-stage inflator. | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
| Side air bag deployment, time to deploy, driver | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
| Side air bag deployment, time to deploy, passenger | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
| Side curtain/tube air bag deployment, time to deploy, driver side | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Side curtain/tube air bag deployment, time to deploy, passenger side | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Pretensioner deployment, time to fire, driver. | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Pretensioner deployment, time to fire, front passenger12 | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Seat track position switch, foremost, status, driver | Mandatory if fitted and used for deployment decision | -1.0 sec | N/A | N/A | N/A | Yes, No. | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Seat track position switch, foremost, status, front passenger12 | Mandatory if fitted and used for deployment passenger | -1.0 sec | N/A | N/A | N/A | Yes, No. | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Occupant size classification, driver | If recorded | -1.0 sec | N/A | 5th percentile female or larger. | N/A | Yes, No. | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Occupant size classification, front passenger12 | If recorded | -1.0 sec | N/A | 6yr old HIII US ATD or Q6 ATD or smaller | N/A | Yes, No. | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
| Safety belt status, rear passengers13 | Mandatory | -1.0 sec | N/A | N/A | N/A | Fastened, not fastened | Planar |  |
|  |  |  |  |  |  |  | Rollover [AEBS] |  |
| Tyre Pressure Monitoring System Warning Lamp Status | Mandatory | -1.0 sec | N/A | N/A | N/A | On, Off | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | AEBS |  |

| Longitudinal acceleration | Mandatory | -5.0 to 0 sec | 2 | -1.5g to +1.5g | ±10 % | 0.1g | Planar |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (pre-crash) |  |  |  |  |  |  | VRU |  |  |
|  |  |  |  |  |  |  | AEBS |  |  |
| Lateral acceleration | Mandatory | -5.0 to 0 sec | 2 | -1.0g to +1.0g | ±10 % | 0.1g | Planar |  |  |
| (pre-crash) |  |  |  |  |  |  | AEBS |  |  |
| Yaw Rate18 | Mandatory | -5 to 0 sec | 2 | -75 deg/sec to +75 deg/sec | ± 10% of the full range of the sensor | 0.1 deg/sec | Planar |  |  |
|  |  |  |  |  |  |  | Rollover |  |  |
|  |  |  |  |  |  |  | AEBS |  |  |
| Traction Control Status | Mandatory if not fitted with Stability control | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, On, Off, Engaged | Planar |  |  |
|  |  |  |  |  |  |  | Rollover |  |  |
| Advanced emergency braking system status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |  |
|  |  |  |  |  |  | Deactivated, | Rollover |  |  |
|  |  |  |  |  |  | On but Non-engaged, | VRU |  |  |
|  |  |  |  |  |  | Warning but Non-engaged, | AEBS |  |  |
|  |  |  |  |  |  | Engaged |  |  |  |
| Cruise Control System Status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Engaged, Faulted, Off, Non-engaged | Planar |  |  |
|  |  |  |  |  |  |  | Rollover |  |  |
|  |  |  |  |  |  |  | VRU |  |  |
|  |  |  |  |  |  |  | AEBS |  |  |
| Adaptive Cruise Control Status (driving automation system level 1) | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Engaged, Faulted, Off, Non-engaged | Planar |  |  |
|  |  |  |  |  |  |  | Rollover |  |  |
|  |  |  |  |  |  |  | VRU |  |  |
|  |  |  |  |  |  |  | AEBS |  |  |
| Vulnerable road user secondary safety system deployment, time to deploy | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | VRU |  |  |
| Vulnerable road user secondary safety system warning indicator status19 | Mandatory | -1.0 sec | N/A | N/A | N/A | On, Off | VRU |  |  |
| Safety belt status mid-position front | Mandatory | -1.0 sec | N/A | N/A | N/A | Fastened, not fastened | Planar |  |  |
|  |  |  |  |  |  |  | Rollover |  |  |
|  |  |  |  |  |  |  | AEBS |  |  |
| Far-side impact centre air bag deployment, time to deploy12 | Mandatory | Event | N/A | 0 to 250 ms | ±2ms | 1ms | Planar |  |  |
|  |  |  |  |  |  |  | Rollover |  |  |
| Lane departure warning system status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | On but not warning, | AEBS |  |
|  |  |  |  |  |  | On – Warning left, |  |  |
|  |  |  |  |  |  | On – Warning right |  |  |
| Corrective steering function status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | On but not engaged, Engaged | AEBS |  |
| Emergency steering function status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | On but not engaged, Engaged | AEBS |  |
| Automatically commanded steering function category A status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | Stand-By | AEBS |  |
|  |  |  |  |  |  | Active20 |  |  |
| Automatically commanded steering function category B1 status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | Stand-By | AEBS |  |
|  |  |  |  |  |  | Active20 |  |  |
| Automatically commanded steering function category B2 status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | Stand-By | AEBS |  |
|  |  |  |  |  |  | Active20 |  |  |
| Automatically commanded steering function category C status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | Stand-By | AEBS |  |
|  |  |  |  |  |  | Active20 |  |  |

| Automatically commanded steering function category D status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | Stand-By | AEBS |  |
|  |  |  |  |  |  | Active20 |  |  |
| Automatically commanded steering function category E status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |  |
|  |  |  |  |  |  | Off, | Rollover |  |
|  |  |  |  |  |  | Stand-By | AEBS |  |
|  |  |  |  |  |  | Active20 |  |  |
| Accident emergency call system status | Mandatory | Event | N/A | N/A | N/A | Faulted, | Planar |  |
|  |  | 0–300 ms or |  |  |  | On but emergency call not automatically triggered, | Rollover |  |
|  |  | 0 to End of |  |  |  | On – Emergency call automatically triggered21 | VRU |  |
|  |  | Event Time |  |  |  |  | AEBS |  |
|  |  | plus 30 ms, |  |  |  |  |  |  |
|  |  | whichever is |  |  |  |  |  |  |
|  |  | shorter. |  |  |  |  |  |  |

| Table-1a Data elements (changed) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data element | Condition for requirement2,3 | Recording interval/time4,5 | Data sample rate | Minimum range | Accuracy6,7 | Resolution8 | Event(s) recorded for9 |
| Speed, vehicle indicated | Mandatory | -5.0 to 0 sec | 210 | 0 km/h to 250 km/h | ±1 km/h | 1 km/h | Planar |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | AEBS |
| Engine throttle, % full (or Accelerator pedal, % full) | Mandatory | -5.0 to 0 sec | 2 | 0 to 100 % | ± 5 % | 1% | Planar |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | VRU |
| Engine throttle, % full | If Recorded | -5.0 to 0 sec | 10 | 0 to 100 % | ± 5 % | 1% | Planar |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | AEBS |

| Accelerator pedal, % full | Mandatory | -5.0 to 0 sec | 10 | 0 to 100 % | ± 5 % | 1% | Planar |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | AEBS |
| Service brake status | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | On, Off | Planar |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | AEBS |
| Time from Event 1 to 2Time interval between this event and the previous event | Mandatory | As needed | N/A | 0 to 5 sec | ±0.1 sec | 0.1 sec | Planar |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | AEBS |
| Engine RPM | Mandatory | -5.0 to 0 sec | 210 | 0 to 10,000 20,000 rpm | ±100 rpm17 | 100 rpm | Planar |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | AEBS |
| Vehicle roll angle18 | If Recorded | 0 -1.0 sec to at least 250 300 ms | 10 | -1080 deg to +1080 deg | ±10 % | 10 deg | Rollover |
| Vehicle roll rate18 | Mandatory if fitted and used for rollover protection system control algorithm | 0 -1.0 sec to at least 250 300 ms | 10 | -240 deg/sec to +240 deg/sec | ±10 % | 4 deg/sec | Rollover |  |
| Anti-lock braking system activity | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, Non-Engaged, Engaged | Planar |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | AEBS |  |
| Stability control | Mandatory | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, On, Off, Engaged | Planar |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | AEBS |  |
| Steering input | Mandatory | -5.0 to 0 sec | 210 | -250 deg CW to + 250 deg CCW. | ±5 % | 1 % 5 deg | Planar |  |
|  |  |  |  |  |  |  | VRU |  |
|  |  |  |  |  |  |  | Rollover |  |
|  |  |  |  |  |  |  | AEBS |  |
| Traction Control Status | Mandatory if not fitted with Stability control | -5.0 to 0 sec | 210 | N/A | N/A | Faulted, On, Off, Engaged | Planar |  |
|  |  |  |  |  |  |  | Rollover |  |

| Table-2 Data elements (new) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data element | Condition for requirement2,3 | Recording interval/time4,5 | Data sample rate | Minimum range | Accuracy6,7 | Resolution8 | Event(s) recorded for9 |
| Pre-Event Synchronization Timer22 | Mandatory | Event | N/A | 0 to 500ms | ±2 ms | 1 ms | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | AEBS |
| Trigger activated | Mandatory | Event | N/A | N/A | N/A | Planar, VRU, Rollover, AEBS | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
|  |  |  |  |  |  |  | AEBS |
| Acceleration control for pedal error (ACPE) status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |
|  |  |  |  |  |  | Off, | VRU |
|  |  |  |  |  |  | On but not intervening, | Rollover |
|  |  |  |  |  |  | Intervening |  |
| [Advanced driver distraction warning (ADDW), number of warnings issued on current trip] | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| [Advanced driver distraction warning (ADDW), time elapsed since its last warning issued on current trip] | Mandatory | Event | N/A | 0 to 240 sec | ±1 sec | 1 sec | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| Blind spot monitoring system status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Off, | Planar |
|  |  |  |  |  |  | Faulted, | VRU |
|  |  |  |  |  |  | On not | Rollover |
|  |  |  |  |  |  | warning, |  |
|  |  |  |  |  |  | On |  |
|  |  |  |  |  |  | warning left- |  |
|  |  |  |  |  |  | side, |  |
|  |  |  |  |  |  | On |  |
|  |  |  |  |  |  | warning |  |
|  |  |  |  |  |  | right-side |  |

| Turn signal switch status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Off, Left, Right | Planar |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| [Driver control assistance system (DCAS) status] | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | Faulted, | Planar |
|  |  |  |  |  |  | Active, | VRU |
|  |  |  |  |  |  | Stand-by, Passive, Inactive, | Rollover |
|  |  |  |  |  |  | Off |  |
| [Driver control assistance system (DCAS), driver monitoring system, number of hands-on requests issued on current trip due to driver disengagement] | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| [Driver control assistance system (DCAS), driver monitoring system, number of eyes-on requests issued on current trip due to driver disengagement] | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |

| [Driver control assistance system (DCAS), driver monitoring system, number of direct control alerts issued on current trip due to driver disengagement] | Mandatory | Event | N/A | 0 to 100 | N/A | 1 | Planar |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| [Driver control assistance system (DCAS), driver monitoring system, time elapsed since its last warning issued on current trip] | Mandatory | Event | N/A | 0 to 60 sec | N/A | 1 sec | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| [Driver control assistance system (DCAS), driver monitoring system, last warning type issued on current trip] | Mandatory | Event | N/A | N/A | N/A | Hands-on request, | Planar |
|  |  |  |  |  |  | Eyes-on request, | VRU |
|  |  |  |  |  |  | Direct Control Alert (due to driver disengagement), | Rollover |
|  |  |  |  |  |  | Direct Control Alert (due to exiting system boundaries), |  |
|  |  |  |  |  |  | No request |  |

| [Driver drowsiness and attention warning (DDAW), number of warnings issued on current trip] | Mandatory | Event | N/A | 0 to 10 | N/A | 1 | Planar |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| [Driver drowsiness and attention warning (DDAW), time elapsed since its last warning issued on current trip] | Mandatory | Event | N/A | 0 to 60 min | ±1 min | 1 min | Planar |
|  |  |  |  |  |  |  | VRU |
|  |  |  |  |  |  |  | Rollover |
| Emergency stop signal status | Mandatory | -5.0 to 0 sec | 2 | N/A | N/A | On, Off | Planar |
|  |  |  |  |  |  |  | VRU |

2  "Mandatory" is subject to the conditions detailed in Section 1.

12 3  "If recorded" means if the data is recorded in non-volatile memory for the purpose of subsequent downloading.

13 For rollover events the time at which the event is determined to have started as defined by the manufacturer.

3 4  Pre-crash data and crash data are asynchronous. The sample time accuracy requirement for pre-crash data [having single data point (e.g., safety belt status, driver)] is -0.1 to 1.0 sec (e.g., T = -1 would need to occur between -1.1 and 0 seconds.).

5  Pre-crash data and crash data are asynchronous. The sample time accuracy requirement for pre-crash data having multiple data points (e.g., vehicle speed) is the sample interval (inverse of sample rate) of the data element. i.e. the last data point may have been captured just before time zero but no more than one sample interval (e.g., data with 2Hz sample rate may have its last sample captured up to 0.5 sec before time zero). All subsequent pre-crash data values are referenced from this data point.

5 6  Accuracy requirement only applies within the range of the physical sensor. If measurements captured by a sensor exceed the design range of the sensor, the reported element shall indicate when the measurement first exceeded the design range of the sensor.

16 7  Relative to the full range of the sensor.

4 8  For data elements with system states, the term "engaged" also means "actively controlling" or "actively intervening" and "not-engaged" also means "on but not controlling". Likewise, "off" also means "deactivated".

6 9  "Planar" includes triggered events in sections 5.3.1.1.1, 5.3.1.1.2, and 5.3.1.1.3

"VRU" includes triggered events in section 5.3.1.1.4.

"Rollover" includes triggered events in section 5.3.1.1.5.

"AEBS" includes triggered events in section 5.3.1.2.

7 10  The ignition cycle at the time of download is not required to be recorded at the time of the crash but shall be reported during the download process.

8 11  The air bag warning lamp is the readiness indicator specified in national air bag requirements and may also illuminate to indicate a malfunction in another part of the deployable restraint system.

9 12 List this element n times, once for each device

18 13 List this element n times, once for each device in 2nd, 3rd, row.

17 14 List this element n - 1 times, once for each stage of a multi-stage air bag system.

10 15 +/- 10 per cent of the full range of the accelerometer used in the Electronic Control Unit (ECU) containing the EDR function as specified in paragraph 6.1.5.

11 16 Format for lateral acceleration recorded in rollover is at the option of the manufacturer.

14 17 These elements do not need to meet the accuracy and resolution requirements in specified crash tests.

15 18 The manufacturer will indicate the direction of positive roll /rate, yaw rate, and roll angle

19 Multiple safety system status indications can be combined into the air bag warning.

20 Faulted = Faulted, Per R79 Off = Off, Standby – ACSF can’t control, Active = ACSF is on but not controlling or ACSF is on and controlling.

21 Triggered means the airbag control unit has sent a message on the bus that emergency call shall be sent via another unit

22 List this element n times, once for each data sample rate of pre-crash data.

II.	Justification

_________________