ETRTO proposal for Action item TA-37-2

ETRTO will prepare a proposal on how to address the core limits, margins and allowances in the same paragraph of the main body by 26 August.

This concept proposal considers [limits setting] proposal of as per TA-37-5

In the main body:

6.1.1.	The abrasion index of the candidate tyre shall not exceed the values given below:

| Category of use |  | Abrasion index (AICT) |
| --- | --- | --- |
| Normal tyre |  | [1.00] |
| Snow tyre |  | [1.00] |
| Special use tyre |  | [Not defined] |

Note: The limits shall be adjusted for the following specific tyre groups as follows

| Tyre group | Stage 1 | Stage 2 |
| --- | --- | --- |
| Standard load tyres | [0.00] | [-0.5] |
| Reinforced or extra load tyres | [+0.10] | [-0.5] |
| Tyres for use in severe snow conditions (3PMSF) | [+0.10] | [+0.10] |
| Normal tyres with a nominal aspect ratio ≤ 40 and a speed category ≥ 300 km/h (speed category symbol Y) | [+0.10] | [-] |
| Tyres with low load index (LI < 77) | [+0.10] | [-] |

The adjustments for specific tyre groups are cumulative, e.g., if a candidate tyre is a reinforced snow tyre for use in severe snow conditions, the limit shall be 1.20 for stage 1 and 1.10 for stage 2.

In Annex 3:

1.11.13.5.	The abrasion index of the candidate tyre shall be calculated from the following equation:

AICT=ALCTALRT-Amargin

Where:

AICT	is the abrasion index of the candidate tyre;

ALCT	is the abrasion level of the candidate tyre, in mg/(km∙t);

ALRT	is the abrasion level of the reference tyre, in mg/(km∙t);

Amargin	is the dimensionless abrasion margin, which accounts for the complexities of the abrasion level measurement methods and the special abrasion characteristics of specific tyre groups as indicated below. Amargin is defined in two steps that account for the evolution of abrasion level measurement methods and tyre technology, as given in the following table: equal to [0.20].

|  |  | Stage 1 | Stage 2 |
| --- | --- | --- | --- |
| Amargin by tyre category of use |  |  |  |
| Normal |  | [0.20] | [0.15] |
| Snow |  | [0.20] | [0.15] |
| Special use |  | [Not defined] | [Not defined] |
| Amargin allowances for specific tyre groups (to be incorporated into Amargin) |  |  |  |
| Tyre for use in severe snow conditions (3PMSF) |  | [+0.10] | [+0.10] |
| Reinforced or extra load tyre (XL) |  | [+0.10] | [-] |
| Tyres with a nominal aspect ratio ≤ 40 and suitable for speeds ≥ 300 km/h |  | [+0.10] | [-] |
| [Tyres with low load index (LI < 77)] |  | [+0.10] | [-] |

Note: Amargin allowances for specific tyre groups are cumulative. E.g., if a candidate tyre is a reinforced snow tyre for use in severe snow conditions, the applicable Amargin shall be 0.20 + 0.1 + 0.1 = 0.4.

2.8.	Processing of Measurement Results

The calculation method for the abrasion index and the abrasion level shall follow the equations:

MlT = MTb – MTa

MlR = MRb - MRa

Where:

MlT	is the mass loss of the candidate tyre, in grams

MlR	is the mass loss of the reference tyre, in grams

MTb	is the mass of the candidate tyre before test cycle, in grams

MTa	is the mass of the candidate tyre after test cycle, in grams

MRb	is the mass of the reference tyre before test cycle, in grams

MRa	is the mass of the reference tyre after test cycle, in grams

The abrasion index (AICT) shall be calculated according to the following equation:

AICT=ALCTALRT-Amargin

Where:

[ALCT	is the abrasion level (mg/(km∙t)) of the candidate tyre,

ALCT = 1000 (mg/kg) x MlT (g)/DT(km)/Fz,T(t)

ALRT		 is the abrasion level (mg/(km∙t)) of the reference tyre,

ALRT = 1000 (mg/kg) x MlR (g)/DR(km)/Fz,R(t)]

DT	is the testing mileage of candidate tyre (km)

DR	is the testing mileage of the reference tyre (km)

Fz,T	is the test load of the candidate tyre (t)

Fz,R	is the test load of the reference tyre (t)

Amargin is the dimensionless abrasion margin, which accounts for the complexities of the abrasion level measurement methods and the special abrasion characteristics of specific tyre groups as indicated below. Amargin is defined in two steps that account for the evolution of abrasion level measurement methods and tyre technology, as given in the following table: equal to [0.20].

|  |  | Stage 1 | Stage 2 |
| --- | --- | --- | --- |
| Amargin by tyre category of use |  |  |  |
| Normal |  | [0.20] | [0.10] |
| Snow |  | [0.20] | [0.10] |
| Special use |  | [Not defined] | [Not defined] |
| Amargin allowances for specific tyre groups (to be incorporated into Amargin) |  |  |  |
| Tyre for use in severe snow conditions (3PMSF) |  | [+0.10] | [+0.10] |
| Reinforced or extra load tyre (XL) |  | [+0.10] | [-] |
| Tyres with a nominal aspect ratio ≤ 40 and suitable for speeds ≥ 300 km/h |  | [+0.10] | [-] |
| Tyres with low load index (LI < 77) |  | [+0.10] | [-] |

Note: Amargin allowances for specific tyre groups are cumulative. E.g., if a candidate tyre is a reinforced snow tyre for use in severe snow conditions, the applicable` Amargin shall be 0.20 + 0.1 + 0.1 = 0.4. The reference tyre used to calculate the abrasion index shall be one of the tyres described in paragraph 2.4.7. of this Annex.