Proposal for amendments to ECE/TRANS/WP.29/GRVA/2026/3

The text reproduced below is based on that of GRVA/2026/3. The modifications to that text are indicated in bold for new characters and strikethrough for deleted characters.

I.	 Proposal

Paragraph 5.3.1. of Annex 8, amend to read:

"5.3.1.	The data elements shall be recorded if the following thresholds are reached or conditions occur:

(a)    Detected collision

(b)    EDR trigger input (excluding last stop trigger)

Vehicles approved in accordance with the provisions of Regulation No. 169, where a Supplementary Restraint System (SRS) is installed, the use of the SRS trigger input alone shall be permitted for the purposes of EDR trigger input.

II.	 Justification

Paragraph 5.3.1(b) currently relies on “EDR trigger input (excluding last stop trigger)”. For vehicles under UN Regulation No. 169, this can encompass multiple trigger sources (e.g., ABS, ESC, sudden deceleration, and—where present—SRS). In UN Regulation No. 160 (passenger cars), the SRS is the practically relevant trigger. The proposed amendment aims to limit time-series EDR trigger input to the SRS related trigger where SRS is installed.

Industry field data from heavy duty fleets indicates that sudden deceleration triggers can occur on the order of ~6,000 events per vehicle per year, with additional frequent ABS/ESC interventions under normal duty cycles; this trigger volume is disproportionate to DSSAD’s intended focus on crash relevant events and drives excessive storage consumption.