Proposal to amend document ECE/TRANS/WP.29/2026/8, based on document ECE/TRANS/WP.29/GRVA/2025/40 as adopted during the 23rd session of GRVA

This proposal aims to correct an unintended modification of the text of paragraph 5.1.2.2. regarding secondary braking system which was introduced by document GRVA/2025/40, unintentionally aligning the wording of the paragraph in UN Regulation No. 13-H with the wording of UN Regulation No. 13.

- Proposal

Paragraph 5.1.2.2., amend to read:

“5.1.2.2.     	Secondary braking system

The secondary braking system shall make it possible by application of the service brake control or by demand from the ADS (as applicable) to halt the vehicle within a reasonable distance in the event of failure of the service braking system. It shall be possible to graduate this braking action. A driver, (if any), shall be able to obtain this braking action from his driving seat without removing his hands from while keeping at least one hand on the steering control. For the purposes of these provisions, it is assumed that not more than one failure of the service braking system can occur at one time.”

- Justification

TF-FADS invites experts from GRVA to consider the proposed changes with the aim to amend document ECE/TRANS/WP.29/2026/8 for adoption at WP.29 in its 199th  session in June 2026:

- The blue text is correcting unintended changes caused by a copy paste issue, by taking over modifications from UN Regulation No. 13 into UN Regulation No. 13-H. The proposed wording is recreating the original wording before the adoption of ECE/TRANS/WP.29/GRVA/2025/40.

- The purple text introduces required changes for the application of the secondary braking system provisions to an ADS, which would otherwise conflict with the original wording (blue text).

.