Proposal for supplement 1 to the 07 Series of Amendments to UN Regulation No. 46 (Safety Glazing Indirect vision)

Submitted by the expert from the Kingdom of the Netherlands on behalf of the Task Force on Automated Vehicles Regulatory Screening*

The text reproduced below was prepared by the Task Force on Automated Vehicles Regulatory Screening (TF-AVRS), with the aim to amend and supersede document ECE/TRANS/GRSG/2025/20. The modifications to ECE/TRANS/GRSG/2025/20 are marked in (bold) red for new or strikethrough for deleted characters.

I.	Proposal

Insert new paragraphs 0. and 0.1. to 0.1.4., to read:

"0.		Introduction

0.1.		Supplement 1 to the 07 series of amendments is introduced to take into account vehicles of categories X and Y 1, as well as vehicles with a manual mode operating at speeds above 6 km/h which are equipped with an  Automated Driving System (ADS) 1.

0.1.1. 	The scope is updated to reflect that this Regulation does not apply to vehicles of categoryies X and Y, since such vehicles do not have a (human) driver and therefore do not need to provide indirect vision.

0.1.2. 	In case of vehicles equipped with an ADS other than vehicles of categoryies X and Y and vehicles of category X without a manual driving mode operated on-board the vehicle, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.

0.1.3.	Vehicles of category X with a manual driving mode that is operated on-board the vehicle using a driver’s seat, in the manual driving mode no special provisions or exemptions apply.

0.1.4.	Vehicles of category X with a manual driving mode that is operated on-board the vehicle not using a driver’s seat, may be approved using alternative requirements, in agreement with the Technical Service and Type Approval Authority provided that the manufacturer, with the safety concept defined for the ADS, can demonstrate that a level of safety, equivalent to that of the requirements set out in this Regulation, is achieved. This may be achieved by e.g. sensors, cameras and brake features, which prevent collision between the vehicle and other vehicles, objects or vulnerable road users at speeds below 6 km/h"

Paragraph 1. - Scope, amend to read:

"1.		Scope

1.1.		This Regulation applies:

(a)	To compulsory and optional devices for indirect vision, set out in the table under paragraph 15.2.1.1.1. of this Regulation for vehicles of category M and N1 and to compulsory and optional devices for indirect vision mentioned in paragraphs 15.2.1.1.3. and 15.2.1.1.4. of this Regulation for vehicles of category L1 with bodywork at least partly enclosing the driver;

(b)	To the installation of devices for indirect visions on vehicles of categories M and N and on vehicles of category L1 with bodywork at least partly enclosing the driver.

(c) 	To the installation of surveillance mirrors and surveillance camera monitor-recording devices with regard to the requirements of paragraphs 15.2.1.2. and 16.3. respectively of this Regulation as applicable. These devices do not need to fulfil any other technical requirements of this Regulation."

1.2.	This Regulation does not apply:

(a)	To devices other than those prescribed under paragraph 1.1.(a) for observing the vision area(s) immediately adjacent to the front and/or the passenger's side of vehicles of category M1, M2, M3, N1 and N2 ≤ 7.5 t.

(b)	To vehicles of category X and Y 1, and to vehicles of category X 1 without a manual mode operated on-board the vehicle."

Paragraph 1.1. and 1.2., footnote 1, amend to read:

"1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Insert new paragraphs 15.1.5. and 15.1.6., to read:

"15.1.5.	The requirements in part II do not apply whilest an ADS x feature is active. In case of vehicles equipped with an ADS, the transitions of user roles and the procedure for those transitions, for example, reversion to manual driving following deactivation of the ADS feature, shall comply with the technical requirements defined in UN Regulation No. [XXX].

15.1.6.	Vehicles of category X with a manual mode intended to be operated on-board the vehicle not using a driver’s seat, may fulfil alternative requirements, in agreement with the Technical Service and Type Approval Authority, provided that the manufacturer, with the safety concept defined for the ADS, can demonstrate that a level of safety, equivalent to that of the requirements set out in the table under paragraph 15.2.1.1.1. below, is achieved. The alternative provisions that are applied and the vehicle’s performance must be duly described in the Test Report."

Insert footnote x, to read:

"x	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev. 8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.