Proposal for supplement [1] to UN Regulation No.168

(to be agreed and submitted to GRPE by TF-AVRS)

Notes:

Modifications to the existing text of the Regulation indicated in black bold for new characters and black strikethrough for deleted characters.

I.	Proposal

Paragraph 3.3., amend to read:

3.3.	Vehicle characteristics and driver (if applicable)

Paragraph 3.3.3., amend to read:

3.3.3.	"Mass in running order" means the mass of the vehicle, with its fuel tank(s) filled to at least 90 per cent of its or their capacity/capacities, including the mass of the driver (if applicable), fuel and liquids, fitted with the standard equipment in accordance with the manufacturer's specifications and, when they are fitted, the mass of the bodywork, the cabin, the coupling and the spare wheel(s) as well as the tools. For vehicles of categories X and Y the driver’s mass shall not be included.

Paragraph 3.3.5., amend to read:

3.3.5.	"Odometer" means an instrument indicating to the driver vehicle user the total distance driven by the vehicle since its production.

Note “a” of Figure 3, amend to read:

a	means the mass of the vehicle, with its fuel tank(s) filled to at least 90 per cent of its or their capacity/capacities, including the mass of the driver (if applicable), fuel and liquids, fitted with the standard equipment in accordance with the manufacturer’s specifications and, when they are fitted, the mass of the bodywork, the cabin, the coupling and the spare wheel(s) as well as the tools. For vehicles of categories X and Y the driver’s mass shall not be included.

Paragraph 6.1.(last section), amend to read:

6.1		The RDE performance shall be demonstrated by performing the necessary tests in the PEMS test family on the road operated over their normal driving patterns, conditions and payloads. The necessary tests shall be representative for vehicles operated on their real driving routes, with their normal load.

Paragraph 6.4.1.3., amend to read:

6.4.1.3.	With agreement of the authority, a PEMS test can also be driven conducted by a different vehicle user witnessed by a Technical Service, provided that at least the tests of the vehicles required by paragraphs 6.4.2.2. 6.4.3.2.and 6.4.2.6. 6.4.3.6. and in total at least 50 per cent of the PEMS tests required by paragraph 6.4.3.7. for validating the PEMS test family are driven conducted by a Technical Service. In such case the Technical Service remains responsible for the proper execution of all PEMS tests pursuant to the requirements of this Regulation.

6.4.1.3.	With the agreement of the authority, a the on-road PEMS test can also be driven carried out by a different vehicle user while witnessed by a Technical Service, provided that at least the tests of the vehicles required by paragraphs 6.4.2.2. 6.4.3.2.and 6.4.2.6. 6.4.3.6. and in total at least 50 per cent of the PEMS tests required by paragraph 6.4.3.7. for validating the PEMS test family are driven carried out by a Technical Service using the vehicle. In such case the Technical Service remains responsible for the proper execution of all PEMS tests pursuant to the requirements of this Regulation.

Paragraph 8.3.1., amend to read:

8.3.1.	Vehicle condition

The vehicle, including the emission related components, shall be in good mechanical condition and shall have been run in and driven at least 3,000 km before the test. The mileage and the age of the vehicle used for RDE testing shall be recorded. All vehicles, and in particular OVC-HEVs vehicles may be tested in any selectable mode, including battery charge mode. On the basis of technical evidence provided by the manufacturer and with the agreement of the responsible authority, the dedicated vehicle user-selectable modes for very special limited purposes shall not be considered (e.g. maintenance mode, race driving, crawler mode). All remaining modes used for forward and for rearwards driving where road and traffic conditions demand this may be considered and the criteria emissions limits shall be fulfilled in all these modes. Modifications that affect the vehicle aerodynamics are not permitted, with the exception of the PEMS installation. The tyre types and pressure shall be according to the vehicle's manufacturer recommendations. The tyre pressure shall be checked prior to the pre-conditioning and adjusted to the recommended values if needed. Driving the vehicle with snow chains is not permitted. Vehicles should not be tested with an empty starter battery. In case the vehicle has problems starting, the battery shall be replaced following the recommendations of the vehicle's manufacturer.

The vehicle's test mass comprises of the driver (if applicable), a witness of the test (if applicable), the test equipment, including the mounting and the power supply devices and any artificial payload. It shall be between the actual mass of the vehicle and the maximum permissible test mass of the vehicle at the beginning of the test and shall not increase during the test. The test vehicles shall not be driven with the intention to generate a passed or failed test due to extreme driving that do not represent normal conditions of use. If necessary, verification of normal driving  may be based on expert judgement made by or on behalf of the granting type approval authority through cross-correlation on several signals, which may include exhaust flow rate, exhaust temperature, CO2, O2 etc. in combination with vehicle speed, acceleration and GNSS data and potentially further vehicle data parameters like engine speed, gear, accelerator pedal position acceleration demand etc.

Paragraph 8.3.2., amend to read:

8.3.2.	Vehicle conditioning for cold start PEMS trip

Before RDE testing, the vehicle shall be preconditioned in the following way:

The vehicle shall be driven, preferably on the same route as the planned RDE testing, or for at least 10 min per type of operation (e.g. urban, rural, motorway) or 30 minutes with a minimum average speed of 30 km/h. The validation test in the laboratory, as in paragraph 8.4., also counts as preconditioning. The vehicle shall subsequently be parked with doors and bonnet closed and kept in engine-off status within moderate or extended altitude and temperatures, in accordance with paragraph 8.1., for between 6 and 72 hours. Exposure to extreme atmospheric conditions (such as heavy snowfall, storm, hail) and excessive amounts of dust or smoke should be avoided.

Before the test start, the vehicle and equipment shall be checked for damages and the presence of warning signals that may suggest malfunctioning. In the case of a malfunction the source of the malfunctioning shall be identified and corrected or the vehicle shall be rejected.

Paragraph 8.3.3., amend to read:

8.3.3.		Auxiliary devices

The air conditioning system or other auxiliary devices shall be operated in a way which corresponds to their typically intended use during real driving operation on the road. Any use shall be documented. The vehicle windows shall be closed when the air conditioning or heating are used.

Paragraph 8.3.4.3., amend to read:

8.3.4.3.	If regeneration occurred during the test, the final emission result without the application of either the Ki -factor or the Ki offset shall be checked against applicable emission limits. If the final emissions are above the emission limits, then the test shall be invalid and repeated once. The completion of the regeneration and stabilisation, through approximately 1 hour of driving, shall be done prior to the start of the second test. The second test is considered valid even if regeneration occurs during it.

Even if the final emission results fall below the applicable emission limits, the occurrence of regeneration may be verified as in paragraph 8.3.4.2. If the presence of regeneration can be proved and with the agreement of the Type Approval Authority, the final results shall be calculated without the application of either the Ki factor or the Ki offset.

Paragraph 9.1.1., amend to read:

9.1.1.		Other requirements

The average speed (including stops) of the urban speed bin shall be between 15 and 40 km/h.

The speed range of the motorway driving shall properly cover a range between 90 and at least 110 km/h. The vehicle’s speed shall be above 100 km/h for at least 5 minutes.

For M2 category vehicles that are equipped with a device permanently limiting vehicle speed to 100 km/h, the speed range of the motorway speed bin shall properly cover a range between 90 and 100 km/h. The vehicle’s speed shall be above 90 km/h for at least 5 minutes.

For those vehicles that are equipped with a device limiting vehicle speed to 90 km/h, the speed range of the motorway speed bin shall properly cover a range between 80 and 90 km/h. The vehicle’s speed shall be above 80 km/h for at least 5 minutes.

In the case that the local speed limits for the specific vehicle being tested prevent compliance with the requirements of this paragraph, the requirements of the following paragraph shall apply:

The speed range of the motorway driving shall properly cover a range between X – 10 and X km/h. The vehicle’s speed shall be above X - 10 km/h for at least 5 minutes. Where X = the local speed limit for the tested vehicle.

Paragraph 9.3., amend to read:

9.3.		RDE test to be performed

The RDE performance shall be demonstrated by testing vehicles on the road, operated over their normal driving patterns, conditions and payloads. RDE tests shall be conducted on paved roads (e.g. off-road operation is not permitted). Either a single RDE trip or two dedicated RDE trips shall be driven in order to prove compliance with the emission requirements against both 3-Phase WLTC and 4-Phase WLTC.

Paragraph 9.3.1., amend to read:

9.3.1.		The design of the trip shall be such as to comprise driving that would in principle cover all of the required shares of speed bins in paragraph 9.2 and comply with all other requirements described in paragraphs 9.1.1. and 9.3, paragraphs 4.5.1. and 4.5.2. of Annex 8 and paragraph 4. of Annex 9.

Paragraph 9.3.2., amend to read:

9.3.2.		The planned RDE trip shall always start with urban operation followed by rural, then motorway or expressway operation, in accordance with the required shares for speed bins in paragraph 9.2. The urban, rural and motorway/expressway operation shall be run consecutively, but may also include a trip which starts and ends at the same point. Rural operation may be interrupted by short periods of urban speed bin when driving through urban areas. Motorway/expressway operation may be interrupted by short periods of urban or rural speed bins, e.g., when passing toll stations or sections of road work.

Paragraph 9.3.3., amend to read:

9.3.3.	The vehicle speed shall normally not exceed 145 km/h. This maximum speed may be exceeded by a tolerance of 15 km/h for not more than 3 per cent of the time duration of the motorway operation. Local speed limits remain in force during a PEMS test, notwithstanding other legal consequences. Violations of local speed limits per se do not invalidate the results of a PEMS test.

Stop periods, defined by vehicle speed of less than 1 km/h, shall account for 6-30 per cent of the time duration of urban operation. Urban operation may contain several stop periods of 10 s or longer. If stop periods in urban driving part are over 30 per cent or there are individual stop periods exceeding 300 consecutive seconds, the test shall be invalid only if the emission limits are not met.

The trip duration shall be between 90 and 120 minutes.

The start and the end points of a trip shall not differ in their elevation above sea level by more than 100 m. In addition, the proportional cumulative positive altitude gain over the entire trip and over the urban operation shall be less than 1,200 m/100 km and be determined in accordance with Annex 10.

Paragraph 9.3.5., insert new paragraph to read:

9.3.5.	For vehicles tested in an ADS mode, it is permissible to assign way-points along the planned route in order to enable  the test to be conducted as planned.

Paragraph 9.4., amend to read:

9.4.	Other trip requirements

If the engine stalls during the test, it may be restarted, but the sampling and data recording shall not be interrupted. If the engine stops during the test, the sampling and data recording shall not be interrupted.

In general the exhaust mass flow shall be determined by measurement equipment functioning independently from the vehicle. With agreement of the approval authority, vehicle ECU data may be used in this respect during type approval.

If the approval authority is not satisfied with the data quality check and validation results of a PEMS test conducted in accordance with Annex 4, the approval authority may consider the test to be invalid. In such case, the test data and the reasons for invalidating the test shall be recorded by the approval authority.

The manufacturer shall demonstrate to the approval authority that the chosen vehicle, driving patterns, conditions and payloads are representative of the PEMS test family. The ambient conditions and payload requirements, as specified in paragraph 8.1. and paragraph 8.3.1. respectively, shall be used ex-ante to determine whether the conditions are acceptable for RDE testing.

The approval authority shall propose a test trip in urban, rural and motorway/expressway operation meeting the requirements of paragraph 9.2. If applicable, for the purpose of trip design, the urban, rural and motorway/expressway parts shall be selected based on a topographic map.

If for a vehicle the collection of ECU data influences the vehicle's emissions or performance, the entire PEMS test family to which the vehicle belongs shall be considered as non-compliant.

For RDE tests performed during type approval, the type approval authority may verify if the test setup and the equipment used fulfil the requirements of Annexes 4 and 5 through a direct inspection or an analysis of the supporting evidence (e.g. photographs, records).

Annex 1, paragraph 12.10., amend to read:

12.10.	Devices or systems with vehicle user selectable modes which influence CO2 emissions, electric energy consumption and/or criteria emissions and do not have a predominant mode: yes/no (1)

Annex 1, explanatory note (h), amend to read:

(h)	The mass of the driver (if applicable) is assessed at 75 kg

Annex 4, Table A4/1, amend to read:

Pedal position Acceleration demand7

Annex 4, paragraph 5.3, amend to read:

5.3	Test end

Excessive idling of the engine after the completion of the trip shall be avoided. The data recording shall continue after the test end (as defined in paragraph 3.8.6. of this Regulation) and until the response time of the sampling systems has elapsed. For vehicles with a signal detecting regeneration, the OBD-check shall be performed and documented directly after data recording and before any further driven distance is driven.

Annex 8, paragraph 3.1., amend to read:

3.1	Definition of averaging windows

The instantaneous CO2 emissions calculated according to Annex 7 shall be integrated using a moving averaging window method, based on a reference CO2 mass.

The usage of the reference CO2 mass is illustrated in Figure A8/2. The principle of the calculation is as follows: The RDE distance-specific CO2 mass emissions are not calculated for the complete data set, but for sub-sets of the complete data set, the length of these sub-sets being determined so as to always match the same fraction of the CO2 mass emitted by the vehicle over the applicable WLTP test (after all appropriate corrections e.g. ATCT are applied, where relevant). The moving window calculations are conducted with a time increment Δt corresponding to the data sampling frequency. These sub-sets used to calculate the vehicle on-road CO2 emissions and its average speed are referred to as ‘averaging windows’ in the following sections. The calculation described in this point shall be run from the first data point (forward), as shown in Figure A8/1.

The following data shall not be considered for the calculation of the CO2 mass, the distance and the vehicle average speed in each averaging window:

The periodic verification of the instruments and/or after the zero drift verifications;

Vehicle ground speed < 1 km/h;

The calculation shall start from when vehicle ground speed is higher than or equal to 1 km/h and include driving events during which no CO2 is emitted and where the vehicle ground speed is higher than or equal to 1 km/h.

The mass emissions MCO2,j shall be determined by integrating the instantaneous emissions in g/s as specified in Annex 7.

Annex 8, paragraph 4.5.1.1., amend to read:

4.5.1.1.	Tolerances around the vehicle CO2 characteristic curve

The upper tolerance of the vehicle CO2 characteristic curve is tol1H=45% for low speed driving and tol1H=40% for medium and high speed driving.

The lower tolerance of the vehicle CO2 characteristic curve is tol1L=25% for ICE and NOVC-HEV vehicles and tol1L=100% for OVC-HEV vehicles.

Annex 8, paragraph 4.5.2.1., amend to read:

4.5.2.1.	Tolerances around the vehicle CO2 characteristic curve

The upper tolerance of the vehicle CO2 characteristic curve is tol1H=45% for low speed driving and tol1H=40% for high speed driving.

The lower tolerance of the vehicle CO2 characteristic curve is tol1L=25% for ICE and NOVC-HEV vehicles and tol1L=100% for OVC-HEV vehicles.

Annex 11, paragraph 2., amend to read:

2.		Symbols, Parameters and Units

Index (k) refers to the category (t=total, u=urban, 1-2=first two phases of the WLTP test)

ICk	is the distance share of usage of the internal combustion engine for an OVC-HEV over the RDE trip

dICE,k	is the distance driven [km], with the internal combustion engine on for an OVC-HEV over the RDE trip

dEV,k	is the distance driven [km], with the internal combustion engine off for an OVC-HEV over the RDE trip

MRDE, k	is the final RDE distance-specific mass of gaseous pollutants [mg/km] or particle number [#/km]

mRDE, k 	is the distance-specific mass of gaseous pollutant [mg/km] or particle number [#/km] emissions, emitted over the complete RDE trip and prior to any correction in accordance with this annex

MCO2,RDE, k 	is the distance-specific mass of CO2 [g/km], emitted over the RDE trip

MCO2,WLTC, k 	is the distance-specific mass of CO2 [g/km], emitted over the WLTC cycle

MCO2,WLTC_CS, k 	is the distance-specific mass of CO2 [g/km], emitted over the WLTC cycle for an OVC-HEV vehicle tested in charge sustaining vehicle operation

rk 	is the ratio between the CO2 emissions measured during the RDE test and the WLTP test

RF k		is the result evaluation factor calculated for the RDE trip

RFL1		is the first parameter of the function used to calculate the result evaluation factor

RFL2	is the second parameter of the function used to calculate the result evaluation factor

Annex 11, paragraph 3.2., amend to read:

3.2.		RDE result evaluation factor for OVC-HEV

The value of the RDE result evaluation factor depends on the ratio rk between the distance-specific CO2 emissions measured during the RDE test and the distance-specific CO2 emitted by the vehicle over the applicable WLTP test conducted in Charge Sustaining vehicle operation including all appropriate corrections. The ratio rk is corrected by a ratio reflecting the respective usage of the internal combustion engine during the RDE trip and on the WLTP test, to be conducted in charge sustaining vehicle operation.

For either the urban or the total driving:

rk=MCO2,RDE, kMCO2,WLTP_CS, t×0.85ICk

where ICk is the ratio of the distance driven either in urban or total trip with the combustion engine activated, divided by the total urban or total trip distance:

ICk=dICE,kdICE,k+dEV,k

With determination of combustion engine operation in accordance with paragraph 3.6.3. of this Regulation.