|  | United Nations | ECE/TRANS/WP.29/GRBP/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 17 September 2024 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Noise and Tyres

Seventy-seventh session

Geneva, 5–9 May 2025

Item Y of the provisional agenda

UN Regulation No. 142 (Tyres Installation)

Proposal for supplement 3 to the 01 Series of Amendments to UN Regulation No. 142 (Tyres Installation)

Submitted by the GRBP Task Force AVRS (Automated Vehicle Regulation Screening)

The text reproduced below was prepared by the expert from the European Commission on behalf of the TF GRBP AVRS, to enable the application of the regulation to vehicles equipped with an ADS. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters.

- Proposal

Paragraph 1., Footnote 1, amend to read:

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document

ECE/TRANS/WP.29/78/Rev.68, para. 2 -

www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions

Insert new paragraphs 2.22. to 2.25, to read:

2.22.	“Automated Driving System (ADS)” means the vehicle hardware and software that are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis1.

2.23.	“Dynamic Driving Task (DDT)” means the real-time operational and tactical functions required to operate the vehicle1.

2.24.	“Category X vehicles” are vehicles of categories M, N, L and T meeting all of the following conditions1:

(a) 	They are equipped with an ADS

(b) 	They are not capable of being driven manually at speeds exceeding 6 km/h

(c) 	They are designed to carry occupants

2.25.	“Category Y vehicles” are vehicles of categories N, L and T meeting all of the following conditions1:

(a) 	They are equipped with an ADS

(b) 	They are not capable of being driven manually at speeds exceeding 6 km/h

(c) 	They are not designed to carry occupants at any time

Paragraph 4.4.1., Footnote 2, amend to read:

2 The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.68, Annex 3 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions

Paragraph 5.2.3.2.2., amend to read:

        - In the case of vehicles normally equipped with normal tyres and occasionally fitted with snow tyres for use in severe snow conditions (i.e. with the alpine or three-peaked mountain snowflake symbol marking) where in such a case the speed category symbol of the snow tyre for use in severe snow conditions shall correspond to a speed either greater than the maximum vehicle design speed or not less than 160 km/h (or both). However, if the maximum vehicle design speed is greater than the speed corresponding to the lowest speed category symbol of the fitted snow tyres for use in severe snow conditions, a maximum speed warning label, specifying the lowest value of the maximum speed capability of the fitted snow tyres for use in severe snow conditions, shall be displayed inside the vehicle in a prominent position readily and permanently visible to the driver. Other tyres with improved snow traction (i.e. with the M+S marking, but without the alpine or three-peaked mountain snowflake symbol marking) shall comply with the requirements of paragraphs 5.2.3.1.1. and 5.2.3.1.2. of this Regulation.

For vehicles equipped with an ADS, it shall be ensured that information is sent to the ADS about the speed corresponding to the lowest speed category symbol of the fitted severe snow tyres.

The speed warning label shall not be required for vehicles of categories X and Y.

Paragraph 5.2.3.2.3., amend to read:

5.2.3.2.3.	In the case of vehicles equipped with special use tyres. However, if the maximum vehicle design speed is greater than the speed corresponding to the lowest speed category symbol of the fitted special use tyres, a maximum speed warning label, specifying the lowest value of the maximum speed capability of the fitted special use tyres, shall be displayed inside the vehicle in a prominent position readily and permanently visible to the driver.

For vehicles equipped with an ADS, it shall be ensured that information is sent to the ADS about the speed corresponding to the lowest speed category symbol of the fitted special use tyres.

The speed warning label shall not be required for vehicles of categories X and Y.

Paragraph 5.2.3.2.4., amend to read:

5.2.3.2.4.	In the case of vehicles of categories M2, M3, N2 or N3 equipped with a speed limitation device (SLD) approved according to UN Regulation No. 89 where in such a case the speed symbol of the tyres shall be compatible with the speed at which the limitation is set. However, if the vehicle manufacturer has foreseen that the maximum vehicle design speed is greater than the speed corresponding to the lowest speed category symbol of the fitted tyres, a maximum speed warning label, specifying the maximum speed capability of the tyres, shall be displayed inside the vehicle in a prominent position readily and permanently visible to the driver.

For vehicles equipped with an ADS, it shall be ensured that information is sent to the ADS about the speed corresponding to the lowest speed category symbol of the fitted tyres.

The speed warning label shall not be required for vehicles of categories X and Y.

Paragraph  5.2.3.2.5., amend to read:

5.2.3.2.5.	In the case of vehicles of categories M1 or N1 equipped with an on- board system fulfilling a speed limitation device / function where in such a case the speed symbol of the tyres shall be compatible with the speed at which the limitation is set.

However, if the vehicle manufacturer has foreseen that the maximum vehicle design speed is greater than the speed corresponding to the lowest speed category symbol of the fitted tyres, a maximum speed warning label, specifying the maximum speed capability of the tyres, shall be displayed inside the vehicle in a prominent position readily and permanently visible to the driver.

For vehicles equipped with an ADS, it shall be ensured that information is sent to the ADS about the speed corresponding to the lowest speed category symbol of the fitted tyres.

The speed warning label shall not be required for vehicles of categories X and Y.

Paragraph  5.2.3.3., amend to read:

5.2.3.3.	The manufacturer shall provide the necessary information about suitable replacement tyres with an appropriate speed capacity in the vehicle owner's handbook or by any other communication means in the vehicle.

For vehicles equipped with an ADS, it shall be ensured that a signal is sent to the ADS whenever a replacement tyre is installed. This signal shall include the speed capacity of the replacement tyre.

Annex 1, Footnote 4, amend to read:

4 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.68, para. 2 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions

Annex 1, Footnote 5, amend to read:

5 Where there is one version with a normal cab and another with a sleeper cab or without a cab, bothall three sets of masses and dimensions are to be stated.

Insert new paragraph Annex 1, 6.2., to read:

6.2.	Vehicle is equipped with an ADS: yes/no

Insert new paragraph Annex 2, Section I, paragraph 2.2., to read:

2.2. 	Vehicle equipped with an ADS: yes/no2

Insert new paragraph Annex 2, Addendum to communication form, paragraph 4.2., to read:

4.2. 	Brief description of the automated driving system (ADS) (if fitted): …