|  | United Nations | ECE/TRANS/WP.29/GRE/2025/xx |  |
| --- | --- | --- | --- |
|  | SLR-74-02 Economic and Social Council |  | Distr.: General DATE Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

xxxxx session

Geneva, xx Month 20xx

Item x of the provisional agenda

Simplification of lighting and light-signalling UN Regulations

Proposal for corrections and clarification of the text of UN Regulation No. 150-01 series (Retro-Reflective Devices)

Submitted by the Informal Working Group on Simplification of Lighting and Light-Signalling Regulations (IWG SLR)*

The text reproduced below was prepared by IWG SLR with the aim to correct the Figures in Annex 24 to UN Regulation No. 150 and to improve the readability of the whole Annex.

I.	Proposal

# 5.	Specific technical requirements

## 5.1.	Technical requirements concerning retro-reflectors of the Classes IA, and IB, IIIA, IIIB and IVA (Symbols "IA", and "IB", IIIA, “IIIB” and “IVA”)

### 5.1.1.	Every retro-reflector of the Classes IA and IB, when tested according to paragraph 5.1.7., shall meet:

### 5.1.3.	Test procedure.

#### 5.1.3.1.	After verification of the general specifications (paragraph 4.) and the specifications of shape and dimensions (Annex 5), the ten samples shall be subjected to the heat resistance test described in Part 1 of Annex 6 and at least one hour after this test examined as to their colorimetric characteristics in paragraph 5.1.5. and RI in paragraph 5.1.4., for an angle of divergence of 20' and an illumination angle 1 = 2 = 0° or if necessary, in the position defined in Part 1 of Annex 4, paragraphs 1.1. and 1.2.

The two retro-reflectors giving the minimum and maximum values shall then be fully tested as shown in paragraph 5.1.4.

These two samples shall be kept by the laboratories for any further checks which may be found necessary.

Four samples out of the remaining eight samples shall be selected at random and divided into two groups of two in each group.

5.1.3.1.1.	Class IA, Class IB, Class IIIA and Class IIIB

The other eight samples shall be divided into four groups of two:

First group:	The two samples shall be subjected successively to the water penetration test (Part 2 of Annex 6) and then, if this test is satisfactory, to the tests for resistance to fuels and lubricants (Parts 1 and 2 of Annex 7).

Second group:	The two samples shall, if necessary, be subjected to the corrosion test in Part 4 of Annex 6, and then to the abrasive-strength test of the rear face of the retro-reflective device in Part 5 of Annex 6.

Third group:	The two samples shall be subjected to the test for stability in time of the optical properties of retro-reflective device in Part 3 of Annex 4.

Fourth group:	The two samples shall be subjected to the resistance to weathering colour-fastness test (Part 67 of Annex 6).

5.1.3.1.2. 	Class IVA

Four samples out of the remaining eight samples shall be selected at random and divided into two groups of two in each group.

First group:	The two samples shall be subjected successively to the water penetration test (Part 2 of Annex 6) and then, if this test is satisfactory, to the tests for resistance to fuels and lubricants (Parts 1 and 2 of Annex 7).

Second group:	The two samples shall, if necessary, be subjected to the corrosion test in Part 4 of Annex 6, and then to the abrasive-strength test of the rear face of the retro-reflective device in Part 5 of Annex 6, these two samples shall also be subjected to the impact test in Part 4 of Annex 8.

#### 5.2.6.3.	The testing of the day-time colour for retro-reflective device shall be carried out according to the method described in paragraph 4.2.2.

Amend to read: …

#### “5.2.6.4.	Luminance factor determined in accordance to Part 2 of Annex 4:

for red colour shall be βv,R≥ 0.03,

for yellow colour shall be βv,R≥ 0.16,

for white colour, it shall be βv,R≥ 0.25.

except classes C, D, E and D/E ”

When measured with a spectrophotometer in accordance with the provisions of CIE document No. 15 (1971) and illuminated with the CIE Standard Illuminant D65 at an angle of 45º to the normal and viewed along the normal (45/0 geometry), the colour of the material in new condition shall be located within the area defined by the chromaticity co-ordinates in Table 8 and comply with the luminance factor.

Table 8

Chromaticity co-ordinates x and y

| Colour |  | 1 | 2 | 3 | 4 | Luminance factor βv,R |
| --- | --- | --- | --- | --- | --- | --- |
| Yellow | x | 0.545 | 0.487 | 0.427 | 0.465 | ≥ 0.16 |
|  | y | 0.454 | 0.423 | 0.483 | 0.534 |  |
| Red | x | 0.690 | 0.595 | 0.569 | 0.655 | ≥ 0.03 |
|  | y | 0.310 | 0.315 | 0.341 | 0.345 |  |

Amend to read: …

5.3.4.2	Retro-reflective devices and fluorescent retro-reflecting material.

Table 89

….

5.3.5.1.2.	The testing of the colour for retro-reflective device (night-time colour) shall be carried out according to the method described in paragraph 4.2.1. and the trichromatic co-ordinates of the red reflected luminous flux shall be within the following limits:

Table 910

5.3.5.2.2.	The testing of the colour of the fluorescent materials (daytime colour) of advance warning triangle of type 1 or type 2 shall be carried out according to the method described in paragraph 4.2.2. and the colour of the material in new condition shall be within an area of which the corner points are determined by the following coordinates as specified in Table 1011:

Table 1011

Annex 1

Communication

5.		Submitted for approval on Date on which the marking material was submitted for approval tests:

# Annex 6

# Environmental Testing

Part 4 -	Resistance to corrosion

4.2.	The coefficient of retro-reflection RA of the retro-reflective areas, when measured after a recovery period of 48 hours as specified in Part 2 of Annex 6, at an entrance angle of β2 = 5 and an observation angle of α = 20', shall be not less than the value in Table 910 or more than the value in Table 1011 respectively. Before measuring, the surface shall be cleaned to remove salt deposits from the saline mist.

Insert new Part 7 to read:

Part 7 - 	Colour-fastness1 of retro-reflective devices of the Classes IA, IB, IIIA, IIIB and IVA

1.	The Type Approval Authority which granted approval shall have the right to check the colour-fastness of a type of retro-reflective device in service.

2.	The Type Approval Authorities of countries other than the country in which approval was granted may carry out similar checks in their territory. If a type of retro-reflector in use exhibits a systematic defect, the said authorities shall transmit any components removed for examination to the Type Approval Authority which granted approval, with a request for its opinion.

3.	In the absence of other criteria, the concept "systematic defect" of a type of retro-reflector in use shall be interpreted in conformity with the intention of paragraph 3.6.1. of this Regulation.

…”

II.	Justification

1.	General remark

The Regulation 150-00 series had in the first step of the simplification process the same content of the specification for retroreflective devices coming from the Regulations R3, R27, R69, R70 and R104. In the next step of simplification in restructuring the chapters and grouping the requirements some errors occur, which had been recognized, when type approval process on the base of Regulation 150-01 series had been requested.

2.	In chapter 5 the colour fastness testing had been applied for the classes IA, IB, IIIA, IIIB and IVA, but in Regulation 150-00 series for the class IVA retro-reflector a different grouping for the samples for testing was specified. For the class IVA a colour-fastness test will be not mandatory.

3.	To indicate this test and how the type approval body has to deal with a new Part 7 was re-inserted in Annex 6 of the Regulation 150-01. This text was already existing in Regulation 150-00.

4.	The testing of day-time colour is specified in chapter 5.2.6.3, but the specification of which class the specification of luminance factor requirement will apply got lost. The exception of the requirement for the luminance factor was amended. It was recognized, that the day-time colour specifications for retroreflective devices of class 5 and class F, as well for rear-marking plates was missing. A table 8 had been re-inserted and the numbering of the following tables hat been corrected.

5.	The communication form had been corrected, with the same text as in rRegulations 148 and 149.

6.	Following up the corrections related to the class IVA reflectors, the Part 7 was already existing in the series 00 of the Regulation 150.

*	In accordance with the programme of work of the Inland Transport Committee for 2018–2019 (ECE/TRANS/274, para. 123 and ECE/TRANS/2018/21/Add.1, cluster 3.1), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

1	Despite the importance of tests to check the colour-fastness of retro-reflective devices, it is in the present state of the art not yet possible to assess colour-fastness by laboratory tests of limited duration.