Proposal for supplement to the 12, 13, 14 and 15 series of amendments  to UN Regulation No. 13 
(Heavy Vehicle Braking)

Proposed amendments to the regulation are indicated in bold for new characters, and strikethrough for deleted characters.

- Proposal

Paragraph 5.2.1.31., amend to read:

“5.2.1.31.	When a vehicle is equipped with the means to indicate emergency braking, activation and de-activation of the emergency braking signal shall only be generated by the application of the service braking system regardless of its means of activation (e.g. by an auxiliary control as specified in paragraph 5.2.1.2.4.) when the following conditions are fulfilled:

- Justification

1.	This proposal aims to align UN R13 on UN R13H following the adoption of document GRVA/2025/45/rev.1 for the UN R13H at the 23rd session of GRVA in September 2025. The justifications below taken over from the document GRVA/205/45rev1.

2.	The parking braking system shall be designed, according to the provisions of paragraph 5.2.1.2.4., so that it can be actuated when the vehicle is in motion. This requirement may be fulfilled by the actuation of the vehicle's service braking system, even partially, by means of an auxiliary control, which could be an Electronic Parking Brake (EPB) control in case of an electronic parking brake system. Therefore, the application of the service braking system via the EPB control is common practice.

3.	The proposal aims to clarify the existing text in order to ensure an aligned and consistent interpretation of the activation conditions for an Emergency Stop Signal (ESS), means the ESS can be generated by application of the service braking system via the EPB control.

___________