|  | United Nations | ECE/TRANS/WP.29/GRSG/2025/39 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 18 July 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on General Safety Provisions

130th session

Geneva, 6–9 October 2025

Item 14(b) of the provisional agenda

Exchange of views on vehicle automation:

Categorization of automated vehicles and autonomous vehicle regulation screening

Proposal for supplement 6 to the original version of UN Regulation No. 158 (Devices for Means of Rear Visibility or Detection)

Submitted by the expert from the European Commission *

The text reproduced below was prepared by the expert from the European Commission on behalf of the Task Force on Automated Vehicles Regulatory Screening 
(TF-AVRS). The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

Proposal

Renumber the first paragraph of the introduction to 0.1:

"0.1.	The purpose of this Regulation is to provide the provisions for means of rear visibility and detection that would improve the driver’s awareness of vulnerable road users behind vehicles when reversing. UN Regulation No. 46. provides the provisions for indirect vision of motor vehicles. This Regulation expands driver’s awareness behind vehicles when reversing. Therefore, some requirements of this Regulation may be satisfied by devices complying with UN Regulation No.46.

This Regulation cannot cover all the traffic conditions and infrastructure features in the type approval process; this Regulation recognises that the performances required in this Regulation cannot be achieved in all conditions (vehicle speed and condition, weather conditions, and traffic scenarios, etc. may affect the system performances). "

Insert a new paragraph 0.2., 0.2.1. to 0.2.3., to read:

"0.2.	For supplement 6 to the original version of the Regulation:

0.2.1.	The Regulation is amended to account for vehicles of categories X and Y.

0.2.2.	In preparing for the introduction of Automated Driving Systems (ADS), GRSG recognises that responsibility for rearward visibility will sit with ADS, if fitted and when an ADS feature is activated. As such, supplement 5 to this regulation exempts vehicles of categories X and Y from the requirements, and limits the requirements for other vehicles equipped with an ADS and when an ADS feature is active.

0.2.3.	In the case of vehicles equipped with an ADS other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply."

Paragraph 1.4., amend to read:

"1.4.	The following vehicles of category M and N1 shall be exempted from this Regulation:

(a) Vehicles where installation of means of rear visibility or detection is incompatible with their on-road use may be partly or fully exempted from this Regulation, subject to the decision of the Type Approval Authority;

(b) Vehicles of categories X and Y.1 "

Insert new footnote 1, to read:

"1 		As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 5.4.1., footnote 1, renumber to footnote 2 and amend to read:

"12 	The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.78 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 12.4., amend to read

"12.4. 	"Vehicles of categories M1, M2, M3, N1, N2 and N3" means those defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3), (document ECE/TRANS/WP.29/78/Rev.78)."

Insert a new paragraph 15.1.3., to read:

"15.1.3.	The requirements in paragraph 15 shall not apply while an ADS feature is active."

Paragraph 15.2.2., amend to read:

"15.2.2. 	The close-proximity rear-view field of vision shall be established using ambinocular vision, the eyes being at the "driver's ocular points" as defined in paragraph 12.1. above. The fields of vision shall be determined when the vehicle is in running order as defined in the consolidated Resolution on the Construction of vehicles (R.E.3) (ECE/TRANS/WP.29/78/Rev. 78, para. 2.2.5.4.), plus for M1 and N1 vehicles one front seat passenger (75 kg). When established through windows, the glazing shall have a total light transmission factor in accordance with UN Regulation No. 43, Annex 24"

Annex 9, paragraph 1.3.3.2., amend to read:

"1.3.3.2. 	Vehicle load.

The vehicle is in running order as defined in the consolidated Resolution on the Construction of vehicles (R.E.3) (ECE/TRANS/WP.29/78/Rev.78, paragraph 2.2.5.4.), plus for M1 and N1 vehicles one front seat passenger (75 kg).

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.