Proposal for amendments 
to document ECE/TRANS/WP.29/GRPE/2025/22

Proposal for the 02 series of amendments to UN Regulation No. 40 (Emission of gaseous pollutants by motorcycles)

This document aims to propose a new series of amendments to UN Regulation No. 40 to align with the provision of Euro 3 emission standard.

The modifications to the working document ECE/TRANS/WP.29/GRPE/2025/22 are marked in bold red for new or strikethrough for deleted characters.

I. Proposal

Figure 2, amend to read:

Figure 2

Flowchart for the number of Test Type I

Any of criteria emissions > Limit

All criteria in the table below Table 6 below within the "first test" row are fulfilled.

No

Any of criteria emissions > Limit

Yes

No

Yes

Third test

Any of criteria emissions > Limit

Yes

Rejected

No

Declared value or mean of three accepted, depending on judgment result of each value

All declared values and emissions accepted

No

No

All criteria in the table below Table 6 below within the "second test" row are fulfilled.

Second test

First test

Yes

Yes

Figure 3, amend to read:

Figure 3

NO

YES

NO

NO

NO

YES

YES

YES

Select three vehicles

Test two vehicles

Series accepted

Test of third vehicle

Computation of the test statistic

According to appropriate Appendix does the test statistic agrees with the criteria for failing falling the series for at least one pollutant?

Series rejected

According to appropriate Appendix does the test statistic agrees with the criteria for passing the series for at least one pollutant?

A pass decision is reached for one or more pollutants

Is a pass decision is reached for all the pollutants?

Series accepted

Test of an additional vehicle

Do the average emissions comply with the requirement?

Checking the conformity of the vehicle

Appendix 2, paragraph 6., amend to read:

6.	Remarks

The following recursive formulae are useful for computing successive values of the test statistic:

dn=1-1ndn-1+ 1ndn

Vn2= 1-1nVn-12+ dn- dn2n-1

(n=2, 3, …; d1= d1;   V1=0)

Table A4/1, amend to read:

Table A4/1

Applicable parts of WMTC as specified in Appendix 11 to Annex 7

| Vehicle Sub classification | Applicable Parts of WMTC as specified in Appendix 11 to Annex 7 |
| --- | --- |
| Class 1 | part 1, reduced vehicle speed in cold condition, followed by part 1, reduced vehicle speed in warm condition |
| Class 2 subdivided in: |  |
| Sub-class 2-1 | part 1, reduced vehicle speed in cold condition, followed by part 2, reduced vehicle speed in warm condition |
| Sub-class 2-2 | part 1, in cold condition, followed by part 2, in warm condition |
| Class 3 subdivided in: |  |
| Sub-class 3-1 | part 1, in cold condition, followed by part 2, in warm condition, followed by part 3, reduced vehicle speed in warm condition |
| Sub-class 3-2 | part 1, in cold condition, followed by part 2, in warm condition, followed by part 3, in warm condition |

Annex 4, clause 3.Test Conditions, paragraph 3.4.5.3.32., amend to read:

3.4.5.3.2.	Optional provisions

The gear choice may be modified according to the following provisions:

The use of gears lower than those determined by the requirements described in paragraph 3.4.5.23.1. is permitted in any cycle phase. Manufacturers’ recommendations for gear use shall be followed if they do not result in gears higher than determined by the requirements of paragraph 3.4.5.23.1.

Annex 4, clause 4. Test procedures, paragraph 4.2.2.3.2.3., amend to read:

4.2.2.3.2.3.	The set running resistance force FE(vj) at the specified vehicle speed on the chassis dynamometer is calculated using the following equation:

FEvj= 13.6 mi 2∆v∆tE						(29)

Annex 7- Appendix 1, amend to read:

## Annex 7- Appendix 1

### Symbols & Abbreviations

Table A7.App 1/1

Symbols used

| Symbol | Definition | Unit |
| --- | --- | --- |
| a | Coefficient of polygonal function | - |
| aT | Rolling resistance force of front wheel | N |
| A | NG / biomethane quantity within the H2NG mixture | per cent vol. |
| b | Coefficient of polygonal function | - |
| bT | Coefficient of aerodynamic function | N/(km/h)2 |
| c | Coefficient of polygonal function | - |
| CCO | Concentration of carbon monoxide | ppm |
| CCO2 | Concentration of CO2 in the diluted exhaust gas contained in the sampling bag | percent vol. |
| CCOcorr | Corrected concentration of carbon monoxide | percent vol. |
| CO2c | Carbon dioxide concentration of diluted gas, corrected to take account of diluent air | percent |
| CO2d | Carbon dioxide concentration in the sample of diluent air collected in bag B | percent |
| CO2e | Carbon dioxide concentration in the sample of diluent air collected in bag A | percent |
| CO2m | Mass of carbon dioxide emitted during the test part | mg/km |
| COc | Carbon monoxide concentration of diluted gas, corrected to take account of diluent air | ppm |
| COd | Carbon monoxide concentration in the sample of diluent air, collected in bag B | ppm |
| COe | Carbon monoxide concentration in the sample of diluent air, collected in bag A | ppm |
| COm | Mass of carbon monoxide emitted during the test part | mg/km |
| CH2 | Concentration of hydrogen in the diluted exhaust gas contained in sampling bag | ppm |
| CH2O | Concentration of H2O in the diluted exhaust gas contained in the sampling bag | percent vol. |
| CH2O-DA | Concentration of H2O in the air used for dilution | percent vol. |
| CHC | Concentration of HC in the diluted exhaust gas contained in the sampling bag | ppm (carbon equivalent) |
| d0 | Standard ambient relative air density | - |
| dCO | Density of carbon monoxide | mg/cm3 |
| dCO2 | Density of carbon dioxide | g/m3 |
| dHC | Density of hydrocarbon | mg/cm3 |
| Dav | Average distance between two battery recharges | km |
| De | Electric range of the vehicle | km |
| DiF | Dilution factor | - |
| DOVC | Distance from externally chargeable vehicle | km |
| S / d | Distance driven in a cycle part | km |
| dNOx | Density of nitrogen oxide | mg/m3 |
| dT | Relative air density under test condition | - |
| Dt | Coast-down time | s |
| Dtai | Coast-down time measured in the first road test | s |
| Dtbi | Coast-down time measured in the second road test | s |
| DTE | Coast-down time corrected for the inertia mass | s |
| DtE | Mean coast-down time on the chassis dynamometer at the reference vehicle speed | s |
| DTi | Average coast-down time at specified vehicle speed | s |
| Dti | Coast-down time at corresponding s vehicle speed | s |
| DTj | Average coast-down time at specified vehicle speed | s |
| DTroad | Target coast-down time | s |
| t | Mean coast-down time on the chassis dynamometer without absorption | s |
| Dv | Coast-down vehicle speed interval (2Dv = v1 – v2) | km/h |
| e | Chassis dynamometer setting error | percent |
| F | Running resistance force | N |
| F* | Target running resistance force | N |
| F*(v0) | Target running resistance force at reference vehicle speed on chassis dynamometer | N |
| F*(vi) | Target running resistance force at specified vehicle speed on chassis dynamometer | N |
| f*0 | Corrected rolling resistance in the standard ambient condition | N |
| f*2 | Corrected coefficient of aerodynamic drag in the standard ambient condition | N/(km/h)2 |
| F*j | Target running resistance force at specified vehicle speed | N |
| f0 | Rolling resistance | N |
| f2 | Coefficient of aerodynamic drag | N/(km/h)2 |
| FE | Set running resistance force on the chassis dynamometer | N |
| FE(v0) | Set running resistance force at the reference s vehicle speed on the chassis dynamometer | N |
| FE(v2) | Set running resistance force at the specified vehicle speed on the chassis dynamometer | N |
| Ff | Total friction loss | N |
| Ff(v0) | Total friction loss at the reference vehicle speed | N |
| Fj | Running resistance force | N |
| Fj(v0) | Running resistance force at the reference vehicle speed | N |
| Fpau | Braking force of the power absorbing unit | N |
| Fpau(v0) | Braking force of the power absorbing unit at the reference vehicle speed | N |
| Fpau(vj) | Braking force of the power absorbing unit at the specified vehicle speed | N |
| FT | Running resistance force obtained from the running resistance table | N |
| H | Absolute humidity | g of water / kg of dry air |
| HCc | Concentration of diluted gases expressed in the carbon equivalent, corrected to take account of diluent air | ppm |
| HCd | Concentration of hydrocarbons expressed in the carbon equivalent, in the sample of diluent air collected in bag B | ppm |
| HCe | Concentration of hydrocarbons expressed in the carbon equivalent, in the sample of diluent air collected in bag A | ppm |
| HCm | Mass of hydrocarbon emitted during the test part | mg/km |
| i | gear number | - |
| K0 | Temperature correction factor for rolling resistance | - |
| Kh | Humidity correction factor | - |
| L | Certification limit values of gaseous pollutant emission | mg/km |
| m | Test vehicle mass | kg |
| ma | Actual mass of the test vehicle | kg |
| mcorr | PM mass corrected for buoyancy | mg |
| mf i | Flywheel equivalent inertia mass | kg |
| mi | Equivalent inertia mass | kg |
| mmix | molar mass of air in balance environment (28.836 gmol-1) | gmol-1 |
| mr | Equivalent inertia mass of all the wheels | kg |
| mri | Equivalent inertia mass of all the rear wheel and vehicle parts rotating with wheel | kg |
| mk | mk is unladen mass of the vehicle | kg |
| mref | mref is reference mass of the vehicle | kg |
| mrid | Rider mass | kg |
| muncorr | PM mass uncorrected for buoyancy | mg |
| Mi | Mass emission of the pollutant i in mg/km | mg |
| M2i | Average mass emission of the pollutant i with an electrical energy/power storage device in minimum state of charge (maximum discharge of capacity) | mg/km |
| M1i | Average mass emission of the pollutant i with a fully charged electrical energy/power storage device | mg/km |
| Mp | Particulate mass emission | mg/km |
| n | Engine speed | min-1 |
| n | Number of data regarding the emission or the test | - |
| N | Number of revolutions made by pump P | - |
| ndvi | Ratio between engine speed in min-1 and vehicle speed in km/h in gear "i" | - |
| ng | Number of forward gears | - |
| nidle | Idling engine speed | min-1 |
| n_max_acc(1) | Upshift engine speed from gear 1 to gear 2 during acceleration phases | min-1 |
| n_max_acc(i) | Up shift engine speed from gear i to gear i+1 during acceleration phases, i>1 | min-1 |
| n_min_acc(i) | Minimum engine speed for cruising or deceleration in gear 1 | min-1 |
| NOxc | Nitrogen oxide concentration of diluted gases, corrected to take account of diluent air | ppm |
| NOxd | Nitrogen oxide concentration in the sample of diluent air collected in bag B | ppm |
| NOxe | Nitrogen oxide concentration in the sample of diluent air collected in bag A | ppm |
| NOxm | Mass of nitrogen oxides emitted during the test part | mg/km |
| p0 | Standard ambient pressure | kPa |
| pa | Ambient/atmospheric pressure | kPa |
| pabs | absolute pressure in balance environment | kPa |
| pd | Saturated pressure of water at the test temperature | kPa |
| pi | Average under-pressure during the test part in the section of pump P | kPa |
| pT | Mean ambient pressure during the test | kPa |
| Pn | Rated power | kW |
| Q | Electric energy balance | Ah |
| ρ0 | Standard relative ambient air volumetric mass | mg/cm3 |
| ρair | density of air in balance environment | mg/cm3 |
| ρweight | density of calibration weight used to span balance | mg/cm3 |
| ρmedia | density of PM sample medium (filter) with filter medium Teflon coated glass fibre (e.g. TX40): ρmedia = 2 300 kg/m3 | mg/cm3 |
| r(i) | Gear ratio in gear i | - |
| R | molar gas constant (8 314 Jmol-1K-1) | Jmol-1K-1 |
| Rf | Response factor to calibrate HC analyser | - |
| RF | Final test result of pollutant emissions, carbon dioxide emission or fuel consumption | mg/km g/km, 1/100km |
| R1 | Test results of pollutant emissions, carbon dioxide emission or fuel consumption for cycle part 1 with cold start | mg/km g/km, 1/100km |
| R2 | Test results of pollutant emissions, carbon dioxide emission or fuel consumption for cycle part 2 with warm condition | mg/km g/km, 1/100km |
| R3 | Test results of pollutant emissions, carbon dioxide emission or fuel consumption for cycle part 1 with warm condition | mg/km g/km, 1/100km |
| Ri1 | First Test Type I results of pollutant emissions | mg/km |
| Ri2 | Second Test Type I results of pollutant emissions | mg/km |
| Ri3 | Third Test Type I results of pollutant emissions | mg/km |
| RS | Reduced speed | - |
| s | Rated engine speed | min-1 |
| S | Accumulated distance in test cycle (paragraph 5.1.1.3. of Annex 1) | km |
| Tamb | absolute ambient temperature of balance environment | °C |
| TC | Temperature of the coolant | °C |
| TO | Temperature of the engine oil | °C |
| TP | Temperature of the spark-plug seat/gasket | °C |
| T0 | Standard ambient temperature | °C |
| Tp | Temperature of the diluted gases during the test part, measured in the intake section of pump P | °C |
| TT | Mean ambient temperature during the test | °C |
| U | Relative humidity | percent |
| v | Specified vehicle speed | km/h |
| V | Total volume of diluted gas | m3 |
| vmax | Maximum design vehicle speed of test vehicle | km/h |
| v0 | Reference vehicle speed | km/h |
| V0 | Volume of gas displaced by pump P during one revolution | m3/rev. |
| v1 | Vehicle speed at which the measurement of the coast-down time begins | km/h |
| v2 | Vehicle speed at which the measurement of the coast-down time ends | km/h |
| vi | Specified vehicle speed selected for the coast-down time measurement | km/h |
| w1 | Weighting factor of cycle part 1 with cold start | - |
| w1warm | Weighting factor of cycle part 1 with warm condition | - |
| w2 | Weighting factor of cycle part 2 with warm condition | - |
| w3 | Weighting factor of cycle part 3 with warm condition | - |

Annex 7- Appendix 5b, paragraph 3.1.2.8, amend to read:

3.1.2.8.	The running resistance determined on the track shall be corrected to the reference ambient conditions as follows:

Equation A7.App5b/5:

Fcorrected = k · Fmeasured

Equation A7.App5b/6:

Where:

RR is the rolling resistance at vehicle speed v (N);

RAERO is the aerodynamic drag at vehicle speed v (N);

RT is the total road load = RR+RAERO (N);

KR is the temperature correction factor of rolling resistance, taken to be equal to: 3.6 · 10-3/K;

t is the road test ambient temperature in K;

t0 is the reference ambient temperature (293.215 K);

dt is the air density at the test conditions (kg/m3);

d0 is the air density at the reference conditions (293.215 K, 101.3 kPa) = 1.189 kg/m3

The ratios RR/RT and RAERO/RT shall be specified by the vehicle manufacturer on the basis of the data normally available to the company and to the satisfaction of the technical service. If these values are not available or if the technical service or responsible authority do not accept these values, the following figures for the rolling/total resistance ratio given by the following formula may be used:

Equation A7.App5b/7:

where:

is the test mass and for each vehicle speed the coefficients a and b are as shown in the following table:

Annex 7 – Appendix 11, paragraph 1.2., amend to read:

1.2. WMTC, cycle part 1

Figure A7.App11/2

WMTC, cycle part 1

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/3, amend to read:

Table A7.App11/3

WMTC., cycle part 1., reduced vehicle speed for vehicle classes 1 and 2-1. 361 to 540 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/4, amend to read:

Table A7.App11/4

WMTC., cycle part 1., reduced vehicle speed for vehicle classes 1 and 2-1. 541 to 600 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/5, amend to read:

Table A7.App11/5

WMTC., cycle part 1, for vehicle classes 2-2 and 3. 0 to 180 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/6, amend to read:

Table A7.App11/6

WMTC., cycle part 1, for vehicle classes 2-2 and 3. 181 to 360 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/7, amend to read:

Table A7.App11/7

WMTC., cycle part 1, for vehicle classes 2-2 and 3. 361 to 540 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/8, amend to read:

Table A7.App11/8

WMTC., cycle part 1, for vehicle classes 2-2 and 3. 541 to 600 s

[Table unchanged]

Annex 7 – Appendix 11, paragraph 1.3., amend to read:

1.3. WMTC., cycle part 2
Figure A7.App11/3

[Figure unchanged]

Annex 7 – Appendix 11, paragraph 1.3.1., amend to read:

1.3.1. The characteristic desired vehicle speed versus test time of WMTC., cycle part 2 is set out in the following tables.

Annex 7 – Appendix 11, Table A7.App11/9, amend to read:

Table A7.App11/9

WMTC., cycle part 2., reduced vehicle speed for vehicle class 2-1. 0 to 180 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/10, amend to read:

Table A7.App11/10

WMTC., cycle part 2., reduced vehicle speed for vehicle class 2-1. 181 to 360 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/11, amend to read:

Table A7.App11/11

WMTC., cycle part 2., reduced vehicle speed for vehicle class 2-1. 361 to 540 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/12, amend to read:

Table A7.App11/12

WMTC., cycle part 2., reduced vehicle speed for vehicle class 2-1. 541 to 600 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/13, amend to read:

Table A7.App11/13

WMTC., cycle part 2, for vehicle classes 2-2 and 3. 0 to 180 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/14, amend to read:

Table A7.App11/14

WMTC., cycle part 2, for vehicle classes 2-2 and 3. 181 to 360 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/15, amend to read:

Table A7.App11/15

WMTC., cycle part 2, for vehicle classes 2-2 and 3. 361 to 540 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/16, amend to read:

Table A7.App11/16

WMTC., cycle part 2, for vehicle classes 2-2 and 3. 541 to 600 s

[Table unchanged]

Annex 7 – Appendix 11, paragraph 1.4., amend to read:

1.4. WMTC., cycle part 3

Figure A7.App11/4

WMTC., cycle part 3.

[Figure unchanged]

Annex 7 – Appendix 11, paragraph 1.4.1., amend to read:

1.4.1. The characteristic desired vehicle speed versus test time of WMTC., cycle part 3 is set out in the following tables.

Annex 7 – Appendix 11, Table A7.App11/17, amend to read:

Table A7.App11/17

WMTC., cycle part 3., reduced vehicle speed for vehicle class 3-1. 0 to 180 s

| time in s | roller speed in km/h | phase indicators |  |  |  | time in s | roller speed in km/h | phase indicators |  |  |  | time in s | roller speed in km/h | phase indicators |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | stop | acc | cruise | dec |  |  | stop | acc | cruise | dec |  |  | stop | acc | cruise | dec |
| 0 | 0.0 | X |  |  |  | 33 | 68.1 |  | X |  |  | 66 | 80.4 |  | X |  |  |
| 1 | 0.0 | X |  |  |  | 34 | 69.1 |  | X |  |  | 67 | 81.7 |  | X |  |  |
| 2 | 0.0 | X |  |  |  | 35 | 69.5 |  | X |  |  | 68 | 82.6 |  | X |  |  |
| 3 | 0.0 | X |  |  |  | 36 | 69.9 |  | X |  |  | 69 | 83.5 |  | X |  |  |
| 4 | 0.0 | X |  |  |  | 37 | 70.6 |  | X |  |  | 70 | 84.4 |  | X |  |  |
| 5 | 0.0 | X |  |  |  | 38 | 71.3 |  | X |  |  | 71 | 85.1 |  | X |  |  |
| 6 | 0.0 | X |  |  |  | 39 | 72.2 |  | X |  |  | 72 | 85.7 |  | X |  |  |
| 7 | 0.0 | X |  |  |  | 40 | 72.8 |  | X |  |  | 73 | 86.3 |  | X |  |  |
| 8 | 0.9 |  | X |  |  | 41 | 73.2 |  | X |  |  | 74 | 87.0 |  | X |  |  |
| 9 | 3.2 |  | X |  |  | 42 | 73.4 |  | X |  |  | 75 | 87.9 |  | X |  |  |
| 10 | 7.3 |  | X |  |  | 43 | 73.8 |  | X |  |  | 76 | 88.8 |  | X |  |  |
| 11 | 12.4 |  | X |  |  | 44 | 74.8 |  | X |  |  | 77 | 89.7 |  | X |  |  |
| 12 | 17.9 |  | X |  |  | 45 | 76.7 |  | X |  |  | 78 | 90.3 |  |  | X |  |
| 13 | 23.5 |  | X |  |  | 46 | 79.1 |  | X |  |  | 79 | 90.6 |  |  | X |  |
| 14 | 29.1 |  | X |  |  | 47 | 81.1 |  | X |  |  | 80 | 90.6 |  |  | X |  |
| 15 | 34.3 |  | X |  |  | 48 | 82.1 |  |  |  | X | 81 | 90.5 |  |  | X |  |
| 16 | 38.6 |  | X |  |  | 49 | 81.7 |  |  |  | X | 82 | 90.4 |  |  | X |  |
| 17 | 41.6 |  | X |  |  | 50 | 80.3 |  |  |  | X | 83 | 90.1 |  |  | X |  |
| 18 | 43.9 |  | X |  |  | 51 | 78.8 |  |  |  | X | 84 | 89.7 |  |  | X |  |
| 19 | 45.9 |  | X |  |  | 52 | 77.3 |  |  |  | X | 85 | 89.3 |  |  | X |  |
| 20 | 48.1 |  | X |  |  | 53 | 75.9 |  |  |  | X | 86 | 89.0 |  |  | X |  |
| 21 | 50.3 |  | X |  |  | 54 | 75.0 |  |  |  | X | 87 | 88.8 |  |  | X |  |
| 22 | 52.6 |  | X |  |  | 55 | 74.7 |  |  |  | X | 88 | 88.9 |  |  | X |  |
| 23 | 54.8 |  | X |  |  | 56 | 74.7 |  |  |  | X | 89 | 89.1 |  |  | X |  |
| 24 | 55.8 |  | X |  |  | 57 | 74.7 |  |  |  | X | 90 | 89.3 |  |  | X |  |
| 25 | 55.2 |  | X |  |  | 58 | 74.6 |  |  |  | X | 91 | 89.4 |  |  | X |  |
| 26 | 53.9 |  | X |  |  | 59 | 74.4 |  |  |  | X | 92 | 89.4 |  |  | X |  |
| 27 | 52.7 |  | X |  |  | 60 | 74.1 |  |  |  | X | 93 | 89.2 |  |  | X |  |
| 28 | 52.8 |  | X |  |  | 61 | 73.9 |  |  |  | X | 94 | 88.9 |  |  | X |  |
| 29 | 55.0 |  | X |  |  | 62 | 74.1 |  | X |  |  | 95 | 88.5 |  |  | X |  |
| 30 | 58.5 |  | X |  |  | 63 | 75.1 |  | X |  |  | 96 | 88.0 |  |  | X |  |
| 31 | 62.3 |  | X |  |  | 64 | 76.8 |  | X |  |  | 97 | 87.5 |  |  | X |  |
| 32 | 65.7 |  | X |  |  | 65 | 78.7 |  | X |  |  | 98 | 87.2 |  |  | X |  |

| time in s | roller speed in km/h | phase indicators |  |  |  | time in s | roller speed in km/h | phase indicators |  |  |  | time in s | roller speed in km/h | phase indicators |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | stop | acc | cruise | dec |  |  | stop | acc | cruise | dec |  |  | stop | acc | cruise | dec |
| 99 | 87.1 |  |  | X |  | 126 | 50.3 |  | X |  |  | 154 | 94.6 |  | X |  |  |
| 100 | 87.2 |  |  | X |  | 127 | 50.6 |  | X |  |  | 155 | 96.0 |  | X |  |  |
| 101 | 87.3 |  |  | X |  | 128 | 51.2 |  | X |  |  | 156 | 97.5 |  | X |  |  |
| 102 | 87.4 |  |  | X |  | 129 | 51.8 |  | X |  |  | 157 | 99.0 |  | X |  |  |
| 103 | 87.5 |  |  | X |  | 130 | 52.5 |  | X |  |  | 158 | 99.8 |  |  |  | X |
| 104 | 87.4 |  |  | X |  | 131 | 53.4 |  | X |  |  | 159 | 99.0 |  |  |  | X |
| 105 | 87.1 |  |  | X |  | 132 | 54.9 |  | X |  |  | 160 | 96.7 |  |  |  | X |
| 106 | 86.8 |  |  | X |  | 133 | 57.0 |  | X |  |  | 161 | 93.7 |  |  |  | X |
| 107 | 86.4 |  |  | X |  | 134 | 59.4 |  | X |  |  | 162 | 91.3 |  |  |  | X |
| 108 | 85.9 |  |  | X |  | 135 | 61.9 |  | X |  |  | 163 | 90.4 |  |  |  | X |
| 109 | 85.2 |  |  |  | X | 136 | 64.3 |  | X |  |  | 164 | 90.6 |  |  |  | X |
| 110 | 84.0 |  |  |  | X | 137 | 66.4 |  | X |  |  | 165 | 91.1 |  |  |  | X |
| 111 | 82.2 |  |  |  | X | 138 | 68.1 |  | X |  |  | 166 | 90.9 |  |  |  | X |
| 112 | 80.3 |  |  |  | X | 139 | 69.6 |  | X |  |  | 167 | 89.0 |  |  |  | X |
| 113 | 78.6 |  |  |  | X | 140 | 70.7 |  | X |  |  | 168 | 85.6 |  |  |  | X |
| 114 | 77.2 |  |  |  | X | 141 | 71.4 |  | X |  |  | 169 | 81.6 |  |  |  | X |
| 115 | 75.9 |  |  |  | X | 142 | 71.8 |  | X |  |  | 170 | 77.6 |  |  |  | X |
| 116 | 73.8 |  |  |  | X | 143 | 72.8 |  | X |  |  | 171 | 73.6 |  |  |  | X |
| 117 | 70.4 |  |  |  | X | 144 | 75.0 |  | X |  |  | 172 | 69.7 |  |  |  | X |
| 118 | 65.7 |  |  |  | X | 145 | 77.8 |  | X |  |  | 173 | 66.0 |  |  |  | X |
| 119 | 60.5 |  |  |  | X | 146 | 80.7 |  | X |  |  | 174 | 62.7 |  |  |  | X |
| 120 | 55.9 |  |  |  | X | 147 | 83.3 |  | X |  |  | 175 | 60.0 |  |  |  | X |
|  |  |  |  |  |  | 148 | 785.4 |  | X |  |  | 176 | 58.0 |  |  |  | X |
| 121 | 53.0 |  |  |  | X | 149 | 87.3 |  | X |  |  | 177 | 56.4 |  |  |  | X |
| 122 | 51.6 |  |  |  | X | 150 | 89.1 |  | X |  |  | 178 | 54.8 |  |  |  | X |
| 123 | 50.9 |  |  |  | X | 151 | 90.6 |  | X |  |  | 179 | 53.3 |  |  |  | X |
| 124 | 50.5 |  |  |  | X | 152 | 91.9 |  | X |  |  | 180 | 51.7 |  |  |  | X |
| 125 | 50.2 |  |  |  | X | 153 | 93.2 |  | X |  |  |  |  |  |  |  |  |

Annex 7 – Appendix 11, Table A7.App11/18, amend to read:

Table A7.App11/18

WMTC., cycle part 3., reduced vehicle speed for vehicle class 3-1. 181 to 360 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/19, amend to read:

Table A7.App11/19

WMTC., cycle part 3., reduced vehicle speed for vehicle class 3-1. 361 to 540 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/20, amend to read:

Table A7.App11/20

WMTC., cycle part 3., reduced vehicle speed for vehicle class 3-1. 541 to 600 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/21, amend to read:

Table A7.App11/21

WMTC., cycle part, 3 for vehicle class 3-2. 0 to 180 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/22, amend to read:

Table A7.App11/22

WMTC., cycle part 3, for vehicle class 3-2. 181 to 360 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/23, amend to read:

Table A7.App11/23

WMTC., cycle part 3, for vehicle class 3-2. 361 to 540 s

[Table unchanged]

Annex 7 – Appendix 11, Table A7.App11/24, amend to read:

Table A7.App11/24

WMTC., cycle part 3, for vehicle class 3-2. 541 to 600 s

[Table unchanged]

Annex 7 – Appendix 12, clause 1., amend to read:

1. Introduction

This explanatory note explains matters specified or described in this Regulation,. including its Annexes or Appendices,. and matters related thereto with regard to the gearshift procedure.

Annex 7 – Appendix 12, paragraph 2.1., amend to read:

2.1. The development of the gearshift procedure was based on an analysis of the gearshift points in the in-use data. In order to establish generalised correlations between technical specifications of the vehicles and desired vehicle speeds to shift gears,. the engine speeds were normalised to the utilisable band between rated engine speed and idling engine speed.

Annex 7 – Appendix 12, paragraph 2.2., amend to read:

2.2. In a second step,. the end speeds (vehicle speed as well as normalised engine speed) for upshifts and downshifts were determined and recorded in a separate table. The averages of these speeds for each gear and vehicle were calculated and correlated with the vehicles’ technical specifications.

Annex 7 – Appendix 12, paragraph 2.4., amend to read:

2.4. In order to find a balanced compromise between the three regions,. a new approximation function for normalised upshift engine speeds versus power-to-mass ratio was calculated as a weighted average of the EU/USA curve (with 2/3 weighting) and the Japanese curve (with 1/3 weighting),. resulting in the following equations for normalised upshift engine speeds:

Equation (1): Normalised upshift engine speed in 1st gear (gear 1)

(1)

Equation (2): Normalised upshift engine speed in gears > 1

(2)

Annex 7 – Appendix 12, paragraph 3.1., amend to read:

3.1. Figure A7.App12/1 shows an example of gearshift use for a small vehicle:

(a) the lines in bold show the gear use for acceleration phases;

(b) the dotted lines show the downshift points for deceleration phases;

(c) in the cruising phases,. the whole engine speed range between downshift engine speed and upshift engine speed may be used.

Annex 7 – Appendix 12, paragraph 3.2., amend to read:

3.2. Where vehicle speed increases gradually during cruise phases,. upshift engine vehicle speeds (v1→2., v2→3 and vi→i+1) in km/h may be calculated using the following equations:

v1→2=0.03×s-nidle+nidle×1ndv2

(3)

v1→22→3=0.5753×e-1.9×Pnmref-0.1×s-nidle+nidle×1ndv1

(4)

vi→i+1=0.5753×e-1.9×Pnmref×s-nidle+nidle×1ndvi-1, i=3 to ng

(5)

Annex 7 – Appendix 12, paragraph 3.3., amend to read:

3.3. In order to allow the technical service more flexibility and to ensure driveability,. the gearshift regression functions should be considered as lower limits. Higher engine speeds are permitted in any cycle phase.

Annex 7 – Appendix 12, paragraph 4.1., amend to read:

4.1.	In order to avoid different interpretations in the application of the gearshift equations and thus to improve the comparability of the test,. fixed-phase indicators are assigned to the vehicle speed pattern of the cycles. The specification of the phase indicators is based on the definition from the Japan Automobile Research Institute (JARI) of the four driving modes as shown in the following table:

II.	Justification

- In April 2024, at the 63rd session of the IWG-EPPR, IMMA promoted the principle of the transposition work of the Regulation UN-GTR No. 2 in incremental steps, starting from the Euro 3 level. The representative of South Africa, Co-Chair of the IWG at the time of the discussions, appreciated the concept and supported the transposition activity in incremental steps, noting that this may offer a basic environmental regulatory instrument to the African Region, where fuel quality challenges persist.

On the other hand, the transposition may contribute to promoting a broader regional adoption of the UNECE WP.29 legal framework, thus fostering technical harmonization and facilitating cross-border trade.

- In Autumn 2024, IMMA shared with the IWG-EPPR the first draft of the 02 series of amendments to UN-Regulation No. 40, that after some revisions by the group, was finally submitted as an informal document to the 92nd session of GRPE (GRPE-92-12), in March 2025. Due to the outdated format and structure of the 01 series of amendments, the submitted informal document could not show the changes between the 01 and the 02 series of amendments.

- The scope of the UN-Regulation No. 40 remained consistent with that of its preceding 01 series of amendments. Following a suggestion from France, the wording of the scope was amended to include the energy efficiency and the readability of the Figure 1 was improved. The technical annexes and appendices were carried over from the UN Regulation GTR No. 2. The Conformity of Production provisions were based on the Regulation EU 44/2014 and the 07 series of amendments to the UN-Regulation No. 83.

- During Spring 2025, after further revisions and minor editorial corrections, the IWG-EPPR, at its 69th session on 30 June, judged the document mature for the formal submission to the 93rd session of GRPE.

- Following the submission of the working document, editorial oversights were corrected in Figures 2 and 3, and Table A4/1. In Annex 4, paragraph 3.4.5.3.2 was updated to correctly reference paragraph 3.4.5.2.1. Further editorial improvements were made in Annex 7 – Appendix 1, with improved wording consistency and punctuation correction in Appendices 11 and 12. Furthermore, the roller speed value at 148 s in Table A7.App11/17 was corrected.

- The recursive formula calculating pass/fail criteria of CoP in Appendix 2 - paragraph 6, was corrected to be aligned with the corresponding formula in UN Regulation No. 83 (as per GRPE-85-09).

- The readability of the formula defining the set running resistance force FE(vj) on the chassis dynamometer was improved without any modification to the formula itself.

- Inconsistencies in the reference ambient temperature were identified, resulting from variations in rounding. Therefore, all the references were aligned to 293.15 K (replacing 293.2 K in Annex 7 – Appendix 5b).

- In the Annex 7- Appendix 12, paragraph 3.2., the subscript of the equation (4) calculating the upshift speed was corrected.