Proposal to amend document GRBP/2025/27 (Proposal for a new UN Regulation on the uniform provisions concerning the approval of tyres with regard to abrasion performance)

Changes proposed by TF TA and included in informal document GRBP-82-29 are marked in bold blue for added text and strike through blue for deleted text.

Changes proposed by TF TA members after GRBP 82 are marked in bold red for added text and strike through red for deleted text.

I.	Proposal

UN Regulation No. [XXX] on uniform provisions concerning the approval of tyres with regard to abrasion performance

Contents

1.	Scope

2.	Definitions

3.	Application for approval

4.	Markings

5.	Approval

6.	Requirements

7.	Modifications of the type of tyre and extension of approval

8.	Conformity of production

9.	Penalties for non-conformity of production

10.	Production definitively discontinued

11.	Names and addresses of Technical Services responsible for conducting approval tests of
Type Approval Authorities

12.	Introductory provisions

Annexes

1	Communication

2	Arrangements of approval marks

3	Procedure for determining the abrasion performance of tyres of class C1

Appendix 1 - Test method (a) - Accelerations calculation

Appendix 2 - Test method (a) - Test report example for the vehicle test method on public open roads

Appendix 3 - Test method (b) - Input of test cycle

Appendix 4 - Test method (b) - Test equipment tolerances

Appendix 5 - Test method (b) - Replacement of sandpaper surface

Appendix 6 - Test method (b) – Test report example for the indoor drum test method

1.		Scope

1.1.	This Regulation applies to new pneumatic tyres* of class C1 with regard to their abrasion performance.

This Regulation applies only to tyres that conform to UN Regulation No. 117.

1.2.	This Regulation does not apply (and approvals to this Regulation shall not be granted) to the following tyres:

1.2.1.	Tyres designed as temporary-use spare tyres and marked "Temporary use only";

1.2.2.	Tyres having a nominal rim diameter code ≤ 13 (or ≤ 330 mm) or ≥ 25 (or ≥ 635 mm);

1.2.3.	Tyres designed for competition;

1.2.4.	Tyres intended to be fitted to road vehicles of categories other than M, N and O;

1.2.5.	Ice grip tyres;

1.2.6.	Tyres fitted with additional devices to improve traction properties (e.g. studded tyres);

1.2.7.	Tyres with a speed category less than 120 km/h (speed speed-category symbol L);

1.2.8.	Tyres designed only to be fitted to vehicles registered for the first time before 1 October 2000;

1.2.9.	Professional off-road tyres;

1.2.10.	[Road legal race tyres;.]

1.2.11. 	[Tyres for use in severe snow conditions with a speed category less than or equal to 160 km/h (speed category symbol Q).]

2.		Definitions

For the purpose of this Regulation, in addition to the definitions contained in UN Regulation No. 30, the following definitions shall apply:

2.1.	"Type of tyre" means tyres which do not differ in such essential characteristics as:

(a)	The manufacturer's name;

(b)	The tyre class (see paragraph 2.5. below);

(c)	The tyre structure;

(d)	The category of use: normal tyre, snow tyre and special use tyre;

[(e)	Whether the tyre is a tyre for use in severe snow conditions or not;]

(f)	The tread pattern (see paragraph 3.2.1. of this Regulation).

2.2.	"Manufacturer" means the person or body who is responsible to the Type Approval Authority (TAA) for all aspects of the type-approval and for ensuring the conformity of production.

2.3.	"Brand name/trademark" means the identification of the brand or trademark as defined by the tyre manufacturer and marked on the sidewall(s) of the tyre. The brand name/trademark may be the same as that of the manufacturer.

2.4.	"Trade description/commercial name" means an identification of a range of tyres as given by the tyre manufacturer. It may coincide with the brand name/trademark.

2.5.	"Tyre class" means one of the following groupings:

2.5.1.	"Class C1 tyres" means tyres conforming to UN Regulation No. 30;

2.6.	"Representative tyre size" means the tyre size which is submitted to the test described in Annex 3 to this Regulation to assess the conformity for the type approval of the type of tyre.

2.7.	"Temporary-use spare tyre" means a tyre different from a tyre intended to be fitted to any vehicle for normal driving conditions; but intended only for temporary use under restricted driving conditions.

2.8.	"Tyre designed for competition" means a tyre intended to be fitted to vehicles involved in motor sport competition and not intended for non-competitive on-road use.

2.9.	["Road legal race tyre" means a tyre approved for use on public roads but primarily designed for track use and/or competition and fulfilling the requirements of paragraph 6.2. of this Regulation.]

2.10.	"Normal tyre" means a tyre intended for normal on-road use.

2.11.	"Snow tyre" means a tyre whose tread pattern, tread compound or construction is primarily designed to achieve in mud and/or snow conditions a performance better than that of a normal tyre with regard to its ability to initiate and control vehicle motion.

2.11.1.	"Tyre for use in severe snow conditions" means a snow tyre or a special use tyre whose tread pattern, tread compound or construction is specifically designed to be used in severe snow conditions and that conforms to UN Regulation No. 117.

2.11.1.1.	"Ice grip tyre" means a class C1 snow tyre that is classified as a tyre for use in severe snow conditions, additionally designed to be used on road surfaces covered with ice and conforming to UN Regulation No. 117.

2.12.	"Special use tyre" means a tyre intended for mixed use both on- and off-road or for other special duty. These tyres are primarily designed to initiate and maintain the vehicle in motion in off-road conditions.

2.12.1.	"Professional off-road tyre" means a special use tyre primarily used for service in severe off-road conditions.

2.13.	"Reinforced tyre" or "extra load tyre" of class C1 means a tyre designed to carry more load at a higher inflation pressure than the load carried by the corresponding standard version tyre at the standard inflation pressure as specified in ISO 4000-1:2024.

2.14.	"Standard Reference Test Tyre" or "SRTT" means a tyre that is produced, controlled and stored in accordance with the standards of ASTM International:

(a)	F3676 – 24 for the size 225/45R17 94V and referred to as "SRTT17S";

(b)	F3675 – 24 for the size 225/45R17 94H and referred to as "SRTT17W".

2.15.	"Candidate tyre" means a tyre, representative of the type that is submitted for approval in accordance with this Regulation and whose performance is evaluated relative to that of a reference tyre.

2.16.	"Reference tyre" means a Standard Reference Test Tyre that is used as a reference in an evaluation programme.

2.17.	"Test tyre" means a candidate tyre or a reference tyre.

2.18.	"Test cycle" means a driving programme of a test tyre on the drum defined by the application of a series of longitudinal and lateral forces.

2.19.	"Abrasion rate" means the mass of material lost from the tyre due to the abrasion process per distance travelled, and expressed in mg/km.

2.20.	"Abrasion level" means the abrasion rate normalized to the load on the tyre, and expressed in mg/(km∙t).

2.21.	"Abrasion index" (AICT) means the dimensionless value for expressing the tyre abrasion level of a candidate tyre relative to that of the applicable Standardized Reference Test Tyre (SRTT).

2.22.	"Approval reference number" means a six- to eight-digit number concatenating:

- the two digits (with a leading zero where applicable, "00" indicating that the approval was granted in accordance with original version of a regulation) that indicate the series of amendments applied to the approval pursuant to Schedule 4, paragraph 3, Section 2(a) of the 1958 Agreement and

- the four to six-digit sequential number (with leading zeros in the case that the sequential number is less than 1000) pursuant to Schedule 4, paragraph 3, Section 3 of the 1958 Agreement, both as contained within the approval number assigned by the Contracting Party pursuant to paragraph 5.2.

both as contained within the approval number assigned by the Contracting Party pursuant to paragraph 5.2.

3.		Application for approval

3.1.	The application for approval of a type of tyre with regard to this Regulation shall be submitted by the tyre manufacturer or by its duly accredited representative. It shall specify:

3.1.1.	Manufacturer’s name and address;

3.1.2.	If applicable, name and address of manufacturer's representative;

3.1.3.	Tyre class (see paragraph 2.5. of this Regulation);

3.1.4.	Category of use (normal, snow, or special use);

[3.1.4.1.	Whether or not the tyre is a tyre for use in severe snow conditions.]

3.1.5.	Tyre structure;

3.1.6.	Brand name(s)/trademark(s), trade description(s)/commercial name(s);

3.1.7.	A list of tyre size designations covered by this application and specifying for each brand name/trademark and/or each trade description/commercial name the applicable tyre size designations and service descriptions [, adding whether "reinforced" (or "extra load") or not, and whether or not the tyre is for use in severe snow conditions].

3.2.	The application for approval shall be accompanied by:

3.2.1.	Details of the major features, with respect to the effects on the abrasion performance of the tyres, including the tread pattern, included in the designated range of tyre sizes. This may be by means of descriptions supplemented by technical data, drawings, photographs or Computer Tomography (CT) scans, and must shall be sufficient to allow the Type Approval Authority or Technical Service to determine whether any subsequent changes to the major features will adversely affect the tyre performance. The effects of changes to minor details of tyre construction on tyre performances will be evident and determined during checks on conformity of production;

3.2.2.	Drawings or photographs of the tyre sidewall, showing the approval marks referred to in paragraph 4., shall be submitted once the production has been established, but no later than one year after the date of granting of type approval;

3.3.	At the request of the Type Approval Authority, the applicant shall submit samples of tyres for test or copies of test reports from the Technical Services, communicated as given in paragraph 11. of this Regulation.

3.4.	With regard to the application, testing may be confined to a representative tyre size of the type of tyre, at the discretion of the Type Approval Authority.

4.		Markings

4.1.	All tyres constituting the type of tyre shall be marked as prescribed by UN Regulations No. 30 and No. 117.

4.2.	In particular, tyres shall bear:

4.2.1.	The manufacturer's name or the brand name/trademark;

4.2.2.	The trade description/commercial name (see paragraph 2.4. of this Regulation). However, the trade description is not required when it coincides with the brand name/trademark;

4.2.3.	The tyre size designation;

4.2.4	The inscription "REINFORCED" (or alternatively "EXTRA LOAD") if the tyre is classified as reinforced;

4.2.5.	The "Alpine Symbol" ("3-peak-mountain with snowflake" conforming to the pictogram described in Annex 7, Appendix 1 to UN Regulation No. 117 from now on referred to as "3PMSF") if the snow tyre or the special use tyre is classified as tyre for use in severe snow conditions;

4.2.6.	The inscription "ET" and/or "POR" if the tyre is classified in the category of use "special". In addition, they may also bear the inscription "M+S" or "M.S" or "M&S".

ET means Extra Tread, and POR means Professional Off-Road.

[4.2.7.	The inscription "[RLR]" if the tyre is classified as a road legal race tyre.]

4.3.	Every tyre conforming to the type of tyre approved under this Regulation shall be marked on at least one of its sidewalls conspicuously with an international approval mark.

4.3.1.	The approval mark shall be situated in the lower area of the tyre on at least one of its sidewalls. However, in the case of tyres identified by the tyre to rim fitment configuration symbol "A" or "U", the marking may be located anywhere on the outside sidewall of the tyre.

4.3.2.	The international approval mark shall consist of the following:

4.3.2.1.	A circle surrounding the letter "E" followed by the distinguishing number4 of the country which has granted approval and

4.3.2.2.	The approval reference number and

4.3.2.3.	One of the suffixes listed below:

| A1 | Stage 1 |
| --- | --- |
| A2 | Stage 2 |

4.3.3.	The approval reference number and the suffix shall be placed close to the circle prescribed in paragraph 4.3.2.1. and with respect to the letter "E" either above or below or to the left or to the right of that circle and shall face in the same direction as the "E".

4.3.4.	If the tyre conforms to type approvals under one or more other Regulations UN regulations annexed to the Agreement in the country which has granted approval under this Regulation, the symbol prescribed in paragraph 4.3.2.1. does not need to be repeated. In such a case the additional numbers and symbols of all the Regulations UN regulations under which approval has been granted in the country which has granted approval under this Regulation shall be placed adjacent to the symbol prescribed in paragraph 4.3.2.1.

4.3.5.	Notwithstanding paragraph 4.3.2., in the case that the approval of a tyre pursuant to this Regulation has been granted by the same Type Approval Authority than as that granting the approval pursuant to UN Regulation No. 30, the approval mark pursuant to UN Regulation No. 30 can be combined with an indication of the applicable series of amendments to which the tyre was approved pursuant to UN Regulation No. [XXX] in the form of 2 digits (for example "00" indicating that the UN Regulation No. [XXX] approval was granted following the original version) and the suffixes "A1" or "A2" using the addition sign "+", as described in Example 1 of Model C in Annex 2 Annex 2 - Appendix 3 to this Regulation, for example "0236378 + 00A1".

4.3.6.	Notwithstanding paragraph 4.3.2., in the case that the approval of a tyre pursuant to this Regulation has been granted by the same Type Approval Authority than as that granting the approval pursuant to UN Regulation No. 117, the approval mark pursuant to UN Regulation No. 117 can be combined with an indication of the applicable series of amendments to which the tyre was approved pursuant to UN Regulation No. [XXX] in the form of 2 digits (for example "00" indicating that the UN Regulation No. [XXX] approval was granted following the original version) and the suffixes "A1" or "A2" using the addition sign "+", as described in Example 2 of Model C in Annex 2 Annex 2 - Appendix 3 to this Regulation, for example "0401523 S2W2R3B + 00A2".

4.3.7.	Annex 2 to this Regulation gives examples of arrangements of approval marks.

4.4.	The markings specified in paragraph 4.2. and the approval mark specified in paragraph 4.3. of this Regulation shall be clearly legible, indelible and raised above or sunk below the tyre surface.

5.		Approval

5.1.	If the candidate tyre submitted for approval pursuant to this Regulation meets the requirements of paragraphs 6. and 7. below, approval of that type of tyre shall be granted.

5.2.	An approval number according to Schedule 4 to the Revision 3 of the 1958 Agreement shall be assigned to the type of tyre approved. The same Contracting Party may not assign the same number to another type of tyre.

5.3.	Notice of approval or extension of approval or refusal of approval of a type of tyre pursuant to this Regulation shall be communicated to the Parties to the Agreement which apply this Regulation by means of a form conforming to the model in Annex 1 to the this Regulation.

5.4.	Contracting Parties applying this Regulation shall continue to accept test reports that were issued in accordance with Annex 10 to UN Regulation No. 117, before the entry into force of Supplement [4] to the 04 Series of Amendments to UN Regulation No. 117.

6.		Requirements

6.1.	Abrasion performance limits, as measured by [one of] the method[s] described in Annex 3 to this Regulation.

6.1.1.	The abrasion index of the candidate tyre shall not exceed the values given below:

| Category of use |  | Abrasion index (AICT) |
| --- | --- | --- |
| Normal tyre |  | [1.00] |
| Snow tyre |  | [1.00] |
| Special use tyre |  | [Not defined] |

Note: The limits shall be adjusted for the following specific tyre groups as follows

| Tyre group | Stage 1 | Stage 2 |
| --- | --- | --- |
| Standard load tyres | — | [-0.05/-0.10] |
| Reinforced or extra load tyres | +0.10 | [-0.05/-0.10] |
| Tyres for use in severe snow conditions (3PMSF) | +0.10 | [+0.10] |
| Tyres with a nominal aspect ratio ≤ 40 and a speed category ≥ 300 km/h (speed-category symbol Y) | +0.10 | [—] |
| Tyres with low load index (LI < 77) | +0.10 | [—] |

The adjustments for specific tyre groups are cumulative, e.g., if a candidate tyre is a reinforced snow tyre for use in severe snow conditions, the limit shall be [1.20] for stage 1 and [1.05] for stage 2.

6.2.	[In order to be classified as road legal race tyre, a normal tyre shall meet the following requirements:

- The void-to-fill ratio shall be less than or equal to 25 per cent and

- The tread depth shall be less than or equal to 6 mm.] and

- The aspect ratio shall be less than or equal to 60.

7.		Modifications of the type of tyre and extension of approval

7.1.	Every modification of the type of tyre, which may influence the performance characteristics approved in accordance with this Regulation, shall be notified to the Type Approval Authority which approved the type of tyre. That Authority may either:

7.1.1.	Consider that the modifications are unlikely to have any appreciable adverse effect on the performance characteristics approved and that the tyre will comply with the requirements of this Regulation; or

7.1.2.	Require further samples to be submitted for test or further test reports from the designated Technical Service.

7.2.	Confirmation or refusal of approval, specifying the modifications, shall be communicated by the procedure given in paragraph 5.3. of this Regulation to the Parties to the Agreement which apply this Regulation.

7.3.	The Type Approval Authority granting the extension of approval shall assign a series number for such an extension which shall be shown on the communication form.

8.		Conformity of production

The conformity of production procedures shall comply with those set out in the 1958 Agreement, Schedule 1 (E/ECE/324-E/ECE/TRANS/505/Rev.3) with the following requirements:

8.1.	Any tyre approved under this Regulation shall be so manufactured as to conform to the performance characteristics of the type of tyre approved and satisfy the requirements of paragraph 6. above;

8.2.	The authority which has granted type approval may at any time verify the conformity control methods applied by the manufacturer. In general, the conformity control methods should take into consideration the production volumes of the type of tyre at each manufacturing facility. The normal frequency of these verifications shall be at least once every two years.;

8.3.	Verification tests shall be carried out on random samples of tyres bearing the approval mark required by this Regulation taken from the series production. Because the test procedure involves testing a number of tyres at the same time, the set shall be considered as being one unit for the purposes of calculating the number of tyres to be tested. The Type Approval Authority shall satisfy itself that all tyres falling within an approved type comply with the approval requirement.;

8.3.1.	The verification tests shall be carried out using the same testing method as that adopted for original approval.;

[8.3.2.	Verification tests shall apply the same abrasion margin (Amargin) value as that applied for the original approval.];

8.4.	Production shall be deemed to conform to the requirements of this Regulation if the abrasion index complies with the limits prescribed in paragraph 6.1. of this Regulation, with an additional allowance of [0.200.25] for considering possible mass production variations.

9.		Penalties for non-conformity of production

9.1.	The approval granted in respect of a type of tyre pursuant to this Regulation may be withdrawn if the requirements laid down in paragraph 8. above are not complied with, or if any tyre of the type of tyre exceeds the limits given in paragraph 8.4.

9.2.	If a Party to the Agreement which applies this Regulation withdraws an approval it has previously granted, it shall forthwith notify the other Contracting Parties applying this Regulation by means of a copy of the approval form conforming to the model in Annex 1 to this Regulation.

10.		Production definitively discontinued

If the holder of an approval completely ceases to manufacture a type of tyre approved in accordance with this Regulation, it shall so inform the Type Approval Authority, which granted the approval. Upon receiving the relevant communication that Authority shall inform thereof the other Parties to the 1958 Agreement applying this Regulation by means of a communication form conforming to the model in Annex 1 to this Regulation.

11.	Names and addresses of Technical Services responsible for conducting approval tests of Type Approval Authorities

11.1.	The Contracting Parties to the 1958 Agreement which apply this Regulation shall communicate to the United Nations Secretariat, the names and addresses of the Technical Services responsible for conducting approval tests and, where applicable, of the approved test laboratories and of the Type Approval Authorities which grant approval and to which forms certifying approval or extension of approval or refusal of approval or withdrawal of approval, or production definitively discontinued, issued in other countries, are to be sent.

11.2.	The Contracting Parties to the 1958 Agreement which apply this Regulation may designate laboratories of tyre manufacturers as approved test laboratories.

11.3.	Where a Contracting Party to the 1958 Agreement applies paragraph 11.2. above, it may, if it so desires, be represented at the tests by one or more persons of its choice.

12.		Introductory provisions

12.1.	As from the official date of entry into force of this Regulation, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type approvals pursuant to this Regulation.

12.2.	As from [1 January 2033], Contracting Parties applying this Regulation shall not be obliged to accept type approvals of tyres, first issued after [31 December 2032], pursuant to this Regulation if the requirements of Stage 2 are not complied with.

12.3.	Until [31 December 2034], Contracting Parties applying this Regulation shall continue to grant extensions to type approvals, first issued before [1 January 2033], pursuant to this Regulation, if the requirements of Stage 2 are not complied with.

12.4.	Until the dates given below, no Contracting Parties applying this Regulation shall mandatorily require the approval pursuant to this Regulation of new types of class C1 tyres.

| Stage 1 | [30 June 2028] |
| --- | --- |
| Stage 2 | [31 December 2032] |

12.5.	Until the dates given below, no Contracting Parties applying this Regulation shall mandatorily require the approval pursuant to this Regulation of all types of class C1 tyres.

| Stage 1 | [30 June 2030] |
| --- | --- |
| Stage 2 | [31 December 2034] |

12.6.	Until [30 June 2032], Contracting Parties applying this Regulation and mandatorily requiring the approval pursuant to this Regulation of all types of class C1 tyres shall continue to allow fitting on a vehicle in use of new class C1 tyres manufactured prior to [30 June 2030] and not type approved pursuant to this Regulation.

[12.6.	Notwithstanding paragraph 12.5, Contracting Parties applying this Regulation and mandatorily requiring the approval pursuant to stage 1 of this Regulation shall continue to allow, for a period of 24 months from the date of requiring the approval for all types of class C1 tyres, the fitting on a vehicle in use of new class C1 tyres that are:

- manufactured prior to the date of requiring the approval pursuant to stage 1 for all types of class C1 tyres and

- that are not type approved pursuant to this Regulation.]

[12.7.	Notwithstanding paragraph 12.5, Contracting Parties applying this Regulation and mandatorily requiring the approval pursuant to stage 2 of this Regulation shall continue to allow, for a period of 24 months from the date of requiring the approval for all types of class C1 tyres, the fitting on a vehicle in use of new class C1 tyres that are:

- manufactured prior to the date of requiring the approval pursuant to stage 2 for all types of class C1 tyres and

- that are not type approved pursuant to stage 2 of this Regulation.]

Annex 1

Communication

(Maximum format: A4 (210 x 297 mm))

|  |  | Issued by: | Name of administration: ...................................... ...................................... ...................................... |
| --- | --- | --- | --- |

Concerning:	Approval granted

Approval extended

Approval refused

Approval withdrawn

Production definitively discontinued

of a type of tyre pursuant to UN Regulation No. [XXX]

Approval No........	Suffix:

1.	Manufacturer's name and address:

2.	If applicable, name and address of manufacturer's representative:

3.	"Tyre class" of the type of tyre:

4.	"Category of use" of the type of tyre:

[4.1.	Tyre for use in severe snow conditions (Yes/No)2]

5.	Tyre structure:

6.	Type of tyre designation:

6.1.	Brand name(s)/trademark(s) of the type of tyre

6.2.	Trade description(s)/commercial name(s) of the type of tyre:

7.	Technical Service and, where applicable, test laboratory approved for purposes of approval or of verification of conformity tests:

8.	Tyre abrasion index of the representative tyre size, paragraph 2.6. of this Regulation, as per the test report in Appendix 2 or 6 to Annex 3: …………… using the vehicle test method on public open roads 2 or the indoor drum method2

9.	Number of report issued by the Technical Service:

10.	Date of report issued by that Service:

11.	Reason(s) of extension (if applicable):

12.	Any remarks:

13.	Place:

14.	Date:

15.	Signature:

16.	Annexed to this communication are:

16.1.	A list of documents in the approval file deposited at the Type Approval Authorities having delivered the approval and which can be obtained upon request.

16.2.	A list of tyre size designations: Specify for each brand name/trademark and/or each trade description/commercial name the list of tyre size designations and service descriptions [, adding whether "reinforced" (or "extra load") or not, and whether or not tyre for use in severe snow conditions].

Annex 2

Arrangements of approval marks

Model A:		Example of a separateSeparate UN Regulation No. [XXX] approval mark

Example 1

|  | ↓ |
| --- | --- |
| 001234 A1 | a/3 |
|  | ↑ |

The above approval mark, affixed to a tyre shows that the tyre concerned has been approved in the Netherlands (E4) pursuant to UN Regulation No. [XXX] (marked by the suffix "A1" (abrasion at stage 1)), under the original version of this regulation (indicated by the first two digits "00") and the serial number 1234.

Model B:		Approval pursuant to UN Regulation No. [XXX] coincident with approval pursuant to other regulations

Example 1

|  |  | ↓ |
| --- | --- | --- |
| 0236454 | 001234 A1 | a/3 |
|  |  | ↑ |

The above approval mark shows that the tyre concerned has been approved in the Netherlands (E4) pursuant to UN Regulation No. [XXX] (marked by the suffix "A1" (abrasion at stage 1)) with approval reference number 001234 and UN Regulation No. 30 with approval reference number 0236454. The first two digits of the approval numbers ("00" and "02") indicate that the approval pursuant to UN Regulation No. [XXX] was granted according to the original version and the approval pursuant to UN Regulation No. 30 according to the 02 series of amendments.

Example 2

|  |  | ↓ |
| --- | --- | --- |
| 0236454 | 001234 A2 | a/3 |
|  |  | ↕ |
| 0425896 S2W2R3B |  | a/3 |
|  |  | ↑ |

The above approval mark shows that the tyre concerned has been approved in the Netherlands (E4) pursuant to UN Regulation No. [XXX] (marked by the suffix "A2" (abrasion at stage 2)) with approval reference number 001234, pursuant to UN Regulation No. 30 with approval reference number 0236454 and pursuant to UN Regulation No. 117 with approval reference number 0425896 for rolling sound emission at stage 2, wet adhesion of tyres in new state at stage 2, rolling resistance at stage 3 and wet adhesion of tyres in worn state (indicated by "S2W2R3B"). The first two digits of the approval numbers ("00", "02", and "04", respectively) indicate that the approval pursuant to UN Regulation No. [XXX] was granted according to the original version, the approval pursuant to UN Regulation No. 30 according to the 02 series of amendments, and the approval pursuant to UN Regulation No. 117 according to the 04 series of amendments.

Model C:		Combinations of markings of approvals issued pursuant to UN Regulations Nos. [XXX], 30 and/or 117

Example 1

|  |  | ↓ |
| --- | --- | --- |
| 0236454 + 00A2 |  | a/3 |
|  |  | ↕ |
| 0425896 S2W2R3B |  | a/3 |
|  |  | ↑ |

The above approval mark shows that the tyre concerned has been approved in the Netherlands (E4) pursuant to UN Regulation No. [XXX] (marked by the "+ 00A2" (abrasion at stage 2)) according to the original version of this regulation, pursuant to UN Regulation No. 30 with approval reference number 0236454 and pursuant to UN Regulation No. 117 with approval reference number 0425896 for rolling sound emission at stage 2, wet adhesion of tyres in new state at stage 2, rolling resistance at stage 3 and wet adhesion of tyres in worn state (indicated by "S2W2R3B").

Example 2

|  |  | ↓ |
| --- | --- | --- |
| 0236454 |  | a/3 |
|  |  | ↕ |
| 0425896 S2W2R3B + 00A1 |  | a/3 |
|  |  | ↑ |

The above approval mark shows that the tyre concerned has been approved in the Netherlands (E4) pursuant to UN Regulation No. [XXX] (marked by the "+ 00A1" (abrasion at stage 1)) according to the original version of this regulation, pursuant to UN Regulation No. 30 with approval reference number 0236454 and pursuant to UN Regulation No. 117 with approval reference number 0425896 for rolling sound emission at stage 2, wet adhesion of tyres in new state at stage 2, rolling resistance at stage 3 and wet adhesion of tyres in worn state (indicated by "S2W2R3B").

Example 3

|  | ↓ |
| --- | --- |
| 0236454 + 04S2W2R3B + 00A1 | a/3 |
|  | ↑ |

The above approval mark shows that the tyre concerned has been approved in the Netherlands (E4) pursuant to UN Regulation No. [XXX] (marked by the "+ 00A1" (abrasion at stage 1)) according to the original version of this regulation, pursuant to UN Regulation No. 30 with approval reference number 0236454 and pursuant to UN Regulation No. 117 according to the 04 series of amendments for rolling sound emission at stage 2, wet adhesion of tyres in new state at stage 2, rolling resistance at stage 3 and wet adhesion of tyres in worn state (indicated by "+ 04S2W2R3B").

Annex 3

Procedure for determining the abrasion performance of tyres of class C1

Introduction

For the calculation of the tyre abrasion index of a candidate tyre, the abrasion level of the candidate tyre is shall be compared to the abrasion level of a standard reference test tyre. It is shall be measured with one of the following test methods:

(a)	vehicle test method on public open roads (paragraph 1. of this Annex);

(b)	indoor drum test method (paragraph 2. of this Annex).

1.	Test method (a) using a vehicle on public open roads

1.1.	This method applies to class C1 tyres in scope of this regulation.

1.2.	Definitions

In addition to relevant definitions set out in paragraph 2 of this Regulation the following definitions shall apply to the vehicle test method on public open roads.

1.2.1.	"Circuit" identifies the roads which will be used for the abrasion test. The circuit may consist of one or several loops, which can be run in any order.

1.2.2.	"Loop" means the section of the circuit having the same starting and ending point. If the same loop is run clockwise and counterclockwise it shall be considered as 2 loops.

1.2.3.	"Shift" means the period of time required to run the circuit (including break time, rotation time between vehicle in convoy or drive in vehicle).

1.2.4.	"Total distance" is the total distance run by a tyre during the test.

1.2.5.	"Electric machine" identifies the energy converter transforming between electrical and mechanical energy.

1.2.6.	"Category of propulsion energy converter" means (i) an internal combustion engine, (ii) an electric machine or (iii) fuel cell.

1.2.7.	"Hybrid vehicle" is a vehicle equipped with a powertrain containing at least two different categories of propulsion energy converters and at least two different categories of propulsion energy storage systems.

1.2.8.	"Hybrid electric vehicle" or "(HEV)" is a hybrid vehicle where one of the propulsion energy converters is an electric machine.

1.2.9.	"Not off-vehicle charging hybrid electric vehicle" or "(NOVC-HEV)" is a hybrid electric vehicle that cannot be charged from an external source.

1.2.10.	"Off-vehicle charging hybrid electric vehicle" or "(OVC-HEV)" is a hybrid electric vehicle that can be charged from an external source.

1.2.11.	"Pure electric vehicle" or "(PEV)" is a vehicle equipped with a powertrain containing exclusively electric machines as propulsion energy converters and exclusively rechargeable electric energy storage systems (REESS) as propulsion energy storage systems.

1.2.12.	"Pure internal combustion engine vehicle" or "(ICE) vehicle" is a vehicle where all propulsion energy converters are internal combustion engines.

1.2.13.	"Traction REESS" means an electric energy storage system that is rechargeable and that provides electric energy for electric propulsion.

1.2.14.	"FWD vehicle" or "(Front Wheel Drive) vehicle" means a vehicle where only the front axle delivers traction torque.

1.2.15.	"RWD vehicle" or "(Rear Wheel Drive) vehicle" means a vehicle where only the rear axle delivers traction torque.

1.2.16.	"4WD vehicle" or "(4 Wheel Drive) vehicle" means a vehicle where the axle traction torque for one axle can be switched off by the driver.

1.2.17.	"AWD vehicle" or "(All Wheel Drive) vehicle" means a vehicle with permanent or vehicle controlled 4 wheels drive.

1.2.18.	"Reference vehicle" identifies the vehicle that will be fitted with the reference tyres.

1.2.19.	"Candidate vehicle" identifies the vehicle that will be fitted with the candidate tyres.

1.2.20.	"Vehicle f2 coefficient" (measured in N/(km/h)2) is the second-order road load coefficient according to UN Regulation No. 154. It is provided at reference conditions.

1.2.21.	"Longitudinal acceleration" (measured in m/s2) is the acceleration in the direction of vehicle movement. Longitudinal acceleration has a positive sign for speed increase and a negative sign for speed decrease (e.g. braking).

1.2.22.	"Lateral acceleration" (measured in m/s2) is the acceleration perpendicular to the direction of vehicle movement. Lateral acceleration has a positive sign when turning left in the direction of the vehicle movement. Lateral acceleration has a negative sign when turning right in the direction of the vehicle movement.

1.2.23.	"Toe IN/OUT angle per wheel on the front (rear) axle" is the average of the measured toe angle of the right front (rear) wheel and the measured toe angle of the left front (rear) wheel.

1.3.	Symbols and abbreviated terms

| Symbol | Unit | Designation |
| --- | --- | --- |
| AICT | No dimension | Abrasion index of the candidate tyre |
| ALCT | mgkm∙t | Abrasion level of candidate tyre at test conditions |
| ALRT | mgkm∙t | Abrasion level of reference tyre at test conditions |
| Amargin | No dimension | Abrasion margin |
| DCi DC | km | Total distance run by a candidate vehicle during the test |
| DRi DR | km | Total distance run by the reference vehicle during the test |
| MCTFi | g | Candidate tyre final mass of tyre i |
| MCTSi | g | Candidate tyre initial mass of tyre i |
| MRTFi | g | Reference tyre final mass of tyre i |
| MRTSi | g | Reference tyre initial mass of tyre i |
| OS | mgkm∙t | Offset of the regression lines of the reference tyre abrasion level of SRTT17S |
| OW | mgkm∙t | Offset of the regression lines of the reference tyre abrasion level of SRTT17W |
| QCi | kg | Test load for each candidate tyre |
| QRi | kg | Test load for each reference tyre |
| SS | mgkm∙t∙°C | Sensitivity of SRTT17S to temperature variation |
| SW | mgkm∙t∙°C | Sensitivity of SRTT17W to temperature variation |
| Ti | °C | Average temperature of the test |
| T | °C | Average temperature of the n tests |

1.4.	Instrumentation

1.4.1.	Instruments for tyre mass measurement

The weight scale shall be able to measure the tyre mass with an accuracy of ± 2  g.

1.4.2.		Instruments for alignment and camber measurement on vehicle

The device shall have an accuracy of ± 0.033 degrees.

1.4.3.	Instruments for vehicle mass measurement per position

The weight scale shall be able to measure the load on each tyre with an accuracy of ± 0.1±0.5 per cent of the reading.

1.4.4.	Instruments for acceleration, distance, and speed measurements

During the test, a continuous evaluation of speed, lateral, and longitudinal acceleration shall be done, with a minimum and recommended sampling rate of 10 Hz. GNSS (Global Navigation Satellite System as defined by ISO 24245:2023) measurement associated with numerical treatment of the positions shall be used. See numerical treatment for GNSS (Global Navigation Satellite System) data in Appendix 1 of this RegulationAnnex.

The distance run by the tyre shall equal the GNSS reported distance plus the distance run without a GNSS signal, unless this distance is estimated by the GNSS itself.

Accelerometers shall not be used.

1.4.5.	Tyre pressure measurement device

The device shall have an accuracy of ± 3  kPa.

1.4.6.	Instruments for weather (rain, snow, ice) measurement

For rain, test drivers shall report mileage with wipers in function (actually wiping the windshield) for each shift.

For snow/ice, test drivers shall report mileage driven with snow or ice on the road for each shift.

1.4.7.	Instruments for temperature measurement

The vehicle external thermometer may be used. Data shall be recorded with time and location on paper or file. Any thermometer positioned to measure external air temperature is acceptable as well. The thermometer shall have a measurement accuracy of ± 1 °C. A Alternatively, a continuous measurement device recording the temperature is acceptablemay be used, as well provided that it fulfils the measurement accuracy described above.

Initial and final measurements shall be done using a calibrated thermometer.

1.4.8.	Instruments for tyre and wheel assembly mass measurement

The weight scale shall be able to measure the tyre and wheel assembly mass with an accuracy of ± 2 g.

1.5.	Tyre, tyre and wheel assembly, and vehicle measurement procedure

1.5.1.		Tyre mass measurement

The tyre shall be cleaned and dried before the mass measurement, with a device or product not removing any rubber from the tyre (e.g. water based non-abrasive cleaner). Any visible stone stones shall be removed from the pattern before mass measurement. The measurement shall be repeated performed 3 times and averaged.

1.5.2.	Tyre and wheel assembly mass measurement

The tyre assembly shall be cleaned and dried before the mass measurement, with a device or product not removing any rubber from the tyre (e.g. water based non-abrasive cleaner). Any visible stone stones shall be removed from the pattern before mass measurement, without air pressure, and without valve core.

The mass measurement shall be performed after checking that the balancing masses are all present on the assembly.

1.5.3.	Vehicle mass measurement procedure

The vehicle shall be cleaned and dried before measurement, with a full fuel tank (ICE vehicleif present), with test ballast as described in paragraph 1.6.12. of this Annex, equipped with the tyres to be tested, and wheels used for the test with drivers’ average weight (i.e. 75 kg). Load Q on each wheel shall be measured.

1.5.4.	Vehicle alignment measurement procedure

The vehicle alignments shall be measured, with a full fuel tank (ICE vehicleif present), with test ballast as described in paragraph 1.6.12. of this Annex, equipped with the tyres to be tested, and wheels used for the test with drivers’ average weight (i.e. 75 kg).

[1.5.5.	Vehicles acceptable suspension system

Vehicles are equipped with suspension variants, offering specific characteristics of vehicle dynamics. Depending on the manufacturer, vehicles differ in the specific suspension variant installed in the chosen vehicle.

Examples:

- Standard suspension with steel springs

- Sport suspension with steel springs

- Regulated sport suspension

- Regulated standard air suspension

- Regulated sports air suspension

- Comfort suspension

- High-stand suspension

The selected suspension variant in the vehicles of a test convoy should be the same for all vehicles (consistent). A mix of vehicles in the convoy with different suspension variants should be avoided due to the reduction of the boundary condition parameters and increase of the comparability of the results.

The chosen suspension variant in the test vehicles should have a minimum of active (controlled) components (especially level control through airs springs), if possible.]

1.6.	Vehicle and circuit requirements

1.6.1.	General requirements

A vehicle where the candidate tyres can be fitted on all 4 positions should be selected as a candidate vehicle. [In the case that it is not possible to find such a vehicle, a vehicle following the prescriptions of paragraph 1.6.6.2. shall be selected.]

Only a vehicle where the reference tyres can be fitted on all 4 positions shall be selected as a reference vehicle.

Alignments setting shall be performed as following:

(a)	Measure and record the alignment values with vehicles in loaded conditions as explained in paragraph 1.5.4.;

(b)	The values measured with loaded conditions will be monitored during the test and will serve as the reference values to respect during the tests.

Alignments (TOEtoe and camber) on both axles of reference vehicle and of each candidate vehicle shall be checked at least:

(c)	At the beginning of the test. The alignment check shall take place at a maximum at of 50 km of distance run before starting the test;

(d)	Optionally at half distance;

(e) 	In the case of an impact that may affect the alignment (e.g. curbstone kerbstone contact, etc);

(f)	At the end of the test. The alignment check shall take place at a maximum at of 50 km of distance run after finishing the test;

(g)	Any additional distance to reach the geometry measurement facility shall not be driven with reference or candidate tyres.

At the end of the test, the alignments shall not vary by more than ± 0.15 ±0.15 degrees for toe and ± 0.3 ±0.3 degrees for camber from the initial measurement under the same condition.

1.6.2.	Vehicles’ acceptable suspension and static tuning for FWD vehicles

1.6.2.1.	The alignment of the Vehicles used for candidate tyresvehicles, measured in loaded condition as described in paragraph 1.5.4., shall fulfil the following requirements:

(a)	Toe IN/OUT angle per wheel on the front axle set to (0 ± 0.1) degrees;

(b)	Camber angle per wheel on the front axle set between -1.2-1.4 degrees to 0 -0.2 degrees;

(c)	Toe IN/OUT angle per wheel on the rear axle between 0.05 degrees and 0.15 degrees;

(d)	Camber angle per wheel on the rear axle between -1.9 degrees and -0.6 degrees.

1.6.2.2.	The alignment of the Vehicle used for reference tyresvehicle, measured in loaded condition as described in paragraph 1.5.4., shall fulfil the following requirements:

(a)	Toe IN/OUT angle per wheel on the front axle set to (0 ± 0.05) degrees;

(b)	Camber angle per wheel on the front axle set between -1.2-1.4 degrees to -0.2 0 degrees;

(c)	Toe IN/OUT angle per wheel on the rear axle between 0.05 degrees and 0.15 degrees;

(d)	Camber angle per wheel on the rear axle between -1.9 degrees and -0.6 degrees;

(e)	In addition, the toe IN/OUT angle in absolute value shall be lower less than or equal to the values used in the candidate vehicles for the front axle.

1.6.3.	Vehicles’ acceptable suspension and static tuning for RWD vehicles

1.6.3.1.	The alignment of the Vehicles used for candidate tyresvehicles, measured in loaded condition as described in paragraph 1.5.4., shall fulfil the following requirements:

(a)	Toe IN/OUT angle per wheel on the front axle set to (0 ± 0.1) degrees;

(b)	Camber angle on the front axle set to (0 ± 0.1) degrees;

(c)	Toe IN/OUT angle per wheel on the rear axle set to (0 ± 0.1) degrees;

(d)	Camber angle on the rear axle set to (0 ± 0.1) degrees.

1.6.3.2.	The alignment of the Vehicle used for reference tyres vehicle, measured in loaded condition as described in paragraph 1.5.4., shall fulfil the following requirement:

(a)	Toe IN/OUT angle per wheel on the front axle set (0 ± 0.05) degrees;

(b)	Camber angle on the front axle set to (0 ± 0.1) degrees;

(c)	Toe IN/OUT angle per wheel on the rear axle set to (0 ± 0.1) degrees;

(d)	Camber angle on the rear axle set to (0 ± 0.1) degrees;

(e) 	In addition, toe IN/OUT in absolute value shall be lower less than or equal to the values used for candidate vehicles for front axle.

1.6.4.	Vehicles’ acceptable suspension and static tuning for 4WD vehicles

4WD vehicles may be used if only one axle is applied as the drive axle. In this case, they are considered as FWD or RWD, depending on the configuration.

1.6.5.	Vehicles’ acceptable suspension and static tuning for AWD vehicles

Vehicles with permanent 4WD shall respect the RWD vehicles settings described in paragraph 1.6.3. of this Annex.

1.6.6.	Special cases

1.6.6.1.	In the case that no vehicle respecting respects the conditions described in paragraphs 1.6.2., 1.6.3., 1.6.4. or 1.6.5., the following process shall be used:

(a)	Measurement with at least 4 different vehicles (if 4 vehicles available or all the available vehicles if less than 4) able to fit the candidate tyres shallmust demonstrate that the settings limits cannot be achieved. The vehicles shall be less than two years old and made by 4 different carmakersvehicle manufacturers.

(b)	Select vehicles (both reference and candidate vehicles) respecting following criteria:

(i)	Front Toe toe IN/OUT angle shall respect the previously given tolerances (0° degree +/- tolerance);

(ii)	Front camber angle shall not differ by more than 0.5°  degrees between Reference reference and Candidate candidate vehicle. Reference vehicle shall have a Front Camber lower less than or equal to the respective value of the candidate vehicle, in absolute value;

(iii)	Rear camber angle shall not differ by more than 0.6° degrees between reference and candidate vehicle. Reference vehicle shall have a rear camber lower less than or equal to the respective value of the candidate vehicle, in absolute value;

(iv)	Rear Toe toe IN/OUT angle shall not differ by more than 0.1° degrees between reference and candidate vehicle. Reference vehicle shall have a rear toe lower less than or equal to the respective value of the candidate vehicle, in absolute value;

(v)	In addition, the following limits shall be respected for alignment of the candidate vehicles, measured in with loaded condition as described in paragraph 1.5.4., shall fulfil the following requirements:

a.	Toe IN/OUT angle per wheel on the front axle set to (0 ± 0.1) degrees;

b.	Camber angle on the front axle set between -1.7 degrees and 0 degree;

c.	Toe IN/OUT angle per wheel on the rear axle set between 0.05 degrees and 0.3 degrees;

d.	Camber angle on the rear axle set between -2.7 degrees and 0.3 degrees.

(vi)	In addition, the following limits shall be respected foralignment of the reference vehicles, measured in with loaded condition as described in paragraph 1.5.4., shall fulfil the following requirements:

a.	Toe IN/OUT angle per wheel on the front axle set to (0 ± 0.05) degrees;

b.	Camber angle on the front axle set between -1.7 degrees and 0 degree;

c.	Toe IN/OUT angle per wheel on the rear axle set between 0.05 degree degrees and 0.3 degrees;

d.	Camber angle on the rear axle set between -2.7 degrees and 0.3 degrees.

[1.6.6.2.	In case no vehicle respecting the conditions described in paragraphs 1.6.1., 1.6.2., 1.6.3., 1.6.4., 1.6.5., or 1.6.6.1. the following process shall be used:

(a)	A vehicle where the candidate tyre can be fitted on one axle shall be selected.

(b)	The candidate tyre shall be fitted on the appropriate axle, while the other axle shall be equipped with a tyre of the same brand and the same pattern, if available.

(c)	LC should not deviate by more than 5 per cent from LR, where

LC	is the ratio of the mass supported by the candidate vehicle’s drive axle and the mass of the candidate vehicle;

LR	is the ratio of the mass supported by the reference vehicle’s drive axle and the mass of the reference vehicle.]

1.6.7.	Vehicle acceptable propulsion energy converter

All the propulsion energy converter types are allowed, as long as they are homogeneous in the convoy. The convoy shall consist of vehicles that belong in to the same vehicle type in terms of the vehicles’ electrification grade (i.e. ICE vehicle or NOVC-HEV or OVC-HEV or PEV).

1.6.8.	Vehicle acceptable transmission system

A FWD vehicle shall be used for the tyre size to be tested when available.

If the tyre size can only be fitted on RWD vehicles, a RWD vehicle shall be used, and the reference tyres shall as wellalso be fitted on a RWD vehicle.

If the tyre size can only be fitted on all wheels driveAWD vehicles, an all wheels driveAWD vehicle shall be used, and the reference tyre shall as wellalso be fitted on all wheels drivean AWD vehicle. If available, a vehicle with similar torque distribution shall be used for both reference tyre and candidate tyre. If not available, the default mode shall be used for both reference vehicle and candidate vehicle.

Vehicles featuring automatic or manual transmission systems are allowed in the same convoy.

1.6.9.	Vehicle driving mode

[Vehicles can be equipped with different driving modes depending on the manufacturer, which can influence the driving behaviour (ESC application) and wheel alignment (air spring suspension). The influence on the wheel alignment depends on the chosen suspension system in the test vehicle.]

If several driving modes are available, the default (or normal or standard) driving mode, if defined by the vehicle manufacturer, shall be selected.

In the case that no default driving mode is defined by the vehicle manufacturer, the use of a representative driving mode shall be agreed with the responsible authority.

1.6.10.	Regenerative braking

The vehicles of the convoy shall have similar regenerative capabilities. This is fulfilled by selecting vehicles of the similar electrification grade (see paragraph 1.6.7.). If the regenerative braking function of a vehicle can be deactivated, the driver is allowed to do so only if all vehicles in the convoy run under the same regenerative braking conditions.

1.6.11.	Vehicle acceptable aerodynamic performances

Aerodynamic performance of the vehicle fitted with reference tyres shall respect the following condition:

The f2 value of the vehicle with reference tyres shall be lower less than or equal to 1.2 times the f2 value of the vehicles with candidate tyres.

The provision defined in this paragraph does not apply when the f2 value of the vehicles is not available to the testing facility.

1.6.12.	Vehicle acceptable mass (depending on the tyre size and tyre load index)

The total vehicle mass shall allow to load the tyre to be loaded with a total load of (67 ± 7) per cent of the total nominal tyre load capacity for 4 tyres.

Example of calculation:

Assuming that the reference tyres load index is 94, which corresponds to a maximum load of 670 kg.

The total load nominal load of the 4 reference tyres would then be: 670* × 4 = 2680 kg.

The loaded vehicle mass shall then be 2680 × *67 % = 1796 kg with a tolerance of 2680* × 7 %, which corresponds to ± 188  kg±188 kg.

Load distribution between front and rear axles shall be as following:

(a)	For FWD vehicles

Front axle load: (56 ± 7) per cent of total vehicle load.

Rear axle load: (44 ± 7) per cent of total vehicle load.

(b)	For AWD/RWD vehicles

Front axle load: (50 ± 7) per cent of total vehicle load.

Rear axle load: (50 ± 7) per cent of total vehicle load.

Ballasting allowing to reach above the loads specified above to be reached is authorized, as long as it does not exceed 85 per cent of the vehicle maximum payload. A minimum ballast of 112.5 kg, representing 1.5 passengers including driver, shall be included.

1.6.13.	Circuit, acceleration, and speed requirements

The circuit shall be a closed loop. Vehicles shall return to the departure point without being transported on a car carrier.

1.6.13.1.	Circuit minimum length

The cCircuit shall be made of one or several closed loops. Vehicles shall return to the departure point. The minimum length shall be 30250 km of different roads. Vehicle The vehicle shall not be transported on a car carrier, except in the case of vehicle/tyre failure.

1.6.13.2.	Driving style distribution

The circuit shall respect the following distribution of acceleration/distance for each one of the represented driving styles:

(a)	Roads representative of highway-like driving style:

(i)	more than 35 per cent of the total distance;

(ii)	The longitudinal acceleration standard deviation shall be in the range from 0.10 m/s2 to 0.45 m/s2;

(iii)	The lateral acceleration standard deviation shall be in the range from 0.15 m/s2 to 1.00 m/s2.

(b)	Roads representative of urban-like driving style:

(i)	more than 25 per cent of the total distance;

(ii)	The longitudinal acceleration standard deviation shall be in the range from 0.45 m/s2 to 0.90 m/s2;

(iii)	The lateral acceleration standard deviation shall be in the range from 0.40 m/s2 to 1.20 m/s2.

(c)	Roads representative of regional-like driving style corresponds to the data points not lying in one of the clusters defined in (a) and (b) of this paragraph;

(d)	In addition, the speed distribution for the circuit shall respect the following conditions:

(i)	The speed shall be lower less than 60 km/h for at least 10 per cent of the global distance;

(ii)	The speed shall be equal to or higher greater than 60 km/h and lower less than 90 km/h for at least 25 per cent of the global distance;

(iii) 	The speed shall be equal to or higher greater than 90 km/h for at least 35 per cent of the global distance.

The calculation of the speed distribution shall be carried out using the 10 Hz data recorded on the global distance.

1.6.13.3.	Global accelerations level

The following provisions regarding the deceleration standard deviation and maximum values shall apply:

1.6.13.3.1.	Standard deviation

The standard deviation of longitudinal and lateral accelerations at the end of the test for the reference vehicle and for each candidate vehicle shall meet the following requirements:

(a)	Longitudinal acceleration: 0.45 m/s2 ± 10 per cent;

(b)	Lateral acceleration: 0.93 m/s2 ± 10 per cent.

In addition, the standard deviations of longitudinal Longitudinal and lateral accelerations at the end of the test standard deviations for each candidate vehicle during the test shall not deviate by more than 5 per cent from those for the reference vehicle from one vehicle to another vehicle of the same convoy.

1.6.13.3.2.	Maximum acceleration

The maximum longitudinal and lateral accelerations at the end of the test for the reference vehicle and for each candidate vehicle shall meet the following requirements:

(a)	Longitudinal acceleration:  ± 5 m/s2 ±5 m/s2 for a distance representing at least 99.98 per cent of the total distance;

(b)	Lateral acceleration: ± 5 m/s2 ±5 m/s2 for a distance representing at least 99.9 per cent of the total distance.

1.6.14.	Speed requirements

Speed, with a measurement tolerance of 10 km/h, shall not exceed the applicable legal limits applying in the respective country where the circuit is located. Additionally, the speed shall not exceed the value of 140 km/h. The maximum tolerance in distance travelled (including measuring tolerance of 10 km/h) is 0.5 per cent (40 km in total for 8000 km driving distance).

1.6.15.	Acceleration and speed monitoring during the test

Acceleration and speed shall be constantly monitored during the test for each car in the convoy.

Details regarding acceleration and speed calculation are provided in Appendix 1 of this Annex.

1.6.16.	Circuit abrasion level

To be usable for tests, the circuit shall respect the following abrasion level specifications for reference tyres:

(a)	SRTT17S: the circuit abrasion level at 20 ℃ shall be in the range from 25 to 75 mg/(km∙t);

(b)	SRTT17W: the circuit abrasion level at 10 ℃ shall be in the range from 25 to 75 mg/(km∙t).

If a circuit uses only one of the reference tyres (e.g. only the SRTT17S), only one of the conditions shall be respected, this being the one for the reference tyre which is used on the circuit.

Abrasion level (mg/(km∙t))

Temperature (test average)

20 °C

s1

s2

Figure 1: The normalised to 20°C abrasion level of the test shall be within s1 and s2.

The calculation of the circuit abrasion level shall be made according to paragraph 1.6.16.1. of this Annex.

1.6.16.1.	The following provisions apply for the measurement of the abrasion level of the reference tyre:

(a)	At least one reference tyre (SRTT17S or SRTT17W) shall be selected. selected;

(b)	 The reference tyre shall be measured at least at 3 different temperatures differing from each other by more than 5 °C covering a temperature range of at least 10 °C;

(bc)	The value of abrasion level for references tyres at 20 °C (SRTT17S) or 10°C (SRTT17W) shall be given by a linear regression;

(cd)	For SRTT17S, at least one measurement shall be done between 15 and 25 ℃;

(de)	For SRTT17W, at least one measurement shall be done between 5 and 15 ℃.

For each of the (at least) 3 sets of reference tyres tested, the ALRTi abrasion level in mgkm∙t at a temperature Ti is available.

Calculation shall be updated each quarter using the results of all performed tests from the previous 4 quarters, starting one year after the circuit’s initial accreditation. Update of slopes and offset at origin may only be performed if the range of temperature covers the provisions of this paragraph.

The sensitivity of SRTT17S and SRTT17W to temperature variation during the test ("SS" and "SW") (i.e. the slope of the regression line of the reference tyre abrasion level to the average test temperature) is shall be calculated by means of the following equation:

SS=i=1nALRTi-ALRT×Ti-Ti=1nT i-T2	for SRTT17S

SW=i=1nALRTi-ALRT×Ti-Ti=1nT i-T2	for SRTT17W

The offsets of the regression lines of the reference tyre abrasion level to the average test temperature during the test are shall be calculated by means of the following equations:

OS=ALRT-SS∙T	for SRTT17S

OW=ALRT-SW∙T	for SRTT17W

The circuit abrasion levels at the relevant temperatures are shall be calculated by means of the following equations:

RACS=OS+SS∙20	for SRTT17S

RACW=OW+SW∙10	for SRTT17W

Where:

S	is the slope of the regression line of the reference tyre abrasion level to the temperatures during the tests;

ALRTi	is the abrasion level of reference tyre at test conditions in mg/(km∙t);

ALRT	is the average abrasion level of the n reference tyre tests at three temperatures in mg/(km∙t);

Ti	is the average temperature of the test in °C;

T	is the average temperature of the n tests in °C.

n	is the number of tests performed.

If the circuit is utilized for both SRTT17S and SRTT17W, the S calculation shall be done for each reference tyre, giving SS and SW values.

1.7.	Weather and climate conditions requirements

1.7.1.	When testing tyres against SRTT17S according to the table in paragraph 1.8. of this Annex, the following weather and climate conditions shall be respected:

(a)	The average ambient temperature during the test shall be within the following range: from 107 °C to 35 °C. However, for tyres designed for maximum speed greater than or equal to 300 km/h, the minimal average ambient temperature during the test shall be greater than or equal to 12 ℃;

(b)	The minimum and maximum ambient temperature during the test shall be within the following range: from 52 °C to 40 °C for at least 90 per cent of the test distance. However, for tyres designed for maximum speed greater than or equal to 300 km/h, the minimal ambient temperature for at least 90 per cent of test distance during the test shall be greater than or equal to 7 ℃;

(c)	No driving under snow or ice conditions is shall be allowed;

(d)	The maximum allowed percentage of the total distance driven under wet conditions is shall be 20 per cent.

1.7.2.	When testing tyres against SRTT17W according to the table in paragraph 1.8. of this Annex, the following weather and climate conditions shall be respected:

1.7.2.1.	Tyres for use in severe snow conditions

(a)	The average temperature during the test shall be within the following range: from -3 °C to 1720 °C;

(b)	The minimum and maximum temperature during the test shall be within the following range: from -7 °C to 2225 °C for at least 90 per cent of the test distance;

(c)	No driving under snow or ice conditions is shall be allowed for more than 5 per cent of the total distance driven;

(d)	The maximum allowed percentage of the total distance driven under wet conditions is 20 per cent.

1.7.2.2.	Tyres not for use in severe snow conditions

(a)	The average temperature during the test shall be within the following range: from -3 °C to 35 °C;

(b)	The minimum and maximum temperature during the test shall be within the following range: from -7 °C to 40 °C for at least 90 per cent of the test distance;

(c)	No driving under snow or ice conditions shall beis allowed for more than 5 per cent of the total distance driven;

(d)	The maximum allowed percentage of the total distance driven under wet conditions is 20 per cent.

1.7.3.	Weather data recording

1.7.3.1.	Wet distance measurement

The wet distance, expressed as a per cent of the distance travelled, corresponds to the distance travelled with wipers on. These This can be manually collected on one vehicle of the convoy. Alternatively, data can be collected from vehicle information (e.g. CAN bus -Controller Area Network- or OBD - On-Board Diagnostics-) when available.

1.7.3.2.	Average ambient temperature

For the calculation of the average temperature, a minimum 5 of 5 measurements per shift on the circuit shall be carried out. The measurements shall include the starting and arrival points. Furthermore, the temperature at the highest altitude reached on the circuit shall be measured. Measurements shall be made on from at least one vehicle of the convoy.

The average temperature of the circuit shall be calculated as the average of all the measured temperatures of all 5 points at the end of the test.

At least at the starting and arrival points, the measurement shall be done with a fixed device respecting requested the accuracy specified in paragraph 1.4.7. The temperature sensor shall be positioned outside in an unobstructed location, exposed to the airflow, and protected from direct solar radiation. The latter may be achieved by any shading screen or similar device.

For measurements on the road, a weather station installed in the vehicle with external temperature sensor may be used. Continuous temperature measurement throughout the test is acceptable. In this case, average, minimum, and maximum measurement should be reported for the full test. The 10 first 10 minutes after departure and after each driver’s break shall be discarded from the minimum, maximum, and average calculation. Both time-based and distance-based average for temperature are acceptable.

1.8.	Standard Reference Test Tyre requirements

For the evaluation of the abrasion performance of the candidate tyre using the vehicle test method on public open roads, the reference tyre shall be selected according to the following table:

|  |  | Reference tyre |  |
| --- | --- | --- | --- |
| Category of use of the candidate tyre |  | SRTT17S | SRTT17W |
| Normal tyre |  | X |  |
| Snow tyre |  |  | X |
|  | Snow tyre that is classified as tyre for use in severe snow conditions |  | X |
| Special use tyre |  | X |  |
|  | Marked with "M+S" or "M.S" or "M&S" |  | X |
|  | Special use tyre that is classified as tyre for use in severe snow conditions |  | X |

Reference tyres SRTT17S and SRTT17W shall be stored in condition recommended in ASTM F3676-23 and ASTM F3675-23 respectively.

1.9.	Preparation and adjustments with respect to tyres

1.9.1.	Tyre fitment on rim and vehicle

New candidate tyres shall be mounted and balanced on a rim permitted by the tyre manufacturer.

New Reference tyres Each new reference tyre for each test shall be fitted on a rim of rim width code 7.57.5 Rim width code rim.

The rim width code of a candidate tyre shall be reported in the test report. Tyres with special fitment requirements, such as asymmetric or directional design, shall also be mounted in accordance with these requirements: the direction of rotation shall be respected and the side of the tyre intended to face the outside of the vehicle outside shall be positioned appropriately.

1.9.2.	Tyre weight mass measurement (without rim)

The tyre weight mass measurement shall be done following the procedure described in paragraph 1.5.1. of this Annex.

No rubber (e.g. mould vents) shall be removed from any tyre (test or reference).

Each tyre weight mass shall be measured:

(a)	Before being fitted on the wheel, to get the initial tyre mass (MRTSiMRTSi) for reference tyres and (MCTSiMCTSi) for candidate tyres;

(b)	After completing the test and dismounting the tyre from the tyre wheel assembly, to get the final tyre masses (MRTFiMRTFi) for reference tyres and (MCTFiMCTFi) for candidate tyres.

1.9.3.	Tyre and wheel assembly mass measurement

The tyre and wheel assembly mass measurement shall be done following the procedure described in paragraph 1.5.2. of this Annex.

Intermediate measurements of tyre and wheel mass assembly are optional.

1.9.4.	Tyre inflation pressure

The inflation pressure of the reference tyres shall be 290 kPa.

Candidate tyres shall be inflated (cold) at their nominal pressure determined by the standard they belong conform to. More specifically:

(a)	For standard load tyres the pressure shall be 250 kPa;

(b)	For reinforced tyres or extra load tyres the pressure shall be 290 kPa;

(c)	The nominal pressure for nominal load as determined by the relevant standard they conform to, if different from (a) and (b).

1.10.	Preparation and adjustments with respect to vehicles

Vehicles for candidate and reference tyres shall be selected following the constraints of requirements set out in paragraph 1.6. of this Annex.

1.10.1.	Vehicle mass measurement

The vehicle mass measurement shall be done following the procedure described in paragraph 1.5.3. of this Annex. The vehicle shall be ballasted following the specifications described in paragraph 1.6.12. of this Annex.

Measurements of each tyre load Q for reference and candidate vehicles are requiredshall be performed.

1.10.2.	Vehicle tuning

Vehicle alignments to shall be tuned following the provisions specified in paragraphs 1.6.2. to 1.6.6. of this Annex.

1.11.	Test method and measurements

1.11.1.	General

The tyre abrasion test shall be run on open roads. A maximum of 4 vehicles are allowed in a convoy. The vehicles shall drive approximately 8000 km along selected circuits with a given driving severity with the aim of exposing each candidate tyre to the same conditions (e.g. severity, drivers, position in convoy, weather).

The tyres are evaluated relatively to a reference tyre. The reference tyre shall be fitted on one convoy's vehicle in order to absorb mainly minimize the influence of temperature variation, but as well as other varying parameters as well.

A candidate vehicle shall be fitted with the same candidate tyres.

The measured performance shall be calculated according to paragraph 1.11.13. of this Annex.

1.11.2.	Test total distance

The total distance driven by each vehicle in the convoy shall be 8000 km ± 300 km. Any total distance outside these boundaries shall invalidate the test.

1.11.3.	Convoy composition and management

The convoy shall be homogeneous regarding the following vehicle parameters:

(a)	Number and position of driven wheels (see paragraph 1.6. of this Annex)

(i) 	FWD vehicles only in the convoy;

(ii) 	RWD vehicles only in the convoy;

(iii) 	AWD vehicles (4 permanent driven wheels) only in the convoy.

(b)	Propulsion energy converters (e.g. pure internal combustion engine vehicle (ICE) vehicles only, NOVC-HEVCs HEVs only, OVC-HEVs only, or PEVs only) in the same convoy. For hybrid vehicles, the provisions described in paragraph 1.6.7. of this Annex shall apply.

The same model and same settings of vehicles shall be used for both reference tyre and candidate tyre provided that:

(c)	The candidate tyre size can be fitted on the same vehicle model as for the reference tyre;

(d)	Loading and alignment parameters are allowed for candidate tyre.

Regarding the maximal distance between vehicles in the convoy, each driver shall be able to have visual contact with the preceding and following vehicles.

Each vehicle shall drive on the right lane (or left lane for left driving countries) when free.

1.11.4.	Vehicle rotation in the convoy and driver rotation on vehicles

Each candidate tyre set, including tested and the reference tyres,tyre set shall run equal parts of the test:

(a)	with all each drivers; and

(b)	 in all each positions in the convoy.

Changes in drivers and vehicle positions may occur within with a tolerance of 10 per cent of the predefined distance in length of the circuit.

During a test, the drivers shall change vehicles, and the vehicles shall change positions within the convoy, so that each candidate tyre set and the reference tyre set are exposed to the same conditions as closely as possible.

1.11.5.	Data measured before, during, and after the test

1.11.5.1.	Before and after the test:Before and after the test, the following measurements shall be done:

(a)	Mass of each tyre;

(b)	Load on each tyre;

(c)	Vehicle alignments (loaded vehicle);

(d)	Tyre pressure after tyre fitment and before dismounting tyres.

1.11.5.2.	During the testDuring the test, the following recordings shall be performed:

(a)	Continuous recordings of parameters needed for the calculation of longitudinal and lateral accelerations on each vehicle;

(b)	Continuous speed measurement on each vehicle;

(c)	Temperature measurement (as specified in paragraph 1.7.3. of this Annex.);

(d)	Tyre pressure each day under cold conditions. By cold conditions it is meant at least 30 minutes after the last stopvehicle is at rest. Tyres shall never be deflated;

(e)	Vehicle alignment of reference vehicle, in loaded condition, accompanied by correction to initial value if relevant for vehicle used for reference tyres. The vehicle alignment measurement shall be carried out 4 3 times during the test, each at roughly a quarter of the test distance.

At intermediate stops, it is recommended but not compulsory to measure:

(f)	Tyre and Wheel wheel assembly mass;

(g)	Vehicle alignment of candidate vehicles, in loaded condition, accompanied by correction to initial value if relevant for vehicle used for candidate tyres.

1.11.6.	Data processing for average temperature

Temperature measurement during the test:

Temperature shall be measured according to paragraph 1.7.3.

1.11.7.	Data processing for test longitudinal and lateral accelerations standard deviation

During each shift, a continuous evaluation of speed, lateral, and longitudinal accelerations shall be carried out. A minimum sampling rate of 10 Hz is recommended. Most common technology is GNSS (global Global Navigation Satellite System) measurement associated with numerical treatment of the positions.

Acceleration data processing is defined in Appendix 1 of this Annex.

1.11.8.	Test validation

The test is considered valid when the following conditions are met:

(a)	Temperatures: minimum, maximum, and average temperatures as calculated in paragraph 1.11.6. of this Annex shall respect specifications defined in paragraph 1.7. of this Annex;

(b)	Accelerations: lateral and longitudinal acceleration shall respect maximum and standard deviation values as calculated in paragraph 1.11.7. of this Annex and shall respect the specifications defined in paragraph 1.6.13.3. of this Annex;

(c)	If more than 1500 600 km GNSS acceleration data are missing for the candidate tyre, the test for this candidate tyre shall beis invalid;

(d)	If more than 1500 600 km GNSS acceleration data are missing for the reference tyre, the whole test shall beis invalid;

(e)	Vehicle alignments at the beginning and end of test shall respect the specifications defined in paragraph 1.6. of this Annex;

(f)	For candidate tyres of category of use snow tyres and special use tyres marked "M+S", either whether or not classified as tyres for use in severe snow conditions, the abrasion level of the SRTT17W normalized at 10 °C (ALRT10) shall be in the range defined in paragraph 1.6.16. of this Annex;

(g)	For other candidate tyres, the abrasion level of the SRTT17S normalized at 20 °C (ALRT20) shall be in the range defined in paragraph 1.6.16. of this Annex;

(h)	A visual inspection of the reference tyres shall show no damage. The tyre sidewall marking shall still be readable. If a reference tyre was losingloses more than 1 cm2 of tread chunking area, the tyre shall be considered as destroyed, and the process for tyre destroyed tyresprocess as described in paragraph 1.11.11. of this Annex shall apply.

1.11.9.	Deviation from nominal circuit

The circuit is considered valid when the following provisions are met altogether:

(a)	The circuit is modified by less than 10 km for the full test or if it is modified by more than 10 km and less than 30 km, for less than 8 shifts;

(b)	The total driven distance remains in thewithin (8000 ± 300) km;

(c)	The abrasion level of reference tyre at 20 °C or 10 °C as applicable is within the ranges specified in paragraph 1.6.16. of this Annex;

(d)	The acceleration limits are within the ranges specified in paragraphs 1.6.13.2. and 1.6.13.3. of this Annex.

When all provisions are met the circuit shall beis considered valid and the distance considered for calculation has toshall be corrected accordingly.

Accidental deviation(s) are acceptable if representing less than 20 per cent of the circuit distance or less than 100 km (whichever is lower), under the condition that the reference tyre abrasion level at 20 °C or 10 °C, as applicable, stays in authorized limits and acceleration standard deviations are respected.

In all other cases, the test is considered not valid and the circuit has toshall be revalidated.

1.11.10.	Vehicle trouble handling

The following provisions apply in the case of vehicle damages to vehicles in the convoy:

(a)	If a vehicle used in the convoy is damaged and cannot be used anymore (e.g. major mechanical failure or accident), it shall be replaced by an identical vehicle that shall be identically loaded and tuned. The replacement vehicle, equipped with the same tyres having started the test, shall run the distance lost due to vehicle failure on the lost segment of the circuit alone without the other vehicles of the convoy;

(b)	If a vehicle used in the convoy is broken down and can be repaired, the lost distance shall be run without other convoy vehicles on the lost segment of the test circuit;

(c)	If the failure occurs on a candidate vehicle and not on the reference vehicle, the convoy may continue the test and the failing vehicle/tyre shall be withdrawn from the convoy. A new set of candidate tyres shall then be used for a new test, starting from scratch.

1.11.11.	Tyre trouble handling

The following provisions apply in the case of tyre damages in the convoy:

(a)	If a tyre used during the test on the reference vehicle or one of the candidate vehicles is damaged by a reparable puncture and if the tyre can be repaired without running without pressure, the added repair mass shall be recorded and taken into account in the final calculation. The use of a spare tyre is permitted for a maximum distance of one loop or maximum of 7.5 per cent of the test distance. The mileage run with the spare tyre shall be recorded and taken into account for the tyre abrasion level;

(b)	If a tyre used during the test is destroyed (or has a non-repairable puncture or is run without pressure), the mass loss of the other tyre tested on the same axle shall beis used twice to perform the final calculation. The spare tyre used to replace the destroyed tyre should have the same size and same pattern as the replaced tyre.

1.11.12.	GNSS trouble handling

If the speed and acceleration recording for one vehicle for during one shift failed for more than 5 10 per cent of the circuit distance (because of missing satellites signalsatellite signals or device failure), the missing data of this shift shall be replaced with one of the other vehicles (preferably the reference vehicle) of the same convoy of the same shift, if validnot be used for the evaluation of the speed, the longitudinal acceleration and the lateral acceleration of that vehicle, and the distance of the shift shall be treated as distance with missing GNSS signal provided that the limits defined in paragraph 1.11.8. of this Annex are respected.

1.11.13.	Data processing for abrasion level calculation.

1.11.13.1.	Reference tyre abrasion level at average test temperature (mgkm∙t)

The average abrasion level at test average temperature of the reference tyre during the test is calculated as following:

ALRT=1000*i=1nMRTSi-MRTFi/DRii=1nQRi/1000

ALRT=106∙i=1nMRTSi-MRTFiDR∙i=1nQRi

Where:

ALRT	is the abrasion level of the reference tyre at test average temperature in mg/(km∙t);

MRTSi	is the mass of the reference tyre at the beginning of the test in g;

MRTFi	is the mass of the reference tyre at the end of the test in g;

DRi	DR	is the total distance of the reference vehicle in km;

QRi		is the test load of the reference tyre in kg;

nN	is the number of reference tyres.

1.11.13.2.	SRTT17S abrasion level calculation at 20 °C

Apply tThe temperature correction determined in paragraph 1.6.16.1. of this Annex shall be applied to the normal reference tyre abrasion level as follows:

ALRT20=ALRT+SS∙20-T

where T is the average temperature of the test

1.11.13.3.	SRTT17W abrasion level calculation at 10 °C

Apply tThe temperature correction determined in paragraph 1.6.16.1. of this Annex shall be applied to the Winter reference tyre abrasion level as follows:

ALRT10=ALRT+SW∙10- T

where T the average temperature of the test

1.11.13.4.	Candidate tyre abrasion level at average test temperature in mg/(km∙t)

The average abrasion level at test average temperature of the candidate tyre during the test shall beis calculated as following:

ALCT=1000*i=1nMCTSi-MCTFi/DCii=1nQCi/1000

ALCT=106∙i=1nMCTSi-MCTFiDC∙i=1nQCi

Where:

ALCT	is the abrasion level of the candidate tyre at test average temperature in mg/(km∙t);

MCTSi	is the mass of the candidate tyre at the beginning of the test in g;

MCTFi	is the mass of the candidate tyre at the end of the test in g;

DCi DC	is the total distance of the candidate vehicle in km;

QCi	is the test load of the candidate tyre in kg;

n	is the number of candidate tyres.

1.11.13.5.	The abrasion index of the candidate tyre shall be calculated from the following equation:

AICT=ALCTALRT-Amargin

Where:

AICT	is the abrasion index of the candidate tyre;

ALCT	is the abrasion level of the candidate tyre, in mg/(km∙t);

ALRT	is the abrasion level of the reference tyre, in mg/(km∙t);

Amargin	is the dimensionless abrasion margin, which accounts for the complexities of the abrasion level measurement methods, and is equal to [0.20].the special abrasion characteristics of specific tyre groups as indicated below. Amargin is defined in two steps that account for the evolution of abrasion level measurement methods and tyre technology, as given in the following table:

|  |  | Stage 1 | Stage 2 |
| --- | --- | --- | --- |
| Amargin by tyre category of use |  |  |  |
| Normal |  | [0.20] | [0.15] |
| Snow |  | [0.20] | [0.15] |
| Special use |  | [Not defined] | [Not defined] |
| Amargin allowances for specific tyre groups (to be incorporated into Amargin) |  |  |  |
| Tyre for use in severe snow conditions (3PMSF) |  | [+0.10] | [+0.10] |
| Reinforced or extra load tyre (XL) |  | [+0.10] | [-] |
| Tyres with a nominal aspect ratio ≤ 40 and suitable for speeds ≥ 300 km/h |  | [+0.10] | [-] |
| [Tyres with low load index (LI < 77)] |  | [+0.10] | [-] |

Note: Amargin allowances for specific tyre groups are cumulative. E.g., if a candidate tyre is a reinforced snow tyre for use in severe snow conditions, the applicable Amargin shall be 0.2 + 0.1 + 0.1 = 0.4.

1.12.	Test report

1.12.1.	The test report shall include the following information:

(a)	Average, minimum, and maximum ambient temperature during the test;

(b)	Percentage of distance covered on wet roads;

(c)	Reference of the circuit used for the test, including the circuit length, driving style distribution, and location;

(d)	Total deviation distance to the nominal distance in km;

(e)	Start and end dates of the test;

(f)	Amargin applied, with indication of any allowances incorporated therein.

1.12.2.	For each reference tyre, the following information shall be reported:

(a)	Model of vehicle used for reference tyre;

(b)	Tyre data, including manufacturer, brand name, trade name, size, LI and load capacity, speed-category symbol, reference pressure, and serial number of the tyres;

(c)	Vehicle tuning at the beginning of the test (Front axle TOE toe and camber, rear axle TOE toe and camber), in loaded condition;

(d)	Vehicle tuning at each intermediate measurement of the test (Front axle TOE toe and camber, rear axle TOE toe and camber), in loaded condition;

(e)	Vehicle tuning at the end of the test (Front axle TOE toe and camber, rear axle TOE toe and camber), in loaded condition;

(f)	Rim width code (7.5);

(g)	Cold inflation pressure at the fitment;

(h)	Cold inflation pressure at 50 per cent of the test;

(i)	Cold inflation pressure at the end to the test;

(j)	Balancing mass at the beginning of the test;

(k)	Balancing mass at the end of the test;

(l)	Initial tyre mass (MRTSiMRTSi) for each reference tyre;

(m)	Final tyre mass (MRTFiMRTFi) for each reference tyre;

(n)	Abrasion level in mg/(km∙t) normalized at 20 °C or 10 °C as applicable;

(o)	Distance run for each reference tyre;

(p)	Standard deviation of longitudinal acceleration for the vehicle fitted with reference tyre;

(q)	Standard deviation of lateral acceleration for the vehicle fitted with reference tyre;

(r)	Percentage of distance covered over the maximum longitudinal acceleration for the vehicle fitted with reference tyre;

(s)	Percentage of distance covered over the maximum lateral acceleration for the vehicle fitted with reference tyre;

(t)	Percentage of time travelled over each speed range (i.e. urban-like, regional-like, and highway-like);

(u)	Measured tyre load for each reference tyre;

(v)	Reference tyres visual inspection report.

1.12.3.	For each candidate tyre, the following information shall be reported:

(a)	Model of vehicle used for candidate tyre;

(b)	Tyre data, including manufacturer, brand name, trade name, size, LI and load capacity, speed-category symbol, reference pressure, and serial number of the tyre;

(c)	Vehicle tuning at the beginning of the test (Front axle TOE toe and camber, rear axle TOE toe and camber) in loaded condition;

(d)	Vehicle tuning at the end of the test (Front axle TOE toe and camber, rear axle TOE toe and camber) in loaded condition;

(e)	Rim width code;

(f)	Cold inflation pressure at the fitment;

(g)	Cold inflation pressure at 50 per cent of the test;

(h)	Cold inflation pressure at the end to the test;

(i)	Balancing mass at the beginning of the test;

(j)	Balancing mass at the end of the test;

(k)	Initial tyre mass (MCTSiMCTSi) for each candidate tyre;

(l)	Final tyre mass (MCTFiMCTFi) for each candidate tyre;

(m)	Measured tyre load for each candidate tyre;

(n)	Distance run for each candidate tyre;

(o)	Standard deviation of longitudinal acceleration for the vehicle fitted with candidate tyre;

(p)	Standard deviation of lateral acceleration for the vehicle fitted with candidate tyre;

(q)	Percentage of distance covered under the maximum longitudinal acceleration for the vehicle fitted with candidate tyre;

(r)	Percentage of distance covered under the maximum lateral acceleration for the vehicle fitted with candidate tyre;

(s)	Percentage of time travelled over each speed range (i.e. urban-like, regional-like, and highway-like).

1.12.4.	Final test results

(a)	The measured result of abrasion level ALRT for the reference tyre during the test at average test temperature as described in paragraph 1.11. of this Annex;

(b)	The measured result of abrasion level ALCT for the candidate tyre during the test at average test temperature as described in paragraph 1.11. of this Annex;

(c)	The final result tyre abrasion index AICT as described in paragraph 11.paragraph 1.11. of this Annex.

2.	Test method (b) using indoor drum

2.1.	This method applies to class C1 tyres in scope of this regulation.

2.2.	Definitions and Terms

In addition to relevant definitions provided in Paragraph 2 of this Regulation the following definitions shall apply to the indoor drum method.

2.2.1.	"Tyre abrasion" means tyre wear that is observed as the loss of tyre mass during usage.

2.2.2.	"Mass loss" means the amount of the mass, expressed in grams, lost due to tyre abrasion.

2.2.3.	"Mean profile depth" is used for the characterization of the surface roughness in macroscale and is described in ISO 13473-1.

2.2.4.	"Micro-roughness" means the surface roughness characterized in microscale and is measured by altered filtering conditions as defined in ISO 13473-1.

2.2.5.	"Tyre coordinate system" is tyre coordinate system specified in ISO 8855.

2.2.65.	"Vertical load" means the tyre normal force of a tyre exerted on the road resulting from the mass supported by the tyre. Tyre normal force is specified in ISO 8855.

2.2.76.	"Lateral force" means the force of a tyre generated in lateral direction during cornering. Tyre lateral force is specified in ISO 8855. It takes a positive sign when turning left and a negative sign when turning right.

2.2.87.		"Longitudinal force" means the force of a tyre generated in the longitudinal direction during acceleration or braking. Tyre longitudinal force is specified in ISO 8855. It takes a positive sign for speed increase and a negative sign for speed decrease (e.g. braking).

2.2.98.	"Loaded radius" means the distance from the tyre axis to the drum outer surface under steady-state conditions at 0 speed and 0 camber as well while the test load and inflation pressure is applied at room temperature and refer to the thermal conditioning of Paragraph 2.6.2. of this Annex.

2.2.109.	"Tyre torque" means the moment on applied to the tyre rotation axle.

2.2.1110.	"Load index" means a numerical code associated with the maximum load that a tyre can carry at the speed indicated by its speed-category symbol under the service conditions specified by the tyre manufacturer.

2.3.	Symbols and Abbreviated terms

In addition to relevant symbols and abbreviated terms provided in Paragraph 1.3. of Annex 3 of this Regulationthis Annex, the following symbols and abbreviated terms apply to the indoor drum method.

| Symbol | Unit | Designation |
| --- | --- | --- |
| T | No dimension | Candidate tyre |
| R | No dimension | Reference tyre |
| Fz | N | Vertical load |
| Fy | N | Lateral force |
| Fx | N | Longitudinal force |
| rL | m | Loaded radius |
| My | Nm | Tyre torque |
| LI | No dimension | Load Index |
| MPD | mm | Mean Profile Depth |

2.4.	Test Method

2.4.1.	General

This test method evaluates the mass loss of the candidate tyre relative to the reference tyre.

In measuring tyre abrasion per distance travelled, it is necessary to control normal load, lateral force, and longitudinal force applied to a test tyre.

This test method uses a tyre abrasion test equipment with a cylindrical flywheel (drum) with an abrasive test surface on the external surface of the drum.

2.4.2.	Drum Tyre Abrasion Test Machine Specifications

2.4.2.1.	Tyre Abrasion Test Equipment

Tyre abrasion test equipment shall consist of a drum, a tyre carriage device, a loading device, and adhesion prevention system. There can be one or two carriage devices.

2.4.2.2.	Drum DiameterSpecification

The tyre abrasion test machine shall have a cylindrical flywheel (drum) built with a rigid and non-deformable material with an external diameter of at least 3 3 m. [The drum circumference shall be measured over test surface segments.]

2.4.2.3.	Test Surface

The test surface shall be applied to external surface of the cylindrical drum. The test surface of drum shall meet the following minimum requirements:

(a)	The test surface of the drum shall have a MPD measured at the start and the end of the drum test not exceeding 2,0 2.0 mm, according to ISO 13473-1;

(b)	The test surface of the drum shall have a micro-roughness measured at the start and end of the drum test within the range from 0.07 mm to 0.4 mm. The measuring method of MPD described in ISO 13473-1 shall apply to the measurement of micro-roughness, except for sampling interval, resampling, high-pass and low-pass filtering, and segment length. The sampling interval shall not be more than 0.033 mm, and samples shall be taken at a fixed interval in the horizontal direction. Re-sample theThe signal shall be re-sampled to either 0.017 mm (preferably) or 0.033 mm spacing. For high-pass and low-pass filtering, the filters shall be of the Butterworth type, 2nd order, and shall have a cut-off at 3.0 mm and 0.1 mm texture wavelength, respectively. The segment length shall be 3.33 mm ± 0.33 mm;

(c)	The test surface of the drum shall be textured with sands, stones, or an alternative material, e.g., aluminium oxide resin;

(d)	The drum surface shall be built with rigid and not deformable material;

(ed)	The test surface, including voids, shall be dry and clean during the entire measurement procedure and for all measurements;

(fe)	The device for measurement of the MPD shallmust fulfil the specifications of ISO 13473-3. The device for measurement of micro-roughness shallmust fulfil the specifications of ISO 13473-3, except for horizontal resolution, which shall not be more than 0.033 mm.

The abrasion level of the SRTT17S reference tyre for all types of surface shall be in the range between 50 45 mg/(km∙t) and [190 135] mg/(km∙t).

The abrasion level of the SRTT17W reference tyre for all types of surface shall be in the range between 35 mg/(km∙t) and [165 135] mg/(km∙t). The abrasion level shall be calculated according to the method in paragraph 2.8. of this Annex. In the case of that sandpaper is used for the surface it shall be replaced as specified in Appendix 5 of this Annex.

When the drum surface no longer satisfies the conditions of the previous periodset out above, the surface shall be replaced. Intermediate inspection of the abrasion rate for the reference tyre is recommended.

2.4.2.4.	Width

The width of the test surface shall always exceed the width of the test tyre contact patch throughout the entire test duration.

2.4.3.	Tyre Carriage and Drive System

The tyre carriage and drive system shall be able to provide dynamic control of:

(a)	tyre Tyre lateral force developed by the drag force produced by tyre slip angle during running;

(b)	Longitudinal tyre force or torque developed by tractive force by the tyre during braking and accelerating.

The maximum allowed deviation from the nominal value of load (Fz), lateral force (Fy), longitudinal force (Fx), and tyre torque (My) during testing is defined as follows:

(c)	Fz: ±50N or 1 per cent whichever is greater;

(d)	Fy: ± 100N or 5 per cent whichever is greater, for the difference between input peaks and actually generated peaks;

(e)	Fx: ± 100N or 5 per cent whichever is greater, for the difference between input peaks and actually generated peaks;

(f)	My: ±40Nm or 5 per cent whichever is greater, for the difference between input peaks and actually generated peaks.

2.4.4.	Adhesion Prevention System

2.4.4.1.	Powder Distribution

The tread abrasion test equipment shall have a powder distribution system to spray a controlled volume of either talc or silica. Mixture of talc and silica is not allowed. The typical powder particle typical size can range from 0,10.1 µm to 100 µm.

The powder distribution system shall spray on the test surface near the test tyre contact patch so that abrasion fragments do not adhere to the tyre or test drum surface. Powder The powder distribution system and materials shall be identical for both reference tyre and candidate tyre during a test and shall remain constant during the entire test. The powder delivery rate (measured by mass per time) applied to the reference and candidate tyres shall be the same, with a maximum ±[2] per cent difference for each test [and be in the range of 40 g/h to 200 g/h for each test tyre].

[The powder delivery rate for each test cycle shall be controlled within ±5 per cent of the average powder delivery rate for each test. This applies to both reference tyre and candidate tyre.]

2.4.4.2.	Nozzle Position

The nozzle position for the powder distribution system shall follow at least one of the following specifications:

(a)	Blow-in type: In a blow-in type system, powder is blown directly to the contact patch through nozzles. In the case of only one nozzle, the centre of the nozzle shall be positioned in symmetrical plane (X = 0). The distance from the nozzles to the centre of the contact patch shall not exceed 35 cm.

In the case of multiple nozzles, they shall be placed parallel to the Y axis and symmetrically distributed respective to the X axis. The distance from the centre of the nozzle array to the centre of the contact patch shall not exceed 35 cm. Nozzles shall be oriented towards contact patch entrance.

(b)	Dispersion type: [In a dispersion-type system, powder is dispersed in the space around the tyre and the drum, but not blown directly onto the contact patch. The contact patch and the tyre are covered with the enclosure coverings, where powder should be evenly dispersed within the enclosure, so that the powder could can be distributed to the contact patch. The nozzle(s) should be placed parallel to the Y axis and symmetrically distributed respective to the X axis.]

2.4.5.	Load, Alignment, Control and Instrumental Accuracies

Measurement of these the parameters listed in Appendix 4 of this Annex shall be sufficiently accurate and precise to provide the required test data. The specific and respective values are provided in Appendix 4 of this Annex.

2.4.6.	Mass Scale

The mass scale for test tyres shall have:

(a)	A mass capacity being able to weigh test tyres;

(b)	An accuracy within ±2 g.

The mass scale shall be duly calibrated following the requirements defined by the manufacturer.

2.4.7.	Reference tyre

For the evaluation of the abrasion performance of the candidate tyre using the indoor drum method, the reference tyre shall be selected according to the following table:

|  |  | Reference tyre |  |
| --- | --- | --- | --- |
| Category of use of the candidate tyre |  | SRTT17S | SRTT17W |
| Normal tyre |  | X |  |
| Snow tyre |  |  | X |
|  | Snow tyre that is classified as tyre for use in severe snow conditions |  | X |
| Special use tyre |  | X |  |
|  | "M+S" or "M.S" or "M&S" |  | X |
|  | Special use tyre that is classified as tyre for use in severe snow conditions |  | X |

Reference tyres SRTT17S and SRTT17W shall be stored in conditions recommended in ASTM F3676-23 and ASTM F3675-23 respectively.

2.5.		Test Conditions

2.5.1.	General

The test consists of a measurement of tyre mass loss in which the tyre is inflated to the cold pressure as specified in Paragraph paragraph 2.5.3. of this Annex and the inflation pressure shall be allowed to build up (i.e. "capped inflation") and not be regulated by machine.

2.5.2.	Test Load

The standard test load Fz on the tyre to be measured shall be calculated from its LI load, corresponding to the maximum mass associated with the LI of the tyre.

The standard test load shall be computed from the values shown in Table 1the table in paragraph 2.5.3.  and shall be kept within the tolerance specified in Appendix 4 of this Annex.

2.5.3.	Tyre Inflation Pressure

The inflation pressure shall be set in accordance with that shown in Table 1the table below with the accuracy specified in Appendix 4 of this Annex and shall be capped.

Table 1

Test loads and inflation pressures

| Tyre type | Tyre of class C1 a |  |
| --- | --- | --- |
|  | Standard load or light load | Reinforced or extra load |
| Load -% of maximum load capacity | 80 | 80 |
| Inflation pressure b a (kPa) | 210 | 250 |
| a For those class C1 tyres belonging to categories which are not shown in ISO 4000-1:2024, Annex B, the inflation pressure shall be the inflation pressure recommended by the tyre manufacturer, corresponding to the maximum tyre load capacity, reduced by 30 kPa. b a The inflation pressure shall be capped with the accuracy specified in Appendix 4 of this Annex. |  |  |

2.5.4.	Testing Conditions (Longitudinal force, lateral force, test speed, running distance)

The following testing conditions shall be met for a test to be considered valid:

(a)	Longitudinal force and lateral force shall be computed from the values shown in Appendix 3 of this Annex. Speed shall be in accordance with that shown in Appendix 3 of this Annex;

(b)	The total running distance of the test shall be 5044 km. The total distance of an actual test shall not differ more than ± 5 per cent from the total input distance;

(c)	The reference tyre shall be mounted on a rim of rim width code 7.5 Rim width code rim. New candidate tyres shall be mounted on any rim requested and approved by the tyre manufacturer;

(d)	The rim width code of the candidate tyre shall be recorded.

(e)	Tyres with special fitment requirements, such as asymmetric or directional design, shall also be mounted in accordance with these requirements: direction of rotation shall be respected;

(ef)	The test shall be performed at null a camber angle of 0°.

2.6.	Test Procedure

2.6.1.	General

The test procedure steps described below shall be followed in the given sequence:

(a)	Both reference and candidate tyres shall be new when starting the test;

(b)	Test tyres with a specified direction of rotation shall be rolling in the forward direction;

(c)	The direction of rolling shall be kept the same throughout the test;

(d)	The abrasion level calculation shall use the actual test run distance.

2.6.2.	Thermal Conditioning

Inflate tThe tyre shall be inflated to not lower than the test pressure and place itplaced in the thermal environment of the test location [between 17 °C and 28 °C] for a minimum of 3 h.

2.6.3.	Pressure Adjustment

After thermal conditioning, the inflation pressure shall be adjusted to the test pressure.

2.6.4.	Thermal Environment

During the test, the ambient temperature shall be kept at [25  °C ± 3 3 °C for normal tyre and 20 20 °C ± 3 3 °C for snow tyre. ]The ambient temperature shall be measured at a distance of not less than 0.15 m and not more than 1 m from the tyre.

The average ambient temperature for reference and candidate tyres during testing shall not differ by more than 2 °C.

2.6.5.	Mass Measurement

The mass of tyre shall be measured before and after 5044 km of run as defined in paragraph 2.6.6. of this Annex for both reference and candidate tyres.

2.6.6.	Test Cycle

2.6.6.1.	Input Condition

Both reference tyre and candidate tyre shall be tested according to the input conditions of Appendix 3 of this Annex. The Appendix 3 test condition of approximately 250 km is defined as one test cycle, including speed transitions, and the test cycle shall be repeated performed 20 times until 5044 km is reached.

2.6.6.2.	Default Test Program (2 positions drum)

Both the reference tyre and the candidate tyre shall be mounted at different positions of one on the same drum. Testing of both reference tyre and candidate tyre shall be conducted at the same time.

Tyres mounted at the two positions shall be exchanged once after the completion of 2500 km. The direction of rotation shall remain constant throughout the test.

A visual inspection of the tyres is recommended after the completion of 2500 km to ensure no tread chunking.

2.6.6.3.	Alternative Test Program (1 position drum)

In the case that testing of reference tyre and candidate tyre is not possible at the same time, the alternative test program may be followed. The following test order for the Reference tyre (R) and Candidate tyre (T) shall be followed:

R (1000 km) – T (2000 km) – R (2000 km) – T (2000 km) – R (2000 km) – T (1000 km)

Repeat aA set of Appendix 3 of this Annex input conditions shall be performed 4 times for 1000 km and 8 times for 2000 km.

A visual inspection of the tyres is recommended around the completion of 2,500 km to ensure no tread chunking.

2.6.6.4.	Test Starting Phase

(a)	The tyres shall touch the drum with a speed that equals 0 km/h.

(b)	 Then theThe test load Fz shall then be applied at a speed equal to 0 km/h or at very low speed.

(c)	 After load application, the speed can be increased to the initial test value 60 km/h with a maximum longitudinal acceleration of 0.125 m/s2 or maximum travelled distance of 3.5 km.

This starting phase shall be free rolling conditions. The distance run during the starting phase shall not be counted.

2.6.7.	Measurement and Recording

Table 2 summarizes the items that shall be measured and recorded:

Table 2

Parameters to The following parameters shall be measured and recorded over the drum test:

| Item | Requirements |
| --- | --- |
| (a) Test speed | Sampling frequency ≥ 1Hz |
| (b) Tyre normal force to the drum surface | Sampling frequency ≥ 1Hz |
| (c) Test inflation pressure | Shall measurebe measured： - Before starting the test; - 3 or more hours after end of the test. Interim measurement during test is optional |
| (d) Ambient temperature measured in ℃, tamb | Sampling frequency ≥ 1Hz |
| (e) Lateral force applied to the test tyre during the test | Sampling frequency ≥ 10 Hz |
| (f) Longitudinal force or torque applied to the test tyre during the test | Sampling frequency ≥ 10 Hz |
| (g) Mass of tyre | Shall be measuredmeasure： - Before starting the test; - after After end of the test. Interim measurement during test is optional |
| (h) MPD and micro-roughness of the test surface; | Shall be measuredmeasure at the timings of： - Before starting the test; - after After end of the test. Interim measurement during test is optional |
| (i) Photograph of tyres after test program | Photograph of tyres after test to record the surface as a proof of compliant test completion in the right way. |
| (j) Drum diameter | Shall be calculated as the drum circumference measured including the thickness of the test surface segments divided by 3.1416. |

During the measurement of the force or torque applied to the test tyre, a moving average over one-wheel revolution may be used to eliminate first and/or second harmonic of the tyre.

During the measurement of the force or torque applied to the test tyre, a low pass filter may be used to eliminate first and/or second harmonic of the tyre.

2.7.	Validation

After a tyre has been subjected to the test procedure specified in paragraph 2.6. of this Annex using a test rim and a valve that undergo no permanent deformation and allow no loss of air, there shall be no visual evidence of tread, sidewall, ply, cord, inner liner, belt or bead separation, chunking, open splices, cracking, broken cords, or rubber adhesion.

A visual inspection of reference tyres shall show no damage on to reference tyres. If a reference tyre loses more than a total of 1 cm2 of tread area (due to chunking or other mechanism), the tyre shall be considered as destroyed and the test shall be considered as invalid.

The following values measured from each parameter shall be with within their tolerances as specified in Table 3the table below. Otherwise, the test results shall be rejected.

Table 3

Validation of Fx, Fy, Fz

| Parameter | Measurement | Value to be verified | Tolerance |
| --- | --- | --- | --- |
| Fx | RMS (a) of G(x) | RMSGx = 0.059 | ±5 % |
| Fy | RMS (a) of G(y) | RMSGy = 0.074 | ±5 % |
| Fx and Fy | RMS (a) of G(x, y) | RMSGxy = 0.095 | ±5 % |
| Fz | RMS of Fz | Prescribed test load RFz | ±50 N or ±1 %, whichever is greater |

Note 1 to entry: (a) For the whole test of total 5044 km, calculate the Root Mean Square (RMS) of G(x), G(y) and Fz using the equations below: RMSGx= 1N × i=1NFxiFzi2 RMSGy= 1N × i=1NFyiFzi2 RMSGxy= 1N × i=1NFxi2+Fyi2Average of Fz2 RMSFz=1N×i=1N (Fzi)2 Where; i is the number of data acquired at a sampling frequency of 10 Hz; N is the total number of data acquired; Fx, Fy and My may be filtered by a low pass filter to remove noise of the output; Fz shall be filtered as the moving average per one second.

2.8.	Processing of Measurement Results

The calculation method for the abrasion index and the abrasion level shall follow the equations:

MlT = MTb – MTa

MlR = MRb - MRa

Where:

MlT	is the mass loss of the candidate tyre, in grams

MlR	is the mass loss of the reference tyre, in grams

MTb	is the mass of the candidate tyre before test cycle, in grams

MTa	is the mass of the candidate tyre after test cycle, in grams

MRb	is the mass of the reference tyre before test cycle, in grams

MRa	is the mass of the reference tyre after test cycle, in grams

The abrasion index (AICT) shall be calculated according to the following equation:

AICT=ALCTALRT-Amargin

Where:

[ALCT	is the abrasion level (mg/(km∙t)) of  the candidate tyre,

ALCT = 1000 (mg/kg) ×x MlT (g)/DT(km)/Fz,T(t)

ALRT	is the abrasion level (mg/(km∙t)) of the reference tyre,

ALRT = 1000 (mg/kg) ×x MlR (g)/DR(km)/Fz,R(t)]

DT	is the testing mileage of candidate tyre (km)

DR	is the testing mileage of the reference tyre (km)

Fz,T	is the test load of the candidate tyre (t)

Fz,R	is the test load of the reference tyre (t)

Amargin	is the dimensionless abrasion margin, which accounts for the complexities of the abrasion level measurement methods, and is equal to [0.20].the special abrasion characteristics of specific tyre groups as indicated below. Amargin is defined in two steps that account for the evolution of abrasion level measurement methods and tyre technology, as given in the following table:

|  |  | Stage 1 | Stage 2 |
| --- | --- | --- | --- |
| Amargin by tyre category of use |  |  |  |
| Normal |  | [0.20] | [0.10] |
| Snow |  | [0.20] | [0.10] |
| Special use |  | [Not defined] | [Not defined] |
| Amargin allowances for specific tyre groups (to be incorporated into Amargin) |  |  |  |
| Tyre for use in severe snow conditions (3PMSF) |  | [+0.10] | [+0.10] |
| Reinforced or extra load tyre (XL) |  | [+0.10] | [-] |
| Tyres with a nominal aspect ratio ≤ 40 and suitable for speeds ≥ 300 km/h |  | [+0.10] | [-] |
| Tyres with low load index (LI < 77) |  | [+0.10] | [-] |

Note: Amargin allowances for specific tyre groups are cumulative. E.g., if a candidate tyre is a reinforced snow tyre for use in severe snow conditions, the applicable` Amargin shall be 0.2 + 0.1 + 0.1 = 0.4. The reference tyre used to calculate the abrasion index shall be one of the tyres described in paragraph 2.4.7. of this Annex.

2.9.	Test report

2.9.1.	The test report shall include the following information:

(a)	Test machine identification;

(b)	Drum diameter (m);

(c)	Test cycle (2 positions /1 position);

(d)	3rd body (talc / silica);

(e)	MPD of test surface (mm): Beginning of test / End of test;

(f)	Tyre class;

(g)	Brand;

(h)	Pattern/trade description;

(i)	Tyre size designation;

(j)	Service description;

(k)	Test load (N);

(l)	Test inflation pressure (kPa);

(m)	Tyre identification;

(n)	3PMSF marking (Y/N);

(o)	M+S marking (Y/N);

(p)	Rim width code;

(q)	Inflation pressure (kPa): Beginning of test / End of test;

(r)	Mass of tyre (g): Before test / After test;

(s)	Test distance (km);

(t)	Abrasion rate (mg/km);

(u)	Abrasion level (mg/(km∙t));

(v)	Abrasion index (only applicable for candidate tyre);

(w)	Average ambient temperature (°C);

(x)	RMS of G(x);

(y)	RMS of G(y);

(z)	Average of Fz;

(aa)	Amargin applied, with indication of any allowances incorporated therein.

Annex 3 – Appendix 1

Accelerations calculation

1.	Input for calculation

1.1.	Required GNSS parameters

The signals shall be derived by evaluation of GNSS (Global Navigation Satellite System as defined by ISO 24245:2023) signals sampled at a frequency of 10 Hz or more.

The following parameters are required for the calculation of longitudinal and lateral acceleration:

(a)	The vehicle velocity speed (v), [m/s]

(b)	The heading (ψ) of the vehicle [rad]longitudinal acceleration (alongitudinal), [m/s²]

(c)	The number of satellites (nsat)lateral acceleration (alateral), [m/s²]

(d)	The sampling frequency (fsample), [Hz]

The accelerations are derived by evaluation of GNSS (GLOBAL NAVIGATION SATELLITE SYSTEMGlobal Navigation Satellite System as defined by ISO 24245:2023) signals. The recommended sampling rate is shall be 10 Hz or more. Otherwise, the below described filtering process is not working.

Filter for measured Valuesvalues

1.2.	Calculation sequenceSpeed jumps detection and correction

Before starting the filtering process, all measured values are shall be checked concerning speed jumps. Speed jumps refer to measurements which are not plausible. For identifying speed jumps, that refer to implausible measurements, the velocity is shall be filtered by using a Butterworth filter with a cut-off frequency of 1 Hz. A threshold of 9 m/s² for the maximum longitudinal acceleration is defined. That means, that a speed change of maximum 9 m/s² ∙ tsampling is still plausible.

A speed jump will be detected if there is a speed difference of 2 ∙ Δspeedpossible. In the case of a detected speed jump, the relevant raw speed values will be replaced by a linear interpolated value.

The sequence for calculating the distance‑based acceleration data used for the determination of the standard deviations of the lateral and longitudinal accelerations shall be as follows:

- Calculate the longitudinal and lateral accelerations at each sampling step.

- Pre-filter the calculated values to exclude data points with excessive acceleration, excessive vehicle speed, or an insufficient number of satellites.

- Apply a low‑pass filter to the acceleration data and the vehicle speed.

- Smooth the remaining values using a moving‑average filter.

- Exclude acceleration data points measured at vehicle speeds below the specified minimum speed.

- Calculate the standard deviations for the longitudinal and lateral accelerations

1.3.	Calculation of accelerations

Definitions:

i	is the discrete time index,

N	is the total number of individual measurement samples used for the calculation,

alongitudinal,i	is the longitudinal acceleration at time step i,

alateral,i	is the lateral acceleration at time step i,

vi	is the vehicle speed at time step i,

nsat,i	is the number of positioning satellites used for the calculation at time step i,

Δψi	is the heading change at time step i,

fₛₐₘₚₗₑ	is the sampling frequency.

Formulas:

The longitudinal component is calculated from the discrete vehicle-speed increments multiplied by the sampling frequency:

alongitudinal,i=Δvi∙fsample

with

Δvi=vi-vi-1  for  i>1  and  Δv1=0

The lateral component is derived from the vehicle speed and the heading change, considering the trigonometric contribution of the heading change over the sampling interval:

alateral,i=vi∙sinΔψi∙fsample

with

Δψi=ψi-ψi-1  for  i>1  and  Δψ1=0

1.4	Pre-filtering

For each discrete time step i, the recorded data shall satisfy all the following requirements:

- Resultant acceleration limit

alongitudinal,i2+alateral,i2≤10 m/s2

- Vehicle speed limit

vi≤250 km/h

(c)	Satellite availability requirement

nsat,i≥5

Otherwise, the data point at time step i shall be discarded.

1.5.1.3.	Butterworth Filter: filter

For filtering the measurement, The data shall be filtered using a second-order Butterworth filter second order with a cut-off frequency of 1 Hz. is shall be used. After this filter the values are smoothed with a "moving average" over 1 second for longitudinal acceleration, all other values over 2 seconds.

All accelerations measured for a velocity less than 7 km/h are excluded.

Python code:

from scipy.signal import butter, filtfilt

# Filter settings: n_order is the order of the filter,

# A scalar or length-2 sequence giving the critical frequencies

N = 2

Wn = 1 / (0.5 * round(data_inp['freq_sample'].mean()))

[b,a] = butter(N, Wn, 'lowpass')

data_filt = filtfilt(b, a, data_inp, padtype = 'odd')

data_inp = data_inp[(data_inp.speed_filt > speed_min)]

1.6.1.4.	Smoothing (moving average)Moving Average:

The acceleration and vehicle speed signals shall be filtered using a moving‑average filter.

For the longitudinal acceleration, a moving‑average window of 1 second shall be used.

For the lateral acceleration and the vehicle speed, a moving‑average window of 2 seconds shall be applied.

Python code:

# averaging over 1 second window

# make sure window is odd

if round(data_inp['freq_sample'].mean()) % 2 != 0:

win = int(round(data_inp['freq_sample'].mean()))

else:

win = int(round(data_inp['freq_sample'].mean()) - 1)

data_inp['accx_filt_movg'] = data_inp['accx_filt'].rolling(window=win,,

center=True, min_periods=1).mean()

# averaging over 2 second window

win = int(round(data_inp['freq_sample'].mean()) * 2 - 1)

data_inp['accy_filt_movg'] = data_inp['accy_filt'].rolling(window=win,

center=True, min_periods=1).mean()

data_inp['speed_filt_movg'] = data_inp['speed_filt'].rolling(window=win,

center=True, min_periods=1).mean()

1.7.	All accelerations measured for a (smoothed) vehicle speed less than 7 km/h shall be excluded.

1.8.1.5.	Distance-based Standard Deviation standard deviation

Calculated accelerations (sampled with a constant frequency) shall beare transferred in into distance-based values: values (one value per metermetre). For this, a distance vector shall be calculated by using smoothed vehicle-speed values and sampling frequencysimple interpolation is used. A linear interpolation shall be used to calculate longitudinal and lateral acceleration values per metre distance.

With these accelerations, the standard deviation can be calculated with the following well-known formulas:

Slongitudinal= 1N-1∙i=1Nalongitudinal,i-μlongitudinal2

σlongitudinal= 1N-1∙i=1Nalongitudinal,i-alongitudinal2

with μlongitudinal=1Ni=1Nalongitudinal,i alongitudinal=1Ni=1Nalongitudinal,i

Slateral= 1N-1∙i=1Nalateral,i-μlateral2

σlateral= 1N-1∙i=1Nalateral,i-alateral2

With  μlateral=1Ni=1Nalateral,i with alateral=1Ni=1Nalateral,i

Python code:

from scipy import interpolate
import numpy as np
data_inp['distance'] = data_inp['speed']/data_inp['fsample']
data_inp['distance'] = data_inp['distance'].cumsum()

# interpolate function for acc
f_accx = interpolate.interp1d(data_inp['distance'], data_inp['accx'])
f_accy = interpolate.interp1d(data_inp['distance'], data_inp['accy'])

# generate array of distance every 1m 
distance_1m = np.array(data_inp['distance'].iloc[0], data_inp['distance'].iloc[-1], 1)

# create an interpolation every 1m for the accelarations - numpy array result
# len of the array same as len of distance_1m
accx_DB = f_accx(distance_1m)
accy_DB = f_accy(distance_1m)

# compute the stadx with ceiling accx_DB and accy_DB
stdax = np.std(accx_DB)
stday = np.std(accy_DB)

2.	Calculation of distribution of driving styles

The following steps shall be followed for the calculation of the distribution of driving styles:

- Cumulate the acceleration data of a vehicle on the circuit for the total test distance of (8 000 ± 300) km; of acceleration data on a vehicle at the circuit. The accelerations data obtained while qualifying the circuit for abrasiveness may be used. The provisions defined for vehicle speed in paragraph 1.6.13.2. (d) of this Annex shall be respected in every single convoy test;

(b)	Split the data in segments of 20 km;

(b)(c)	Compute the longitudinal and lateral acceleration standard deviations (Slongitudinal ; Slateralσlongitudinal and σlateral) for each complete 20-km segment, starting with the segment from 0 to 20 km and progressing in 1-km increments, over the nominal test distance of (8,000 ± 300) km, using following the method described in paragraph 1. of this appendix Appendix;

(c)(d)	For each segment with (Slongitudinal ; Slateral) σlongitudinal and σlateral data, count which road driving style (as defined in paragraph 1.6.13.2. of this Annex) it belongs to. This shall be done by attributing it to urban-like driving style if matching the corresponding criteria, to highway-like driving style if matching the corresponding criteria, or to regional-like driving style if not matching the urban-like or highway-like driving style;

(d)(e)	The share of points in one road driving style equals to the number of points in the road driving style divided by the total number of segments. This share shall respect the shares defined in paragraph 1.6.13.2. of this Annex.

Annex 3 – Appendix 2

Test report example for vehicle test method on public open roads

1.	Type Approval Authority or Technical Service:

2.	Name and address of manufacturer:

3.	Test report No.:

4.	Brand name and trade description:

5.	Tyre Class: C1

6.	Category of use:

6.1.	M+S marking in the case of special use tyres (Yes/No)

6.2.	3PMSF marking (Yes/No)8

6.3.	XL marking (Yes/No)8

6.4.	Tyres with a nominal aspect ratio ≤ 40 and suitable for speeds ≥ 300 km/h (Yes/No)8

6.5.	[Tyres with low load index (LI < 77) (Yes/No)8]

7.	Comments (if any):

8.	Date:

9.	Signature:

| Tyre abrasion rate test report |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test conditions: |  |  |  |  |  |  |  |  |
| Test Starting date: |  |  |  | Test end date: |  |  |  |  |
| Test temperatures: (degree C) |  |  |  |  |  |  |  |  |
| Average: |  |  | Minimum: |  |  | Maximum: |  |  |  |  |
| Percentage of distance covered on wet road: |  |  |  | (add snow) |  |  |  |  |
| Circuit used for test: |  |  |  |  |  |  |  |  |
| Reference: |  |  |  | Location: |  |  |  |  |
| Nominal circuit length: |  |  |  | Total distance covered: |  |  |  |  |
| Highway-like driving distance: |  |  |  | Regional-like style distance: |  |  |  |  |
| Urban-like driving style distance: |  |  |  | Total deviation distance: |  |  |  |  |
| Slope of reference tyre sensitivity to temperature: |  |  |  |  |  |  |  |  |

|  | Candidate tyre/vehicle |  |  |  | Reference tyre/vehicle |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Left | Front right | Rear left | Rear right | Front Left |  | Front right | Rear left | Rear right |
| Vehicle information: |  |  |  |  |  |  |  |  |  |
| Vehicle model: |  |  |  |  |  |  |  |  |  |
| Standard deviation X acceleration: |  |  |  |  |  |  |  |  |  |
| % of distance covered under the maximum longitudinal acceleration: |  |  |  |  |  |  |  |  |  |
| Standard deviation Y acceleration: |  |  |  |  |  |  |  |  |  |
| % of distance covered under the maximum lateral acceleration: |  |  |  |  |  |  |  |  |  |
| Toe at test start: |  |  |  |  |  |  |  |  |  |
| Camber at test start: |  |  |  |  |  |  |  |  |  |
| Toe at test end: |  |  |  |  |  |  |  |  |  |
| Camber at test end: |  |  |  |  |  |  |  |  |  |
| Load per position: |  |  |  |  |  |  |  |  |  |
| Tyre information: |  |  |  |  |  |  |  |  |  |
| Tyre brand: |  |  |  |  |  |  |  |  |  |
| Tyre pattern: |  |  |  |  |  |  |  |  |  |
| Tyre size designation: |  |  |  |  |  |  |  |  |  |
| Tyre load index: |  |  |  |  |  |  |  |  |  |
| Tyre speed index: |  |  |  |  |  |  |  |  |  |
| Serial number (if available): |  |  |  |  |  |  |  |  |  |
| Cold inflation pressure (fitment): |  |  |  |  |  |  |  |  |  |
| Cold inflation pressure 50% test: |  |  |  |  |  |  |  |  |  |
| Cold inflation pressure (test end): |  |  |  |  |  |  |  |  |  |
| Balancing mass (test beginning): |  |  |  |  |  |  |  |  |  |
| Balancing mass (test end): |  |  |  |  |  |  |  |  |  |
| Rim width code: |  |  |  |  |  | 7.5 | 7.5 | 7.5 | 7.5 |
| Initial tyre mass: |  |  |  |  |  |  |  |  |  |
| Final tyre mass: |  |  |  |  |  |  |  |  |  |
| Distance run by each tyre: |  |  |  |  |  |  |  |  |  |
| Reference tyre abrasion level in mg/(km∙t) normalized at 20°C (or 10°C): |  |  |  |  |  |  |  |  |  |
| Tyres visual inspection report: |  |  |  |  |  |  |  |  |  |
| Test results: |  |  |  |  |  |  |  |  |  |
| Tyre Abrasion level in mg/(km∙t): |  |  |  |  |  |  |  |  |  |
| Amargin applied: |  |  |  |  |  |  |  |  |  |  |
| Tyre Abrasion index (AICT): |  |  |  |  |  | N/A |  |  |  |

Annex 3 – Appendix 3

Input of test cycle

In order to calculate the input forces Fx and Fy, longitudinal and lateral acceleration indices, as G(x) and G(y) respectively, are introduced as below.

For torque control testing machine, tyre torque (My) is calculated with longitudinal force (Fx) and loaded radius (RL) following the equations provided below:

Fx ＝ Fz  ×  G(x)  or My = Test load(Fz) ×G(x) × RL

Fy ＝ Fz  ×  G(y)

Fz is the test load defined in paragraphs 2.2.6.2.2.5. and 2.5.2. of this Annex.

G(x) and G(y) represent the index compared to the standard acceleration due to earth gravity (g= 9.80665 m/s2). Alternatively, the local earth gravity may be defined.

The table below Table A1 defines the time, G(x), G(y), and speed of test cycle. In Table T1the table below, T represents the total test duration from the beginning of the test. At a point of test duration T, the values of G(x) and G(y) shall be equal to those listed in the table belowTable A1.

G(x) and G(y) shall change linearly between two adjacent points. Therefore, the values of Fx and Fy will also change linearly from one point to another. The following graphs show samples of linear change for Fx or Fy with respect to T.

T means the driving time from starting test.

The value of G(x) and G(y) at driving time T is mentioned in the table belowTable A1.

G(x) and G(y) between each point changes linearly through those two points.

Graph AFigure .1

Example of Fx , with a test load of 5727N

Graph AFigure .2

Example of Fy, with a test load of 5727N

Table A1.

Input of test cycle

| T | v | G(x) | G(y) |
| --- | --- | --- | --- |
| (s) | (kph) |  |  |
| 0 | 60 | 0.000 | 0 |
| 50 | 100 | 0.000 | 0.000 |
| 373.2 | 100 | 0.025 | 0.005 |
| 388.4 | 100 | 0.025 | 0.185 |
| 418.7 | 100 | 0.055 | -0.175 |
| 446.5 | 100 | 0.085 | 0.155 |
| 471.7 | 100 | 0.115 | -0.115 |
| 491.9 | 100 | 0.145 | 0.125 |
| 509.6 | 100 | 0.175 | -0.085 |
| 522.3 | 100 | 0.205 | 0.065 |
| 532.4 | 100 | 0.235 | -0.025 |
| 540.0 | 100 | 0.265 | 0.035 |
| 545.0 | 100 | 0.295 | 0.005 |
| 547.5 | 100 | 0.325 | 0.005 |
| 556.8 | 100 | 0.025 | 0.005 |
| 574.5 | 100 | 0.025 | -0.205 |
| 602.3 | 100 | -0.005 | 0.155 |
| 620.0 | 100 | -0.035 | -0.085 |
| 632.6 | 100 | -0.065 | 0.065 |
| 645.2 | 100 | -0.095 | -0.055 |
| 657.9 | 100 | -0.125 | 0.065 |
| 662.9 | 100 | -0.155 | 0.005 |
| 668.8 | 100 | 0.025 | 0.005 |
| 678.9 | 100 | 0.025 | 0.125 |
| 699.1 | 100 | 0.055 | -0.115 |
| 719.3 | 100 | 0.085 | 0.095 |
| 737.0 | 100 | 0.115 | -0.085 |
| 747.1 | 100 | 0.145 | 0.065 |
| 757.2 | 100 | 0.175 | -0.025 |
| 764.8 | 100 | 0.205 | 0.035 |
| 769.9 | 100 | 0.235 | 0.005 |
| 774.9 | 100 | 0.265 | 0.035 |
| 777.4 | 100 | 0.295 | 0.005 |
| 785.9 | 100 | 0.025 | 0.005 |
| 796.0 | 100 | 0.025 | -0.115 |
| 816.2 | 100 | -0.005 | 0.125 |
| 826.3 | 100 | -0.035 | -0.025 |
| 833.9 | 100 | -0.065 | 0.035 |
| 841.4 | 100 | -0.095 | -0.025 |
| 849.0 | 100 | -0.125 | 0.035 |
| 851.6 | 100 | -0.155 | 0.005 |
| 857.4 | 100 | 0.025 | 0.005 |
| 865.0 | 100 | 0.025 | 0.095 |
| 885.2 | 100 | 0.055 | -0.115 |
| 902.9 | 100 | 0.085 | 0.095 |
| 915.6 | 100 | 0.115 | -0.055 |
| 923.1 | 100 | 0.145 | 0.035 |
| 930.7 | 100 | 0.175 | -0.025 |
| 935.8 | 100 | 0.205 | 0.035 |
| 940.8 | 100 | 0.235 | 0.005 |
| 945.9 | 100 | 0.265 | 0.035 |
| 953.5 | 100 | 0.025 | 0.005 |
| 961.0 | 100 | 0.025 | -0.085 |
| 978.7 | 100 | -0.005 | 0.095 |
| 986.3 | 100 | -0.035 | -0.025 |
| 991.3 | 100 | -0.065 | 0.035 |
| 996.4 | 100 | -0.095 | 0.005 |
| 1001.5 | 100 | -0.125 | 0.035 |
| 1006.5 | 100 | 0.025 | 0.005 |
| 1014.1 | 100 | 0.025 | 0.095 |
| 1031.8 | 100 | 0.055 | -0.085 |
| 1049.5 | 100 | 0.085 | 0.095 |
| 1059.6 | 100 | 0.115 | -0.025 |
| 1067.1 | 100 | 0.145 | 0.035 |
| 1072.2 | 100 | 0.175 | 0.005 |
| 1077.2 | 100 | 0.205 | 0.035 |
| 1082.3 | 100 | 0.235 | 0.005 |
| 1084.8 | 100 | 0.265 | 0.005 |
| 1092.4 | 100 | 0.025 | 0.005 |
| 1100.0 | 100 | 0.025 | -0.085 |
| 1112.6 | 100 | -0.005 | 0.065 |
| 1120.2 | 100 | -0.035 | -0.025 |
| 1125.3 | 100 | -0.065 | 0.035 |
| 1130.3 | 100 | -0.095 | 0.005 |
| 1135.4 | 100 | -0.125 | 0.035 |
| 1140.4 | 100 | 0.025 | 0.005 |
| 1148.0 | 100 | 0.025 | 0.095 |
| 1165.7 | 100 | 0.055 | -0.085 |
| 1180.8 | 100 | 0.085 | 0.065 |
| 1188.4 | 100 | 0.115 | -0.025 |
| 1193.5 | 100 | 0.145 | 0.035 |
| 1198.5 | 100 | 0.175 | 0.005 |
| 1203.6 | 100 | 0.205 | 0.035 |
| 1206.1 | 100 | 0.235 | 0.005 |
| 1208.6 | 100 | 0.265 | 0.005 |
| 1216.2 | 100 | 0.025 | 0.005 |
| 1223.8 | 100 | 0.025 | -0.085 |
| 1236.4 | 100 | -0.005 | 0.065 |
| 1241.5 | 100 | -0.035 | 0.005 |
| 1246.5 | 100 | -0.065 | 0.035 |
| 1251.6 | 100 | -0.095 | 0.005 |
| 1256.6 | 100 | -0.125 | 0.035 |
| 1261.7 | 100 | 0.025 | 0.005 |
| 1269.3 | 100 | 0.025 | 0.095 |
| 1284.4 | 100 | 0.055 | -0.085 |
| 1299.6 | 100 | 0.085 | 0.065 |
| 1307.2 | 100 | 0.115 | -0.025 |
| 1312.2 | 100 | 0.145 | 0.035 |
| 1317.3 | 100 | 0.175 | 0.005 |
| 1322.3 | 100 | 0.205 | 0.035 |
| 1324.8 | 100 | 0.235 | 0.005 |
| 1331.6 | 100 | 0.025 | 0.005 |
| 1339.2 | 100 | 0.025 | -0.085 |
| 1351.8 | 100 | -0.005 | 0.065 |
| 1356.8 | 100 | -0.035 | 0.005 |
| 1361.9 | 100 | -0.065 | 0.035 |
| 1366.9 | 100 | -0.095 | 0.005 |
| 1372.0 | 100 | -0.125 | 0.035 |
| 1377.1 | 100 | 0.025 | 0.005 |
| 1382.1 | 100 | 0.025 | 0.065 |
| 1397.3 | 100 | 0.055 | -0.085 |
| 1409.9 | 100 | 0.085 | 0.065 |
| 1417.5 | 100 | 0.115 | -0.025 |
| 1422.5 | 100 | 0.145 | 0.035 |
| 1427.6 | 100 | 0.175 | 0.005 |
| 1432.6 | 100 | 0.205 | 0.035 |
| 1435.2 | 100 | 0.235 | 0.005 |
| 1441.9 | 100 | 0.025 | 0.005 |
| 1447.0 | 100 | 0.025 | -0.055 |
| 1457.1 | 100 | -0.005 | 0.065 |
| 1462.1 | 100 | -0.035 | 0.005 |
| 1467.2 | 100 | -0.065 | 0.035 |
| 1472.2 | 100 | -0.095 | 0.005 |
| 1477.3 | 100 | -0.125 | 0.035 |
| 1482.3 | 100 | 0.025 | 0.005 |
| 1487.4 | 100 | 0.025 | 0.065 |
| 1502.5 | 100 | 0.055 | -0.085 |
| 1515.2 | 100 | 0.085 | 0.065 |
| 1522.7 | 100 | 0.115 | -0.025 |
| 1527.8 | 100 | 0.145 | 0.035 |
| 1532.9 | 100 | 0.175 | 0.005 |
| 1535.4 | 100 | 0.205 | 0.005 |
| 1541.3 | 100 | 0.025 | 0.005 |
| 1546.3 | 100 | 0.025 | -0.055 |
| 1556.4 | 100 | -0.005 | 0.065 |
| 1561.5 | 100 | -0.035 | 0.005 |
| 1566.5 | 100 | -0.065 | 0.035 |
| 1571.6 | 100 | -0.095 | 0.005 |
| 1576.6 | 100 | -0.125 | 0.035 |
| 1581.7 | 100 | 0.025 | 0.005 |
| 1586.8 | 100 | 0.025 | 0.065 |
| 1601.9 | 100 | 0.055 | -0.085 |
| 1614.5 | 100 | 0.085 | 0.065 |
| 1619.6 | 100 | 0.115 | 0.005 |
| 1624.6 | 100 | 0.145 | 0.035 |
| 1629.7 | 100 | 0.175 | 0.005 |
| 1632.2 | 100 | 0.205 | 0.005 |
| 1638.1 | 100 | 0.025 | 0.005 |
| 1643.2 | 100 | 0.025 | -0.055 |
| 1650.8 | 100 | -0.005 | 0.035 |
| 1655.8 | 100 | -0.035 | 0.005 |
| 1660.9 | 100 | -0.065 | 0.035 |
| 1663.4 | 100 | -0.095 | 0.005 |
| 1665.9 | 100 | -0.125 | 0.005 |
| 1671.0 | 100 | 0.025 | 0.005 |
| 1676.0 | 100 | 0.025 | 0.065 |
| 1691.2 | 100 | 0.055 | -0.085 |
| 1703.8 | 100 | 0.085 | 0.065 |
| 1708.9 | 100 | 0.115 | 0.005 |
| 1713.9 | 100 | 0.145 | 0.035 |
| 1719.0 | 100 | 0.175 | 0.005 |
| 1721.5 | 100 | 0.205 | 0.005 |
| 1727.4 | 100 | 0.025 | 0.005 |
| 1732.4 | 100 | 0.025 | -0.055 |
| 1740.0 | 100 | -0.005 | 0.035 |
| 1745.1 | 100 | -0.035 | 0.005 |
| 1747.6 | 100 | -0.065 | 0.005 |
| 1750.1 | 100 | -0.095 | 0.005 |
| 1752.7 | 100 | -0.125 | 0.005 |
| 1757.7 | 100 | 0.025 | 0.005 |
| 1762.8 | 100 | 0.025 | 0.065 |
| 1775.4 | 100 | 0.055 | -0.055 |
| 1788.0 | 100 | 0.085 | 0.065 |
| 1793.1 | 100 | 0.115 | 0.005 |
| 1798.1 | 100 | 0.145 | 0.035 |
| 1800.7 | 100 | 0.175 | 0.005 |
| 1803.2 | 100 | 0.205 | 0.005 |
| 1809.1 | 100 | 0.025 | 0.005 |
| 1814.1 | 100 | 0.025 | -0.055 |
| 1821.7 | 100 | -0.005 | 0.035 |
| 1826.8 | 100 | -0.035 | 0.005 |
| 1829.3 | 100 | -0.065 | 0.005 |
| 1831.8 | 100 | -0.095 | 0.005 |
| 1834.3 | 100 | -0.125 | 0.005 |
| 1839.4 | 100 | 0.025 | 0.005 |
| 1844.5 | 100 | 0.025 | 0.065 |
| 1857.1 | 100 | 0.055 | -0.055 |
| 1867.2 | 100 | 0.085 | 0.035 |
| 1872.2 | 100 | 0.115 | 0.005 |
| 1877.3 | 100 | 0.145 | 0.035 |
| 1879.8 | 100 | 0.175 | 0.005 |
| 1882.3 | 100 | 0.205 | 0.005 |
| 1888.2 | 100 | 0.025 | 0.005 |
| 1893.3 | 100 | 0.025 | -0.055 |
| 1900.9 | 100 | -0.005 | 0.035 |
| 1905.9 | 100 | -0.035 | 0.005 |
| 1908.5 | 100 | -0.065 | 0.005 |
| 1911.0 | 100 | -0.095 | 0.005 |
| 1913.5 | 100 | -0.125 | 0.005 |
| 1918.6 | 100 | 0.025 | 0.005 |
| 1923.6 | 100 | 0.025 | 0.065 |
| 1936.2 | 100 | 0.055 | -0.055 |
| 1946.4 | 100 | 0.085 | 0.035 |
| 1951.4 | 100 | 0.115 | 0.005 |
| 1956.5 | 100 | 0.145 | 0.035 |
| 1959.0 | 100 | 0.175 | 0.005 |
| 1964.0 | 100 | 0.025 | 0.005 |
| 1969.1 | 100 | 0.025 | -0.055 |
| 1976.7 | 100 | -0.005 | 0.035 |
| 1981.7 | 100 | -0.035 | 0.005 |
| 1984.2 | 100 | -0.065 | 0.005 |
| 1986.8 | 100 | -0.095 | 0.005 |
| 1989.3 | 100 | -0.125 | 0.005 |
| 1994.4 | 100 | 0.025 | 0.005 |
| 1999.4 | 100 | 0.025 | 0.065 |
| 2009.5 | 100 | 0.055 | -0.055 |
| 2019.6 | 100 | 0.085 | 0.035 |
| 2024.7 | 100 | 0.115 | 0.005 |
| 2029.7 | 100 | 0.145 | 0.035 |
| 2032.3 | 100 | 0.175 | 0.005 |
| 2037.3 | 100 | 0.025 | 0.005 |
| 2042.4 | 100 | 0.025 | -0.055 |
| 2049.9 | 100 | -0.005 | 0.035 |
| 2055.0 | 100 | -0.035 | 0.005 |
| 2057.5 | 100 | -0.065 | 0.005 |
| 2060.0 | 100 | -0.095 | 0.005 |
| 2062.6 | 100 | -0.125 | 0.005 |
| 2067.6 | 100 | 0.025 | 0.005 |
| 2070.1 | 100 | 0.025 | 0.035 |
| 2080.3 | 100 | 0.055 | -0.055 |
| 2087.8 | 100 | 0.085 | 0.035 |
| 2092.9 | 100 | 0.115 | 0.005 |
| 2097.9 | 100 | 0.145 | 0.035 |
| 2100.5 | 100 | 0.175 | 0.005 |
| 2105.5 | 100 | 0.025 | 0.005 |
| 2108.0 | 100 | 0.025 | -0.025 |
| 2115.6 | 100 | -0.005 | 0.035 |
| 2120.7 | 100 | -0.035 | 0.005 |
| 2123.2 | 100 | -0.065 | 0.005 |
| 2125.7 | 100 | -0.095 | 0.005 |
| 2129.9 | 100 | 0.025 | 0.005 |
| 2132.5 | 100 | 0.025 | 0.035 |
| 2142.6 | 100 | 0.055 | -0.055 |
| 2150.2 | 100 | 0.085 | 0.035 |
| 2155.2 | 100 | 0.115 | 0.005 |
| 2160.3 | 100 | 0.145 | 0.035 |
| 2162.8 | 100 | 0.175 | 0.005 |
| 2167.8 | 100 | 0.025 | 0.005 |
| 2170.4 | 100 | 0.025 | -0.025 |
| 2177.9 | 100 | -0.005 | 0.035 |
| 2180.5 | 100 | -0.035 | 0.005 |
| 2183.0 | 100 | -0.065 | 0.005 |
| 2186.4 | 100 | 0.025 | 0.005 |
| 2188.9 | 100 | 0.025 | 0.035 |
| 2199.0 | 100 | 0.055 | -0.055 |
| 2206.6 | 100 | 0.085 | 0.035 |
| 2211.6 | 100 | 0.115 | 0.005 |
| 2214.2 | 100 | 0.145 | 0.005 |
| 2218.4 | 100 | 0.025 | 0.005 |
| 2220.9 | 100 | 0.025 | -0.025 |
| 2228.5 | 100 | -0.005 | 0.035 |
| 2231.0 | 100 | -0.035 | 0.005 |
| 2233.5 | 100 | 0.025 | 0.005 |
| 2236.1 | 100 | 0.025 | 0.035 |
| 2246.2 | 100 | 0.055 | -0.055 |
| 2253.7 | 100 | 0.085 | 0.035 |
| 2258.8 | 100 | 0.115 | 0.005 |
| 2261.3 | 100 | 0.145 | 0.005 |
| 2265.5 | 100 | 0.025 | 0.005 |
| 2268.1 | 100 | 0.025 | -0.025 |
| 2275.6 | 100 | -0.005 | 0.035 |
| 2278.2 | 100 | -0.035 | 0.005 |
| 2280.7 | 100 | 0.025 | 0.005 |
| 2283.2 | 100 | 0.025 | 0.035 |
| 2293.3 | 100 | 0.055 | -0.055 |
| 2300.9 | 100 | 0.085 | 0.035 |
| 2306.0 | 100 | 0.115 | 0.005 |
| 2308.5 | 100 | 0.145 | 0.005 |
| 2312.7 | 100 | 0.025 | 0.005 |
| 2315.2 | 100 | 0.025 | -0.025 |
| 2322.8 | 100 | -0.005 | 0.035 |
| 2325.3 | 100 | -0.035 | 0.005 |
| 2327.8 | 100 | 0.025 | 0.005 |
| 2330.4 | 100 | 0.025 | 0.035 |
| 2340.5 | 100 | 0.055 | -0.055 |
| 2348.1 | 100 | 0.085 | 0.035 |
| 2353.1 | 100 | 0.115 | 0.005 |
| 2355.6 | 100 | 0.145 | 0.005 |
| 2359.9 | 100 | 0.025 | 0.005 |
| 2362.4 | 100 | 0.025 | -0.025 |
| 2370.0 | 100 | -0.005 | 0.035 |
| 2372.5 | 100 | -0.035 | 0.005 |
| 2375.0 | 100 | 0.025 | 0.005 |
| 2377.5 | 100 | 0.025 | 0.035 |
| 2385.1 | 100 | 0.055 | -0.025 |
| 2392.7 | 100 | 0.085 | 0.035 |
| 2397.7 | 100 | 0.115 | 0.005 |
| 2400.3 | 100 | 0.145 | 0.005 |
| 2404.5 | 100 | 0.025 | 0.005 |
| 2407.0 | 100 | 0.025 | -0.025 |
| 2412.1 | 100 | -0.005 | 0.035 |
| 2414.6 | 100 | -0.035 | 0.005 |
| 2417.1 | 100 | 0.025 | 0.005 |
| 2419.6 | 100 | 0.025 | 0.035 |
| 2427.2 | 100 | 0.055 | -0.025 |
| 2434.8 | 100 | 0.085 | 0.035 |
| 2439.9 | 100 | 0.115 | 0.005 |
| 2442.4 | 100 | 0.145 | 0.005 |
| 2446.6 | 100 | 0.025 | 0.005 |
| 2449.1 | 100 | 0.025 | -0.025 |
| 2454.2 | 100 | -0.005 | 0.035 |
| 2456.7 | 100 | -0.035 | 0.005 |
| 2459.2 | 100 | 0.025 | 0.005 |
| 2461.8 | 100 | 0.025 | 0.035 |
| 2469.3 | 100 | 0.055 | -0.025 |
| 2476.9 | 100 | 0.085 | 0.035 |
| 2482.0 | 100 | 0.115 | 0.005 |
| 2484.5 | 100 | 0.145 | 0.005 |
| 2488.7 | 100 | 0.025 | 0.005 |
| 2491.2 | 100 | 0.025 | -0.025 |
| 2496.3 | 100 | -0.005 | 0.035 |
| 2498.8 | 100 | -0.035 | 0.005 |
| 2501.3 | 100 | 0.025 | 0.005 |
| 2503.9 | 100 | 0.025 | 0.035 |
| 2511.4 | 100 | 0.055 | -0.025 |
| 2519.0 | 100 | 0.085 | 0.035 |
| 2524.1 | 100 | 0.115 | 0.005 |
| 2526.6 | 100 | 0.145 | 0.005 |
| 2530.8 | 100 | 0.025 | 0.005 |
| 2533.3 | 100 | 0.025 | -0.025 |
| 2538.4 | 100 | -0.005 | 0.035 |
| 2540.9 | 100 | -0.035 | 0.005 |
| 2543.4 | 100 | 0.025 | 0.005 |
| 2546.0 | 100 | 0.025 | 0.035 |
| 2553.5 | 100 | 0.055 | -0.025 |
| 2561.1 | 100 | 0.085 | 0.035 |
| 2563.7 | 100 | 0.115 | 0.005 |
| 2566.2 | 100 | 0.145 | 0.005 |
| 2570.4 | 100 | 0.025 | 0.005 |
| 2572.9 | 100 | 0.025 | -0.025 |
| 2578.0 | 100 | -0.005 | 0.035 |
| 2580.5 | 100 | -0.035 | 0.005 |
| 2583.0 | 100 | 0.025 | 0.005 |
| 2585.5 | 100 | 0.025 | 0.035 |
| 2593.1 | 100 | 0.055 | -0.025 |
| 2600.7 | 100 | 0.085 | 0.035 |
| 2603.2 | 100 | 0.115 | 0.005 |
| 2606.6 | 100 | 0.025 | 0.005 |
| 2609.1 | 100 | 0.025 | -0.025 |
| 2614.2 | 100 | -0.005 | 0.035 |
| 2616.7 | 100 | -0.035 | 0.005 |
| 2619.2 | 100 | 0.025 | 0.005 |
| 2621.8 | 100 | 0.025 | 0.035 |
| 2629.3 | 100 | 0.055 | -0.025 |
| 2636.9 | 100 | 0.085 | 0.035 |
| 2639.4 | 100 | 0.115 | 0.005 |
| 2642.8 | 100 | 0.025 | 0.005 |
| 2645.3 | 100 | 0.025 | -0.025 |
| 2650.4 | 100 | -0.005 | 0.035 |
| 2652.1 | 100 | 0.025 | 0.005 |
| 2654.6 | 100 | 0.025 | 0.035 |
| 2662.2 | 100 | 0.055 | -0.025 |
| 2669.8 | 100 | 0.085 | 0.035 |
| 2672.3 | 100 | 0.115 | 0.005 |
| 2675.7 | 100 | 0.025 | 0.005 |
| 2678.2 | 100 | 0.025 | -0.025 |
| 2683.2 | 100 | -0.005 | 0.035 |
| 2684.9 | 100 | 0.025 | 0.005 |
| 2687.4 | 100 | 0.025 | 0.035 |
| 2695.0 | 100 | 0.055 | -0.025 |
| 2702.6 | 100 | 0.085 | 0.035 |
| 2705.1 | 100 | 0.115 | 0.005 |
| 2708.5 | 100 | 0.025 | 0.005 |
| 2711.0 | 100 | 0.025 | -0.025 |
| 2716.1 | 100 | -0.005 | 0.035 |
| 2717.8 | 100 | 0.025 | 0.005 |
| 2720.3 | 100 | 0.025 | 0.035 |
| 2727.9 | 100 | 0.055 | -0.025 |
| 2735.5 | 100 | 0.085 | 0.035 |
| 2738.0 | 100 | 0.115 | 0.005 |
| 2741.3 | 100 | 0.025 | 0.005 |
| 2743.9 | 100 | 0.025 | -0.025 |
| 2748.9 | 100 | -0.005 | 0.035 |
| 2750.6 | 100 | 0.025 | 0.005 |
| 2753.1 | 100 | 0.025 | 0.035 |
| 2760.7 | 100 | 0.055 | -0.025 |
| 2765.8 | 100 | 0.085 | 0.035 |
| 2768.3 | 100 | 0.115 | 0.005 |
| 2771.7 | 100 | 0.025 | 0.005 |
| 2774.2 | 100 | 0.025 | -0.025 |
| 2779.2 | 100 | -0.005 | 0.035 |
| 2780.9 | 100 | 0.025 | 0.005 |
| 2783.5 | 100 | 0.025 | 0.035 |
| 2791.0 | 100 | 0.055 | -0.025 |
| 2796.1 | 100 | 0.085 | 0.035 |
| 2798.6 | 100 | 0.115 | 0.005 |
| 2802.0 | 100 | 0.025 | 0.005 |
| 2804.5 | 100 | 0.025 | -0.025 |
| 2809.6 | 100 | -0.005 | 0.035 |
| 2811.2 | 100 | 0.025 | 0.005 |
| 2813.8 | 100 | 0.025 | 0.035 |
| 2821.4 | 100 | 0.055 | -0.025 |
| 2826.4 | 100 | 0.085 | 0.035 |
| 2828.9 | 100 | 0.115 | 0.005 |
| 2832.3 | 100 | 0.025 | 0.005 |
| 2834.8 | 100 | 0.025 | -0.025 |
| 2839.9 | 100 | -0.005 | 0.035 |
| 2841.6 | 100 | 0.025 | 0.005 |
| 2844.1 | 100 | 0.025 | 0.035 |
| 2851.7 | 100 | 0.055 | -0.025 |
| 2856.7 | 100 | 0.085 | 0.035 |
| 2859.2 | 100 | 0.115 | 0.005 |
| 2862.6 | 100 | 0.025 | 0.005 |
| 2865.1 | 100 | 0.025 | -0.025 |
| 2870.2 | 100 | -0.005 | 0.035 |
| 2871.9 | 100 | 0.025 | 0.005 |
| 2874.4 | 100 | 0.025 | 0.035 |
| 2882.0 | 100 | 0.055 | -0.025 |
| 2887.0 | 100 | 0.085 | 0.035 |
| 2889.6 | 100 | 0.115 | 0.005 |
| 2892.9 | 100 | 0.025 | 0.005 |
| 2895.5 | 100 | 0.025 | -0.025 |
| 2900.5 | 100 | -0.005 | 0.035 |
| 2902.2 | 100 | 0.025 | 0.005 |
| 2904.7 | 100 | 0.025 | 0.035 |
| 2912.3 | 100 | 0.055 | -0.025 |
| 2917.4 | 100 | 0.085 | 0.035 |
| 2919.9 | 100 | 0.115 | 0.005 |
| 2923.3 | 100 | 0.025 | 0.005 |
| 2925.8 | 100 | 0.025 | -0.025 |
| 2930.8 | 100 | -0.005 | 0.035 |
| 2932.5 | 100 | 0.025 | 0.005 |
| 2935.0 | 100 | 0.025 | 0.035 |
| 2942.6 | 100 | 0.055 | -0.025 |
| 2947.7 | 100 | 0.085 | 0.035 |
| 2950.2 | 100 | 0.025 | 0.005 |
| 2952.7 | 100 | 0.025 | -0.025 |
| 2957.8 | 100 | -0.005 | 0.035 |
| 2959.5 | 100 | 0.025 | 0.005 |
| 2962.0 | 100 | 0.025 | 0.035 |
| 2969.6 | 100 | 0.055 | -0.025 |
| 2974.6 | 100 | 0.085 | 0.035 |
| 2977.2 | 100 | 0.025 | 0.005 |
| 2979.7 | 100 | 0.025 | -0.025 |
| 2984.7 | 100 | -0.005 | 0.035 |
| 2986.4 | 100 | 0.025 | 0.005 |
| 2988.9 | 100 | 0.025 | 0.035 |
| 2996.5 | 100 | 0.055 | -0.025 |
| 3001.6 | 100 | 0.085 | 0.035 |
| 3004.1 | 100 | 0.025 | 0.005 |
| 3006.6 | 100 | 0.025 | -0.025 |
| 3011.7 | 100 | -0.005 | 0.035 |
| 3013.4 | 100 | 0.025 | 0.005 |
| 3015.9 | 100 | 0.025 | 0.035 |
| 3023.5 | 100 | 0.055 | -0.025 |
| 3028.5 | 100 | 0.085 | 0.035 |
| 3031.0 | 100 | 0.025 | 0.005 |
| 3036.1 | 100 | 0.025 | 0.005 |
| 3041.2 | 100 | -0.005 | 0.035 |
| 3042.8 | 100 | 0.025 | 0.005 |
| 3045.4 | 100 | 0.025 | 0.035 |
| 3052.9 | 100 | 0.055 | -0.025 |
| 3058.0 | 100 | 0.085 | 0.035 |
| 3060.5 | 100 | 0.025 | 0.005 |
| 3065.6 | 100 | 0.025 | 0.005 |
| 3070.6 | 100 | -0.005 | 0.035 |
| 3072.3 | 100 | 0.025 | 0.005 |
| 3074.8 | 100 | 0.025 | 0.035 |
| 3082.4 | 100 | 0.055 | -0.025 |
| 3087.5 | 100 | 0.085 | 0.035 |
| 3090.0 | 100 | 0.025 | 0.005 |
| 3095.1 | 100 | 0.025 | 0.005 |
| 3100.1 | 100 | -0.005 | 0.035 |
| 3101.8 | 100 | 0.025 | 0.005 |
| 3104.3 | 100 | 0.025 | 0.035 |
| 3111.9 | 100 | 0.055 | -0.025 |
| 3116.9 | 100 | 0.085 | 0.035 |
| 3119.5 | 100 | 0.025 | 0.005 |
| 3124.5 | 100 | 0.025 | 0.005 |
| 3129.6 | 100 | -0.005 | 0.035 |
| 3131.3 | 100 | 0.025 | 0.005 |
| 3133.8 | 100 | 0.025 | 0.035 |
| 3141.4 | 100 | 0.055 | -0.025 |
| 3146.4 | 100 | 0.085 | 0.035 |
| 3149.0 | 100 | 0.025 | 0.005 |
| 3154.0 | 100 | 0.025 | 0.005 |
| 3159.1 | 100 | -0.005 | 0.035 |
| 3160.7 | 100 | 0.025 | 0.005 |
| 3163.3 | 100 | 0.025 | 0.035 |
| 3170.8 | 100 | 0.055 | -0.025 |
| 3175.9 | 100 | 0.085 | 0.035 |
| 3178.4 | 100 | 0.025 | 0.005 |
| 3183.5 | 100 | 0.025 | 0.005 |
| 3188.5 | 100 | -0.005 | 0.035 |
| 3190.2 | 100 | 0.025 | 0.005 |
| 3192.7 | 100 | 0.025 | 0.035 |
| 3197.8 | 100 | 0.055 | 0.005 |
| 3202.8 | 100 | 0.085 | 0.035 |
| 3205.4 | 100 | 0.025 | 0.005 |
| 3210.4 | 100 | 0.025 | 0.005 |
| 3215.5 | 100 | -0.005 | 0.035 |
| 3217.2 | 100 | 0.025 | 0.005 |
| 3219.7 | 100 | 0.025 | 0.035 |
| 3224.7 | 100 | 0.055 | 0.005 |
| 3229.8 | 100 | 0.085 | 0.035 |
| 3232.3 | 100 | 0.025 | 0.005 |
| 3237.4 | 100 | 0.025 | 0.005 |
| 3242.4 | 100 | -0.005 | 0.035 |
| 3244.1 | 100 | 0.025 | 0.005 |
| 3246.6 | 100 | 0.025 | 0.035 |
| 3251.7 | 100 | 0.055 | 0.005 |
| 3256.7 | 100 | 0.085 | 0.035 |
| 3259.3 | 100 | 0.025 | 0.005 |
| 3264.3 | 100 | 0.025 | 0.005 |
| 3269.4 | 100 | -0.005 | 0.035 |
| 3271.1 | 100 | 0.025 | 0.005 |
| 3273.6 | 100 | 0.025 | 0.035 |
| 3278.6 | 100 | 0.055 | 0.005 |
| 3283.7 | 100 | 0.085 | 0.035 |
| 3286.2 | 100 | 0.025 | 0.005 |
| 3291.3 | 100 | 0.025 | 0.005 |
| 3296.3 | 100 | -0.005 | 0.035 |
| 3298.0 | 100 | 0.025 | 0.005 |
| 3300.5 | 100 | 0.025 | 0.035 |
| 3305.6 | 100 | 0.055 | 0.005 |
| 3310.6 | 100 | 0.085 | 0.035 |
| 3313.2 | 100 | 0.025 | 0.005 |
| 3318.2 | 100 | 0.025 | 0.005 |
| 3323.3 | 100 | -0.005 | 0.035 |
| 3325.0 | 100 | 0.025 | 0.005 |
| 3327.5 | 100 | 0.025 | 0.035 |
| 3332.5 | 100 | 0.055 | 0.005 |
| 3335.1 | 100 | 0.085 | 0.005 |
| 3337.6 | 100 | 0.025 | 0.005 |
| 3342.6 | 100 | 0.025 | 0.005 |
| 3347.7 | 100 | -0.005 | 0.035 |
| 3349.4 | 100 | 0.025 | 0.005 |
| 3351.9 | 100 | 0.025 | 0.035 |
| 3357.0 | 100 | 0.055 | 0.005 |
| 3359.5 | 100 | 0.085 | 0.005 |
| 3362.0 | 100 | 0.025 | 0.005 |
| 3367.1 | 100 | 0.025 | 0.005 |
| 3372.1 | 100 | -0.005 | 0.035 |
| 3373.8 | 100 | 0.025 | 0.005 |
| 3376.3 | 100 | 0.025 | 0.035 |
| 3381.4 | 100 | 0.055 | 0.005 |
| 3383.9 | 100 | 0.085 | 0.005 |
| 3386.4 | 100 | 0.025 | 0.005 |
| 3391.5 | 100 | 0.025 | 0.005 |
| 3394.0 | 100 | -0.005 | 0.005 |
| 3395.7 | 100 | 0.025 | 0.005 |
| 3398.2 | 100 | 0.025 | 0.035 |
| 3403.3 | 100 | 0.055 | 0.005 |
| 3405.8 | 100 | 0.085 | 0.005 |
| 3408.3 | 100 | 0.025 | 0.005 |
| 3413.4 | 100 | 0.025 | 0.005 |
| 3415.9 | 100 | -0.005 | 0.005 |
| 3417.6 | 100 | 0.025 | 0.005 |
| 3420.1 | 100 | 0.025 | 0.035 |
| 3425.2 | 100 | 0.055 | 0.005 |
| 3427.7 | 100 | 0.085 | 0.005 |
| 3430.2 | 100 | 0.025 | 0.005 |
| 3435.3 | 100 | 0.025 | 0.005 |
| 3437.8 | 100 | -0.005 | 0.005 |
| 3439.5 | 100 | 0.025 | 0.005 |
| 3442.0 | 100 | 0.025 | 0.035 |
| 3447.1 | 100 | 0.055 | 0.005 |
| 3449.6 | 100 | 0.085 | 0.005 |
| 3452.1 | 100 | 0.025 | 0.005 |
| 3457.2 | 100 | 0.025 | 0.005 |
| 3459.7 | 100 | -0.005 | 0.005 |
| 3461.4 | 100 | 0.025 | 0.005 |
| 3463.9 | 100 | 0.025 | 0.035 |
| 3469.0 | 100 | 0.055 | 0.005 |
| 3471.5 | 100 | 0.085 | 0.005 |
| 3474.0 | 100 | 0.025 | 0.005 |
| 3479.1 | 100 | 0.025 | 0.005 |
| 3481.6 | 100 | -0.005 | 0.005 |
| 3483.3 | 100 | 0.025 | 0.005 |
| 3485.8 | 100 | 0.025 | 0.035 |
| 3490.9 | 100 | 0.055 | 0.005 |
| 3493.4 | 100 | 0.085 | 0.005 |
| 3495.9 | 100 | 0.025 | 0.005 |
| 3501.0 | 100 | 0.025 | 0.005 |
| 3503.5 | 100 | -0.005 | 0.005 |
| 3505.2 | 100 | 0.025 | 0.005 |
| 3507.7 | 100 | 0.025 | 0.035 |
| 3512.8 | 100 | 0.055 | 0.005 |
| 3515.3 | 100 | 0.085 | 0.005 |
| 3517.8 | 100 | 0.025 | 0.005 |
| 3522.9 | 100 | 0.025 | 0.005 |
| 3525.4 | 100 | -0.005 | 0.005 |
| 3527.1 | 100 | 0.025 | 0.005 |
| 3529.6 | 100 | 0.025 | 0.035 |
| 3534.7 | 100 | 0.055 | 0.005 |
| 3537.2 | 100 | 0.085 | 0.005 |
| 3539.7 | 100 | 0.025 | 0.005 |
| 3544.8 | 100 | 0.025 | 0.005 |
| 3547.3 | 100 | -0.005 | 0.005 |
| 3549.0 | 100 | 0.025 | 0.005 |
| 3551.5 | 100 | 0.025 | 0.035 |
| 3556.6 | 100 | 0.055 | 0.005 |
| 3559.1 | 100 | 0.085 | 0.005 |
| 3561.6 | 100 | 0.025 | 0.005 |
| 3566.7 | 100 | 0.025 | 0.005 |
| 3569.2 | 100 | -0.005 | 0.005 |
| 3570.9 | 100 | 0.025 | 0.005 |
| 3573.4 | 100 | 0.025 | 0.035 |
| 3578.5 | 100 | 0.055 | 0.005 |
| 3581.0 | 100 | 0.085 | 0.005 |
| 3583.5 | 100 | 0.025 | 0.005 |
| 3588.6 | 100 | 0.025 | 0.005 |
| 3591.1 | 100 | -0.005 | 0.005 |
| 3592.8 | 100 | 0.025 | 0.005 |
| 3595.3 | 100 | 0.025 | 0.035 |
| 3600.3 | 100 | 0.055 | 0.005 |
| 3602.9 | 100 | 0.085 | 0.005 |
| 3605.4 | 100 | 0.025 | 0.005 |
| 3610.5 | 100 | 0.025 | 0.005 |
| 3613.0 | 100 | -0.005 | 0.005 |
| 3614.7 | 100 | 0.025 | 0.005 |
| 3617.2 | 100 | 0.025 | 0.035 |
| 3619.7 | 100 | 0.055 | 0.005 |
| 3622.2 | 100 | 0.085 | 0.005 |
| 3624.8 | 100 | 0.025 | 0.005 |
| 3629.8 | 100 | 0.025 | 0.005 |
| 3632.3 | 100 | -0.005 | 0.005 |
| 3634.0 | 100 | 0.025 | 0.005 |
| 3636.6 | 100 | 0.025 | 0.035 |
| 3639.1 | 100 | 0.055 | 0.005 |
| 3641.6 | 100 | 0.085 | 0.005 |
| 3644.1 | 100 | 0.025 | 0.005 |
| 3649.2 | 100 | 0.025 | 0.005 |
| 3651.7 | 100 | -0.005 | 0.005 |
| 3653.4 | 100 | 0.025 | 0.005 |
| 3655.9 | 100 | 0.025 | 0.035 |
| 3658.5 | 100 | 0.055 | 0.005 |
| 3661.0 | 100 | 0.085 | 0.005 |
| 3663.5 | 100 | 0.025 | 0.005 |
| 3668.6 | 100 | 0.025 | 0.005 |
| 3671.1 | 100 | -0.005 | 0.005 |
| 3672.8 | 100 | 0.025 | 0.005 |
| 3675.3 | 100 | 0.025 | 0.035 |
| 3677.8 | 100 | 0.055 | 0.005 |
| 3680.4 | 100 | 0.085 | 0.005 |
| 3682.9 | 100 | 0.025 | 0.005 |
| 3687.9 | 100 | 0.025 | 0.005 |
| 3690.5 | 100 | -0.005 | 0.005 |
| 3692.1 | 100 | 0.025 | 0.005 |
| 3694.7 | 100 | 0.025 | 0.035 |
| 3697.2 | 100 | 0.055 | 0.005 |
| 3699.7 | 100 | 0.085 | 0.005 |
| 3702.2 | 100 | 0.025 | 0.005 |
| 3707.3 | 100 | 0.025 | 0.005 |
| 3709.8 | 100 | -0.005 | 0.005 |
| 3711.5 | 100 | 0.025 | 0.005 |
| 3714.0 | 100 | 0.025 | 0.035 |
| 3716.6 | 100 | 0.055 | 0.005 |
| 3719.1 | 100 | 0.085 | 0.005 |
| 3721.6 | 100 | 0.025 | 0.005 |
| 3726.7 | 100 | 0.025 | 0.005 |
| 3729.2 | 100 | -0.005 | 0.005 |
| 3730.9 | 100 | 0.025 | 0.005 |
| 3733.4 | 100 | 0.025 | 0.035 |
| 3735.9 | 100 | 0.055 | 0.005 |
| 3738.5 | 100 | 0.085 | 0.005 |
| 3741.0 | 100 | 0.025 | 0.005 |
| 3746.0 | 100 | 0.025 | 0.005 |
| 3748.6 | 100 | -0.005 | 0.005 |
| 3750.3 | 100 | 0.025 | 0.005 |
| 3752.8 | 100 | 0.025 | 0.035 |
| 3755.3 | 100 | 0.055 | 0.005 |
| 3757.8 | 100 | 0.085 | 0.005 |
| 3760.4 | 100 | 0.025 | 0.005 |
| 3765.4 | 100 | 0.025 | 0.005 |
| 3767.9 | 100 | -0.005 | 0.005 |
| 3769.6 | 100 | 0.025 | 0.005 |
| 3772.1 | 100 | 0.025 | 0.035 |
| 3774.7 | 100 | 0.055 | 0.005 |
| 3777.2 | 100 | 0.085 | 0.005 |
| 3779.7 | 100 | 0.025 | 0.005 |
| 3784.8 | 100 | 0.025 | 0.005 |
| 3787.3 | 100 | -0.005 | 0.005 |
| 3789.0 | 100 | 0.025 | 0.005 |
| 3791.5 | 100 | 0.025 | 0.035 |
| 3794.0 | 100 | 0.055 | 0.005 |
| 3796.6 | 100 | 0.085 | 0.005 |
| 3799.1 | 100 | 0.025 | 0.005 |
| 3804.1 | 100 | 0.025 | 0.005 |
| 3806.7 | 100 | -0.005 | 0.005 |
| 3808.4 | 100 | 0.025 | 0.005 |
| 3810.9 | 100 | 0.025 | 0.035 |
| 3813.4 | 100 | 0.055 | 0.005 |
| 3815.9 | 100 | 0.085 | 0.005 |
| 3818.5 | 100 | 0.025 | 0.005 |
| 3823.5 | 100 | 0.025 | 0.005 |
| 3826.0 | 100 | -0.005 | 0.005 |
| 3827.7 | 100 | 0.025 | 0.005 |
| 3830.3 | 100 | 0.025 | 0.035 |
| 3832.8 | 100 | 0.055 | 0.005 |
| 3835.3 | 100 | 0.085 | 0.005 |
| 3837.8 | 100 | 0.025 | 0.005 |
| 3842.9 | 100 | 0.025 | 0.005 |
| 3845.4 | 100 | -0.005 | 0.005 |
| 3847.1 | 100 | 0.025 | 0.005 |
| 3849.6 | 100 | 0.025 | 0.035 |
| 3852.2 | 100 | 0.055 | 0.005 |
| 3854.7 | 100 | 0.085 | 0.005 |
| 3857.2 | 100 | 0.025 | 0.005 |
| 3862.3 | 100 | 0.025 | 0.005 |
| 3864.8 | 100 | -0.005 | 0.005 |
| 3866.5 | 100 | 0.025 | 0.005 |
| 3869.0 | 100 | 0.025 | 0.035 |
| 3871.5 | 100 | 0.055 | 0.005 |
| 3873.2 | 100 | 0.025 | 0.005 |
| 3878.3 | 100 | 0.025 | 0.005 |
| 3880.8 | 100 | -0.005 | 0.005 |
| 3882.5 | 100 | 0.025 | 0.005 |
| 3885.0 | 100 | 0.025 | 0.035 |
| 3887.5 | 100 | 0.055 | 0.005 |
| 3889.2 | 100 | 0.025 | 0.005 |
| 3894.3 | 100 | 0.025 | 0.005 |
| 3896.8 | 100 | -0.005 | 0.005 |
| 3898.5 | 100 | 0.025 | 0.005 |
| 3901.0 | 100 | 0.025 | 0.035 |
| 3903.5 | 100 | 0.055 | 0.005 |
| 3905.2 | 100 | 0.025 | 0.005 |
| 3910.3 | 100 | 0.025 | 0.005 |
| 3912.8 | 100 | -0.005 | 0.005 |
| 3914.5 | 100 | 0.025 | 0.005 |
| 3917.0 | 100 | 0.025 | 0.035 |
| 3919.5 | 100 | 0.055 | 0.005 |
| 3921.2 | 100 | 0.025 | 0.005 |
| 3926.3 | 100 | 0.025 | 0.005 |
| 3928.8 | 100 | -0.005 | 0.005 |
| 3930.5 | 100 | 0.025 | 0.005 |
| 3933.0 | 100 | 0.025 | 0.035 |
| 3935.5 | 100 | 0.055 | 0.005 |
| 3937.2 | 100 | 0.025 | 0.005 |
| 3942.3 | 100 | 0.025 | 0.005 |
| 3944.8 | 100 | -0.005 | 0.005 |
| 3946.5 | 100 | 0.025 | 0.005 |
| 3951.5 | 100 | 0.025 | 0.005 |
| 3954.1 | 100 | 0.055 | 0.005 |
| 3955.7 | 100 | 0.025 | 0.005 |
| 3960.8 | 100 | 0.025 | 0.005 |
| 3963.3 | 100 | -0.005 | 0.005 |
| 3965.0 | 100 | 0.025 | 0.005 |
| 4015.0 | 60 | 0.000 | 0.000 |
| 5188.3 | 60 | 0.025 | 0.005 |
| 5221.7 | 60 | 0.025 | 0.365 |
| 5291.3 | 60 | 0.055 | -0.355 |
| 5358.1 | 60 | 0.085 | 0.335 |
| 5416.6 | 60 | 0.115 | -0.325 |
| 5458.3 | 60 | 0.145 | 0.245 |
| 5486.2 | 60 | 0.175 | -0.085 |
| 5497.3 | 60 | 0.205 | 0.065 |
| 5505.6 | 60 | 0.235 | -0.025 |
| 5511.2 | 60 | 0.265 | 0.035 |
| 5514.0 | 60 | 0.295 | 0.005 |
| 5523.3 | 60 | 0.025 | 0.005 |
| 5562.2 | 60 | 0.025 | -0.415 |
| 5637.4 | 60 | -0.005 | 0.365 |
| 5693.1 | 60 | -0.035 | -0.235 |
| 5737.6 | 60 | -0.065 | 0.215 |
| 5779.3 | 60 | -0.095 | -0.235 |
| 5801.6 | 60 | -0.125 | 0.095 |
| 5818.3 | 60 | -0.155 | -0.055 |
| 5826.7 | 60 | -0.185 | 0.035 |
| 5835.0 | 60 | -0.215 | -0.025 |
| 5837.8 | 60 | -0.245 | 0.005 |
| 5847.1 | 60 | 0.025 | 0.005 |
| 5877.7 | 60 | 0.025 | 0.335 |
| 5941.7 | 60 | 0.055 | -0.325 |
| 5991.8 | 60 | 0.085 | 0.275 |
| 6033.6 | 60 | 0.115 | -0.175 |
| 6047.5 | 60 | 0.145 | 0.065 |
| 6055.8 | 60 | 0.175 | -0.025 |
| 6061.4 | 60 | 0.205 | 0.035 |
| 6067.0 | 60 | 0.235 | 0.005 |
| 6074.4 | 60 | 0.025 | 0.005 |
| 6107.8 | 60 | 0.025 | -0.355 |
| 6169.0 | 60 | -0.005 | 0.335 |
| 6202.4 | 60 | -0.035 | -0.175 |
| 6227.5 | 60 | -0.065 | 0.095 |
| 6246.9 | 60 | -0.095 | -0.085 |
| 6258.1 | 60 | -0.125 | 0.065 |
| 6266.4 | 60 | -0.155 | -0.025 |
| 6269.2 | 60 | -0.185 | 0.005 |
| 6276.6 | 60 | 0.025 | 0.005 |
| 6307.3 | 60 | 0.025 | 0.335 |
| 6360.1 | 60 | 0.055 | -0.265 |
| 6404.7 | 60 | 0.085 | 0.245 |
| 6429.7 | 60 | 0.115 | -0.085 |
| 6440.9 | 60 | 0.145 | 0.035 |
| 6446.4 | 60 | 0.175 | 0.005 |
| 6452.0 | 60 | 0.205 | 0.035 |
| 6454.8 | 60 | 0.235 | 0.005 |
| 6462.2 | 60 | 0.025 | 0.005 |
| 6490.0 | 60 | 0.025 | -0.295 |
| 6542.9 | 60 | -0.005 | 0.275 |
| 6562.4 | 60 | -0.035 | -0.085 |
| 6576.3 | 60 | -0.065 | 0.065 |
| 6587.4 | 60 | -0.095 | -0.055 |
| 6595.8 | 60 | -0.125 | 0.035 |
| 6601.4 | 60 | -0.155 | 0.005 |
| 6604.1 | 60 | -0.185 | 0.005 |
| 6611.6 | 60 | 0.025 | 0.005 |
| 6636.6 | 60 | 0.025 | 0.275 |
| 6686.7 | 60 | 0.055 | -0.235 |
| 6725.7 | 60 | 0.085 | 0.215 |
| 6739.6 | 60 | 0.115 | -0.055 |
| 6745.2 | 60 | 0.145 | 0.035 |
| 6750.7 | 60 | 0.175 | 0.005 |
| 6753.5 | 60 | 0.205 | 0.005 |
| 6760.0 | 60 | 0.025 | 0.005 |
| 6785.1 | 60 | 0.025 | -0.265 |
| 6835.2 | 60 | -0.005 | 0.275 |
| 6849.1 | 60 | -0.035 | -0.055 |
| 6857.4 | 60 | -0.065 | 0.035 |
| 6865.8 | 60 | -0.095 | -0.025 |
| 6871.3 | 60 | -0.125 | 0.035 |
| 6874.1 | 60 | -0.155 | 0.005 |
| 6880.6 | 60 | 0.025 | 0.005 |
| 6905.7 | 60 | 0.025 | 0.275 |
| 6950.2 | 60 | 0.055 | -0.205 |
| 6983.6 | 60 | 0.085 | 0.185 |
| 6994.7 | 60 | 0.115 | -0.025 |
| 7000.3 | 60 | 0.145 | 0.035 |
| 7005.9 | 60 | 0.175 | 0.005 |
| 7008.7 | 60 | 0.205 | 0.005 |
| 7015.2 | 60 | 0.025 | 0.005 |
| 7040.2 | 60 | 0.025 | -0.265 |
| 7082.0 | 60 | -0.005 | 0.215 |
| 7093.1 | 60 | -0.035 | -0.025 |
| 7101.4 | 60 | -0.065 | 0.035 |
| 7109.8 | 60 | -0.095 | -0.025 |
| 7115.4 | 60 | -0.125 | 0.035 |
| 7118.1 | 60 | -0.155 | 0.005 |
| 7124.6 | 60 | 0.025 | 0.005 |
| 7146.9 | 60 | 0.025 | 0.245 |
| 7185.9 | 60 | 0.055 | -0.175 |
| 7213.7 | 60 | 0.085 | 0.155 |
| 7224.8 | 60 | 0.115 | -0.025 |
| 7230.4 | 60 | 0.145 | 0.035 |
| 7233.2 | 60 | 0.175 | 0.005 |
| 7238.7 | 60 | 0.025 | 0.005 |
| 7261.0 | 60 | 0.025 | -0.235 |
| 7297.2 | 60 | -0.005 | 0.185 |
| 7305.5 | 60 | -0.035 | -0.025 |
| 7313.9 | 60 | -0.065 | 0.035 |
| 7319.5 | 60 | -0.095 | 0.005 |
| 7325.0 | 60 | -0.125 | 0.035 |
| 7327.8 | 60 | -0.155 | 0.005 |
| 7334.3 | 60 | 0.025 | 0.005 |
| 7353.8 | 60 | 0.025 | 0.215 |
| 7392.8 | 60 | 0.055 | -0.175 |
| 7412.2 | 60 | 0.085 | 0.095 |
| 7420.6 | 60 | 0.115 | -0.025 |
| 7426.2 | 60 | 0.145 | 0.035 |
| 7428.9 | 60 | 0.175 | 0.005 |
| 7434.5 | 60 | 0.025 | 0.005 |
| 7454.0 | 60 | 0.025 | -0.205 |
| 7490.2 | 60 | -0.005 | 0.185 |
| 7498.5 | 60 | -0.035 | -0.025 |
| 7506.9 | 60 | -0.065 | 0.035 |
| 7512.4 | 60 | -0.095 | 0.005 |
| 7515.2 | 60 | -0.125 | 0.005 |
| 7520.8 | 60 | 0.025 | 0.005 |
| 7540.3 | 60 | 0.025 | 0.215 |
| 7576.5 | 60 | 0.055 | -0.175 |
| 7595.9 | 60 | 0.085 | 0.095 |
| 7601.5 | 60 | 0.115 | 0.005 |
| 7607.1 | 60 | 0.145 | 0.035 |
| 7609.9 | 60 | 0.175 | 0.005 |
| 7615.4 | 60 | 0.025 | 0.005 |
| 7634.9 | 60 | 0.025 | -0.205 |
| 7665.5 | 60 | -0.005 | 0.155 |
| 7673.9 | 60 | -0.035 | -0.025 |
| 7679.4 | 60 | -0.065 | 0.035 |
| 7685.0 | 60 | -0.095 | 0.005 |
| 7687.8 | 60 | -0.125 | 0.005 |
| 7693.4 | 60 | 0.025 | 0.005 |
| 7712.8 | 60 | 0.025 | 0.215 |
| 7749.0 | 60 | 0.055 | -0.175 |
| 7765.7 | 60 | 0.085 | 0.095 |
| 7771.3 | 60 | 0.115 | 0.005 |
| 7774.1 | 60 | 0.145 | 0.005 |
| 7776.9 | 60 | 0.175 | 0.005 |
| 7782.4 | 60 | 0.025 | 0.005 |
| 7801.9 | 60 | 0.025 | -0.205 |
| 7832.5 | 60 | -0.005 | 0.155 |
| 7840.9 | 60 | -0.035 | -0.025 |
| 7846.4 | 60 | -0.065 | 0.035 |
| 7849.2 | 60 | -0.095 | 0.005 |
| 7852.0 | 60 | -0.125 | 0.005 |
| 7857.6 | 60 | 0.025 | 0.005 |
| 7877.1 | 60 | 0.025 | 0.215 |
| 7910.5 | 60 | 0.055 | -0.145 |
| 7924.4 | 60 | 0.085 | 0.065 |
| 7929.9 | 60 | 0.115 | 0.005 |
| 7932.7 | 60 | 0.145 | 0.005 |
| 7937.4 | 60 | 0.025 | 0.005 |
| 7954.1 | 60 | 0.025 | -0.175 |
| 7984.7 | 60 | -0.005 | 0.155 |
| 7993.0 | 60 | -0.035 | -0.025 |
| 7998.6 | 60 | -0.065 | 0.035 |
| 8001.4 | 60 | -0.095 | 0.005 |
| 8004.2 | 60 | -0.125 | 0.005 |
| 8009.7 | 60 | 0.025 | 0.005 |
| 8026.4 | 60 | 0.025 | 0.185 |
| 8057.1 | 60 | 0.055 | -0.145 |
| 8068.2 | 60 | 0.085 | 0.065 |
| 8073.8 | 60 | 0.115 | 0.005 |
| 8076.5 | 60 | 0.145 | 0.005 |
| 8081.2 | 60 | 0.025 | 0.005 |
| 8097.9 | 60 | 0.025 | -0.175 |
| 8122.9 | 60 | -0.005 | 0.125 |
| 8128.5 | 60 | -0.035 | 0.005 |
| 8134.1 | 60 | -0.065 | 0.035 |
| 8136.8 | 60 | -0.095 | 0.005 |
| 8139.6 | 60 | -0.125 | 0.005 |
| 8145.2 | 60 | 0.025 | 0.005 |
| 8161.9 | 60 | 0.025 | 0.185 |
| 8192.5 | 60 | 0.055 | -0.145 |
| 8203.6 | 60 | 0.085 | 0.065 |
| 8209.2 | 60 | 0.115 | 0.005 |
| 8212.0 | 60 | 0.145 | 0.005 |
| 8216.6 | 60 | 0.025 | 0.005 |
| 8233.3 | 60 | 0.025 | -0.175 |
| 8255.6 | 60 | -0.005 | 0.125 |
| 8261.2 | 60 | -0.035 | 0.005 |
| 8266.7 | 60 | -0.065 | 0.035 |
| 8269.5 | 60 | -0.095 | 0.005 |
| 8272.3 | 60 | -0.125 | 0.005 |
| 8277.9 | 60 | 0.025 | 0.005 |
| 8294.6 | 60 | 0.025 | 0.185 |
| 8322.4 | 60 | 0.055 | -0.115 |
| 8333.5 | 60 | 0.085 | 0.065 |
| 8339.1 | 60 | 0.115 | 0.005 |
| 8341.9 | 60 | 0.145 | 0.005 |
| 8346.5 | 60 | 0.025 | 0.005 |
| 8360.4 | 60 | 0.025 | -0.145 |
| 8382.7 | 60 | -0.005 | 0.125 |
| 8388.3 | 60 | -0.035 | 0.005 |
| 8391.1 | 60 | -0.065 | 0.005 |
| 8393.8 | 60 | -0.095 | 0.005 |
| 8398.5 | 60 | 0.025 | 0.005 |
| 8415.2 | 60 | 0.025 | 0.185 |
| 8440.2 | 60 | 0.055 | -0.115 |
| 8451.4 | 60 | 0.085 | 0.065 |
| 8456.9 | 60 | 0.115 | 0.005 |
| 8460.6 | 60 | 0.025 | 0.005 |
| 8474.6 | 60 | 0.025 | -0.145 |
| 8494.0 | 60 | -0.005 | 0.095 |
| 8499.6 | 60 | -0.035 | 0.005 |
| 8502.4 | 60 | -0.065 | 0.005 |
| 8505.2 | 60 | -0.095 | 0.005 |
| 8509.8 | 60 | 0.025 | 0.005 |
| 8523.7 | 60 | 0.025 | 0.155 |
| 8548.8 | 60 | 0.055 | -0.115 |
| 8557.1 | 60 | 0.085 | 0.035 |
| 8562.7 | 60 | 0.115 | 0.005 |
| 8566.4 | 60 | 0.025 | 0.005 |
| 8580.3 | 60 | 0.025 | -0.145 |
| 8599.8 | 60 | -0.005 | 0.095 |
| 8605.4 | 60 | -0.035 | 0.005 |
| 8608.2 | 60 | -0.065 | 0.005 |
| 8610.9 | 60 | -0.095 | 0.005 |
| 8615.6 | 60 | 0.025 | 0.005 |
| 8629.5 | 60 | 0.025 | 0.155 |
| 8654.5 | 60 | 0.055 | -0.115 |
| 8662.9 | 60 | 0.085 | 0.035 |
| 8665.7 | 60 | 0.115 | 0.005 |
| 8669.4 | 60 | 0.025 | 0.005 |
| 8683.3 | 60 | 0.025 | -0.145 |
| 8702.8 | 60 | -0.005 | 0.095 |
| 8708.4 | 60 | -0.035 | 0.005 |
| 8711.1 | 60 | -0.065 | 0.005 |
| 8713.9 | 60 | -0.095 | 0.005 |
| 8718.6 | 60 | 0.025 | 0.005 |
| 8732.5 | 60 | 0.025 | 0.155 |
| 8752.0 | 60 | 0.055 | -0.085 |
| 8760.3 | 60 | 0.085 | 0.035 |
| 8763.1 | 60 | 0.115 | 0.005 |
| 8766.8 | 60 | 0.025 | 0.005 |
| 8780.7 | 60 | 0.025 | -0.145 |
| 8797.4 | 60 | -0.005 | 0.095 |
| 8800.2 | 60 | -0.035 | 0.005 |
| 8803.0 | 60 | -0.065 | 0.005 |
| 8806.7 | 60 | 0.025 | 0.005 |
| 8820.6 | 60 | 0.025 | 0.155 |
| 8840.1 | 60 | 0.055 | -0.085 |
| 8848.5 | 60 | 0.085 | 0.035 |
| 8851.2 | 60 | 0.115 | 0.005 |
| 8854.9 | 60 | 0.025 | 0.005 |
| 8866.1 | 60 | 0.025 | -0.115 |
| 8882.8 | 60 | -0.005 | 0.095 |
| 8885.6 | 60 | -0.035 | 0.005 |
| 8888.3 | 60 | -0.065 | 0.005 |
| 8892.1 | 60 | 0.025 | 0.005 |
| 8906.0 | 60 | 0.025 | 0.155 |
| 8925.5 | 60 | 0.055 | -0.085 |
| 8933.8 | 60 | 0.085 | 0.035 |
| 8936.6 | 60 | 0.115 | 0.005 |
| 8940.3 | 60 | 0.025 | 0.005 |
| 8951.4 | 60 | 0.025 | -0.115 |
| 8968.1 | 60 | -0.005 | 0.095 |
| 8970.9 | 60 | -0.035 | 0.005 |
| 8973.7 | 60 | -0.065 | 0.005 |
| 8977.4 | 60 | 0.025 | 0.005 |
| 8988.5 | 60 | 0.025 | 0.125 |
| 9008.0 | 60 | 0.055 | -0.085 |
| 9013.6 | 60 | 0.085 | 0.035 |
| 9016.4 | 60 | 0.115 | 0.005 |
| 9020.1 | 60 | 0.025 | 0.005 |
| 9031.2 | 60 | 0.025 | -0.115 |
| 9045.1 | 60 | -0.005 | 0.065 |
| 9047.9 | 60 | -0.035 | 0.005 |
| 9050.7 | 60 | -0.065 | 0.005 |
| 9054.4 | 60 | 0.025 | 0.005 |
| 9065.6 | 60 | 0.025 | 0.125 |
| 9085.0 | 60 | 0.055 | -0.085 |
| 9090.6 | 60 | 0.085 | 0.035 |
| 9093.4 | 60 | 0.115 | 0.005 |
| 9097.1 | 60 | 0.025 | 0.005 |
| 9108.2 | 60 | 0.025 | -0.115 |
| 9122.1 | 60 | -0.005 | 0.065 |
| 9124.9 | 60 | -0.035 | 0.005 |
| 9127.7 | 60 | -0.065 | 0.005 |
| 9131.4 | 60 | 0.025 | 0.005 |
| 9142.6 | 60 | 0.025 | 0.125 |
| 9162.0 | 60 | 0.055 | -0.085 |
| 9167.6 | 60 | 0.085 | 0.035 |
| 9170.4 | 60 | 0.115 | 0.005 |
| 9174.1 | 60 | 0.025 | 0.005 |
| 9185.2 | 60 | 0.025 | -0.115 |
| 9199.2 | 60 | -0.005 | 0.065 |
| 9201.9 | 60 | -0.035 | 0.005 |
| 9204.7 | 60 | 0.025 | 0.005 |
| 9215.9 | 60 | 0.025 | 0.125 |
| 9235.3 | 60 | 0.055 | -0.085 |
| 9240.9 | 60 | 0.085 | 0.035 |
| 9243.7 | 60 | 0.025 | 0.005 |
| 9254.8 | 60 | 0.025 | -0.115 |
| 9268.7 | 60 | -0.005 | 0.065 |
| 9271.5 | 60 | -0.035 | 0.005 |
| 9274.3 | 60 | 0.025 | 0.005 |
| 9282.7 | 60 | 0.025 | 0.095 |
| 9299.4 | 60 | 0.055 | -0.055 |
| 9304.9 | 60 | 0.085 | 0.035 |
| 9307.7 | 60 | 0.025 | 0.005 |
| 9318.8 | 60 | 0.025 | -0.115 |
| 9332.8 | 60 | -0.005 | 0.065 |
| 9335.5 | 60 | -0.035 | 0.005 |
| 9338.3 | 60 | 0.025 | 0.005 |
| 9346.7 | 60 | 0.025 | 0.095 |
| 9363.4 | 60 | 0.055 | -0.055 |
| 9368.9 | 60 | 0.085 | 0.035 |
| 9371.7 | 60 | 0.025 | 0.005 |
| 9380.1 | 60 | 0.025 | -0.085 |
| 9394.0 | 60 | -0.005 | 0.065 |
| 9396.8 | 60 | -0.035 | 0.005 |
| 9399.6 | 60 | 0.025 | 0.005 |
| 9407.9 | 60 | 0.025 | 0.095 |
| 9421.8 | 60 | 0.055 | -0.055 |
| 9427.4 | 60 | 0.085 | 0.035 |
| 9430.2 | 60 | 0.025 | 0.005 |
| 9438.5 | 60 | 0.025 | -0.085 |
| 9449.7 | 60 | -0.005 | 0.065 |
| 9452.4 | 60 | -0.035 | 0.005 |
| 9455.2 | 60 | 0.025 | 0.005 |
| 9463.6 | 60 | 0.025 | 0.095 |
| 9477.5 | 60 | 0.055 | -0.055 |
| 9483.1 | 60 | 0.085 | 0.035 |
| 9485.8 | 60 | 0.025 | 0.005 |
| 9494.2 | 60 | 0.025 | -0.085 |
| 9505.3 | 60 | -0.005 | 0.065 |
| 9508.1 | 60 | -0.035 | 0.005 |
| 9510.9 | 60 | 0.025 | 0.005 |
| 9519.2 | 60 | 0.025 | 0.095 |
| 9533.2 | 60 | 0.055 | -0.055 |
| 9538.7 | 60 | 0.085 | 0.035 |
| 9541.5 | 60 | 0.025 | 0.005 |
| 9549.9 | 60 | 0.025 | -0.085 |
| 9561.0 | 60 | -0.005 | 0.065 |
| 9563.8 | 60 | -0.035 | 0.005 |
| 9566.6 | 60 | 0.025 | 0.005 |
| 9574.9 | 60 | 0.025 | 0.095 |
| 9588.8 | 60 | 0.055 | -0.055 |
| 9594.4 | 60 | 0.085 | 0.035 |
| 9597.2 | 60 | 0.025 | 0.005 |
| 9605.5 | 60 | 0.025 | -0.085 |
| 9616.7 | 60 | -0.005 | 0.065 |
| 9619.4 | 60 | -0.035 | 0.005 |
| 9622.2 | 60 | 0.025 | 0.005 |
| 9630.6 | 60 | 0.025 | 0.095 |
| 9644.5 | 60 | 0.055 | -0.055 |
| 9650.1 | 60 | 0.085 | 0.035 |
| 9652.8 | 60 | 0.025 | 0.005 |
| 9661.2 | 60 | 0.025 | -0.085 |
| 9669.5 | 60 | -0.005 | 0.035 |
| 9672.3 | 60 | -0.035 | 0.005 |
| 9675.1 | 60 | 0.025 | 0.005 |
| 9683.5 | 60 | 0.025 | 0.095 |
| 9697.4 | 60 | 0.055 | -0.055 |
| 9700.2 | 60 | 0.085 | 0.005 |
| 9702.9 | 60 | 0.025 | 0.005 |
| 9708.5 | 60 | 0.025 | -0.055 |
| 9716.9 | 60 | -0.005 | 0.035 |
| 9719.6 | 60 | -0.035 | 0.005 |
| 9722.4 | 60 | 0.025 | 0.005 |
| 9730.8 | 60 | 0.025 | 0.095 |
| 9744.7 | 60 | 0.055 | -0.055 |
| 9747.5 | 60 | 0.085 | 0.005 |
| 9750.3 | 60 | 0.025 | 0.005 |
| 9755.8 | 60 | 0.025 | -0.055 |
| 9764.2 | 60 | -0.005 | 0.035 |
| 9767.0 | 60 | -0.035 | 0.005 |
| 9769.7 | 60 | 0.025 | 0.005 |
| 9778.1 | 60 | 0.025 | 0.095 |
| 9792.0 | 60 | 0.055 | -0.055 |
| 9794.8 | 60 | 0.085 | 0.005 |
| 9797.6 | 60 | 0.025 | 0.005 |
| 9803.1 | 60 | 0.025 | -0.055 |
| 9811.5 | 60 | -0.005 | 0.035 |
| 9814.3 | 60 | -0.035 | 0.005 |
| 9817.1 | 60 | 0.025 | 0.005 |
| 9825.4 | 60 | 0.025 | 0.095 |
| 9839.3 | 60 | 0.055 | -0.055 |
| 9842.1 | 60 | 0.085 | 0.005 |
| 9844.9 | 60 | 0.025 | 0.005 |
| 9850.5 | 60 | 0.025 | -0.055 |
| 9858.8 | 60 | -0.005 | 0.035 |
| 9861.6 | 60 | -0.035 | 0.005 |
| 9864.4 | 60 | 0.025 | 0.005 |
| 9869.9 | 60 | 0.025 | 0.065 |
| 9883.9 | 60 | 0.055 | -0.055 |
| 9886.6 | 60 | 0.085 | 0.005 |
| 9889.4 | 60 | 0.025 | 0.005 |
| 9895.0 | 60 | 0.025 | -0.055 |
| 9903.3 | 60 | -0.005 | 0.035 |
| 9906.1 | 60 | -0.035 | 0.005 |
| 9908.9 | 60 | 0.025 | 0.005 |
| 9914.5 | 60 | 0.025 | 0.065 |
| 9925.6 | 60 | 0.055 | -0.025 |
| 9928.4 | 60 | 0.085 | 0.005 |
| 9931.2 | 60 | 0.025 | 0.005 |
| 9936.7 | 60 | 0.025 | -0.055 |
| 9945.1 | 60 | -0.005 | 0.035 |
| 9947.9 | 60 | -0.035 | 0.005 |
| 9950.7 | 60 | 0.025 | 0.005 |
| 9956.2 | 60 | 0.025 | 0.065 |
| 9967.4 | 60 | 0.055 | -0.025 |
| 9970.1 | 60 | 0.085 | 0.005 |
| 9972.9 | 60 | 0.025 | 0.005 |
| 9978.5 | 60 | 0.025 | -0.055 |
| 9986.8 | 60 | -0.005 | 0.035 |
| 9989.6 | 60 | -0.035 | 0.005 |
| 9992.4 | 60 | 0.025 | 0.005 |
| 9998.0 | 60 | 0.025 | 0.065 |
| 10009.1 | 60 | 0.055 | -0.025 |
| 10011.9 | 60 | 0.085 | 0.005 |
| 10014.7 | 60 | 0.025 | 0.005 |
| 10020.2 | 60 | 0.025 | -0.055 |
| 10028.6 | 60 | -0.005 | 0.035 |
| 10030.4 | 60 | 0.025 | 0.005 |
| 10036.0 | 60 | 0.025 | 0.065 |
| 10047.1 | 60 | 0.055 | -0.025 |
| 10049.9 | 60 | 0.085 | 0.005 |
| 10052.7 | 60 | 0.025 | 0.005 |
| 10058.3 | 60 | 0.025 | -0.055 |
| 10066.6 | 60 | -0.005 | 0.035 |
| 10068.5 | 60 | 0.025 | 0.005 |
| 10074.1 | 60 | 0.025 | 0.065 |
| 10085.2 | 60 | 0.055 | -0.025 |
| 10088.0 | 60 | 0.085 | 0.005 |
| 10090.8 | 60 | 0.025 | 0.005 |
| 10096.3 | 60 | 0.025 | -0.055 |
| 10104.7 | 60 | -0.005 | 0.035 |
| 10106.5 | 60 | 0.025 | 0.005 |
| 10112.1 | 60 | 0.025 | 0.065 |
| 10123.2 | 60 | 0.055 | -0.025 |
| 10126.0 | 60 | 0.085 | 0.005 |
| 10128.8 | 60 | 0.025 | 0.005 |
| 10134.4 | 60 | 0.025 | -0.055 |
| 10142.7 | 60 | -0.005 | 0.035 |
| 10144.6 | 60 | 0.025 | 0.005 |
| 10150.1 | 60 | 0.025 | 0.065 |
| 10161.3 | 60 | 0.055 | -0.025 |
| 10164.0 | 60 | 0.085 | 0.005 |
| 10166.8 | 60 | 0.025 | 0.005 |
| 10172.4 | 60 | 0.025 | -0.055 |
| 10180.7 | 60 | -0.005 | 0.035 |
| 10182.6 | 60 | 0.025 | 0.005 |
| 10188.2 | 60 | 0.025 | 0.065 |
| 10196.5 | 60 | 0.055 | -0.025 |
| 10199.3 | 60 | 0.085 | 0.005 |
| 10202.1 | 60 | 0.025 | 0.005 |
| 10207.7 | 60 | 0.025 | -0.055 |
| 10216.0 | 60 | -0.005 | 0.035 |
| 10217.9 | 60 | 0.025 | 0.005 |
| 10223.4 | 60 | 0.025 | 0.065 |
| 10231.8 | 60 | 0.055 | -0.025 |
| 10234.6 | 60 | 0.085 | 0.005 |
| 10237.3 | 60 | 0.025 | 0.005 |
| 10242.9 | 60 | 0.025 | -0.055 |
| 10251.3 | 60 | -0.005 | 0.035 |
| 10253.1 | 60 | 0.025 | 0.005 |
| 10258.7 | 60 | 0.025 | 0.065 |
| 10267.0 | 60 | 0.055 | -0.025 |
| 10269.8 | 60 | 0.085 | 0.005 |
| 10272.6 | 60 | 0.025 | 0.005 |
| 10278.2 | 60 | 0.025 | -0.055 |
| 10286.5 | 60 | -0.005 | 0.035 |
| 10288.4 | 60 | 0.025 | 0.005 |
| 10293.9 | 60 | 0.025 | 0.065 |
| 10302.3 | 60 | 0.055 | -0.025 |
| 10305.1 | 60 | 0.085 | 0.005 |
| 10307.9 | 60 | 0.025 | 0.005 |
| 10313.4 | 60 | 0.025 | -0.055 |
| 10321.8 | 60 | -0.005 | 0.035 |
| 10323.6 | 60 | 0.025 | 0.005 |
| 10329.2 | 60 | 0.025 | 0.065 |
| 10337.5 | 60 | 0.055 | -0.025 |
| 10340.3 | 60 | 0.085 | 0.005 |
| 10343.1 | 60 | 0.025 | 0.005 |
| 10348.7 | 60 | 0.025 | -0.055 |
| 10357.0 | 60 | -0.005 | 0.035 |
| 10358.9 | 60 | 0.025 | 0.005 |
| 10364.4 | 60 | 0.025 | 0.065 |
| 10372.8 | 60 | 0.055 | -0.025 |
| 10375.6 | 60 | 0.085 | 0.005 |
| 10378.4 | 60 | 0.025 | 0.005 |
| 10383.9 | 60 | 0.025 | -0.055 |
| 10392.3 | 60 | -0.005 | 0.035 |
| 10394.1 | 60 | 0.025 | 0.005 |
| 10399.7 | 60 | 0.025 | 0.065 |
| 10408.1 | 60 | 0.055 | -0.025 |
| 10410.8 | 60 | 0.085 | 0.005 |
| 10413.6 | 60 | 0.025 | 0.005 |
| 10419.2 | 60 | 0.025 | -0.055 |
| 10427.5 | 60 | -0.005 | 0.035 |
| 10429.4 | 60 | 0.025 | 0.005 |
| 10435.0 | 60 | 0.025 | 0.065 |
| 10443.3 | 60 | 0.055 | -0.025 |
| 10446.1 | 60 | 0.085 | 0.005 |
| 10448.9 | 60 | 0.025 | 0.005 |
| 10454.4 | 60 | 0.025 | -0.055 |
| 10462.8 | 60 | -0.005 | 0.035 |
| 10464.6 | 60 | 0.025 | 0.005 |
| 10470.2 | 60 | 0.025 | 0.065 |
| 10478.6 | 60 | 0.055 | -0.025 |
| 10481.3 | 60 | 0.085 | 0.005 |
| 10484.1 | 60 | 0.025 | 0.005 |
| 10489.7 | 60 | 0.025 | -0.055 |
| 10498.0 | 60 | -0.005 | 0.035 |
| 10499.9 | 60 | 0.025 | 0.005 |
| 10505.5 | 60 | 0.025 | 0.065 |
| 10513.8 | 60 | 0.055 | -0.025 |
| 10516.6 | 60 | 0.085 | 0.005 |
| 10519.4 | 60 | 0.025 | 0.005 |
| 10525.0 | 60 | 0.025 | -0.055 |
| 10533.3 | 60 | -0.005 | 0.035 |
| 10535.2 | 60 | 0.025 | 0.005 |
| 10540.7 | 60 | 0.025 | 0.065 |
| 10549.1 | 60 | 0.055 | -0.025 |
| 10551.9 | 60 | 0.085 | 0.005 |
| 10554.6 | 60 | 0.025 | 0.005 |
| 10560.2 | 60 | 0.025 | -0.055 |
| 10568.6 | 60 | -0.005 | 0.035 |
| 10570.4 | 60 | 0.025 | 0.005 |
| 10576.0 | 60 | 0.025 | 0.065 |
| 10584.3 | 60 | 0.055 | -0.025 |
| 10586.2 | 60 | 0.025 | 0.005 |
| 10589.0 | 60 | 0.025 | -0.025 |
| 10597.3 | 60 | -0.005 | 0.035 |
| 10599.2 | 60 | 0.025 | 0.005 |
| 10604.7 | 60 | 0.025 | 0.065 |
| 10613.1 | 60 | 0.055 | -0.025 |
| 10615.0 | 60 | 0.025 | 0.005 |
| 10617.7 | 60 | 0.025 | -0.025 |
| 10626.1 | 60 | -0.005 | 0.035 |
| 10627.9 | 60 | 0.025 | 0.005 |
| 10633.5 | 60 | 0.025 | 0.065 |
| 10641.9 | 60 | 0.055 | -0.025 |
| 10643.7 | 60 | 0.025 | 0.005 |
| 10646.5 | 60 | 0.025 | -0.025 |
| 10654.8 | 60 | -0.005 | 0.035 |
| 10656.7 | 60 | 0.025 | 0.005 |
| 10662.3 | 60 | 0.025 | 0.065 |
| 10670.6 | 60 | 0.055 | -0.025 |
| 10672.5 | 60 | 0.025 | 0.005 |
| 10675.3 | 60 | 0.025 | -0.025 |
| 10683.6 | 60 | -0.005 | 0.035 |
| 10685.5 | 60 | 0.025 | 0.005 |
| 10691.0 | 60 | 0.025 | 0.065 |
| 10699.4 | 60 | 0.055 | -0.025 |
| 10701.2 | 60 | 0.025 | 0.005 |
| 10704.0 | 60 | 0.025 | -0.025 |
| 10712.4 | 60 | -0.005 | 0.035 |
| 10714.2 | 60 | 0.025 | 0.005 |
| 10719.8 | 60 | 0.025 | 0.065 |
| 10728.1 | 60 | 0.055 | -0.025 |
| 10730.0 | 60 | 0.025 | 0.005 |
| 10732.8 | 60 | 0.025 | -0.025 |
| 10741.1 | 60 | -0.005 | 0.035 |
| 10743.0 | 60 | 0.025 | 0.005 |
| 10748.6 | 60 | 0.025 | 0.065 |
| 10756.9 | 60 | 0.055 | -0.025 |
| 10758.8 | 60 | 0.025 | 0.005 |
| 10761.5 | 60 | 0.025 | -0.025 |
| 10769.9 | 60 | -0.005 | 0.035 |
| 10771.7 | 60 | 0.025 | 0.005 |
| 10777.3 | 60 | 0.025 | 0.065 |
| 10785.7 | 60 | 0.055 | -0.025 |
| 10787.5 | 60 | 0.025 | 0.005 |
| 10790.3 | 60 | 0.025 | -0.025 |
| 10798.7 | 60 | -0.005 | 0.035 |
| 10800.5 | 60 | 0.025 | 0.005 |
| 10806.1 | 60 | 0.025 | 0.065 |
| 10814.4 | 60 | 0.055 | -0.025 |
| 10816.3 | 60 | 0.025 | 0.005 |
| 10819.1 | 60 | 0.025 | -0.025 |
| 10827.4 | 60 | -0.005 | 0.035 |
| 10829.3 | 60 | 0.025 | 0.005 |
| 10834.8 | 60 | 0.025 | 0.065 |
| 10843.2 | 60 | 0.055 | -0.025 |
| 10845.0 | 60 | 0.025 | 0.005 |
| 10847.8 | 60 | 0.025 | -0.025 |
| 10856.2 | 60 | -0.005 | 0.035 |
| 10858.0 | 60 | 0.025 | 0.005 |
| 10863.6 | 60 | 0.025 | 0.065 |
| 10871.9 | 60 | 0.055 | -0.025 |
| 10873.8 | 60 | 0.025 | 0.005 |
| 10876.6 | 60 | 0.025 | -0.025 |
| 10884.9 | 60 | -0.005 | 0.035 |
| 10886.8 | 60 | 0.025 | 0.005 |
| 10889.6 | 60 | 0.025 | 0.035 |
| 10897.9 | 60 | 0.055 | -0.025 |
| 10899.8 | 60 | 0.025 | 0.005 |
| 10902.6 | 60 | 0.025 | -0.025 |
| 10908.1 | 60 | -0.005 | 0.035 |
| 10910.0 | 60 | 0.025 | 0.005 |
| 10912.8 | 60 | 0.025 | 0.035 |
| 10921.1 | 60 | 0.055 | -0.025 |
| 10923.0 | 60 | 0.025 | 0.005 |
| 10925.8 | 60 | 0.025 | -0.025 |
| 10931.3 | 60 | -0.005 | 0.035 |
| 10933.2 | 60 | 0.025 | 0.005 |
| 10936.0 | 60 | 0.025 | 0.035 |
| 10944.3 | 60 | 0.055 | -0.025 |
| 10946.2 | 60 | 0.025 | 0.005 |
| 10949.0 | 60 | 0.025 | -0.025 |
| 10954.5 | 60 | -0.005 | 0.035 |
| 10956.4 | 60 | 0.025 | 0.005 |
| 10959.2 | 60 | 0.025 | 0.035 |
| 10967.5 | 60 | 0.055 | -0.025 |
| 10969.4 | 60 | 0.025 | 0.005 |
| 10972.1 | 60 | 0.025 | -0.025 |
| 10977.7 | 60 | -0.005 | 0.035 |
| 10979.6 | 60 | 0.025 | 0.005 |
| 10982.4 | 60 | 0.025 | 0.035 |
| 10990.7 | 60 | 0.055 | -0.025 |
| 10992.6 | 60 | 0.025 | 0.005 |
| 10995.3 | 60 | 0.025 | -0.025 |
| 11000.9 | 60 | -0.005 | 0.035 |
| 11002.8 | 60 | 0.025 | 0.005 |
| 11005.5 | 60 | 0.025 | 0.035 |
| 11013.9 | 60 | 0.055 | -0.025 |
| 11015.8 | 60 | 0.025 | 0.005 |
| 11018.5 | 60 | 0.025 | -0.025 |
| 11024.1 | 60 | -0.005 | 0.035 |
| 11026.0 | 60 | 0.025 | 0.005 |
| 11028.7 | 60 | 0.025 | 0.035 |
| 11037.1 | 60 | 0.055 | -0.025 |
| 11038.9 | 60 | 0.025 | 0.005 |
| 11041.7 | 60 | 0.025 | -0.025 |
| 11047.3 | 60 | -0.005 | 0.035 |
| 11049.2 | 60 | 0.025 | 0.005 |
| 11051.9 | 60 | 0.025 | 0.035 |
| 11057.5 | 60 | 0.055 | 0.005 |
| 11059.4 | 60 | 0.025 | 0.005 |
| 11062.1 | 60 | 0.025 | -0.025 |
| 11067.7 | 60 | -0.005 | 0.035 |
| 11069.6 | 60 | 0.025 | 0.005 |
| 11072.3 | 60 | 0.025 | 0.035 |
| 11077.9 | 60 | 0.055 | 0.005 |
| 11079.8 | 60 | 0.025 | 0.005 |
| 11082.6 | 60 | 0.025 | -0.025 |
| 11088.1 | 60 | -0.005 | 0.035 |
| 11090.0 | 60 | 0.025 | 0.005 |
| 11092.8 | 60 | 0.025 | 0.035 |
| 11098.3 | 60 | 0.055 | 0.005 |
| 11100.2 | 60 | 0.025 | 0.005 |
| 11103.0 | 60 | 0.025 | -0.025 |
| 11108.5 | 60 | -0.005 | 0.035 |
| 11110.4 | 60 | 0.025 | 0.005 |
| 11113.2 | 60 | 0.025 | 0.035 |
| 11118.7 | 60 | 0.055 | 0.005 |
| 11120.6 | 60 | 0.025 | 0.005 |
| 11123.4 | 60 | 0.025 | -0.025 |
| 11128.9 | 60 | -0.005 | 0.035 |
| 11130.8 | 60 | 0.025 | 0.005 |
| 11133.6 | 60 | 0.025 | 0.035 |
| 11139.1 | 60 | 0.055 | 0.005 |
| 11141.0 | 60 | 0.025 | 0.005 |
| 11143.8 | 60 | 0.025 | -0.025 |
| 11149.4 | 60 | -0.005 | 0.035 |
| 11151.2 | 60 | 0.025 | 0.005 |
| 11154.0 | 60 | 0.025 | 0.035 |
| 11159.6 | 60 | 0.055 | 0.005 |
| 11161.4 | 60 | 0.025 | 0.005 |
| 11164.2 | 60 | 0.025 | -0.025 |
| 11169.8 | 60 | -0.005 | 0.035 |
| 11171.6 | 60 | 0.025 | 0.005 |
| 11174.4 | 60 | 0.025 | 0.035 |
| 11180.0 | 60 | 0.055 | 0.005 |
| 11181.8 | 60 | 0.025 | 0.005 |
| 11184.6 | 60 | 0.025 | -0.025 |
| 11187.4 | 60 | -0.005 | 0.005 |
| 11189.2 | 60 | 0.025 | 0.005 |
| 11192.0 | 60 | 0.025 | 0.035 |
| 11197.6 | 60 | 0.055 | 0.005 |
| 11199.5 | 60 | 0.025 | 0.005 |
| 11202.2 | 60 | 0.025 | -0.025 |
| 11205.0 | 60 | -0.005 | 0.005 |
| 11206.9 | 60 | 0.025 | 0.005 |
| 11209.7 | 60 | 0.025 | 0.035 |
| 11215.2 | 60 | 0.055 | 0.005 |
| 11217.1 | 60 | 0.025 | 0.005 |
| 11219.9 | 60 | 0.025 | -0.025 |
| 11222.6 | 60 | -0.005 | 0.005 |
| 11224.5 | 60 | 0.025 | 0.005 |
| 11227.3 | 60 | 0.025 | 0.035 |
| 11232.9 | 60 | 0.055 | 0.005 |
| 11234.7 | 60 | 0.025 | 0.005 |
| 11237.5 | 60 | 0.025 | -0.025 |
| 11240.3 | 60 | -0.005 | 0.005 |
| 11242.1 | 60 | 0.025 | 0.005 |
| 11244.9 | 60 | 0.025 | 0.035 |
| 11250.5 | 60 | 0.055 | 0.005 |
| 11252.3 | 60 | 0.025 | 0.005 |
| 11255.1 | 60 | 0.025 | -0.025 |
| 11257.9 | 60 | -0.005 | 0.005 |
| 11259.8 | 60 | 0.025 | 0.005 |
| 11262.5 | 60 | 0.025 | 0.035 |
| 11268.1 | 60 | 0.055 | 0.005 |
| 11270.0 | 60 | 0.025 | 0.005 |
| 11272.7 | 60 | 0.025 | -0.025 |
| 11275.5 | 60 | -0.005 | 0.005 |
| 11277.4 | 60 | 0.025 | 0.005 |
| 11280.2 | 60 | 0.025 | 0.035 |
| 11285.7 | 60 | 0.055 | 0.005 |
| 11287.6 | 60 | 0.025 | 0.005 |
| 11290.4 | 60 | 0.025 | -0.025 |
| 11293.2 | 60 | -0.005 | 0.005 |
| 11295.0 | 60 | 0.025 | 0.005 |
| 11297.8 | 60 | 0.025 | 0.035 |
| 11303.4 | 60 | 0.055 | 0.005 |
| 11305.2 | 60 | 0.025 | 0.005 |
| 11308.0 | 60 | 0.025 | -0.025 |
| 11310.8 | 60 | -0.005 | 0.005 |
| 11312.6 | 60 | 0.025 | 0.005 |
| 11315.4 | 60 | 0.025 | 0.035 |
| 11321.0 | 60 | 0.055 | 0.005 |
| 11322.8 | 60 | 0.025 | 0.005 |
| 11325.6 | 60 | 0.025 | -0.025 |
| 11328.4 | 60 | -0.005 | 0.005 |
| 11330.3 | 60 | 0.025 | 0.005 |
| 11333.1 | 60 | 0.025 | 0.035 |
| 11338.6 | 60 | 0.055 | 0.005 |
| 11340.5 | 60 | 0.025 | 0.005 |
| 11343.3 | 60 | 0.025 | -0.025 |
| 11346.0 | 60 | -0.005 | 0.005 |
| 11347.9 | 60 | 0.025 | 0.005 |
| 11350.7 | 60 | 0.025 | 0.035 |
| 11356.2 | 60 | 0.055 | 0.005 |
| 11358.1 | 60 | 0.025 | 0.005 |
| 11360.9 | 60 | 0.025 | -0.025 |
| 11363.7 | 60 | -0.005 | 0.005 |
| 11365.5 | 60 | 0.025 | 0.005 |
| 11368.3 | 60 | 0.025 | 0.035 |
| 11371.1 | 60 | 0.055 | 0.005 |
| 11372.9 | 60 | 0.025 | 0.005 |
| 11375.7 | 60 | 0.025 | -0.025 |
| 11378.5 | 60 | -0.005 | 0.005 |
| 11380.4 | 60 | 0.025 | 0.005 |
| 11383.2 | 60 | 0.025 | 0.035 |
| 11385.9 | 60 | 0.055 | 0.005 |
| 11387.8 | 60 | 0.025 | 0.005 |
| 11390.6 | 60 | 0.025 | -0.025 |
| 11393.4 | 60 | -0.005 | 0.005 |
| 11395.2 | 60 | 0.025 | 0.005 |
| 11398.0 | 60 | 0.025 | 0.035 |
| 11400.8 | 60 | 0.055 | 0.005 |
| 11402.6 | 60 | 0.025 | 0.005 |
| 11405.4 | 60 | 0.025 | -0.025 |
| 11408.2 | 60 | -0.005 | 0.005 |
| 11410.1 | 60 | 0.025 | 0.005 |
| 11412.8 | 60 | 0.025 | 0.035 |
| 11415.6 | 60 | 0.055 | 0.005 |
| 11417.5 | 60 | 0.025 | 0.005 |
| 11420.3 | 60 | 0.025 | -0.025 |
| 11423.0 | 60 | -0.005 | 0.005 |
| 11424.9 | 60 | 0.025 | 0.005 |
| 11427.7 | 60 | 0.025 | 0.035 |
| 11430.5 | 60 | 0.055 | 0.005 |
| 11432.3 | 60 | 0.025 | 0.005 |
| 11435.1 | 60 | 0.025 | -0.025 |
| 11437.9 | 60 | -0.005 | 0.005 |
| 11439.7 | 60 | 0.025 | 0.005 |
| 11442.5 | 60 | 0.025 | 0.035 |
| 11445.3 | 60 | 0.055 | 0.005 |
| 11447.2 | 60 | 0.025 | 0.005 |
| 11450.0 | 60 | 0.025 | -0.025 |
| 11452.7 | 60 | -0.005 | 0.005 |
| 11454.6 | 60 | 0.025 | 0.005 |
| 11457.4 | 60 | 0.025 | 0.035 |
| 11460.2 | 60 | 0.055 | 0.005 |
| 11462.0 | 60 | 0.025 | 0.005 |
| 11464.8 | 60 | 0.025 | -0.025 |
| 11467.6 | 60 | -0.005 | 0.005 |
| 11469.4 | 60 | 0.025 | 0.005 |
| 11472.2 | 60 | 0.025 | 0.035 |
| 11475.0 | 60 | 0.055 | 0.005 |
| 11476.9 | 60 | 0.025 | 0.005 |
| 11479.6 | 60 | 0.025 | -0.025 |
| 11482.4 | 60 | -0.005 | 0.005 |
| 11484.3 | 60 | 0.025 | 0.005 |
| 11487.1 | 60 | 0.025 | 0.035 |
| 11489.8 | 60 | 0.055 | 0.005 |
| 11491.7 | 60 | 0.025 | 0.005 |
| 11494.5 | 60 | 0.025 | -0.025 |
| 11497.3 | 60 | -0.005 | 0.005 |
| 11499.1 | 60 | 0.025 | 0.005 |
| 11501.9 | 60 | 0.025 | 0.035 |
| 11504.7 | 60 | 0.055 | 0.005 |
| 11506.6 | 60 | 0.025 | 0.005 |
| 11509.3 | 60 | 0.025 | -0.025 |
| 11512.1 | 60 | -0.005 | 0.005 |
| 11514.0 | 60 | 0.025 | 0.005 |
| 11516.8 | 60 | 0.025 | 0.035 |
| 11519.5 | 60 | 0.055 | 0.005 |
| 11521.4 | 60 | 0.025 | 0.005 |
| 11524.2 | 60 | 0.025 | -0.025 |
| 11527.0 | 60 | -0.005 | 0.005 |
| 11528.8 | 60 | 0.025 | 0.005 |
| 11531.6 | 60 | 0.025 | 0.035 |
| 11534.4 | 60 | 0.055 | 0.005 |
| 11536.2 | 60 | 0.025 | 0.005 |
| 11539.0 | 60 | 0.025 | -0.025 |
| 11541.8 | 60 | -0.005 | 0.005 |
| 11543.7 | 60 | 0.025 | 0.005 |
| 11546.4 | 60 | 0.025 | 0.035 |
| 11549.2 | 60 | 0.055 | 0.005 |
| 11551.1 | 60 | 0.025 | 0.005 |
| 11553.9 | 60 | 0.025 | -0.025 |
| 11556.7 | 60 | -0.005 | 0.005 |
| 11558.5 | 60 | 0.025 | 0.005 |
| 11561.3 | 60 | 0.025 | 0.035 |
| 11564.1 | 60 | 0.055 | 0.005 |
| 11565.9 | 60 | 0.025 | 0.005 |
| 11568.7 | 60 | 0.025 | -0.025 |
| 11571.5 | 60 | -0.005 | 0.005 |
| 11573.4 | 60 | 0.025 | 0.005 |
| 11576.1 | 60 | 0.025 | 0.035 |
| 11578.9 | 60 | 0.055 | 0.005 |
| 11580.8 | 60 | 0.025 | 0.005 |
| 11583.6 | 60 | 0.025 | -0.025 |
| 11586.3 | 60 | -0.005 | 0.005 |
| 11588.2 | 60 | 0.025 | 0.005 |
| 11591.0 | 60 | 0.025 | 0.035 |
| 11593.8 | 60 | 0.055 | 0.005 |
| 11595.6 | 60 | 0.025 | 0.005 |
| 11598.4 | 60 | 0.025 | -0.025 |
| 11601.2 | 60 | -0.005 | 0.005 |
| 11603.0 | 60 | 0.025 | 0.005 |
| 11605.8 | 60 | 0.025 | 0.035 |
| 11608.6 | 60 | 0.055 | 0.005 |
| 11610.5 | 60 | 0.025 | 0.005 |
| 11613.2 | 60 | 0.025 | -0.025 |
| 11616.0 | 60 | -0.005 | 0.005 |
| 11617.9 | 60 | 0.025 | 0.005 |
| 11620.7 | 60 | 0.025 | 0.035 |
| 11623.5 | 60 | 0.055 | 0.005 |
| 11625.3 | 60 | 0.025 | 0.005 |
| 11628.1 | 60 | 0.025 | -0.025 |
| 11630.9 | 60 | -0.005 | 0.005 |
| 11632.7 | 60 | 0.025 | 0.005 |
| 11635.5 | 60 | 0.025 | 0.035 |
| 11638.3 | 60 | 0.055 | 0.005 |
| 11640.2 | 60 | 0.025 | 0.005 |
| 11642.9 | 60 | 0.025 | -0.025 |
| 11645.7 | 60 | -0.005 | 0.005 |
| 11647.6 | 60 | 0.025 | 0.005 |
| 11650.4 | 60 | 0.025 | 0.035 |
| 11653.1 | 60 | 0.055 | 0.005 |
| 11655.0 | 60 | 0.025 | 0.005 |
| 11657.8 | 60 | 0.025 | -0.025 |
| 11660.6 | 60 | -0.005 | 0.005 |
| 11662.4 | 60 | 0.025 | 0.005 |
| 11665.2 | 60 | 0.025 | 0.035 |
| 11668.0 | 60 | 0.055 | 0.005 |
| 11669.8 | 60 | 0.025 | 0.005 |
| 11672.6 | 60 | 0.025 | -0.025 |
| 11675.4 | 60 | -0.005 | 0.005 |
| 11677.3 | 60 | 0.025 | 0.005 |
| 11680.0 | 60 | 0.025 | 0.035 |
| 11682.8 | 60 | 0.055 | 0.005 |
| 11684.7 | 60 | 0.025 | 0.005 |
| 11687.5 | 60 | 0.025 | -0.025 |
| 11690.3 | 60 | -0.005 | 0.005 |
| 11692.1 | 60 | 0.025 | 0.005 |
| 11694.9 | 60 | 0.025 | 0.035 |
| 11697.7 | 60 | 0.055 | 0.005 |
| 11699.5 | 60 | 0.025 | 0.005 |
| 11702.3 | 60 | 0.025 | -0.025 |
| 11705.1 | 60 | -0.005 | 0.005 |
| 11707.0 | 60 | 0.025 | 0.005 |
| 11709.7 | 60 | 0.025 | 0.035 |
| 11712.5 | 60 | 0.055 | 0.005 |
| 11714.4 | 60 | 0.025 | 0.005 |
| 11717.2 | 60 | 0.025 | -0.025 |
| 11719.9 | 60 | -0.005 | 0.005 |
| 11721.8 | 60 | 0.025 | 0.005 |
| 11724.6 | 60 | 0.025 | 0.035 |
| 11727.4 | 60 | 0.055 | 0.005 |
| 11729.2 | 60 | 0.025 | 0.005 |
| 11732.0 | 60 | 0.025 | -0.025 |
| 11734.8 | 60 | -0.005 | 0.005 |
| 11736.6 | 60 | 0.025 | 0.005 |
| 11739.4 | 60 | 0.025 | 0.035 |
| 11742.2 | 60 | 0.055 | 0.005 |
| 11744.1 | 60 | 0.025 | 0.005 |
| 11746.8 | 60 | 0.025 | -0.025 |
| 11749.6 | 60 | -0.005 | 0.005 |
| 11751.5 | 60 | 0.025 | 0.005 |
| 11754.3 | 60 | 0.025 | 0.035 |
| 11757.1 | 60 | 0.055 | 0.005 |
| 11758.9 | 60 | 0.025 | 0.005 |
| 11761.7 | 60 | 0.025 | -0.025 |
| 11764.5 | 60 | -0.005 | 0.005 |
| 11766.3 | 60 | 0.025 | 0.005 |
| 11769.1 | 60 | 0.025 | 0.035 |
| 11771.9 | 60 | 0.055 | 0.005 |
| 11773.8 | 60 | 0.025 | 0.005 |
| 11776.5 | 60 | 0.025 | -0.025 |
| 11779.3 | 60 | -0.005 | 0.005 |
| 11781.2 | 60 | 0.025 | 0.005 |
| 11784.0 | 60 | 0.025 | 0.035 |
| 11786.7 | 60 | 0.055 | 0.005 |
| 11788.6 | 60 | 0.025 | 0.005 |
| 11791.4 | 60 | 0.025 | -0.025 |
| 11794.2 | 60 | -0.005 | 0.005 |
| 11796.0 | 60 | 0.025 | 0.005 |
| 11798.8 | 60 | 0.025 | 0.035 |
| 11801.6 | 60 | 0.055 | 0.005 |
| 11803.4 | 60 | 0.025 | 0.005 |
| 11806.2 | 60 | 0.025 | -0.025 |
| 11809.0 | 60 | -0.005 | 0.005 |
| 11810.9 | 60 | 0.025 | 0.005 |
| 11813.6 | 60 | 0.025 | 0.035 |
| 11816.4 | 60 | 0.055 | 0.005 |
| 11818.3 | 60 | 0.025 | 0.005 |
| 11821.1 | 60 | 0.025 | -0.025 |
| 11823.9 | 60 | -0.005 | 0.005 |
| 11825.7 | 60 | 0.025 | 0.005 |
| 11828.5 | 60 | 0.025 | 0.035 |
| 11831.3 | 60 | 0.055 | 0.005 |
| 11833.1 | 60 | 0.025 | 0.005 |
| 11835.9 | 60 | 0.025 | -0.025 |
| 11838.7 | 60 | -0.005 | 0.005 |
| 11840.6 | 60 | 0.025 | 0.005 |
| 11843.3 | 60 | 0.025 | 0.035 |
| 11846.1 | 60 | 0.055 | 0.005 |
| 11848.0 | 60 | 0.025 | 0.005 |
| 11850.8 | 60 | 0.025 | -0.025 |
| 11853.5 | 60 | -0.005 | 0.005 |
| 11855.4 | 60 | 0.025 | 0.005 |
| 11858.2 | 60 | 0.025 | 0.035 |
| 11861.0 | 60 | 0.055 | 0.005 |
| 11862.8 | 60 | 0.025 | 0.005 |
| 11865.6 | 60 | 0.025 | -0.025 |
| 11868.4 | 60 | -0.005 | 0.005 |
| 11870.2 | 60 | 0.025 | 0.005 |
| 11873.0 | 60 | 0.025 | 0.035 |
| 11875.8 | 60 | 0.055 | 0.005 |
| 11877.7 | 60 | 0.025 | 0.005 |
| 11883.2 | 60 | 0.025 | 0.005 |
| 11886.0 | 60 | -0.005 | 0.005 |
| 11887.9 | 60 | 0.025 | 0.005 |
| 11890.7 | 60 | 0.025 | 0.035 |
| 11893.4 | 60 | 0.055 | 0.005 |
| 11895.3 | 60 | 0.025 | 0.005 |
| 11900.9 | 60 | 0.025 | 0.005 |
| 11903.6 | 60 | -0.005 | 0.005 |
| 11905.5 | 60 | 0.025 | 0.005 |
| 11908.3 | 60 | 0.025 | 0.035 |
| 11911.1 | 60 | 0.055 | 0.005 |
| 11912.9 | 60 | 0.025 | 0.005 |
| 11918.5 | 60 | 0.025 | 0.005 |
| 11921.3 | 60 | -0.005 | 0.005 |
| 11923.1 | 60 | 0.025 | 0.005 |
| 11925.9 | 60 | 0.025 | 0.035 |
| 11928.7 | 60 | 0.055 | 0.005 |
| 11930.5 | 60 | 0.025 | 0.005 |
| 11936.1 | 60 | 0.025 | 0.005 |
| 11938.9 | 60 | -0.005 | 0.005 |
| 11940.8 | 60 | 0.025 | 0.005 |
| 11943.5 | 60 | 0.025 | 0.035 |
| 11946.3 | 60 | 0.055 | 0.005 |
| 11948.2 | 60 | 0.025 | 0.005 |
| 11953.7 | 60 | 0.025 | 0.005 |
| 11956.5 | 60 | -0.005 | 0.005 |
| 11958.4 | 60 | 0.025 | 0.005 |
| 11961.2 | 60 | 0.025 | 0.035 |
| 11963.9 | 60 | 0.055 | 0.005 |
| 11965.8 | 60 | 0.025 | 0.005 |
| 11971.4 | 60 | 0.025 | 0.005 |
| 11974.2 | 60 | -0.005 | 0.005 |
| 11976.0 | 60 | 0.025 | 0.005 |
| 11978.8 | 60 | 0.025 | 0.035 |
| 11981.6 | 60 | 0.055 | 0.005 |
| 11983.4 | 60 | 0.025 | 0.005 |
| 11989.0 | 60 | 0.025 | 0.005 |
| 11991.8 | 60 | -0.005 | 0.005 |
| 11993.6 | 60 | 0.025 | 0.005 |
| 11996.4 | 60 | 0.025 | 0.035 |
| 11999.2 | 60 | 0.055 | 0.005 |
| 12001.1 | 60 | 0.025 | 0.005 |
| 12006.6 | 60 | 0.025 | 0.005 |
| 12009.4 | 60 | -0.005 | 0.005 |
| 12011.3 | 60 | 0.025 | 0.005 |
| 12014.0 | 60 | 0.025 | 0.035 |
| 12016.8 | 60 | 0.055 | 0.005 |
| 12018.7 | 60 | 0.025 | 0.005 |
| 12024.3 | 60 | 0.025 | 0.005 |
| 12027.0 | 60 | -0.005 | 0.005 |
| 12028.9 | 60 | 0.025 | 0.005 |
| 12031.7 | 60 | 0.025 | 0.035 |
| 12034.5 | 60 | 0.055 | 0.005 |
| 12036.3 | 60 | 0.025 | 0.005 |
| 12041.9 | 60 | 0.025 | 0.005 |
| 12044.7 | 60 | -0.005 | 0.005 |
| 12046.5 | 60 | 0.025 | 0.005 |
| 12049.3 | 60 | 0.025 | 0.035 |
| 12052.1 | 60 | 0.055 | 0.005 |
| 12053.9 | 60 | 0.025 | 0.005 |
| 12059.5 | 60 | 0.025 | 0.005 |
| 12062.3 | 60 | -0.005 | 0.005 |
| 12064.1 | 60 | 0.025 | 0.005 |
| 12066.9 | 60 | 0.025 | 0.035 |
| 12069.7 | 60 | 0.055 | 0.005 |
| 12071.6 | 60 | 0.025 | 0.005 |
| 12077.1 | 60 | 0.025 | 0.005 |
| 12079.9 | 60 | -0.005 | 0.005 |
| 12081.8 | 60 | 0.025 | 0.005 |
| 12084.6 | 60 | 0.025 | 0.035 |
| 12087.3 | 60 | 0.055 | 0.005 |
| 12089.2 | 60 | 0.025 | 0.005 |
| 12094.8 | 60 | 0.025 | 0.005 |
| 12097.5 | 60 | -0.005 | 0.005 |
| 12099.4 | 60 | 0.025 | 0.005 |
| 12102.2 | 60 | 0.025 | 0.035 |
| 12105.0 | 60 | 0.055 | 0.005 |
| 12106.8 | 60 | 0.025 | 0.005 |
| 12112.4 | 60 | 0.025 | 0.005 |
| 12115.2 | 60 | -0.005 | 0.005 |
| 12117.0 | 60 | 0.025 | 0.005 |
| 12119.8 | 60 | 0.025 | 0.035 |
| 12122.6 | 60 | 0.055 | 0.005 |
| 12124.5 | 60 | 0.025 | 0.005 |
| 12130.0 | 60 | 0.025 | 0.005 |
| 12132.8 | 60 | -0.005 | 0.005 |
| 12134.7 | 60 | 0.025 | 0.005 |
| 12137.4 | 60 | 0.025 | 0.035 |
| 12140.2 | 60 | 0.055 | 0.005 |
| 12142.1 | 60 | 0.025 | 0.005 |
| 12147.6 | 60 | 0.025 | 0.005 |
| 12150.4 | 60 | -0.005 | 0.005 |
| 12152.3 | 60 | 0.025 | 0.005 |
| 12155.1 | 60 | 0.025 | 0.035 |
| 12157.9 | 60 | 0.055 | 0.005 |
| 12159.7 | 60 | 0.025 | 0.005 |
| 12165.3 | 60 | 0.025 | 0.005 |
| 12168.1 | 60 | -0.005 | 0.005 |
| 12169.9 | 60 | 0.025 | 0.005 |
| 12172.7 | 60 | 0.025 | 0.035 |
| 12175.5 | 60 | 0.055 | 0.005 |
| 12177.3 | 60 | 0.025 | 0.005 |
| 12182.9 | 60 | 0.025 | 0.005 |
| 12185.7 | 60 | -0.005 | 0.005 |
| 12187.5 | 60 | 0.025 | 0.005 |
| 12190.3 | 60 | 0.025 | 0.035 |
| 12193.1 | 60 | 0.055 | 0.005 |
| 12195.0 | 60 | 0.025 | 0.005 |
| 12200.5 | 60 | 0.025 | 0.005 |
| 12203.3 | 60 | -0.005 | 0.005 |
| 12205.2 | 60 | 0.025 | 0.005 |
| 12208.0 | 60 | 0.025 | 0.035 |
| 12210.7 | 60 | 0.055 | 0.005 |
| 12212.6 | 60 | 0.025 | 0.005 |
| 12218.2 | 60 | 0.025 | 0.005 |
| 12220.9 | 60 | -0.005 | 0.005 |
| 12222.8 | 60 | 0.025 | 0.005 |
| 12225.6 | 60 | 0.025 | 0.035 |
| 12228.4 | 60 | 0.055 | 0.005 |
| 12230.2 | 60 | 0.025 | 0.005 |
| 12235.8 | 60 | 0.025 | 0.005 |
| 12238.6 | 60 | -0.005 | 0.005 |
| 12240.4 | 60 | 0.025 | 0.005 |
| 12243.2 | 60 | 0.025 | 0.035 |
| 12246.0 | 60 | 0.055 | 0.005 |
| 12247.8 | 60 | 0.025 | 0.005 |
| 12253.4 | 60 | 0.025 | 0.005 |
| 12256.2 | 60 | -0.005 | 0.005 |
| 12258.1 | 60 | 0.025 | 0.005 |
| 12260.8 | 60 | 0.025 | 0.035 |
| 12263.6 | 60 | 0.055 | 0.005 |
| 12265.5 | 60 | 0.025 | 0.005 |
| 12271.0 | 60 | 0.025 | 0.005 |
| 12273.8 | 60 | -0.005 | 0.005 |
| 12275.7 | 60 | 0.025 | 0.005 |
| 12278.5 | 60 | 0.025 | 0.035 |
| 12281.2 | 60 | 0.055 | 0.005 |
| 12283.1 | 60 | 0.025 | 0.005 |
| 12288.7 | 60 | 0.025 | 0.005 |
| 12291.5 | 60 | -0.005 | 0.005 |
| 12293.3 | 60 | 0.025 | 0.005 |
| 12296.1 | 60 | 0.025 | 0.035 |
| 12298.9 | 60 | 0.055 | 0.005 |
| 12300.7 | 60 | 0.025 | 0.005 |
| 12306.3 | 60 | 0.025 | 0.005 |
| 12309.1 | 60 | -0.005 | 0.005 |
| 12310.9 | 60 | 0.025 | 0.005 |
| 12313.7 | 60 | 0.025 | 0.035 |
| 12316.5 | 60 | 0.055 | 0.005 |
| 12318.4 | 60 | 0.025 | 0.005 |
| 12323.9 | 60 | 0.025 | 0.005 |
| 12326.7 | 60 | -0.005 | 0.005 |
| 12328.6 | 60 | 0.025 | 0.005 |
| 12331.3 | 60 | 0.025 | 0.035 |
| 12334.1 | 60 | 0.055 | 0.005 |
| 12336.0 | 60 | 0.025 | 0.005 |
| 12341.6 | 60 | 0.025 | 0.005 |
| 12344.3 | 60 | -0.005 | 0.005 |
| 12346.2 | 60 | 0.025 | 0.005 |
| 12349.0 | 60 | 0.025 | 0.035 |
| 12351.8 | 60 | 0.055 | 0.005 |
| 12353.6 | 60 | 0.025 | 0.005 |
| 12359.2 | 60 | 0.025 | 0.005 |
| 12362.0 | 60 | -0.005 | 0.005 |
| 12363.8 | 60 | 0.025 | 0.005 |
| 12366.6 | 60 | 0.025 | 0.035 |
| 12369.4 | 60 | 0.055 | 0.005 |
| 12371.2 | 60 | 0.025 | 0.005 |
| 12376.8 | 60 | 0.025 | 0.005 |
| 12379.6 | 60 | -0.005 | 0.005 |
| 12381.4 | 60 | 0.025 | 0.005 |
| 12384.2 | 60 | 0.025 | 0.035 |
| 12387.0 | 60 | 0.055 | 0.005 |
| 12388.9 | 60 | 0.025 | 0.005 |
| 12394.4 | 60 | 0.025 | 0.005 |
| 12397.2 | 60 | -0.005 | 0.005 |
| 12399.1 | 60 | 0.025 | 0.005 |
| 12401.9 | 60 | 0.025 | 0.035 |
| 12404.6 | 60 | 0.055 | 0.005 |
| 12406.5 | 60 | 0.025 | 0.005 |
| 12412.1 | 60 | 0.025 | 0.005 |
| 12414.8 | 60 | -0.005 | 0.005 |
| 12416.7 | 60 | 0.025 | 0.005 |
| 12419.5 | 60 | 0.025 | 0.035 |
| 12422.3 | 60 | 0.055 | 0.005 |
| 12424.1 | 60 | 0.025 | 0.005 |
| 12429.7 | 60 | 0.025 | 0.005 |
| 12432.5 | 60 | -0.005 | 0.005 |
| 12434.3 | 60 | 0.025 | 0.005 |
| 12437.1 | 60 | 0.025 | 0.035 |
| 12439.9 | 60 | 0.055 | 0.005 |
| 12441.8 | 60 | 0.025 | 0.005 |
| 12447.3 | 60 | 0.025 | 0.005 |
| 12450.1 | 60 | -0.005 | 0.005 |
| 12452.0 | 60 | 0.025 | 0.005 |
| 12454.7 | 60 | 0.025 | 0.035 |
| 12457.5 | 60 | 0.055 | 0.005 |
| 12459.4 | 60 | 0.025 | 0.005 |
| 12464.9 | 60 | 0.025 | 0.005 |
| 12467.7 | 60 | -0.005 | 0.005 |
| 12469.6 | 60 | 0.025 | 0.005 |
| 12475.2 | 60 | 0.025 | 0.005 |
| 12477.9 | 60 | 0.055 | 0.005 |
| 12479.8 | 60 | 0.025 | 0.005 |
| 12485.4 | 60 | 0.025 | 0.005 |
| 12488.1 | 60 | -0.005 | 0.005 |
| 12490.0 | 60 | 0.025 | 0.005 |

Annex 3 – Appendix 4

Test equipment tolerances

Table B.1

Instrumentation accuracy

| Parameter | Control accuracy | Instrumentation accuracy |
| --- | --- | --- |
| Tyre forces (using filtered values) | Fz: ± 50 N or 1 % using filtered values whichever is greater. | at full scale: Fz: ± 1 % |
|  | Fy: ± 100 N or 5 % using filtered values whichever is greater, for the difference between input peaks and actually generated peaks. | at full scale: Fy: ± 1 % |
|  | Fx: ± 100 N or 5 % using filtered values whichever is greater, for the difference between input peaks and actually generated peaks. | at full scale: Fx: ± 1 % |
| Tyre torque | My: ± 40 Nm or 5 % using filtered values whichever is greater, for the difference between input peaks and actually generated peaks. | at full scale: My: ± 1 % |
| Inflation pressure | Not applicable | ± 3 kPa |
| Mass scale | Not applicable | ± 2 g |
| Test duration | For the test time durations, the total time of an actual test shall not differ more than ± 5 % from the total input time, 69.39h (249800s). The interval of measurement shall be more than 1Hz. | The accuracy of measuring one second is within ± 0.02 s |
| Camber angle | (0 ±+/- 0.1) degrees | (0 ±+/- 0.1) degrees |
| Temperature | ±3 °C | ±0.5 °C |
| Speed | ±2 km/h | ± 0.1 % at full scale or ± 0.3 km / h, whichever is greater |
| Loaded radius | Not applicable | ±1 mm |

Annex 3 – Appendix 5

Replacement of sandpaper surface (if applicable)

Sandpaper A sandpaper surface shall be replaced when it does not meet the specifications described in paragraph 2.4.2.3. of this Annex; partial replacement is also allowed. In the case that the drum diameter is 3 m, Sandpaper the sandpaper surface should be replaced either when the running distance reaches:

(a)	20000 km for 2 positions drum Running distance reached 20000 km for 2 positions drum in case of 3 m,

(b)	 40000 km for 1 position drum in case of 3 m, [or partial replacement is also allowed]

In case ofIn the case that the drum diameter is not 3 3 m, the following formula applies:

Road surface replacement distance (km) = Road surface replacement distance (standard value) x Drum diameter owned by each testing institution (m) / Standard drum diameter (m)

Where:

Road surface replacement distance (standard value) = 20000 km;

Standard drum diameter = 3 m.

Annex 3 – Appendix 6

Example of a test report for indoor drum test method

The test report shall include the following information.

| Test report number: |  |  | Test starting date: | ~ |  |
| --- | --- | --- | --- | --- | --- |
| Test machine identification: |  |  | Test end date: |  |  |
| Drum circumference diameter (m),): |  |  |  |  |  |
| Test cycle (2 positions /1 position): |  |  |  | Beginning of test | End of test |
| Talc or Silica: | Talc or Silica |  | MPD of test surface (mm): |  |  |
|  |  |  | Micro-roughness of test surface (mm) |  |  |

| Type of test tyre |  | Reference tyre | Candidate tyre |
| --- | --- | --- | --- |
| Tyre class: |  |  |  |
| Brand: |  |  |  |
| Pattern/trade description: |  | SRTT… |  |
| Tyre size designation: |  |  |  |
| Service description: |  |  |  |
| Test load (N): |  |  |  |
| Test inflation pressure (kPa): |  |  |  |
| Tyre identification: |  |  |  |
| 3PMSF marking (Yes/No)7: |  |  |  |
| M+S marking (Yes/No) 7: |  |  |  |
| XL marking (Yes/No)7: |  |  |  |
| Tyres with a nominal aspect ratio ≤ 40 and suitable for speeds ≥ 300 km/h (Yes/No)7: |  |  |  |
| [Tyres with low load index (LI < 77) (Yes/No)7:] |  |  |  |
| Rim width code: |  | 7.5 |  |
| Inflation pressure (kPa) | Beginning of test: |  |  |
|  | End of test: |  |  |
| Mass of tyre (g) | Before test: |  |  |
|  | After test: |  |  |
| Test distance (km): |  |  |  |
| Abrasion rate (mg/km): |  |  |  |
| Abrasion level (mg/(km∙t)): |  |  |  |
| Amargin applied: |  |  |  |
| Abrasion index (AICT): |  |  |  |
| Average ambient temp. (°C): |  |  |  |
| RMS of G(x): |  |  |  |
| RMS of G(y): |  |  |  |
| RMS of G(x,y): |  |  |  |
| Average of Fz: |  |  |  |
| Amount of powder sprayed relative to reference tyre: |  |  |  |
| Remarks: |  |  |  |

II.	Justification

- The amendments proposed are the result of the work of the Task Force Tyre Abrasion done since the submission of document ECE/TRANS/WP.29/GRBP/2025/27.

* For the purpose of this Regulation "tyres" means "pneumatic tyres".

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3).

Class C1 tyres correspond to "passenger car tyres" in ISO 4000-1:2024.

Some of these requirements may be specified separately in UN Regulation No. 30.

4	The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3).

Distinguishing number of the country which has granted/extended/refused/withdrawn approval 
(see approval provisions in the Regulation).

Strike out what does not apply.

According to Schedule 4 to Revision 3 of the 1958 Agreement.

According to paragraph 4.3.2.3. of this Regulation

strike out what does not apply.