Modifications to ECE/TRANS/WP.29/GRSP/2025/15*

I.	Proposal

Section 0., at the end, insert new paragraphs 0.1. to 0.1.3., to read:

“0.1.	For supplement [3] to the 10 series of amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.2.	The Regulation was originally drafted for vehicles with driver and manual driving controls and it is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and manual driving controls. In the absence of driver/manual driving controls, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of category Xcategories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.”

Paragraph 1.2., amend to read:

“1.2.	At the request of the manufacturer, it also applies to safety-belts designated for installation on side-facing seats in vehicles of category M3 (Class II, III or B1).

This Regulation does not apply to vehicles of category Y.”

Paragraph 1., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 2.15.1., amend to read:

“2.15.1.	"A front passenger seat" means any seat where the "foremost H-point" of the seat in question is in or in front of the vertical transverse plane through the driver's R-point. For vehicles of category X, the "R" point of the most forward passenger seat shall be taken instead of the R-point of the driver's seat.”

Insert new paragraphs 2.26. to 2.27., to read:

“2.26.	"ADS feature of type 1 (ADSF-1)" means an ADS1 feature which includes an ADS fallback response requiring a fallback user.

2.27.	"ADS feature of type 2 (ADSF-2)" means an ADS feature which does not include an ADS fallback response requiring a fallback user.”

Paragraph  6.2.5.3.1.5., amend to read:

“6.2.5.3.1.5.	In cases where the operation of a retractor depends on an external signal or power source, the design shall ensure that the retractor locks automatically upon failure or interruption of that signal or power source. However, this requirement need not be met in the case of a retractor with multiple sensitivities, provided only one sensitivity is dependent on an external signal or power source and the failure of the signal or power source is indicated to the driver by optical and/or acoustical means or whilst an ADSF-2 is active, transmitted as a logic signal to the ADS.”

Paragraph 6.4.1.4.1.1., amend to read:

“6.4.1.4.1.1.	In the case of an occupant having a steering control in front the driver, contact of the chest with the steering control assembly would be allowed, if the latter meets the requirements of UN Regulation No. 12 and provided contact does not occur at a speed higher than 24 km/h. For this assessment the seat shall be considered to be in the positions specified in paragraph 7.7.1.5. below.”

Paragraph 7.10.1. (i), amend to read:

“	(i)	For any restraint system other than the one for the driver the contour of the contact should be evaluated versus the movement of the manikin in the sled by the technical service in this report.”

Annex 9, paragraph 2., amend to read:

“2.		In the case of safety-belts fitted with a type 4N retractor, it shall be indicated in the installation instructions and on any packaging that this belt is not suitable for installation in motor vehicles used for the carriage of passengers having not more than nine seats, including that of the driver.”

II.	Justification

See paragraph 0. in the proposal.

________________

*	Proposal for supplement [3] to the 10 series of amendments to UN Regulation No. 16 (Safety-belts)