Proposal for amendments to ECE/TRANS/WP.29/2026/25 (Proposal for the 09 series of amendments to UN Regulation No. 83 (Emissions of M1 and N1 vehicles))

This document proposes an amendment to the 09 series of amendments to UN Regulation No. 83 which was adopted at the 198th session of WP.29 in March 2026.

Proposal

Paragraph 12.2.3.(a), amend to read:

"12.2.3.	…

(a)	The requirements for vehicles approved under the character GL or GE as defined in Table A3/1, Annex 3 of this Regulation, as amended by the 09 series of amendments;

…"

Annex 3, Table A3/1, replace to read:

# "Table A3/1

Letters with reference to emission standard and vehicle category*

| Emission Character (1) (2) | Emission standard | Sub-character for this regulation (see Table A3/2) | Sub-character for OBM and EVP (see Table A3/3) | Vehicle category | Sub-character for brake emissions (see Table A3/4) |
| --- | --- | --- | --- | --- | --- |
| FL | Euro 7A | MA, ME or MG | OA | M1, N1 | RA |
| FE | Euro 7A | MB, MF or MG | OB | N2 (Euro 7ext3) | RB |
| GL | Euro 7B | MG | OA or OB | M1, N1, N2 (Euro 7ext3) | RC |
| GS | Euro 7BS | NA | PA | M1 (SVM), N1 (SVM) | RD |
| GT | Euro 7BT | NB | PB | N2 (Euro 7ext3, SVM) | RE |

(1)	First letter of Character: F = Euro 7A, G = Euro 7B

(2)	Second letter of Character: L = large volume manufacturer, E = Euro 7ext-vehicle, S = SVM, T = Euro 7ext-vehicle from SVM

(3)	As defined in Article 5(2) of Regulation (EU) 2024/1257

(*)	Contracting parties outside the European Union are not obliged to accept an approval to emission standards Euro 7BS or Euro 7BT but may choose to set out criteria for acceptance in their national regulations.”

Annex 3, Table A3/2, replace to read:

# "Table A3/2

Sub-character table with reference to vehicle category and powertrain

| Sub-character | Description | Vehicle category (5) | Powertrain |
| --- | --- | --- | --- |
| MA | R154-04 | M1, N1 | ICEV |
| ME | R154-04 UF EC (4) | M1, N1 | NOVC-HEV, OVC-HEV |
| MB | R154-04 | N2 | ICEV |
| MF | R154-04 UF EC (4) | N2 | NOVC-HEV, OVC-HEV |
| ME | R154-04 | M1, N1 | PEV, FCHV |
| MF | R154-04 | N2 | PEV, FCHV |
| MG | R154-04 – WITH OBFCM-2 (6) | M1, N1, N2 | All |
| NA | R154-04 | M1 (SVM), N1 (SVM) | All |
| NB | R154-04 | N2 (SVM) | All |

(4)	For OVC-HEVs using the utility factor from Euro 6e-bis-FCM (EC), applying the parameter dnec for the determination of the fractional UF according to Appendix 5 of Annex B8 to UN Regulation No. 154.

(5)	When referred to ‘N2’ in this column, this is only applicable for Euro 7ext vehicles as defined in Article 5(2) of Regulation (EU) 2024/1257.

(6)	OBFCM-2: lifetime retention value for all applicable powertrains, additional parameters for OVC-HEV and new requirements for PEVs according to Paragraph 11 and Appendix 5 to UN Regulation No.154.

Annex 3, Table A3/3, column header of third column, amend to read:

"Vehicle category (1) or product type"

Annex 3, Table A3/3, add footnote (1) to read:

"(1) When referred to ‘N2’ in this column, this is only applicable for Euro 7ext vehicles as defined in Article 5(2) of Regulation (EU) 2024/1257.

Annex 3, add a new Table A3/4 to read:

"Table A3/4

Sub-character table for compliance with UN Regulation No. [179] on Brake Particle Emissions

| Sub-character | Description | Vehicle category (1) | Powertrain |
| --- | --- | --- | --- |
| RA | Compliance to UN Regulation No. [179] in its original version | M1, N1 | All |
| RB |  | N2 | All |
| RC | Compliance to 01 series of amendments of UN Regulation No. [179] | M1, N1, N2 | All |
| RD |  | M1 (SVM), N1 (SVM) | All |
| RE |  | N2 (SVM) | All |

(1) When referred to ‘N2’ in this column, this is only applicable for Euro 7ext vehicles as defined in Article 5(2) of Regulation (EU) 2024/1257. "