Proposal for supplement [2] to UN Regulation No.168

(to be agreed and submitted to GRPE by TF-AVRS)

Notes:

Modifications to the existing text of the Regulation indicated in bold for new characters and strikethrough for deleted characters.

I.	Proposal

Paragraph 1., amend to read:

1. 	Scope and application

This Regulation aims at providing a worldwide harmonized method to determine the levels of Real Driving Emissions (RDE) of gaseous compounds and particles from light-duty vehicles.

This Regulation applies to the type approval of vehicles of categories M11 with a reference mass not exceeding 2,610 kg and vehicles of categories M2 and N11 with a reference mass not exceeding 2,610 kg and a technical permissible maximum laden mass not exceeding 3,500 kg with regard to their Real Driving Emissions.

At the manufacturer's request, type approval granted under this Regulation may be extended from vehicles mentioned above to vehicles of categories M1 with a reference mass not exceeding 2,840 kg and vehicles of categories M2 and N1 with a reference mass not exceeding 2,840 kg and a technical permissible maximum laden mass not exceeding 3,500 kg and which meet the conditions laid down in this Regulation.

Pure Electric Vehicles and Fuel Cell Vehicles are out of the scope of this Regulation.

Paragraph 1., new Footnote 1, insert to read:

"1 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2. - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 3.3., amend to read:

"3.3.	Vehicle characteristics and driver (if applicable)"

Paragraph 3.3.3., amend to read:

"3.3.3.	"Mass in running order" means the mass of the vehicle, with its fuel tank(s) filled to at least 90 per cent of its or their capacity/capacities, including the mass of the driver, fuel and liquids, fitted with the standard equipment in accordance with the manufacturer's specifications and, when they are fitted, the mass of the bodywork, the cabin, the coupling and the spare wheel(s) as well as the tools. For vehicles of categories X and Y1 the mass of the driver shall not be included."

Paragraph 3.3.5., amend to read:

"3.3.5.	"Odometer" means an instrument indicating to the driver the total distance driven by the vehicle since its production to the driver and/or sends a signal to the ADS1."

Note “a” of Figure 3, amend to read:

"a	means the mass of the vehicle, with its fuel tank(s) filled to at least 90 per cent of its or their capacity/capacities, including the mass of the driver, fuel and liquids, fitted with the standard equipment in accordance with the manufacturer’s specifications and, when they are fitted, the mass of the bodywork, the cabin, the coupling and the spare wheel(s) as well as the tools. For vehicles of categories X and Y the mass of the driver shall not be included."

Paragraph 5.4.1., Footnote 1, renumber to Footnote 2 and amend to read:

"12 The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.68 – Annex 3, https://unece.org/transport/standards/transport/vehicle-regulations-wp29/resolutionshttps://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 6.4.1.3., amend to read:

"6.4.1.3. 	With agreement of the authority, a PEMS test can also be driven by a different operator witnessed by a Technical Service, provided that at least the tests of the vehicles required by paragraphs 6.4.3.2. and 6.4.3.6. and in total at least 50 per cent of the PEMS tests required by paragraph 6.4.3.7. for validating the PEMS test family are driven by a Technical Service. In such case the Technical Service remains responsible for the proper execution of all PEMS tests pursuant to the requirements of this Regulation.”

6.4.1.3.	With agreement of the authority, a PEMS test can also be drivenconducted by a different operatorvehicle user witnessed by a Technical Service, provided that at least the tests of the vehicles required by paragraphs 6.4.2.2. and 6.4.2.6. and in total at least 50 per cent of the PEMS tests required by paragraph 6.4.3.7. for validating the PEMS test family are drivenconducted by a Technical Service. In such case the Technical Service remains responsible for the proper execution of all PEMS tests pursuant to the requirements of this Regulation."

"6.4.1.3.	With the agreement of the authority, athe on-road PEMS test can also be drivencarried out by a different vehicle user while witnessed by a Technical Service, provided that at least the tests of the vehicles required by paragraphs 6.4.2.2. and 6.4.2.6. and in total at least 50 per cent of the PEMS tests required by paragraph 6.4.3.7. for validating the PEMS test family are drivencarried out by a Technical Service using the vehicle. In such case the Technical Service remains responsible for the proper execution of all PEMS tests pursuant to the requirements of this Regulation."

Paragraph 8.3.1., amend to read:

"8.3.1.	Vehicle condition

The vehicle, including the emission related components, shall be in good mechanical condition and shall have been run in and driven at least 3,000 km before the test. The mileage and the age of the vehicle used for RDE testing shall be recorded.

All vehicles, including vehicles of categories X and Y, and in particular OVC-HEVs vehicles may be tested in any selectable mode, including battery charge mode. Vehicles equipped with an ADS, other than those of categories X and Y, may be tested with an ADS feature being active. On the basis of technical evidence provided by the manufacturer and with the agreement of the responsible authority, the dedicated driveruser-selectable modes for very special limited purposes shall not be considered (e.g. maintenance mode, race driving, crawler mode). All remaining modes used for forward and for rearwards driving where road and traffic conditions demand this may be considered and the criteria emissions limits shall be fulfilled in all these modes. Modifications that affect the vehicle aerodynamics are not permitted, with the exception of the PEMS installation.

The tyre types and pressure shall be according to the vehicle's manufacturer recommendations. The tyre pressure shall be checked prior to the pre-conditioning and adjusted to the recommended values if needed. Driving the vehicle with snow chains is not permitted.

Vehicles should not be tested with an empty starter battery. In case the vehicle has problems starting, the battery shall be replaced following the recommendations of the vehicle's manufacturer.

The vehicle's test mass comprises of the driver (if applicable), a witness of the test (if applicable), the test equipment, including the mounting and the power supply devices and any artificial payload. It shall be between the actual mass of the vehicle and the maximum permissible test mass of the vehicle at the beginning of the test and shall not increase during the test.

The test vehicles shall not be driven with the intention to generate a passed or failed test due to extreme driving that do not represent normal conditions of use. If necessary, verification of normal driving may be based on expert judgement made by or on behalf of the granting type approval authority through cross-correlation on several signals, which may include exhaust flow rate, exhaust temperature, CO2, O2 etc. in combination with vehicle speed, acceleration and GNSS data and potentially further vehicle data parameters like engine speed, gear, accelerator pedal positionacceleration demand etc."

Paragraph 9.3.5., insert new paragraph to read:

"9.3.5.	Special provisions for vehicles equipped with an ADS

For vehicles of categories X and Y, all tests in this annex shall be conducted, and all respective requirements shall be fulfilled. [RDE] tests shall be conducted using dedicated methods, which may include assigning way points to define the test route, selection of different modes or any other method subject to agreement between the vehicle manufacturer and the technical service, ensuring that the test accurately reflects normal driving patterns.

9.3.5.1.	For vehicles equipped with an ADS, other than those of categories X and Y, the tests in this annex shall be performed at least in the manual driving mode. Tests do not have to be performed in ADS mode providing the manufacturer can demonstrate to the technical service that the same [RDE] performance can be achieved whilst an ADS feature is active. However, testing to verify this shall be performed at the request of the technical service."

Annex 1, Part 2, paragraph 12.10., amend to read:

"12.10.	Devices or systems with driveruser-selectable modes which influence CO2 emissions, electric energy consumption and/or criteria emissions and do not have a predominant mode: yes/no (1)"

Annex 1, explanatory note (h), amend to read:

"(h) The mass of the driver (if applicable) is assessed at 75 kg."

Annex 4, Table A4/1, amend to read:

"Pedal position Acceleration demand7		%		Sensor or ECU"