Modifications to ECE/TRANS/WP.29/GRSP/2025/16*

I.	Proposal

Insert a new paragraph 0., to read:

“0.	Introduction

0.1.	For supplement 1 to the 12 series of amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.2.	The Regulation was originally drafted for vehicles with driver and manual driving controls and it is the intention of this amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and manual driving controls. In the absence of driver/manual driving controls, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of category X, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.”

Paragraph 1., at the end, add a new paragraph to read:

“	This Regulation does not apply to vehicles of category Y.”

Paragraph 1., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 4.4.1 footnote 2, amend to read:

“2	The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.78, Annex 3 - https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Annex 14, paragraph 3.1.7.8., amend to read:

“3.1.7.8.	The upper legs of the driver dummy shall rest against the seat cushion to the extent permitted by placement of the feet. Adjust the spacing of the legs so that the centreline of the knees and ankles is 200 mm (±10mm) apart and ensure that the knees are level.”

Annex 15, paragraph 1., amend to read:

“1.	Purpose

Procedures for folding or retracting head restraints in all designated seating positions equipped with head restraints, except the a driver’s designated seating position.”

II.	Justification

See paragraph 0. in the proposal.

________________

*	Proposal for supplement 1 to the 12 series of amendments to UN Regulation No. 17 (Strength of seats, their anchorages and head restraints)