|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/18 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General xxx Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-eighth session

Geneva, 1–4 December 2025

Item 20 of the provisional agenda

Collective amendments to UN Regulations that impact automated vehicles

Proposal for supplement 7 to the 03 series of amendments to UN Regulation No. 29 (Cabs of commercial vehicles)

Submitted by the expert from Germany on behalf of the GRSP Task Force on Automated Vehicles Regulation Screening (TF-AVRS) *

The text reproduced below was prepared by the expert from Germany on behalf of TF-AVRS, to enable the application of the regulation to vehicles equipped with an Automated Driving System (ADS). The modifications to the existing text of the UN Regulation are marked in bold for new or strikethrough for deleted characters. This document is superseding informal document GRSP-77-63.

I.	Proposal

Insert a new paragraph 0., to read:

“0.	Introduction

0.1.	For supplement 7 to the 03 series of amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.2. 		The Regulation was originally drafted for vehicles with driver and manual driving controls in a cab mounted at the front of the vehicle. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and manual driving controls inside the cab. In the absence of driver/manual driving controls inside the cab, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3.	Cabs for vehicles of category X mounted in other positions than the vehicle front shall not be taken into account for the frontal collision simulation (test A).

0.1.4. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.”

Paragraph 1., amend to read:

“1.	This Regulation applies to:

Vehicles of category N1 with regard to the protection of the occupants of the cab.

This Regulation does not apply to vehicles of category Y.”

Paragraph 1., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 5.1.4., amend to read:

“5.1.4.	Test A (frontal impact) shall only be conducted on Cab-over-Engine vehicles and for vehicles of category X having the cab in the front of the vehicle.”

Annex 1, paragraph 1.4., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Annex 1, Part 1, paragraph 2.7., amend to read:

“2.7.	Driving Cab (cab-over-engine or bonnet or in case of vehicles of category X the position of the cab)2”

Annex 3, paragraph 3., amend to read:

“3.	If fitted, Tthe cab shall be equipped with the steering mechanism, steering wheel, instrument-panel and the driver and passenger seats. The steering wheel and the seating position shall be adjusted to their positions for normal use as prescribed by the manufacturer.”

Annex 3, insert a new paragraph 5.3.4., to read:

“5.3.4.	If fitted, Tthe cab shall be equipped with the steering mechanism, steering wheel, instrument-panel and the driver and passenger seats. The steering wheel and the seating position shall be adjusted to their positions for normal use as prescribed by the manufacturer.”

II.	Justification

See paragraph 0. in the proposal.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.