|  | United Nations | ECE/TRANS/WP.29/GRSG/2026/13 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 28 January 2026 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on General Safety Provisions

131st session

Geneva, 13–17 April 2026

Item 13(b) of the provisional agenda

Exchange of views on vehicle automation:

Categorization of automated vehicles and autonomous vehicle regulation screening

Proposal for supplement 6 to the original version of 
UN Regulation No. 161 (Devices against Unauthorised Use)

Submitted by the expert from the Kingdom of the Netherlands*

The text reproduced below was prepared by the expert from the Kingdom of the Netherlands on behalf of the Task Force on Automated Vehicles Regulatory Screening 
(TF-AVRS). It is based upon ECE/TRANS/WP.29/GRSG/2025/41 as amended by GRSG-130-51. The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

Proposal

Insert new paragraphs 0. and 0.1. to 0.1.2., to read:

"0.		Introduction

0.1.		Supplement 6 to the original version is introduced to take into account vehicles of category X and Y1, as well as other vehicles with a manual mode operating at speeds above 6 km/h which are equipped with an automated Driving System (ADS)1.

0.1.1. 	The Regulation was originally drafted for vehicles with a driver and manual driving controls. It is the intention of this amendment to keep the spirit of the Regulation and to extend its application to vehicles without a driver and without manual driving controls in the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.2. 	In case of vehicles equipped with an ADS, in a manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active, the relevant ADS requirements apply."

Paragraph 1.1., footnote 1, amend to read:

"1		As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.68. (https://unece.org/transport/standards/transport/vehicle-regulations-wp29/resolutions https://unece.org/transport/vehicle-regulations/wp29/resolutions)."

Paragraph 2.5., amend to read:

"2.5.	Device to prevent unauthorized use means a system designed to prevent unauthorized normal activation of the engine or other source of main engine power of the vehicle in combination with at least one system which:

(a)	locks the steering; or

(b)	locks the transmission; or

(c)	locks the gearshift control; or

(d)	locks brakes.

In the case of a system which locks brakes, deactivation of the device shall not automatically release the brakes contrary to the driver's intention unless a driver or an ADS specifically requests to do so."

Insert new paragraphs 2.13. and 2.14., to read:

"2.13.	"ADS feature of type 1 (ADSF-1)" means an ADS feature which includes an ADS fallback response requiring a fallback user;

2.14.	"ADS feature of type 2 (ADSF-2)" means an ADS feature which does not include an ADS fallback response requiring a fallback user."

Paragraph 4.4.1., footnote 2, amend to read:

"2	As defined in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev. 68, para. 2 (https://unece.org/transport/standards/transport/vehicle-regulations-wp29/resolutions https://unece.org/transport/vehicle-regulations/wp29/resolutions)"

Annex 9, paragraph 4.3.2., amend to read:

"4.3.2.	The requirements in paragraph 4.3.1. shall not apply:

- during a remote-control manoeuvring and remote-control parking as defined in UN Regulation No. 79;

- for vehicles of categories X and Y1; or

- when an ADSF-2 is activated by an authorized registered digital key".

*	In accordance with the programme of work of the Inland Transport Committee for 2026 as outlined in proposed programme budget for 2026 (A/80/6 (Sect. 20), table 20.7), the World Forum will develop, harmonize and update UN Regulations to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.