|  | United Nations | ECE/TRANS/WP.29/2026/7 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 23 December 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

198th session

Geneva, 10–13 March 2026

Item 4.12.1 of the provisional agenda

1958 Agreement:

Consideration of additional proposals for 
amendments to existing UN Regulations submitted 
by the Working Parties subsidiary to the World Forum, if any

Proposal for a Supplement 3 to the 15 series of amendments to UN Regulation No. 13 (Heavy vehicle braking)

Submitted by the Working Party on Automated/Autonomous and Connected Vehicles*

The text reproduced below was adopted by the Working Party on Automated/Autonomous and Connected Vehicles (GRVA) at its twenty-third session (ECE/TRANS/WP.29/GRVA/23, para. 23). It is based on: ECE/TRANS/WP.29/GRVA/2025/39 as amended by informal document 
GRVA-23-14. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee (AC.1) for consideration at their March 2026 sessions.

Paragraph 1.2.5., shall be deleted.

Paragraph 2.3., amend to read:

2.3.	"Braking system" means the combination of parts whose function is progressively to reduce the speed of a moving vehicle or bring it to a halt, or to keep it stationary if it is already halted; these functions are specified in paragraph 5.1.2. The system consists of the control (if any), the transmission, and the brake proper;

Paragraph 2.5., amend to read:

2.5.	"Transmission" means the combination of components comprised between the control and/or ADS (as applicable), and the brake and linking them functionally. The transmission may be mechanical, hydraulic, pneumatic, electric or mixed. Where the braking power is derived from or assisted by a source of energy independent of the driver, the reserve of energy in the system is likewise part of the transmission.

The transmission is divided into two independent functions: the control transmission and the energy transmission. Whenever the term "transmission" is used alone in this Regulation, it means both the "control transmission" and the "energy transmission". The control and supply lines between towing vehicles and trailers shall not be considered as parts of the transmission.

Paragraph 2.34.2.1., amend to read:

2.34.2.1.	"Directional control" means a function within a vehicle stability function that assists the driver or the ADS, in the event of under steer and over steer conditions, within the physical limits of the vehicle in maintaining the direction intended by the driver or the ADS in the case of a power-driven vehicle and assists in maintaining the direction of the trailer with that of the towing vehicle in the case of a trailer.

Paragraph 5.1.2.1., amend to read:

5.1.2.1.	Service braking system

The service braking system shall make it possible to control the movement of the vehicle and to halt it safely, speedily and effectively, whatever its speed and load, on any up or down gradient. It shall be possible to graduate this braking action. A driver, if any, shall be able to achieve this braking action from his driving seat without removing his hands from the steering control.

Paragraph 5.1.2.2., amend to read:

5.1.2.2.	Secondary braking system

The secondary braking system shall make it possible to halt the vehicle within a reasonable distance in the event of failure of the service braking system. It shall be possible to graduate this braking action. A driver, if any, shall be able to obtain this braking action from his driving seat while keeping at least one hand on the steering control. For the purposes of these provisions, it is assumed that not more than one failure of the service braking system can occur at one time.

Paragraph 5.1.2.3., amend to read:

5.1.2.3.	Parking braking system

The parking braking system shall make it possible to hold the vehicle stationary on an up or down gradient even in the absence of the driver, the working parts being then held in the locked position by a purely mechanical device. A driver, if any, shall be able to achieve this braking action from his driving seat, subject, in the case of a trailer, to the provisions of paragraph 5.2.2.10. of this Regulation. The trailer air brake and the parking braking system of the towing vehicle may be operated simultaneously provided that the driver or the ADS (as appropriate) is able to check, at any time, that the parking brake performance of the vehicle combination, obtained by the purely mechanical action of the parking braking system, is sufficient.

Paragraph 5.1.4.7., amend to read:

5.1.4.7. 	It shall be possible to verify, in a frequent and simple way, the correct operational status of those electronic systems which have control over braking. If special information is needed, this shall be made freely available.

Paragraph 5.2.1.2.1., amend to read:

5.2.1.2.1.	Except for vehicles of categories X and Y, there shall be at least two controls, independent of each other and readily accessible to the driver from his normal driving position.

For all categories of vehicles, except M2, M3, X and Y, every brake control (excluding an endurance braking system control) shall be designed such that it returns to the fully off position when released. This requirement shall not apply to a parking brake control (or that part of a combined control) when it is mechanically locked in an applied position;

Paragraph 5.2.1.2.7.2., amend to read:

5.2.1.2.7.2.	If the service braking force and transmission depend exclusively on the use, controlled by the driver or the ADS, of an energy reserve, there shall be at least two completely independent energy reserves, each provided with its own transmission likewise independent; each of them may act on the brakes of only two or more wheels so selected as to be capable of ensuring by themselves the prescribed degree of secondary braking without endangering the stability of the vehicle during braking; in addition, each of the aforesaid energy reserves shall be equipped with a warning device as defined in paragraph 5.2.1.13. below. In case of compressed-air braking systems, in each service braking circuit in at least one of the air reservoirs a device for draining and exhausting is required in an adequate and easily accessible position;

Paragraph 5.2.1.26.4., amend to read:

5.2.1.26.4.	After the powertrain has been deactivated and/or the key removed, it shall remain possible to apply the parking braking system, whereas releasing by using the parking brake control shall be prevented.

However, the parking braking system may also be released when this action is part of an operation of a remote-control system fulfilling the technical requirements of an ACSF of Category A as specified in the 02 series of amendments to UN Regulation No. 79 or later series of amendments.

Furthermore, the parking brake shall be automatically applied, at least when the vehicle is detected to be stationary and additionally any of the following conditions is fulfilled:

(a)	The ignition/start switch which controls the electrical energy for the braking equipment has been switched off and/or the key removed;

(b)	The driver is deemed to leave the driving seat (e.g. via a detection of door opening, unfastening of seat belt). Alternatively, for vehicles of category M, the parking brake shall be automatically applied if no input by the ADS, nor any control by the driver nor any brakes application are detected for a time greater than 30 seconds.

However, the automatic application of the parking braking system may be suppressed by the driver (e.g. during maintenance operation, manoeuvring situations, to avoid park brake freezing in winter conditions) with a dedicated action (e.g. by pedal actuation, a switch).

Paragraph 5.2.1.26.5., amend to read:

5.2.1.26.5.	If the parking braking system detects a request (generated automatically or by the driver or the ADS):

(a)	To fully apply the parking brake (i.e. to reach the mechanically locked position of the parking brake), or

(b)	To gradually control the parking brake action,

The actuation of the warning as required in paragraph 2.6. of Annex 8 may be delayed until the parking brake system has detected the correct clamping of the parking brake. The yellow warning signal specified in paragraph 5.2.1.29.1.2. shall be displayed at the latest 10s rafter the request for a full parking brake application, in the case the stable state is not reached.

Paragraph 5.2.2.24.6.3., amend to read:

5.2.2.24.6.3.	A towing trailer may only be operated in conjunction with a power-driven vehicle which is equipped with at least a pneumatic and an electric control line, as per paragraph 5.1.3.1.2. In the event of such a trailer being connected to a power-driven vehicle equipped with only an electric control line according to paragraph 5.1.3.1.3. it is considered that this combination is not compatible. In this case the towing trailer, when electrically connected to the power-driven vehicle, shall automatically apply the brakes of the trailer or remain braked. The driver shall be warned by the separate yellow warning signal specified in paragraph 5.2.1.29.2.

Paragraph 5.3., amend to read:

5.3.	Special provisions for vehicles equipped with an Automated Driving System

The braking system of any vehicle equipped with an Automated Driving System shall fulfil the following requirements

Paragraph 5.3.3., amend to read:

5.3.3. 	Whilst an ADS feature is active, warnings (e.g. defect warning signals, signals from a warning device, failure status) and other information intended for the driver as specified in this UN Regulation (including those received from a trailer) shall be transmitted to the ADS.

The means by which it is ensured that existing detected faults are transmitted to the ADS before an ADS feature becomes active (e.g. previously detected faults which remain present) shall be documented by the manufacturer and demonstrated in accordance with Annex 18.

Insert new paragraph 5.3.4. to read:

5.3.4.	Without prejudice to the requirements of other applicable regulations, the braking control(s) and transmission links between the braking control(s) and brake(s) may be disabled or disconnected whilst an ADS feature is active.

Annex 4, insert new paragraphs 1.1.3. and 1.1.4. to read:

1.1.3.		For vehicles of categories X and Y, all tests in this annex shall be conducted, and all respective requirements shall be fulfilled. In the absence of manual driving controls, braking tests shall be conducted using dedicated activation methods, which may include:

(a)	A test mode allowing to manually control or trigger the braking functions, or;

(b)	Any other method subject to agreement between the vehicle manufacturer and the technical service, ensuring that the evaluation accurately reflects real-world ADS braking performance.

Wherever this annex details a control being actuated or a force being applied, that shall be understood as a braking demand being made through the selected activation method above. The manufacturer shall demonstrate that the test activation method accurately replicates ADS braking performance, and a detailed description of the method used shall be included in the test report. The braking demand made shall be recorded in the test report alongside the results of each test.

1.1.4.	For vehicles equipped with an ADS, other than those of categories X and Y, the tests in this annex shall be performed at least in the manual driving mode. Tests do not have to be performed in ADS mode providing the manufacturer can demonstrate to the technical service that the same braking performance can be achieved when braking demands are made by the ADS. However, testing to verify this shall be performed at the request of the technical service.

Annex 19 – Part 2, paragraph 1.1.4.1.4., amend to read:

1.1.4.1.4.	Engine management:

Control of the engine, or any other source(s) of motive power, to be shown to be independent from control demand by the driver or the ADS (as applicable).

Annex 19 – Part 2, paragraph 1.1.4.1.6., amend to read:

1.1.4.1.6.	Differential type/differential lock(s):

Effect of self-locking or driver/ADS selected locking (as applicable) to be shown, e.g. function maintained, reduced or switched-off.

Annex 21, paragraph 2.1.1., amend to read:

2.1.1.	Where a vehicle is equipped with a vehicle stability function as defined in paragraph 2.34. of this Regulation, the following shall apply:

In the case of directional control the function shall have the ability to automatically control individually the speed of the left and right wheels on each axle or an axle of each axle group by selective braking based on the evaluation of actual vehicle behaviour in comparison with a determination of vehicle behaviour demanded by the driver or the ADS (as applicable).

In the case of roll-over control the function shall have the ability to automatically control the wheel speeds on at least two wheels of each axle or axle group by selective braking or automatically commanded braking based on the evaluation of actual vehicle behaviour that may lead to vehicle roll-over.1

In both cases, the function is not required:

(a)	When the vehicle speed is below 20 km/h;

(b)	Until the initial start-up self-test and plausibility checks have been completed;

(c)	When the vehicle is being driven in reverse;

(d)	When it has been automatically or manually disabled, or disabled by an ADS. In this case, the following conditions shall apply as appropriate:

(i)	When a vehicle is equipped with a means to automatically 	disable the vehicle stability function to provide increased 	traction by modifying the functionality of the drive train, the disablement and its re-instatement shall be automatically linked to the operation which changes the functionality of the drive train;

(ii)	When a vehicle is equipped with a means to manually disable the vehicle stability function, the vehicle stability function shall be automatically reinstated at the initiation of each new ignition cycle;

(iii)	A constant optical warning signal shall inform the driver that the vehicle stability function has been disabled. The yellow warning signal specified in paragraph 2.1.5. below may be used for this purpose. The warning signals specified in paragraph 5.2.1.29. of this Regulation shall not be used.

While an ADS feature is active, the signal shall be transmitted to the ADS.

Annex 21, paragraph 2.1.2., amend to read:

2.1.2.	To realise the functionality defined above a vehicle stability function shall include, in addition to selective braking and/or automatically commanded braking, at least the following:

(a)	The ability to control engine power output;

(b)	In the case of directional control: The determination of actual vehicle behaviour from values of yaw rate, lateral acceleration, wheel speeds, and from the control demand by the driver or the ADS (as applicable) to the braking and steering systems and to the engine. Only on-board generated information shall be used. If these values are not directly measured, the evidence of the appropriate correlation with directly measured values under all driving conditions (e.g. including driving in a tunnel) shall be shown to the Technical Service at the time of type approval;

(c)	In the case of roll-over control: The determination of actual vehicle behaviour from values of the vertical force on the tyre(s) (or at least lateral acceleration and wheel speeds) and from the control demand by the driver or the ADS (as applicable) to the braking system and to the engine. Only on-board generated information shall be used. If these values are not directly measured, the evidence of the appropriate correlation with directly measured values under all driving conditions (e.g. including driving in a tunnel) shall be shown to the Technical Service at the time of type approval;

(d)	In the case of a towing vehicle equipped according to paragraph 5.1.3.1. of this Regulation: The ability to apply the service brakes of the trailer via the respective control line(s) independently of the driver or the ADS.

Annex 21, paragraph 2.1.4., amend to read:

2.1.4.	Interventions of the vehicle stability function shall be indicated to the driver by a flashing optical warning signal fulfilling the relevant technical requirements of Regulation No. 121 or, whilst an ADS feature is active, transmitted to the ADS as defined in paragraph 5.3.3. The indication shall be present as long as the vehicle stability function is in an intervention mode. The warning signal specified in paragraph 5.2.1.29.1.2. of this Regulation shall not be used for this purpose.

Additionally, interventions by systems related to the vehicle stability function (including traction control, trailer stability assist, corner brake control, other similar functions that use throttle individual torque control to operate and share common components with vehicle stability function, and ESC or VSF intervention on the steering angle of one or more wheels for the purpose of vehicle stability) may also be indicated to the driver by this flashing optical warning signal or, if an ADS feature is active, transmitted to the ADS in accordance with paragraph 5.3.3.

Annex 21, paragraph 2.1.5., amend to read:

2.1.5.	A vehicle stability function failure or defect shall be detected and indicated to the driver by an optical warning signal fulfilling the relevant technical requirements of Regulation No. 121, or transmitted to the ADS (as applicable).

The warning signal specified in paragraph 5.2.1.29.1.2. of this Regulation shall not be used for this purpose.

The warning signal shall be constant and remain displayed as long as the failure or defect persists and the ignition (start) switch is in the "on" (run) position."

Annex 21, paragraph 2.1.6., amend to read:

2.1.6.	In the case of a power-driven vehicle equipped with an electric control line and electrically connected to a trailer with an electric control line the driver shall be warned by a specific optical warning signal fulfilling the relevant technical requirements of Regulation No. 121 whenever the trailer provides the information "VDC Active" via the data communications part of the electric control line. The optical signal defined in paragraph 2.1.4. above may be used for this purpose.

While an ADS feature is active, the optical warning to the driver is not required. The warning signal shall be transmitted to the ADS in accordance with paragraph 5.3.3.

Annex 21 – Appendix 2, paragraph 2.1., amend to read:

"2.1.	The validity of the applied modelling and simulation tool shall be verified by means of comparisons with a practical vehicle test(s). The test(s) utilised for the validation shall be those which, without control action, would result in loss of	directional control (under-steer and over-steer) and/or roll-over control as appropriate to the functionality of the stability control function installed on a vehicle.

During the test(s) the following motion variables, as appropriate, shall be recorded or calculated in accordance with ISO 15037 Part 1:2006 or Part 2:2002 as relevant:

(a)	Yaw velocity;

(b)	Lateral acceleration;

(c)	Wheel load or wheel lift;

(d)	Forward velocity;

(e)	Control demand by the driver or the ADS (as applicable).

Annex 22, paragraph 3.6., amend to read:

3.6.	The vehicle user's handbook provided by the manufacturer shall include a warning about the consequences of not checking the compatibility of the automated connector between the towing vehicle and the trailer. Information about mixed mode operation shall also be provided if applicable.

To enable the driver to check the compatibility, vehicles fitted with an automated connector shall have a marking specifying the category according to paragraph 2. of this annex. For categories B and D also the type of the installed automated connector shall be shown. This marking shall be indelible and visible to a user when standing on the ground beside the vehicle.

*	In accordance with the programme of work of the Inland Transport Committee for 2026 as outlined in proposed programme budget for 2026 (A/80/6 (Sect. 20), table 20.7), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para.2 https://unece.org/transport/standards/transport/vehicleregulations-wp29/resolutions