Modifications to ECE/TRANS/WP.29/GRSP/2025/36*

The text reproduced below was prepared by the expert from Germany on behalf of TF-AVRS, to enable the application of the regulation to vehicles equipped with an Automated Driving System (ADS).

The base proposal was initially submitted by France to the 77th session GRSP as informal document GRSP-77-67. This proposal was then reproduced as ECE/TRANS/WP.29/GRSP/2025/36 for the 78th session.

Amendments to the current text of UN Regulation No.127 are marked in bold for new or strikethrough for deleted characters. Amendments to ECE/TRANS/WP.29/GRSP/2025/36 are marked in red.

I.	Proposal

Insert a new paragraph 0., to read:

“0.	Introduction

0.1.		For supplement 4 to the 04 series of amendments:

0.1.1.		The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.2. 		The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 		In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of categoriesy X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.”

Paragraph 1., amend to read:

“1.	This Regulation applies to motor vehicles of categories M1 and N1.

However, vehicles of category N1 where the driver’s position "R-point" is either forward of the front axle or longitudinally rearwards of the front axle transverse centreline by a maximum of 1,100 mm, are exempted from the requirements of this Regulation. For vehicles of category X, the "R" point of the most forward passenger seat shall be taken instead of the R-point of the driver's seat.

This Regulation does not apply to vehicles of category M1 above 2,500 kg maximum mass and which are derived from N1 category vehicles, and where the driver’s position "R-point" is either forward of the front axle or longitudinally rearwards of the front axle transverse centreline by a maximum of 1,100 mm; for these vehicle categories Contracting Parties may continue to apply the requirements already in force for that purpose at the time of acceding to this Regulation. For vehicles of category X, the "R" point of the most forward passenger seat shall be taken instead of the R-point of the driver's seat.

This Regulation does not apply to vehicles of category Y.”

Paragraph 1, footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.68, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 2.27., amend to read:

“2.27.	"Mass in running order" means the nominal mass of a vehicle as determined by the sum of unladen vehicle mass and driver’s mass, if applicable.

For vehicles of category X, the driver’s mass is not included.”

Paragraph 2.29., amend to read:

“2.29.	"Normal ride attitude" means the vehicle positioned on a flat horizontal surface with its mass in running order, with the tyres inflated to manufacturer recommended pressures, the front wheels in the straight-ahead position and with a passenger mass placed on the front passenger seat. The front seats are placed at the nominal mid-track position. The suspension shall be set in normal running condition as specified by the manufacturer for a speed of 40 km/h.

For vehicles of category X, two passenger masses are placed on the front seats instead of the driver's and one passenger masses.”

Paragraph 4.4.1 footnote 7, amend to read:

“7	The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.68, Annex 3 - https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 5.3.1.2., amend to read:

“5.3.1.2.	ARHSS can be engaged for use in exceptional low speed use cases (e.g. flood/heavy snow) on public roads. In such cases, ARHSS shall automatically return to the normal ride height when the vehicle speed exceeds 25 km/h or when the system is manually disengaged by the driver or controlled by the ADS or when an ADS feature is performing the DDT8. If an ARHSS for use in exceptional low speed use cases is engaged, it is indicated to the driver at least by an optical warning signal.”

Add Paragraph 5.3.1.2, footnote 8 :

“8	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 1.17 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

II.	Justification

See paragraph 0. in the proposal.