|  | United Nations | ECE/TRANS/WP.29/GRBP/2026/4 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 27 November 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Noise and Tyres

Eighty-third session

Geneva, 10–13 February 2026

Item 3 of the provisional agenda

UN Regulation No. 51 (Noise of M and N Categories of Vehicles)

Proposal for Supplement 11 to the 03 series of amendments to UN Regulation No. 51 (Noise of M and N Categories of Vehicles)

Submitted by the Subgroup on UN Regulations Nos. 51 and 138*

The text reproduced below was prepared by the Subgroup on UN Regulations Nos. 51 and 138 reflecting the outcome of the group’s work. The proposed amendments are intended to introduce a “default mode” and specify the requirements to be applied to the emission of Enhanced Exterior Sound (EES), with the corresponding transitional provisions, and to draw a limit curve to introduce electric vehicles (EVs) and hybrid electric vehicles (HEVs) which can be propelled with no internal combustion engine operating in the scope of ASEP (Annex 7), as agreed during the eighty-second session of GRBP (informal document GRBP-82-39).

- Proposal

Paragraph 2.24., amend to read:

"2.24.	Table of Symbols

…

| … |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| nanchor,κ nanchor | 1/min | Annex 7 |  | 3.1. |  | reported vehicle engine speed for gear ratio i from Annex 3 |  |
| aanchor,κ | m/s² | Annex 7 | 3.1. |  | reported vehicle acceleration for gear ratio [i] from Annex 3; value to be reported and used for calculations to the first decimal place |  |
| vanchor,κ vanchor | km/h | Annex 7 | 3.1. | reported vehicle test speed for gear ratio i at BB’ from Annex 3; value to be reported and used for calculations to the first decimal place |
| aanchor,κ aanchor | m/s² | Annex 7 | 3.1. | reported vehicle acceleration for gear ratio i[i] from Annex 3; value to be reported and used for calculations to the first decimal place |
| LASEP κ,j | dB(A) | Annex 7 | 3.3. |  | [sound pressure level calculated for a gear ratio [i] and for a speed (engine or vehicle) to reflect the estimated linear sound level increase regarding the engine speed (ICEs) or the vehicle speed (EVs)] sound pressure level calculated for a gear k at test point j to reflect the estimated linear sound level increase as a function of the engine speed (ICEs) or the vehicle speed (EVs) |  |
| ΔLASEP, j | dB | Annex 7 | 3.4. |  | [estimated delta sound pressure level for ASEP to reflect the relation of the measured performance versus performance at the anchor point; value to be used for calculations to the first decimal place] |  |
| Lκj | dB(A) | Annex 7 | 3.4.3.5. |  | … |  |
| … |  |  |  |  |  |  |

…"

Add a new paragraph 2.29., to read

“[2.29.	“Active Sound Source” means a physical part installed on the vehicle

- which is powered, and

- one of whose purposes is to emit sound to the exterior of the vehicle.]"

Add a new paragraph 2.30., to read

"[2.30.	"Enhanced Exterior Sound (EES)" means any sound

- generated by one or more Active Sound Sources / active systems installed on a vehicle enhancing the vehicle’s sound emission, and

- which is not subject to another UN Regulation.]"

Add a new paragraph 2.31., to read

"2.31.	"Vehicle Master Control Switch" means the device by which the vehicle’s on-board electronics system is brought, from being switched off, as in the case where a vehicle is parked without the driver being present, to [normal] operation mode (i.e. the vehicle status is "ready to drive").[]"

Paragraph 6.2., amend to read:

“6.2.	Specifications regarding sound levels

[Electrified vehicles HEVs of categories M1 and N1 which can be propelled in the normal mode, in reverse or at least one forward drive gear, without an internal combustion engine (ICE) operating shall comply with the requirements for electric vehicles when the ICE is not operating and with the requirements for ICE vehicles when the ICE is operating.]

Paragraph 6.2.3., amend to read:

"6.2.3.	Additional sSound eEmission pProvisions (ASEP)

The Requirements of Annex 7Additional Sound Emission Provisions (ASEP) apply only to vehicles of categories M1 and N1 equipped with an internal combustion engine.

Vehicles are deemed to fulfil the requirements of Annex 7, if the vehicle manufacturer provides technical documents to the type approval authority showing, that the difference between maximum and minimum engine speed of the vehicles at BB' for any test condition inside the ASEP control range (as defined in paragraph 2.3. of Annex 7 to of this Regulation (including Annex 3 conditions) does not exceed 0.15 x S. This article is intended especially for non-lockable transmissions with variable gear ratios (CVT).

…

The sound  …  in a significant manner.

Any electric sound enhancement system for the purpose of the exterior sound emission emission of EES shall be operational activated during the type-approval test."

Add a new paragraph 6.2.3.3. and subparagraphs, to read:

"6.2.3.3.	Specifications for EVs and HEVs regarding the emission of EES, when [purely electrically propelled]

6.2.3.3.	Specifications for EVs and HEVs regarding the emission of EES, when the [vehicle is only propelled by electric motor(s)] and without any operating internal combustion engine during [the test].

6.2.3.3.1.	Operation of EES

When the vehicle is switched on by the vehicle master control switch, any EES shall be deactivated.

Any activation of EES shall require an intentional action of the driver.

The EES shall be capable of being deactivated.

6.2.3.3.2.	Vehicle Sound Limits

In the speed range equal to or lower than 20 km/h including standstill the sound emitted by the vehicle shall comply with the requirements of UN-Regulation No. 138 paragraph 6.

In the speed range from 20 km/h to 80 km/h the sound emitted by the vehicle shall comply with the requirements of this Regulation.

6.2.3.3.1.	When the vehicle is switched on by the vehicle master control switch, any EES shall be deactivated.

6.2.3.3.2. 	Any activation of EES shall require an intentional action of the driver.[]

6.2.3.3.3.	The sound emitted by the vehicle shall comply with the requirements of UN-Regulation No. 138 paragraph 6 in the speed range equal and lower than 20 km/h including standstill.

6.2.3.3.4.	The sound emitted by the vehicle shall comply with the requirements of Annex 7 of this Regulation.

6.2.3.3.5.	The EES shall be deactivatable.”

Paragraph 6.2.3.3. (former), renumber as paragraph 6.2.3.4.

Add new paragraphs 11.18., 11.19. and 11.20., to read:

Option 1

"11.18. 	[Supplement [11] does not apply to existing UN type approvals and their extensions, granted prior to the date of entry into force of Supplement [11].

11.19.	As from the official date of entry into force of Supplement [11] to the 03 series of amendments to this Regulation, no Contracting Party applying this UN Regulation shall refuse to grant or refuse to accept UN type approvals according to Supplement [11] to the 03 series of amendments to this UN Regulation."

Option 2

"{11.18. 	[For vehicles without EES,] Supplement [11] does not apply to existing UN type approvals and their extensions, granted prior to the date of entry into force of Supplement [11].

11.19.	As from the official date of entry into force of Supplement [11] to the 03 series of amendments to this Regulation, no Contracting Party applying this UN Regulation shall refuse to grant or refuse to accept UN type approvals according to Supplement [11] to the 03 series of amendments to this UN Regulation.}

11.20. 	[{For vehicles with EES,} Uu]ntil [12-18-24] months after the date of entry into force of the Supplement [11] to the 03 series of amendments to this UN Regulation, Contracting Parties applying this UN Regulation can continue to grant extensions to existing UN type approvals to the 03 series of amendments to this UN Regulation, granted prior to the date of entry into force of Supplement [11]. "

Option 3

"

11.18. 	Until [12-18-24] months after the date of entry into force of the Supplement [11] to the 03 series of amendments to this UN Regulation, Contracting Parties applying this UN Regulation can continue to grant extensions to existing UN type approvals to the 03 series of amendments to this UN Regulation, granted prior to the date of entry into force of Supplement [11]."

Annex 1, Appendix 1, add a new item 1.1.10., to read:

"1.1.10.	[System](s) generating EES, Make and Type: "

Annex 1 – Appendix 2, add a new item 7.2., to read:

"7.2.	Details of any [system(s)] generating EES (e.g., number and position of actuators and [active sound sources]) [including a description of all a possibilities possibility to activate and deactivate the EES]"

Annex 3,

Paragraph 2.2.3.3., amend to read:

"2.2.3.3.	Active Sound [Systems] generating EES

Any active sound devices [system] generating EES, subject to paragraph 6.2.3. and Annex 7 of this Regulation, either for noise control, or sound enhancement, shall operate … measurements."

Paragraph 3.1.2.1.4.3., amend to read:

"3.1.2.1.4.3.	Vehicles with only one gear ratio, like but not limited to Battery Electric Vehicles (BEV) and Fuel Cell Vehicles (FCV)

…

The achieved acceleration awot_test , but no more than 2.0 m/s², is then used for the calculation of the partial power factor kp (see paragraph 3.1.2.1.3.) instead of awot ref.[For Annex 7 calculations, the true achieved acceleration shall be used.]"

Annex 7,

Add a new paragraph 1.1., to read:

"1.1.	Vehicles when purely electrically propelled [with propulsion technologies other than combustion engines] shall comply with Analysis method 1 (Slope assessment method) only. "

Paragraph 2.3., amend to read:

"2.3.	Control rRange

The ASEP …  below.

The control range is specified as:

|  |  | For vehicles in operating conditions in accordance with Annex 7, paragraph 1.1. |  | For vehicles in all other operating conditions |
| --- | --- | --- | --- | --- |
| Vehicle speed vAA_ASEP |  | vAA ≥ > 20 km/h |  | vAA ≥ 20 km/h |
| Vehicle acceleration aWOT_ASEP |  | aWOT ≤ 5.0 m/s2 | aWOT ≤ 5.0 m/s2 |  |
| Vehicle acceleration aWOT_ASEP |  | aWOT ≤ 5.0 m/s2 |  |
| Engine speed nBB_ASEP |  | not applicable |  | nBB ≤ 2.0 * PMR-0.222 * S or nBB ≤ 0.9 * S, whichever is the lowest |
| Vehicle speed vBB_ASEP for vehicles tested in Annex 3 with |  |  |  |  |
|  | - locked gear | vBB ≤ 80 km/h |  | If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but no more than 80 km/h. For any other gear, the maximum vehicle speed is 70 km/h. |
|  | - non-locked gears | vBB ≤ 80 km/h |  | For vehicles tested in non-locked transmission conditions the maximum vehicle speed is 80 km/h. |
|  | - non-locked gears | vBB ≤ 80 km/h |  |
| Gear selection |  | only if applicable, e.g. for hybrid electric vehicles: gears κ ≤ gear i as determined in Annex 3 |  | gears κ ≤ gear i as determined in Annex 3 |
| Transmission condition for vehicles tested in Annex 3 with |  |  |  |  |
|  | - locked gear | Geari, geari-1,… |  | Geari, geari-1,… |
|  | - non-locked gears | Non-locked gears |  | Non-locked gears |

Vehicle speed VAA_ASEP: 	vAA ≥ 20 km/h

Vehicle acceleration aWOT_ASEP:	aWOT ≤ 5.0 m/s2

Engine speed nBB_ASEP:	nBB ≤ 2.0 * PMR-0.222 * S or

nBB ≤0.9 * S, whichever is the lowest

For vehicles subject to paragraph 1.1. of this Annex, the manufacturer shall take measures to achieve an acceleration aWOT_ASEP within the acceleration control range.

For vehicles subject to paragraph 1.1. of this Annex, the manufacturer shall take measures to achieve an acceleration within the control range in such a way that each test point P1 to P4 as specified in paragraph 2.4. of this Annex reaches the given target speed within the specified tolerance.

Table 1 in Appendix 1 to Annex 3 provides examples for valid measures to enable a test condition within the above specified acceleration boundaries. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

Vehicle speed VBB_ASEP:

If the vehicle, in the lowest valid gear does not achieve the maximum engine speed nBB_ASEP below 70 km/h, increase the vehicle speed in that gear to reach the maximum engine speed nBB_ASEP, but not beyond 80 km/h.

For any other gear, the maximum vehicle speed is 70 km/h.

For vehicles tested in non-locked transmission conditions, the maximum vehicle speed is 80 km/h.

Gears	κ ≤ gear i as determined in Annex 3

Transmission conditions:

| Annex 3 gear selection | Annex 7 gear selection |
| --- | --- |
| Locked | Geari, geari-1,… |
| Non-locked | Non-locked |

"

Paragraph 3.1., amend to read:

"3.1.	Determination of the anchor point

…

For vehicles subject to paragraph 1.1. of this Annex:

Lanchor is the higher sound pressure level of Lwot of left and right sides of gear ratio selected for the test;

vanchor is the average of vBB,wot of the 4 runs of gear ratio selected for the test reported from Annex 3, but limited to 60 km/h for further calculation.

aanchor is the measured acceleration and is not limited to 2.0 m/s² as required in paragraph 3.1.2.1.4.3. of Annex 3 for the determination of the kp-factor, and is the average of awot,test reported from Annex 3, as defined in paragraph 3.1.2.1.2. of Annex 3.”

Paragraph 3.2.2., amend to read:

"3.2.2.	Slope of the regression line for each gear ratio κ

…

For vehicles subject to paragraph 1.1. of this annex, Slopeκ shall be set to a constant value of 0.25 dB/(km/h) for further calculation.

For vehicles subject to paragraph 1.1. of this annex, the sound measurements shall be evaluated as function of vehicle speed and Slopeκ shall be set to a constant value of 0.25 dB/(km/h) for further calculation."

Paragraph 3.3., amend to read:

"3.3.	Calculation of the linear sound level increase expected for each measurement

…

For vehicles subject to paragraph 1.1. of this Annex:

The sound level LASEP,κ,j for measurement point j and gear ratio κ shall be calculated using the vehicle [speed] measured for each measurement point, using the constant slope specified in paragraph 3.2.2. above to the specific anchor point for each gear ratio.

For vBB_κ,j ≤ vanchor:

LASEP κ,j=Lanchor+Slopeκ-Y×(vBB_κ,j- vanchor)

For vBB_κ,j > vanchor:

LASEP κ,j=Lanchor+Slopeκ×vBB_κ,j- vanchor

Where Y= 0.05 dB/(km/h)"

Add a new paragraph 3.4., to read:

"3.4.	Calculation of the sound level increase regarding performance

For tests of purely electrically propelled vehicles, the sound level ΔLASEP,j shall be calculated:

For vBB, test,  j × awot, test, j≤ vanchor× aanchor:

ΔLASEPj =0 dB

For vBB test,j × awot, test, κ,j> vanchor× aanchor :

ΔLASEP,j = 2-4-6-8dB ×log10 vBB test,j ×awot,test,jvanchor× aanchor

For all other tests the sound level ΔLASEP,j is set to 0 dB."

Paragraph 3.4. (former), renumber as paragraph 3.5.

Paragraph 3.5. (former), renumber and amend to read:

"3.56.	Specifications

Every individual  … below:

Lκ,j - ΔLASEP,j ≤ LASEP,κ.j + x

With:

x = 3 dB(A) + limit value - Lurban	for vehicle  tested with non-locked transmission  conditions

x = 2 dB(A) + (limit value³ - Lurban),	 of Annex 3 for all other vehicles

and only for vehicles

- tested with non-locked transmission conditions given by multiple gear ratios [or]

[x = 3 dB + (limit value - Lurban) of Annex 3]

- [having multiple electric propulsion sources or]

- [(EV only) having an Lcrs_rep greater than Lwot_rep of Annex 3]:

x = [2-4] dB + (limit value - Lurban) of Annex 3

If at any point the measured sound level at a point exceeds the limit, … specification. "

Annex 9, paragraph 1., amend to read:

"[1.	General

The Real Driving Additional Sound Emission Provisions (RD-ASEP) apply only to vehicles of categories M1 and N1 equipped with:

-	an internal combustion engine (ICE) for propulsion of the vehicle, or

-	any other propulsion technology fitted with an exterior sound enhancement a system generating EES.]"

II.	Justification

- This working document reflects the outcome of the Informal Working Group on Real Driving - Additional Sound Emission Provisions (IWG RD - ASEP) Subgroup on UN Regulations Nos. 51 and 138.

- The group will continue its work to resolve the square brackets and to provide justifications to the proposed changes.

- On the definition of EES: It turned out difficult to be clear in wording about what sound sources should be considered for the EES. The definition aims on distinguishing sound that should be seen parasitic to another purpose and sound that is made for the purpose of making that sound. A signatory sound that results of an ignition order and cylinder arrangement of an ICE should be seen as such a parasitic sound, although also that sound is probably “enhanced” if it were only by reducing other noise and using all available SPL for this signatory sound. The EES that the definition intends to describe is sound that is generated and amplified and is not parasitic to another primary purpose. This type of sound is referred to as active sound (still between brackets) but other wording to support the intention described could be used as well.

- On the paragraph 6.2.3 Additional Sound Emission Provision: This paragraph describes that an EES is default switched OFF and needs an intentional action from the driver to be switched on. Further requirements safeguard that the driver is in control of the EES that is produced by the vehicle and thus can bear responsibility for the EES that the vehicle is emitting.

- On the transitional provisions: The supplement aims on implementing the sound limits of Annex 7 with immediate effect. Data analysis of a broad range of the existing EV show that (1) EES is not yet widely implemented and (2) current EVS are within the limits that are set. The transitional provision is further intended to give industry a reasonable time to implement the paragraph 6.2.3, although also these requirements are commonly fulfilled.

- On the limits in Annex 7: Paragraph 3.4 describes an additional sound space for vehicles that accelerate above the V_anchor. In theory the Anchor speed and acceleration is respectively 55km/h and 2m/s2 the Annex 7 ASEP limit is at 5m/s2 and 80km/h. The highest outcome of ΔLASEP,j would be [2-4-6-8] * 0,56 dB.

- On the conditions to add 1dB to the limit (x=3) in paragraph 3.6

….no clear justification as of now.

*	In accordance with the programme of work of the Inland Transport Committee for 2026 as outlined in proposed programme budget for 2026 (A/80/6 (Sect. 20), table 20.7), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

as already defined in UN Regulations Nos. 141 and 174.

See recommendations provided by informal document GRB-68-03 in its latest version as guidance for technical interpretation[, which is included in the list of documents for reference of GRBP].

Example: changing a mode or operating a switch.

The Analysis method 2 (Lurban assessment) and the Reference Sound Assessment are not applicable for these vehicles.

As applicable for the approved  type of vehicle

As applicable for the approved  type of vehicle

As applicable for the approved  type of vehicle