|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/23 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 22 September 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-eighth session

Geneva, 1–4 December 2025

Item 20 of the provisional agenda

Collective amendments to UN Regulations that impact automated vehicles

Proposal for supplement 1 to the 04 series of amendments to UN Regulation No. 137 (Frontal impact with focus on restraint systems)

Submitted by the expert from Germany on behalf of the GRSP Task Force on Automated Vehicles Regulation Screening (TF-AVRS) *

The text reproduced below was prepared by the expert from Germany on behalf of TF-AVRS, to enable the application of the regulation to vehicles equipped with an Automated Driving System (ADS). The modifications to the existing text of the UN Regulation are marked in bold for new or strikethrough for deleted characters. This document is superseding informal document GRSP-77-69.

I.	Proposal

Title, amend to read:

Uniform provisions concerning the approval of passenger cars vehicles in the event of a frontal collision with focus on the with regard to restraint systems, and with regard to fuel system integrity and protection against electrical shock in the event of a frontal collision

Paragraph 0., amend to read:

“0.	Introduction to the 04 series of amendments to UN Regulation No. 137

0.1.		For the 04 series of amendments:

0.1.1.		Currently, under UN Regulation No. 137 in its 03 series of amendments …

…

… this particular case.

0.2.	For supplement 1 to the 04 series of amendments:

0.2.1.	The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.2.2. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.2.3. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.”

Paragraph 1., amend to read:

“1.	Scope

This Regulation applies to vehicles of category M11 with a maximum permissible mass not exceeding 3,500 kg and to vehicles of category N1.

This regulation does not apply to vehicles of category Y.”

Paragraph 1., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 2.4.2., amend to read:

“2.4.2.b.	The structure, dimensions, lines and materials of the part of the vehicle forward of the transverse plane through the "R" point of the driver's seat, in so far as they have a negative effect on the results of the impact test prescribed in this Regulation. for vehicles of category X, the "R" point of the most forward passenger seat shall be taken instead of the R-point of the driver's seat.”

Paragraph 2.10., amend to read:

“2.10.	"Passenger airbag" means an airbag assembly intended to protect occupant(s) in seats other than the a driver's in the event of a frontal collision.”

Paragraph 5.2., amend to read:

5.2.		Specifications for the restraint system test (Full Width Rigid Barrier test)

The vehicle shall be tested and approved in accordance with the method described in Annex 3.

The vehicle which, in agreement with the Technical Service, is considered as having the worst case effect on the result of the injury criteria specified in paragraph 5.2.1. shall be selected for this test.

The test of the vehicle carried out in accordance with the method described in Annex 3. shall be considered satisfactory if all the conditions set out in paragraphs 5.2.1. to 5.2.7. below are all satisfied at the same time.

Additionally, vehicles equipped with electric power train shall meet the requirements of paragraph 5.2.8. This can be met by a separate impact test at the request of the manufacturer and after validation by the Technical Service, provided that the electrical components do not influence the occupant protection performance of the vehicle type as defined in paragraphs 5.2.1. to 5.2.5. of this Regulation. In case of this condition the requirements of paragraph 5.2.8. shall be checked in accordance with the methods set out in Annex 3 to this Regulation, except paragraphs 2., 5. and 6. of Annex 3.

A dummy corresponding to the specifications for Hybrid III fiftieth percentile (see footnote 1 of Annex 3) fitted with a 45° ankle angle and meeting the specifications for its adjustment shall be installed in driver’s seat.

A dummy corresponding to the specifications for Hybrid III fifth percentile (see footnote 1 of Annex 3) fitted with a 45° ankle angle and meeting the specifications for its adjustment shall be installed in the outboard passenger’s seat.

For vehicles of category X, the manufacturer shall propose the position of the dummy on the front seat, provided that both dummies, the Hybrid III fiftieth percentile male dummy and the Hybrid III fifth percentile female dummy are used.”

Paragraph 5.2.3.1., amend to read:

“5.2.3.1.	In the case of automatically activated door locking systems which are installed optionally and/or which can

- be de-activated by the driver by the driver in case of manual driving , or

- be deactivated in case of vehicles of category X,

this requirement shall be verified by using one of the following 2 test procedures, at the choice of the manufacturer:”

Paragraph 5.2.4.2., amend to read:

“5.2.4.2.	In the case of vehicles equipped with automatically activated door locking systems which are installed optionally and/or which can

- be de-activated by the driver by the driver in case of manual driving , or

- be deactivated in case of vehicles of category X,

this requirement shall be verified by using one of the following 2 test procedures, at the choice of the manufacturer:”

Paragraph 6.1., amend to read:

“6.1.		[For a vehicle fitted with a passenger airbag intended to protect occupants other than the a driver, this information shall consist of the warning label described in paragraph 6.2. below.]

For a vehicle fitted with airbag assemblies intended to protect the driver and occupants other than the driver, compliance with paragraphs 8.1.8. to 8.1.9. of UN Regulation No. 16 as amended by the 08 series of amendments, with paragraphs 8.1.9. to 8.1.10. of UN Regulation No. 16 as amended by the 09 series of amendments or with paragraphs 5.1.9. to 5.1.10. of UN Regulation No. 173 shall be demonstrated as from 1 September 2020 for new vehicle types. Before this date the relevant requirements of the preceding series of amendments apply.”

Annex 3, paragraph 1.4.3.5.2., amend to read:

1.4.3.5.2.	In the case of vehicles equipped with an automatically activated door locking system that is installed optionally and/or which can be de-activated by the driver in case of manual driving , or be deactivated in case of vehicles of category X, one of the following two procedures shall be used at the choice of the manufacturer:

Annex 3, paragraph 1.4.3.5.2.2., amend to read:

“1.4.3.5.2.2.	The side doors on the driver one side shall be unlocked and the system overridden for these doors; for the side doors on the other vehicle passenger side, the system may be activated in order to lock these doors automatically before the moment of impact. At the choice of the manufacturer, these doors shall be locked manually before the start of propulsion of the vehicle. This test is deemed to be fulfilled, if the unlocked and locked doors are reversed.”

Annex 3, paragraph 1.4.3.11.1., amend to read:

1.4.3.11.1.	Position of front seats

Seats adjustable longitudinally shall be placed so that their "H" point, determined in accordance with the procedure set out in annex 6 is in the middle position of travel or in the nearest locking position thereto, and at the height position defined by the manufacturer (if independently adjustable for height). In the case of a bench seat, the reference shall be to the "H" point of the driver's place. For vehicles of category X, the "H" point of the most forward passenger place shall be taken instead of the H-point of the driver's place.

Annex 3, paragraph 2.1.1., amend to read:

“2.1.1.		A dummy corresponding to the specifications for Hybrid III fiftieth percentile male dummy meeting the specifications for its adjustment shall be installed in the driver seat in accordance with the conditions set out in Annex 5. A dummy corresponding to the specifications for Hybrid III fifth percentile female dummy1 meeting the specifications for its adjustment shall be installed in the front passenger seat in accordance with the conditions set out in Annex 5. For vehicles of category X, the manufacturer shall propose the position of the dummy on the front seat, provided that both dummies, the Hybrid III fiftieth percentile male dummy and the Hybrid III fifth percentile female dummy are used.”

Annex 5, paragraph 1.2.2., amend to read:

“1.2.2.	Outer passenger

The plane of symmetry of the dummy shall be symmetrical with that of the driver dummy relative to the longitudinal median plane of the vehicle. In the absence of a driver dummy, the plane of symmetry of the dummies shall be vertical and parallel to the vehicle's longitudinal centreline using the Y-coordinate of the R-point as defined by the manufacturer. If the seating position is determined by the shape of the bench, such seat shall be regarded as a separate seat.”

***

II.	Justification

See paragraph 0. in the proposal.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.