|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/11 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 19 September 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-eighth session

Geneva, 1–4 December 2025

Item 20 of the provisional agenda

Collective amendments to UN Regulations 
that impact automated vehicles

Proposal for supplement 3 to the 04 series of amendments to UN Regulation No. 11 (Door latches and hinges)

Submitted by the expert from Germany on behalf of the GRSP Task Force on Automated Vehicles Regulation Screening (TF-AVRS) *

The text reproduced below was prepared by the expert from Germany on behalf of TF-AVRS, to enable the application of the regulation to vehicles equipped with an ADS. The modifications to the existing text of the UN Regulation are marked in bold for new or strikethrough for deleted characters. This document is superseding informal document GRSP-77-57.

I.	Proposal

Insert a new paragraph 0., to read:

“0.	Introduction

For supplement 3 to the 04 series of amendments:

The Regulation is amended to account for vehicles of category X1. Vehicles of category Y1 are not in the scope of this Regulation.

0.1.1.	The Regulation was originally drafted for vehicles with driver and manual driving controls and it is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and manual driving controls. In the absence of driver/manual driving controls, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.2. 	In case of vehicles equipped with an Automated Driving System (ADS)1 other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements apply.”

Paragraph 1., amend to read:

“1.	Scope: This Regulation applies to vehicles of categories M1 and N1 with respect to latches and door retention components such as hinges and other supporting means on doors, which can be used for the entry or exit of the occupants and/or can present the risk of occupants being thrown from a vehicle as a result of impact.

This regulation does not apply to vehicles of category Y.”

Paragraph 1., footnote 1, amend to read:

“1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions”

Paragraph 2.9., amend to read:

“2.9.	"Door closure warning system" is a system that will activate a visual signal located where it can be clearly seen by the driver or will transmit a logical signal to the ADSF-2 (if applicable), when a door latch system is not in its fully latched position and while the vehicle ignition is activated.”

Paragraph 2.24., amend to read:

“2.24.	"Side front door" is a door that, in a side view, has 50 per cent or more of its opening area forward of the rearmost point on the driver's seat back, when the seat back is adjusted to its most vertical and rearward position, providing direct access for passengers to enter or depart the vehicle; in case of vehicles with an ADSF-2, For vehicle of category X, the front-most seat back is considered instead of the driver's seat back.”

Paragraph 2.25., amend to read:

“2.25.	"Side rear door" is a door that, in a side view, has 50 per cent or more of its opening area to the rear of the rearmost point on the driver's seat back, when the driver's seat is adjusted to its most vertical and rearward position, providing direct access for passengers to enter or depart the vehicle; in case of vehicles equipped with an ADSF-2 For vehicle of category X,, the front-most seat back is considered instead of the driver's seat back.”

Insert new paragraphs 2.28. to 2.29., to read:

“2.28.	"ADS feature of type 1 (ADSF-1)" means an ADS1 feature which includes an ADS fallback response requiring a fallback user.

2.2930.	"ADS feature of type 2 (ADSF-2)" means an ADS feature which does not include an ADS fallback response requiring a fallback user.

2.31. 		"Vehicle master control switch" means the device by which the vehicle’s on-board electronics system is brought from being switched off, as in the case where a vehicle is parked without the driver being present, to a normal operation mode (i.e. the vehicle status is "ready to drive").”

Insert new paragraph 5.3., to read:

“5.3.	For vehicles whilst while an ADSF-2 is active while carrying occupants, a door closure warning system shall transmit a logical signal to the ADSF-2, provided the ADS complies with the technical requirements of UN Regulation No. [XXX] applicable to the scope of this UN Regulation.”

Paragraph 6.3.2., amend to read:

6.3.1.1. 	If fitted, a full locking system may only be activated when the ignition key or vehicle control master switch is not in the engine running mode and shall be installed in combination with at least one of the following measures:

Paragraph 6.3.2., amend to read:

“6.3.2.	Rear Side Doors

Each rear side rear door shall be equipped with at least one locking device which, when engaged, prevents operation of the interior door handle or other interior latch release control and requires separate actions to unlock the door and operate the interior door handle or other interior latch release control.

For vehicles whilst while an ADSF-2 is active while carrying occupants, all doors adjacent to a seating position suitable for the transport of children shall be equipped with such locking device.”

Paragraph 6.3.2.1., amend to read:

“6.3.2.1.	The locking device may be a:

(a)	A Cchild safety lock system; or

(b)	A lock release/engagement device located within the interior of the vehicle and readily accessible to the driver of the vehicle or an occupant seated adjacent to the door; or

(c)    	A lock release/engagement device readily accessible to the occupant adjacent to the door, or

(d)	A lock release/engagement device controlled by the ADS for vehicles whilst an ADSF-2 is active and while carrying occupants, provided the ADS complies with the technical requirements of UN Regulation No. [XXX]. Means shall be installed to facilitate the egress of all occupants in an emergency case.”

Paragraph 6.3.2.2., amend to read:

“6.3.2.2. 	Either system described in paragraph 6.3.2.1. (a), (b) and (bc) shall be permitted as an additional locking feature. For vehicles whilst an ADSF-2 is active and while carrying occupants, either system described in paragraph 6.3.2.1. (a), (c) and (d) shall be permitted as an additional locking feature.”

Annex 1, insert new paragraph 9.1., to read:

“9.1. 	Vehicle is equipped with ADSF-2? 	Yes/No/Optional”

Annex 4, paragraph 2.2.3.3., amend to read:

“2.2.3.3. 	Transverse setup 1. Orient the vehicle or body in white so that its transverse axis is aligned with the axis of the acceleration device, simulating a driver left hand-side impact.” an impact on one side of the vehicle.

Annex 4, paragraph 2.3.3.3., amend to read:

“2.3.3.3.	Transverse setup 1. Orient the door subsystem(s) on the acceleration device in the direction of a driver left hand-side impact. an impact on one side of the vehicle”

II.		Justification

See paragraph 0. in the proposal.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.